| ARGENTINA |  |
| --- | --- |
| 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? 9.0 Are there any relevant experiences that you wish to share with IALA and other VTS Centres? | Through triangular green icons. Yes. The contact by radio and the use of charts decreased. The VTS Centre is operated with greater security. The space had to be adapted and the personnel trained. The behaviour of the ship in restricted waters is best known; the operators make decisions with better knowledge of the situation of ships making traffic more dynamic. We would like to report that Prefectura Naval Argentina has 13 AIS stations throughout the coastal stations, together with the communication system, Radar System and Security video cameras. This allows the Maritime Authority to provide maritime security and safety which is required by international bodies. |
| AUSTRALIA |  |
| Adelaide 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? | Yes – added functions Limited. Needs more training |
| Brisbane 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? | Symbol only. 10 targets as AIS capable Yes |
| Cairns 4.4 Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? | An AIS symbol differentiates vessels where AIS is currently the primary sensor from radar or Inmarsat C . The past tracks of vessels are differentiated by the position source of each individual position received (radar, AIS and Inmarsat C) being colour coded. There has been no appreciable impact on the workload of VTSOs to date. No |
| Fremantle 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? | All displayed with triangle Some reduction of communications Given operators greater situational awareness |
| Gladstone 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? 9.0 Are there any relevant experiences that you wish to share with IALA and other VTS Centres? | Yellow diamond on top of radar target No Rely on AIS to provide more accurate positions, courses, speeds etc than radar. Very good when use integrated with other systems |
| Hay Point 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? | Different shaped symbol No Yes, More updated information |
| Melbourne 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? | Not yet but with right software is likely to |
| Port Kembla 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? | Vessel icon showing broad direction and time trail tracks – PKPC system toggles to enhance target information display are yet to be confirmed and activated. No Generally yes |
| Reef 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? 9.0 Are there any relevant experiences that you wish to share with IALA and other VTS Centres? | An AIS symbol differentiates vessels where AIS is currently the primary sensor from radar or Inmarsat C The past tracks of vessels are differentiated by the position source of each individual position received (radar, AIS and Inmarsat C) being colour coded. There has been no appreciable impact on the workload of VTSOs to date. AIS has complimented radar and Inmarsat C to provide enhanced close track monitoring capabilities. The questionnaire is a great initiative and should provide some useful information on the status of AIS deployment and the functionality being utilised from the shore. With crystal ball gazing it may provide the basis for a further, more detailed, questionnaire. |
| Tasmanian Ports 4.4. Please describe how AIS targets and tracks are displayed? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? 9.0 Are there any relevant experiences that you wish to share with IALA and other VTS Centres? | Vessel icon with red vector and paste track painted on screen for 60 mins. Yes – better scheduling for Port Operations available. With further development of AIS, our business of Port Control will be enhanced. |
| BELGIUM/NL |  |
| Scheldt 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? 9.0 Are there any relevant experiences that you wish to share with IALA and other VTS Centres? | AIS only tracks in the colour pink, radar tracks only in white, radar tracks with AIS + after the ships name Yes especially when there is a conflict between radar tracks and AIS tracks because of wrong info from the vessel Identifications are faster and better and all the things I mentioned before What the authority needs are sanctions. A few captains/owners of vessels don’t repair or don’t show the accurate AIS info. A traffic leader can only inform the vessel but not order to do the right thing. |
| BELGIUM |  |
| Ghent 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? 9.0 Are there any relevant experiences that you wish to share with IALA and other VTS Centres? | As per type + dangerous good o/b. No Visualisation adds to assessment of traffic Very cost effective means; Need to expand AIS to all nautical traffic, including barges and leisure craft. |
| Wandelaar 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? | Different colour and shapes to the radar tracks. No More accuracy in ETA prediction. |
| BERMUDA |  |
| 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? | No Assists in the establishing of initial communication with ships detected. |
| CANADA |  |
| Halifax 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? 9.0 Are there any relevant experiences that you wish to share with IALA and other VTS Centres? | AIS targets displayed as triangle. No No In areas where our radar has trouble (i.e. small targets, close quarter situations, areas where radar targets become distorted) AIS seems to do a good job. Impressed with range of AIS. |
| Les Escoumins 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 9.0 Are there any relevant experiences that you wish to share with IALA and other VTS Centres? | AIS targets are displayed as triangles Yes – we keep more of a constant vigil on our screens and intervene when the situation necessitates. A very good tool. It allows us to make better decisions for all of our sectors, and not only those under radar coverage. On the other hand, we can’t forget that radar data is much more reliable since AIS information is dependent on the correct operation of shipboard equipment. |
| Montreal 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? | AIS targets are displayed as triangles. No No – However, AIS allows us to provide much more precise information and to have a better traffic picture of the non-radar areas. |
| Placentia 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? | Standard IALA approved icons No No |
| Quebec 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? 9.0 Are there any relevant experiences that you wish to share with IALA and other VTS Centres? | AIS targets are displayed as triangles No No The traffic situation is better understood, which supports any intervention required by the VTS operator. |
| Rivière-au-Renard 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? | AIS targets are displayed by triangles. Yes – we must exercise greater vigilance in the detection of non-identified targets as well as the movement of suspect ships. Yes – AIS permits us to refer to ship movements in real-time, as opposed to simulated tracks. |
| Sarnia 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? 9.0 Are there any relevant experiences that you wish to share with IALA and other VTS Centres? | Different target shape to identify AIS as source. A small triangle represents AIS target. Yes. It has increase workload as there have been instances where erratic behaviour of observed AIS traffic has required involvement of VTSO. No but keep this in context of having only 1 test bed site. Our AIS test site has provided useful information in critical situations and after accidents. It is an extremely important tool in VTS operations. |
| Tofino 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? 9.0 Are there any relevant experiences that you wish to share with IALA and other VTS Centres? | Standard IALA icons No Provides the ability to identify inbound vessels on radar prior to establishing communications with these vessels at the required check in points. Overall the addition of AIS improves the level of service that we can provide the mariner and will ultimately positively affect the workload and communications of the MCTSO as procedures are established using AIS to its full capability. |
| CHILE |  |
| 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? | They are displayed by different colours according to their navigation status and the shape of these are represented according to IALA’s recommendations. The operator has more time available to perform other activities. |
| CHINA |  |
| Chengshantou 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? | All AIS targets are displayed with a triangle. No No |
| Dalian 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 9.0 Are there any relevant experiences that you wish to share with IALA and other VTS Centres? | Read AIS data and create a track same as radar track, and correlate with radar tracks. No Ships can know each other through AIS, and reduce the VHF communication time. AIS helps VTS operators recognise the ships and enhance the efficiency of VTS. There are many incorrect static information comes from the bad inputting of the ship. And the reliability of AIS device (include other voyage device linked to AIS) should be enhanced. |
| Guangzhou 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? | A rectangle􀀁􀂴 stands for a radar target. And a diamond 􀂲stands for an AIS target.􀀁 Yes, Tracking a target with AIS is more stable and reduce some VHF workload. Not yet we decide the vessels with AIS equipment will not report to VTS Centre when passing report line. |
| Huanghua 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? | A diamond stands for a target with AIS while a circle stands for a target without AIS. YES reduce the level of VHF radio communication In some extent due to the reduction of operator workload. |
| Lianyungang 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? | Different colour or symbol for different type of Vessels. No No |
| Ningbo 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? | All AIS targets are displayed with a triangle. Yes, a result of a reduction of VHF radio Communications No |
| Xiamen 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? 9.0 Are there any relevant experiences that you wish to share with IALA and other VTS Centres? | Target displayed prefix “+” , track displayed with a triangle. No Some extent. It changes the manner. It is easy to locate the ship as reporting. Investigation indicated that the voyager and pilot use AIS in identification of the other ship. Most of ship officer had not been trained on AIS operation. Some ships installed the AIS but did not apply for the MMSI number. So they input MMSI number randomly. |
| DENMARK |  |
| Great Belt 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? 9.0 Are there any relevant experiences that you wish to share with IALA and other VTS Centres? | Triangle with course and speed vector Heading and ROT, if this is available No, the reduction in VHF communications is replaced by checking AIS data. Not beyond that mentioned in part 7 From our point of view AIS cannot be used as stand alone source for information. ID must be verified by VHF Position must be verified by VHF or DF Draught must be verified by VHF Proper working AIS network might improve NAS given by VTS This questionnaire is a summary of input from all VTS operators, supervisors and manager at VTS Belt Traffic |
| ESTONIA |  |
| Tallinn 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? | Vessel’s symbol according to length No |
| FINLAND |  |
| 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? 9.0 Are there any relevant experiences that you wish to share with IALA and other VTS Centres? | They are integrated so, that operator see only one track. Operator gets also information if there is something wrong in the AIS information. Yes -> AIS reduced workload -> VTSO can use more time for traffic surveillance Not yet, but we are planning changes to reporting and services in general. AIS has improved Finnish VTS-work very many ways. At the moment we are still in situation where we are planning our future services (via AIS) and our development in VTS. However it is clear that we can use more “power” to traffic surveillance and we need to use less “power” to reporting and some other routine procedures. We like to develop more this excellent tool. One way is to develop common understanding in IALA’s VTM-committee, how to develop AIS for VTS/SRS purposes. One important thing is that maybe we can have vessels route plans through AIS in the future. Route plan in electronic format opens very many interesting “doors” for future VTS/VTM development. |
| GREECE |  |
| Piraeus Rafina Lavrio 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? 9.0 Are there any relevant experiences that you wish to share with IALA and other VTS Centres? | Use of rectangle scheme over the track Yes. Less workload due to the decrement of required communications. No In order to improve the received AIS data through AIS contact data forms, that would lead to the integrity and accuracy of the offered VTS services, it would be wise to be determined in terms of regulations, that every vessel or target that is detected by radar will be obliged to be equipped with an AIS device. The relevant position information and accuracy through AIS text messages must be obligatory and reliably transmitted by all the users. Otherwise, the Traffic Organisation and Navigational Assistant Services cannot be improved. |
| ICELAND |  |
| Reykjavik 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? | Uniform Display No No |
| IRELAND |  |
| Commissioners for Irish Lights 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? | Generally no discrimination No No |
| Cork 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? 9.0 Are there any relevant experiences that you wish to share with IALA and other VTS Centres? | A circle for unidentified radar tracks. A diamond for AIS tracks Only one operator. Slightly reduced workload. We are using the benefits in all areas of the organisation. All persons with a legitimate requirement now have access to live data regarding the position of vessels. This reduces the requirement to contact the VTS operator. I am also harbour master of the port of Bantry which has low traffic levels and no VTS. The AIS is now set up to automatically send an SMS message to the harbour master when a vessel enters or leaves the port limits. This is a feature which greatly enhances safety as prior to this vessels could enter the bay undetected. |
| JAPAN |  |
| 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? | The size of the AIS target is set to three stages according to the length of the ship. Please see attached fig 2 and fig 3. Yes. Workload of VTSOs for providing information has increased to communicate with SOLAS vessels installed AIS. Yes. AIS can detect vessel’s movements more widely than radar. The function has produced the improvement of efficiency in addition to safety. |
| LATVIA |  |
| Liepaja 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? | AIS targets are displayed as tactical symbols with an added MMSI number/ships name. Target’s course and speed could also been observed because of directional vector is displayed No No |
| Riga 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? | AIS targets are displayed as tactical symbols with an added MMSI number/ships name. Target’s course and speed could also been observed because of directional vector is displayed. No No |
| Ventspils 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? | Standard No YES, it helps to check ships on arrival much faster |
| LITHUANIA |  |
| Klaipeda 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? | Different colour for type of vsl No Yes – Traffic Image accuracy |
| NETHERLANDS |  |
| Amsterdam 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? | White all observed targets, Yellow as target is activated for plot and/or info labels Yes, better information service of the VTS especially for crossing traffic and early identification of contacts without radio contact. Yes, by early identification of contacts, vessels can be contacted directly. Mainly of important use for the many crossing traffic of the DWR. |
| Delfzijl and Eemshaven 4.4. Please describe how AIS targets and tracks are displayed? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? | They are displayed by name and by clicking on them you get more info Yes, we know more in advance and are more accurate |
| Den Helder 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 9.0 Are there any relevant experiences that you wish to share with IALA and other VTS Centres? | As another type of Letter (Capitol) No If ships use AIS more accurate and format is more complete it will help to fill databases like SafeSeaNet. If there is still a lot of traffic (most of it) that has no AIS obligation, voice communication will not reduce. |
| Dordtrecht 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? | AIS targets are labelled with an UPPERCASE label Not yet determined Not at this moment, AIS is additional |
| Rotterdam 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 9.0 Are there any relevant experiences that you wish to share with IALA and other VTS Centres? | Visualisation is equal to radar targets and tracks. Specific AIS information is shown in the labels: # in front means AIS-only information + behind means AIS information available and not suppressed by the VTS-system − behind means AIS information available but suppressed by the VTS-system (usually because errors in the information are suspected) All received AIS information of a particular ship can be shown in a pop-up window. The VTS-display indicates which areas of the port are covered by radar. Too early to determine. If VHF radio communications will indeed increase, the workload will rise. It is not quite clear yet whether our problems are the result of inaccurate information coming from the ships, or the result of misinterpretation within the shore-based systems. |
| Scheepvaart Begeleiding 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? | Different colours – Nav data AIS & VTS pictures does not give correct position No |
| Scheldt 9.0.Are there any relevant experiences that you wish to share with IALA and other VTS Centres? | Trial stage. Evaluation due in 2007. Antenna support under construction Link wit other systems forseen |
| Scheldt (2) 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? 9.0 Are there any relevant experiences that you wish to share with IALA and other VTS Centres? | Integrated into radar display. Different colours for AIS/Radar targets Yes. Dealing with ???? information Not really AIS is OK, but integration into radar displays proved to be difficult (biggest problem is due to incorrect position information, resulting in double tracks) |
| NEW ZEALAND |  |
| Wellington 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? 9.0 Are there any relevant experiences that you wish to share with IALA and other VTS Centres? | AIS targets are displayed and whenever acquired leave a trail. All targets are of the same colour. No AIS has improved the quality monitoring and is user friendly. In our experience, AIS has been the best thing since radar. It’s far more cost effective than radar and can provide more information HOWEVER it does not cover all targets and like all electronic navigation systems it has its limitations. It can malfunction, be subjected to errors (most commonly when the input from the ship’s crew is incorrect or the data has not been updated). One of the ways it has helped the Communication Officers is by taking the guesswork out of the positions of specific target, as targets are accurately displayed and identified on a screen. This is particularly useful when a vessel is making her way to the Pilot Station and has failed to give the appropriate notice of her arrival. We have found that it is not uncommon that when a ship’s AIS is not transmitting, the VTS is the first to bring it to their attention. |
| NORWAY |  |
| Brevik Kvitsoey Fedje Varde 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? 9.0 Are there any relevant experiences that you wish to share with IALA and other VTS Centres? | All ship movement (regardless of source) presented on Traffic picture. Targets based on AIS signal presented with diamond shaped centre of vector. Yes. Marginal reduction (blind zone procedures is skipped) Identification of radar tracks is simplified. For conventional VTS mission procedures to be reconstructed to take into account not only the increased information available to VTS centre, but also addressing the increased information already available onboard. For traffic monitoring and response mission AIS is the element enabling the mission. In our experience there are quite a lot of ships, that are poorly equipped when it comes to presenting the received AIS information onboard. A full benefit of AIS in the VTS service can not be gained before this issue is further addressed and regulated. The VTS operator has no guarantee of receipt of text messaging or binary messages unless the ship replies. With establishment of new VTS’s one should have the option to include both text messaging and binary messages. |
| OMAN |  |
| Salalah 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? 9.0 Are there any relevant experiences that you wish to share with IALA and other VTS Centres? | No Yes. By monitoring Vsl information on AIS, saves on VHFcomunication and logging details. Linking VTS stations regionally/Internationally by suitable means would offer better vessel safety and monitoring of vessels especially in emergencies. |
| POLAND |  |
| Gdynia 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? 9.0 Are there any relevant experiences that you wish to share with IALA and other VTS Centres? | Fused targets are displayed Yes. Not considerably but e.g. safety related messages feature is used. Introduction of the AIS outside the VTS area imposed additional duties on VTS operators which are now also responsible for additional traffic monitoring. VTSOs have to revise correctness of the AIS data in comparison to VTS databases. AIS has improved detection and tracking of the vessels within and outside VTS area. It has shortened voice communication with vessels. Due to the fact that content of the AIS data should be frequently verified VTSO workload has extended. AIS is an important sensor used for the Vessel Traffic Monitoring outside the VTS area. In Poland MarSSIES – Maritime Safety and Security Information Exchange System is used for the purpose of the national traffic monitoring and exchange of safety and security related data. AIS Display in the system is an important module which gives allied services access to the comprehensive traffic overview. AIS data is also important in crisis management at sea, it supports pollution combating, polluter tracing or SAR operations co-ordination. Not only it gives a detailed historical track of the vessels but also serve as a tool for statistical analysis. Due to the fact that some of the static or voyage related AIS data are incorrect VTSO workload has extended. |
| Szczecin 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? 9.0 Are there any relevant experiences that you wish to share with IALA and other VTS Centres? | Fused radar/AIS targets are displayed Yes . Not considerably but e.g. safety related messages feature is used. Introduction of the AIS outside the VTS area imposed additional duties on VTS operators which are now also responsible for additional traffic monitoring. VTSOs have to revise correctness of the AIS data in comparison to VTS databases. AIS has improved detection and tracking of the vessels within and outside VTS area. It has shortened voice communication with vessels. Due to the fact that content of the AIS data should be frequently verified VTSO workload has extended. AIS is an important sensor used for the Vessel Traffic Monitoring outside the VTS area. AIS Display in the system is an important module which gives allied services access to the comprehensive traffic overview. AIS data is also important in crisis management at sea, it supports pollution combating, polluter tracing or SAR operations co-ordination. Due to the fact that some of the static or voyage related AIS data are incorrect VTSO workload has extended. |
| PORTUGAL |  |
| Lisbon 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? | Only a different shape (tracks are circled, AIS are rhombus) Yes. Less radio contacts for identification No |
| Lisbon (2) 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? | Symbology is available and used for this Setubal VHF traffic decreased with AIS and consequently VTSO’s workload There is no information because VTS radars and AIS equipment were installed simultaneously |
| RUSSIA |  |
| Nakhodka 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? 9.0 Are there any relevant experiences that you wish to share with IALA and other VTS Centres? | Triangle Marks and target tracks Yes. A reduction in VHF radio traffic. In a moderate way AIS facilitates ship identification in a significant way. AIS assist in ship tracking beyond radar ranges and in radar blind areas. |
| Vladivostok 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? 9.0 Are there any relevant experiences that you wish to share with IALA and other VTS Centres? | Triangle Marks and target tracks Yes. A reduction in VHF radio traffic. In a moderate way AIS facilitates ship identification in a significant way. AIS assist in ship tracking beyond radar ranges and in radar blind areas. |
| Rosmorport 4.4. Please describe how AIS targets and tracks are displayed? 9.0 Are there any relevant experiences that you wish to share with IALA and other VTS Centres? | Targets are displayed by a triangle. Tracks of AIS targets are not displayed. A VTS has been operating for a month time only so we have no any relevant and important experiences to share but I’m sure that we will soon. |
| SINGAPORE |  |
| Singapore 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? | No The AIS has enhanced the safety of navigation by providing instant identification of vessels to both other vessels and shore authorities. It has also cut down communications on VHF fro identifying vessels. |
| SOUTH AFRICA |  |
| National Ports Authority 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? | Different symbols when AIS correctly installed, usually merged with radar target. Positive impact – vessels arrive already identified and displaying stat & dyn info Yes, improved detection range (average 60 miles) and this results in more efficient scheduling and planning of resources (berths, pilots, tugs, berthing services, etc). |
| SWEEDEN |  |
| West Coast / Gothenburg 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? | Tracks are not displayed, Targets are orange and target name displayed in yellow. No No |
| TAIWAN |  |
| Kaohsiung 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? 9.0 Are there any relevant experiences that you wish to share with IALA and other VTS Centres? | Dangerous cargo vessel display in yellow, others and in black & white No Sometimes if we can’t contact a ship by VHF we will transmit message to her to call us back through VHF AIS is an aid only, not for navigation purposes. Since some of the information can be changed by users, ship’s name and position are not dependable. We had better check. AIS information with other monitors to make sure the information. |
| UNITED KINGDOM |  |
| Aberdeen 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? 9.0 Are there any relevant experiences that you wish to share with IALA and other VTS Centres? | Normal AIS targets shown in black, Domain breach/collision danger shown in red. No No AIS targets may lag behind the radar image. We have noticed that sometimes AIS targets can be displayed on AIS live but not on our Transas equipment. Transas are investigating this anomaly. |
| Dover 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? 9.0 Are there any relevant experiences that you wish to share with IALA and other VTS Centres? | Designated target plot & vector distinct from radar. Less manual’logging’ required. Main benefit is to enable forward planning of vessel entries & to identify precise location of individual vessels in congested waters. The uses that AIS provides for us is quite limited but nevertheless it has proved very beneficial. For instance when determining which ferry berths can be used at any particular time when ferries are running off schedule a scan of the near continental ports readily indicates which ferries are still alongside and therefore will not be requiring a berth in the near future. As an additional information source it is now considered invaluable. |
| Liverpool 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? | By a diamond shape Yes, requires monitoring and administration No, but it has improved our facility to identify targets early, thus provide better information particularly to our Pilots |
| London 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? 9.0 Are there any relevant experiences that you wish to share with IALA and other VTS Centres? | See attachment to questionnaire No significant change but VTS Operators have greater confidence in the image presented. Only insofar as the accuracy and confidence in presented data is much improved. The PLA now has 2 years experience in operating a fully integrated AIS system. AIS is used as the primary tracking medium. This is possible with the Sofrelog system as it is based on a singe primary tracking solution rather than a “resolved”, “best-fit” or “pool of errors” concept. Close cooperation is required with the system provider so that tracking algorithms and correlation processes are understood by operators and so that operators can recognise when AIS transmissions are in error. The ability to display representative ship shapes at the lower range scales greatly enhances situational awareness. Two features of the Sofrelog system that we feel adds to operator confidence and management of AIS are: - Tracking by AIS but AIS tracking data overlaid on radar video. - Facility for operator to easily inhibit AIS tracking when AIS errors are detected, or to clear picture once a vessel is berthed alongside. Future enhancements that we are urgently seeking to include are: - Automatic validation of Static data against internal databases and/or external registry database - Ability to recognise latency in AIS transmitted data. |
| Medway 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? | Diamond with straight line vector to indicate course made good and dotted line to indicate heading off set at end of vector is a rate of turn vector. Yes, with positive identification it has reduced the amount of time the VTSO spends in that task. Also the need to acquire lost targets has been reduced due to the improved tracking integrity No |
| Severn 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? | By a triangle with a dot in the centre Yes. Reduced workload Knowing a target's aspect can help to understand some of the decisions taken by mariners. |
| Sullom Voe 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? | Monchrome/ box with ships name on screen/ Tracks not displayed as would clutter up display. Target table available. No, AIS makes life easiler for VTS staff Yes. Much clearer traffic picture. Vessel identification no longer a problem. Better traffic management and harbour resources. |
| Sullom Voe (2) 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? | Vector and (speed) same colour Less inaccurate ETAs Yes – No helicopter surveillance |
| UKRAINE |  |
| 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? | Targets are displayed with the help of their own conventional symbols, routes – with the help of the dashed lines Yes, the workload of personnel has been reduced Yes, vessels identification has been improved |
| UNITED STATES |  |
| Berwick Bay 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? 9.0 Are there any relevant experiences that you wish to share with IALA and other VTS Centres? | Colors/shapes (e.g., ) No AIS has provided users with greater situational awareness, and it has allowed them to assist investigators and law enforcement personnel with targeting vessels for their specific purposes. AIS is not real-time and should never take the place of radar. AIS only applies to AIS-equipped vessels, and in these instances AIS has been useful. If an AIS user places his unit in a “moored” navigational status, he will report every three minutes. The user can get underway and continue to report every three minutes until the transponder detects three knots over ground. At that point, the unit will begin to report every two to ten seconds based on movement and speed. |
| Houston 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? | Use of different colors for type of vessel. Additionally special colors for vessels carrying CDCs and are deep/wide relative to the channel. Yes. Again, no quantifiable data at hand but operators are largely relieved of the burden of manually tracking participants. Eased operator workload of manual tracking and plot maintenance. |
| New Orleans 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? 9.0 Are there any relevant experiences that you wish to share with IALA and other VTS Centres? | use of different colours for type of vessels, navigation status No. Workload will be reduced when mariners and equipment reliably reflect correct vessel data such as AIS destination, composition of tow, unique vessel characteristics. No AIS is an essential tool for VTS managers. It requires the VTS community and the marine community to work together to get 100% compliance. Once this milestone has been reached then mariners will report AIS issues, rely on their equipment and that will make the task of the VTS community much easier. AIS coupled with ECDIS systems are the keys in making AIS the tool that mariners are looking for. Lower cost AIS only transponders are also essential for fishing vessels, larger recreational vessels and unique marine operations. One suggestions is that AIS receivers have a alert feature that allows mariners to punch a button on their AIS and it will alert all other AIS users that are at short range. That will alert these users that they need to listen up as someone close is using this feature as a last resort to get their attention. For example a deep draft ship calling a fishing vessel ahead of him. Under this scenario the mariner on the deep draft vessel would hit the alert button on his AIS unit after trying to raise the vessel on his VHF. The alert tone would be received on the fishing vessels transmit only AIS beacon and sound an alert tone in the wheel house. Hearing that tone the fishing vessel operator would turn up the volume of his VHF and scan the visible and radar horizon for the close aboard vessel that sounded the alert. Another thing not to forget is AIS is a tool and as such can be missed used/interpreted, as RADAR was when it was first required on ships. An all AIS environment will not detect those vessels not required to carry the device. It is important from a VTS stand point that RADAR, CCTV and any other detection device are important in the safe and effective management of traffic. |
| New York 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? 9.0 Are there any relevant experiences that you wish to share with IALA and other VTS Centres? | AIS targets are displayed uniformly. The VTS Sector Operator links the AIS with a Vessel Data Card created during the sail plan filing with the VTS. The colour and symbology of the icon is reflective of VTS determination of risk, cargo type or special operating conditions. Yes! The Sector Operator is constantly cross referencing the data with the radar return to insure data integrity and accuracy of VTS navigational advisories. Dramatic changes have occurred. The AIS functions provides critical data input that assists in the formation of navigational safety advisories and traffic routing solutions. It provides greater transparency in vessel operations and of vessel presence. It is not to be depended upon as a sole informational source. AIS is a useful tool used to assist the VTS in identifying vessels actions when operating in a VTS area. However, the VTS in Sector New York recognises that AIS data is easily manipulated and is subject to human input as well as human error. For these reasons AIS data is not referenced alone when the VTS provides navigational advisories or vessel routing solutions. AIS is utilised as a data point in conjunction with radar and CCTV so as to insure a high level of accuracy and data integrity. When AIS is required on all vessels and becomes a secure and tamper proof means of data sourcing will AIS begin to fully realize its potential in a VTS environment. |
| Port Arthur 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? 9.0 Are there any relevant experiences that you wish to share with IALA and other VTS Centres? | - Vessel name, course, and speed displayed. - CDC and EHC cargoes color coded to RED. - Deep draft vessels color coded. - Radar track only vessels color coded. - Different shapes for deep draft and shallow draft. - Tow description annotated (Wide, Long, load, empty, no. barges, etc) - Moored vessels minimized. - Pilot number on-board annotated (deep draft only) Yes. Fewer position reports. Easier access to basic data. Fewer requests by vessels for intermediate advisories. Voice communications are reduced in that the number of reporting points is reduced and in that vessels, using their on-board AIS units, may request fewer traffic advisories. Shore to ship AIS message capability has not affected the manner in which our VTS Centre operates. On-board AIS units are generally not user friendly, accessible, timely, or verifiable enough for effective shore to ship or ship to shore message traffic in lieu of voice communications. - OSV’s, certain towing vessels, and fishing vessels not required to carry AIS negatively impact Maritime Domain Awareness. - AIS is a valuable tool, but not considered a reliable replacement for radar, camera, or voice comms. - Relating to Message traffic through AIS units: - In this area, over 80% of the traffic is towing vessels with a single operator in the wheelhouse, neither manual data entry nor deciphering is practical or safe while driving. - Existing AIS on-board units are generally poorly designed for data entry and message traffic retrieving. - Voice comms to vessels provide instant verification and feedback. There appears to be no corresponding timely feedback through message traffic to vessels. - MMSI numbers should be permanently assigned to AIS units at the factory – like 406 Mz EPIRBS. - In this area, a single towing vessel will carry up to 6 barges. AIS data pertains only to the towing vessel and, except for the identifying data on the towing vessel, the information is incomplete. - Port Arthur VTS was a start-up VTS in February 2006. In the process of starting operations, we have progressed from operating with vhf only; to AIS and vhf only; to AIS, vhf, and radar; to AIS, vhf, radar, and camera. We have learned to minimize radio comms and enhance MDA for mariners by using a broadcast format for deep draft vessel movements. We have reduced required reporting points for all vessels IF they are AIS equipped. |
| Prince William Sound 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? | Different colours and shapes depending upon type of vessel, cargo, and status. Yes, less manual track manipulation is required with AIS data. No |
| Puget Sound 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? 9.0 Are there any relevant experiences that you wish to share with IALA and other VTS Centres? | Different icons for AIS than for Radar. Yes . Operators are not having to reacquire tracks to pass from one radar system to another. Tracking requires less work in this sense. However, there has been some increase in workload as operators now have to validate AIS data). Not a great deal. The most profound change has been in our ability to retain the same track without having to re-acquire between the many radar sits in our AOR. The other change is that we have continuous tracking of vessels in our non-radar areas. However, this has not significantly changed the “manner” in which we operate. It has significantly enhanced the quality of our services. The system is working but just as with any computer system; dynamic and voyage related information input by the vessels crew is what others see. If it is incorrect then it can potentially increase risk. A worldwide data standard needs to be used. Also: - AIS does not equal VTS. - AIS carriage requirements should align with VTS participation requirements. - Public vessels should be required to transmit AIS in VTS areas unless threat level dictates otherwise. - Radar still remains a vital tool. The need for high quality radar remains. Cannot invest solely in AIS infrastructure now. - We have not received significant pressure to reduce VTS calling requirements. In fact many users still like to communicate with the VTS and other vessels on the radio regardless of the presence of AIS data. |
| San Fransisco 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? 9.0 Are there any relevant experiences that you wish to share with IALA and other VTS Centres? | IAW CGVTS track display conventions with an “A” next to the ICON indicating an AIS track (as opposed to an “R” indicating a radar track) A reduction in workload in that it is easier to maintain an accurate and up to date display, less time is spent acquiring and correlating radar tracks, easier export of information within the VTS AOR for MDA and as well as for port partners and stakeholders. VTS SF doesn’t employ the Shore to Ship system. The implementation of AIS has greatly improved the capabilities of our VTS. It allows us to monitor traffic in real time; covers areas previous shrouded by radar blind spots; and provides readily available information. |
| St Marys River 4.4. Please describe how AIS targets and tracks are displayed? 7.0. Is there an impact on the workload of VTSOs, as a result of the introduction of AIS? 8.0. Has AIS changed the manner in which your VTS Centre operates? If so, how? 9.0 Are there any relevant experiences that you wish to share with IALA and other VTS Centres? | Shapes and colors are used to differentiate the various vessel characteristics and handling requirments. Yes to the positive less admin; less time spent mining for vessel movement data. Without AIS this VTS had to rely on verbal position reports. now we have real time positioning data to help us with tfc mgmt decisions. For vessels equipped with AIS, we’ve reduced the need for voice position reports improved our awareness of shipping activity. we’ve also expanded the VTS’ role in the regions ice breaking mission making that operation safer and more efficient. We look forward to the day when we will be able to connect our system with that of our Canadian and St Lawrence seaway partners to produce a seamless link of traffic mgmt data across the great lakes navigation system. We’ve used AIS data to resolve pollution and marine casualty cases; the data storage and playback capabilites enabled us to identify potential polluters, locate valuable equipment (lost rudder), and point out conning mistakes made by waterway users for training and investigative purposes. |