Annex 6

Performance requirements for
closed circuit television Service in VTS

Table of Contents

1.	INTRODUCTION	2

1.1	BACKGROUND	2

1.2	OBJECTIVES OF cctv EQUIPMENT	2

2.	REFERENCES	2

3.	DEFINITIONS	3

4.	FUNCTIONAL REQUIREMENTS	3

4.1	Characteristics of the CCTV	3

4.2	Reliability, accuracy, range, and resolution	3

4.3	Malfunctions, warnings, alarms and indications	3

5.	OPERATIONAL REQUIREMENTS	3

5.1	Ergonomy	3

5.2	Operational controls	3

5.3	Detection Performance	4

5.4	Recording and Replaying of Data	4

5.5	Software requirements	4

5.6	Availability	4

6.	DESIGN AND INSTALLATION	4

6.1	Durability and resistance to environmental conditions	4

6.2	Interference	4

6.3	Power supply	4

6.4	Site selection and Installation	4

6.5	Maintenance	5

7.	INTERFACING	5

8.	BACK-UP AND FALL-BACK ARRANGEMENTS	5

9.	SAFETY PRECAUTIONS	5

10.	MARKING AND IDENTIFICATION	5

11.	DOCUMENTATION	5

# INTRODUCTION

In 1997 the IMO Maritime Safety Committee adopted Regulations for Vessel Traffic Services (VTS) that have since been included in SOLAS Chapter V (Safety of Navigation) as Regulation 12.This Regulation specifies the responsibilities of contracting governments to arrange for the establishment of VTS in certain vulnerable areas under their control.

## BACKGROUND

The performance requirements placed on the CCTV service varies depending on traffic density, levels of VTS, special regional features and the coverage of the VTS area. The purpose of this document is to describe the general performance requirements of these sensors.

## OBJECTIVES OF cctv EQUIPMENT

CCTV information may be integrated into VTS applications to provide the operator a real-time assessment of the situation in the VTS area of responsibility.  Information collected from this equipment can be provided to ships to assist in assessing the waterway conditions.

# REFERENCES

Electronics Industry Association (EIA) Recommended Standard RS-170

Relevant SOLAS requirements; SOLAS Chapter V (Safety of Navigation) as Regulation 8-2

General resolutions, such as resolution A.694(17) - General Requirements for Shipborne Radio Equipment forming Part of the Global Maritime Distress and Safety System (GMDSS) and for Electronic Navigational Aids;

Resolutions with more specific requirements, such as:

NMEA 0183 – Standard for Marine Electronic Devices (IEC equivalent 60945)

IEC 529 "Degrees of protection provided by enclosures (IP Code)"

IEC 721-3-6 "Classification of environmental conditions, Part 3: Classification of groups of environmental parameters and their severities; Ship environment"

IEC 60945 "Maritime navigation and radiocommunication equipment and systems - General requirements, methods of testing and required test results"

# DEFINITIONS

As defined by Electronics Industry Association (EIA), Motion Picture Experts Group (MPEG) and Joint Photographic Experts Group (JPEG).

# FUNCTIONAL REQUIREMENTS

## Characteristics of the CCTV

Cameras may be used as a more cost effective solution to radar or in conjunction with radar as an additional sensor, depending on the level of risk.  CCTV may be effective in a limited area not covered by other sensors or provide supplemental information with other sensors (e.g., identification).

The camera may be capable of automatically tracking a vessel selected by the VTS Operator. In addition, there should be the possibility for the VTS Operator to manually de-select any acquired target and manually select another target and/or area, in order for the VTS Operator to perform specified monitoring tasks, such as pilot embarkation and disembarkation.

Where more than one camera is installed to cover a VTS area, it may be desirable for the output from multiple cameras to be provided in one composite picture.

VTS authorities should consider the need for low-light level, infrared, colour, intensified and laser-gated low light level, as well as digital image processing and video compression of CCTV installations.

Characteristics to be considered with regard to CCTV installations include resolution, focus uniformity, shading, lag/image retention, spectral sensitivity, blooming, and light sensitivity.

## Reliability, accuracy, range, and resolution

The reliability, accuracy, range, and resolution should satisfy the minimum requirements of the VTS and as determined by EIA and MPEG.

## Malfunctions, warnings, alarms and indications

Refer to relevant requirements of Resolution A.686(17) and Resolution MSC.39(63).

# OPERATIONAL REQUIREMENTS

## Ergonomy

Regarding CCTV controls, one should consider the VTSO position and the use of the CCTV.  It is desirable for the CCTV to be controlled from a single operator position (e.g., touch screen, mouse click, and joystick for CCTV slewing and automatic tracking).

## Operational controls

The camera may be controllable manually in pan, tilt, and zoom modes.  The extent of this control will be dependent on geography of the waterway, camera installation site, etc.  Camera housings are generally equipped with means for keeping the lens operational (e.g., heaters, lens wipers, protective coating, washers).

## Detection Performance

Depending upon the circumstances, cameras should allow the VTSO to identify the type and possibly the name of the vessels concerned.

The type of vessel should be capable of being identified normally at a minimum range of 3 NM from the camera location, but this will depend on the individual circumstances, including local topography. The identity of a vessel by shape, color and other features should be capable of being determined normally at a minimum range of 1 NM, but again this will depend on the individual circumstances. These ranges are based on a nominal visibility in excess of 10 nmi.

## Recording and Replaying of Data

The data should be recorded automatically and capable of being replayed, if required by the VTS Authority.  Replay of CCTV should not interfere with ongoing operations.  This may require a separate display system for playback.  The VTS Authority must determine the quality of recording and playback (e.g., frames per second, resolution, period).

## Software requirements

The software requirements for CCTV is a matter for the VTS Authority to determine.

## Availability

The requirements for the availability of CCTV is a matter for the VTS Authority to determine.

# DESIGN AND INSTALLATION

## Durability and resistance to environmental conditions

Electronics installed externally should be in an environmental enclosure .  As regards environmental conditions, the IEC requirements should be applied as far as relevant.

## Interference

CCTV equipment is industrial equipment and therefore complies with applicable international standards and regulations.  Refer to the IEC requirements (IEC 60945) as far as relevant.

## Power supply

IEC requirements should be applied as far as relevant.  In remote locations, authorities should consider use of alternative power (e.g., solar panels, wind vanes, etc.) in addition to generators and/or uninterruptible power supplies.

## Site selection and Installation

Requirements concerning the installation of CCTVs, wiring and the arrangement of the equipment in the VTS Centre should be determined.  Operational requirements will determine where CCTVs are to be located and how many are required.  Sites for CCTVs should be selected based upon optimizing the views of areas relevant to the VTS.  Other considerations include:

- Availability of power

- Lighting (natural and artificial light) ,

- Camera housing,

- Stability of the mounting and susceptibility to vibration,

- Lightning protection

- Maintenance and repair,

- Accessibility requirement and protection against theft and vandalism,

- Collocation with existing VTS, AtoN, or other suitable infrastructure

Note:  Special care must be given to the stability of the camera mounting and preventing glare from direct sunlight.  It may be appropriate to consider the use of software to mitigate unavoidable vibration.

## Maintenance

Requirements in addition to IMO Resolution A.694(17) concerning maintenance should be determined if necessary.

# INTERFACING

For the interfacing of CCTV services to VTS equipment, several different standards are in use  e.g.,  EIA, MPEG, JPEG.  A time stamp and source should be provided for recording and playback purposes.

# BACK-UP AND FALL-BACK ARRANGEMENTS

Redundant cameras of the same type are not normally installed in VTS.  However, different camera types may be collocated and can provide backup.

# SAFETY PRECAUTIONS

No specific safety requirements  in addition to resolution A.694(17).

# MARKING AND IDENTIFICATION

National and/or local regulations may require the posting of signs to notify the public that they are under surveillance.

# DOCUMENTATION

Documentation should be provided in accordance with the IALA VTS Manual.