Annex 4

Performance requirements for 
Radiocommunications in VTS

Table of Contents

1.	INTRODUCTION	2

1.1	BACKGROUND	2

1.2	OBJECTIVES OF RADIOCOMMUNICATIONS EQUIPMENT	2

2.	REFERENCES	2

3.	DEFINITIONS	3

4.	FUNCTIONAL REQUIREMENTS	3

4.1	Characteristics of Radiocommunications equipment	3

4.2	Malfunctions, warnings, alarms and indications	3

5.	OPERATIONAL REQUIREMENTS	4

5.1	Radiocommunications Coverage	4

5.2	Recording and Playback of Data	4

5.3	Availability	4

6.	DESIGN AND INSTALLATION	5

6.1	Durability and resistance to environmental conditions	5

6.2	Interference	5

6.3	Power supply	5

6.4	Site selection and Installation	5

6.5	Maintenance	5

7.	INTERFACING	6

8.	BACK-UP AND FALL-BACK ARRANGEMENTS	6

9.	SAFETY PRECAUTIONS	6

10.	DOCUMENTATION	6

# INTRODUCTION

In 1997 the IMO Maritime Safety Committee adopted Regulations for Vessel Traffic Services (VTS) that have since been included in SOLAS Chapter V (Safety of Navigation) as Regulation 12.This Regulation specifies the responsibilities of contracting governments to arrange for the establishment of VTS in certain vulnerable areas under their control.

## BACKGROUND

The performance requirements placed on the radiocommunications service varies depending on traffic density, levels of VTS, special regional features and the coverage of the VTS area. The purpose of this document is to describe the general performance requirements of this service.

## OBJECTIVES OF RADIOCOMMUNICATIONS EQUIPMENT

Radiocommunications equipment is typically integrated into VTS applications to provide the operator a real-time assessment of the situation in the VTS area of responsibility as well as a means to deliver timely services to VTS participants.  Information collected and disseminated via this equipment can assist in assembling the traffic image and in supporting safe navigation of the VTS area.

# REFERENCES

Relevant SOLAS requirements; SOLAS Chapter IV (Radiocommunications)

SOLAS Chapter V (Safety of Navigation) – Regulation 12

SOLAS Chapter V (Safety of Navigation) – Regulation 19

Resolution A.694(17) - General Requirements for Shipborne Radio Equipment forming Part of the Global Maritime Distress and Safety System (GMDSS) and for Electronic Navigational Aids;

IEC 529 "Degrees of protection provided by enclosures (IP Code)"

IEC 721-3-6 "Classification of environmental conditions, Part 3: Classification of groups of environmental parameters and their severities; Ship environment"

IEC 60945 "Maritime navigation and radiocommunication equipment and systems - General requirements, methods of testing and required test results"

# DEFINITIONS

As defined by ITU-R and IEC.

# FUNCTIONAL REQUIREMENTS

## Characteristics of Radiocommunications equipment

Radiocommunication is the central ingredient in the operation of VTS.  Radiocommunications links are used to collect position, safety, and general information from shipboard personnel and remote sensing devices. These links are also the primary means through which services are delivered to VTS participants. Components of the VTS radiocommunications equipment may include:

Very High Frequency (VHF) voice radio - It is common for the VTS to have its own independent VHF network for their use within specifically designated VHF Channels.  This network may comprise of one or more VHF Channels in different sectors of the VTS Area.  The VTS Authority may require specific VHF Channels to be designated by the National Radio Authority for specific types of operations.  The VHF equipment must comply with national and international regulations.

Medium Frequency (MF) and High Frequency (HF) voice and data – MF and HF may be used on a regional basis where long range communication is required.   The VTS Authority may require specific channels to be designated by the National Radio Authority for specific types of operations.  The equipment must comply with national and international regulations.

Radio Direction Finding (RDF) - VTS Authorities may use RDF receivers to determine which vessels may be transmitting.  In order to ensure accurate identification, the use of two or more separate RDF bearing stations are required.  All bearings should be automatically displayed on the VTSO display. The bearings should remain visible at least as long as the vessel is transmitting a signal. The use of RDF may decrease as AIS usage by VTS Authorities increases.  However, RDF is unlikely to become obsolete as, depending on national requirements, some vessels may not be equipped with AIS.  RDF is not suitable for continuous tracking.

Automatic Identification System (AIS) – See other IALA documents on this equipment (Recommendations A-123, A-124, A-126 and IALA Guidelines 1029 and 1030 on Operational and Technical aspects of AIS)

The shipborne equipment must meet the functional requirements of the relevant IMO performance standards and the ITU-R Radio Regulations.

## Malfunctions, warnings, alarms and indications

Refer to relevant requirements of resolution A.686(17) and resolution MSC.39(63).

# OPERATIONAL REQUIREMENTS

## Radiocommunications Coverage

VTS Radiocommunications shall be in accordance with ITU-R Radio Regulations.  Depending upon the circumstances, radiocommunication equipment should be capable of receiving signals from appropriately equipped ships.  VHF voice radio reception is dependent upon the line-of-sight distance between VTS receive site and the ship antenna heights.   This is also true with AIS, although there are performance differences between VHF voice radio and AIS.  As a minimum, the radiocommunications range should facilitate VTS ship communications before the ship enters a VTS area of responsibility.

## Recording and Playback of Data

Radiocommunications data should be recorded automatically and capable of being replayed onto a separate replay system, if required by the VTS Authority. A time stamp and source should be provided for recording and syncronised playback purposes.

## Availability

The requirements for the availability of radiocommunications equipment is a matter for the VTS Authority to determine.

# DESIGN AND INSTALLATION

## Durability and resistance to environmental conditions

Electronics installed externally should be in an environmental enclosure .  As regards environmental conditions, the IEC requirements should be applied as far as relevant.

## Interference

Radiocommunications equipment is industrial equipment and therefore complies with applicable international standards and regulations.  Refer to the IEC requirements (IEC 60945) as far as relevant.  Care must be taken when selecting antenna sites that adjacent equipment does not interfere with, nor is interfered with, VTS radiocommunications equipment.  Radiocommunications shall be in accordance with ITU-R Radio Regulations and national regulations.

## Power supply

IEC requirements should be applied as far as relevant.  In remote locations, authorities should consider use of alternative power (e.g., solar panels, wind vanes, etc.) in addition to generators and/or uninterruptible power supplies.

## Site selection and Installation

Requirements concerning the installation of radiocommunciation equipment, wiring and the arrangement of the equipment in the VTS Centre and in remote sites should be determined.  Operational requirements will determine where radiocommunications transceivers and antennas are to be located and how many are required.  Sites for radiocommunications equipment should be selected based upon optimizing the coverage of the VTS area.  .  Care must be taken when co-locating antenna sites that proper separation is maintained to avoid interference.  To avoid channel saturation, consideration should be given to subdividing the VTS area into communications sectors based upon channel use with adjacent sectors using separate channels.  Other considerations include availability of power, protection against vandalism, housing, and collocation with existing VTS, AtoN, or other suitable infrastructure

## Maintenance

In addition to the requirements of IMO Assembly Resolution A.694(17), siting considerations for radiocommunications equipment should address maintenance,repair, and accessibility requirements.

# INTERFACING

For the interfacing of radiocommunications services to VTS equipment, several different standards are in use, including the 4-wire E&M standard.

For the interface between a VTS and its users, VHF voice is the standard practice.  AIS data communications should follow ITU-R Rec. M.1371-1 and IALA A-124.

# BACK-UP AND FALL-BACK ARRANGEMENTS

The provision of redundant radiocommunications equipment is recommended.  Additionally, consideration should be given to an emergency mobile communications capability as a means to re-establish communications capabilities.

# SAFETY PRECAUTIONS

No specific safety requirements  in addition to resolution A.694(17).

# DOCUMENTATION

Documentation should be provided in accordance with the IALA VTS Manual.