Input paper: ENAV26-5.1.11.0

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM	□  ENG		□  PAP					X   Input

X  ENAV	□  VTS							□   Information

Agenda item			5.1.11

Technical Domain / Task Number 2	…………………………………

Working Group			WG3

Author(s) / Submitter(s)			KIM Juntae (Korean Register)

JANG Tae-Hoon (KMNET)

CHOI Seung-Hyun (Hanwha System)

Field testing results for VDES Testbed equipment 
in the Smart-Navigation Project

Summary

In the Smart-Navigation project, the plan to develop testbed for testing e-Navigation service data transmission using VDES communication from 2019 to 2020 was established. VDES testbed, which consists of Management system, Shore/ship VDES gateways and test service host/client, was designed and implemented in 2019, and the related information for the testbed and interface design was submitted as ENAV24-6.1.13 in IALA ENAV 26 meeting. In 2020, the testbed equipment were installed at the VDES field testing sites together with VDES shore station and ship station, and field tests at sea areas were conducted to verify the operability of various functions of the test bed, including the function of transmitting virtual e-Navigation service data through the VDES communication network.

This paper aims to provide the information on the results of development and field testing of the VDES testbed equipment conducted in Korean sea areas.

Purpose of the document

This document provides information on field testing results for VDES testbed equipment conducted in Korean sea areas in connection with VDES communication.

Related documents

ENAV24-6.1.13 The design of VDES testbed and gateway under Smart-Navigation Project

Background

Smart-Navigation Project

Smart-Navigation project funded by Korean Ministry of Oceans and Fisheries, was initiated in 2016, and is now underway for the development and construction of Korean e-Navigation testbed.

Korean Register has been leading developing and testing of VDES testbed equipment in the Smart-Navigation Project.

Figure 1. SMART-NAVIGATION Project

VDES Testbed building and operation

The VDES test network equipment was developed and tested according to the following schedule.

Design and Development : 2019. 01 ~ 12.

Installation : 2020. 01 ~ 06.

Operation and Test : 2020. 07 ~ 08.

The design, implementation and basic function tests for VDES testbed equipment were finished by 2019, VDES testbed equipment were installed in the VDES shore station located in Haenam area in Korea by June 2020, and also installed at the VDES ship station located at the navigation bridge of ship navigating between Mokpo port and Jeju port.

VDES test BED development and configuration for acutual sea test

Test bed architecture and Functions of each component

Figure 2. Configuration of VDES Testbed

Figure 2 shows the equipment configuration for field testing of the VDES test bed. The VDES Test bed consists of one VDES Test Center/Shore Station and two VDES Ship Stations.

- The VDES Management system provides device monitoring, configuration setting of VDES base station and gateway, traffic data storage, user interface, and ASM message generation.

- The e-Navigation service host device includes an MMS host simulator and can input virtual e-Nav service data.

- VDES shore gateway supports network connection and interoperability between VDES base station and e-Nav service host equipment. It functions a network protocol conversion and supports conversion of e-Nav service messages into VDES PI sentences.

VDES Ship Station includes VDES ship station, test e-Navigation service client, VDES ship Gateways and associated network devices.

- The test e-Nav service client receives the message provided by the service host and provides a user interface.

- The VDES ship gateway performs the almost same functions as the shore gateway..

VDES Ship Station #2 was virtually added to enable service testing and actually located together within VDES Ship Station #1. The e-Navigation service client #1 is connected to the VDES communication network, and the e-Navigation service client #2 is connected to the LTE communication network. This configuration is for testing e-Nav service data transmission test between different communication technologies and enable to test VDE messages transmission too.

The interface diagram of the test bed is shown in Figure 3.

Figure 3. VDES Test bed interface diagram

VDES Test Center / Shore station installation

Figure 4. VDES Test Center and VDES Base Station

The VDES Test center and the VDES Shore station were installed at the same place on the top of Mt. Duryun in Haenam area in the Republic of Korea.

The Test center consists of testbed management system, VDES shore gateway, and test e-Navigation service host equipment, and VDES shore gateway supports the function routing received messages to appropriate VDES Shore Station among connected multiple Stations.

VDES Shore Station consists of VDES Base Station Equipment, antennas and network devices. The VDES Base Station (KEVSS-100) developed by the Smart-Navigation Project provides ASM and VDE-TER communication functions including GPS receiving.

Ship station installation

Figure 5. Ship Station #1

The ship station was installed at the port side in the navigation bridge of M/V “Queen Marry” which is RoRo Passenger ship navigating between Mokpo port and Jeju port in Korea.

- VDES Ship Station Equipment (KEVMS) provides ASM and VDE-TER functions including GPS module.

- VDES Ship Gateway supports network connectivity between VDES Mobile Station and test e-Navigation service clients.

- The test e-Navigation service client includes the MMS client simulator, service data input and log collection function essentially required for testing. However, graphic display equipment to display S-100 based service data like ECDIS are not included in the testbed.

- LTE router is used to test the transmission of e-Navigation service data and text messages between two different communication means of VDE-TER and LTE.

Test Environment

VDES Shore station and test center was installed on the top of Mt. Duryun and VDES ship station and testbed equipment was installed in the M/V ‘Queen Marry’ which navigates between Mokpo Port and Jeju Port as shown in the figure 6.

During ship’s navigation, text message transmission tests and an e-Navigation service data transmission test were conducted using the testbed equipment.

Figure 6. Test sea area

field test list and testing results

Field test list

Table 1 shows the test items for the sea area test of the testbed equipment.

- Table 1. Functional Test Items of Sea Area Test

| Test identifier | Test Items | Applied Equip. |
| --- | --- | --- |
| PT-01 | Check whether the ship location is displayed on the map screen Check whether the moving path of the ship registered in the management system is displayed normally on the screen | Management System/ Client #1 |
| PT-02 | - Check whether the ASM message transmission record of the management system is displayed normally (Broadcast and Unicast) - Check whether the ASM message reception record of the ASM client is displayed normally - Check whether the ASM message transmission record of the ASM client is displayed normally - Check whether the ASM message reception record of the management system is displayed normally | Management System/ Client #1 |
| PT-03 | Virtual e-Nav service test data transmission and reception (Accident Warning Service) - Ship #2 Accident warning signal transmission - Confirmation of reception of warning signal from service host - Ship #1 warning signal reception and file download confirmation - Ship #1 View downloaded data | HOST/ Client #1, #2 |
| PT-04 | VDE message transmission and reception through heterogeneous communication network - Send VDE message including attachments from Ship #1 to Ship #2, Ship #2 receives messages through heterogeneous communication (LTE) - Send VDE message including attachments from Ship #2 to Ship #1 via heterogeneous communication (LTE), Ship #1 receives VDES messages through polling | HOST/ Client #1, #2 |

Ship location reception and map display test (PT-01)

Test details: PT-01 is a ship location reception and map display test. VDES Ship Station #1 transmits the ship's GPS location information to the VDES Shore Station in ASM message format at intervals once a minute. The received ASM message is delivered to the management system to record the location information of each ship position and display the ship's positioning information on the screen of the management system.

Test result: It was confirmed at real sea test that the ASM message receiving function and the map display function were successfully operated linked with the VDES communication network (Figure 7).

Figure 7. VDES Ship location indication and tracking

Transmission/reception of ASM short text message between management system and ship station #1 (PT-02)

Test details: The ASM short message input from the management system is transmitted from the VDES Shore Station to the VDES Ship Station #1 through the ASM channel, and the received message is confirmed by the Ship Service Client #1. Then Ship Service Client #1 sends a response message to the management system.

Test result: It was confirmed at real sea test that ASM short message input and transmission function works good (Figure 8 & Figure 9)

Figure 8. Screen for sending message from management system to ship station #1 and receiving ship station #1

Figure 9. Message transmission and management system reception screen from ship station #1 to Management system

Test maritime data standard (S-10x) data transmission and reception (Accident warning service) (PT-03)

Test details: PT-03 is the test that transmits and receives data from the test maritime data standard (S-10X), and virtually simulates a part of the accident-prone ship monitoring support service, one of the e-Navigation services. The accident-prone ship monitoring support service evaluates the risk of collision and aground based on the position information of the ship on land, provides information so that the ship can recognize the dangerous situation, and provides real-time risk for early response on land in the event of a marine accident. As a service for sharing situation information with related organizations, a service scenario that warns other ships of dangerous situations due to fire or floating objects on the sea was simulated. The flow of service message transmission in the test network is shown in Figure 10.

Figure 10. Message transmission flow according to accident warning service scenario

1) Service Client #2, which is connected to the LTE communication network, transmits a danger situation warning message that there is a collision dangerous object in the nearby sea area to the Service Host located in the shore station via LTE network. After received at the Test Service Host, it returns an Ack message to Service Client #2. At this time, the display of Service Client #2 shows a red alarm (Accident).

2) Service Host inquires the management system for the MRN(Maritime Resource Name) list of nearby ships within the set distance (3km) based on the ship location information having Service Client #2.

3) Service Host prepares to provide warning messages and image files to ships corresponding to the searched MRN list.

4) Service Client #1 connecting to VDES communication requests service data from the Service Host via the VDE-TER channel.

5) Service Host checks whether MRN of Service Client #1 is included to the searched MRN list, and if yes, the service host transmits a danger warning message together with image file to Service Client #1 via the VDE-TER channel.

6) When a dangerous situation warning message is received by Service Client #1, a warning message is displayed on the display, and after an image file is downloaded by user, the alarm status is recovered to the “Normal” status.

Test result:

- Based on the simulated test scenario, it was confirmed that the provision of virtual e-Navigation service data works good via the VDES communication network.

- In addition, it was confirmed that the routing function of service host works effectively from the Service Host to the destination Service Client regardless of the connected communication type (VDE-TER and LTE) using MRN information. (Figure 11 & Figure 12)

Figure 11. Service client #2 dangerous situation warning transmission and warning status change

Figure 12. Service Client #1 Change the warning status of dangerous situations and check the provided data

Message transmission test via heterogeneous communication network between VDE-TER and LTE(PT-04)

Test details:

    - Service Client #1 using VDES communication sends short text messages and file attachment to Service Client #2 using VDE-TER channel though the service host.

    - Service Host checks the recipient(MRN information) of the received message, and if recipient is service client #2, service host prepares to deliver it to Service Client #2.

    - Service Client #2 using the LTE network sends a polling request for any received message to the Test Service Host, and then, the service host delivers the message received from Service Client #1 to Service Client #2. The sequence of test message delivery is shown in Figure 13.

Test result: It was confirmed that the message and file transfer function between heterogeneous communication (LTE-VDES) works successfully through real sea area test. (Figure 14 & Figure 15)

Figure 13. VDES / LTE message transmission and reception

Figure 14. Client #1 file upload and transfer

Figure 15. Client #2 message reception and file download

Analysis of VDE-TER Message transmission performance

In the above PT-05 test, we reviewed the message transmission efficiency according to various data sizes of attachment files. When sending messages containing attachments from service client #1 to the service host via VDE Channel Link ID 17(100kHz BW, PI/4-QPSK), the results from Table 2 were obtained by measuring each message transmission time.

- Table 2. VDE Message transmission performance with attachment file

| Test Case No. | attached file size(A) (byte) | comm. time (B) (sec) | A/B (byte / sec) |
| --- | --- | --- | --- |
| 1 | 2852 | 04.760 | 599 |
| 2 | 3796 | 06.888 | 551 |
| 3 | 4745 | 06.973 | 680 |
| 4 | 5694 | 08.477 | 672 |
| 5 | 6643 | 10.108 | 657 |
| 6 | 7592 | 09.358 | 811 |
| 7 | 8541 | 11.354 | 752 |
| 8 | 9800 | 11.085 | 884 |
| 9 | 10749 | 12.816 | 839 |
| Average |  |  | 716 |

The average transfer rate from VDES shipping station to coast station is around 716 byte/sec(5,728bps) as measured by increasing the size of the attachment from 3 Kbyte to 11 Kbyte. Attachments were ASCII Text files in this test.

Conclusion

VDES Testbed equipment for testing e-Navigation service data transmission have been designed and developed in the Smart-Navigation project in last year. When we started this research program, we met the following unsolved issues.

How can VDES messages communicate between VDES and TCP/IP based service applications?

How should we design gateway protocol to interconnect VDES and service application referring to international standard such as IEC 61162 series?

How can the exchange of VDES message between VDES communication network and other communication means(e.g. LTE) make enable?

Is TCP/IP connection valid via VDES communication?

Is the provision of e-Navigation service enable via VDES communication?

Although allowed time for research was not enough for us, but it was very meaningful that we found solutions and operable design to address the above issues since the resolution of these issue is essential for future service provision using VDES.

This year, field tests were carried out in real sea areas to verify the functionality and operability for VDES testbed equipment which has been developed in 2019. And it was confirmed our design philosophy works good according to test results. And we could learned the below lessons from our field testing.

VDES Messages can communicate easily between VDES and service application using VDES gateway. Therefore, IALA may consider the introduction of VDES gateway concept and protocol design considering various service applications(most service applications are using TCP/IP protocol and HTTP(S)).

Direct TCP/IP connection may not be suitable to be applied in VDES communication in the context of inefficiency of data overhead and difficulty to maintain continuous TCP connection.

MRN(Maritime Resource Name) can be used for communication address for service application client instead of MMSI and IP/port address and it also support the exchange of VDE message between VDES and other communication networks using additional routing solutions. It is noteworthy that MRN may be used for communication address for routing among service clients.

It is believed that VDES communication will work good for future e-Navigation service provision on condition that service data sizes are reasonably small to be suitable for VDES performance, in this regard, more test and research for service data transmission will be necessary to figure out optimized data sizes which can be properly transmitted through VDES.

References

- IALA Guideline G1139 Ed.2: The Technical Specification of VDES

IEC 61162 Series: Maritime navigation and radio communication equipment and systems — Digital interfaces

Maritime navigation and radio-communication equipment and systems-Digital interface-Part 450

D1.29-MCP-VDES-inter-operability-report

MMS_Specification_0.8.3 on Website: https://maritimeconnectivity.net/

Action requested of the Committee

The Committee is requested to review the information and take appropriate action.