Input paper: ENAV26-5.1.11.1

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM		□  ENG		□  PAP						□  Input

X  ENAV		□  VTS								X  Information

Agenda item				5.1.11

Technical Domain / Task Number 	…………………………………

Working Group				WG3

Author(s) / Submitter(s)			KIM Wonyong(COMESTA),  KIM Jaehyun(Ace Antenna),

KIM Jeonghyun(I-Storm), KIM Juntae(Korea Register),

# VDES field testing results at sea in the Smart-Navigation

# Summary

COMESTA, Ace Antenna, I-Storm and Korean Register have been developing the VDES base station and Ship station in the Smart-Navigation project in the Republic of Korea.

Our VDES communication equipment are complied with the IALA G1139 Ed.2 specification, and field tests were carried out at sea to verify the actual performance in maritime environment this year.

The VDES field tests were conducted in Korean sea areas from June to August 2020 and PSR(Packet Success Ratio) was measured for ASM and VDE with the target distance up to 100KM.

## Purpose of this document

Recognizing the important need to verify the performance required for the VDES communication operation in ports and coastal areas, this paper provides the information on the VDES field testing results at sea which were carried out in the Republic of Korea this year.

## Related documents

IALA Guideline G1139 Edition 2(2018/12), The Technical Specification of VDES

ENAV24-6.1.12, Initial VDES sea trial and test plan under Smart-Navigation Project

# Background

Smart-Navigation project funded by the Korean Ministry of Oceans and Fisheries, was initiated in 2016, and is now underway in the goal of the development and construction of Korean e-Navigation testbed.

COMESTA, Ace Antenna, I-Storm with the support of Korean Register has been developing VDES Communication Equipment in the Smart-Navigation Project.

Figure 1  SMART-NAVIGATION Project

# OVERVIEW of vdes base station and Ship station

## VDES Base Station (KEVSS-100)

The VDES base station includes ASM, VDE-TER, and GPS, and is complied with IALA Guideline G1139 ed.2 issued in December 2018 by IALA.

The below Figure 2 shows the external appearance and internal configuration of KEVSS-100. The main hardware configuration consists of RF front end subsystem, baseband subsystem and control subsystem. In addition, GPS module are included to enable independent operation by itself.

Figure 2  KEVSS-100

Basic specifications of KEVSS-100 are as below;

[Features of KEVSS-100]

- Software Defined Radio (SDR) Solution

- Complete VDES solution

- Two channel receiver

- VDES (ASM + VDE-TER)

- Upgradeable and future proof

- Output power: 12.5W

- RF Range covered: 156.025 - 162.025 MHz

- Interfaces:

- Ethernet and serial ports

- LCD touch display and keypad

- USB Debug port

## VDES Ship Station (KEVMS)

The VDES Ship Station (KEVMS) integrates the functions of ASM, VDE-TER, and GPS, and is complied with IALA Guideline G1139 ed.2 issued in December 2018 by IALA.

The below Figure 3 shows the external appearance and internal configuration of KEVMS. The main hardware configuration consists of RF front end subsystem, baseband subsystem and control subsystem. In addition, GPS module are included to enable independent operation by itself.

Figure 3  KEVMS

Basic specification of KEVMS for ship station is as below;

[Features of KEVMS]

- Software Defined Radio (SDR) Solution

- Complete VDES solution

- One channel receiver

- Fully compliant VDES (ASM + VDE-TER)

- Upgradeable and future proof

- Output power: Max 12.5W

- RF Range covered: 157.1875 - 167.2875 MHz, ASM 1,2

- Interfaces:

- Ethernet and serial ports

- LCD touch display and keypad

- USB Debug port

# VDES filed test OVERVIEW

## General

The VDES base station was installed at Mt. Duryun in Haenam area in the Republic of Korea, and the VDES Ship station was installed at M/V ‘Queen Mary’, which is Ro-Ro Passenger ship navigating between Mokpo Port and Jeju Port. The purpose of this field test is to verify transmission and reception function of the developed VDES, and to measure the effect of multipath propagation and coastal areas of Korea, and to check the interoperability of AIS interference.

- Measurement of ASM PSR (Packet Success Ratio): Link ID 5

- Measurement of VDE-TER PSR (Packet Success Ratio): Link ID 11. 17. 19

Figure 4  Test Configuration

## Shore station and ship station installation for Testing

### Location and environment

VDES ship station for VDES field test was installed in port side of the navigation bridge in M/V “Queen Mary”, which navigates from Mokpo port to Jeju port, and VDES shore station was located in Mt. Duryun in Haenam area in the Republic of Korea. In the below figure, the shore station, the ship station, and the VTS located in the vicinity are indicated. The ship station operates along the designated route from Mokpo to Jeju and measured the performance through transmission and reception tests for ASM and VDE-TER packets during operation. The test environment was selected as a place where the results of the effects of LOS, NLOS, and AIS can be analyzed, and the communication performance according to various environments can be measured.

Figure 5  Test areas of Mokpo/Jeju/Haenam

The base station was installed at the top of Mt. Duryun with an altitude of 675m considering the maximum test distance up to 100km. The ship station was installed in the bridge in the M/V “Queen MARY” with the height of 20m above sea level. The tested maximum distance from the coast station to the shipping station is about 103 km, and quite high altitude islands are located between the shore station and the ship station route.

Figure 6  Shore station and Ship station installation

|  | Setting for Testing |  |
| --- | --- | --- |
|  | Shore Station | Ship Station |
| Location of Shore Station | Haenam | Bridge of Queen Mary |
| RF Transmit Power | 12.5 W | 12.5 W |
| Link ID for ASM and VDE | Link ID 5, Link ID 11, 17, 19 | Link ID 5, Link ID 11, 17, 19 |
| Bandwidth | 25kHz, 100kHz | 25kHz, 100kHz |
| Antenna Height | 2 m(675 m above sea level) | 1.5 m(20 m above sea level) |

## Test Setup

For testing, the test data in the ASM and VDE Link Packet are jointly organized in Payload as shown in the table below. This data can be used to analyse the impact of shore to ship distance, location and speed on PSR.

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Packet Index | 16 | Packet identifier. (0-65535) It increases by 1 for every packet transmission. |
| Longitude | 28 | Longitude in 1/10000 (±180°, East = positive (as per 2’s complement), West = negative (as per 2’s complement). 181° = (6791AC0h) = not available = default) |
| Latitude | 27 | Latitude in 1/10000 (±90°, North = positive (as per 2’s complement), South = negative (as per 2’s complement). 91° = (3412140h) = not available = default) |
| SOG | 10 | Speed over ground in 1/10 knot steps (0-102.2 knots) 1023 = not available, 1022 = 102.2 knots or higher |
| COG | 9 | Course over ground (0-359) (511 indicates not available = default) |
| Slot Number | 12 | Slot number when transmission over VHF. |
| Spare | 2 | Should be zero. (00b) |
| Total | 104 |  |

### ASM Test Setup

The ASM transmitter receives a BBM message per second from HMI and sends it to the ASM channel by adding test data if the ASM ID is "1004(dec)" and the ASM receiver generates a PISDM message (similar to the VSM message) and sends it to HMI. The receiving HMI logs the data if the ASM ID is "1004(dec)". The ASM transceiver test message flow is shown below.

Figure 7  ASM Message flow diagram

The ASM link message used Message ID 2 (Broadcast Message) and the packet structure is as follows;

Figure 8  ASM test packet structure

### VDE-TER Test Setup

The VDE-TER transmitter receives data in the format of binary data (RaUDP) of 61162-450 from HMI, and if the value of the SOA (Status of Acquisition) field is "1004(dec)" it adds test data, properly constructs the binary by the Link ID selected in VDE Link and sends it to the VDE channel. VDE transport packets were sent from TDMA channel 1 and were fragmented to 18 (Link ID 11, Link ID 17) and 13 (Link ID 19) according to the selected Link ID, and no other link packets were sent except for Start/Continuation/End Fragmentation messages (#74, #75, #76) for smooth PER measurement. The VDE receiver reassembles the received frames and transmits them to the HMI. Receive-side HMI logs the data if the value of the SOA field is "1004(dec)" The VDE transceiver test data flow is shown below.

Figure 9  VDE Message flow diagram

The structure of the VDE transmission packet is as follows;

Figure 10  VDE test packet structure

# TESTing RESULTs

## Link Budget Analysis prior to testing

A number of islands are located between the VDES shore station and the routes of the VDES ship station, so Link Budget simulations were conducted to estimate the impact on field test and to compare with actual test results. According to M/V Queen Mary's route, a high mountain is located between the base station and ship’s route, and Link Budget analysis shows a large difference in received power. Points were identified and marked on the route according to the received power.

Figure 11  Analysis of link budget according to route of “Queen MARY”

Received power differences are mostly affected by mountainous terrain and should be considered in PSR analysis. The figure 11 shows an example of link budget calculated by location.

## Testing results for ASM

In order to measure the performance of ASM packet transmission/reception in the marine environment, ASM messages were periodically transmitted. The measured PSR(Packet Success Ratio) are as follows;

### ASM Link ID 5 (ship to shore)

Test Date: 2020.07.01

| Test Mode | Packet numbers of TX | Packet numbers of Good RX | PSR |
| --- | --- | --- | --- |
| Link ID 5 | 14710 | 14656 | 99.63 % |

### ASM Link ID 5 (shore to ship)

Test Date: 2020.07.02

| Test Mode | Packet numbers of TX | Packet numbers of Good RX | PSR |
| --- | --- | --- | --- |
| Link ID 5 | 3816 | 3654 | 95.75 % |

Test date: 2020.07.26

| Test Mode | Packet numbers of TX | Packet numbers of Good RX | PSR |
| --- | --- | --- | --- |
| Link ID 5 | 7429 | 6689 | 90.04 % |

## Test results for VDE-TER

In order to measure the performance of VDE packet transmission/reception in the marine environment, VDE messages were periodically transmitted. The measured PSR(Packet Success Ratio) are as follows.

### VDE Link ID 11(ship to shore)

2020.07.01/02

| Test Mode | Packet numbers of TX | Packet numbers of Good RX | PSR |
| --- | --- | --- | --- |
| Link ID 11 | 12353 | 12154 | 99.9 % |

### VDE Link ID 11 (shore to ship)

Test date: 2020.07.01/02

| Test Mode | Packet numbers of TX | Packet numbers of Good RX | PSR |
| --- | --- | --- | --- |
| Link ID 11 | 10602 | 10593 | 98.5 % |

### VDE Link ID 19 (ship to shore)

Test date: 2020.07.26.

| Test Mode | Packet numbers of TX | Packet numbers of Good RX | PSR |
| --- | --- | --- | --- |
| Link ID 19 | 1261 | 1261 | 100 % |

### VDE Link ID 19 (shore to ship)

Test date: 2020.07.26.

| Test Mode | Packet numbers of TX | Packet numbers of Good RX | PSR |
| --- | --- | --- | --- |
| Link ID 19 | 1634 | 1345 | 87.7 % |

## ASM/VDE-TER PSR over 100km

The following shows the PSR measured when Queen Mary entered Jeju Port and was located more than 100 km away from the base station.

### ASM Link ID 5 (ship to shore)

Test Date: 2020.07.26 / Distance: 103Km (@ Jeju port)

| Test Mode | Packet numbers of TX | Packet numbers of Good RX | PSR |
| --- | --- | --- | --- |
| Link ID 5 | 1555 | 1552 | 99.8 % |

### ASM Link ID 5 (shore to ship)

Test Date: 2020.07.26. / Distance: 103Km (@ Jeju port)

| Test Mode | Packet numbers of TX | Packet numbers of Good RX | PSR |
| --- | --- | --- | --- |
| Link ID 5 | 1538 | 1444 | 93.9 % |

### VDE Link ID 11(ship to shore)

Test Date: 2020.07.26. / Distance: 103Km (@ Jeju port)

| Test Mode | Packet numbers of TX | Packet numbers of Good RX | PSR |
| --- | --- | --- | --- |
| Link ID 11 | 1944 | 1944 | 100 % |

### VDE Link ID 11 (shore to ship)

Test Date: 2020.07.26. / Distance: 103Km (@ Jeju port)

| Test Mode | Packet numbers of TX | Packet numbers of Good RX | PSR |
| --- | --- | --- | --- |
| Link ID 11 | 2214 | 2108 | 95.2 % |

### VDE Link ID 19 (ship to shore)

Test Date: 2020.07.26. / Distance: 103Km (@ Jeju port)

| Test Mode | Packet numbers of TX | Packet numbers of Good RX | PSR |
| --- | --- | --- | --- |
| Link ID 19 | 845 | 777 | 92 % |

### VDE Link ID 19 (shore to ship)

Test Date: 2020.07.26. / Distance: 103Km (@ Jeju port)

| Test Mode | Packet numbers of TX | Packet numbers of Good RX | PSR |
| --- | --- | --- | --- |
| Link ID 19 | 1014 | 101 | 10.0 % |

## Analysis of Measured PSR

The results of several maritime tests can be found to have significant differences in reception performance, depending on time and location of the ship station. The performance differences by location were able to identify the results consistent with those predicted through the link budget, and the performance differences in the same area are deduced mainly due to the weather.

It has been confirmed that the degradation of receiving performance of the ship station rather than receiving of the base station occurred through repeated measurements. The major factors can be estimated by Multipath and Interference and interference from adjacent AIS could be found to have a serious adverse effect on ASM/VDE reception.

- Multipath Effect

Figure 12  Multipath fading in marine environment

- Interference

Figure 13  Measured interference By Queen Mary AIS

# Conclusion

VDES communication equipment has been developed in the Smart-Navigation Project from 2016, and it comes to final stage to verify the performance of our VDES through the field test at sea areas this year. The field test was conducted to verify that the developed VDES was properly working in the actual marine environment, and both ASM and VDE were in good operation.

Although ASM and VDE were generally communicated well at the 100km distance, which was the final goal of the task, it was confirmed that in the case of Link ID 19, receiving performance at the ship station side did not meet expectations. It is believed that these problems can be overcome by limiting the operating distance or by applying the equalizer. In addition, Packet Loss occurred more frequently than CRC errors in the actual sea environment, and the main reason is assumed to be the training sequence, which is too short for maritime environmental situations where multiple pathways and interference signals exist.

Over the years, we have been developed VDES prototypes based on IALA Guideline G1139 and were able to gain some meaningful experiences through the actual field test at Korean sea areas. We hope that the information on actual field testing results of VDES conducted in Korea will be referenced by other organizations participating in the development or standardization work of VDES, and we also plan to share the further analysis study regarding the use of pilot symbols and the extension of the training sequence length in order to overcome the maritime environment such as multipath confirmed through the test results.

# Action requested of the Committee

The Committee is requested to review the information and take appropriate action.