The role of VTS in limited access/restricted Areas

1. SCOPE

This guideline aims at defining a special application of measures and procedures for establishing and managing particular areas where limitations to navigation may need to be or have been implemented.

The purpose of these guidelines is to assist VTS authorities in defining the role of the VTS in the process of establishing and managing such area.

2. REFERENCES

Reference documentation is IMO Resolution A.857(20), the IALA VTS Manual and IMO Ships’ Routeing Guide (GPSR).

3. DEFINITION AND CLARIFICATION

Restricted area: defined area in which navigation is limited or prohibited for reasons of safety, security and environmental interests. These areas are established by an authority which may not necessarily be the VTS authority (e.g. Harbour Master, Coastguard, Ministry of Environment, etc).

In establishing such an area, the boundaries and restrictions should be promulgated to shipping through all possible means (see Annex A for examples).

4. ESTABLISHING A RESTRICTED AREA

Through risk assessment a responsible authority can identify the need for establishing a restricted area. Frequency and/or type of shipping in the vicinity of such areas can potentially increase risks and vulnerabilities and therefore should be taken into consideration when determining the restricted area. The VTS Authority should have a consultative or supportive role in the establishment process of restricted areas. As VTS has the most expertise and experience regarding traffic movement in its area, it should advise the responsible authority about the consequences for shipping and for the access to harbours. In some cases a VTS Authority may advise that establishing such an area is not appropriate.

An analysis of statistical traffic data and of potential dangerous situations in the affected area may be provided by the VTS authority to evaluate the consequences - not only to shipping but also to human interests, the environment, offshore installations, etc. - in terms of frequency and outcome. Consideration must be given to the location of the area; in particular the applicability of legislation with regard to, for example, the right of free passage as defined in UNCLOS.

While assessing the need to establish restrictions to navigation, responsible authorities should also consider the management and enforcement of the restrictions. Furthermore, responsible authorities should first evaluate whether there is a need for a VTS In the area, or whether there is a need for making modifications to an existing VTS system. Such modifications may include: increasing the VTS area, installing new equipment, changing the type of service the VTS provides, etc. Risk assessment and cost-benefit analysis are both useful tools in the establishment of the restricted area and the means of monitoring and enforcing the restrictions.

5. IMPLEMENTATION AND MANAGING

Particular care should be given to ensure that control measures will effectively regulate shipping entering this area and to monitor and take appropriate action against any violations.

In circumstances where the risk related to navigation is increased, a VTS may be one of the most efficient tools through which such a risk can be dealt with in order to reduce it to an acceptable level.

When it is decided that the monitoring and regulation of shipping should be performed by VTS, the responsible authority has to ensure that the VTS has the necessary tools, procedures, and authorisation to do so.

To this end the VTS authority may adopt particular provisions such as to:

- impose specific navigation measures (e.g. recommended route)

- implement specific reporting obligations

- arrange for the exchange of related information with other organisations involved

- monitor/co-ordinate safety zone to prevent unauthorized access

- insert boundary co-ordinates and set appropriate alarms for the area.

- modify existing operating procedures

- arrange appropriate response measures

- provide information of infraction to enforcement authorities

Any such provisions must be in compliance with national or international legislation.

6. EVALUATING

After establishing a restricted area an evaluation process of its effect will be needed in order to identify problems through risk assessment, and the VTS authority should participate in this process. This evaluation may, for example, lead to adjustments of the area or its restrictions, or an adjustment of the management of the area.

Notes

- Review VTS Manual (Ch.8/Ch.6) for definitions and references to areas

- If the area is owned by another authority how does VTS provide support?

- Look at chart symbology for restricted/limit access areas

Annex:

Examples of management tools for restricted areas

Offshore installations:

- Safety zone: Ensure safety of the installation by means of a “Buffer-zone”

- Surveillance measures (e.g. radar, AIS, CCTV) placed ashore or on the installation itself: To identify vessels which may create a hazardous situation when approaching the installation.

- Guard vessels: Ensure safety of the installation through physical intervention with approaching vessels.

- Aids to navigation (AtoN): Indicating the location of the installation or the restricted area around it.

- Navigational warnings, Notices to Mariners (NtM), and virtual AIS marking: Communicating any restrictions to shipping.

Offshore renewable energy installations (OREI) expand on different types, e.g. windfarms, wave generators

- Safety zone: Ensure safety of the installation by means of a “Buffer-zone”

- Surveillance measures (e.g. radar, AIS, CCTV) placed ashore or on the installation itself: To identify vessels which may create a hazardous situation when approaching the installation.

- Guard vessels: Ensure safety of the installation through physical intervention with approaching vessels.

- AtoN: Indicating the location of the installation or the restricted area around it.

- Navigational warnings, NtM, and virtual AIS marking: Communicating any restrictions to shipping.

National parks

- Surveillance measures (e.g. radar, AIS, CCTV): To determine whether vessels enter the area without permission.

- AtoN: Indicating the location of the restricted area.

- Charts, NtM, virtual AIS marking: Communicating any restrictions to shipping.

Military exercise areas

- Surveillance measures (e.g. radar, AIS, CCTV): To determine whether vessels enter the area without permission.

- Guard vessels / aircraft: Ensure safety of shipping in the area through physical intervention with approaching vessels.

- AtoN: Indicating the location of the restricted area.

- Navigational warning, NtM, virtual AIS marking: Communicating any restrictions to shipping.

- Shore based marking: Ensure safety of shipping and pleasure craft operating near exercise area.

Sites of Special Scientific Interest (SSSI)

- Surveillance measures (e.g. radar, AIS, CCTV): To determine whether vessels enter the area without permission.

- AtoN: Indicating the location of the restricted area.

- Charts, NtM, virtual AIS marking: Communicating any restrictions to shipping.