AISM Association Internationale de Signalisation Maritime      IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

IALA Guideline No. ####

On

Navigational Safety within Marine Spatial Planning

Edition 1

[Date issued]

[Previous Edition; Date issued]

10, rue des Gaudines

78100 Saint Germain en Laye, France

Telephone: +33 1 34 51 70 01  Fax:  +33 1 34 51 82 05

e-mail:  contact@iala-aism.org       Internet:  www.iala-aism.org

Document Revisions

Revisions to the IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |

Table of Contents

To update, right click anywhere on the table, then update field / update entire table.

Document Revisions	2

Table of Contents	3

Index of Tables	3

Index of Figures	3

Guideline on Navigational Safety within Marine Spatial Planning	4

1.	Introduction	4

2.	FRAMEWORK: ASPECTS AND REFERENCES	4

2.1	Aspects of MSP:	4

2.2	REFERENCES	5

Additional references are provided in annex x	5

3.	THe Marine spatial planning process	6

4.	spatial demands	7

4.1	Potential effects on maritime safety	7

4.2	Data needed	10

4.3	Marking with AtoN	10

4.4	Decommissioning	10

5.	Geographical Information System (GIS)	10

5.1	Data requirements	10

	A GIS is considered to be an essential tool for supporting the Marine Spatial Planning process.	11

	A GIS webservice to facilitate consultation and promulgation of information to assist and improve the quality of decisions made in the process of formulating a Marine Spatial Plan is recommended.	11

	Geographical information that aligns with standards contained in the IHO’s developing S-100 Geospatial Information Registry will be increasingly helpful for Marine Spatial Planning, noting that IALA is working on two S-100 domains, VTS and AtoN.	11

	Advice on the application of GIS needs to be generic;	11

	Does the existing Guideline 1057 on the Use of Geographical Information Systems by Aids to Navigation Authorities need to be reviewed?	11

	Guideline 1057 will need to be referenced in a Guideline on Marine Spatial Planning;	11

5.2	Content of a GIS	12

5.2.1	Actual situation manoeuvrable space	12

5.2.2	Identification present risk areas	12

5.2.3	Identification/description service provision	13

5.2.4	Identification short term measures (as designed using GIS)	13

5.2.5	Identification developments traffic and planned utilization of space	13

5.2.6	Identification future risk areas (based on GIS analysis)	13

5.2.7	Identification mid -and long-term measures	13

5.3	GIS Components	14

5.3.1	Layer Properties	14

5.3.2	Sources of data and information for the GIS	14

5.3.3	Metadata	14

5.3.4	GIS Data and the S-100 Registry	14

5.4	Web-based viewer	14

5.5	Training	14

5.6	Transparency	14

6.	Risk management	14

6.1	Risk Assessment	15

6.2	Risk Acceptance	16

6.2.1	Risks to people	16

6.2.2	Risks to environment	17

6.2.3	Risks to property and assets	18

6.2.4	Risks to business	18

6.3	Risk Mitigation	18

7.	Conclusions	18

8.	acronyms AND ABBREVIATIONS	18

9.	Definitions and Interpretations	19

ANNEX A	links to relevant sources	21

ANNEX B	Case studies	24

ANNEX C	Categorisation of GIS layers for MSP	25

ANNEX D	List of risk sources	28

	Office 2003, go to Format / Bullets and Numbering / Restart numbering (lower left in the box)	30

	Office 2007, go to down arrow next to Numbering icon and select Set Numbering Value	30

[end dutch version]

Index of Tables

Table 1	Title required	14

Index of Figures

Figure 1	Title required	14

Guideline on Navigational Safety within Marine Spatial Planning

# Introduction

  - Background

For millennia the seas have been the preserve of the mariner alone. In the second half of the 20th century the seas became not only trade routes and a source of food but also a source of energy. Oil and gas installations became the first significant man-made developments that were often sited in proximity to or amidst navigable waters. In the 21st century the drive to reduce carbon emissions and improve energy security has led to a global expansion of Offshore Renewable Energy Installations (OREI). Many sites are either under construction or being planned, with wind energy being the technology of choice for many countries.

The construction of large offshore wind farms and tidal and wave sites requires coherent planning to ensure the safety of navigation, achieve environmental protection and provide for energy needs. Although wind farms presently are the most space-consuming, in the future other interests such as aquaculture, environmental protection and preservation and exploitation of natural resources may increase. Significant areas of navigable waters may no longer be available to the mariner alone.

As a result of the workshop on the Application of Aids to Navigation within Marine Spatial Planning, attended by delegates from diverse range of backgrounds and the continued work of the IALA ARM Committee, this Guideline on Navigational Safety within Marine Spatial Planning was initiated. It will contribute to facilitating engagement in the process and inter-stakeholder co-operation at local, national and international level.

  - Purpose

The purpose of this Guideline is to inform Aids to Navigation (AtoN) and other maritime authorities of the main elements of the Marine Spatial Planning (MSP) process. Specific navigational concerns should be considered when assessing the impact on existing marine traffic routing and navigational safety caused by offshore developments. It will also provide information to other MSP stakeholders of the underlying navigation factors to be taken into account during the process.

This document describes the MSP process and provides guidance on the role of AtoN and other maritime authorities have in contributing to the navigational assessment elements of MSP. It is important that preparation and planning takes place to ensure that safety at sea and navigation requirements are adequately addressed. This Guideline also provides reference to other industry documents for further technical and procedural details.

  - Scope

The MSP process brings together multiple users of marine areas to make informed, coordinated decisions about how to use marine resources sustainably and reduce conflicts. Users may include shipping, offshore energy, aquaculture, fishing, government, conservation and recreation entities. MSP has its origins in marine ecological and environmental protection but has evolved to encompass economic and navigational safety concerns. It also provides means for optimising utilisation of sea areas with respect to ecological, social and economical values and improving long-term international policy development, efficiency of shipping and maritime safety. MSP should therefore not only be seen as a national or cross-border issue but should also take into consideration international navigational interests.

# FRAMEWORK:	ASPECTS AND REFERENCES

The MSP process is not new. Conflicting interests have occurred before, but solutions could be found on a relatively small scale. However, offshore space has become increasingly valuable and utilised and a more holistic spatial planning approach is required. Integrated Pollution Prevention and Control (IPPC) emission regulations have stimulated a considerable demand for OREI developments. These developments may take up large sea areas and may have significant effects on navigation, the environment and economies.

# Aspects of MSP:

It is important that early preparation and planning takes place to ensure that safety at sea and navigation requirements are adequately addressed. Therefore, administrations should be proactive in the development of their MSP rather than reactive.

The following aspects may be considered in developing marine spatial plans to ensure that navigational safety is included:

- Economic impacts such as shipping and fisheries industries.

- Energy targets (e.g. renewable energy capacity).

- Environmental requirements (e.g. Marine Protection Areas).

- Singular, multi, or co-uses of an area.

- Cumulative effects of adjacent project infrastructure.

- Marine-related activities.

Additional explanations of the above bullets can be found in Annex X.

The main purpose of MSP is to achieve a balanced approach towards navigational safety, environmental protection, economic effects and communication (information management). Typically, a marine spatial plan is not based upon one solution. Multiple options are usually presented to address key issues over the plan development using an iterative and continual learning approach. Plan developers should take into account jurisdictional issues between adjacent states. The MSP process in every country may be subject to international laws and regulations, national legislation, laws, rules, regulations and other guidance documents.

## REFERENCES

The table below lists guidance documents recommended for review to support MSP development efforts or subtopics like the acceptable distance between wind farms and shipping routes.

| Document title | Purpose |
| --- | --- |
| Offshore Renewable Energy Installations Guidance on UK Navigational Practice, Safety and Emergency Response Issues (MGN 371), MCA 2004 | Highlights issues that need to be taken into consideration when assessing the impact on navigational safety and emergency response caused by offshore renewable energy installation developments. Includes a “shipping route” template. |
| Offshore Renewable Energy Installations Guidance to Mariners Operating In The Vicinity Of UK Wind Farms (MGN 372), MCA 2005 | Highlights issues that need to be taken into account when planning and undertaking voyages in the vicinity of offshore renewable energy installations. |
| Guidance On The Assessment Of The Impact Of Offshore Wind Farms, Methodology for Assessing the Marine Navigational Risks of Offshore Wind Farms, DTI 2005 | Makes recommendations on how to conduct a Risk Assessment for an offshore wind farm development |
| Marine Spatial Planning, a step-by-step approach toward Ecosystem-based planning, UNESCO 2009 | Aimed primarily at those responsible for the spatial planning and management of marine areas |
| The Shipping Industry and Marine Spatial Planning – a professional approach, The Nautical Institute 2013 | Aimed at mariners and maritime professionals who should play a role in MSP, and MSP professionals that want to better understand maritime stakeholders’ requirements. |
| Marine Planning Handbook, National Ocean Council 2013 | Provide information and guidance to regions that choose to establish regional planning bodies and develop marine (spatial) plans. |
| Directive 2014/89/EU | Establishing a framework for marine spatial planning |
| Interaction Between Offshore Wind Farms And Maritime Navigation, PIANC 2016 MarCom WG 161. | The final report of the WG will provide an approach, guidelines and recommendations to assess the required manoeuvring space in the vicinity of offshore windfarms and the minimal distance between shipping lanes and sea areas far offshore windfarms, in order to ensure a minimal risk level for navigation. |

# Additional references are provided in annex x

# THe Marine spatial planning process

A basic ten-step MSP development process is diagrammed below. The paragraphs following the flowchart explain the steps identified and provide guidance to the authority/authorities responsible for maritime safety identified in step 1. The planning process should include stakeholder interaction in multiple stages of the project.

*Flow chart provided courtesy of UNESCO.

Step 1 – Identify the need and establish an authority

An Administration should identify or establish a leading MSP authority or authorities. The leading authority [Competent authority] should coordinate the link between other Administration’s agencies interested in MSP.

Step 2 – Obtain financial support

The leading authority should recognise that resources, such as financial, personnel, equipment, survey and other appropriate data, which will be needed to participate in this process.

Step 3 - Organise the process through pre-planning

The leading authority needs to ensure proper capacity or desired skills. It may coordinate with other national representatives as appropriate to ensure that all MSP issues are covered and supported according to current requirements. This should include developing a work plan that identifies key work products and resources such as information and data needed to complete the output of the planning on time.

In addition, the lead authorities may identify public educational issues and stakeholders to include in the planning process.

Step 4 – Organise stakeholder participation

The leading authority should organise planning meetings that involve the appropriate stakeholders. A cross section of stakeholder expertise should be considered. The planning meetings´ agendas will indicate which stakeholders to include and how they will be involved in the MSP process.

Step 5 – Define and analyse existing conditions

Current information such as AtoN, VTS, radio navigation and communication capability, traffic routes and AIS tracking, previous risk assessments, hydrographic and meteorological information, should be gathered. In addition gaps in data/information should be identified and where new studies are required. Analysis of current information will identify existing and possible conflicts and synergies between waterway users.

Step 6 – Define and analyse future conditions

The MSP process should ensure that future safety of navigation issues are identified. Risk assessments will need to be conducted and possible mitigations identified. Mitigations may include; spatial/temporal separations, new routes, changes to AtoN / VTS strategy, or the need for new resources such as coastal monitoring emergency response capability.

The leading authority should identify possible costs and benefits for each proposed scenario and how the MSP will be monitored in the future.

Finally, the plan should be co-ordinated with neighbouring national maritime administrations and as far as possible comply with international conventions, regulations and guidelines.

Step 7 – Prepare and approve the MSP

The leading authority should identify the best options for the MSP including risk mitigation and cost issues. The plan should be communicated to all stakeholders and identify any pre-implementation issues that will need to be addressed. These may include public relations, commissioning equipment/materials, and engaging the appropriate personnel. The plan will then be finalised and approved.

Step 8 – Implement and enforce the MSP

The leading authority will ensure the promulgation of the MSP. At this step the MSP should be executed including issues of regulatory changes, notifications, step by step implementation and enforcement activities. Project management techniques should be utilised to ensure that the implementation is timely, efficient and well co-ordinated with other authorities, agencies and stakeholders. Monitoring activities of the plan should be activated during this stage.

Step 9 – Monitor and evaluate the MSP

Monitoring activities as defined should be carried out to assess and evaluate the effectiveness of the plan. During this stage, periodic progress reports should be published about the performance of the plan, and any problems identified and analysed.

Step 10 – Adapt the spatial management process

All information gained during the entire implementation of the MSP should be used to amend the existing plan to ensure continuous improvement of the plan.

# Geographical Information System (GIS)

A Geographical Information System (GIS) is an essential tool for storing, visualising and managing geographical data across borders.  GIS is a system designed to capture, store, manipulate, analyse, manage, and present all types of spatial or geographical data.  GIS allows us visualize, question, analyse, and interpret data to understand relationships, patterns, and trends.

One of the best ways to capture, manage, analyse and display geographic data is through a Geographical Information System (GIS). A GIS integrates hardware, software, and data, and can capture the three-dimensional aspects of marine data as well as temporal aspects, such as how oceanographic processes or human activities change throughout the year.  In transboundary MSP, which usually compiles a large amount of geographic information, a GIS can be a powerful tool in the management and treatment of this information. It allows exchange of information between countries, combination of information layers and visualization of possible spatial conflicts.  A GIS can therefore be the basis for a common system of information developed within transboundary projects, representing the spatial extent, time and frequency of maritime activities, as well as the distribution and conditions. (from TPEA)

A GIS is considered to be an essential tool for supporting the Marine Spatial Planning process.  Therefore, it is recommended that Competent Authorities develop a GIS webservice that will allow the user to facilitate consultation and promulgation of information to assist and improve the quality of decisions made in the process of formulating a Marine Spatial Plan.

The IALA Guideline 1057 entitled “On the use of Geographic Information Systems (GIS) by Aids to Navigation (AtoN) Authorities” should be used by a Competent Authority to identify and determine the type and capabilities of its GIS with a view towards the GIS having the data and capabilities to support and aid the MSP process.

## Data requirements

The Competent Authority must determine the minimum data that should be portrayed in a GIS.  This data should include:

Hydrographic data presently displayed on applicable charts.  This includes but is not limited to:

Area identification

Routing measures

Anchorages

Shallow water

- A GIS is considered to be an essential tool for supporting the Marine Spatial Planning process.

- A GIS webservice to facilitate consultation and promulgation of information to assist and improve the quality of decisions made in the process of formulating a Marine Spatial Plan is recommended.

- Geographical information that aligns with standards contained in the IHO’s developing S-100 Geospatial Information Registry will be increasingly helpful for Marine Spatial Planning, noting that IALA is working on two S-100 domains, VTS and AtoN.

- Advice on the application of GIS needs to be generic;

- Does the existing Guideline 1057 on the Use of Geographical Information Systems by Aids to Navigation Authorities need to be reviewed?

- Guideline 1057 will need to be referenced in a Guideline on Marine Spatial Planning;

- Data ownership issues e.g. mapping at sea, is a difficulty and part of the problem in terms of not having the essential information needed.

- Hydrographic officers are very keen to ensure maps are up to date and accurate so quality control and management of this is essential. There is a need for information to be more freely available and in Australia RASTER maps are produced, paper maps printed from this and ENC data, but no extra resource.  As such, RASTER-based product production being stopped and this may give opportunity to others to use RASTER-type data.

- Paper and RASTER chart scales at the moment do not provide location of individual turbines. This raises the question of who pays for chart updates and it could be that developers do this. Essential to enable accurate marking.

## Content of a GIS

Body text

A geographic information system (GIS) is a system designed to capture, store, manipulate, analyze, manage, and present all types of geographical data. A GIS can be thought of as a system: it digitally creates and "manipulates" spatial areas that may be jurisdictional, purpose, or application-oriented.  GIS Applications are tools that allow users to create interactive queries (user-created searches), analyse spatial information, edit data in maps, and present the results of all these operations.  GIS is considered to be an essential tool for MSP.

MSP is a step-by-step approach and should be inclusive of all relevant stakeholders from the very beginning of the process and take into account their various interests.  It should also be iterative, always seeking to improve and refine.  GIS plays an important and essential part of these processes.

The minimum data that should be portrayed when developing GIS are:

Area identification

- Vessel Traffic Separation Schemes (VTSSs)

- Deep Water Routes (DWRs)

- Anchorage areas

- Shallow waters

- Offshore structures (oil, gas) (incl. safety zones)

- Fish farms

- Windmill farms (incl. safety zones)

- Areas to be avoided (sensitive environment)

- Pipelines

- Communication and power cabling

- Restricted areas for certain activities

Traffic flows

- Intensity

- Diversity

- Scale enlargement expected

- Mandatory, non-mandatory, expected on-board equipment

### Actual situation manoeuvrable space

- Detailed overview and full description of the use of the marine space

- Accurate mapping overall area and specific areas

- Specific areas

- Routing systems

- Offshore activities

- Up-to-date and accessible databases

- Recognizable Competent Authority/Authorities

### Identification present risk areas

- Incidents / Accidents

  - past mid-term figures (3-5 years)

    - type of incidents / accidents

    - causes

    - Types of vessels involved

    - nationality/flag of vessels involved

    - damages / losses

- Operational registrations

- Enforcement protocols

- Verdicts/decisions National  Board of Transport Safety

    - Validations

    - Conclusions

    - Proposed measures for improvement

### Identification/description service provision

- Detailed regional and local overview and description of the available (shore based) infrastructure and provided services

    - Supporting vessel traffic management instruments,

- Transparency responsibilities and liabilities

### Identification short term measures (as designed using GIS)

Consequences:  for each and every aspect

- Identification additional measures, potential solutions and services. In most cases reactive, such as:

  - Modification of existing routing

  - Modification of Marking

Additional provision of information

### Identification developments traffic and planned utilization of space

- Outlook

  - Mid-term expectations ( >  10 years)

  - Long-term expectations (> 20 years+)

  - Developments in shipping (all areas, trends, growth)

  - Growth expectations of ports

  - Evolvements of types of goods, transport of persons

  - Offshore developments

- Development Vision (“What will be the picture then?” )

  - Mid-term ( >  10 years)

    - Long-term (> 20 years+)

  - Tune where appropriate plans and Vision with neighboring administrations and Competent Authorities

### Identification future risk areas (based on GIS analysis)

### Identification mid -and long-term measures

Consequences:  for each and every aspect

Identification additional measures, potential solutions and services. In most cases proactive.

Text on how GIS can help in design of effective measures. Measures itself described under Risk Mitigation Measures.

## GIS Components

GIS layers maybe categorised into the following:

- General

- Present

- Future

- Variable Constraints

- Test Beds

Further details of the above categorisation is presented in Annex ???.

### Layer Properties

Layer properties should include easily understood information about the data presented to the user (for instance a legend) otherwise the information presented is useless.

### Sources of data and information for the GIS

The sources of information to be displayed on GIS are almost limitless; however, these should be presented to the GIS in a consistent and agreed manner.

### Metadata

Metadata can be described as “data about data.”  It is important that metadata attributes are easily accessible and well defined.  Metadata should be designed in order that the information is easily exchanged and presented consistently.

#### Maintenance of metadata

Data can consist of fixed or variable data.  It is essential that these data are updated at regular frequencies.  Data must be continually validated for currency, quality and accuracy.  It is incumbent on the owner of the data source to ensure that the data is current and reliable.

### GIS Data and the S-100 Registry

It is recommended that GIS data and their relevant product specifications are incorporated into the S-100 registry.  IALA may consider co-ordinating the input into the relevant S-100 domain.

## Web-based viewer

A web-based GIS viewer is a simplified way to present geospatial information to non-specialist parties in a pre-defined structure.  Web-based viewers are “read only” and the viewer cannot interact/change the GIS programme or parameters.

## Training

GIS systems are complexand as such, authorities should consider training programmes for personnel working with GIS.  Hands on experience and current use are essential to maintain skill levels and get the most from GIS systems.

## Transparency

The process of MSP planning should be transparent and accessible to all stakeholders.  This could include open access to GIS systems or web-based viewers.

# Risk management

Within the context of this Guideline, “Risk Management” refers to the strategic risk management connected to policy making and planning processes.  In [Guideline 1018] the (maritime) risk management process is described in depth based on the break down as defined in Fig##.  In this Guideline the aspects particularly relevant to MSP (planning of OREI) will be highlighted.

Fig## Risk Management process scheme, from [Guideline 1018]

The risk assessment for offshore facilities, e.g., OREI starts in the project phase which, through the location, design and layout of the facility, aims to reduce the safety risks as much as technically possible and economically viable.

The initial risk assessment is an overall assessment based on the present knowledge of the project and of the maritime environment (including vessel movements and future scenarios). Subsequently the risk assessment has to be updated as the detail of the project is further developed.

A risk that needs to be considered is when the permitting authority does not recognise a recommendation from a maritime authority.tbd!

In general, development of facilities must not impair the safe usage of Traffic Separation Schemes, Inshore Traffic Zones, recognised sea-lanes, approaches and safe access to anchorages, harbours and places of refuge.

## Risk Assessment

The Assessment of the risk of major hazards shall be in accordance with Guideline 1018, and follow recognised methods for risk assessment e.g. the IALA risk management toolbox or the IMO adopted Formal Safety Assessment methodology (FSA).

The IALA Risk Management Toolbox includes the Quantitative Tool – IWRAP, the Qualitative Tool – PAWSA and Simulation Tools. According to SOLAS, Coastal States are obliged to provide Aids to Navigation in accordance with the volume of traffic and the degree of Risk. The complementarity of the three components PAWSA, IWRAP and Simulation enable national authorities to better fulfil their obligations under SOLAS.

A recognised risk assessment tool like PAWSA or FSA is a structured and systematic methodology, aimed at enhancing maritime safety, including protection of life, health, the marine environment and property by using risk analysis and cost benefit assessment. These can be used as tools to assist:

- In the identification of risks introduced by the facility

- in the evaluation of new measures for maritime safety and protection of the marine environment

- in making a comparison between existing and possibly improved measures, with a view to achieving a balance between the various technical and operational issues, including the human element

- in making a comparison between maritime safety or the protection of the marine environment and costs.

The assessment of risks regarding the safety of navigation may include, but is not limited to:

- Present and future trends in shipping traffic

- Traffic density and distribution

- Bathymetry

- Meteorological conditions

- Identification of additional measures, potential solutions and services

- Shipping routes and channels, TSS and other routing measures, including IMO adopted routing measures.

- Existing AtoN, VTS and MSI-VHF coverage

- Interference to GNSS, radio and radar signals, etc.

- Ports and harbour approaches

- Pilot em- and debarkation areas

- Environmentally sensitive areas

- Outflow of environmentally harmful substances

- Search and Rescue

- Ports of refuge or safe anchorages

- Fishing industry

- Aquacultures

- Leisure craft

- Geological resources (oil & gas, aggregate mining)

- Renewable energy (wind, wave, tidal generators)

- Abandoned installations

- Wave energy test sites

- Pipes, cables, (historic) wrecks (+diving)

- Restricted areas, military, physical, environmental

- Dumping areas

- Hydrographic information

- Staff/manning/pilot regulations

- Shore developments with marine effects

- Technological advancements in AtoN (GPS, AIS, eNav services)

- Future autonomous shipping

- Other local conditions

## Risk Acceptance

When deciding on acceptability of risks, it is useful to distinguish between different types of risk, by the nature of the consequences involved.  Risks can pertain to

- People (safety)

- Environment

- Property and assets

- Business

    - Risks to people

The safety of people not involved in the process or activity that causes the risk is expressed in individual risk and societal risk.

The individual risk indicates the likelihood for a person that stays in a particular location for one year, is killed by the consequences of an event. Thus, risk contours with a constant likelihood may be mapped. A criterion for acceptability can be that the 10-6 contours must not include inhabited areas.

Societal risk is usually expressed as a FN curve, which shows the likelihood of an incident (F) depending on the number of fatalities (N). The area under this probability curve is the Potential Loss of Life (PLL).

Fig xxx Hypothetical example of an FN Curve

Different acceptability criteria may be used for the shape of the FN curve and/or the PLL. Using PLL only implies that an incident killing 1000 people is considered equivalent to 1000 incidents killing 1 person, while the (political) impact may not be the same.

Although a MSP usually addresses sea areas with no human inhabitants, the risk to people has to be taken into consideration.

### Risks to environment

A thorough Environmental Impact Assessment has to be done, taking the following into consideration (during construction as well as in operation phase), but not limited to:

- Present and future trends in shipping traffic

- Traffic density and distribution

- Bathymetry

- Noise

- Light

- Environmentally sensitive areas

- Protected habitats

- Outflow of environmentally harmful substances

- Ports of refuge and safe anchorages

- Fishing industry

- Aquacultures

- Abandoned installations (possible decommissioning)

- Dumping areas

### Risks to property and assets

In addition to the monetary value and inconvenience of the potential loss, the value that stakeholders would pay for this loss and the value that stakeholders would gain from the intended facilities, should be taken into consideration.

### Risks to business

The potential risks to business and loss of reputation of ports and harbours (lesser vessels visiting due to increased risk) which may be subjected to the negative effect of the intended facilities, should be taken into consideration.  The vice versa may also be applicable.

## Risk Mitigation

Risk should be mitigated by a balanced approach, ensuring that the MSP process makes adequate provision for all related activities required.

It is important for each country to apply their conditions and requirements in the application of their MSP processes.

On a case-by-case basis, National Authorities may consider:

- Establishing:

- TSS

- Traffic lanes and other routing measures

- Exclusion or Safety Zones and Areas to be Avoided

- Separations areas

in order to prohibit or restrict vessels from entering or leaving areas of offshore structures.

- Establish additional, or reposition AtoN

- Establish or extend VTS

Offshore structures should be adequately marked following Guideline O-139. The use of virtual AtoN may be considered. In some cases the offshore structures may be used to accommodate radar scanners or VHF relay stations to improve VTS coverage.  Potentially, VTS outside national territories may contribute to safety of navigation in international waters populated with offshore installations.

All measures mentioned above must be reflected on the nautical charts and publications and promulgated through Maritime Safety Information (MSI) services.

# Conclusions

Xxxx

Annex X

# EXPLANATION OF CONSIDERED ASPECTS

The following aspects that should be considered when developing the MSP to ensure navigational safety are explained below:

Aspects

Examples

Impacts

Possible mitigation

Shipping

Aspect:	Fairways, Routes, TSS

Ports, anchorage area.

Pilot de- and embarkation area.

Explanation:	Normally handled exports and imports via trade routes by sea.

Consideration:	Planning phase design guidelines.

Sea traffic monitoring and planning.

Refuge harbours.

Ice and weather.

Infrastructure

Aspect:	Infrastructure is defined as facilities for the transportation of goods, passengers and services, and for the transmission of energy.

Explanation:	Bridges and tunnels for roads, train tracks and airports can come into conflict with shipping.

Consideration:	Sea traffic monitoring and planning.

Avoiding to force sea traffic to new routes.

Demands for air draft (vertical clearance).

Clearance for airport traffic

Commercial Fishing

Aspect:	The fishing activity varies geographically and over time.

Explanation:	Fishing is a mobile activity that requires access to fishing areas.

Consideration:	Competition with other activities concerns access to fishing areas and landing harbours.

Avoid fishing activities in fairways, routes and TSS.

Aquaculture

Aspect:		Fish farming.

Mussel and oyster farming, etc.

Seaweed farming

Explanation:	May be an obstacle for shipping if the industry expands and aquaculture develops further out to sea.

Consideration:	Aquaculture will be deployed of safety distance from fairways, routes and TSS.

Marking of aquaculture i.a.w. IALA recommendation O-139.

The suitability of safety distances may be assessed on the basis of risk analysis tools.

Recreation and tourism

Aspect:		Leisure craft.

Explanation:	 Many different types of outdoor activities take place in or by the sea (fishing, sailing, kayaking, skating, bathing, diving, bird watching and other nature related activities).

Consideration:	Separation of sea traffic if there is conflict between commercial vessel and leisure craft.

Environmental protection

Aspect:	Marine Protected Areas (MPA)

Natura 2000

Explanation:	Marine habitats that require comprehensive, long-term protection can become protected areas.

Consideration:	The EU Marine Directive 2008.

Access to MPA for maintenance of AtoN.

Research and Environmental monitoring

Aspect:	Oceanographic research.

Hydrographic surveys and shipping.

Explanation:	Through research and environmental monitoring, information is developed that can form a basis for planning work.

Consideration:	Regularly conducts measurements of water depth.

Energy

Aspect:	Renewable energy (wind, wave, tidal generators).

Test sites for OREI.

Oil and gas platforms.

Explanation:	The energy sector's use of the sea involves both the production and transfer of energy.

Consideration:	Facilities of energy production (cables, pipelines, offshore constructions).

National interests in relation to other coastal states.

Safety distance to fairways, routes, TSS and marking of offshore facilities i.a.w. NTI MSP recommendation and IALA O-139.

Etcetera – see other sections and references.

Consider to establish a safety zone of (max) 500 meters around offshore constructions (e.g. oil and gas platforms) i.a.w. UNCLOS.

Radio communication, radar etc

Aspect:	Interference.

Explanation:	Especially wind farms may interfere with radio and radar signals.

Consideration:	Navigation, VTS, Meteorological, airport and military radars.

VTS and MSI communication, VHF and AIS, etc.

Extraction and storage of materials

Aspect:		Geological resources.

Carbon capture and storage.

Explanation:	Extraction and storage of materials is expected to increase.

Consideration:	Marking by AtoN of establish area i.a.w. IALA recommendation O-139.

Cultural history

Aspect:	Wrecks

Explanation:	The seabed is home to a cultural heritage that tells a story covering everything from shipping to the fishing industry.

Consideration:	Areas with high cultural values need to be identified.

Marking of wrecks close to fairways, routes and TSS i.a.w. IALA MBS.

Defence

Aspect:	Military restricted areas.

Military exercise areas.

Explanation:	Exercising areas and shooting ranges are used to achieve and maintain the armed forces capability.

Consideration:	Avoid waterways close to military areas.

Co-ordination of military exercises and operations close to waterways.

## Data needed

Future needs for the co-ordination of maritime activities include the depending on reliable information of current traffic movements and hydrographic surveys.

For more detailed information reference to chapter 6 (GIS) and Annex XX

## Marking with AtoN

Marking with AtoN in accordance with IALA MBS (Maritime Buoyage System) and NAVGUIDE (e.g. restricted areas, offshore constructions, fairways, routes, TSS).

xxx

## Decommissioning

Marking of working area around offshore constructions in accordance to IALA O-139.

Survey operation after recovery of offshore constructions.

After all constructions including cabels and pipelines are removed, it is important to verify the current depth in the area by a survey operation in accordance to IHO S-44.

In the event that an obstruction remains which constitutes a danger for the mariner, it is recommended to mark the location based on a risk assessment.

# acronyms AND ABBREVIATIONS

AIS – Automatic Identification System

AtoN – Aid(s) to Navigation

GIS – Geographic Information System

GOC – (Global Ocean Commission)

GNSS – Global Navigational Satellite System

ICZM – Coastal Zone Management or Integrated coastal management (ICM)

MSI – Maritime Safety Information

MSP – Marine Spatial Planning

MSP – Maritime Spatial Planning

MSP – Also: Maritime Services Portfolio as used in e-Navigation

OREI – Offshore Renewable Energy Installation

TSS – Traffic Separation Scheme

UNCLOS – United Nations Convention on the Law of the Sea.

VTS – Vessel Traffic Services

WOC (World Ocean Council)

# Definitions and Interpretations

GIS – Geographic Information System

Besides displaying different layers on a geographic background, GIS can be used to automatically add safety distances around objects, detect conflicting spatial features, etc. In Section  GIS is discussed in depth.

GOC (Global Ocean Commission)

The Global Ocean Commission, an independent body of international leaders based at Somerville College with the aim of reversing the degradation of the ocean. See http://www.globaloceancommission.org.

ICZM – Integrated Coastal Zone Management or Integrated coastal management (ICM)

A process for the management of the coast using an integrated approach, regarding all aspects of the coastal zone, including geographical and political boundaries, in an attempt to achieve sustainability.

MSP – Marine Spatial Planning

Marine spatial planning is a process that brings together multiple users of the ocean – including energy, industry, government, conservation and recreation – to make informed and coordinated decisions about how to use marine resources sustainably. MSP generally uses maps to create a more comprehensive picture of a marine area – identifying where and how an ocean area is being used and what natural resources and habitat exist. It is similar to land-use planning, but for marine waters. (wikipedia 2013)

Marine spatial planning is a public process of analyzing and allocating the spatial and temporal distribution of human activities in marine areas to achieve ecological, economic, and social objectives that usually have been specified through a political process. Characteristics of  marine spatial planning include ecosystem-based, area-based, integrated, adaptive, strategic and participatory. (UNESCO-IOC)

MSP – Maritime Spatial Planning

It's about planning when and where human activities take place at sea – to ensure these are as efficient and sustainable as possible. Maritime spatial planning involves stakeholders in a transparent way in the planning of maritime activities (see also Integrated coastal zone management). (EU Maritime Affairs)

Essentially both Marine Spatial Planning and Maritime Spatial Planning address the same process. Which term is used depends on the perspective of the user: marine ecology or maritime activities.

MSP – Also: Maritime Services Portfolio as used in e-Navigation

OREI – Offshore Renewable Energy Installation

Those Installations include wind farms, wave energy and tidal energy installations. Offshore wind farms, which represent a vast majority of present OREI, require much more sea area per kW than the latter two.

UNCLOS – United Nations Convention on the Law of the Sea.

Coastal states are obliged to allow innocent passing of vessels through their waters. Important in this context is that UNCLOS allows a coastal state to declare a range with maximum 500m radius around an offshore structure as safety zone, forbidden for any traffic.

WOC (World Ocean Council)

The World Ocean Council brings together the multi-sectoral ocean business community to catalyze global leadership and collaboration in ocean sustainability and "Corporate Ocean Responsibility" – see http://www.oceancouncil.org.

- links to relevant sources

SEANERGY 2020

A comprehensive overview of the laws, conventions and agreements that influence MSP in Europe is given by the EU project Seanergy 2020, see [Seanergy, 2011]. SEANERGY 2020 was an EU funded project – Intelligent Energy Europe programme – and ran from May 2010 to April 2012. It was coordinated by the European Wind Energy Association.

The project provided an in-depth analysis of the national and international Maritime Spatial Planning (MSP) practices, policy recommendations for developing existing and potentially new MSP for the development of offshore renewable power generation, and promoted acceptance of the results. The international instruments are distinguished from EU and regional instruments in the report. The perspective of the report is that of an OREI developer.

The EU Integrated Maritime Policy

The Integrated Maritime Policy seeks to provide a more coherent approach to maritime issues, with increased coordination between different policy areas. It focuses on:

- Issues that do not fall under a single sector-based policy e.g. "blue growth" (economic growth based on different maritime sectors).

- Issues that require the coordination of different sectors and actors e.g. marine knowledge.

Specifically it covers these cross-cutting policies:

- Blue growth

- Marine data and knowledge

- Maritime spatial planning

- Integrated maritime surveillance

- Sea basin strategies

http://ec.europa.eu/maritimeaffairs/policy/index_en.htm

The EU MSP Directive 2014

DIRECTIVE 2014/89/EU OF THE EUROPEAN PARLIAMENT AND OF THE COUNCIL

of 23 July 2014

establishing a framework for maritime spatial planning

http://eur-lex.europa.eu/legal-content/EN/TXT/PDF/?uri=CELEX:32014L0089&from=EN

Almodovar, M. et al., 2014.  TPEA Good Practice Guide.  Liverpool: University of Liverpool

Lessons for Cross- Border MSP from Transboundary Planning in the European Atlantic.  This good practice guide is the outcome of a project co-funded by the European Commission (DG Mare) called Transboundary Planning in the European Atlantic (TPEA), which ran from December 2012 to May 2014.  The aim of the project was to demonstrate approaches to transboundary maritime spatial planning in the European Atlantic region.

- http://www.tpeamaritime.eu/wp/wp-content/uploads/2014/11/TPEA_bestpract_34_download.pdf

Look at IALA reference documents and assess interrelationship with this document.

Geoportal and links for MSP in France (ministry of ecology, sustainability and energy - http://www.geolittoral.equipement.gouv.fr)

Australian Maritime Spatial Information System – AMSIS -http://www.ga.gov.au/marine/jurisdiction/amsis.html

ISO standards / MEDIN (European)

United Nations Conference on Trade and Development (2012).Review of Maritime Transport (2012).

International Maritime Organization (2012).Maritime Knowledge Centre – International Shipping Facts and Figures – Information Resources on Trade, Safety, Security, Environment.

England marine planning portal, a consultative tool for viewing and commenting upon information for MSP http://www.4coffshore.com/offshorewind/

The Shipping Template	MCA

See MGN 371 Offshore renewable energy installations

http://webarchive.nationalarchives.gov.uk/20140609050314/https://www.gov.uk/government/publications/mgn-371-offshore-renewable-energy-installations-oreis

draft interim guidance for mariners operating in the vicinity of wind farms 	MCA

http://webarchive.nationalarchives.gov.uk/20080613013029/http://www.redensigngroup.org/draft_interim_guidance.pdf

Guidance on the assessment of the impact of offshore wind farms	- DTI /MCA / DfT / BMT

http://webarchive.nationalarchives.gov.uk/20130605183043/http://www.dft.gov.uk/mca/mcga07-home/shipsandcargoes/mcga-shipsregsandguidance/mcga-windfarms/offshore-renewable_energy_installations/guidance-on_the_assessment_of_the_impact_of_offshore_wind_farms.htm

MSP a step-by-step approach	UNESCO

http://www.unesco-ioc-marinesp.be/goto.php?id=ac1dd209cbcc5e5d1c6e28598e8cbbe8&type=docs

Marine Spatial Planning 	the Nautical Institute

http://www.nautinst.org/download.cfm?docid=9423102B-A083-4C8D-94B6BB215544BB42

Marine Planning Handbook	National Ocean Council

http://www.whitehouse.gov//sites/default/files/final_marine_planning_handbook.pdf

Interaction between offshore wind farms and maritime navigation	PIANC WG161

(tbd)

- Case studies

  - In Australian example, NE shipping management plan maps shipping, likely increase in use, and assesses this against collision, powered grounding, drift grounding, structure failure / foundering, fire / explosion, oil spill. Maps of change give colours that indicate what additional AtoN / VTS measures need to be taken. HOWEVER – Need to ensure that approach to assessment of risk is appropriate for the scale / intensity of use in an area i.e. the North Sea will be different from the Antarctic.

Possible process from A Description of the Marine Planning System for England (Defra, UK, 2011):

- Categorisation of GIS layers for MSP

Annex A – Categorisation of GIS layers for MSP

General

Relevant Sea area

- Overview of the Sea area with borderlines

- Parts of Continental Shelves

- 12 miles zones

- EEZ

- Traffic separation schemes (TSS)

- Deep Water Routes (DW)

- Ferry and RoRo lines (routes)

- Ports (geographic locations)

Present

- Infrastructure and Operations

  - VTS centers

  - VTS coverage

  - VTS areas of responsibility

  - VTS areas of attention

  - VHF coverage

  - Radar coverage

  - AIS base stations + coverage (shore- & offshore based)

  - DGNSS stations + coverage (shore- & offshore based), incl. accuracy DGPS IALA beacons, RTK stations whereavailable, AIS stations providing message type 17

  - Terrestrial  Loran stations plus coverage ( incl. accuracy)

  - d-Loran stations plus coverage, incl. accuracy

  - R-Mode, incl. accuracy

  - Coastguard Centers (MRCC/JRCC)

  - SAR areas

  - NAV areas

  - A1 + A2 + A3 areas for GMDSS

  - Stations providing Maritime Safety Information (MSI) + coverage

- Offshore Constructions (present)

  - Existing oil and gas platforms, inclusive safety areas

  - Existing windmill farms

  - Existing pipelines and cables

  - Existing Remote ports

  - Fish farms

- Present Specific Areas

  - Environmental protected areas

  - Anchorage areas

  - Military Practice areas

  - Dumping areas

  - Economical interest areas (Oil and Gas fields)

  - All other restricted areas relevant for accessibilityand/or safety

- Present Traffic

  - Density of present traffic (AIS and additional sources) on a year-to-year basis

  - Density of present traffic (Route Topology Model)

  - Density maps (varieties of types of ships)

  - Overall density map including general total of  ship movements

  - Density maps cargo flows

  - Ports  (incl. figures of incoming/outgoing traffic)

  - Ports and intermodality streams

  - Pleasure craft concentration areas

  - Main Fishing areas

- Accidents

  - Accidents and incidents

  - Accidents, distinguish types of accident and incidents

    - Collision

    - Grounding

    - Fire

    - Loss of cargo

    - Human factors

    - Reported near misses

    - Pollution

    - Types of vessels involved in accidents

  - Accidents and depth contours

  - Accidents and wind farms, oil platforms

- Dangerous areas and hot spots

  - Indication of present “dangerous areas”

  - Indication based on Route Topology Model

Future

- Offshore constructions

  - Foreseen oil and gas platforms inclusive foreseen safety areas

  - Foreseen windmill farms

  - Foreseen pipelines and cables

  - Foreseen remote ports

  - Foreseen fish farms

  - Foreseen Marine Spatial Planningplans/policies of country or ports

  - (Planned, under investigation or foreseen)

- Future Traffic

  - Density predictions

  - Density and limited space (oil platforms, wind farms, restricted areas etc.)

  - Ports growth expectations

- Foreseen dangerous areas and hot spots

  - Indication of foreseen “dangerous areas” and “traffic hotspots” based on increase of variables

Variable constraints

- Constraints on accessibility and velocity of traffic

  - Time constraints (symbols of tidal windows for seaports and presence of locks)

  - Restrictions mandatory pilot services for ports

  - Wind force and directions in certain periods of the year

- List of risk sources

| source | Risk | considerations |
| --- | --- | --- |
| Offshore oil&gas |  |  |
| De-comissioning | - taken out of production  AtoN failure - platform itself dark - lack of attention - structural integrity | Enforcing difficult Who is responsible? Was AtoN working or not |
| production | Environmental risk when hit | 500m exclusion zone |
| Wind farms |  |  |
| Gravity base or Jacket instead of monopile | Large damage to ship | Exclusion zone Differentiate? Confusion. |
| Ramming/ drifting | Damage to ship (type of construction/ type of collision) | Rounded forms Direction current/wind? tug |
| Division in smaller wind farms | Passage for smaller vessels may lead to other accidents SAR |  |
| Layout of windfarm | Outlying turbine | Regular layout. Fishermen: wind direction Boundaries determined by other factors SAR |
| Maintenance | Extra ship movements Risk from passing leisure boats Tourist attraction | Accommodation platform |
| Construction phase | Tourist attraction… |  |
| Turbine breakdown | Floating blade Ships in area |  |
| Other inexperienced companies jump in | - |  |
| Submarines | Collision | Sonar on corners (Gemany, Estonia) |
|  |  | Foghorn internal structures? |
|  |  | Aviation lights only when aircraft near Infrared lighting |
| Ice | Buoys / lights damaged (Ice on blades: turbine stops) | Deicing |
| Underwater structures |  |  |
|  | Visibility |  |
|  | UKC |  |
|  | Many small sites: proliferation of special marks. Different ways of marking | Big fields: cardinal marks |
|  | Unregulated expansion |  |
| Different types of aquaculture | Also deepsea + accommodation vessel |  |
|  | Off-station |  |
| Maintenance | Raised cross-bars |  |
| Virtual AtoN | May fail Not everybody can use it Security issues | Monitor responsibility Only when off-station? |
| Submarines |  | No sonar on fish farm |
| Floating device | Only marks visible | Requirements for marks |
| Changing traffic patterns due to offshore activities |  |  |
| Changing bathymetry |  |  |

Guidelines should have Annexes.  Appendices are attached to Annexes.

ANNEX HEAD1

Body Text

To restart the Annex Heading numbering at 1:

- Office 2003, go to Format / Bullets and Numbering / Restart numbering (lower left in the box)

- Office 2007, go to down arrow next to Numbering icon and select Set Numbering Value

Annex Heading 2

Body text

Annex Heading 3

Body Text Indent

Annex Heading 4

Body Text Indent 2

- Appendix title

- APPENDIX Heading 1

Followed by Body Text

  - Appendix Heading 2

Followed by Body Text

    - Appendix Heading 3

Followed by Body Text Indent

Clarke, K. C., 1986. Advances in geographic information systems, computers, environment and urban systems, Vol. 10, pp. 175–184