External Liaison Report

| Meeting Title | 28th CIE Session in the International Year of Light |  |  |
| --- | --- | --- | --- |
| Location | Manchester, UK | Date | 29-4/07/15 |

# Justification

| Purpose of Meeting | To attend Presentations on up to date practices, Attend Division meetings and to participate in Technical Committees representing the Maritime Sector |
| --- | --- |
| Paper/Presentation | Apparent Intensity of Flashing Lights, Poster from Manchester University on LEDs in Fog, IALA/CIE Liaison Report |
| Side Meetings | See below |
| Other |  |

# Report

## Introduction

The International Commission on Illumination - also known as the CIE from its French title, the Commission Internationale de l´Eclairage - is devoted to worldwide cooperation and the exchange of information on all matters relating to the science and art of light and lighting, colour and vision, photobiology and image technology.
With strong technical, scientific and cultural foundations, the CIE is an independent, non-profit organization that serves member countries on a voluntary basis. Since its inception in 1913, the CIE has become a professional organization and has been accepted as representing the best authority on the subject and as such is recognized by ISO as an international standardization body.

The success of an organization such as the CIE depends upon the effectiveness of its technical committees. Indeed, the objectives of the CIE could not be attained without a suitable and active committee structure which draws upon the expertise of people from all the member countries. Each major subject of interest to the CIE was assigned to one of seven Divisions:

- Division 1: Vision and Colour

- Division 2: Measurement of Light and Radiation

- Division 3: Interior Environment and Lighting Design

- Division 4: Lighting and Signalling for Transport

- Division 5: Exterior Lighting and Other Applications

- Division 6: Photobiology and Photochemistry

- Division 7: Not allocated

- Division 8: Image Technology

The GLA’s and IALA have a long history of representation at CIE. Of particular interest are divisions 1,2 & 4.

Every four years the CIE hold a Conference in conjunction with Division meetings.

The conference was well attended with 492 delegates from over 40 Countries.

With over 70 technical presentations, 6 workshops, 82 posters, Division Meetings and Technical Committees it was impossible to attend all. However, items of interest to GLA/IALA are as follows:-

## Presentations

CONE FUNDAMENTALS: PAST, PRESENT AND FUTURE

Viénot, F.

Since the establishment of the 1931 standard colorimetric observer, considerable knowledge

on colour vision mechanisms has been acquired. Establishing colorimetry directly on

Physiology is a new approach to colour specification. CIE technical committee 1-36 (TC 1-36) was established for proposing chromaticity diagrams based on the best set of colour-matching functions and cone fundamentals available to date. A physiologically designed chromaticity diagram named the MacLeod-Boynton chromaticity diagram (MacLeod and Boynton, 1979) is proposed as well as a cone-fundamental-based (xF , yF) chromaticity diagram. All data are now fixed. Tables are available in the CIE publications (CIE, 2006; CIE in press).

This paper will have an impact on IALA Recommendations. The Author will review the ‘In press’ publication when announced and report on the implications.

VALIDATION OF THE USE OF ZERO-LENGTH PHOTOMETRY IN THE

GONIOPHOTOMETRY OF SOLID-STATE LIGHTING DEVICES

Bergen, A.S.J.

A long test distance is important for far-field goniophotometry of luminaires which have narrow beam angles; significant non-luminous spaces between luminous areas; steep luminous intensity gradients; tight cut-offs; or which have other glare control issues. However

implementing a long test distance in a laboratory can be expensive in terms of the allocation

of real-estate; cost of building construction; and room environmental control.

In goniophotometry of maritime signalling devices such as lighthouse beacons and port entry

lights, it is common to use a parabolic mirror for foldback. The detector is placed in the focal

plane of the mirror: thus it is effectively in the far-field of the light source’s luminous intensity

distribution. This method also has application in the goniophotometry of LED luminaires

where a short test distance may result in errors in the cases outlined above. This paper

details the implementation of a goniophotometer using this method and provides validation of measurement of a specially-designed LED luminaire.

This paper was produced on the back of the author promoting the benefits of light measurement capability at IALA. The system was installed for SeaLite, a manufacturer and supplier of Marine AtoNs and replicates the system used by R&RNAV.

THE APPARENT INTENSITY OF A FLASHING LIGHT

Nicholson, M.,Tutt, I.

Flashing signal lights are viewed by mariners at night from a distance, so the light appears as a point source. In order to understand how the human observer perceives a flashing point

source light over a range of illuminance levels, a visual experiment was carried out at Harwich, UK, where a number of observers compared the brightness of a flashing light to that of a steady light of known illuminance. This experiment was based on work carried out in 1933 by Toulmin-Smith and Green, at Farnborough, UK (ref RPT-11-MN-IT-2015).

This paper was well received and there seemed to be genuine interest in work on supra-threshold flashing lights because many realise that signal lights need to convey information, and that cannot be reliably achieved at threshold levels. There is enough interest within CIE to start a Technical Committee on supra-threshold work but the terms of reference would need to be carefully thought through. Unfortunately, there doesn't seem to be the will or the money available in most organisations to carry out experimental work but there is expertise in abundance to interpret R&RNAV work in this field. That alone could be very useful.

## Workshops

Adaptive Lighting and Visibility

This workshop was specifically aimed at the ‘Street Lighting’ or ‘Roads’ transport sector and looked at switching on street lighting during reduced visibility and dimming or switch off in reduced traffic. However, there are a number of parallels with other transport sectors, such as, Balancing the benefits and cost, Bayesian Analysis, Levels of Service and Managing expectations. R&RNAV are about to commence a project on assessing the effectiveness of switching on AtoN in reduced visibility and how this can be achieved and will take account of the experience of the road transport sector and feedback to CIE on the outcomes of the project.

CIE Illuminant L (LED) Pros, Cons and Challenges

This workshop discussed whether CIE should develop an illuminant standard/standards for LEDs. It is important to note that a standard illuminant is a set of theoretical spectral power values based on an ideal. E.g. Illuminant D65 is a theoretical approximation of sunlight and is used to calculate/design how surfaces might appear when sunlight is reflected off them.

The general feeling was that, at present, there was no requirement to develop a standard illuminant for LEDs.

## Division Meetings and Technical Committees

The author attended the Division 2 (Measurement of Light and Radiation) meeting and presented the IALA/CIE Liaison Report. A brief overview of the structure, history and membership of IALA was given. A more detailed section on the impact of CIE publications on IALA publications was presented. A delegation from CIE visited IALA at their last meeting in Paris. The Division Director (Peter Blattner) thanked the author for arranging the visit and also asked that thanks be passed on to IALA. It is apparent that CIE took a number of positives away from IALA that are reflected in the new Code of Procedure. The general format of the meeting is much like IALA with a plenary dealing with administration details as well as report from other bodies. They then spilt out into Technical Committees and return to a final plenary to discuss the progress. Technical Committees of interest to the GLA/IALA are as follows:-

TC2-70 Standards for the Measurement of Reflectance and Transmittance Properties of Materials (D. Rich)

This TC has been closed by Central Bureau due to lack of progress.

TC2-49 Photometry of Flashing Light (Y. Ohno)

This TC did not meet in Manchester as the Technical Report is its final draft stage. There have been a number of disagreements amongst TC members as to validity of the proposed impulse response function q(t), which is the mathematical model of how the eye responds to a flash of light. At the time of writing the author is unsure whether the report will be published. However, all parties are agreed that the R&RNAV work on brightness matching of different flash profiles is key to choosing the correct impulse response function needed to finalise the convolution method.

TC2-65 Photometric Measurements in the Mesopic Range (T. Goodman)

The work on this TC is nearly complete. After discussing the photometric quantities and their symbols, agreement was reached within the TC and a report will be forthcoming in a few months.

TC2-71 CIE Standard on Test Methods for LED Lamps, Luminaires and Modules (Y. Ohno)

This TC has concluded its work and produced a Standard (CIE S 025/E:2015) Test Method for LED Lamps, LED Luminaires and LED Modules. The author will study the Standard and determine any impact it may have on light measurements of LEDs.

TC2-74 Goniospectroradiometry of Optical Radiation Sources (J. Pan)

R&RNAV have pioneered this measurement technique and is a member of this technical committee to ensure that any technical output does not preclude those developed by R&RNAV.

Other Technical Committees (TC’s) attended, of interest to the GLAs are as follows:-

TC4-47 – The Application of LEDs in Transport Signalling and Lighting (S. Jenkins)

This TC has been somewhat hijacked by the very strong representation in Division 4 from the Road transport sector. The Marine section was written four years ago and needs updating to reflect the current status. However, only minor updates are required.

It was decided that this technical report should be split into two separate reports, one for roads, and one for the others (Aviation, Marine and Rail).

Minor updates are required with a view to publication in the latter part of 2015.

TC1-84 – Definition of a Visual Field for Conspicuity

This TC is progressing well and the author proposed the adoption of a 3-step perception model (as used by IALA) as well as incorporating an experienced observer vs. non-experienced observer model. However, more visual data is required for this. The author was requested to provide RPT-03-MN15 Conspicuity of GLA Daymarks as input.

## Side Meetings

The author was approached by a number of the academic community with regard to the possibility of collaboration work.  Contact details were taken should any suitable topics arise.

John Bullough from the Lighting Research Centre (USA) asked if the GLA had done any work on fixed flashing lights as the road transport sector had started to use them for marking road works. The author agreed to send R&RNAV Report RPT-28-MN-13 on Fixed Flashing Lights.

Prof. Ron Gibbons (Division 4 Director) asked the author to be a member of the new Joint Technical Committee on the Revision of CIE143:2001 International recommendations for colour vision requirements for transport which was originally requested by IMO.

# Actions

- MN review the ‘Cone Fundamentals’ publication when announced and report on the implications.

- MN to input to CIE on the outcome of the R&RNAV project on AtoN in reduced visibility.

- MN to pass on thanks to IALA for hosting the delegation from CIE.

- MN to review CIE S 025/E:2015 and report any implications for LED light measurements

- MN to review TC4-47 Signals document.

- MN to provide RPT-03-MN15 Conspicuity of GLA Daymarks as input to TC1-84

- MN to send R&RNAV Report RPT-28-MN-13 on Fixed Flashing Lights to John Bullough.

Author:	M. Nicholson	Date:	23rd July 2015

Approved:		Date: