Input paper:	     ENG3-9.9

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM	X  ENG		□  PAP					□   Input

□  ENAV	□  VTS							X   Information

Agenda item			9

Technical Domain / Task Number	TD#1

Author(s) / Submitter(s)			Mr. Ju-Seop Han/Korea Association of Aids to Navigation

Development of LED Large Beacon(LLB) in Korea

Aids to Navigation(AtoN) is important facility to give visible navigation information to a ship. The previous distribution planning of AtoN was carried out by using a expert's experience and a marine map. The AtoN beacon is important facility to improve safety and sailing efficiency of ship during day and night. Metal halide lamp, currently using dominant light source of the beacon, is applied high voltage and has short lifetime due to high lamp voltage. The necessity  for AtoN technology has through the application of modern light sources.

The LED light source by replacing the pre-existing old group was clear and reducing the power and maximize the available light. This is a system that can support the effectiveness of the safe navigation and operation of a vessel. A metal halide lamp used as a light source, such as a large conventional rotating beacon is take typically 5 to 10 minutes until lighting and required restart time is about 8 to 15 minutes. Under severe cold, it takes up to 20 minutes until normal lighting. These disadvantages (pre-heating and cooling time), necessary from external environment which is typical in marine, make a beacon difficult to quick lighting and turn off. To compensate for such disadvantages, many lighthouses facilitate additional separate high temperature re-lighting unit. In addition, fatal disadvantage of a high lamp voltage is short life time, which requires abundant work for replacing lamps with high cost or even impossible for an autonomous lighthouse.

To overcome such disadvantages, especially critical for a large beacon, Korea has developed a rotating LED large beacon by Korea Photonics Technology Institute (KOPTI) and Korea Association of Aids to Navigation (KAAN).  We applied LED modules and a large reflecting mirror and developed the LED large beacon(LLB) with variable luminous intensity from 20M to 27M (T=0.74) including the light output and flashing control system. Large LED Beacon was applied to light optic technology, optimized heat dissipation and compact mechanical design technologies.

The design requirements of Korean LLB are as follows;

- Longer life time of LLB

- Durability of optical components under marine environment

- Automatic switching-off on emergency occasion

- Minimizing maintenance cost

To satisfy above requirements, we have designed a proper LLB system which has following characteristics. The vertical divergence angle is measured of ±3.2 degrees and the horizontal divergence angle ±4.5 degrees (FWHM). Optical system of beacon is configured to 2 converging lens, reflector, and compound optical system of the Fresnel lens. For efficient LED heat dissipation, Non-directional rapidly inverse spread devices and using a reflector rotational force convection LED heat dissipation is developed. As a structure capable of cooling the heat sink of the outside air flows into the housing by the rotating force, it is possible to minimize of weight and volume the heat sink like a forced convection system. Table 1 shows the summarized system specification of the designed LLB developed in Korea.

Table 1 System Specification of designed LED Large Beacon developed in Korea

|  | Max. Power[W] | Optical Performance |  | Max. Intensity | Auxiliary Function |
| --- | --- | --- | --- | --- | --- |
|  |  | Vertical | Horizontal |  |  |
| Value | 700W | ±3.2° | ±4.5° | 3,700,000 | Automatic switching |

The Korean LLB system consists of the three major modules. First, High Power Lighting Engine (HPLE), which includes LEDs, MCPCB, a series of optics with mechanical holders, rapid thermal dissipation pipes illuminates narrow beam to the second components, a rotational optical component (i.e., a Mirror and a folded Fresnel lens) which is run by BLDC motor and hold by a mechanical fixture.  The third components are mostly electronics and electrical modules allowing maximum 700W and drive COB LEDs (36ea) and the motor stably even under emergency occasion. The Figure 1 show HPLE and the rest of components are illustrated in Fig. 2

Korean LLB adopted highly durable and efficient optics and electronics which has resulted in a high energy efficient beacon. The use of COB LEDs (36ea) with a meantime between failure (MTBF) of 100,000 hours is particularly effective for minimizing maintenance cost and under emergency occasion which needs no additional battery based requirements in overall design. The main controller system monitors status of driving condition and detects malfunction of the drivers and automatically switch off in case of emergency condition. And the main controlling system communicates with the drivers through 14 different channels where LEDs consumes 60W power each. Table 2 shows representative driving characteristic of the HPLE.

Table 1 Driving characteristic of the HPLE

|  | Input Voltage | Output Voltage | Typical Power Consumption of single module group | Auxiliary Function |
| --- | --- | --- | --- | --- |
| Value | 4.7Vdc ~ 60Vdc | 0Vdc ~ 60Vdc | 60W(35 Vdc, 2A) | Analog Dimming, Operating Current Monitoring |

2nd optical lens

1st optical lens

LED modules

Heat pipes

BLDC motor

Fig.1 Scheme of HPLE in LLB with rotating BLDC motor

Major mechanical design consideration of the LLB is as follows;

- Minimizing total weight to reduce the  dynamic capacity of a motor as well as its longer life time

- Preventing inflow of any insects from marine environment.

- Rapid thermal dissipation mechanism with rotating mechanical components

Mechanical analysis of the LLB shows that junction temperature(Tj) of the LED is less than 85℃ under which guarantee the life time of the LED is more than 50,000 hr. Maximum case temperature was about 70℃ at room temperature (25℃).

In the optical simulation results, the 1st optical lens forms diverging angle of the LEDs with ±3.74° at optical efficiency of 86% and vertical diverging angle through the designed Fresnel lens are about ±3.2°. The system complexity which is inevitable for the design requirement with a series of optics and 36 LEDs causes more diverging angle of LLB because of mechanical tolerance.

The Korean LLB, comprising an HPLE of 670 watts, with a high reliability and a stationary intensity of 3,700,000 candles, giving a range of more than 27 nautical miles. Measured Vertical divergence of the lens in HPE at 50% of the peak intensity is about ±3.2°. Programmable flash characters and tuneable power consumption allow Korean LLB to be adjustable other middle range beacons for various purposes.

Fig.2 Main mechanical structure (left) and overall design of LLB (middle) with the driving boards (right)

Future Tasks

In future tasks, we will provide visibility measurement results in the field.