Input paper: 	     ENG3-10.29.1

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM	  ENG		□  PAP					□  Input

□  ENAV	□  VTS							  Information

Agenda item

Technical Domain / Task Number 2	…………………………………

Author(s) / Submitter(s)			…Leif Arne Larsen………………

# Comparison of Solar Model Performance

# Summary

This paper presents information from the RETScreen model, for comparison with the IALA solar model during EEP3 WG2. Given the reference load information representing a buoy electrical system in appendix 1, the indication is that 1 battery bank and 3 solar modules are required when using the RETScreen model (Alt 1).

## Purpose of the document

Forward to WG2 for information to their on-going work.

## Related documents

ENG2 Working Paper 11.2.3

Guideline 1039

# Background

During EEP2, Task Group 2 reviewed the input data that had been provided in the form of the results form a number of solar models. This allowed for a comparison of solar sizing techniques so that it would inform the group as to any update that could be applied to the IALA solar calculator.

During this review, which was focussed around a lighthouse system, it was felt that the members should rerun their models, but this time base it on a buoy system. This it is believed will determine if there are any factors associated with an afloat situation, that was not identified earlier.

Members were requested to use their own calculators and submit the results for comparisons and discussion. Trinity House has provide the load, location, battery and solar module data presented in appendix 1

# Discussion

## IALA solar model

The load data detailed in Appendix 1 was applied to the IALA energy model. The output indicating that to allow for a 43 day period of no sun, the system needs to have 2 solar modules and 2 bank of batteries (140 Ah) as detailed in Figure 1 below.

| IALA Solar Model (embedded) | IALA HA Buoy |
| --- | --- |

IALA Solar Model Summary

## RETScreen solar model alt.1

The load data detailed in Appendix 1 was applied to the Retscreen energy model. The output indicating that to allow for a 35 (70/2) day period of no sun, the system needs to have 3 solar modules and 1 bank (70 Ah) of batteries as detailed in Figure 2,3,4  below.

| RETScreen Solar Model Alt 1.(embedded) | Alt 1 Retscreen HA Buoy - SC minimum og autonomi som IALA |
| --- | --- |

RETScreen Solar Model Alt 1 summary

RETScreen Solar Model Alt 1 battery capacity

RETScreen Solar Model Alt 1 production/load

## RETScreen solar model alt.2

The load data detailed in Appendix 1 was applied to the Retscreen energy model. The output indicating that to allow for a 61 (122/2) day period of no sun, the system needs to have 2 solar modules and 2 bank of batteries ‘(140 Ah) as detailed in Figure 5,6,7 below.

RETScreen Solar Model Alt 2 summary

RETScreen Solar Model Alt 2 battery capacity

RETScreen Solar Model Alt 2 production/load

| RETScreen Solar Model Alt 2.(embedded) | Alt 2 Retscreen HA Buoy - SC minimum og autonomi som IALA |
| --- | --- |

Solar Model Data

Figure 1 - Photo of HA Buoy

Back Ground Information

This station is known as the HA Buoy

Type: Safe water mark

Location – At the entrance to Harwich and Felixstowe

Lat: 51º 56.756' N

Lon: 001º 30.665' E

Figure 2 - Location of HA the Buoy

# Hardware Details

Solar Module Type: BP525 - 25W panel – See data sheet

Solar Panel orientation: vertical on the sides. Maximum of 5

Battery type: Sonnenschein Gel battery type GF12 063 Y = 70Ah @C20 to 1.80v @ 30°C = 80.6Ah @ C100 to 1.80v @ 30°C – See data sheet

Table 1 - Data Sheets

| Solar Module Data Sheet |  |
| --- | --- |
| Battery Data Sheet |  |
| Charge regulator |  |

Lantern

Solar Modules

Charge regulator

Battery

Figure 3 - System Block Diagram

System Concept

# Effective Loads

## Main Light

LED Lantern, 5nm white light

Flash Character of Isophase 5s (flash length 2.5s on + 2.5s off)

Operations is night time only

Power night time operation = 0.85W

Power during the daytime = 0.048W

## Charge Regulator

Continuous load = 0.84W

# System fundamentals

The battery system voltage is 12v

The system autonomy needs to be 30 days without any solar input.

100% of the battery capacity is available for use.

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain