VTS30-output-XX

IALA VTS Committee

#### Liaison note to the IALA Council

on

#### THE ROLE AND POSITION OF VTS IN VTM

Aim of the document is to

Inform the IALA Council in accordance with the results of this document

The IALA Council is requested to

- take notice of the results and views of the VTS Committee as reflected in this document

- approve the developed vision, definitions, clarifications and views on the various aspects.

VTS30-output-XX

IALA VTS Committee

#### THE ROLE AND POSITION OF VTS IN VTM

1.	Introduction

The IALA VTS Committee was tasked to develop a view of the future role and tasks of VTS within the concept of VTM in respect to “allied and other services” such as Law Enforcement, Security, Search-and-Rescue (SAR) and Incident management.

2.	Consideration of the task

The VTS Committee studied the task as mentioned above and concluded that a thorough explanation of the role and position of VTS within the VTM functional framework required a more extensive evaluation of present and future services by VTS and the relationship between VTS and the stakeholders within the VTM framework.  Therefore, it was decided that the results on the subject should include:

- a view on the role and position of VTS within the VTM framework;

- views on possible changes to current VTS measures and services (primary services);

- views on the possible contribution of VTS towards current and future services such as:

              - Security;

              - Law Enforcement;

              - Remedial services;

              - Allied services;

              - Other or supportive services.

The possible stakeholders related to the current and future VTS services were identified.  Also studied were present legislation, guidelines and manuals in order to identify necessary modifications and inconsistencies.

Although the document was not finalized completely due to the complexity of the subject the Committee decided that the results achieved so far should be shared with the IALA Council.

The possible contribution of VTS towards Security is reflected in a separate document (VTS30-output-XX) for approval by the Council.

The task, to be continued in the Work Programme 2010-2014, will be completed at VTS31 focussing on the possible contribution of VTS within the VTM framework towards the current and future services as law enforcement, remedial services, allied services and other or supportive services.

3.	Objectives of this document

The objectives of this document are to:

- Inform the IALA Council about the views of the VTS Committee concerning the role and position of VTS within the VTM functional framework

- Indicate the views and proposals considered necessary for discussion in the VTS Committee with respect to the provision of measures and services by VTS to stakeholders within the VTM framework;

- Identify the relationships between VTS and its related stakeholders in order to:

  - further develop the functional requirements for measures and services within the VTM framework;

  - develop the functional architecture for both the VTM framework and the e-navigation concept; and

  - identify other related issues, which may need the attention of the Committee, other committees, working groups or bodies, in order to be investigated, developed or resolved.

- Give guidance to the relevant discussions in both the VTS and e-NAV Committees.

The document includes:

Paragraph 4	-	Summary of the functional framework of VTM

Paragraph 5	-	Further clarification on VTM

Paragraph 6 	-	Strategic, tactical and operational domain of VTM

  - strategic, tactical and operational efforts

New definition:		Supervisory Control

Paragraph 7	-	The role and position of VTS within the VTM functional framework

New definition:		the role of VTS within VTM

New definition:		the position of VTS within VTM

New definition:		the information position of VTS within VTM

Summary and considerations in respect to

- Relevant legislation, Recommendations and Guidelines

- The existing and future role and position of VTS

- Information management and exchange

- Vision and developments

Paragraph 8	-	Services of VTS within the VTM framework

  - the primary services

  - INS, NAS, TOS

    - Functions INS, NAS, TOS positioned in the strategic, tactical and operational domain

    - Benefits for INS, NAS and TOS due to the positioning of VTS within the VTM framework

    - Actions requested and recommendation

4.	The functional framework of VTM1

The IALA definition of VTM is:

“Vessel Traffic Management is the functional framework of harmonized measures and services to enhance the safety, security and efficiency of shipping and the protection of the marine environment in all navigable waters”.

- The functional framework is the set of arrangements on a global, supranational, national and/or regional scale to set the conditions for safe, secure and efficient maritime traffic and for the enhancement of efficient and effective use of the resources engaged in order to enable the collaboration among the different public and private stakeholders involved.  These arrangements should be consistent, coherent and recognized by present and future (inter)national legislation and guidelines.

•   Arrangements are agreements, international treaties, understandings or other legal mandates between two or more stakeholders within the functional VTM framework on the implementation of measures, the provision of services, the coordination of efforts, the allocation of resources and any other conditions needed to attain the aims of VTM;

•   Measures are the efforts by Competent Authorities to set the conditions to be respected and the provision of services to meet the aims of VTM and the compelling needs of stakeholders;

•  A service within VTM is an activity or a combination of activities, provided by one or more stakeholders, for the benefit of VTM stakeholders.

The aims of VTM are:

- Safety of shipping and the environment;

- Efficiency of maritime transport;

- Security of shipping, ports and infrastructure;

- Meeting stakeholder demands for reliable and current information support efficient use of maritime infrastructure, the effective and reliable operation of other logistic or nautical processes.

These aims are to be achieved through the provision of a functional framework of measures and services that enable stakeholders (ship-borne and shore-based authorities/organizations at local, regional, national and international levels) to interact and exchange information to enhance:

- The decision making process in matters concerning maritime safety and security, efficiency of
navigation and of vessel traffic;

- The prevention and control of marine pollution and emissions from vessels;

- The strategic planning of vessel movements in confined and congested waters;

- The monitoring of vessel traffic in all navigable waters worldwide;

- The assistance to safe and secure berth to berth navigation

- The efficient management of vessels movements in all navigable waters;

- The services to maritime safety and security, efficiency of navigation and prevention and control of
marine pollution from vessels;

- The operation of allied services;

- The services to vessels’ routeing and waterway management;

- The optimal utilisation of the marine infrastructure and/or assets;

- The services to maritime transport and logistics;

- The services to contingency response, search and rescue, and incident and accident response.

Measures and services within the functional framework of VTM are executed on strategic, tactical and operational levels to maximize the safety, security and efficiency of vessel traffic in interaction with its operating environment worldwide, and to meet stakeholders’ demands for reliable and valuable information.

### 5	Further clarification on VTM

As defined in IMO Resolution A.857(20):

“VTS is a service, implemented by a Competent Authority, designed to improve the safety and efficiency of vessel traffic and to protect the environment.  The service should have the capability to interact with the traffic and respond to traffic situations developing in the VTS area”

However, on several occasions (seminars, symposiums, meetings), it was quite clear that there was confusion and misunderstanding how to define Vessel Traffic Management.  Although with the best intentions, this resulted in a variety of subjective statements on how VTM may be interpreted.

The IALA Council at its 45th session approved the definition and scope of VTM, as reflected in doc. VTS29/output/6 liaison note to Council; the VTS Committee established the appropriate clarification:

VTM is the global functional framework for all navigable waters, in which among other activities, VTS may be the central instrument.  This framework shall be supported by information management.

VTM therefore is not:

- “VTS+”;

- VTMIS

- a series of interconnected VTS centres;

- one or more ship monitoring systems;

- a large area “VTM centre.”;

- equivalent to e-navigation; or

- equivalent to IALA.NET.

# 6. 	Strategic, tactical and operational domain of VTM

VTM has to be seen as the overall (overarching), structured and functional framework:

- of strategic, tactical and operational efforts in interaction with its operating environment worldwide;

- to meet stakeholders’ demands for reliable and valuable information;

- to enable collaboration among public and private stakeholders; as well as

- to optimise the utilisation of the marine infrastructure.

It is recognized that stakeholders within VTM operate at a strategic, tactical and/or operational level, in order to ensure coordinated efforts among these various stakeholders.  VTS29 agreed the definitions of these efforts that encompass all measures and services that are employed in the course of berth-to-berth navigation and associated activities, including the deployment, the maintenance and the delivery of the measures and services to meet the (evolving) requirements of the stakeholders in the maritime domain.

Conclusion:

VTS as a stakeholder within VTM can operate in the strategic, tactical and operational domain of VTM.  The role and position of VTS, however, can vary for the strategic, tactical and operational levels.

6.1	Strategic efforts
The strategic efforts target the pro-active part (preparation and planning) of:

- the conditions for safe, secure and efficient vessel traffic;

- berth-to-berth navigation; and

- the timely exchange of valuable information associated with the first two bullets.

Communication at the strategic level implies the exchange of relevant information, in quality and quantity, needed to anticipate and prepare for the execution of a task or to assess the circumstances that could lead to a hazardous or undesired occurrence.

These strategic efforts may result in measures, such as policymaking.  The time horizon may range from several years to several hours.

6.2 	Tactical efforts
The tactical efforts target the active part (monitoring and decision making) of

- the safe, secure and efficient handling of vessel traffic;

- berth-to-berth navigation;

- the enforcement of or compliance with rules and regulations, and

- timely exchange of valuable information associated with the three bullets mentioned in 7.1.1

This includes communication at tactical level: the exchange of relevant information, in quality and quantity, needed to take proper action in the execution of a task.

These tactical efforts include measures and services, among others,

- monitoring of vessel traffic;

- supervision of navigation and vessel traffic handling, and

- the supervisory control of potential hazardous occurrences in confined waters.

The time horizon may range from several hours to several minutes.

In addition, tactical efforts include vessel traffic services in their aim of supporting the decision making process in the navigation of vessels and to influence their intentions, in order to improve safety and the efficiency of vessel traffic in interaction with its operating environment.

6.2.1	Clarification supervisory control

In discussing the tactical efforts of VTS within the VTM functional framework, especially the active efforts regarding the safe and efficient handling of maritime traffic were studied. The question was raised how to handle and control these active efforts. "Supervisory control" seemed to be the right concept to describe the handling and control of these efforts and activities.

It was recognized that :

- “Supervisory control” should be linked with the role and position of VTS within the VTM framework.

- In case the scope of the “Supervisory control” -concept is defined too broad and/or new tasks to VTS and responsibilities to the VTS Authority are suggested, this concept may not be supported by current regulations and recommendations. In particular the "control" of certain activities within the VTM framework could be beyond the capability of the VTS Authority.

Consequently a new definition for “Supervisory control” in this context was developed: 
"Supervisory control is an adaptive way of supervising all activities within the VTM framework, connected to the role and position of VTS in VTM, to ensure  the requirements of both the VTM Stakeholders and the VTS Authority are met".

Clarification: meeting the requirements includes the identification, assessment and control of the risks associated.

6.3	Operational efforts
The operational efforts target the reactive part (response to occurrences) of

- berth to berth navigation in interaction with its operating environment, and

- the ‘real-time’ exchange of valuable information.

This includes communication at operational level: the exchange of relevant information, in quality and quantity, to guide real time decision making.

These operational efforts include:

- the safe conduct of navigation;

- navigational assistance from a VTS;

- maritime pilotage

- tugboat and linesmen assistance.

Note: it is recognized that maritime pilotage also can occur as a tactical effort.

The time horizon may range from several minutes to immediate.

7.	The role and position of VTS within the VTM functional framework

In order to prevent confusion it should be made clear that, in accordance with the VTS Manual, the acronym VTS is used when referring to the “service”. Otherwise, specifically referring to the organization, the wording VTS Authority will be used. Furthermore it should be noted that the stakeholder of VTS is the VTS authority.

Definition:	the role of VTS within VTM is considered its functional contribution to the aims of VTM in its capacity to interact with other stakeholders.

Definition:	the position of VTS within VTM is considered its status relative to other stakeholders, in its capacity to contribute to the aims of VTM.

Although the VTM functional framework is still a concept, the existing role and position of VTS within a VTM functional framework are considered to determine the relationship between the VTS Authority and the other  stakeholders within VTM, and to what extent they interact with each other.

This interaction may benefit other stakeholders, as well as the aims of VTM, when the VTS Authority can provide them with other vital information, in addition to the provision of INS, NAS and TOS to shipping.

The more this information is valued by the receiving stakeholders, the stronger the position of VTS related to the information exchange will be.

A strong position of VTS provides the VTS Authority with the opportunity to take a central role in the management of information within the VTM functional framework. Holding a strong information position may result in a greater contribution to the benefits of VTM.

Definition:	the information position of VTS within VTM is the position in which VTS is placed relative to other stakeholders, that identifies and defines from a functional perspective its capability to collect, manage, maintain, process, provide and/or distribute data and information.

Clarification:	the information position of VTS within VTM determines its capability to manage the information exchange between stakeholders in the VTM framework, and to set the preconditions for the use and re-use of the information exchanged.

Being in the strategic, tactical and operational domain of VTM, the VTS Authority is considered to take responsibility in:

- preventing the development of dangerous situations;

- sustaining the vessel traffic flow;

- informing the stakeholders appropriately about risks of vessel traffic in the VTS area that may develop in accidents or incidents;

- To provide support to and inform other stakeholders appropriately in accordance with current and future tasks of VTS; and

- To provide support to and inform other stakeholders within the VTM functional framework in accordance with specific arrangements with those stakeholders.

Hence, a VTS should define its relationships with all stakeholders and for each relationship its subsequent information position. The information position of the VTS Authority should include a description of:

- its institutional position (legal mandate, legal structure and resources);

- its tasks and responsibilities (mission, policy, objectives);

- its information requirements (data needed for VTS tasks, to satisfy stakeholder needs, service/data sharing arrangements between the VTS Authority and stakeholders, data management, verification and validation of external data, distribution mechanisms, harmonized standards and protocols); and

- its provision of information services, including the re-use of information for and information exchange with other stakeholders (legal aspects, liability issues, confidentiality, property rights, data protection – both public and private data-).

This implies that the functionality of VTS should not be extended towards VTMIS, but that the information position of VTS should be enhanced, so that information exchange between VTS and its environment within the VTM framework is firmly established.

In order to determine the role and position of VTS within VTM, as well as the (future) provision and execution of the measures and services, a number of aspects and considerations in respect to legislation, the existing role of VTS, the compelling need for enhanced information within the maritime domain and conceptual developments should be taken into account.

### 7.1	Relevant legislation , Recommendations and Guidelines

- UNCLOS and other UN-conventions related to the maritime domain

- IMO Resolution A857(20)

- Other relevant IMO Regulations and Recommendations as mentioned in IMO Res. A857(20) and published after 1997

- IALA Recommendations and Guidelines related to VTS

- IALA VTS Manual 2008

- International and regional developments and initiatives affecting the maritime domain

# 7.2	The existing and future role and position of VTS

Legal constraints upon the provision and execution of current and future VTS measures and services should be considered by VTS Authorities. This may have an impact on the existing and future role and position of VTS:

- The existing role of VTS focuses on the safety and efficiency of vessel traffic, protection of the marine environment and the relationship with allied services;

- According to Resolution A.857(20), the primary services of VTS should be provided within a declared VTS area.  However, with respect to other existing and future functions, the role and position of VTS within the VTM framework may extend beyond the VTS area, as the VTM framework is applicable to all navigable waters.

- It should be recognized that VTM can exist without VTS. This depends on geographical and traffic circumstances and/or organizational arrangements in some areas. If this is the case, measures and services within the VTM framework, which are usually designated to VTS centres, may be executed or provided by other organizations (e.g. Coastguard, monitoring stations and reporting services);

- The VTS Authority is not always the competent authority for other measures and services within the VTM framework;

- The role of VTS in the VTM concept should be consistent with its role in the e-Navigation concept;

- The measures and services provided in the maritime domain are still evolving as a result of the developments as identified for VTM. This may lead to changes in the role or position of current VTS within the VTM framework and to new capabilities for the VTS Authority;

- The role of VTS within the VTM framework needs to be adaptable and sustainable;

- It is the responsibility of national, regional or local competent authorities, to decide on and to organize the provision and execution of the necessary measures and services within VTM and those for VTS.

# Information management and exchange

The increasing number of participants and stakeholders with interest in the maritime domain requires further consideration on information management and -exchange:

- The needs and increasing interest of certain participants and stakeholders in the information and services VTS may provide;

- Information exchange needs harmonization, standardization and interoperability;

- The present information capabilities of VTS may not fulfil all of the information needs and requirements of stakeholders within the VTM framework

- In principle, the distribution of information by VTS should be made available appropriately to stakeholders within the VTM framework. However, the wider distribution of data by VTS may be restricted by legal constraints. Aspects such as property rights, confidentiality, ownership, liability and security of data are still subject to study and investigation;

## 7.4	Vision and developments

The role and position of VTS within VTM deals with:

- The need for Vessel Traffic Management;

- The VTM functional framework, of which VTS is part, should be user driven;

- The user requirements (including needs and specifications) of individual stakeholders within the VTM framework are still subject to further identification or development, which is considered to be an ongoing process;

- The strategy, implementation and potential impact of the e-Navigation concept;

- Primary services of VTS within the VTM framework

It is recognized that a strong information position of VTS is  of increasing interest to other stakeholders in the maritime domain, in order to fulfil their compelling needs for more, better, reliable and timely information. The primary VTS Services, in their existing state, shall contribute to the VTM framework constrained by preconditions for use and re-use of information.  Their contribution shall in conjunction with other stakeholders make VTM more robust, while improving each stakeholder’s individual position within the VTM framework. This improved position is therefore expected to enhance the primary services of the VTS.

8.1	Primary services

VTS should at least comprise an information service and may also include others, such as navigational assistance service or a traffic organization service, or both of these services, defined in Resolution A.857(20), as follows:

- An information service is a service to ensure that essential information becomes available in time for on-board navigational decision-making.

- A navigational assistance service is a service to assist on-board navigational decision-making and to monitor its effects.

- A traffic organization service is a service to prevent the development of dangerous maritime traffic situations and to provide for the safe and efficient movement of vessel traffic within the VTS area.

8.1.1	Information Service (INS)

Information Service (INS)  : An Information Service provides essential and timely information to assist the on-board decision-making process.

The function of INS is outlined below and identifies the major activities that may be expected from this service type, together with an indication of the role that it can be expected to undertake.

This service type involves maintaining a traffic image and allows interaction with traffic

and response to developing traffic situations. An INS provides essential and timely marine information to assist the on-board decision-making process, which may include:

- The position, identity, intention and destination of vessels;

- Amendments and changes in promulgated information concerning the VTS area such as boundaries, procedures, radio frequencies, reporting points;

- The mandatory reporting of movements; and

- Meteorological and hydrological conditions, notices to mariners, status of aids to navigation; limited manoeuvrability that may impose restrictions on the navigation of other vessels, or any other potential hindrances.

8.1.1.1 Function positioned in the strategic, tactical and/or operation domain.

The INS functions may appear in the strategic, tactical and/or operational domain.

8.1.1.2 	Benefits for INS due to the positioning of VTS in the VTM framework

It is foreseen that the nature of INS provided will change little. However, it is expected that the exchange of information by conventional means will decrease and will be replaced by automated and digitalised options for the collection, storage and exchange of information. Therefore it is anticipated, especially for INS, that the way the service is executed will change.

It is expected that the provision of INS will benefit from the positioning of VTS within VTM.

8.1.1.3 	Actions requested

- The inconsistencies in the definitions from the VTS manual 2008 should be reviewed

8.1.2 	Navigational Assistance Service (NAS)

Navigational Assistance Service (NAS) : A Navigational Assistance Service may be provided in addition to an Information Service and/or Traffic Organisation Service. It is a service to assist in the on-board navigational decision-making process and is provided at the request of a vessel, or when deemed necessary by the VTS. A Navigational Assistance Service provides essential and timely navigational information to assist the on-board decision making process and may inform, advise and/or instruct vessels accordingly.

The function of NAS is outlined below and identifies the major activities that may be expected from this service type, together with an indication of the role that it can be expected to undertake.

A Navigational Assistance Service may be provided in addition to an Information Service or Traffic Organisation Service. It is a service that provides essential and timely navigational information to assist in the on-board navigational decision-making process. It may also involve navigational advice and/or instruction.

Navigational Assistance may be provided at the request of a vessel or when a navigational situation is observed and intervention by VTS is deemed necessary. Such assistance requires positive identification and continuous communication throughout the process. It is important that the provision of Navigational Assistance is agreed between the vessel and the VTS providing the service. Acceptance by the vessel of the Navigational Assistance Service should be established, and the beginning and the end of navigational assistance should be clearly stated.

Navigational Assistance may involve the provision of information, such as:

- Course and speed made good by a vessel;

- Position relative to fairway axis, navigational features and/or way-points;

- Proximity to navigational hazards; and

- Positions, identities, intentions and any restrictions of surrounding traffic.

Navigational Assistance may also involve the additional provision of advice and/or instruction, and may include or require:

- An assessment of the suitability of the vessel to respond to the advice provided including an assessment of linguistic ability;

- A review of vessel characteristics including manoeuvrability relative to the area in which the service is provided and any defects or deficiencies;

- An assessment of the environmental conditions;

- An assessment of the implications of the cargo carried;

- A review of the proposed sailing plan;

- Recommendations on measures to maintain the sailing plan noting that any advice on courses and speeds should be result orientated;

- The use of message markers;

- The use of a dedicated frequency; and

- Restriction of other traffic movement.

8.1.2.1 Function positioned in the strategic, tactical and/or operation domain.

8.1.2.2 	Benefits for NAS due to the positioning of VTS in the VTM framework

Although new technologies on board is not a result from the positioning ofVTS within VTM, the requests for NAS from vessels may decrease due to the increasing availability of valid information on board for decision making, and it is anticipated that the vessel will become more self-sufficient.

Although NAS may be less affected than INS and TOS by the positioning of VTS within VTM, the provision of NAS from the shore may increase due to an increasing demand by authorities to ensure safe and efficient use of the navigable waters and protection of the environment .

It is expected that the provision of NAS will benefit from the positioning of VTS within VTM.

8.1.2.3 	Actions requested

- The inconsistencies in the definitions from the VTS manual 2008 should be reviewed

8.1.3 	Traffic Organisation Service (TOS)

Traffic Organisation Service (TOS) : A Traffic Organisation Service is a service to provide for the safe and efficient movement of traffic and to identify and manage potentially dangerous traffic situations. A Traffic Organisation Service provides essential and timely information to assist the on-board decision-making process and may advise, instruct or exercise the authority to direct movements.

8.1.3.1 Function positioned in the strategic, tactical and/or operation domain.

8.1.3.2 	Benefits for TOS due to the positioning of VTS in the VTM framework

More emphasis on the traffic organisation service is foreseen, in particular on:

- advanced traffic planning and traffic monitoring, and

- co-operation with and coordination between allied services, port operations, emergency services and adjacent VTS.

It is expected that the provision of TOS will benefit from the positioning of VTS within VTM.

8.1.3.3 	Actions requested

- The identified  inconsistencies in definitions from the VTS manual 2008 should be reviewed

8.1.4	Recommendation:

Considering the conclusions made by VTS24 and VTS25 that VTS should have a major and central role within a global VTM framework.  It may also be concluded that:

- the quality of its services and management will become more visible to other stakeholders, and

- its information position may become more prominent.

Consequently and in addition to IMO Assembly Resolution A.857(20), IMO MSC Circ.1065, and IALA recommendation V-103, a Quality Management System (QMS) could be considered and new competencies probably need to be addressed.

1 VTS29-output-6 Liaison note to IALA Council VTM Concept and Scope, approved by Council at its 45th Session, May 2009

VTMIS has been used to describe the promotion and enabling of information exchange with allied services, and between VTS centres.  The scope of VTMIS however is too limited to be used for the entire information management with respect to the VTM framework.

e-Navigation however is supportive to the VTM framework and will provide capabilities to support the provision of measures and services within the VTM framework.

To be revised into “maritime pilotage on board” in case of inconsistency with current legislation.

In those areas where no VTS is established, but services within VTM are provided, the organizations providing those services should also define their relationships with other stakeholders and identify their information position as described

VTS manual 2008

VTS Manual 2008

VTS Manual 2008

VTS Manual 2008

This outlook may be influenced by the confidence that the ship will have in the reliability and integrated presentation of the information, in the quality of the systems and the ability to use this information for effective decision making.

VTS Manual 2008