# Annex

# Draft guide on the provision of VTS in international waters –

# as seen by Harmonisation working group in the Baltic region and Norway (HWG)

# Overview

The purpose of Vessel Traffic Services (VTS) in international waters is to improve the safety and efficiency of navigation, safety of life at sea and the protection of the marine environment and/or the adjacent shore area, worksites and offshore installations from possible adverse effects of maritime traffic.

The principles of vessel traffic services in international waters are governed by a hierarchy of regulatory requirements and guidelines.  Key requirements and guidelines include:

- UNCLOS

- SOLAS Regulation V/12-1 “Vessel traffic services”

- IMO Resolution A.857(20) Guidelines for Vessel Traffic Services

- IMO Resolution A.851(20) General Principles for Ship Reporting Systems and Ship Reporting Requirements

- Resolution MSC.43(64) Guidelines and Criteria for Ship Reporting Systems

- IMO Resolution A918(22) IMO Standard Marine Communication Phrases

- IALA Vessel Traffic Services Manual (2008)

Chapter V “Safety of Navigation” of the SOLAS 1974 Convention, Regulation V-12 “Vessel Traffic Services”, states among others that:

- “Vessel traffic services contribute to safety of life at sea, safety and efficiency of navigation and protection of the marine environment, adjacent shore areas, work sites and offshore installations from possible adverse effects of maritime traffic.”

- “Contracting Governments planning and implementing VTS shall, wherever possible, follow the guidelines developed by the Organization.”

Implicit in the statement that contracting governments, wherever possible, shall follow the VTS guidelines developed by the Organisation, is the need to ensure consistency in the approach taken to the delivery of vessel traffic service and to avoid confusion for the mariner trading between various jurisdictions about the delivery of VTS services.

IMO Resolution A.857(20) Guidelines for Vessel Traffic Services define a Vessel Traffic Service (VTS) as a:

Service implemented by a Competent Authority, designed to improve the safety and efficiency of vessel traffic and to protect the environment. The service should have the capability to interact with the traffic and to respond to traffic situations developing in the VTS area.

## Scope

The aim of this document is to provide guidance on the delivery of VTS in international waters to ensure that interaction with participating vessels to assist onboard decision making is consistent between VTS centres, is timely and is clearly understood by the participating vessels.

# References

| IMO | SOLAS Regulation V/12 “Vessel Traffic Services” |
| --- | --- |
|  | Resolution A.857(20) Guidelines For Vessel Traffic Services |
|  | Resolution A.851(20) General Principles for Ship Reporting Systems and Ship Reporting Requirements |
|  | Resolution A.918(22) IMO Standard Marine Communication Phrases |
| IALA | Recommendation V-127 On Operational Procedures for Vessel Traffic Services |
|  | VTS Manual (2008) |
|  | Recommendation V-128 – Operational and Technical Performance Requirements for VTS Equipment Edition 3.0 |
|  | IALA Recommendation V-103 - VTS Operator training |

# Abbreviations

| IALA | International Association for Marine Aids to Navigation and Lighthouse Authorities |
| --- | --- |
| IMO | International Maritime Organization |
| MSC | Maritime Safety Committee (Standing Committee of IMO) |
| SOLAS | Convention on the Safety of Life at Sea |
| SMCP | Standard Marine Communication Phrases |
| VTS | Vessel Traffic Services |
| VTSO | Vessel Traffic Services Operator |
| OOW | Officer of the Watch |
| COG | Course over Ground |
| SOG | Speed over Ground |

# Definitions

The following terms are used in conjunction with these guidelines:

| Message Markers |  |  |
| --- | --- | --- |
| Information |  | This indicates that the interaction from a VTS Centre “is restricted to observed facts, situations” Note: This marker is preferably used for navigational and traffic information, etc. Consequences of ignoring INFORMATION will be up to the recipient 2 |
| Warning |  | This indicates that the interaction from a VTS Centre “implies the intention of the sender to inform others about danger.” Note: This means that any recipient of a WARNING should pay immediate attention to the danger mentioned. Consequences of ignoring a WARNING will be up to the recipient 2 |
| Question |  | This indicates that the following message is of an interrogative character. Note: The use of this marker removes any doubt as to whether a question is being asked or a statement is being made, especially when interrogatives such as what, where, why, who, how are additionally used at the beginning of the question. The recipient is expected to return an answer 2 |
| Answer |  | This indicates that the following message is the reply to a previous question. Note: Note that an answer should not contain another question. |
| Request |  | This indicates that the following message is asking for action from others with respect to the vessel. Note: The use of this marker is to signal: I want something to be arranged or provided,e.g. ship´s stores requirements, tugs, permission, etc. 2 |
| Intention |  | This indicates that the following message informs others about immediate navigational action intended to be taken. Note: The use of this message marker is logically restricted to messages announcing navigational actions by the vessel sending this message 2 |
| General Definitions |  |  |
| Competent Authority |  | The authority made responsible, in whole or in part, by the Government for safety, including environmental safety, and efficiency of vessel traffic and the protection of the environment. 1 |
| Information Service |  | A service to ensure that essential information becomes available in time for on-board navigational decision-making. |
| Participating Vessel |  | Vessels navigating in an area where vessel traffic services are provided should make use of these services. Depending upon governing rules and regulations, participation in a VTS may be either voluntary or mandatory. Vessels should be allowed to use a VTS where mandatory participation is not required. 1 |
| Vessel Traffic Services (VTS)in international waters |  | A service implemented by a Contracting Government or Governments or the Competent Authority, designed to improve the safety and efficiency of vessel traffic and to protect the environment. The service should have the capability to interact with the traffic and to respond to traffic situations developing in the VTS area outside territorial waters. |
| VTS Authority |  | The authority with responsibility for the management, operation and coordination of the VTS, interaction with participating vessels and the safe and effective provision of the service. 1 |
| VTS Operator (VTSO) |  | An appropriately qualified person performing one or more tasks contributing to the services of the VTS. 1 |
| VTS Traffic Image |  | The surface picture of vessels and their movements in a VTS area. 1 |
| VTS Centre |  | The centre from which the VTS is operated. 1 |
| 1 | IMO Resolution A.857(20) Guidelines For Vessel Traffic Services |  |
| 2 | IMO Resolution A.918(22) IMO Standard Marine Communication Phrases |  |

# Provision of VTS in international waters

VTS in international waters is a service that provides essential and timely information to assist in the onboard navigational decision-making process.

VTS in international waters is limited to providing an Information Service (INS), and – as stated in the VTS Manual (2008) – “An Information Service does not participate in on-board decision-making”.

Nevertheless, it should be noted that the VTS Manual (2008) also states that this service type involves maintaining a traffic image and allows interaction with traffic and response to developing traffic situations. An Information Service provides essential and timely marine information to assist the on-board decision-making process, which may include:

- The position, identity, intention and destination of vessels;

- Amendments and changes in promulgated information concerning the VTS area such as boundaries, procedures, radio frequencies, reporting points;

- The mandatory reporting of movements; and

- Meteorological and hydrological conditions, notices to mariners, status of aids to navigation; limited manoeuvrability that may impose restrictions on the navigation of other vessels, or any other potential hindrances.

In addition to what is stated in the VTS Manual an Information Service may also:

- Receiving Sailing Plans (SP), including in advance forwarded SP, and distribute the data and information accordingly, and by doing so assist vessels in their reporting obligation(s).

## Responding to traffic situations developing in the VTS area in international waters

- IMO Resolution A.857(20) Guidelines for Vessel Traffic Services  states that:

“A VTS should at all times be capable of generating a comprehensive overview of the traffic in its service area combined with all traffic influencing factors.”

The VTS should be able to compile a traffic image, which is the basis for its capability to respond to traffic situations developing in its service area. The VTS traffic image allows the VTS operator to evaluate situations and make decisions accordingly.

To respond to traffic situations developing in the VTS area the acquired data should be processed and evaluated by trained and qualified VTS personnel. Conclusions from the evaluation should subsequently be communicated to participating vessels in a timely manner.

Examples of developing traffic situations where the VTS Centre may deem it necessary to provide timely information to (a) specific vessel(s) in order to ensure that essential information becomes available for on-board navigational decision-making include:

- Risk of grounding

- Vessel deviating from the recommended track or sailing plan

- Vessel unsure of its position or unable to determine its position

- Vessel unsure of the route to its destination

- Assistance to a vessel to an anchoring position

- Vessel navigational or manoeuvring equipment casualty

- Inclement conditions (e.g., low visibility, high winds)

- Potential collision between vessels

- Potential collision with a fixed object or hazard

- Assistance to a vessel to support the unexpected incapacity of a key member of the bridge team

## Interaction between a VTS and a vessel

### Master / Pilot / VTS Relationship

When providing Information Service the VTS should take care that VTS operations do not encroach upon the master's responsibility for safe navigation, or disturb the traditional relationship between master and pilot (IMO Resolution A.857(20) Guidelines For Vessel Traffic Services).

Further, masters of vessels and pilots should be aware of their responsibilities in receiving communications using IMO Resolution A.918(22) IMO Standard Marine Communication Phrases (SMCP) message markers.

Decisions concerning the actual navigation and the manoeuvring of the vessel remain with the master. Neither a sailing plan, nor requested or agreed changes to the sailing plan can supersede the decisions of the master concerning the actual navigation and manoeuvring of the vessel.

#### Master requesting information

The master of a participating vessel may at any time request additional information from the VTS should he experience a developing situation, which may endanger his vessel. Examples of such developing situations are shown in 5.1 above.

When receiving such additional information from the VTS the master should be made aware of the limitations on VTS equipment and the specific responsibilities of the Master for collision avoidance.

#### Observed / Deemed Necessary

The VTS may when it is deemed necessary provide timely and relevant information to (a) specific vessel(s) in order to respond to a developing situation, thereby ensuring that essential information becomes available in time for on-board navigational decision-making.

### Delivery of Information Service

When providing Information Service it is important that the interaction from a VTS centre to assist the onboard decision making is conducted in a timely manner, is unambiguous and clearly understood by both parties and is not open to misinterpretation. In providing guidance for the delivery of VTS services IMO Resolution A.857(20) Guidelines for Vessel Traffic Services recommends that in the provision of information and warning:

- IMO Standard Marine Communication Phrases should be used where practicable (IMO Resolution A.918(22) IMO Standard Marine Communication Phrases).

- Messages addressed to a specific vessel must always contain the name or call sign of the recipient so that there is no doubt to whom the content of the message is directed.

IMO Resolution A.857(20) Guidelines For Vessel Traffic Services states that in the interaction between a participating vessel and a VTS centre:

- “In any VTS message directed to a vessel or vessels it should be made clear whether the message contains information, advice, warning…”, and

- “IMO Standard Communication Phrases should be used where practicable.”

#### Communications Using Message Markers

There are six types of Communication Message Markers that may be used in VTS in international waters. Message Markers are used to emphasise the content of the message or to ensure that the message is properly understood, particularly when language difficulties are apparent between the VTS and the vessel.  These are:

- INFORMATION

- WARNING

- QUESTION

- ANSWER

- REQUEST

- INTENTION

It is at the discretion of the VTS or the bridge team whether to use one of the message markers and if so which of them to apply depending on the user’s qualified assessment of the situation. The application of message markers is described in the IMO Resolution A.918(22).

Examples of the use of message markers QUESTION, ANSWER, REQUEST and INTENTION are provided in the SMCP. The use of the message markers INFORMATION and WARNING – and combinations thereof – are particularly relevant for VTS in international waters.

#### INFORMATION

IMO Resolution 857(20) states that any VTS “…should have the capability to interact with the traffic and to respond to traffic situations developing in the VTS area.” In order to perform this interaction with participating vessels the VTS in international waters should make full use of the possibilities contained within Information Service and the message marker INFORMATION.

Examples of the provision of an INFORMATION Message during the delivery of Information Service to a vessel include:

- Course and speed over the ground by a vessel;

- Position relative to fairway axis, navigational features and/or way-points;

- Proximity to navigational hazards; and

- Positions, identities, intentions and any restrictions of surrounding traffic.

For example:

| WARNING Example 1 | “INFORMATION, According to my equipment, vessel “No Name” will overtake you on your starboard side in the vicinity of Buoy “….”. |
| --- | --- |
| Example 2 | “INFORMATION, Next high water at Port “YY” predicted to be “….” at a height of “ZZ” metres. |
| Example 3 | “INFORMATION, A naval exercise is taking place in the vicinity of lighthouse “Rocky-Shore” |
| Example 4 | “INFORMATION, According to my equipment, your course over ground is not in aligned with the fairway axis |

The provision of information during the delivery an Information Service may also include Warnings.  SMCP defines WARNING as a communication whereby the message implies the intention of the sender to inform others about danger.  It may be used to convey potentially dangerous situations or observed developing situations.

As such, it is a relay of information extracted from the VTS sensors and the traffic image and, in the professional opinion of the VTSO, the message should be communicated to inform a vessel about potential danger.

The contents of a Warning Message should be immediately assessed onboard the vessel in conjunction with any additional information which may not be available to the VTS centre. Corrective action should be taken where necessary.  Subject to the response of the vessel, a Warning Message may be followed by further messages using INFORMATION.

Examples of the provision of a Warning Message during the delivery of an Information Service to a participating vessel include:

| Example 1 | "WARNING. Obstruction in the fairway. Submerged container …. degrees, distance …. meters from …. Buoy.” |
| --- | --- |
| Example 2 | “WARNING. According to my equipment you are running into shallow water.” |
| Example 3 | “WARNING. According to my equipment, you will pass close to the outgoing vessel bearing …… degrees distance … nautical miles” |

#### Combination of INFORMATION and WARNING

| Example 1 | "WARNING. Obstruction in the fairway. Submerged container …. degrees, distance …. meters from …. Buoy.” “INFORMATION Least depth above the container is … meters” |
| --- | --- |
| Example 2 | “WARNING. According to my equipment you are running into shallow water.” “INFORMATION. Deep Water Route is in bearing …degrees distance … nautical miles from you” |
| Example 3 | “WARNING. According to my equipment, you will pass close to the outgoing vessel bearing …… degrees distance … nautical miles “INFORMATION. Outgoing vessel is deep draught tanker “Dirty Oil” bound for Panama” |
| Example 4 | "WARNING. According to my equipment, you are diverging from the recommended track.” “INFORMATION The recommended track is … degrees” |
| Example 5 | “WARNING. Visibility in the approach channel is less than …... nautical miles” “INFORMATION. Emergency anchorage is in bearing … degrees distance … nautical miles from you |
| Example 6 | “WARNING. You will meet crossing traffic in position ….” “INFORMATION. Crossing traffic is vessel “Blue Moon”. “Blue Moon” has reported inoperative AIS signal” |

## Other Considerations

When providing Information Service as part of VTS in international waters there is a need to give emphasis to equipment capabilities, staffing, training and the legal basis.

### Equipment Capabilities

Consideration should be given to the quality of the traffic image available, the communications capability and the equipment availability. For further information refer to IALA Recommendation V-128 – Operational and Technical Performance Requirements for VTS Equipment .

### Staffing and Training

It is important that VTS personnel should be trained and practiced in the delivery of Information Service as part of VTS in international waters. For further information see IALA Recommendation V-103 on Standards for Training and Certification of VTS Personnel.

### Legal

It is important that consideration is given to the national and international legal basis for the provision of Information Service as part of VTS in international waters.

## Operational Procedures

All details for the provision of an Information Service as part of VTS in international waters, including the terminology used, should be contained in the Standard Operating Procedures (SOP) of the VTS Centre.

Further information and guidance on preparing operational procedures is provided in the IALA Recommendation V-127 On Operational Procedures for Vessel Traffic Services.

## Promulgation of Information and Service

The service offered to the mariner by a VTS in international waters should be promulgated to vessels in the appropriate internationally recognised marine publications. This should include details of the VTS, its capabilities, service provided, rules, regulations, requirements and procedures. The information promulgated should be verified and up-dated at least at annual intervals.

Several of these Resolutions (and other regulatory documents) may need to be amended in order to reflect the possibility of providing VTS outside territorial waters.