Future use of the VHF Radio Spectrum in the Port Environment

Executive Summary

Background

The demands on port communications have highlighted a shortage of VHF channels in the marine band available to fulfil the ever increasing requirements of the ports community.  Interference between VTS Centres is becoming increasingly evident as they compete for allocations from the limited number of available voice radio communications channels and the burgeoning requirement for mechanisms to facilitate data transfer, including real time kinematics, provides a catalyst for an examination of the allocation of the VHF radio spectrum in a more radical way.

Aim

The aim of this paper is to influence the allocation of the marine VHF spectrum to provide greater efficiencies in the allocation of frequencies resulting in increased safety due to a reduction in interference and increased clarity of communication.

The current situation

Current provision in the International Maritime Mobile (IMM) VHF band is for a number of simplex and duplex channels within a range from 156 to 162 MHz with a frequency spacing of 25kHz.  There is some provision for a reduction in spacing to 12.5kHz where national variation is permitted and it is understood that proposals are being considered to expand the use of 12.5kHz frequency spacing.  Such provision is likely to be too little, too late and a more radical approach is appropriate.

The International Civil Aviation Organization (ICAO) has mandated the reduction of VHF communication channel spacing in Europe from 25 kHz to 8.33 kHz for all aircraft operating above 20,000 feet.

Technological Advances

Among the emerging enabling technologies an example isTerrestrial Trunked Radio (TETRA).  A digital trunked mobile radio standard developed by the European Telecommunications Standards Institute. The purpose of the TETRA standard was to meet the needs of traditional Professional Mobile Radio (PMR) user organisations.  The technology involved has the potential to provide the basis for a maritime communication system in the International Maritime Mobile frequency band.  The allocation of user groups would facilitate both specific voice communication and also data transmission.  The latter would be useful in the port environment as a method for broadcasting tidal and navigational information.   There are, however, other systems under development which have  the potential to satisfy this particular requirement.

There is evidence from industry that, even with today’s technology, 6.25kHz frequency spacing is possible and that this could form the basis of a realistic target for future development.  It may, however, be appropriate to be less prescriptive when considering data transfer requirements which would, in turn, lead to the concept of a more fundamental review of allocations and a more flexible approach to frequency allocations.

There is also emerging pressure from governments to charge for the use of spectrum.  Application of technology would increase the utilisation of available bandwidth increasing availability of dedicated channels for port and maritime operations and may even result in efficiencies that could lead to the release of spectrum in the longer term.

Constraints

The frequency allocations in the International Maritime Mobile VHF band are well established by international agreement.   Changes to this would require a phased implementation plan with a substantial lead-in time to allow for changes to mandatory carriage requirements for appropriate radio equipment.

Conclusions

In order to provide a resilient and flexible maritime communication system with a sufficient number of channels for future use in ports and approaches, a radical review is necessary.  Systems are now available to make more efficient use of available spectrum and it is proposed that TETRA technology should form the basis for development and integration into the eNavigation concept.  The long lead time to implement such a change has acted as a deterrent to the development of an effective strategic plan.  eNavigation, however, provides the ideal incentive for action now to satisfy the long term demands of the industry and to improve safety.

Roy Stanbrook

IHMA

23rd July 2009