Maritime Radio Communications Manual (MARCOM Manual) December -----

The International Association of Marine Aids to Navigation and Lighthouse Authorities (IALA) exists to:

harmonise standards for aids to navigation systems worldwide;

facilitate the safe and efficient movement of shipping, and;

enhance the protection of the maritime environment.

This Maritime Radio Communications Manual (MARCOM Manual) has been developed by IALA to assist in the selection of radio communication systems required to support e-Navigation and GMDSS.  The current status of radio communication in the maritime mobile bands is discussed and forms the basis of projections for future developments needed to support e-navigation, GMDSS and the development of Maritime Autonomous Systems (Surface).

The MARCOM Manual provides guidance to IALA members regarding potential future developments, which will enable members to identify areas requiring resource allocation and research activity.

DOCUMENT HISTORY

| Date | Details | Approval |
| --- | --- | --- |
| December 2009 | 1st issue | Council 46 |
| December 2012 | Entire document General revision to update terminology Sections 3.4.3 (VHF) and 4 (securing spectrum) Changes to status of e-navigation and maturing communications concepts Outcomes of WRC-12 / preparation for WRC-15 | Council 54 |
| December 2017 | Entire document General revision to reflect outcomes of WRC-15 / preparation for WRC-19 | Council 65 |
| December 2022 | Entire document to reflect IALA documentation structure (IALA Manual) Revisions to reflect outcomes of WRC-19 / preparation for WRC-23 | Council ?? |

JCJ Thoughts

Add in references (with hot links) to relevant IALA documents

Current VTS Manual uses a blue box format (that can be for IALA to confirm)

Think about what makes this useful to the IALA member…

Need to include IMT references, maybe something on 3GPP and the maritime vertical?

CONTENTS

1.1.1	Aim and Vision of IALA	6

1.2	Scope and Objectives	6

2.1	General	6

2.2	Regulatory	7

2.3	Commercial	7

2.4	Operational	7

2.5	Technical	7

3.1	Maritime Communication Requirements	8

3.2	General Overview of Existing Communication System Technologies	8

3.2.1	Low Frequency Band (LF)	8

3.2.2	Medium Frequency / High Frequency Band (MF/HF)	8

3.2.2.1	Digital Selective Calling (DSC)	9

3.2.2.2	Voice Communication	9

3.2.2.3	Data Communication	9

3.2.2.4	Narrowband Direct Printing (NBDP)	9

3.2.2.5	Navigational Telex (NAVTEX)	9

3.2.2.6	Differential Global Navigational Satellite System (DGNSS)	10

3.2.3	Very High Frequency Band (VHF)	10

3.2.3.1	Digital Selective Calling (DSC)	10

3.2.3.2	Voice Communication	10

3.2.3.3	121.5 MHz Locating Beacon	10

3.2.3.4	Automatic Identification System (AIS)	10

3.2.3.5	Man Overboard Device (MOB)	12

3.2.3.6	Autonomous Maritime Radio Devices (AMRD)	12

3.2.3.7	Two-way VHF Radiotelephone Apparatus	12

3.2.3.8	Regional Data Communication Systems	12

3.2.3.9	Satellite Data Communication	12

3.2.4	Ultra-High Frequency Band / Super High Frequency Band (UHF/SHF)	12

3.2.4.1	Emergency Position Indicating Radio Beacon (EPIRB)	12

3.2.4.2	On-board Communication	13

3.2.4.3	Satellite Voice and Data Communication	13

3.2.4.4	Enhanced Group Call (EGC)	13

3.2.4.5	Long Range Identification and Tracking (LRIT)	13

3.2.4.6	Global Navigation Satellite System (GNSS)	13

3.2.4.7	X-Band and S-Band Radar, Radar Beacon (RACON) and Radar SART	13

3.2.4.8	International Mobile Telecommunications-2020	14

3.2.4.9	Other Communication Technologies Using the UHF/SHF Band	14

3.3	Future Development of Maritime Radio Communications	14

3.3.1	Low Frequency Band (LF)	14

3.3.1.1	Enhanced Long Range Navigation (eLORAN)	14

3.3.2	Medium Frequency / High Frequency Band (MF/HF)	14

3.3.2.1	Digital Data Communication Using MF/HF Band	14

3.3.2.2	NAVDAT (Navigational Data)	15

3.3.3	Very High Frequency Band (VHF)	15

3.3.3.1	Automatic Identification System (AIS)	15

3.3.3.2	Digital Data Communication	16

3.3.3.3	Digital communication	17

3.3.3.4	Digital Voice Communication	18

3.3.3.5	R-Mode	18

3.3.4	Ultra High Frequency Band / Super High Frequency Band (UHF/SHF)	18

3.3.4.1	Satellite Communication Using the UHF/SHF Band	18

3.3.4.2	Public Mobile Wireless Communications	18

3.3.4.3	On-board Communications	18

3.4	Overview of the Current and Future Voice and Data Communication Technologies	18

3.5	Vision: Automated Selection Process of Available Communication Technologies	20

3.6	Modernisation of GMDSS	20

3.6.1	Current GMDSS System Components	21

3.6.2	Possible System Components for the Modernisation of GMDSS	21

4.1	Situation with Respect to Existing Spectrum Usage	22

4.2	WRC-19 Agenda Items [full section – update to WRC-23 agenda items ]	22

4.2.1	Agenda Item 1.11	23

4.3	Preliminary Agenda for WRC-23	23

E1.1	VHF data exchange system: data exchange between terrestrial stations	63

E1.2	VHF data exchange system: data exchange between satellites and terrestrial stations	63

E4.1	Number of ships	64

E4.2	AIS Base Stations	64

E4.3	Airborne AIS	64

E4.4	AIS AtoN, Real or Virtual, and AtoN Monitoring	64

E4.5	AIS-SART	65

E4.6	SAR Related Equipment Using AIS-SART Technology	65

E4.7	Application-Specific Messages	65

E4.7.1	International	65

E4.7.2	Regional	65

E4.7.3	Mandatory Application Specific Messaging in Certain Regions	65

E4.8	AIS Messages (23, 24, 25, 26 and 27)	65

E4.9	GNSS Differential Corrections	65

E4.10	FATDMA Usage	65

E5.1	Increased Use of AIS Shore to Ship Data Transmissions	66

E5.2	Increased Use of Ship-to-Shore Data Transfer	66

E5.3	Increased Use of Ship-to-Ship Data Transmissions	66

E5.4	Base Stations	66

E5.5	Repeater Stations	67

E5.6	Future Messaging	67

E5.7	Satellite Detection of AIS	67

E5.8	Future GMDSS Platform	67

E5.9	PLB (Personal Locating Beacon)	67

E5.10	Use of AIS for PNT (Position, Navigation and Timing) and Ranging	67

E5.11	Coordinated Channel Management for Future e-Navigation VHF Digital Data Communications	68

E6.1	General Thoughts	68

E6.2	 “VHF Data Exchange” (VDE) Plan	68

E7.1	Background	68

E7.2	VHF Data Communications	68

E7.3	AIS and VDE Channel Plan	69

E7.4	VDE, ASM and their Relationship with AIS	71

E7.5	Legacy Strategy	72

E7.6	Benefits of Future AIS / VDE for the Users	72

LIST OF TABLES AND FIGURES

Table 1 - Geographical Classification of e-Navigation Data and Voice Communication Techniques Ship-Shore	20

Table 2 - Geographical Classification of e-Navigation Data and Voice Communication Techniques Ship-Ship	21

Table 3 - Comparison of Communications Technologies	22

Table 4 - Current GMDSS System Components	24

Table 5 - Potential System Components for Modernisation of GMDSS	25

Table 6 - System Description Table	31

Table 7 - Maritime Frequency Allocations	39

Table 8 - Summary of Spectrum Requirements of New Commercial Data Links	46

Table 9 - Summary of Maritime Radio Spectrum	48

Table 10 - ITU Recommendations and Reports for Maritime Radio Applications	55

Table 11 - VHF Data Exchange System (VDES)	73

Figure 1- VDES functions and frequency use – full system	65

Figure 2 – VHF Data Communications Channel	71

Figure 3 - Appendix 18 to the ITU Radio Regulations	72

# Introduction

The International Association of Marine Aids to Navigation and Lighthouse Authorities (IALA) was formed in 1957 as a non-government, non-profit making, technical association that provides a framework for aids to navigation authorities, manufacturers, and consultants from all parts of the world to work with a common effort. The focus of IALA is on the harmonisation of standards, the safe and efficient movement of shipping and enhancing the protection of the maritime natural environment.

.

### Aim and Vision of IALA

The aim of IALA is:

Fostering the safe, economic, and efficient movement of vessels by improvements and harmonisation of aids to navigation worldwide.

while the Vision of IALA is:

A holistic approach (e-Navigation) to Safe marine navigation in a world of larger and faster ships, changing waterway use, changing economy, and advancing technology while applying stringent standards.

With this in mind, IALA is maintaining focus work on maritime radiocommunications to support both the vision of IALA and e-Navigation (https://www.iala-aism.org/about-iala/strategy-definition/).

## Scope and Objectives

The core objective of the Maritime Radio Communications plan (MARCOM) is to state the IALA vision for the efficient use of Radio Spectrum in the Maritime Mobile Service, the need for an agreed infrastructure of communications between ships and ashore considering current, developing, and future Radio Communication Systems for the maritime sector.

The MARCOM Manual is designed to support radio communication within an increasingly digital maritime environment, including e-navigation and Maritime Autonomous Surface Ships (MASS).  It should assist in the formulation of policy for National and International spectrum allocation and usage, and provide input to the ongoing work of the International Telecommunications Union (ITU).

# Background

The digitalisation of the maritime environment, including e-Navigation and MASS, depends upon efficient, reliable and robust ship-ship, ship-shore or shore-ship electronic data transfer. Noting the different requirements for communications ship to ship; ship to/from shore and shore to shore, it is important to  facilitate and monitor the development of digital communications solutions.  Studies provide a basis for analysis of candidate technologies and the opportunity to provide strategic guidance on implementation of various applicable technologies.

## General

The challenge facing radio communication in the maritime sector is the need for greater data bandwidth and lower latency as the communications services migrate from analogues to digital systems.

Spectrum, as a finite resource, is under increasing demand from all users globally and national licensing / regulatory authorities have been to use pricing / cost to drive efficiency improvements.

Co-operation and coordination between International, Intergovernmental, and other organizations including IMO, ITU, ICAO, WMO, IHO, IALA, IEC, IMEA, CIRM, and RTCM is necessary to ensure an effective approach to maritime communications to support e-Navgation and other developments.

Co-ordination with ITU is required to secure access to radio spectrum to support e-Navigation communications technology.

IALA national administrations are encouraged to ask their national representatives at ITU to support the MARCOM plan.

## Regulatory

Radio spectrum is regulated by the ITU, which includes not only the frequencies but the technologies and standards for the systems employed.

The carriage requirement for radio communications equipment on SOLAS convention vessels is identified by the IMO including AIS, DSC, voice communications and radar equipment and communications equipment to support GMDSS and LRIT.

IALA has highlighted the requirement to provide an optimum mix of Aids to Navigation for the users, which includes SOLAS convention vessels; domestic commercial vessels; and recreational craft.

This MARCOM plan reflects the requirements for effective and secure communications identified by IMO, in addressing the needs of the maritime industry, including domestic commercial vessels and recreational craft (MSC98).

## Commercial

Growth of the world fleet requires that both commerce and public correspondence communications systems to cope with the traffic with the load on AIS exceeding the optimal 50% load in some areas.

Analogue voice systems that occupy a channel for the duration of the call can be viewed as an inefficient means of relaying information.  Developments in digital data transfer, including digital voice, can enhance efficiency of spectrum use and provide additional capability to meet increased demands for communications and data transfer. Increased digital data exchange can cope with multiple simultaneous users and is more resistant to overloading due to growth in the number of participants.

The option of more spectrum in the maritime mobile service is not readily available, so alternatives must be found to support future growth.  Existing spectrum allocated to maritime use will need to be fully utilized.

## Operational

Operational aspects include the development and implementation of e-Navigation, the widespread reliance on GNSS and its role underpinning position, navigation and timing and the growing deployment of local and specific traffic monitoring schemes and Vessel Traffic Services. This is leading to a change in the balance between traditional navigation skills, the importance of understanding digital data and maritime informatics.   Challenges are raised as the role of the human element within technological advances such as ECDIS and IBS are recognised and addressed.  Training and competence with the changing technologies are included within the operational aspect.  .

Before adopting any technology, human factors must be addressed (safety, liability, on-board training and duty of care). e-Navigation implementation requires the active participation of international bodies such as IMO, ITU, IHO, IALA,, PIANC, IEC, WMO, RTCM, ETSI and CIRM.

Reference - IMO MSC.1 Circ.1512 – Guideline on Software quality assurance and human centred design for e-navigation.

## Technical

Significant changes underpinning services and systems are expected over the next decade being precipitated by the rapid deployment of satellite systems, the rapid growth of high bandwidth, low latency radio data communication systems and the merging of data and voice services.

Technical developments in digital data exchange and communications includes the work of the International Telecommunications Union (ITU) on the International Mobile Telecommunications 2020 (IMT-2020) and development of the VHF Data Exchange System (VDES).

# Developments in Maritime Communications

The maritime domain uses radio communication to support safe navigation, efficient operations, and commercial aspects (commerce and public correspondence).  Because many of these technologies have been developed with a single application in mind, vessels needs to carry many different types of communications equipment, to be able to receive relevant data.

Developments in the maritime environment, including e-Navigation and MASS require an assessment of the communications needs as well as an understanding of the radio spectrum available to the community.

## Maritime Communication Requirements

The maritime domain uses communication links for essential safety of life applications (Distress, Urgency and Safety), routine operational activities and commercial applications (commerce and general correspondence).

Current and future needs require a design of the communications architecture to be flexible with the ability grow to encompass other applications and technologies as they are identified.

| Safety |  | Operational |  | Commercial |
| --- | --- | --- | --- | --- |
| - AIS position reports - AIS AtoN - Digital Selective Calling - Long Range Identification and Tracking - Differential GNSS - NAVTEX/SafetyNET - VTS coordination - SAR - Distress and Urgency alerting/calling |  | - Weather data - Ship reporting - Notifications to coastal States - Port arrival notification - Maritime Information Overlays - Port & VTS surveillance feeds - Electronic chart updates - Access to vessel and equipment manuals - Remote maintenance and service - Telemedicine |  | - Voyage orders - Commercial port services - Operational reports - Cargo telemetry - Point of Sale - Crew personal communications - Passenger Internet access - Crew training - Infotainment |

Overview of use of maritime communication

Figure 1 shows some example uses of communications technologies to address the key requirements for safety (including distress, urgency and safety); operations and commerce or commercial use.   .

## General Overview of Existing Communication System Technologies

This section details the current technologies (as of 2022) used for maritime communications by frequency band. Please refer to the table in Annex A for system details, Annex B for maritime spectrum allocations and Annex D for specific ITU technical characteristics associated with the systems described in section 3.3 and 3.4.

### Low Frequency Band (LF)

There is some use of the LF radio spectrum by the maritime community.  While Loran C is now no longer in use, other systems such as eLoran and research projects are underway to evaluate the use, if any, of this spectrum.

### Medium Frequency / High Frequency Band (MF/HF)

The MF/HF radio spectrum is used by the maritime community for communication of voice and data. MF/HF transmissions support both general, Maritime Safety Information (MSI) and distress related communications using DSC, NBDP, voice and data.  These communications take place across the maritime mobile service bands within 1.6-26.5 MHz as defined in Appendix 17 to the ITU Radio Regulations, whilst distress related communications are consigned to a small set of specific channels as indicated in Appendix 15 to the ITU Radio Regulations.  Channel bandwidths are typically 0.5 kHz (DSC and NBDP) and 3 kHz (voice and data).

#### Digital Selective Calling (DSC)

DSC is an element of the Global Maritime Distress and Safety System (GMDSS) and enables a radio station to establish contact with, and transfer information to, another station or group of stations, for distress or general communications over medium to long range distances using a unique MMSI (ITU-R M.585).  DSC is primarily used for distress alerting, urgency and safety calling within ship-to-ship, ship-to-shore, and shore-to-ship prior to initiating distress, urgency and safety communications using radiotelephone or MF/HF radiotelex.

DSC distress alerts are used to initiate emergency communications with ships and rescue co-ordination centres.  DSC is intended to eliminate the need for manual watch keeping on a ship's bridge or on shore to monitor continuously radio receivers on the distress and safety frequencies.

Six specific MF/HF frequencies are also set aside for Digital Selective Calling (DSC) distress and safety communications, one in each communication sub-band up to 16 MHz band.

#### Voice Communication

The MF/HF radio spectrum is used by the maritime community for communication of voice in ship-ship, shore-ship, and ship-shore modes of operation.  General voice communication takes place across the band 1.6-26.5 MHz Channel bandwidths are typically 3 kHz.

#### Data Communication

Current and emerging HF digital modulation schemes provide new opportunities utilizing data transmission in this frequency band (1.6-26.5 MHz).  The relevant technologies are described in the Recommendation ITU-R M.1798.  The Recommendation ITU-R M.1798-2, published on February 2021, includes four systems.

System 1 is an HF data services modem protocol using Orthogonal Frequency Division Multiplexing (OFDM) and uses 4/8-PSK modulation to 32 sub-carriers.

System 2 is an electronic mail system using the Pactor-III protocol with QPSK modulation to 18 sub-carriers.

Note - System 1 and System 2 use 3 kHz channels for the data rate of 3 kbps or below.

System 3 is a 10-20 kHz wideband HF data system for internet access and electronic mail services using OFDM.  This system uses QAM modulation to 228 sub-carriers at 10 kHz bandwidth or 460 sub-carriers at 20 kHz bandwidth for the data rate up to 51 kbps.  All three systems are IP level-compatible making interoperability possible.

System 4 is a wideband HF data system for Point-To-Point (PTP) communication for shore-to-ship, ship-to-shore, and ship-to-ship for exchanging digital data.  The system operates on the HF maritime bands of 4-27.5 MHz in a radio communication channel with a 10 kHz bandwidth, providing data rate up to 51 kbit/s.  The system establishes a communication link using FSK and then exchanges data using OFDM.

#### Narrowband Direct Printing (NBDP)

NBDP (also known as radio telex) uses HF channels of 0.5 kHz using FSK modulation and supports low speed data transmissions (100 bps) in the maritime mobile service bands within 1.6-26.5 MHz., NBDP is an element of GMDSS and can be used as the text-based distress follow-up communications and general communications between ship-to-ship, ship-to-shore and shore-to-ship.

IMO has indicated that NBDP could be removed as a required system under the GMDSS.

#### Navigational Telex (NAVTEX)

NAVTEX is an international, automated system for instantly distributing Maritime Safety Information (MSI) such as maritime navigational warnings, weather forecasts and warnings, search and rescue notices and similar information to ships.  A small, low-cost, and self-contained smart printing radio receiver (NAVTEX receiver) is an element of GMDSS and installed on the ship’s bridge.  Messages are broadcasted in English on 518 kHz, while 490 kHz and 4209.5 kHz are used to broadcast in English and/or local language.  The messages are coded with the broadcasting stations, type of messages, and the serial number of the message.  The time of broadcasting is internationally co-ordinated by areas (NAVAREA) to share the same frequency.

#### Differential Global Navigational Satellite System (DGNSS)

The IALA Differential GNSS coastal radio beacon network broadcasts corrections and integrity information to maritime users in the LF/MF bands (between 283.5 and 325 kHz).  Data rates can be 50, 100 or 200 bps.  This is a data broadcast system from shore-to-ship.

### Very High Frequency Band (VHF)

Voice communication uses the maritime VHF band (156.025-162.025 MHz) as the primary means of ship-shore, shore-ship, and ship-ship communication system.  It is used for distress, safety information and general communications.  Channel spacing is currently 25 kHz although the use of 12.5 kHz channels on an interleaved basis is allowed within Appendix 18 of the Radio Regulations as described in accordance with Recommendation ITU-R M.1084 to improve spectrum efficiency.

#### Digital Selective Calling (DSC)

DSC is a technique using digital codes which enables a radio station to establish contact with, and transfer information to, another station or group of stations, for distress or general communications using channel 70 (156.525 MHz).  DSC is an element of the GMDSS and is used for distress alerting; urgency and safety calling within ship-to-ship; ship-to-shore and shore-to-ship prior to initiating distress, urgency and safety communications.  Additionally, DSC may be used for AIS channel management in specific geographic areas.  DSC distress alerts, which consist of a preformatted distress message, are used to initiate emergency communications with ships and rescue coordination centres.

DSC can also be used to call individual stations, groups of stations, or "all stations" in radio range.  Each DSC-equipped ship, shore station and group is assigned a unique 9-digit MMSI (ITU-R M.585).

#### Voice Communication

Voice communication in VHF is currently analogue, and uses the maritime mobile band (156.025 to 162.025 MHz).  It is used for distress, safety and general communications.  Hand-held units are generally utilized for on-board communications.  Primary channels used for distress and safety communications by voice are Ch 6, Ch 13 and Ch 16.  The use of other channels is designated in Appendix 18 to the ITU Radio Regulations.

#### 121.5 MHz Locating Beacon

The frequency 121.5 MHz is an aeronautical emergency frequency.  121.5 MHz radio beacons were developed in the mid-seventies for installation on aircraft, as Emergency Locator Transmitters (ELTs).  However, they can also be used on board ship as part of Emergency Position-Indicating Radio Beacons (EPIRBs) or in Personal Locator Beacons (PLBs).

#### Automatic Identification System (AIS)

##### Primary Purpose of Automatic Identification System

AIS is a Self-Organised Time Division Multiple Access (SOTDMA) data exchange system used by ships and shore authorities to improve the safety of navigation by assisting in the efficient navigation of ships, protection of the environment, and operation of Vessel Traffic Services (VTS). AIS data includes identification, position, course, and speed.  AIS assists VTS and the vessel's watch keeping officers and allow maritime authorities to track and monitor vessel movements. The AIS technical specification is ITU-R M.1371-5.

AIS uses VHF Channels AIS 1 (161.975 MHz) and AIS 2 (162.025 MHz) or regional channels in defined geographical areas. Additionally, AIS has the capability for data exchange by application specific messages for navigation and safety related purposes.  The VHF Data link (VDL) loading should be considered when using application specific messages.

##### Automatic Identification System variations

There are various types of AIS and used for different purposes and include:

AIS base stations

AIS repeaters

AIS AtoN types 1, 2 and 3

Class A AIS used on SOLAS vessels

Class B Carrier Sense

Class B SOTDMA

AIS SART

AIS Application Specific Messages (AIS ASMs) are identified in IMO SN.1/Circ. 289, with examples of use of AIS ASM provided in IMO SN.1/Circ. 290.  Regionally, ASMs are managed by IALA, through the IALA e-Navigation Portal

Reference: IMO SN.1/Circular 289 and 290.

##### Automatic Identification System Search and Rescue Transmitter (AIS-SART)

AIS-SART is a locating device (alternative to radar SART).  As an element of GMDSS, AIS-SART is used to locate survival craft and distressed vessels.  The AIS-SART has no receiver and operates up to 96 hours on a primary battery.  The position and time synchronization of the AIS-SART is derived from a built-in GNSS receiver.

##### VHF Data Exchange System

ITU introduced a standard with options for 25 kHz, 50 kHz, and 100 kHz channels at an on air (not throughput) data rates up to 307.2 kbps to improve spectrum efficiency in 2012.  The VHF Data Exchange System (VDES) (ITU-R M.2090) resulted and is an extension of the AIS technology and has two primary components known as ASM and VDE components. The ASM uses Phase Shift Keyed (PSK) and the VDE a range of different modulation methods dependent on the Radio Frequency environment. VDES has both a bi-directional Terrestrial and a Satellite component. A VDES transceiver can communicate with AIS, ASM and VDE Terrestrial and Satellite systems.

Reference [to include – IALA and ITU references]

VDES consists of four components: AIS + ASM + VDE-TER + VDE-SAT

AIS uses channels AIS 1, AIS 2, 75 and 76, for terrestrial communications (AIS 1 and AIS 2) and satellite uplinks (AIS 1, AIS 2, 75 and 76)

ASM uses channels ASM 1 and ASM 2, for both terrestrial communications and satellite uplinks

VDE-TER uses channels 24, 84, 25, 85, 1024, 1084, 1025, 1085, 2024, 2084, 2025 and 2085 for terrestrial communications

VDE-SAT uses channels 1024, 1084, 1025, 1085,1026, 1086, 2024, 2084, 2025, 2085, 2026 and 2086, for both satellite uplink and downlink communications.

|  | VHF Data Exchange System (Recommendation ITU-R M.2092-0) |  | VHF Data Exchange System (Recommendation ITU-R M.1371-5) |  |
| --- | --- | --- | --- | --- |
| Sub-group | Application Specific Message (ASM) | VHF Data Exchange (VDE) | AIS for safety of navigation | AIS long range |
| Radio channels | - Channels ASM 1 and ASM 2 - World-wide dedicated channels (WRC-15 agreed, including Sat uplink) | - Channels 24,84, 25, 85, 26, 86 (WRC-15 agreed VDE-TER; target VDE-SAT WRC-19) | - AIS-1 & AIS-2 (simplex) | - Channels 75 and 76 (simplex) WRC-12 |
| Functionality | - Marine safety information - Marine security information - SSRMs - General purpose information communication | - General purpose data exchange - Robust high speed data exchange | - Safety of navigation - Maritime and inland distress and safety communications (Subject to inclusion in GMDSS Modernization by IMO) | - Space detection of AIS - Future SAR |
| Message types for AIS protocol | - IMO SN.1/ Circ.289 international application specific messages - Regional application specific messages - Base Station |  | - Vessel identification - Vessel dynamic data - Vessel static data - Voyage related data - Aids to Navigation - Base Station | - Space detection of AIS - Other messages for support of future SAR |
| Sub functionality | - Area warnings and advice - Meteorological and hydrological data - Traffic management - Ship-shore data exchange - Channel management | - High message payload | - Ship to ship collision avoidance - VTS - Tracking of ships - Locating in SAR - VDL control (by Base Station) | - Detection of vessels by coastal states beyond range of coastal AIS base stations - Future distress alerting (Subject to inclusion in GMDSS Modernization by IMO) |

#### Man Overboard Device (MOB)

There are various systems available that operate on either DSC based or AIS based technology as well as others that utilize Bluetooth or other technologies.  The MOB devices must be regarded as location aids in emergency situations and not as distress alert systems.

#### Autonomous Maritime Radio Devices (AMRD)

WRC-19 agreed introduction of autonomous maritime radio devices (AMRD) Group A, which enhance the safety of navigation, and Group B, which do not enhance the safety of navigation.

A modification was developed to a footnote in Appendix 18 to allow the use of AMRD Group A on channel 70 and AIS 1 and AIS 2 using DSC and/or AIS technology. A new footnote designates 160.9 MHz (channel 2006) to AMRD Group B using AIS technology.

#### Two-way VHF Radiotelephone Apparatus

Two-way VHF radiotelephone apparatus is defined in SOLAS Chapter III.  In practice, this apparatus is used as an element of GMDSS.  The apparatus is either a portable or fixed transceiver for use in survival craft.  It is used for on scene communications between survival craft, to other vessels and rescue units.

Communication is on Ch16 (156.8 MHz) and at least one other simplex channel.  The transceiver can be used for on board communications using a secondary battery; however, the primary battery must be used for GMDSS purposes.

#### Regional Data Communication Systems

In various regions, VHF data communication systems exist for data exchange between shore to ship and ship to shore.  Those systems are commercial and used primarily for vessel tracking, search areas in SAR operation, etc.

#### Satellite Data Communication

Satellite communications in the VHF band are commercially available.

Ship to satellite communications occurs in the maritime VHF band, providing tracking and data messages (AIS, ASM).

There is currently no satellite to ship communications in the maritime VHF band, however services are available for SMS, weather, and tracking, and these can be used for maritime purposes. VDES satellite component is in development (Refer to Annex E for further information on VDES).

### Ultra-High Frequency Band / Super High Frequency Band (UHF/SHF)

#### Emergency Position Indicating Radio Beacon (EPIRB)

Emergency radio beacons are alerting and tracking transmitters which aid in the detection and location of ships, aircraft, and people in distress.  They are radio beacons that interface with COSPAS/SARSAT, the international satellite system for search and rescue (SAR).  When activated, such beacons send out a distress alert signal that, when detected by non-geostationary satellites, can be located by the combination of Doppler shift measurement and triangulation.

An Emergency Position-Indicating Radio Beacon (EPIRB) is an element of GMDSS and used to send a distress alert signal to the COSPAS/SARSAT satellite system for the purposes of notifying Search and Rescue (SAR) organizations.  Information is retransmitted, at approximately 50 seconds intervals, to the satellites in the frequency band 406.0 to 406.1 MHz.  The EPIRB is also equipped with a 121.5 MHz beacon transmitter for homing by SAR aircraft, modulated with a swept audio tone and may also have an incorporated AIS-SART based transmitter (EPIRB AIS).

#### On-board Communication

UHF hand-held and fixed radios are commonly used on vessels for on-board communications including communications with workers on the dock or berth when alongside.  These radios are typically constrained to radiating less than 2 W in the band 450-470 MHz and are used for voice and data communication.

#### Satellite Voice and Data Communication

Satellite communications in the UHF band is commonly deployed on vessels to fulfil several distress, safety and general communications purposes.

Satellite communication links can support analogue and digital voice, broadband connectivity, e-mail, SMS, crew calling, telex, facsimile, remote monitoring, tracking (position reporting), chart and weather updates and Inmarsat FleetNET services.

Satellite systems are commercially provided services, which may have global or regional coverage.  These systems may be geostationary or non-geostationary. Non-geostationary systems are often in Low Earth Orbit (LEO) or Medium Earth Orbit (MEO).

Inmarsat and Iridium are elements of the GMDSS for distress alerting, urgency, and safety calling.

Other Geostationary systems include Thuraya.

Non-geostationary satellite systems include Globalstar, SpaceX, OneWeb and Orbcomm.

#### Enhanced Group Call (EGC)

Enhanced Group Calling (EGC), allows broadcast messages to be made to selected groups of ship stations located anywhere within satellite coverage.  Four geostationary Inmarsat satellites provide near worldwide coverage for SafetyNET except for the polar regions. SafetyNET and NAVTEX are recognized by the GMDSS as the primary means for disseminating maritime safety information.  Ships subject to the Safety of Life at Sea Convention (SOLAS) operating outside areas covered by NAVTEX must carry an Inmarsat-C SafetyNET receiver or equivalent Iridium transceiver.

#### Long Range Identification and Tracking (LRIT)

LRIT is an IMO-mandated scheme through which all passenger ships, cargo ships and mobile offshore drilling units engaged on international voyages must report their position on a regular basis (at least 4 times a day) to their flag administration.  In general satellite communications is used as the method for transmission of these reports, which are received and authenticated by an authorized service provider before being passed to the data centre.  The data from the vessel can be augmented with additional information by the shore-based authorities.  Other States may be entitled to request this information from the flag administration.

#### Global Navigation Satellite System (GNSS)

Global Navigation Satellite Systems (GNSS) are used for positioning, navigation and timing (PNT) and as an essential input into other ship systems.  Current signals are in the frequency-band around 1 GHz to 2.5 GHz.

#### X-Band and S-Band Radar, Radar Beacon (RACON) and Radar SART

Radar systems are commonly deployed and typically operate in two bands: X-band from 9.2 to 9.5 GHz and S-band from 2.9 to 3.1 GHz.  The radars are used for target detection and to support identification and for coastal and port navigation.  These bands are also used by radar transponders, namely the RACON and radar Search and Rescue Radar Transponder (SART) both create identifiable patterns when interrogated by vessel radars.  Racons are used to highlight the location of a visual Aid to Navigation (AtoN) or hazards on a ship’s radar screen and a radar SART is used to aid in locating a survival craft in a SAR operation.  RACONs operate in S- and X-band and SARTs are X-band only.

The radar SART is a locating device in the GMDSS and used to locate a survival craft or distressed vessel by creating a series of dots on a rescuing ship's radar display.  A radar-SART will only respond to a 9 GHz X-band (3 cm wavelength) radar.

The adoption of new technology radar within the maritime community introduces improvements in radar performance and the reduced peak transmission power associated with these radars can reduce the triggering range of RACONs and SARTs.

#### International Mobile Telecommunications-2020

The International Mobile Telecommunications-2020 enables both voice, data, and Internet of Things (IoT) communications. Although some systems have achieved up to 100Km range from ideally located shore side LTE base stations, most of IMT systems are limited in range and they would therefore be confined to supporting data transfer within the littoral state, port, or harbour environment.

IMT allows for Public and / or Private networks to be established. Where Private networks are established and are using 5G technologies, the data rates under ideal circumstances can exceed 1Gb/s.

The Private IMT2020 networks being established within various ports are being used for both the delivery of maritime focused services (i.e., connecting CCTV and high-speed data links to shore and providing data connections to ships in the coverage area) and the provision of services to commercial operators within the port environment.

Some IMT2020 network and service providers can offer GNSS independent positioning services within the port and port approach environments dependent on the coverage of the IMT2020 systems.

#### Other Communication Technologies Using the UHF/SHF Band

Various other communication technologies in the UHF to SHF bands are being used (or considered) for general maritime communications, namely Wi-Fi, WiMax, and short-range devices like ZigBee and Bluetooth links.  These offer the possibility of high-speed data transfer.

## Future Development of Maritime Radio Communications

This section details future technologies or technologies not widely used for maritime radio communications by frequency band.  Please refer to the table in Annex 3 for technical details.

### Low Frequency Band (LF)

#### Enhanced Long Range Navigation (eLORAN)

The enhanced Loran (eLoran) system is an experimental system to provide data channel modulated onto the approximately 100 kHz signals.  Two formats for this data channel are currently available, known as Eurofix and 9th pulse respectively.  Both techniques offer data rates below 100 bps although higher rate concepts have been proposed.

### Medium Frequency / High Frequency Band (MF/HF)

#### Digital Data Communication Using MF/HF Band

The likely increase in ship traffic in Polar regions, because of receding ice fields, may increase the requirements for HF communications, since geostationary satellites do not cover these areas. Various system operators are studying increased data rates using 3 kHz channels, which may produce data rates of 19.2 kbps, and other solutions to likewise increase data speeds.

Furthermore, a new data communication system using 10-20 kHz bandwidth for data rates up to 51 kbps, has been incorporated in the Recommendation ITU-R M.1798-2.  Appendix 17 to the Radio Regulations was revised at the World Radiocommunication Conference 2012 (WRC-12).  The revision of AP17 will implement new digital bands for 3 kHz systems as well as wideband systems. WRC-19 has also approved use of band 495-505 kHz for use of high-speed Navigational Data service (NAVDAT).

#### NAVDAT (Navigational Data)

The NAVDAT is an MF radio system, used in the maritime mobile service, operating in the 500 kHz band for digital broadcasting of maritime safety and security related information from shore-to-ship.

WRC-12 approved the worldwide exclusive usage of the frequency band 495 - 505 kHz for the maritime mobile service.  The NAVDAT system utilizes an OFDM modulation in this 10 kHz bandwidth which provides a flow rate of about 15/25 Kbit/s (more than 300 times the NAVTEX transmission), featuring:

Possibility to transmit any type of text, graphs, pictures, data etc with encryption if required;

Automatic reception;

Possibility to use Single Frequency Network (SFN) technology, with no need for time slot allocation on the same frequency.

WRC-19 added a footnote to the 495-505 kHz frequency band in the frequency allocation table of Article 5 of RR, and specified its use as NAVDAT. In addition, NAVDAT was added to the footnote at 415-495 kHz and 505-526.5 kHz in addition to radiotelography (NAVTEX).

WRC-19 also lists the NAVDAT frequencies in the Maritime HF Band Frequency Table in RR Appendix 17.

The MF NAVDAT system is described in the Recommendation ITU-R M.2010-1: Characteristics of a digital system, named Navigational Data for broadcasting maritime safety and security related information from shore-to-ship in the 500 kHz band, published by ITU on January 2019 .

The HF NAVDAT system is described in the Recommendation ITU-R M.2058-0: Characteristics of a digital system, named navigational data for broadcasting maritime safety and security related information from shore-to-ship in the maritime HF frequency band, published by ITU on Feb 2014.

### Very High Frequency Band (VHF)

#### Automatic Identification System (AIS)

It is recognized that by design AIS is not an ideal candidate for high speed and/or high volume data communications.  However, AIS is a proven maritime data system, with ships equipped and shore infrastructure established.  The future AIS channel plan should consider at least the following:

Channels for Safety of Navigation purposes:

  - AIS 1 and AIS 2 are exclusively allocated to AIS usage in both Region 2 (Americas, effective 2025) and Region 3 (Asian/Pacific, effective 2013), but not in Region 1 (Europe/Africa) where they are still shared with land mobile and fixed services (WRC-12);

  - Taking account of the future agenda item for the modernization of the GMDSS at WRC-18, and if IMO includes AIS as a component of the GMDSS our goal is to apply for exclusive world-wide allocation for AIS 1 and 2 frequencies in the maritime mobile service.

Channels for satellite detection of AIS.

  - Since the satellites have very wide footprint, the frequencies for satellite detection of AIS are exclusive for maritime mobile service.  Appendix 18 of Radio Regulation assigned the CH75 and CH76 for satellite detection of AIS.  These channels are guard band channels of CH16 and are the only maritime dedicated channels except CH16 and CH70 within Appendix 18.

  - AIS satellite long range messaging on channels 75 and 76 was approved worldwide effective 1 January 2013 with a primary allocation in Region 2 (Americas), secondary elsewhere (WRC-12)

  - Non-AIS operations on channels 75 and 76 were limited to 1watt on 1 January 2013 (WRC-12).

  - AIS SARTs are recognised in GMDSS.  In the future, other aspects of AIS may be part of GMDSS.  The distress alerting, urgency and safety communications should be by both terrestrial and satellite communications; therefore, dedicated maritime frequencies are needed.  The satellite detection of AIS is a one-way system (from earth to satellite); however, in the GMDSS, the acknowledgement (from satellite to earth) is essential (a two way system).  The CH75 and CH76 could be used for this purpose; however, WRC-12 approved these channels in the Earth-to-space direction. At WRC-12 channels 27, 87, 28 and 88 have been identified for possible testing of AIS, under Note YYY in Appendix 18.

Channels for ASM purposes:

  - In considering the future AIS system for terrestrial (non-satellite) data communication purposes, an additional two frequencies will be needed.  These frequencies would be used to support the transmission of area warnings and advice, meteorological and hydrographical data, traffic management information and general ship-to-shore data exchanges.

  - At WRC-15 VHF channels 27 and 28 are each split into two simplex channels. The channels 2027 and 2028 designated as ASM 1 and ASM 2 are used for application specific messages (ASM) as described in the most recent version of Recommendation ITU-R M.2092. These channels are envisaged to be part of the VHF Data Exchange System (VDES) (see Annex E).

Channel for channel management purposes:

  - In order to fully use available VHF spectrum, channel 70 was agreed to manage and co-ordinate usage of the VHF Data Link.

Further details are explained in ANNEX E.

#### Digital Data Communication

The use of VHF for digital data transfer has yet to be broadly implemented by the maritime community.  To this end, ITU-R has developed and published Recommendation ITU-R M.1842 – “Characteristics of VHF radio systems and equipment for the exchange of data and electronic mail in the maritime mobile service RR Appendix 18 channels”.  Also, WRC-12 identified digital bands within the Appendix 18 band.

Development of the Recommendation ITU-R M.1842 was originally proposed based on the shore-based network system trialled in Norway.  This system deploys a network of radio modems capable of switching between nine narrowband duplex VHF channels in the maritime mobile band. These nine channels can be combined into one 225 kHz wideband channel.  The wideband radio was proven to have insufficient EMC characteristics, indicated instability and the throughput slowed down when the signal strength was low.  This system is described in the Report ITU-R M.2127, but not incorporated in the Recommendation ITU-R M.1842.

Narrowband radio 25 kHz duplex on the other hand has been proven in Norway to be very robust and stable and can perform a variety of services giving good throughput of data.  This system is incorporated in Recommendation ITU-R M.1842 together with other techniques to be applied more widely - from 25 kHz bandwidth systems providing 43 kbps data rates through to 100 kHz systems offering 307 kbps.  The Recommendation ITU-R M.1842 currently specifies four systems for the modulation schemes and general characteristics for the transceivers.  The consideration of the standardized communication protocols is on-going at ITU-R.

ITU has issued new Recommendation ITU-R M.2092-0 - Technical characteristics for a VHF data exchange system in the VHF maritime mobile band.  VHF data exchange system (VDES) integrates the components of VHF data exchange (VDE), application specific messages (ASM) and the automatic identification system (AIS) in the VHF maritime mobile band.  This recommendation provides the technical characteristics of ASM and VDE.

IALA is aware that further consideration of how best to achieve spectral efficiency in this band is worthwhile.  This consideration needs to address coverage range, robustness, channel spacing, and modulation schemes.

The data speed/coverage trade off in the coastal range must be considered to achieve a cost effective service.

WRC-12 VHF Data Results:

VHF Appendix 18 was modified to permit digital systems on channels: 24, 84, 25, 85, 26 and 86 for Region 2 from 1st Jan 2013 and 80, 21, 81, 22, 82, 23, 83, 24, 84, 25, 85, 26, 86 worldwide except Region 2 and specific countries (footnote D1)) from 1 Jan 2017;

Testing of future AIS applications on channels 27, 28, 87 and 88 is permitted from 1 January 2013;

160.9 MHz (channel 2006) is reserved for experimental use for future applications or systems, e.g. MOB and AIS from 1 January 2013.

WRC-15 VHF Data Results:

VHF Appendix 18 was modified to permit digital systems on channels: 24, 84, 25, 85, 26 and 86 for Region 2 WRC-15 agreed on regulatory provisions and frequency allocations to enable new Automatic Identification System (AIS) applications and other possible new applications to improve maritime radiocommunication.  These new applications for data exchange are intended to improve the safety of navigation;

WRC-15 made new allocations in the bands 161.9375-161.9625 MHz and 161.9875-162.0125 MHz for the maritime mobile-satellite service in the uplink and amended the channelling arrangement for VHF maritime frequencies contained in ITU RR Appendix 18;

While the proposed allocation for the maritime mobile-satellite service (MMSS) in the downlink in the requested frequency band (161.7875-161.9375 MHz) was not agreed at WRC-15, the use of satellite communications for VHF data exchange was agreed.  WRC-15 agreed to further studies of compatibility between MMSS and incumbent services in the same and adjacent frequency bands, for consideration by WRC-19.

WRC-19 VHF Data Results:

WRC-19 made new secondary allocations in the bands 157.1875-157.3375 MHz and 161.7875-161.9375 MHz for the maritime mobile-satellite service in the uplink/downlink and amended the channelling arrangement for VHF maritime frequencies contained in ITU RR Appendix 18;

The frequency bands 157.1875 157.3375 MHz and 161.7875-161.9375 MHz (corresponding to channels: 24, 84, 25, 85, 26, 86, 1024, 1084, 1025, 1085, 1026, 1086, 2024, 2084, 2025, 2085, 2026 and 2086) are identified for the utilization of the VHF Data Exchange System (VDES). The VDES terrestrial and satellite components are described in the most recent version of Recommendation ITU R M.2092. These channels shall not be used for feeder links. The channels may be merged using multiple 25 kHz contiguous channels to form channel bandwidths of 50, 100 or 150 kHz;

The channels 1024, 1084, 1025 and 1085 are identified for ship-to-shore, shore-to-ship and ship-to-ship communications, but ship-to-satellite and satellite-to-ship communications may be possible without imposing constraints on ship-to-shore, shore-to-ship and ship-to-ship communications;

The channels 2024, 2084, 2025 and 2085 are identified for shore-to-ship and ship-to-ship communications, but ship-to-satellite and satellite-to-ship communications may be possible without imposing constraints on shore-to-ship and ship-to-ship communications;

The channels 1026, 1086, 2026 and 2086 are identified for ship-to-satellite and satellite-to-ship communications and are not used by the terrestrial component of VDES;

The channels 24, 84, 25 and 85 are identified for ship-to-shore and shore-to-ship communications.

Further details are explained in Annex E – VHF Data Exchange System VDES.

#### Digital communication

A number of digital communication systems are available for use with Maritime Aids to Navigation and on board ship and for ship to shore and shore to ship depending on application. These are based on IMT-2020 technologies and include:

LoraWAN used for data collection from sensors from shore side systems as well on board ship

NB-IoT and LTE-M being part of the IMT system and used for lower rate data services

There are a growing number of proprietary digital data communication system that are becoming available as a result of the rapid growth of the Internet of Things (IoT) sector.

#### Digital Voice Communication

Digital voice communication may, in the long term, replace the present analogue VHF voice communication service, i.e. ship-to-ship/ship-to-shore/shore-to-ship. As this develops, the introduction of mixed digital / analogue equipment should be encouraged. It is recognized that global digitization will make spectrum use more efficient, but this will take some time to complete.

#### R-Mode

<inputs required>

### Ultra High Frequency Band / Super High Frequency Band (UHF/SHF)

#### Satellite Communication Using the UHF/SHF Band

In the future, navigational satellite payloads may include transponders connected with GMDSS and may function as additional SAR resources. (i.e. return link capability is possible functionality within Galileo).

#### Public Mobile Wireless Communications

Public mobile wireless communications (public correspondence) such as IMT-2020 technologies are being used by mariners and ports / port authorities in coastal waters and could be further developed to support evolving maritime communication needs. In addition, 5G coverage is expanding and several satellite service providers have announced that they are working to deploy satellite based 5G services that would be available to the maritime sector.

#### On-board Communications

Limited availability of spectrum for on-board communications gives rise to congestion and interference. Consideration of this issue at ITU is of utmost importance as well as developing new techniques for communication in this band.

## Overview of the Current and Future Voice and Data Communication Technologies

Radio communication technologies are characterized by performance parameters that include range, bandwidth, latency, and the need for infrastructure.

The tables below list, in the column headings, the communications techniques that are presently available for maritime voice and data communications.  It also includes some techniques which may become available in the short to medium term.  Separate tables are provided for ship-to-shore and for ship-to-ship communications.

Six geographical regions are listed at the left column, defined by the ranges of a selection from the available communications technologies.  The GMDSS sea areas, which approximately correspond, are also noted in the second column.

The suggested most likely candidates for voice and data communications, for e-Navigation, in each of the six geographical areas are indicated by yellow colour, with ‘E’ for existing technologies, and ‘F’ for future technologies or technologies currently not widely used in the maritime domain.

Table 1 – Overview of Communications Technologies

| Communication Technology | On air data rate | Available data rate | Primary constraints | Infrastructure | Coverage | Transmission | Maritime / public |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NAVDAT | 12-18 kbps |  |  | Based on NAVTEX | 250/300 NM | Broadcast Terrestrial | Maritime |
| VDES VDE | 307 kbps |  | The data rate is proportional to the signal strength (the higher the signal strength, the higher the data rate) | VHF Data link, RR Appendix 18 channels | 15 NM-65 NM Satellite component provides further coverage | Addressed / Broadcast Terrestrial / Satellite | Maritime |
| VDES ASM | 19.2 kbps |  | ASM has a clear mode and an error corrected mode. The clear mode has a higher data rate. | VHF Data link, RR Appendix 18 channels | approx 15 NM-65 NM Satellite component provides further coverage | Addressed / Broadcast Terrestrial / Satellite | Maritime |
| Wi-Fi (IEEE 802.11ac) | 1,300 kbps |  |  | Routers / Access points | 50m | Addressed | Public |
| WiMax | 75 Mbps |  |  | Routers / Access points | 2-5 km | Addressed | Public |
| Digital VHF | 9.6 – 19.2 kbps |  |  | Base station / mobile radios | approx 1 5NM-65 NM | Addressed | Maritime |
| Digital HF | 19.2 kbps |  |  | Base station / mobile radios | Global | Addressed | Maritime |
| 4G (including LTE) | 600 Mbps |  |  | 4G Base stations | 5-30 km (3-6 NM) | Addressed | Public |
| 5G | 1,200 Mbps |  |  | 5G base stations | 5-30 km (3-6 NM) | Addressed | Public |
| GEO Satellite |  |  |  |  |  |  |  |
| Inmarsat C | 600 bps |  |  | Satellite service | Global, spot beams | Addressed / broadcast | Maritime |
| Inmarsat GX | 50 Mbps |  |  | Satellite functioning on Ka band | Global, spot beams | Addressed / broadcast | Cross Industry |
| LEO Satellite |  |  |  |  |  |  |  |
| Iridium | Up to 134 kbps |  |  | Satellite functioning on L band | Global, dependent on constellation size | Addressed / broadcast | Cross Industry (Iridium Pilot Maritime) |

## Future Vision: Automated Selection Process of Available Communication Technologies

Table 1 identifies distinct communication technologies but do not consider the optimization that could be achieved through a system that automatically selects the most appropriate technology for the area of operation and the type of data to be exchanged.

Some existing marine communication technologies, such as VDES (including AIS), automatically carry out data communications without any need for the ship’s crew to intervene in the communications process.  In the specific case of AIS, while the crew sets certain voyage parameters at the start of each voyage, and can change the navigational status at any time, the communications process is automatic.)  Other existing marine communications technologies, such as VHF voice communications, require the ship’s crew to adjust the communications apparatus. In the case of VHF voice this involves deciding on and selecting the appropriate communications channel.

An ideal e-Navigation communications system would operate automatically, selecting the best communications technology, channel, and characteristics in accordance with the ship’s location, and the type of data to be exchanged.  This automatic process would be managed in accordance with rules and the needs of the mariner that might include the following:

Need to avoid latency – such as when exchanging safety and navigation data with other vessels or receiving it from electronic systems ashore or on the water surface;

Ability to delay the sending or receipt of data – such as the sending of non-urgent administrative data, or the receipt of chart corrections for the current voyage, a subsequent voyage or for a port to be visited later in the present voyage;

Value / Cost of communications;

Importance of the data for commercial services on board;

Rate of data transfer (capacity of transmission).

Some of the rules managing seamless data communications would be programmed into the system, for example those concerned with safety of navigation.  Other rules might be set by the ship’s operator in accordance with their procedures and the operator’s commercial model.  Further rules might be adjusted, from time to time, by the ship’s crew.

## Modernisation of GMDSS

The current GMDSS system was designed over 25 years ago and is currently being reviewed by IMO.  This is the first full review since its implementation in 1999 and recognises that technology has developed significantly in that time.  Existing technology elements within the GMDSS have also evolved, although the functions have not been altered.  The current system remains sound, however there are GMDSS elements where improvement could be made, e.g. managing the cessation of international telex, and reviewing the continued use of narrow-band direct-printing in certain sea areas.

The elements that will be identified may need to be examined and reviewed as a matter of some urgency.  The 14th session of IMO Sub-Committee on Radiocommunications and Search and Rescue (COMSAR 14, held on March 2010) initiated a Scoping Exercise and a Work Plan to define the requirements for the GMDSS Review and Modernization.  The Scoping Exercise was finalized at COMSAR16 (March 2012) and the Review will be take place over a three-year period (2013-2015).

A further two-year period is envisaged (2015-2017) for the GMDSS modernization plan.  This will be followed by development of legal instruments, revision/development of relevant performance standards and an implementation period.

While supporting the IMO’s systematic approach, IALA should also be active in the review, push forward initiatives to ITU with a view to the fact that items for consideration at WRC-19 need to be produced as agenda items at WRC-

### Current GMDSS System Components

The GMDSS functions are classified into alerting (distress/urgency/safety); communications (distress/urgency/safety), locating in SAR, homing on location in SAR; on scene communications; promulgation of maritime safety information (MSI); and general communications.  There are 4 sea areas identified in GMDSS – Sea Area A1; Sea Area A2; Sea Area A3 and Sea Area A4.  To achieve the functions of GMDSS, the equipment listed in Table 2 - Current GMDSS System Components is used according to the GMDSS sea areas.

Table 2 - Current GMDSS System Components

| Sea area | Distress/Urgency Safety Alerting | Distress/urgency Safety Comms | Locating | Homing | On scene Comms | MSI Promulgation | General Comms |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A1 | VHF DSC EPIRB | VHF R/T | Radar SART AIS-SART | EPIRB (121.5 MHz) | VHF Rx/Tx | NAVTEX SafetyNET | VHF R/T |
| A2 | VHF DSC MF DSC EPIRB | VHF R/T MF R/T |  |  |  |  | VHF R/T MF R/T, NBDP |
| A3 | VHF DSC MF DSC HF DSC Inmarsat EPIRB | VHF R/T MF R/T HF R/T, NBDP Inmarsat |  |  |  | NAVTEX SafetyNET HF MSI | VHF R/T HF/MF R/T, NBDP Inmarsat |
| A4 | VHF DSC MF DSC HF DSC EPIRB | VHF R/T MF R/T HF R/T, NBDP |  |  |  | HF MSI | VHF R/T HF/MF R/T, NBDP |

R/T – Radio Telephony

GMDSS is under review at the IMO.  The problems in the current GMDSS include the following:

Perceived high rate of false distress alerting by DSC and EPIRB;

complicated operation of DSC; and

decline in use of the NBDP

Efforts have been made to address these issues, such as updating ITU-R Recommendations and IEC standards.  The AIS-SART was introduced in January 2010 as an alternative to the radar SART.  The AIS-SART is defined as a locating device.  Field trials have proved that the AIS-SART is detectable by SAR aircraft at a much greater range than 121.5 MHz beacons in 406 MHz EPIRBs and can also be detected by low earth orbit AIS satellites.

### Possible System Components for the Modernisation of GMDSS

IMO COMSAR sub-committee finalized the draft work plan on ‘Review and modernization of the Global Maritime Distress and Safety System’ at its 16th session in March 2012, and this was approved by Maritime Safety Committee at its 90 session in May 2012.  The work is continuing and includes a detailed review on a number of items, including: the possible inclusion of AIS functions, LRIT functions and SSAS functions; the role of Narrow Band Direct Printing (NBDP) and MF/HF DSC; expected evolution of satellite EPIRB systems; and the further evolution of Maritime Safety Information broadcast systems.

While AIS (AIS-SART) is currently included in the GMDSS as a locating device, the review of possible inclusions of AIS functions in the GMDSS modernization work plan could see additional uses for AIS, which may include homing.  Other digital data technologies, such as VDES, may be used to address GMDSS functions.

WRC-12 allocated 495-505 kHz (plus 505-510 kHz in Region 2) to the maritime mobile service on an exclusive primary basis, this band could be used for promulgation of maritime safety and security related information in accordance with the Recommendation ITU-R M.2010. This could deliver relatively large volumes of information with a high data rate and may be a supplementary system to the current NAVTEX.

The VHF Data Exchange System (VDES) may also be a candidate for promulgation of Maritime Safety Information (shore-ship) and emerging ship-ship / ship-shore applications.

The MF/HF data communications and VHF Data Exchange System (VDES) may be candidates for general communications in addition to the existing radiotelephone and satellite communications.

The above ideas are summarized in Table 3.

Table 3 - Potential System Components for Modernisation of GMDSS

| Sea area | Distress/Urgency Safety Alerting | Distress/urgency Safety Comms | Locating/Homing | On scene Comms | MSI Promulgation | General Comms |
| --- | --- | --- | --- | --- | --- | --- |
| A1 | AIS *1 EPIRB-AIS *2 | VHF R/T, Data | AIS-SART EPIRB-AIS *2 | VHF R/T | AIS *4 VDE NAVDAT SafetyNet | VHF R/T, Data MF/HF R/T, Data Satellite *3 |
| A2 A3 | AIS *1 EPIRB-AIS *2 MF/HF Data Satellite *4 | VHF R/T, Data MF/HF R/T, Data Satellite *2 |  |  |  |  |
| A4 | AIS *1 EPIRB-AIS *2 MF/HF Data | VHF R/T, Data MF/HF R/T, Data |  |  | AIS *4 HF Data | VHF R/T, Data MF/HF R/T, Data |

*1	using terrestrial and satellite messages

*2	EPIRB with built-in AIS-SART function

*3	global or regional satellite services

*4	using application specific messages

R/T	Radio Telephony

# Securing the Spectrum

## Situation with Respect to Existing Spectrum Usage

Section 3.3 described the existing maritime technologies, which use the radio spectrum and indicated the bands in which they operate.  Whilst there is consideration of variations to the technologies which may be used, all such variations take, as their base assumption, the ongoing use of existing spectrum allocations.  These variations may give rise to a need to change the channelization within certain bands.

Please refer to the table in Annex A for system details, Annex B for maritime spectrum allocations and Annex D for specific ITU technical characteristics associated with the systems described in section 3.3 and 3.4.

## WRC-19 Agenda Items [full section – update to WRC-23 agenda items ]

A number of the agenda items for the 2019 World Radiocommunications Conference (WRC-19) may affect the maritime use of the spectrum, especially:

### Agenda Item 1.11

Agenda item 1.11 to consider possible regulatory actions to support the modernization of the Global Maritime Distress and Safety System and the implementation of e-navigation, in accordance with Resolution 361 (Rev.WRC-19);

1	to consider possible regulatory actions, based on the ITU Radiocommunication Sector (ITU R) studies, taking into consideration the activities of IMO, as well as information and requirements provided by IMO, to support GMDSS modernization;

2	to consider possible regulatory actions, including spectrum allocations based on the ITU Radiocommunication Sector (ITU R) studies, for the maritime mobile service, supporting e-navigation;

3	to consider regulatory provisions, if any, based on the results of ITU R studies, referred to in invites ITU R below, to support the introduction of additional satellite systems into the GMDSS.

## Preliminary Agenda for WRC-23

Resolution 812 (WRC-19) has a direct affect for maritime spectrum use:

- to consider improving the utilization of the VHF maritime frequencies in Appendix 18, in accordance with Resolution 363 (WRC-19)

# Summary

The IALA Maritime Radio Communications Plan identifies current and future technologies used for maritime communication and addresses the IALA position regarding maritime related agenda items for WRC-15 and WRC-19, and also looks ahead to WRC-23.

e-Navigation is a driving factor for a digital concept for the maritime sector. Developments in digital communications have a profound and long-term impact on the way the maritime sector operates.  It is foreseen that e-navigation will be supported by many current and future technologies identified in this plan.  The communications infrastructure should be designed to enable authorized seamless information transfer on board ship, between ships, between ship and shore and between shore authorities and other parties.  This infrastructure will have to be capable of not only supporting future e-Navigation applications, but will also have to support legacy applications.  It will therefore be bandwidth intensive and possibly rely upon a range of technologies.

Securing spectrum, or changing the use of spectrum within existing allocations, is important for e-Navigation and other developments due to the planning timescales required to protect or extend the use of the radio spectrum through the ITU WRC.

Modern services on land request a ubiquitous connectivity on the vessel which is not addressed by current maritime networks.  Dedicated service connectivity to improve services (for example logistic chain or port arrival and clearance) would benefit from the concept of a network at sea. The network at sea defines a fully meshed connectivity between all vessels and land stations which covers direct and multi-hop connections. The network comprises of a terrestrial and a satellite component and both components are currently developed with a long-range connectivity through the new VDES system for terrestrial and satellite connectivity. In the near future increasing demands, such as the maritime connectivity platform, require a broadband connectivity that requires to being developed. In the future, it is envisioned that there will be a network on the sea connecting the shore and vessels, and between vessels themselves, using digital technologies such as VDES, broadband maritime and satellite communications.

In general, there are ongoing concerns relating to spectrum allocation and management for e-Navigation.  Protection of the maritime mobile service spectrum allocation needs to continue.  Particular emphasis should be given to following items:

development of frequency plans for the 450 to 470 MHz band so as to protect on-board use; and

the use of PLT/PLB at a national level as and when the possibility or necessity arises;

In relation to the forthcoming WRC meeting, spectrum requirements to support e-Navigation should be addressed.  In particular:

on board communications needs;

the requirements for VDES, including VDE-SAT;

protection of HF RT/WT allocations.

Finally, the plan recognises the need for maritime communications to develop automated processes for selecting the best communications technology, channel, and characteristics in accordance with the ship’s location, and the type of data to be exchanged.

The plan also suggests modernization of GMDSS could include the modern digital technologies such as VHF Data exchange System (VDES), AIS, and data and voice communications in MF/HF and VHF bands.

The e-Navigation concept and future user requirements are rapidly being developed but it is difficult to speculate what specific systems and spectrum will be required to achieve e-Navigation.  As e-Navigation develops the IALA Maritime Radio Communication Plan, as a living document, needs to be reviewed and updated.

# Glossary of Terms

AIS	Automatic Identification System

AMRD	Autonomous Maritime Radio Devices

ARPA	Automatic Radar Plotting Aid

AtoN(s)	Aid(s) to Navigation

BeiDou	China Navigation Satellite System

CDMA	Code Division Multiple Access

DGNSS	Differential Global Navigation Satellite System

DGPS	Differential Global Positioning System

ECDIS	Electronic Chart Display Information System

EGC	Enhanced Group Call

eLoran	Enhanced Loran

ELT	Emergency Locator Transmitters

EPIRB	Emergency Position-Indicating Radio Beacon

EPIRB AIS	Emergency Position-Indicating Radio Beacon with AIS

ETSI	European Telecommunications Standards Institute

FDD	Frequency Division Duplex

GAGAN	GPS-Aided Geo Augmented Navigation (System) (India)

GALILEO	European GNSS (not an acronym)

GEO	Geostationary Orbit

GMDSS	Global Maritime Distress and Safety System

GLONASS	Global Navigation Satellite System (Russian Federation)

GNSS	Global Navigation Satellite System

GPRS	General Packet Radio Service

GPS	Global Positioning System (US)

GSM	Global System for Mobile Communications

HF	High Frequency (3 – 30 MHz)

HSDPA	High Speed Downlink Packet Access

IALA	International Association of Marine Aids to Navigation and Lighthouse Authorities

IBS	Integrated Bridge System

ICAO	International Civil Aviation Organization

IEC	International Electrotechnical Commission

IHO	International Hydrographic Organization

IMO	International Maritime Organization

IMEA	International Maritime Electronics Alliance

INS	Integrated Navigation System

IP	Internet Protocol

ITU	International Telecommunication Union

ITU-R	ITU Radiocommunication Sector

LEO	Low Earth Orbit

LF	Low Frequency (30 – 300 kHz)

LORAN	Long Range Navigation system

LRIT	Long Range Identification and Tracking

LTE	Long Term Evolution

MASS	Maritime Autonomous Systems (Surface)

MEO	Medium Earth Orbit

MF	Medium Frequency (300 – 3 000 kHz)

MMS	Maritime Mobile Service

MMSS	Maritime Mobile Satellite Service

MRCP	Maritime Radio Communications Plan

NBDP	Narrow Band Direct Printing

NAVDAT	Navigational Data (the system name)

NAVTEX	Navigational Telex (the system name)

OFDM	Orthogonal Frequency Division Multiplexing

PIANC	Permanent International Association of Navigation Congresses

PLB	Personal Locator Beacon

PLT	Power Line Transmission

PMR	Private Mobile Radio

PNT	Position, Navigation and Timing

QAM	Quadrature Amplitude Modulation

QPSK	Quadrature Phase Shift Keying

RACON	RAdar BeaCON

RCC	Rescue Coordination Center

RAIM	Receiver Autonomous Integrity Monitoring

RR	Radio Regulations

SBAS	Satellite Based Augmentation System

SHF	Super High Frequency (3 – 30 GHz)

SMS	Short Message Service

SOLAS	Safety of Life at Sea (IMO Convention)

TDD	Time Division Duplex

TDMA	Time Division Multiple Access

TETRA	Terrestrial Trunked Radio

UHF	Ultra High Frequency (300 – 3 000 MHz)

UMB	Ultra Mobile Broadband

UMDM	Universal Maritime Data Model

UMTS	Universal Mobile Telecommunications System

UWB	Ultra Wideband

VDE	VHF Data Exchange

VDE-ASM	VHF Data Exchange System – Application Specific Messages

VDES	VHF Data Exchange System

VDE-SAT	VHF Data Exchange System – Satellite

VDE-TER	VHF Data Exchange System - Terrestrial

VDL	VHF Data Link

VDR	Voyage Data Recorder

VHF	Very High Frequency (30 – 300 MHz)

VTS	Vessel Traffic Service

Wi-Fi	Wireless Fidelity

WiMax	Worldwide Interoperability for Microwave Access

# ANNEX A – System Description

Table 6 - System Description Table

| System | Band | Frequency Channel | Bandwidth Data rate | Status | Life Span | Ownership | Mode | Service | Purposes | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Distress and safety communication within GMDSS |  |  |  |  |  |  |  |  |  |  |
| # MF/HF Voice | # MF/HF | # 2182 kHz 4125 kHz 6215 kHz 8291 kHz 12290 kHz 16420 kHz | # 3 kHz | # Current | # Long | # Regional and International | # Analogue voice. | # Mobile to mobile Fixed to mobile Mobile to fixed | # Distress communication | # Long distance, > 250 nm |
| # MF/HF DSC | # MF/HF | # 2187.5 kHz 4207.5 kHz 6312.0 kHz 8414.5 kHz 12577 kHz 16804.5 kHz | # 0.5 kHz 100 bps | # Current | # Long | # International | # Data (Digital) | # Mobile to mobile Fixed to mobile Mobile to fixed. | # Distress alerting | # Long distance > 250 nm |
| # 121.5 DF | # VHF | # 121.5 MHz |  | # Current | # Short | # International | # Carrier (Analogue) | # Mobile to mobile | # Location. | # Line of sight |
| # VHF DSC | # VHF | # 156.525 MHz # (Ch 70) | # 25 kHz 1200 bps | # Current | # Long | # International | # Data (Digital) | # Mobile to mobile Fixed to mobile Mobile to fixed | # Distress alerting | # approx 15NM-65NM |
| # VHF voice | # VHF | # 156.300 MHz (Ch 06) 156.650 MHz (Ch 13) 156.800 MHz (Ch 16) | # 25 kHz | # Current | # Long | # International | # Voice | # Mobile to mobile Fixed to mobile Mobile to fixed | # Distress communication | # Line of sight |
| # Two way VHF voice | # VHF | # 156.025 - 161.950 MHz | # 25 kHz | # Current | # Long | # International | # Voice | # Mobile to mobile | # On scene communication | # Line of sight |
| # AIS-SART | # VHF | # 161.975 MHz, 162.025 MHz | # 25 kHz 9600 bps/ TDMA | # Current | # Long | # International | # digital | # Mobile to mobile | # Location | # approx 15NM-65NM |
| # EPIRB | # UHF | # 406 MHz |  | # Current | # Long | # International | # Digital | # Mobile to satellite | # Distress alerting / Location / Homing | # COSPAS-SARSAT Satellite; Global coverage |
| # Satellite INMARSAT C, B, F | # UHF | # Tx 1626.5 to 1646.5 MHz Rx 1525.0 to 1545.0 MHz |  | # Current | # Long | # 3rd party | # Digital. Voice and data. | # Satellite to earth Earth to satellite | # Distress alerting, distress communication. | # Global coverage |
| # RADAR SART, X-Band | # SHF | # 9.2 – 9.5 GHz |  | # Current | # Medium | # International | # analogue | # Mobile to mobile | # Homing | # Line of sight |
| # EPIRB-AIS | # UHF / VHF | # 406 MHz # 161.975 MHz (AIS 1) # 162.025 MHz (AIS 2) |  | # Current | # Long | # International | # Digital | # Mobile to satellite # Mobile to mobile | # Distress alerting / Location / Homing | # COSPAS-SARSAT Satellite; # Global coverage |
| # Maritime safety information promulgation |  |  |  |  |  |  |  |  |  |  |
| # NAVTEX | # MF/HF | # 518 kHz 490 kHz and 4209.5 kHz (for local language) | # 0.5 kHz | # Current | # Medium | # International | # Text | # Fixed to mobile | # Reception of maritime safety information. | # Long distance, > 250 nm Broadcast to mobile only. |
| # Narrow band direct printing. (NBDP) | # HF | # 4210 kHz 6314 kHz 8416.5 kHz 12509 kHz 16806.5 kHz 19680.5 kHz 22376.0 kHz 26100.5 kHz | # 0.5 kHz | # Current | # Medium | # International. | # Text | # Mobile to mobile Fixed to mobile Mobile to fixed | # Reception of maritime safety information | # Long distance, > 250 nm |
| # EGC | # UHF | # Tx 1626.5 - 1646.5 MHz Rx 1525.0 - 1545.0 MHz |  | # Current | # Long | # International | # Digital data. | # Satellite to mobile | # Reception of maritime safety information (Safety Net) | # Global coverage |
| # Safety of navigation |  |  |  |  |  |  |  |  |  |  |
| # Loran and eLoran | # LF | # 90-110 KHz |  | # Current | # Very Long | # Regional | # Carrier (Analogue) | # Fixed to mobile | # Positioning | # Long distance |
| # DGNSS | # MF | # 283.5 to 325 kHz | # 50/100 bps | # Current | # Medium. | # Regional. | # Digital | # Fixed to mobile | # Augmentation of positioning | # Medium distance |
| # AIS | # VHF | # 161.975 MHz (AIS 1) 162.025 MHz (AIS 2) | # 25 kHz 9600 bps/ TDMA | # Current | # Long | # International | # Data (Digital) | # Mobile to mobile Fixed to mobile Mobile to fixed | # Collision avoidance, Environmental protection, Security, VTS Augmentation of positioning, communication for navigational and safety related purposes. | approx 15NM-65NM LEO Satellite detection for tracking Part of VDES |
| # Satellite AIS | # VHF | # Channels 75 & 76 within RR App 18 | # 25 kHz # 9600 bps # TDMA | # Current | # Long term | # International | # Data # (Digital) | # Mobile to satellite. | # Security. Environmental protection. # Vessel tracking. | Closely related to current AIS. Global Same transponder on ship Part of VDES |
| # AIS MMS channel management | # VHF | # Channel 70 # (AIS 1 and AIS 2) | # 25kHz # 9600bps # TDMA | # Current | # Long term | # International | # Data # (Digital) | # Mobile to mobile. # Fixed to mobile. | # To manage data transfer over channels in the MMS. | E.g. “go to channel XX to get chart update for Tokyo Bay. |
| # AIS AtoN | # VHF | # 161.975MHz (AIS 1) 162.025MHz (AIS 2) | # 25 kHz 9600 bps/ TDMA | # Current | # Long | # International | # Data (Digital) | # AtoN (fixed/floating) to mobile/shore | # Navigational aid | # approx 15NM-65NM |
| # VHF voice | # VHF Mobile | # 156.025 to 161.950 MHz | # 25 kHz | # Current | # Long | # International | # Voice | # Mobile to mobile. Fixed to mobile. Mobile to fixed. | # communication for navigational and safety related purposes | # Line of sight |
| # GNSS | # UHF | # GPS: # 1227.6 MHz 1575.42 MHz GLONASS: 1559-1610 MHz 1240-1260 MHz BeiDou 1.561098 GHz 1.589742 GHz 1.20714 GHz 1.26852 GHz Galileo 1.164-1.215 GHz 1.260-1.300 GHz 1.559-1.592 GHz |  | # Current Current Current Future | # Long Long Long # Long | # International International Regional International | # Digital | # Satellite to earth | # Positioning | # Global coverage |
| # LRIT | # Satellite | # Tx 1626.5 to 1646.5 MHz Rx 1525.0 to 1545.0 Mhz |  | # Current | # Short | # 3rd party | # Data (Digital) | # Inmarsat | # Security. Vessel tracking. Environmental protection. | # Global coverage |
| # Radar S Band |  | # 2.9 to 3.1 GHz |  | # Current | # Long | # International |  | # Mobile and fixed | # Collision avoidance, Navigational aid. | # Line of sight |
| # Radar X Band |  | # 9.2 to 9.5 GHz |  | # Current | # Long | # International |  | # Mobile and fixed. | # Collision avoidance, Navigational aid | # Line of sight |
| # racon |  | # 2.9 to 3.1 GHz and 9.2 to 9.5 GHz |  | # Current | # Long | # International |  | # Fixed. | # Navigational aid | # Line of sight |
| # General communication |  |  |  |  |  |  |  |  |  |  |
| # HF Voice | # MF/HF | # 1.6 to 26.5 MHz | # 3 kHz | # Current | # Long | # Regional and International | # Analogue voice. (ALE) | # Mobile to mobile Fixed to mobile Mobile to fixed | # General voice communication | # Long distance, > 250 nm |
| # Narrow band direct printing (NBDP) | # MF & HF. | # 1.6 to 26.5 MHz | # 0.5 kHz | # Current | # Long for GMDSS Short for general commun-ication | # International | # Text | # Mobile to mobile. Fixed to mobile. Mobile to fixed. | # General text communication | # Long distance, > 250 nm |
| # HF Data | # MF/HF | # 1.6 to 26.5 MHz | # 0.5 – 3 kHz | # Current | # Long | # Regional and International | # Digital data | # Fixed to mobile Mobile to fixed | # General data communication | # Long distance, > 250 nm |
| # VHF voice | # VHF | # 156.025 to 161.950 MHz | # 25 kHz | # Current | # Long | # international | # Voice | # Mobile to mobile Fixed to mobile Mobile to fixed | # General voice communication | # Line of sight |
| # VHF data | # VHF | # 156.025 - 161.950 MHz | # 25 kHz | # Current | # Long | # regional | # data | # Mobile to mobile Fixed to mobile Mobile to fixed | # General data communication | # approx 15NM-65NM |
| # UHF # on-board | # UHF | # 457.5125-457.5875/ 467.5125-467.5875 MHz | # 6.25/12.5/25 kHz | # Current | # Long | # International | # Analogue voice/ Digital voice/ data | # Mobile to mobile. # Internal comms. | # On-board communication | # Line of sight |
| # Satellite | # UHF | # Tx 1626.5 - 1646.5 MHz Rx 1525.0 - 1545.0 MHz |  | # Current | # Long | # global 3rd party | # Digital. Voice and data. | # Satellite to earth Earth to satellite | # General voice and data communication | # Global coverage |
| # GSM | # UHF |  |  | # Current | # Long | # regional 3rd party | # Digital. Voice and data. | # Cellular Mobile-fixed-mobile. | # General voice and data communication | # Medium range in locality |
| # GPRS | # UHF |  |  | # Current | # Long | # regional 3rd party | # Digital # Voice and data | # Cellular Mobile-fixed-mobile. | # General voice and data communication | # Short range in locality |
| # UMTS/3G | # UHF |  |  | # Current | # Long | # regional 3rd party | # Digital. Voice and data. | # Cellular. Mobile-fixed-mobile. | # General voice and data communication | # Short range in locality |
| # IEEE 802.11 (Wi-Fi) | # UHF/ SHF | # 2.4 – 2.5 GHz 5.17 – 5.33 GHz 4.9 GHz | # 11 Mbps 54 - 300 Mbps 54 Mbps | # Current | # Long | # local 3rd party | # Digital data | # LAN Peer-to peer | # Internet access | # Sort in locality < 100m <10 km |
| # IEEE 802.16 (WiMax) | # UHF/ SHF | # 2.5 GHz 3.3 GHz | # 75 Mbps | # Current | # Long | # local 3rd party | # Digital data | # LAN Peer-to peer | # Internet access | # Short range in locality < 50 km |
| # IEEE 802.15.4 (ZigBee) | # UHF | # 868-870 MHz 902 – 928 MHz 2.4 GHz | # 20 kbps 40 kbps 250 kbps | # Current | # Long | # local 3rd party | # Digital data |  |  | # Short range in locality < 30 km |

| # Future development |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # HF Digital data | # HF | # 4 to 26 MHz | # 10 to 20 kHz. | # On and after 1 Jan 2017 | # Long term | # International | # Data (Digital) | # Mobile to mobile Fixed to mobile Mobile to fixed | # Binary data communication for navigational and safety related purposes. | # KENTA (France) Has range advantage over VHF 40 to 250 NM (>250 sky wave) |
| # ASM | # VHF | # 161.950 MHz (ASM 1) # 162.000 MHz (ASM 2) | # 25 kHz Min 9600 bps TDMA | # On and after 1 Jan 2019 | # Long term | # International | # Data (Digital) | # Mobile to mobile Fixed to mobile Mobile to fixed Mobile to satellite | # Binary data communication for navigational and safety related purposes. Security and tracking. | # Closely related to current AIS. Same transponder on ship # Part of VDES |
| # VHF Digital data. - ECDIS chart data - Digital voice - Virtual channels - UMDM support - Internet access | # VHF # Mobile | # 156.025 to # 161.950 MHz | # 25 to 100 kHz. | # On and after 1 January 2013 in Region 2, # 1 January 2017 in Regions 1 & 3 | # Long term | # International | # Data (Digital) | # Mobile to mobile # Fixed to mobile Mobile to fixed | # Binary data communication for navigational and safety related purposes. | # Rec. ITU-R M.1842-1 Rec. ITU-R M.2092-Managed or autonomous systems. # Relates to VDES |

# Annex B – Maritime Frequency Allocations

The table below sets out the various current maritime frequency allocations as allocated by ITU Radio Regulations.  Due to the size, footnotes to the allocation in the ITU Radio Regulation are not included.  Please refer to Article 5 in the ITU Radio Regulation, Edition of 2016.

Table 7 - Maritime Frequency Allocations

| Frequency | Region 1 Allocation | Region 2 Allocation |  | Region 3 Allocation | Status of Maritime Use |
| --- | --- | --- | --- | --- | --- |
| 90 – 110 kHz | # RADIONAVIGATION 5.62 # Fixed 5.64 |  |  |  | # No constraints |
| # 285 – 315 kHz | # AERONAUTICAL RADIONAVIGATION # MARITIME RADIONAVIGATION (radiobeacons) # 5.73 5.74 | # AERONAUTICAL RADIONAVIGATION # MARITIME RADIONAVIGATION (radiobeacons) 5.73 |  |  | # No constraints |
| # 315 – 325 kHz | # AERONAUTICAL RADIONAVIGATION # Maritime radionavigation (radiobeacons) 5.73 5.75 | # MARITIME RADIONAVIGATION (radiobeacons) 5.73 # Aeronautical radionavigation |  | # AERONAUTICAL RADIONAVIGATION # MARITIME RADIONAVIGATION (radiobeacons) 5.73 | # Secondary in Region 1 |
| # 415 – 435 kHz | # MARITIME MOBILE 5.79 # AERONAUTICAL RADIONAVIGATION | # MARITIME MOBILE 5.79 # Aeronautical radionavigation 5.80 5.77 5.78 5.82 |  |  | # No constraints |
| # 435 - 472 | # MARITIME MOBILE 5.79 # Aeronautical radionavigation 5.77 5.82 |  |  |  | # No constraints |
| # 472 - 479 | MARITIME MOBILE 5.79 Amateur 5.A123 Aeronautical radionavigation 5.77 5.80 5.82 5.B123 |  |  |  |  |
| # 479 – 495 kHz | MARITIME MOBILE 5.79 5.79A Aeronautical radionavigation 5.77 5.82 | MARITIME MOBILE 5.79 5.79A Aeronautical radionavigation 5.80 5.77 5.82 |  |  | # No constraints |
| # 495 – 505 kHz | # MARITIME MOBILE |  |  |  | # No constraints |
| # 505 – 510 kHz | MARITIME MOBILE 5.79 5.79A 5.84 AERONAUTICAL RADIONAVIGATION | # MARITIME MOBILE 5.79 |  | # MARITIME MOBILE 5.79 5.79A 5.84 # AERONAUTICAL RADIONAVIGATION # Aeronautical mobile # Land mobile | # No constraints |
| # 510 – 526.5 kHz | MARITIME MOBILE 5.79 5.79A 5.84 AERONAUTICAL RADIONAVIGATION | # 510 – 525 KHz # MARITIME MOBILE 5.79A 5.84 # AERONAUTICAL RADIONAVIGATION |  | # MARITIME MOBILE 5.79 5.79A 5.84 # AERONAUTICAL RADIONAVIGATION # Aeronautical mobile # Land mobile | # No constraints |
| # 1606.5 – 1625 kHz | FIXED MARITIME MOBILE 5.90 LAND MOBILE 5.92 | ## 1605 – 1625 kHz # BROADCASTING 5.89 5.90 |  | # FIXED # MOBILE # RADIOLOCATION # RADIONAVIGATION 5.91 | # Region 1 only and subject to restrictions on permitted coverage area |
| # 1635 – 1705 kHz | FIXED MARITIME MOBILE 5.90 LAND MOBILE 5.92 5.96 | # FIXED # MOBILE # BROADCASTING 5.89 # Radiolocation 5.90 |  | # FIXED # MOBILE # RADIOLOCATION # RADIONAVIGATION 5.91 | # Region 1 only and subject to restrictions on permitted coverage area |
| # 1705 – 1800 kHz | FIXED MARITIME MOBILE 5.90 LAND MOBILE 5.92 5.96 | # FIXED # MOBILE # RADIOLOCATION # AERONAUTICAL RADIONAVIGATION |  | # FIXED # MOBILE # RADIOLOCATION # RADIONAVIGATION 5.91 | # Region 1 only and subject to restrictions on permitted coverage area |
| # 2045 – 2065 kHz | FIXED MARITIME MOBILE LAND MOBILE 5.92 | # FIXED # MOBILE |  |  | # Region 1 only |
| # 2065 – 2107 kHz | FIXED MARITIME MOBILE LAND MOBILE 5.92 | # MARITIME MOBILE 5.105 5.106 |  |  | # No constraints |
| # 2107 – 2160 kHz | FIXED MARITIME MOBILE LAND MOBILE 5.92 | # FIXED # MOBILE |  |  | # Region 1 only |
| # 2170 – 2173.5 kHz | # MARITIME MOBILE |  |  |  | # No constraints |
| # 2173.5 – 2190.5 kHz | # MOBILE (distress and calling) # 5.108 5.109 5.110 5.111 |  |  |  | # Distress and calling |
| # 2190.5 – 2194 kHz | # MARITIME MOBILE |  |  |  | # No constraints |
| # 2625 – 2650 kHz | MARITIME MOBILE MARITIME RADIONAVIGATION 5.92 | # FIXED # MOBILE |  |  | # Region 1 only |
| # 4000 – 4063 kHz | # FIXED # MARITIME MOBILE 5.127 5.126 |  |  |  | # No constraints |
| # 4063 – 4438 kHz | # MARITIME MOBILE 5.79A 5.109 5.110 5.130 5.131 5.132 5.128 |  |  |  | # No constraints |
| # 6200 – 6525 kHz | # MARITIME MOBILE 5.109 5.110 5.130 5.132 5.137 |  |  |  | # No constraints |
| # 8100 – 8195 kHz | # FIXED # MARITIME MOBILE |  |  |  | # No constraints |
| # 8195 – 8815 kHz | # MARITIME MOBILE 5.109 5.110 5.132 5.145 5.111 |  |  |  | # No constraints |
| # 12230 – 13200 kHz | # MARITIME MOBILE 5.109 5.110 5.132 5.145 |  |  |  | # No constraints |
| # 16360 – 17410 kHz | # MARITIME MOBILE 5.109 5.110 5.132 5.145 |  |  |  | # No constraints |
| # 18780 – 18900 kHz | # MARITIME MOBILE |  |  |  | # No constraints |
| # 19680 – 19800 kHz | # MARITIME MOBILE 5.132 |  |  |  | # No constraints |
| # 22000 – 22855 kHz | # MARITIME MOBILE 5.132 5.156 |  |  |  | # No constraints |
| # 25070 – 25210 kHz | # MARITIME MOBILE |  |  |  | # No constraints |
| # 26100 – 26175 kHz | # MARITIME MOBILE 5.132 |  |  |  | # No constraints |
| # 154 – 156.4875 MHz | # FIXED # MOBILE except aeronautical mobile (R) 5.226 5.A114 | # FIXED # MOBILE 5.226 | # FIXED # MOBILE 5.226 5.A114 |  | # Operation protected by footnote 5.226 |
| # 156.4875 – 156.5625 MHz # (CH10, 70, 11) | # MARITIME MOBILE (distress and calling via DSC) ###### 5.111 5.226 5.227 |  |  |  | # No constraints for CH70 # CH10 and CH11 are protected by footnote 5.227 |
| # 156.5625 – 156.7625 MHz | # FIXED # MOBILE except aeronautical mobile (R) 5.226 | # FIXED # MOBILE 5.225 5.226 |  |  | # Operation protected by footnote 5.226 |
| # 156.7625 – 156.7875 MHz # (CH75) | # MARITIME MOBILE # Mobile-satellite (Earth-to-space) # 5.111 5.226 5.G110 # (Ch 75) | # MARITIME MOBILE # MOBILE-SATELLITE (Earth-to-space) # 5.111 5.226 5.G110 # (Ch 75) |  | # MARITIME MOBILE # Mobile-satellite (Earth-to-space) # 5.111 5.226 5.G110 # (Ch 75) | # Operation protected by footnote 5.226 |
| # 156.7875 – 156.8125 MHz # (CH16) | # MARITIME MOBILE (distress and calling) 5.111 5.226 |  |  |  | # No constraints |
| # 156.8125 – 156.8375 MHz # (CH76) | # MARITIME MOBILE # Mobile-satellite (Earth-to-space) 5.111 5.226 5.G110 (Ch 76) | # MARITIME MOBILE # MOBILE-SATELLITE (Earth-to-space) 5.111 5.226 5.G110 |  | # MARITIME MOBILE # Mobile-satellite (Earth-to-space) 5.111 5.226 5.G110 | # Operation protected by footnote 5.226 |
| # 156.8375 - 161.9625 MHz | # FIXED # MOBILE except aeronauticalmobile 5.226 | FIXED MOBILE 5.226 |  |  | # Operation protected by footnote 5.226 |
| # 161.9375-161.9625 MHz | # FIXED # MOBILE except aeronautical mobile # Maritime mobile-satellite (Earth-to-space) 5.228AA # 5.226 | FIXED MOBILE Maritime mobile-satellite (Earth-to-space) 5.228AA 5.226 |  |  | # Operation protected by footnote 5.226 |
| # 161.9625 - 161.9875 MHz # (AIS 1) | # FIXED # MOBILE except aeronautical mobile # Mobile-satellite (Earth-to-space) 5.F110 5.226 5.A110 5.B110 | # AERONAUTICAL MOBILE (OR) # MARITIME MOBILE # MOBILE-SATELITE (Earth-to-space) 5.C110 5.D110 |  | # MARITIME MOBILE # Aeronautical mobile (OR) 5.E110 # Mobile-satellite (Earth-to-space) 5.F110 5.226 | # No constraints in Region 2. # Operation protected by footnote 5.226 in Regions 1 and 3 |
| # 161.9875 - 162.0125 MHz | # FIXED # MOBILE except aeronautical mobile Maritime mobile-satellite (Earth-to-space) 5.228AA 5.226 5.229 | FIXED MOBILE Maritime mobile-satellite (Earth-to-space) 5.228AA 5.226 |  |  | # Operation protected by footnote 5.226 |
| # 162.0125 - 162.0375 MHz # (AIS 2) | # FIXED # MOBILE except aeronautical mobile # Mobile-satellite (Earth-to-space) 5.F110 5.226 5.229 5.A110 5.B110 | # AERONAUTICAL MOBILE (OR) # MARITIME MOBILE # MOBILE-SATELITE (Earth-to-space) 5.C110 5.D110 |  | # MARITIME MOBILE # Aeronautical mobile (OR) 5.E110 # Mobile-satellite (Earth-to-space) 5.F110 5.226 | # No constraints in Region 2. # Operation protected by footnote 5.226 in Regions 1 and 3 |
| # 162.0375 - 174 MHz | # FIXED # MOBILE except aeronautical mobile 5.226 5.229 | FIXED MOBILE 5.226 5.230 5.231 5.232 |  |  | # Operation protected by footnote 5.226 |
| # 406 – 406.1 MHz | # MOBILE-SATELLITE (Earth-to-space) # 5.265 5.266 5.267 |  |  |  | # No constraints |
| # 456 – 459 MHz | # FIXED # MOBILE 5.286AA 5.271 5.287 5.288 |  |  |  | # Footnote 5.287 specifies maritime mobile use |
| # 460 – 470 MHz | # FIXED # MOBILE 5.286AA # Meteorological-satellite (space-to-Earth) 5.287 5.288 5.289 5.290 |  |  |  | # Footnote 5.287 specifies maritime mobile use |
| # 1525 – 1530 MHz | # SPACE OPERATION (space-to-Earth) # FIXED # MOBILE-SATELLITE (space-to-Earth) 5.208B 5.351A # Earth exploration-satellite # Mobile except aeronautical mobile 5.349 5.341 5.342 5.350 5.351 5.352A 5.354 | # SPACE OPERATION (space-to-Earth) # MOBILE-SATELLITE (space-to-Earth) 5.208B 5.351A # Earth exploration-satellite # Fixed # Mobile 5.343 5.341 5.351 5.354 |  | # SPACE OPERATION (space-to-Earth) # FIXED # MOBILE-SATELLITE (space-to-Earth) 5.208B 5.351A # Earth exploration-satellite # Mobile 5.349 5.341 5.351 5.352A 5.354 | # Complex |
| # 1530 – 1535 MHz | # SPACE OPERATION (space-to-Earth) # MOBILE-SATELLITE (space-to-Earth) 5.208B 5.351A 5.353A # Earth exploration-satellite # Fixed # Mobile except aeronautical mobile 5.341 5.342 5.351 5.354 | SPACE OPERATION (space-to-Earth) MOBILE-SATELLITE (space-to-Earth) 5.208B 5.351A 5.353A Earth exploration-satellite Fixed Mobile 5.343 5.341 5.351 5.354 |  |  | # Complex |
| # 1535 – 1559 MHz | # MOBILE-SATELLITE (space-to-Earth) 5.208B 5.351A 5.341 5.351 5.353A 5.354 5.355 5.356 5.357 5.357A 5.359 5.362A |  |  |  | # Complex |
| # 1616 – 1610.6 MHz | # MOBILE-SATELLITE (Earth-to-space) 5.351A # AERONAUTICAL RADIONAVIGATION 5.341 5.355 5.359 5.364 5.366 5.367 5.368 5.369 5.371 5.372 | # MOBILE-SATELLITE (Earth-to-space) 5.351A # AERONAUTICAL RADIONAVIGATION # RADIODETERMINATION- SATELLITE (Earth-to-space) ### 5.341 5.364 5.366 5.367 5.368 5.370 5.372 |  | # MOBILE-SATELLITE (Earth-to-space) 5.351A # AERONAUTICAL RADIONAVIGATION # Radiodetermination-satellite (Earth-to-space) ### 5.341 5.355 5.359 5.364 5.366 5.367 5.368 5.369 5.372 | # Complex |
| # 1610.6 – 1613.8 MHz | # MOBILE=SATELLITE (Earth-to-space) 5.351A # RADIO ASTRONOMY # AERONAUTICAL RADIONAVIGATION # 5.149 5.341 5.355 5.359 5.364 5.366 5.367 5.368 5.369 5.371 5.372 | # MOBILE=SATELLITE (Earth-to-space) 5.351A # RADIO ASTRONOMY # AERONAUTICAL RADIONAVIGATION # RADIODETERMINATION-SATELLITE (Earth-to-space) # 5.149 5.341 5.364 5.366 5.367 5.368 5.370 5.372 |  | # MOBILE=SATELLITE (Earth-to-space) 5.351A # RADIO ASTRONOMY # AERONAUTICAL RADIONAVIGATION # Radiodetermination-satellite (Earth-to-space) # 5.149 5.341 5.355 5.359 5.364 5.366 5.367 5.368 5.369 5.372 | # Complex |
| # 1613.8 – 1626.5 MHz | # MOBILE=SATELLITE (Earth-to-space) 5.351A # AERONAUTICAL RADIONAVIGATION # Mobile-satellite (space-to-Earth) 5.208B # 5.341 5.355 5.359 5.364 5.365 5.366 5.367 5.368 5.369 5.371 5.372 | # MOBILE=SATELLITE (Earth-to-space) 5.351A # AERONAUTICAL RADIONAVIGATION # RADIODETERMINATION-SATELLITE (Earth-to-space) # Mobile-satellite (space-to-Earth) 5.208B # 5.341 5.364 5.365 5.366 5.367 5.368 5.370 5.372 |  | # MOBILE=SATELLITE (Earth-to-space) 5.351A # AERONAUTICAL RADIONAVIGATION # Mobile-satellite (space-to-Earth) 5.208B # Radiodetermination-satellite (Earth-to-space) # 5.341 5.355 5.359 5.364 5.365 5.366 5.367 5.368 5.369 5.372 | # Complex |
| # 1626.5 – 1660 MHz | # MOBILE-SATELLITE (Earth-to-space) 5.351A ### 5.341 5.351 5.353A 5.354 5.355 5.357A 5.359 5.362A 5.374 5.375 5.376 |  |  |  | # Complex |
| # 2900 – 3100 MHz | # RADIOLOCATION 5.424A # RADIONAVIGATION 5.426 ### 5.425 5.427 |  |  |  | # No constraints |
| # 9200 – 9300 MHz | EARTH EXPLORATION-SATELLITE (active) 5.474A 5.474B 5.474C # RADIOLOCATION # MARITIME RADIONAVIGATION 5.472 ### 5.473 5.474 5.474D |  |  |  | # No constraints |
| # 9300 – 9500 MHz | # RADIONAVIGATION # EARTH EXPLORATION-SATELLITE (active) # SPACE RESEARCH (active) # RADIOLOCATION ### 5.427 5.474 5.475 5.475A 5.475B 5.476A |  |  |  | # No constraints |

# Annex C – Current and Evolving Wireless Telecommunications Technologies in the Maritime Environment

There is an ever increasing number of wireless technologies that aim to deliver voice and data connectivity to mobile users and which may be of use in the maritime environment.  Aside from their technical capabilities, there may be regulatory issues and associated technical restrictions which prevent them being fully exploited.  Such issues may include:

the ITU Radio Regulations do not permit maritime mobile operation in the frequency bands concerned;

frequencies are licensed on a national basis, such that there is no continuity of operation from country to country (this may even require equipment to be switched off when entering certain jurisdictions);

the system parameters may have the potential to cause interference to ship-borne equipment;

planning parameters may make use at sea (or even in ports) complex or difficult.

The following terrestrial technologies are currently in the process of either being rolled-out or standardized on an international basis and thus may be candidates for use in the maritime environment, particularly for commercial port services:

digital PMR (to replace analogue PMR for on-board communications): TETRA, TETRAPOL, P25, dPMR, DMR, TDMA;

3G Mobile: UMTS (TDD & FDD), cdma2000, TD-SCDMA, IEEE802.20 (including iBurst), HSDPA;

3.5G Mobile: WiMax;

4G Mobile: LTE, UMB.

In addition to these commercial service technologies, there are various technologies which may offer potential solutions under the banner of ‘license exempt’ or ‘low-power’ technologies such as Wi-Fi, Bluetooth, ZigBee and UWB.  The range of such services, however, is exceptionally limited (normally to 100 meters or much less) and the frequencies employed are globally allocated for the purpose (e.g. the 2.4 GHz ISM band) such that there is no need for a specific consideration of the impact on spectrum of their usage in a maritime environment.

Table 8 details, for each of the commercial technologies, the frequency band(s) in which they ‘prefer’ to operate, i.e. those for which there are known services, as well as the general range of frequencies over which they are specified to operate.

Table 8 - Summary of Spectrum Requirements of New Commercial Data Links

| Technology | Frequency Range | Notes |
| --- | --- | --- |
| TETRA | # 380 – 400 MHz 410 – 430 MHz 450 – 470 MHz 806 – 821 // 851 – 866 MHz | # EN 300 392 |
| # TETRA 2 (TEDS) |  | # EN 302 561 |
| # TETRAPOL | # 70 – 933 MHz |  |
| # P25 | # 136 – 870 MHz |  |
| # DMR | # 30 – 900 MHz | # TS 102 361 |
| # dPMR | # 30 – 900 MHz | # EN 301 166 |
| # GSM (incl EDGE) | # 380 – 400 MHz 410 – 430 MHz 450 – 470 MHz 478 – 496 MHz 698 – 746 MHz 747 – 792 MHz 806 – 866 MHz 824 – 894 MHz 870 – 921 MHz 876 – 925 MHz 880 – 960 MHz 1710 – 1880 MHz 1850 – 1990 MHz |  |
| # CDMA2000 | # 450 – 470 MHz 790 – 862 MHz 824 – 894 MHz 880 – 960 MHz 1710 – 1880 MHz 1850 – 1990 MHz 1820 – 2170 MHz 2300 – 2400 MHz 2500 – 2690 MHz |  |
| # W-CDMA (UMTS) |  |  |
| # TD-CDMA | # 1900 – 1920 MHz 2010 – 2025 MHz (2570 – 2620 MHz) | # ‘TDtv’ applications have been trialed in some EU countries |
| # TD-SCDMA |  |  |
| # WiMax | # 2 – 11 GHz 10 – 66 GHz | # 3400 – 3600 MHz is being seen as a potential target following WRC-07 and EU WAPECS moves |
| # 802.20 (MBWA) | # Below 3.5 GHz |  |
| # WiBro | # 2.3 – 2.4 GHz | # Currently Korea only |
| # iBurst (HC-SDMA) | # Below 3.5 GHz | # Often uses 1785 – 1805 MHz or 1900 – 1920 MHz |

LTE and UMB are 4G extensions to UMTS and CDMA2000 respectively.  At present, it is envisaged that existing spectrum assignments will be re-farmed such that no new spectrum will be necessary.

From the above, the bands of particular interest, and where there are most likely to be mobile communication systems providing value in maritime applications are:

380 to 400 MHz;

410 to 430 MHz;

450 to 470 MHz;

790 to 862 MHz (the ‘Digital Dividend’);

824 to 849 // 869 to 894 MHz (US ‘800 MHz’ cellular band);

880 to 915 // 925 to 960 MHz (EU ‘900 MHz’ cellular band);

1710 to 1785 // 1815 to 1880 MHz (EU ‘1800 MHz’ cellular band);

1850 to 1910 // 1930 to 1990 MHz (US ‘1900 MHz’ cellular band);

1880 to 1920 MHz (EU ‘3G TDD’ band);

1920 to 1980 // 2110 to 2170 MHz (EU ‘3G’ band);

2010 to 2015 MHz (EU ‘3G TDD’ band);

2400 to 2483.5 MHz (‘2.4 GHz’ license exempt, low-power band);

2500 to 2690 MHz (‘3G expansion’ band);

3400 to 3600 MHz

Table 9 shows, based on the latest version of the ITU Radio Regulations, the allocations in those bands (including any footnotes), highlighting the position of maritime services in those bands.

Table 9 - Summary of Maritime Radio Spectrum

| Frequency (MHz) | Region 1 Allocation | Region 2 Allocation | Region 3 Allocation | Status of Maritime Use |
| --- | --- | --- | --- | --- |
| 380 – 387 | FIXED MOBILE 5.254 |  |  | # No constraints |
| # 387 – 390 | FIXED MOBILE Mobile-satellite (space-to-Earth) 5.208A 5.208B 5.254 5.255 |  |  | # No constraints |
| # 390 – 399.9 | FIXED MOBILE 5.254 |  |  | # No constraints |
| # 410 – 420 | FIXED MOBILE except aeronautical mobile SPACE RESEARCH (space-to-space) 5.268 |  |  | # No constraints |
| # 420 – 430 | FIXED MOBILE except aeronautical mobile Radiolocation 5.269 5.270 5.271 |  |  | # No constraints |
| # 450 – 455 | FIXED MOBILE 5.286 AA 5.209 5.271 5.286 5.286A 5.286B 5.286C 5.286D 5.286 E |  |  | # No constraints |
| 455 – 456 | # FIXED # MOBILE 5.286AA 5.209 5.271 5.286A 5.286B 5.286C 5.286 E | # FIXED # MOBILE 5.286AA # MOBILE-SATELLITE (Earth-to-space) 5.286A 5.286B 5.286C 5.209 | # FIXED # MOBILE 5.286AA 5.209 5.271 5.286A 5.286B 5.286C 5.286 E | # No constraints |
| # 456 – 459 | FIXED MOBILE 5.286AA 5.271 5.287 5.288 |  |  | # No constraints |
| # 459 – 460 | # FIXED # MOBILE 5.286AA 5.209 5.271 5.286A 5.286B 5.286C 5.286 E | # FIXED # MOBILE 5.286AA # MOBILE-SATELLITE (Earth-to-space) 5.286A 5.286B 5.286C 5.209 | # FIXED # MOBILE 5.286AA 5.209 5.271 5.286A 5.286B 5.286C 5.286 E | # No constraints |
| # 460 – 470 | FIXED MOBILE 5.286AA Meteorological-satellite (space-to-Earth) 5.287 5.288 5.289 5.290 |  |  | # No constraints |
| # 790 – 806 | # FIXED # BROADCASTING # MOBILE except aeronautical mobile 5.316B 5.317A 5.312 5.314 5.315 5.316 5.316A 5.319 | # BROADCASTING # Fixed # MOBILE 5.313B 5.317A 5.293 5.309 5.311A | # FIXED # MOBILE 5.313A 5.317A # BROADCASTING 5.149 5.305 5.306 5.307 5.311A 5.320 | # No constraints |
| # 806 – 862 | # FIXED # BROADCASTING # MOBILE except aeronautical mobile 5.316B 5.317A 5.312 5.314 5.315 5.316 5.316A 5.319 | # FIXED # MOBILE 5.317A # BROADCASTING 5.317 5.318 | # FIXED # MOBILE 5.313A 5.317A # BROADCASTING 5.149 5.305 5.306 5.307 5.311A 5.320 | # No constraints |
| # 862 – 890 | # FIXED # MOBILE except aeronautical mobile 5.317A # BROADCASTING 5.322 5.319 5.323 | # FIXED # MOBILE 5.317A # BROADCASTING 5.317 5.318 | # FIXED # MOBILE 5.313A 5.317A # BROADCASTING 5.149 5.305 5.306 5.307 5.311A 5.320 | # No constraints |
| # 890 – 902 | # FIXED # MOBILE except aeronautical mobile 5.317A # BROADCASTING 5.322 Radiolocation 5.323 | # FIXED # MOBILE except aeronautical mobile 5.317A # Radiolocation 5.318 5.325 | # FIXED # MOBILE 5.317A # BROADCASTING # Radiolocation 5.327 | # No constraints |
| # 902 – 928 | # FIXED # MOBILE except aeronautical mobile 5.317A # BROADCASTING 5.322 # Radiolocation 5.323 | # FIXED # Amateur # MOBILE except aeronautical mobile 5.325A # Radiolocation 5.150 5.325 5.326 | # FIXED # MOBILE 5.317A # BROADCASTING # Radiolocation 5.327 | # No constraints |
| # 928 – 942 | # FIXED # MOBILE except aeronautical mobile 5.317A # BROADCASTING 5.322 # Radiolocation 5.323 | # FIXED # MOBILE except aeronautical mobile 5.317A # Radiolocation 5.325 | # FIXED # MOBILE 5.317A # BROADCASTING # Radiolocation 5.327 | # No constraints |
| # 942 – 960 | # FIXED # MOBILE except aeronautical mobile 5.317A # BROADCASTING 5.322 5.323 | # FIXED # MOBILE 5.317A | # FIXED # MOBILE 5.317A # BROADCASTING 5.320 | # No constraints |
| # 1710 – 1930 | FIXED MOBILE 5.384A 5.388A 5.388B 5.149 5.341 5.385 5.386 5.387 5.388 |  |  | # No constraints |
| # 1930 – 1970 | # FIXED # MOBILE 5.388A 5.388B 5.388 | # FIXED # MOBILE 5.388A 5.388B # Mobile-satellite (Earth-to-space) 5.388 | # FIXED # MOBILE 5.388A 5.388B 5.388 | # No constraints |
| # 1970 – 1980 | FIXED MOBILE 5.388A 5.388B 5.388 |  |  | # No constraints |
| # 1980 – 2010 | FIXED MOBILE MOBILE-SATELLITE (Earth-to-space) 5.351A 5.388 5.389A 5.389B 5.389F |  |  | # No constraints |
| # 2010 – 2015 | # FIXED # MOBILE 5.388A 5.388B 5.388 | # FIXED # MOBILE # MOBILE-SATELLITE # (Earth-to-space) 5.388 5.389C 5.389E | # FIXED # MOBILE 5.388A 5.388B 5.388 | # No constraints |
| # 2110 – 2120 | FIXED MOBILE 5.388A 5.388B SPACE RESEARCH (deep space) (Earth-to-space) 5.388 |  |  | # No constraints |
| # 2120 – 2160 | # FIXED # MOBILE 5.388A 5.388B 5.388 | # FIXED # MOBILE 5.388A 5.388B # Mobile-satellite (space-to-Earth) 5.388 | # FIXED # MOBILE 5.388A 5.388B 5.388 | # No constraints |
| # 2160 – 2170 | # FIXED # MOBILE 5.388A 5.388B 5.388 | # FIXED # MOBILE # MOBILE-SATELLITE (space-to-Earth) 5.388 5.389C 5.389E | # FIXED # MOBILE 5.388A 5.388B 5.388 | # No constraints |
| # 2400 – 2450 | # FIXED # MOBILE 5.384A # Amateur # Radiolocation 5.150 5.282 5.395 | # FIXED # MOBILE 5.384A # RADIOLOCATION # Amateur 5.150 5.282 5.393 5.394 5.396 |  | # No constraints |
| # 2450 – 2483.5 | # FIXED # MOBILE # Radiolocation 5.150 | # FIXED # MOBILE # RADIOLOCATION 5.150 |  | # No constraints |
| # 2500 – 2520 | # FIXED 5.410 # MOBILE except aeronautical mobile 5.384A 5.412 | # FIXED 5.410 # FIXED-SATELLITE (space-to-Earth) 5.415 # MOBILE except aeronautical mobile 5.384A 5.404 | # FIXED 5.410 # FIXED-SATELLITE (space-to-Earth) 5.415 # MOBILE except aeronautical mobile 5.384A # MOBILE-SATELLITE (space-to-Earth) 5.351A 5.407 5.414 5.414A 5.404 5.415A | # No constraints |
| # 2520 – 2535 | # FIXED 5.410 # MOBILE except aeronautical mobile 5.384A # BROADCASTING-SATELLITE 5.413 5.416 5.339 5.405 5.412 5.417C 5.417D 5.418B 5.418C | # FIXED 5.410 # FIXED-SATELLITE (space-to-Earth) 5.415 # MOBILE except aeronautical mobile 5.384A # BROADCASTING-SATELLITE 5.413 5.416 5.339 5.417C 5.417D 5.418B 5.418C | # FIXED 5.410 # FIXED-SATELLITE (space-to-Earth) 5.415 # MOBILE except aeronautical mobile 5.384A # BROADCASTING-SATELLITE 5.413 5.416 5.403 5.414A 5.415A | # No constraints |
| # 2535 – 2655 | # FIXED 5.410 # MOBILE except aeronautical mobile 5.384A # BROADCASTING-SATELLITE 5.413 5.416 5.339 5.412 5.417C 5.417D 5.418B 5.418C | # FIXED 5.410 # FIXED-SATELLITE (space-to-Earth) 5.415 # MOBILE except aeronautical mobile 5.384A # BROADCASTING-SATELLITE 5.413 5.416 5.339 5.417C 5.417D 5.418B 5.418C | # FIXED 5.410 # MOBILE except aeronautical mobile 5.384A # BROADCASTING-SATELLITE 5.413 5.416 5.339 5.417A 5.417B 5.417C 5.417D 5.418 5.418A 5.418B 5.418C | # No constraints |
| # 2655 – 2670 | # FIXED 5.410 # MOBILE except aeronautical mobile 5.384A # BROADCASTING-SATELLITE 5.208B 5.413 5.416 # Earth exploration-satellite (passive) # Radio astronomy # Space research (passive) 5.149 5.412 | # FIXED 5.410 # FIXED-SATELLITE (Earth-to-space) (space-to-Earth) 5.415 # MOBILE except aeronautical mobile 5.384A # BROADCASTING-SATELLITE 5.413 5.416 # Earth exploration-satellite (passive) # Radio astronomy # Space research (passive) 5.149 5.208B | # FIXED 5.410 # FIXED-SATELLITE (Earth-to-space) 5.415 # MOBILE except aeronautical mobile 5.384A # BROADCASTING-SATELLITE 5.413 5.416 # Earth exploration-satellite (passive) # Radio astronomy # Space research (passive) 5.149 5.208B 5.420 | # No constraints |
| # 2670 – 2690 | # FIXED 5.410 # MOBILE except aeronautical mobile 5.384A # Earth exploration-satellite (passive) # Radio astronomy # Space research (passive) 5.149 5.412 | # FIXED 5.410 # FIXED-SATELLITE (Earth-to-space) (space-to-Earth) 5.208B 5.415 # MOBILE except aeronautical mobile 5.384A # Earth exploration-satellite (passive) # Radio astronomy # Space research (passive) 5.149 | # FIXED 5.410 # FIXED-SATELLITE (Earth-to-space) 5.415 # MOBILE except aeronautical mobile 5.384A # MOBILE-SATELLITE (Earth-to-space) 5.351A 5.419 # Earth exploration-satellite (passive) # Radio astronomy # Space research (passive) 5.149 | # No constraints |
| # 3400 – 3500 | # FIXED # FIXED-SATELLITE (space-to-Earth) # Mobile 5.430A # Radiolocation 5.431 | # FIXED # FIXED-SATELLITE (space-to-Earth) # Amateur # Mobile 5.431A # Radiolocation 5.433 5.282 | # FIXED # FIXED-SATELLITE (space-to-Earth) # Amateur # Mobile 5.432B # Radiolocation 5.433 5.282 5.432 5.432A | # Secondary |
| # 3500 – 3600 | # FIXED # FIXED-SATELLITE (space-to-Earth) # Mobile 5.430A # Radiolocation 5.431 | # FIXED # FIXED-SATELLITE (space-to-Earth) # MOBILE except aeronautical mobile # Radiolocation 5.433 | # FIXED # FIXED-SATELLITE (space-to-Earth) # MOBILE except aeronautical mobile 5.433A # Radiolocation 5.433 | # Secondary in Region 1 |

None of the footnotes relating to the use of these bands specifically preclude any use for maritime services.

With the exception of the band 3400 – 3600 MHz, the mobile service (which includes, unless explicitly stated, the maritime mobile service) has a primary allocation meaning that the use of these bands for maritime applications will pose no regulatory issues at the ITU.  In the band 3400 – 3600 MHz, many of the footnotes relate to the upgrading of the mobile service from secondary to primary status, however it is known that in some parts of the world (especially in high rainfall areas such as around the equator) this band is still heavily used for satellite downlinks and as such its development for mobile services is likely to remain constrained.

Conclusions

There does not appear to be any necessity to expend significant effort on the standardization or regulation of these mobile communication technologies to make them suitable for the maritime community.

# Annex D – Development of New Characteristics

The following tables summarizes the status of the various ITU technical characteristics relating to maritime communication applications both for those already existing as well as for those foreseen in e-Navigation.  They outline where existing characteristics are likely to need updating or elsewhere new characteristics need to be developed.  If changes are required they will take place through the appropriate ITU Study Groups.

ITU Study Groups (SG) are responsible for developing and maintaining Recommendations, some of which are used as technical background for amending the frequency allocations at the periodic World Radiocommunications Conferences (WRC).  Study groups are established and assigned study questions by a Radiocommunication Assembly (RA) to prepare Recommendations for approval by ITU Member States.  In the case of e-Navigation two SG’s are likely to be involved in the development of material to be agreed at WRC, namely SG4 and SG5.

SG4 relates to satellite services, covering systems and networks for the following service:

Fixed-satellite service;

Mobile-satellite service;

Broadcasting-satellite service; and

Radiodetermination-satellite service.

SG5 relates to terrestrial services, covering systems and networks for the following services:

Fixed service;

Mobile service;

Radiodetermination service;

Amateur service; and

Amateur-satellite service.

Below the SG’s there are subgroups, such as Working Parties (WP) and Task Groups (TG) that are established to study the questions assigned to the different Study Groups. For example, SG4 has the following WP’s:

Working Party 4A - Efficient orbit/spectrum utilization for Fixed Satellite Service (FSS) and Broadcasting Satellite Service (BSS);

Working Party 4B - Systems, air interfaces, performance and availability objectives for FSS, BSS and Mobile Satellite Service (MSS), including IP-based applications and satellite news gathering;

Working Party 4C - Efficient orbit/spectrum utilization for MSS and Radiodetermination Satellite Service (RDSS);

SG5 has the following WP’s:

Working Party 5A - Land mobile service above 30 MHz excluding IMT; wireless access in the fixed service; amateur and amateur-satellite services;

Working Party 5B - Maritime mobile service including Global Maritime Distress and Safety System (GMDSS); aeronautical mobile service and radiodetermination service;

Working Party 5C - Fixed wireless systems; HF and other systems below 30 MHz in the Fixed and Land Mobile Services;

Working Party 5D - IMT Systems;

Task Group 5/1 (TG 5/1) - WRC-19 agenda item 1.13

For maritime GMDSS and e-Navigation applications any necessary revision to, or creation of, technical characteristics will take place in one of two WP’s – SG4 WP4C and SG5 WP5B.

Table 10 - ITU Recommendations and Reports for Maritime Radio Applications

| Application |  | Current characteristics |  | Amendments | Mechanism |
| --- | --- | --- | --- | --- | --- |
| DSC | # VHF | # Recommendation ITU-R M.493-14 | # Digital selective-calling system for use in the maritime mobile service. | # The revision work on Recommendation ITU-R M.493-13 is on going at WP 5B in cooperation with IMO, IALA, IEC and CIRM | # N/A |
|  |  | # Recommendation ITU-R M.541-10 | # Operational Procedures for the Use of Digital Selective-Calling Equipment in the Maritime Mobile Service |  |  |
|  |  | # Recommendation ITU-R M.693-1 | # Technical characteristics of VHF emergency position-indicating radio beacons using digital selective calling (DSC VHF EPIRB) |  |  |
|  |  | # Recommendation ITU-R M.689-3 | # International Maritime VHF Radiotelephone Systems with Automatic Facilities Based on DSC Signaling Format |  |  |
|  |  | # Recommendation ITU-R M.821-1 | # Optional Expansion of the Digital Selective-Calling System for Use in the Maritime Mobile Service |  |  |
|  | # MF/HF | # Recommendation ITU-R M.493-14 | # Digital selective-calling system for use in the maritime mobile service. | # The revision work on Recommendation ITU-R M.493-13 is on going at WP 5B in cooperation with IMO, IALA, IEC and CIRM | # N/A |
|  |  | # Recommendation ITU-R M.541-10 | # Operational Procedures for the Use of Digital Selective-Calling Equipment in the Maritime Mobile Service |  |  |
|  |  | # Recommendation ITU-R M.1173-1 | # Technical characteristics of single-sideband transmitters used in the maritime mobile service for radiotelephony in the bands between 1 606.5 kHz (1 605 kHz Region 2) and 4 000 kHz and between 4 000 kHz and 27 500 kHz |  |  |
|  |  | # Recommendation ITU-R M.1082-1 | # International maritime MF/HF radiotelephone system with automatic facilities based on DSC signaling format |  |  |
|  |  | # Recommendation ITU-R M.476-5 | # Direct-Printing Telegraph Equipment in the Maritime Mobile Service |  |  |
|  | # Satellite | # Recommendation ITU-R M.541-10 | # Operational Procedures for the Use of Digital Selective-Calling Equipment in the Maritime Mobile Service | # DSC over satellite is not currently defined explicitly through a technical characteristic. However, it could be appropriate to develop a complementary characteristic to those available for VHF and HF. | # This task should be coordinated through WP4C/SG4 under existing question 227/4. |
| LRIT | # HF |  |  | # The use of HF and Satellite data links to support the LRIT application should not need a new technical characteristic, as they are simply applications supported by existing data links. However, there may be merit in amending the existing recommendations in relation to HF and satellite communications to explicitly recognize the use of LRIT. | # The work would have to be addressed through WP4C/SG4 Questions 84-4/4 (Iridium), 87-4/4 (satellite) coordinating with WP5B/SG5 for HF. |
|  | # Satellite |  |  |  |  |
| AIS |  | # Recommendation ITU-R M.1371-5 | # Technical characteristics for an automatic identification system using time division multiple access in the VHF maritime mobile band | # WRC-12 allocated two channels for satellite detection of AIS, six channels for possible testing of future AIS applications and one channel for experimental use for future applications. | WRC-15 agreed that VHF channels 27 and 28 are each split into two simplex channels from 1 January 2019. The channels 2027 and 2028 designated as ASM 1 and ASM 2 are used for application specific messages (ASM) as described in the most recent version of Recommendation ITU-R M.2092. # . |
| VHF Comms | # Voice | # Recommendation ITU-R M.1084-5 | # Interim solutions for improved efficiency in the use of the band 156-174 MHz by stations in the maritime mobile service | # Under e-Navigation, a general push to make more efficient use of the spectrum available for maritime voice communications may take place as existing channels are increasingly allocated to digital services. # The existing characteristics (e.g. M.1084) propose possible interim solutions such as the use of 12.5 kHz channel spacing. It may be desirable to develop a new technical characteristic to describe a permanent solutions for e.g. 12.5 kHz or 8.33 kHz voice channels in the future communication environment. | # This task should be coordinated through WP5B/SG5. |
|  |  | # Recommendation ITU-R M.489-2 | # Technical characteristics of VHF radiotelephone equipment operating in the maritime mobile service in channels spaced by 25 kHz |  |  |
|  | # Data | # Recommendation ITU-R M.1842-1 | # Characteristics of VHF radio system and equipment for the exchange of data and electronic mail in the maritime mobile service RR Appendix 18 channels | # A new technical characteristic will be required to support the use of VHF channels for digital data transmission – assuming that AIS is not selected as the maritime domain datalink of choice. # The current characteristic for VHF data exchange refers to the specific solution implemented by Telenor in Norway. If this solution is accepted by the maritime community, this characteristic could be amended. However, in the more likely scenario of a new datalink technology being proposed a new characteristic will be required. # WP 5B is working on the standardization of communication protocols for full implementation of standardized VHF data communications. | # This task should be coordinated through WP5B/SG5. |
|  |  | # Recommendation ITU-R M.2092-0 | # Technical characteristics for a VHF data exchange system in the VHF maritime mobile band | # VHF data exchange system (VDES) integrates the components of VHF data exchange (VDE), application specific messages (ASM) and the automatic identification system (AIS) in the VHF maritime mobile band. This recommendation provides the technical characteristics of ASM and VDE. |  |
|  | # SAR | # Recommendation ITU-R M.489-2 | # Technical characteristics of VHF radiotelephone equipment operating in the maritime mobile service in channels spaced by 25 kHz | # No e-Nav related comms change is anticipated for the use of 121.5 MHz for short range SAR purposes. However, were the VHF channels to be revised the SAR associated characteristic (M.489) would in any event be updated. | # N/A |
| HF Comms | # Voice | # Recommendation ITU-R M.1173-1 | # Technical characteristics of single-sideband transmitters used in the maritime mobile service for radiotelephony in the bands between 1 606.5 kHz (1 605 kHz Region 2) and 4 000 kHz and between 4 000 kHz and 27 500 kHz | # No e-Nav related comms change is envisaged for HF voice services. However, if there is a broad encroachment of digital techniques into the band there may be a need to revise existing voice characteristics to accommodate data. | # N/A |
|  |  | # Recommendation ITU-R M.1082-1 | # International maritime MF/HF radiotelephone system with automatic facilities based on DSC signalling format |  |  |
|  | # Data | # Recommendation ITU-R M.1081-1 | # Automatic HF facsimile and data system for maritime mobile users | # There are a number of existing technical characteristics relating to data exchange in the HF band that include both outmoded legacy technologies as well as candidate future technologies. # There needs to be either an update to M.1798, or else a new characteristic to describe the selected HF digital data scheme to be used under e-Navigation. Currently M.1798 and F.1821 describes multi-channel HF data links such as the system currently deployed by Globe Wireless for commercial services (email, etc.). | # WRC-12 decided channelling arrangements in AP17, effecting on 1 January 2017 for the implementation of HF data communications. |
|  |  | # Recommendation ITU-R M.1798-1 | # Characteristics of HF radio equipment for the exchange of digital data and electronic mail in the maritime mobile service |  |  |
|  |  | # Recommendation ITU-R M.627-1 | # Technical characteristics for HF maritime radio equipment using narrow-band phase-shift keying (NBPSK) telegraphy |  |  |
|  |  | # Recommendation ITU-R F.1821-0 | # Characteristics of advanced digital high frequency (HF) radiocommunication systems |  |  |
| NAVTEX / NBDP | # MSI | # Recommendation ITU-R M.625-4 | # Direct-Printing Telegraph Equipment Employing Automatic Identification in the Maritime Mobile Service | # If under e-Navigation the NAVTEX data is carried across different communication links there may be a need to provide a new characteristic to accommodate it. For this purpose, WP 5B developed Recommendation M.2010. Also, WRC-12 exclusively allocated 495-505 kHz to MMS for digital broadcasting of maritime safety and security related information using M.2010. |  |
|  |  | # Recommendation ITU-R M.540-2 | # Operational and Technical Characteristics for an Automated Direct- Printing Telegraph System for Promulgation of Navigational and Meteorological Warnings and Urgent Information to Ships - Section 8B - Maritime Mobile Service; Telegraphy and Related Subjects. |  |  |
|  |  | # Recommendation ITU-R M.688-0 | # Technical characteristics for a high frequency direct-printing telegraph system for promulgation of high seas and NAVTEX-type maritime safety information |  |  |
|  |  | # Recommendation ITU-R M.492-6 | # Operational procedures for the use of direct-printing telegraph equipment in the maritime mobile service |  |  |
|  |  | # Recommendation ITU-R M.476-5 | # Direct-Printing Telegraph Equipment in the Maritime Mobile Service |  |  |
| NAVDATA | # Data | # Recommendation ITU-R M.2010-0 | # Characteristics of a digital system, named Navigational Data for broadcasting maritime safety and security related information from shore-to-ship in the 500 kHz band | # This technical characteristic may be used for e-Navigation for digital broadcasting maritime safety and security related information from shore to ship on 495-505 kHz with high data rate. | # WRC-12 allocated 495-505 kHz to the MMS exclusively for the high speed digital broadcasting, effective on 1 January 2013. |
|  |  | # Recommendation ITU-R M.2058-0 | # Characteristics of a digital system, named navigational data for broadcasting maritime safety and security related information from shore-to-ship in the maritime HF frequency band | # This technical characteristic may be used for e-Navigation for digital broadcasting maritime safety and security related information from shore to ship on 4, 6, 8, 12, 16 and 22 MHz bands with high data rate. |  |
| On board Comms | # UHF | # Recommendation ITU-R M.1174-3 | # Technical characteristics of equipment used for on-board vessel communications in the bands between 450 and 470 MHz | # WRC-15 Agenda Item 1.15 will consider spectrum demands for on-board communication and there may be a need to update the existing characteristics to support the implementation of digital PMR. | # This task should be coordinated through WP5B/SG5 most likely under a new question. |
|  | # Wi-Fi | # Report ITU-R F.2086-1 | # Technical and operational characteristics and applications of broadband wireless access in the fixed service | # If Wi-Fi stations are to be adopted for on-board data communications there may be a need for a new characteristic associated with is use in the maritime mobile domain. | # This task should be coordinated through WP5B/SG5 most likely under a new question. |
| Satellite Comms | # Broadband | # Recommendation ITU-R S.1709-1 | # Technical characteristics of air interfaces for global broadband satellite systems | # Broadband over satellite is an existing application of current satellite systems and does not warrant a specific characteristic for the maritime domain | # N/A |
|  | # MSI | # Recommendation ITU-R S.1709-1 | # Technical characteristics of air interfaces for global broadband satellite systems | # The transmission of MSI over broadband may warrant a characteristic to define the data layers. | # This task should be coordinated through WP4C/SG4 under existing question 87-4/4. |
| DGNSS |  | # Recommendation ITU-R M.823-3 | # Technical characteristics of differential transmissions for global navigation satellite systems from maritime radio beacons in the frequency band 283.5-315 kHz in Region 1 and 285-325 kHz in Regions 2 and 3 | # The current characteristic M.823-3 will ultimately need to be updated to accommodate new developments in GNSS such as Galileo and COMPASS. | # This task should be coordinated through WP5B/SG5. |
| Port Services | # WiMax / 2G / 3G / 4G |  |  | # The maritime community will make use of technologies developed and standardized for other applications in port. No maritime specific characteristics are required. | # N/A |
| Inter-Ship Broadband | # WiMax | # Recommendation ITU-R M.1801-2 | # Radio interface standards for broadband wireless access systems, including mobile and nomadic applications, in the mobile service operating below 6 GHz | # Under e-Navigation, ad-hoc mesh WiMax networks could be deployed to provide broadband coastal networks. This would require a specific form of implementation of WiMax that would necessitate a specific technical characteristic. | # This task should be coordinated through WP5B/SG5 possibly under existing questions 212/8 and 238/8. |
| Loran | # eLoran data channel | # Recommendation ITU-R M.589-3 | # Technical characteristics of methods of data transmission and interference protection for radionavigation services in the frequency bands between 70 and 130 kHz | # The existing technical characteristic will need to be revised to accommodate the definitive 9th pulse, Eurofix and/or new modulation schemes selected for the eLoran data channel. | # This task should be coordinated through WP5B/SG5 under a new question. |
| Radar | # SART | # Recommendation ITU-R M.628-5 | # Technical Characteristics for Search and Rescue Radar Transponders | # Relates to 9 GHz SART, so no change required. | # N/A |
|  | # Racon | # Recommendation ITU-R M.824-4 | # Technical parameters of radar beacons (Racons) | # No e-Nav related comms change. | # N/A |
|  | # Radio determination | # Recommendation ITU-R M.1460-2 | # Technical and operational characteristics and protection criteria of radiodetermination radars in the 2 900-3 100 MHz band | # No e-Nav related comms change. | # N/A |
| EPIRB | # VHF | # Recommendation ITU-R M.690-3 | # Technical characteristics of emergency position-indicating radio beacons (EPIRBs) operating on the carrier frequencies of 121.5 MHz and 243 MHz | # No e-Nav related comms change. | # N/A |
|  | # Satellite (COSPAS / SARSAT) | # Recommendation ITU-R M.633-4 | # Transmission characteristics of a satellite emergency position-indicating radio beacon (satellite EPIRB) system operating through a satellite system in the 406 MHz band | # No e-Nav related comms change. | # N/A |
|  | # Satellite | # Recommendation ITU-R M.632-3 | # Transmission Characteristics of a Satellite Emergency Position Indicating Radio Beacon (Satellite EPIRB) System Operating Through Geostationary Satellites in the 1.6 GHz Band | # No e-Navigation related communications change. | # N/A |

# E3	AIS Today

AIS is technically defined by Recommendation ITU-R M.1371, mandatory for SOLAS vessels and for other vessels on a regional basis, and used voluntarily;

AIS has proved to be a powerful tool for various applications in the field of navigation, and distribution of safety related information, however these applications have not yet fully been exploited;

Radio Regulations Appendix 15 (WRC-07) defines the frequencies used for GMDSS, and this includes the frequencies AIS1 and AIS 2 used by AIS-SART stations.

# E4	Increasing Use of AIS

The use of AIS is increasing rapidly, threatening to degrade the performance or to overload the current AIS frequencies AIS 1 and AIS 2.

## E4.1	Number of ships

Full implementation of SOLAS requirement for AIS is completed. Increased use of mandatory AIS on non-SOLAS vessels is evident:

i)	USA (USA commercial vessels);

ii)	Europe (EU Directive for fishing vessels requiring Class-A AIS);

iii)	Europe (EU Directive for inland vessels requiring Inland AIS (Class-A derivative));

iv)	Korea;

v)	India;

vi)	Mexico;

vii)	Australia.

Increased voluntary use of AIS:

i)	both Class-A and Class-B units used on smaller vessels including rapidly increasing numbers of pleasure vessels.

Class-B increasing use:

- Class-B (CSTDMA) visibility will eventually be reduced due to polite behaviour;

- Class-B (SOTDMA) has been introduced.

## E4.2	AIS Base Stations

Coastal and inland AIS infrastructure continues to grow.  This growth is driven by safety, security and environmental protection considerations.  The use of AIS data gathered over time is now integrated into many analysis systems both for trend analysis and to determine possible anomalous vessel behaviours.

In addition, within Europe, EU Directive 59 requires that all EU member countries maintain coastal AIS coverage.

## E4.3	Airborne AIS

More SAR aircraft are being fitted with AIS stations.

## E4.4	AIS AtoN, Real or Virtual, and AtoN Monitoring

AIS AtoN – real and virtual – are being used or approved by competent authorities.

A separate message for AtoN monitoring (often Message 6) is being used in most cases where an AIS AtoN station is deployed.

In addition, AIS AtoN stations are being deployed by commercial organizations to mark offshore platforms, wind farms, etc.

## E4.5	AIS-SART

IMO carriage option for the AIS-SART started in January 2010.  AIS-SART broadcasts eight times per minute on AIS 1 and AIS 2. :

## E4.6	SAR Related Equipment Using AIS-SART Technology

Several kinds of AIS-MOB using AIS-SART technology have been deployed on the market and this device is identified to use unique IDs as defined in Recommendation ITU-R M.585-6.

EPIRB-AIS, AIS-MOB and Diver device using AIS-SART technology are being introduced on the market.  IMO NAV58 considered the issue relating to these devices and decided that diver locating devices should not operate on the frequencies AIS 1 and AIS 2 for routine diver locating.  It was further considered that the frequencies AIS 1 and AIS 2 should only be used when a diver was in a non-routine situation, that in these cases the device was similar to a MOB device and, therefore, the parameter and appropriate message for MOB should apply.

## E4.7	Application-Specific Messages

Application specific messages are being used more on AIS1 and 2; this leads to future developments under a combined application.

WRC-15 agreed that VHF channels (duplex) 27 and 28 are to be split into two simplex channels from 1 January 2019.  The channels 2027 and 2028 have been designated as ASM 1 and ASM 2, and are to be used for application specific messages (ASM) as described in the most recent version of Recommendation ITU-R M.2092.

### E4.7.1	International

IMO SN/Circ.289 provides guidance on the use of international application specific messages. This circular retains  most of the international application-specific messages that were developed for trial and the addition of new messages were created such as;

- a new meteorological-hydrographic message with a dynamic length of 2 to 5 slots;

- an area message for navigational warnings.

### E4.7.2	Regional

There is extensive use of safety-related ASM regionally, including the use of AMSs for the monitoring of Aids to Navigation (AtoN).  In addition, there are uses of ASM within certain waterways such as the St. Lawrence Seaway and the River Thames.

IALA maintains a registry of these regional application specific messages.

### E4.7.3	Mandatory Application Specific Messaging in Certain Regions

Additional application specific messages are being used for inland AIS in Europe.  These include the development of dedicated ship stations, which have been type approved.

## E4.8	AIS Messages (23, 24, 25, 26 and 27)

Messages 23, 24, 25, 26 and 27 were added in recent editions of Recommendation ITU R M.1371;

Additional frequencies were approved at WRC12 for receiving Message 27 by Satellites (Channels 75 and 76).

## E4.9	GNSS Differential Corrections

Use of Message 17 for GNSS correction data dissemination is increasing.  Message 17 is already being broadcast in the Gulf of Finland and Tokyo Bay.  Germany, the Netherlands and England are planning the introduction of message 17.

## E4.10	FATDMA Usage

Competent authorities are increasingly using FATDMA slot allocations for various uses:

i)	AIS AtoN;

ii)	base station slot reservation broadcasts (Message 20);

iii)	base station regular broadcasts (Message 17);

iv)	reserved slots for ship replies;

v)	safety text messages;

vi)	meteorological and hydrographic messages (Message 8).

There is an indication that the VHF Data Link will become more crowded and competent authorities resort to FATDMA reservations to ensure that their services are protected.

# E5	Additional Future Use of AIS / VDES

## E5.1	Increased Use of AIS Shore to Ship Data Transmissions

In a future e-Navigation scenario, the transmission of data from shore-to-ship is expected to increase significantly with the advent of mandatory ECDIS, which provides a graphical display of information.  There could be opportunities for transmission of this data by AIS or by VDE, as part of VDES.

Examples:

- Ship off track warning;

- Territorial water advice;

- Environmental area warning;

- Navigational hazards;

- Weather warnings;

- Military test ranges and submarine exercises.

It is anticipated that some of the present voice communications between shore and ship will move to data messaging:

i)	ship reporting;

ii)	routing information.

## E5.2	Increased Use of Ship-to-Shore Data Transfer

- Ship reporting (by AIS or VDE):

- Requesting data:

i)	weather data;

ii)	docking data;

iii)	routing information.

## E5.3	Increased Use of Ship-to-Ship Data Transmissions

- Reduced use of voice communications in favour of data messages.

## E5.4	Base Stations

- There will be more fixed stations, some of these being private;

- Development of limited base stations will be driven by:

i)	private ports;

ii)	bridges and locks;

iii)	canals;

iv)	offshore commercial facilities.

## E5.5	Repeater Stations

- Repeaters will be used to extend coverage of shore AIS base station networks, ship-ship AIS communications, and AIS AtoN range:

i)	requiring double the VDL slot count to transfer data;

ii)	and are deployed to extend base station coverage when the usual base station-to- base station data links are unavailable.

VDE may assist with mitigating the impact on the VDL.

## E5.6	Future Messaging

- Will be required as the suite of Recommendation ITU-R M.1371 messages increases:

i)	Originally 22 message types, now 27 have been defined.

- Improved set of messages are expected:

i)	replacement for some old messages, when they have proved not to support proper data modelling (e.g. static ship data);

ii)	reduction of “tailored messages” to “essentially needed messages”;

iii)	new flexibility for applications.

## E5.7	Satellite Detection of AIS

- Satellite detection of AIS messages is now an accepted capability.

- Separate frequencies for satellite detection of AIS were requested from within Appendix 18 because the tuning range of the shipborne AIS Class A is limited to these frequencies. Report ITU-R M.2084 indicated that the interference environment resulting from the existing services in those bands must be taken into account in determining the feasibility of accommodating satellite AIS, due to the large satellite antenna footprint that overlaps both land and sea. Separate operating frequencies in addition to AIS 1 and AIS 2 are needed that are not subject to terrestrial use.

- Appendix 18 contains only 4 frequencies (channels 16, 70, 75 and 76) that are exclusively dedicated to maritime use (channels 75 and 76 have been approved to be shared with this service).  This proposal meets the intent of footnote n) to Appendix 18 for interference mitigation.

- Based on the above situation, WRC-12 approved the use of channels 75 and 76 for this purpose (long range AIS) and assigned these channels to the maritime mobile satellite service.

## E5.8	Future GMDSS Platform

- IMO modified the performance standard for the 406 MHz EPIRB to include use of AIS. This suggests the use of AIS, when combined with another technology, may be useful as a GMDSS locating / homing device.

- IMO has recognized AIS-SART as part of GMDSS.  The potential for additional use of AIS and VDE as a SAR tool is being considered as part of GMDSS modernization review being carried out by the IMO.

## E5.9	PLB (Personal Locating Beacon)

- There are initiatives to develop a PLB / MOB / Diver applications based on the AIS-SART technology.  These may be considered as part of the development of AMRDs.

## E5.10	Use of AIS for PNT (Position, Navigation and Timing) and Ranging

- Improved time transfer capabilities shore to ship by using high precision time bases has been proposed.

- The e-Navigation Committee envisages a “Ranging” mode for AIS and trials were undertaken in Northern Europe as part of ACCSEAS project

i)	where the timing of AIS messages are used for position determination;

ii)	by ship and shore.

## E5.11	Coordinated Channel Management for Future e-Navigation VHF Digital Data Communications

- Channel management should be relocated to other channels such as 28B (2028) and 27B (2027) protecting distress and safety messaging on channel 70 and avoiding interference from other services.

- Designation of additional AIS frequencies may enhance the coordination of regional channel management.

- Using TDMA protocol to coordinate the usage of the VHF data link will enable channel management for other e-Navigation data services as well as AIS.

- Any additional protocols on Channel 70 are required to co-exist and not interfere with current DSC requirements and operation.

# E7	Description of VHF Data Communications Requirements

## E7.1	Background

Prior to WRC-12, IALA created ITU document 5B/801 “Three essential elements of e-Navigation communications”, which originated in IALA as output document e-Nav10-output-18.

Two of these essential elements were concerned with AIS and with VHF channels for data exchange. The third of these essential elements is MF radio communications, near 500 kHz. It is not considered in this ANNEX, which is restricted to VHF matters.

The objective of this document 5B/801 was to secure at WRC-12 additional AIS channels for satellite detection, and additional VHF channels to relieve the loading on AIS1 and AIS2 with the objective of optimizing the use of AIS1 and AIS2 for their original purpose. The result of WRC-12 was in accordance with these objectives.

## E7.2	VHF Data Communications

VHF data communications will provide robust high-speed/large volume data exchange between ships and between ship and shore. The AIS system is not capable of handling, nor is intended for, this high-speed/large volume data exchange.

Taking into account the channels identified by WRC-12 as described above, channels channels 80, 21, 81, 22, 82, 23, 83, 24, 84, 25, 85, 26, and 86 will use the modulation technique described in ITU-R M.1842-1, and WRC-19 agreed channels 24, 84, 25, 85, 26, and 86 will use the modulation technique described in Recommendation ITU-R M.2092-0 rather than Recommendation ITU-R M.1842-1..

These may be used as discrete data communications channels, or a number may be combined into a single wide-bandwidth channel.

From 1 January 2019, the channels 24, 84, 25 and 85 may be merged in order to form a unique duplex channel with a bandwidth of 100 kHz in order to operate the VDES terrestrial component described in the most recent version of Recommendation ITU R M.2092.

All VHF data communication channels are illustrated in the Figure 2.

- Figure 2 – VHF Data Communications Channel

## E7.3	AIS and VDE Channel Plan

- Safety of Navigation purposes

i)	The AIS 1 and AIS2 are internationally allocated on a sharing basis. However, WRC-12 allocated these frequencies exclusive to maritime mobile service in both Region 2 effective in 2025 and Region 3 effective in 2013, but not in Region 1 (sharing basis continues).

- Satellite detection of AIS and future GMDSS

i)	VHF Channels 75 and 76, dedicated maritime frequencies, have been approved for monitoring from space of Message 27, sharing with low power devices for navigational related communications only.

ii)	in the future GMDSS could include AIS technologies. The distress alerting, urgency and safety communications may be by both terrestrial and satellite communications; therefore exclusive maritime mobile service frequencies are needed. The satellite detection of AIS is a one way system (Earth-to-space) and could be considered as an additional alerting means. Methods of acknowledgement via alternative return links may also be considered but will require RR modifications by future WRC.

- Data communication purposes.

In considering the future AIS system for terrestrial (non-satellite) low volume data communication purposes, an additional two frequencies will be needed.  These frequencies have already been selected within the Appendix 18.  These are shown in Table 10.

Also, the Appendix 18 covers 156-162.025 MHz; however the current channeling arrangements only apply to limited parts of the band.  As shown in the figure below, the blue bands (sharing basis) and red bands (maritime exclusive) are channelized. The yellow bands (sharing basis) are not channelized.  The yellow bands are available for maritime mobile as well as terrestrial services and candidate frequencies may be selected on a sharing basis.

- Figure 3 - Appendix 18 to the ITU Radio Regulations

Table 11 - VHF Data Exchange System (VDES)

|  | VHF Data Exchange System (Recommendation ITU-R M.2092-0) |  | VHF Data Exchange System (Recommendation ITU-R M.1371-5) |  |
| --- | --- | --- | --- | --- |
| Sub-group | Application Specific Message (ASM) | VHF Data Exchange (VDE) | AIS for safety of navigation | AIS long range |
| Radio channels | - Channels ASM 1 and ASM 2 - World-wide dedicated channels (WRC-15 agreed, including Sat uplink) | - Channels 24,84, 25, 85, 26, 86 (WRC-15 agreed VDE-TER; target VDE-SAT WRC-19) | - AIS-1 & AIS-2 (simplex) | - Channels 75 and 76 (simplex) WRC-12 |
| Functionality | - Marine safety information - Marine security information - SSRMs - General purpose information communication | - General purpose data exchange - Robust high speed data exchange | - Safety of navigation - Maritime and inland distress and safety communications (Subject to inclusion in GMDSS Modernization by IMO) | - Space detection of AIS - Future SAR |
| Message types for AIS protocol | - IMO SN.1/ Circ.289 international application specific messages - Regional application specific messages - Base Station |  | - Vessel identification - Vessel dynamic data - Vessel static data - Voyage related data - Aids to Navigation - Base Station | - Space detection of AIS - Other messages for support of future SAR |
| Sub functionality | - Area warnings and advice - Meteorological and hydrological data - Traffic management - Ship-shore data exchange - Channel management | - High message payload | - Ship to ship collision avoidance - VTS - Tracking of ships - Locating in SAR - VDL control (by Base Station) | - Detection of vessels by coastal states beyond range of coastal AIS base stations - Future distress alerting (Subject to inclusion in GMDSS Modernization by IMO) |

‘VDES provides a summary of the technical assignment of various VHF channels for communication including protocol and types of messages to meet the functionality required by user needs.

## E7.4	VDE, ASM and their Relationship with AIS

To summarize the VDE plan and its relationship with AIS:

VHF Data Exchange (VDE)

  - The four contiguous channels 25, 85, 26, 86 will be used for data exchange using the modulation technique described in ITU-R M.1842-1 Annex 4:

    - These may be used as separate channels or combined into a single 100 KHz broadband channel.

  - The two contiguous channels 24 and 84 may also be used for data exchange along the coastlines and waterways using the modulation techniques described in ITU-R M.1842-1 Annex 1 or Annex 3.

Application Specific Message (ASM)

  - The duplex channels 27 and 28 are each split into two simplex channels from 1 January 2019.  The channels 2027 and 2028 designated as ASM 1 and ASM 2 are used for application specific messages (ASM) as described in the most recent version of Recommendation ITU-R M.2092.

AIS

  - The existing AIS frequencies AIS-1 and AIS-2 (both are simplex channels) will be used exclusively for safety of navigation, primarily position reporting and identification, ship to ship and ship to shore;

  - the simplex channels 75 and 76 will be used for satellite detection of AIS using AIS Message 27, long range AIS broadcast message.

## E7.5	Legacy Strategy

- Future advances in AIS will require updates to legacy systems.

- Legacy AIS functionality will be maintained.

- AIS channel management, using Channel 70, already available internationally, should be maintained until it is replaced by another internationally agreed method of channel management.

## E7.6	Benefits of Future AIS / VDE for the Users

- Dedicated frequencies will provide for greater safety and integrity of data link, providing better protection for the transfer of safety related messages.

- Use of AIS for maritime information dissemination will mean that NAVTEX / NAVDAT could be supplemented and enhanced by VDE

- The possibility will be created to support the future function of AIS in GMDSS, with the benefit of making distressed vessels visible to all resources (land, sea and air) in the vicinity.

- Future shipboard AIS / VDE installations could provide seamless communication means, supporting a modular concept for communication needs in the VHF band, and enabling scalability and inter-operability of communication solutions intended for SOLAS and well as Non-SOLAS vessels while remaining backward compatible with existing AIS equipment.

- Satellite detection of AIS will be facilitated.

- The rapidly increasing use of data transfer for the efficiency of navigation will be facilitated.

NCSR 4 / 29 (Report of NCSR 4 to the MSC)

The self-contained printer requirement was removed from IMO performance standards (MSC.148 (77)) subject to provision of a dedicated display device and a printer port for the installations on or after 1 July 2005.

http://www.iala-aism.org/asm/

Inmarsat B, C and Fleet 77 are elements of GMDSS

Locating is defined as ‘to find a mobile unit in distress or the location of survivors’.

Homing is defined to determine the bearing to the transmitting station.

Note, there may be benefit in using less than 25kHz, however this is not yet identified in ITU-R M.1842.

Industrial, Scientific and Medical.  These bands are used by devices such as industrial heaters and driers and domestic equipment such as microwave ovens.

This technology is widely used for government agencies and may not be available to commercial enterprise.

According to 3GPP TS 45.005