Input paper:   ENAV30-5.1.3.2

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM		□  ENG		□  PAP						□  Input

X  ENAV		□  VTS								X  Information

Agenda item 				5.1

Technical Domain / Task Number 2	…………………………………

Working Group				WG3

Author(s) / Submitter(s)			Jeonghyeon KIM(I-Storm, Republic of Korea),

Woojin JEONG(WApplE Cloud, Republic of Korea)

Juntae KIM, Jinho YOO (Korean Register)

# Design of VDES Gateway Technology to implement IP network over VDES

# Summary

This document describes the method configuring IP network between a VDES ship station and a VDES shore station to enable IP-based communication with a WAN (Wide Area Network) on land and provides information on the structure of the VDES IP gateway device to be used for this purpose.

By using VDES IP Gateway, even if public IP addresses are not allocated to all ship stations, virtual IP addresses corresponding to MMSI (Maritime Mobile Service Identity) can be arbitrarily allocated to enable communication with a specific server through WAN.

## Purpose of the document

This document aims to explain the structure of an IP network over VDES that can configure an IP-based network to connect to the Internet using VDES from computers or terminal devices on board.

## Related documents

[1]	ITU-R M.2092-1:2022, Technical characteristics for a VHF data exchange system in the VHF maritime mobile band

[2] 	IEC 61162-450 Maritime navigation and radiocommunication equipment and systems – Digital interfaces – Part450: Multiple talkers and multiple listeners – Ethernet interconnection

# Background

‘IP network over VDES’ has been designed to construct IP network to VDES ship-to-ship communication in the KASS (Korea Autonomous surface ship) project. For the sake of this, an IP gateway is required to efficiently handle and convert the IP packets and VDES packets from ship and shore station in order to configure IP network which enable to connect land based internet network using VDES communication.

IP address to identify the ship station is necessary to configure IP network in a ship station and to send packets generated from onboard devices to the Internet through the shore station, however, allocating and managing IP address to all vessels engaged in IP communication require very complex protocol. Therefore, the management of IP address to efficiently identify a vessel in low speed communication network such as VDES and to handle IP packets are needed.

# design of IP network over vdes

## Architecture and functions of IP Network between Ship and Shore

IP network between ship station and shore station should be formed in order to connect devices in ships at sea to specific servers on shore networks using IP communication. The overall structure of the IP network over VDES that can use VDES as a transport medium for such an IP network is shown in Figure 1.

Architecture of IP network between Ship and Shore

Ship VDES IP Gateway performs the following roles.

- Forms a LAN (Local Area Network) for devices in the ship station.

- Process IP packet to be processed inside LAN.

- IP packets to be transmitted outside the LAN are delivered to IP Gateway Software through Virtual NIC (Network Interface Card) according to the router settings.

- IP packet is capsulated by PI (Presentation Interface) and transmitted to VDES transceiver.

- The PI data received from the VDES transceiver is reassembled into IP packets and transmitted to the destination device of the ship through the Virtual NIC.

Shore VDES IP Gateway performs the following roles.

- It is connected to the Internet via WAN.

- Form a LAN for devices within the shore station.

- Process IP packet to be processed inside LAN.

- IP packets that need to be sent outside the LAN are forwarded to the IP Gateway Software through the Virtual NIC according to the router settings.

- PI data received from the VDES transceiver is converted into IP packets and transmitted to the Internet through the Virtual NIC.

- Among the IP packet received from the Internet, the ship station packets within the shore station management area is capsulated by PI and transmitted to the VDES transceiver.

VDES is a communication network that provides relatively low speed. Therefore, it can be seen that the amount or speed of data that the IP gateway must process is relatively small. For such a communication network and data processing scale system, we designed a structure to receive and process Internet Protocol packets from IP Gateway Software through Virtual NIC (Network Interface Card). This design can simplify the structure of the system and make software development easier.

## Design of Ship VDES IP Gateway

Ship VDES IP Gateway

The IP network structure of Ship Gateway is shown in Figure 2. The Ship Gateway forms a LAN with onboard computers or terminal devices. Devices inside the LAN must be assigned an IP statically or dynamically.

When data is transmitted from a client connected to a LAN to a specific server on the Internet, packets are constructed according to application protocols such as HTTP and FTP, and sent and forwarded to the OS kernel along with destination IP and port information. The OS constructs a packet to send by performing processing according to each layer protocol such as TCP/UDP(1) and IP(1). This packet goes through data link layer processing and is forwarded to the router specified in the client's network configuration. In this case, we set the router to Ship Gateway to forward the packets to Ship Gateway.

All IP packets that need to be sent outside the LAN range are entered into the Ship Gateway's physical NIC. Ship Gateway is delivered to Gateway software through Virtual NIC after Data Link Layer processing in OS kernel. A virtual NIC is a software-generated network interface that handles IP layer packets. Therefore, packets received by the gateway software over the virtual NIC contain application, TCP/UDP(2) and IP(2) layer data sent by the client. Gateway software analyzes and processes header information such as destination address, source address, destination port, and source port in the received packet. This packet is composed of the Binary Image Transfer data structure specified in IEC 61162-450 and transmitted through the VDES transceiver. The data structure for IEC 61162-450 binary image transmission is shown in Figure 3.

IEC 61162-450 data format to transmit IP packet

## Design of Shore VDES IP Gateway

Shore VDES IP Gateway

The IP network structure of Shore Gateway is shown in Figure 4. Shore Gateway forms a LAN within the ship station. The IEC 61162-450 data received through the VDES Transceiver is transferred to the gateway software and reconstructed into the Application Data, TCP/UDP(2), and IP(2) packets. Shore Gateway also forwards the recovered packet to the OS Kernel through the Virtual NIC, it is processed by the OS and delivered to the server through the internet connected to ethernet.

The IP packet that the Shore Gateway receives from the Internet and needs to be transmitted to the ship is delivered to the ship through the reverse procedure.

## IP Address management

The payload transmitted from the ship station to the coast station includes IP layer, TCP/UDP layer, and application layer data. The source address and destination address of the IP layer indicate the source and destination addresses as IP addresses. In IP network over VDES, the important IP addresses to be managed are the IP address assigned to the Virtual NIC of the Ship Gateway and the IP address assigned to the Virtual NIC of the Shore Gateway. These two addresses are IP addresses arbitrarily assigned by the gateway software and are not related to each other. The reason these two addresses are important is as follows.

- The source IP address field of the IP layer header that Ship Gateway Software receives through the Virtual NIC all has the IP address of the Virtual NIC. (IP(2) in Fig. 2)

- When Shore Gateway Software restores data received through VDES, the source address of the IP layer header has the IP address of the Ship Gateway's Virtual NIC.

- The destination address of the IP layer header of the response packet received through the Virtual NIC from the Shore Gateway is the same as the Virtual NIC IP address of the Ship Gateway.

- When Ship Gateway Software restores data received through VDES, the destination address of the IP layer header has the IP address of the Ship Gateway's Virtual NIC.

Therefore, the IP address that can identify the ship station when the software of Ship Gateway and Shore Gateway transmits the IP packet of the client of the ship station to the Internet is the IP address assigned to the Virtual NIC of the Ship Gateway.

Ship Gateway's Virtual NIC has an IP address arbitrarily assigned by the Gateway software. Therefore, it cannot be guaranteed that the IP address of the Virtual NIC in each ship is uniquely assigned to the ship station as an independently determined IP address. In other words, since several ship stations communicating with the same shore station VDES transceiver can use the same IP address, it is impossible to use it as a source address or destination address for identifying a ship station in an IP network.

To solve this, the coast station can think of a way to allocate the IP address of the virtual NIC whenever each ship station connects, but considering the VDES communication environment, a protocol for controlling various situations such as delay time and vessel mobility is required. Also, it is possible to use a unique IP address by assigning a public IP to the ship station, but in this case, there is no guarantee that the network connected through the VDES performs routing of packets using this public IP normally.

Therefore, there is a need for a method to allow the ship to access the Internet by providing a method for the coast station to identify the ship station even if the generated packet is transmitted to the coast station using an IP address that can be arbitrarily allocated by the ship. In this document, MMSI is used as a unique number to identify a ship even if such a redundant IP address is used by the ship. It is assumed that the MMSI is uniquely assigned to each VDES device of the ship station and that the IP gateway already knows it. The operation method of the IP address designed in this document is as follows.

- Ship Gateway Software randomly assigns IP address of Virtual NIC.

- The source address of all IP layer headers transmitted from Ship Gateway to Shore Gateway has a randomly assigned address.

- Gateway Software adds its own MMSI when transmitting data in IEC 61162-450 format through VDES. In our case, the source MMSI and the coast station VDES MMSI are added to the Header field of Figure 3, and the destination MMSI is added.

- The Shore Gateway stores the source MMSI field of the received IEC 61162-450 header and the source address of the IP header and allocates a new IP. This new IP is an IP that has not yet been assigned to another ship, and it is assigned from an address range that Virtual NIC can recognize as a LAN. For example, if the Virtual NIC has an address of 200.0.0.1, the address that can be allocated can be 200.0.0.2 ~ 200.0.0.254. It stores the source MMSI, the source address of the IP header received from the ship station, and the mapping table of the newly allocated IP address.

- The Shore Gateway changes the source address of the IP header received from the ship to the newly allocated IP address and transmits it to the Internet through the Virtual NIC.

- When the Shore Gateway receives a response packet from the Internet through the Virtual NIC, the destination address of the IP header is recorded as the address newly assigned by the Shore Gateway. The Shore Gateway can find out the IP address and MMSI information originally sent by the ship station from the previously stored mapping table, records the destination address field of the IP header as the original address, and transmits it to the corresponding ship station using the MMSI.

- When the Ship Gateway receives and restores a packet, the destination address in the IP header is the address assigned to its Virtual NIC. Therefore, when a packet is transmitted through this interface, the response packet is delivered to the device that originally sent the packet.

## Example of Internet connection using IP network over VDES

The IP address change process and Internet access process of IP network over VDES will be described by taking the case of accessing http://www.google.com using a web browser from a client PC connected to the LAN of the ship station.

### IP packet from client to server on Ship VDES IP Gateway

Figure 5 shows the procedure for processing the HTTP Request message generated by the client of the ship station.

HTTP Request packet process of Ship gateway

  - Ship Gateway configures LAN for ship station devices and has an IP of 192.168.1.1, and all connected devices are assigned an address of 192.168.1.x.

  - The Client PC is connected to the ship station's LAN and is assigned an IP address of 192.168.1.200.

  - The Virtual NIC created by Gateway Software has 100.0.0.99 as its address.

  - When http://www.google.com is accessed from the web browser of the client PC, a packet is created in which the HTTP request is set to the application layer data and the destination port of the TCP layer is set to 80 as shown in Figure 5. The source port is arbitrarily designated, and here it is assumed to be designated as 10000. At this time, the source address of the IP layer is set to 192.168.1.200, which is the IP of the client PC, and the destination address is designated as the IP address of www.google.com obtained using DNS (Domain Name Service).

  - IP packet goes through transport layer and is delivered to Gateway software through Physical NIC and Virtual NIC according to the configuration of Ship Gateway. The router's IP address translation process is performed using a protocol such as NAT (Network Address Translation).

  - IP layer packet is transmitted using the payload field of IEC 61162-450 Binary Image transfer data format in Figure 3. At this time, the MMSI of the ship station and the MMSI of the coast station are added to the header of IEC 61162-450 and transmitted. Figure 5 shows an example of transmission using “100” for the ship station’s MMSI and “1” for the coast station’s MMSI.

  - This data is transmitted to the VDES Transceiver and transmitted to the VDES communication network.

### IP packet from client to server on Shore VDES IP Gateway

The procedure for processing the data received from the ship station through VDES is shown in Figure 6.

Received packet process of Shore gateway

- IEC 61162-450 data received through VDES is restored to IP layer packet transmitted by the ship. Source MMSI is used as an identifier to identify the ship that transmitted this packet.

- The source address of the recovered IP layer packet has 100.0.0.99 transmitted by the ship. Since this address is an address allocated by the ship station itself, it cannot be used for ship identification by the coast station. Also, if this address is transmitted to the Internet network as it is, the response packet may not be returned to the coast station Virtual NIC. Therefore, Shore VDES IP Gateway reassigns a unique IP address to identify each ship station. Here, 200.0.0.10, which is the IP address range of 200.0.0.1, which is the coast station Virtual NIC, is allocated.

- The Shore VDES IP Gateway stores the mapping relationship between the Source MMSI, Source Address, and New Source Address sent by the ship station. This mapping table is needed when transmitting the response packet to the ship station.

- Change the source address field of the IP layer to 200.0.0.10, which is the changed address, and transmit it to the Internet network through the Virtual NIC and Physical NIC, and the packet is delivered to the destination www.google.com server.

### IP packet from server to client on Shore VDES IP Gateway

Figure 7 shows the procedure for processing the HTTP Response message transmitted from the Internet server to the ship station through the Shore gateway.

HTTP Response packet process of Shore gateway

HTTP Response packet from server on Internet is received from Shore gateway through Internet routing and delivered to Gateway software through Virtual NIC. In this data, the destination address field of the IP header has a value of 200.0.0.10 newly allocated when the HTTP Request packet is transmitted. Gateway software can find the original address "100.0.0.99" for MMSI "100" by looking up this IP address in the mapping table. Therefore, the destination address of the IP header is replaced with the original address, 100.0.0.99, and transmitted to the ship with MMSI “10” through the VDES transceiver.

### IP packet from server to client on Ship VDES IP Gateway

The procedure for processing the data received from the ship station through VDES is shown in Figure 8.

Received packet process of Ship gateway

The IP packet restored from the VDES received packet is transmitted to the Virtual NIC and delivered to the client that sent the HTTP Request according to the LAN network routing.

## Test configuration for IP network over VDES

The operation of the IP network was verified by building a test system for testing the IP network over VDES system described in this document as shown in Figure 9.

Block diagram of IP network over VDES test system

In this test system, two VDES transceiver simulators to emulate the VDES radio communication were set up as a ship station and a coast station respectively, and a serial wire connection of 57.6kbps was used. The actual test configuration for IP network over VDES is as follows.

Test setup of IP network over VDES

## Test result

The test environment was configured as shown in Figure 9 and Figure 10, and the system operation was verified by accessing website:www.goole.com from the web browser of the ship station terminal device (PC), searching for "VDES", and monitoring the packet conversion process at each step.

Information on the main parameters used in this test environment is shown in the table below.

Main Parameters for Test Environment

| Item | Parameter |
| --- | --- |
| IP address of terminal device (ship side) | 192.168.200.10 |
| Virtual NIC IP address of VDES gateway (ship side) | 100.0.0.1 |
| Virtual NIC IP address of VDES gateway (shore side) | 192.168.53.1 |
| MMSI of VDES transceiver (ship side) | 200 |
| MMSI of VDES transceiver (shore side) | 100 |
| Destination for test | google.com |

Search for “VDES” through google.com in the ship terminal device and check the HTTP packet received from the ship terminal device as shown in Figure 11 (1) by monitoring the packet on the physical NIC of the ship VDES Gateway.

HTTP Request packet received on physical NIC of ship gateway

In the Virtual NIC of the ship VDES gateway, it can be confirmed that this packet is changed to the virtual NIC IP (100.0.0.1) as shown in Figure 12 (1) and received.

HTTP Request packet received on virtual NIC of ship gateway

IP packet received from Virtual NIC is capsulated in IEC61162-450 format in gateway software and transmitted to VDES transceiver. At this time, SrcMMSI and DestMMSI information is added in the -450 header as shown in Figure 13.

Fragmented IEC 61162-450 packet header on ship gateway

The processing of packets received by the shore VDES gateway from the VDES shore transceiver is monitored as shown in Figure 14 (1). This packet is restored through the IEC 61162-450 defragmentation process, and a new IP address (192.168.53.101) is allocated using the SrcMMSI field of the header and the IP address (100.0.0.1) to reach the Internet through the WAN.

Process of shore gateway

The processing of the IP packet received from the target server is shown in Figure 14 (2). The destination IP (192.168.53.101) is converted to the original IP address (100.0.0.1) by the MMSI-IP mapping table and transmitted to the ship station (MMSI 200).

The information received from the ship station goes through a series of processing again and is converted in the order of (2) in Figure 12 and (2) in Figure 11 in the Virtual NIC and transmitted to the ship terminal device.

Finally, the result screen of searching for “VDES” in the web browser of the ship terminal device is shown in Figure 15.

Search “VDES” result using IP network over VDES

# Conclusion

This document describes how to configure an IP network using VDES. Since VDES provides low-speed communication, it is possible to make IP gateway structure simple by processing packets in gateway software using Virtual NIC. In addition, by designing a coast station to manage the IP address using the ship station's MMSI, IP communication is possible even if the ship station does not use a unique IP address. In the future, this IP network over VDES concept will be extended to the design and construction for IP network between ships in the KASS project.

# References

None

# Action requested of the Committee

The Committee is requested to note the information and take appropriate action.

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain