VTS33	Input paper

Agenda item	14

Task Number

Author(s)	M. Boisgontier, T Le Poder	, B Manoury

# SHIP and SHORE VHF installations

# Interference between AIS and radio mobile

# Summary

The Automatic Identification System (AIS) is based on a shipborne transponder that shares the maritime mobile VHF band 156 MHz to 162 MHz frequencies with VHF mobile radio transmissions and is, as the matter of fact, subject of compatibility problem.

The document VTS31/15/5 e-NAV8/9/17 proposed modifications of IALA papers for explanation of radio interferences between AIS transponder and VHF transmitter receiver on the ship.  VTS32/8/6 asked for forward this problem to e-NAV9.  At the same time, it appeared that these phenomena was evoked in many papers and needed to be highlighted with a quantitative analysis and some experimentation.

These experiments took longer than expected but results are available for VTS33 and e-NAV10 committees.

## Purpose of the document

The document presents the tests observed with

3 type of AIS equipment: AIS class A (included base station and AtoN), AIS class B and AIS receiver not in compliance with IEC specification,

three scenarios :

in shore site with low input signals (scenario 1)

in shore site with high input signals (scenario 2)

on board of three different ships (scenario 3)

## Related documents

The reference documents are the following

for VHF equipment : Recommendation IEC 60489-3 (61097-7), ETSI EN 300 086

for AIS equipment : Recommendation ITU-R M.1371 (Technical Characteristics for a Universal Shipborne Identification System Using Time Division Multiple Access In the VHF Maritime Mobile Band), Recommendation IEC 61993-2 (class A), IEC 62287 (class B), IEC 62 320-1 (base station), IEC 62 320-2 (AtoN).

# Background

## Test procedure

Scenario 1 : the AIS equipment is located in shore site near Brest and receive the AIS data of two distant AIS base stations: Pointe du Raz of 20 NM distant and Portzic station 1,5 NM distant.  A local frequency generator simulate a VHF transmitter and interfere with the AIS received signal by the antenna with a modulated frequency in duplex band (160 to 162 MHz) and  in simplex band (156 to 157 MHz).  The local generator is modulated with audio signal of 1 kHz with frequency deviation of 1.5 kHz.

Scenario 2 : the AIS wanted signal is generated by a test bed formed with sentence AIS generator coupled with sentence AIS analyser.  The AIS signal is mixed with a local generator as unwanted signal.

Scenario 3 : the AIS equipment and the VHF transceiver of three different ships was used.

## Test results

The tests measure the message errors when there are

100% reception (no perturbation)

50% reception

0% reception (all messages are deleted or blocked)

# Scenario 1

VHF generator : AEROFLEX 2945

AIS receiver : class A L177111RX PRESCOM  and AtoN KANNAD

AIS receiver : class B Seven Star 2 FURUNO

AIS receiver : not IEC compliant : COMAR SLR 200

mixer : EAGLE 5-1000 MHz RLB 150*3 (measure 6 dB insertion loss  and 20 dB  isolation)

We utilize  "AIS  dispatcher 1.0" and "AIS decoder"  free software

AIS receiver

COMAR SLR 200

Mixer

AIS analyser

and decoder

VHF Generator

with 1kHz modulation

Mixer

## Localisation of the AIS base stations

Portzic base station (MMSI 2288187)

Distance 1.5 nm

Reception site

Raz base station (MMSI 2275365)

Distance 20 nm

We select message AIS 20 on the two base stations Raz (MMSI 227 5365) distant of 20 NM from the  reception site and Portzic (MMSI 228 8187), distant of 1.5 NM from the reception site.

With AIS receiver COMAR SLR 200 and -25 dBm unwanted signal at 156.8 MHz, all messages of the two AIS base stations are received

With AIS receiver COMAR SLR 200 and -22 dBm unwanted signal at 156.8 MHz, 50% of message of Raz base station ( MMSI 227 5365) are deleted, all messages of Portzic base station (MMSI 228 8187) are received

With AIS receiver COMAR SLR 200 and -20 dBm unwanted signal at 156.8 MHz, all messages of the two base stations are deleted.

(*) All level are measured on the input of the AIS receiver.

We exchange the receiver COMAR SLR 200 for class A and class B equipment and renew the measurements.  The results were as follows :

level of unwanted signal at 156,8 MHz

for a reception rate from Raz base station (RSSI 43 )

| AIS receiver | 100% reception | 50% reception | 0% reception |
| --- | --- | --- | --- |
| Class A (Prescom analyser) |  |  |  |
| Not IEC compliant (COMAR SLR 200) | -25 dBm | -22 dBm | -20 dBm |
| Class A AtoN KANNAD | -25 dBm | -21 dBm | -19 dBm |

level of unwanted signal at 156,8 MHz

for a reception rate from Portzic base station (RSSI 93)

| AIS receiver | 100% reception | 50% reception | 0% reception |
| --- | --- | --- | --- |
| Class A (Prescom analyser) |  |  |  |
| Not IEC compliant (COMAR SLR 200) | -21 dBm | -19 dBm | -18 dBm |
| Class A AtoN KANNAD | -11 dBm |  | -8 dBm to -9 dBm |

level of unwanted signal at 161,5 MHz

for a reception rate from Raz base station (RSSI 43 )

| AIS receiver | 100% reception | 50% reception | 0% reception |
| --- | --- | --- | --- |
| Class A (Prescom analyser) |  |  |  |
| Not IEC compliant ( COMAR SLR 200) | -45 dBm | -43 dBm | -42 dBm |
| Class A AtoN KANNAD | -36 dBm | - 34 dBm | -33 dBm |

level of unwanted signal at 161,5 MHz

for a reception rate from Portzic base station (RSSI 93)

| AIS receiver | 100% reception | 50% reception | 0% reception |
| --- | --- | --- | --- |
| Class A (Prescom analyser) |  |  |  |
| Not IEC compliant ( COMAR SLR 200) | -32 dBm | -31 dBm | -30dBm |
| Class A AtoN KANNAD | -22 dBm | -20 dBm | -18 dBm |

# Scenario 2

VHF

generator

Sentence AIS

generator

Sentence

AIS analyser

Sentence

AIS

generator

Sentence

AIS

analyser

fig. Measure equipments

VHF generator : AEROFLEX 2945 and AGILENT N9310A

AIS receiver : class A L177111RX PRESCOM (AIS class A test bed) and AtoN KANNAD

AIS receiver : not IEC compliant : COMAR SLR 200

mixer : EAGLE 5-1000 MHz RLB 150*3 (measure 6 B insertion loss and 20 dB isolation for frequencies 156 to 162 MHz)

In this scenario, we fixed the level of the AIS signal and increase the level of the unwanted signal in order to obtain the wanted rate of message received.  (Sentence analyser and generator are moved to a distance for reduce direct coupling.)

The receiver is a class A.  We chose the message type "1" alternative on AIS A and B, the repetition frequency is 5 sec.

The level of the AIS signal is fixed at -83 dBm, we increased the unwanted signal progressively.

Level of the unwanted signal at 156.8 MHz

| AIS level on class A receiver | 100% reception | 50% reception | 0% reception |
| --- | --- | --- | --- |
| -83 dBm (AIS test bed) | -1 dBm | 2 dBm | 4 dBm |
| -80 dBm (AtoN beacon) |  | 3 dBm | 5 dBm |

Level of the unwanted signal at 161.5 MHz

| AIS level on class A receiver | 100% reception | 50% reception | 0% reception |
| --- | --- | --- | --- |
| -80 dBm (AtoN beacon and test bed) | -18 dBm | -17 dBm | -16 dBm |

a VHF unwanted signal at 156.8 MHz with a level of -1 dBm cause no perturbation of the AIS receiving signal (we read a receive strength signal  indicator (RSSI) 127 to 132),

a VHF unwanted signal at 156.8 MHz with a level of 2 dBm cause a AIS message a little corrupted,

a VHF unwanted signal at 156.8 MHz with a level of 4 dBm block the AIS reception signal, (this level is 5 dBm for the AIS AtoN),

If the frequency of the unwanted signal is 161.5 MHz (nearer of the AIS frequencies than 156,8 MHz), the AIS messages are blocked at -16 dBm

With the same class A receiver, we increase the AIS signal (the AIS transmitter shall be move at distance because of direct coupling problems)

Level of the unwanted signal at 156.8 MHz

| AIS level on class A receiver | 100% reception | 50% reception | 0% reception |
| --- | --- | --- | --- |
| -60 dBm on AIS test bed | 4 dBm |  | 10 dBm |
| -60 dBm on AtoN |  | 10 dBm | 11 dBm |

Level of the unwanted signal at 161,5 MHz

| AIS level on class A receiver | 100% reception | 50% reception | 0% reception |
| --- | --- | --- | --- |
| -60 dBm on AIS test bed | -4 dBm |  | 0 dBm |
| -60 dBm on AtoN |  | 0 dBm | 1 dBm |

a VHF unwanted signal at 156.8 MHz with a level of +4 dBm give no perturbation of the AIS receiving signal.  If the unwanted signal increases at 10 dBm or 11 dBm, the AIS signal is blocked.

If the unwanted signal is at 161.5 MHz frequency, the AIS signal is blocked at  0 dBm or 1 dBm.

Level of the unwanted signal at 161.975 MHz or 162.025 MHz (adjacent channel)

| AIS level on class A receiver | 100% reception | 50% reception | 0% reception |
| --- | --- | --- | --- |
| -60 dBm on AIS test bed |  |  | channel A for -27 dBm at 162.025 MHz |
| -60 dBm on AIS test bed |  |  | channel B for -27 dBm at 161.975 MHz |

For the AIS receiver COMAR SLR 200 (not IEC compliant)

Level of the unwanted signal at 156.8 MHz

| AIS level on Comar SLR 200 receiver | 100% reception | 50% reception | 0% reception |
| --- | --- | --- | --- |
| -60 dBm | -7dBm | -6 dBm | -4 dBm |
| -50 dBm | -8 dBm |  | -5 dBm |
| -40 dBm | -8 dBm | -6 dBm | -5 dBm |
| -30 dBm | -7 dBm | -5 dBm to -6 dBm | -4 dBm |

The AIS reception is blocked with a VHF unwanted signal with a level of -4 dBm, this level is stable even if the AIS signal increase.

We suppose that the blocked unwanted signal level is 10 dBm for a class A receiver for an AIS signal from -60 dBm and more.

# SCENARIO 3

## Test aboard the Armorique buoy tender (498 UMS, length 48 m)

Equipment : Display MaxSea, receiving AIS, radio VHF RT 2048 of Sailor, antenna on radio mast.

Ship buoy tender Armorique

The local VHF transmitter is off, we can see on the Maxsee display the AIS track of the ship alongside the quay and away: Alcyon in 1.17 nautical mile, Tréburon  0.52 nautical mile, Colin 0.12 nautical mile, Recouvrance 0.124 nautical mile, Abeille Bourbon 0.06 nautical mile, Eship Base 15.9 mile.

We command the transmission on channel 11 (frequency 156.550 MHz): the tracks remain displayed, but the tracks in progress are blocked.  We noted that the software Maxsea maintain the track in progress 30 sec. even if there are no reception.

AIS tracks near the Armorique

Radio VHF equipment

After 3 minutes of VHF transmission on channel 11, some tracks goes out: track of A. Bourbon (0.06 nm),  Eship Baz (15.9 nm)...

After 10 minutes of VHF transmission on channel 11: track AIS of the ship Andre again in progress in 1.14 nautical mile, tracks of ships alongside the quay are not cancelled.

## Try aboard the tug boat of open sea Abeille Bourbon (length 80 m, 3200 ums)

AIS antenna

VHF antennae

Equipment : Receiver AIS Furuno universal FA – 100, control screen MaxSea, transmitter VHF Furuno FM 8500, antenna VHF installed 25 m high, we check a distance of 10 meter between AIS and VHF antennae.

Tug Abeille Bourbon

After transmission on channel 69, track of ship Typhon 19.2 mile is blocked, track of ship A. Colin 13.8 mile distance is not blocked, track of ship buoy tender Armorique (0228155700) is not blocked while his course 1.5 mile to 6.4 mile from Abeille  Bourbon.

Track of buoy tender Armorique 6 mile from ship Abeille Bourbon in progress

New test with channel 77 the 3 august 2011

Tracks of the base station Pointe du Raz (distant 24 mile) goes out after 40 or 60 sec transmission.

Some tracks of ship near Sein island at 24 miles or farther at 41 miles in the south of Raz goes out after 60 sec transmission, track of ship less far are not blocked for instance: track Roselend at 17 miles and other ships at 13 miles.

## Experimentation aboard a small boat

Ship SNS151 (15 m) of SNSM “Le Conquet” the 6th of august 2011.

Equipment: VHF FURUNO and SAILOR 25 W, AIS FURUNO, control screen MAXSEA.

Transmission channel VHF 72.  Track ship «rien d'impossible» 4.17 miles blocked, and farther blocked, these tracks remains displayed 60 seconds even if there is no reception.  They are blocked and then flagged by a cross some seconds after the end of the VHF transmission they appear again.

AIS antenna

Track of ship “Enez Eussa” at 0.49 mile not blocked and displayed even if they are VHF transmission.

VHF antennae

# Discussion

## Scenarios

Scenario 1 shows that a weak AIS signal received by a shore installation can be blocked if a VHF local signal in the band 156 MHz to 162 MHz is transmitted in the same time.

The level of the local signal (unwanted and blocking signal) depend of the strength of the wanted AIS signal (in relation with the distance of the AIS station), the type of the receiver, the frequency of the local signal.

This result confirm our experimentation in 2001 with AIS class A receiver SAAB transponder model R30 for base station, we had find a level of unwanted signal at -9 dBm in 156 MHz band.

Scenario 2 shows the variation of the level and the frequency of the blocking signal with the level of AIS wanted signal.  For a class A receiver and strengthened AIS level, levels of 5 and 10 dBm shall be considered as upper limit for the unwanted signal.

Scenario 3 shows that, the AIS track can be blocked by a local VHF transmission and can disappear.  If the ship has a good installation, this phenomenon appear for simplex channel and for long distance.  If the VHF transmitter is in weak power mode, the track remains. For small ship or for local transmission in duplex mode or for no compliance receiver, the AIS track is blocked.

## Conformity to the requirements

The test CEI 61993-2 15.4.7, "blocking and desensitization" use a wanted signal (AIS signal) -104 dBm and an unwanted signal between ±1 MHz to ±10 MHz of the AIS frequencies.  The blocking ratio for any frequency within the specified ranges shall not be less than 84,0 dB.  The level of the blocking signal is -20 dBm.  The unwanted signal is unmodulated.

The test CEI 62 320-1 9.3.11.2 (base station), "blocking and desensitization" use a wanted signal (AIS signal) -101 dBm and an unwanted signal (blocking signal modulated) at 156,025 and 156,3 MHz at -15 dBm for a PER of 20%.  The unwanted signal is modulated 400Hz.

The test CEI 62 320-2 (AtoN) 7.1.2.7.2, "blocking and desensitization" use a wanted signal (AIS signal) -101 dBm and an unwanted signal (blocking signal) -15 dBm at ± 5 MHz for a PER of 20%.  The unwanted signal is unmodulated.

Comment : We have used a wanted signal with a level -80 dBm because the measurement is easier and we have choice an unwanted signal modulated 1000 Hz with a deviation of 1,5 kHz which is more representative of voice modulation.  This modulation frequency has nevertheless a weak effect on the results.

## Variation of the level of the blocking signal and the level of the AIS signal

We have measured (scenario 2) a blocking signal of 4 and 5 dBm for a wanted AIS signal of -80 dBm and a blocked signal of 10 dBm for a wanted AIS signal of -60 dBm. We consider that the  AIS receiver is compliant with the blocking signal -20 dBm for AIS signal of -104 dBm.  We can consider a difference of 84 dB between AIS signal and blocking signals.

We can deduce that the level of the blocking signal at 156,8 MHz for the sensitivity of the receiver AIS of -107 dBm is -107 dBm + 84 dB = -23 dBm.

## Shore station

In the shore station, the best sensitivity of the AIS signal shall be considered.  That means that the sensitivity of the AIS signal shall be -107 dBm.  The level of the blocking signal is therefore -23 dBm in simplex frequency band or higher for frequencies of the duplex band, near the AIS frequencies.

If the VHF transmitter is in the same place of the receiver and has a power of 25 watts (44 dBm) at 156.8 MHz and no gain in the antennae, the isolation shall be :

44 dBm + 23 dBm = 67 dB

If the shore station use duplex channel, the isolation shall be higher, depends of the distance in between.

These values are very important and special dispositions are needed for the protection of the AIS reception.

## ship installation

For the ship installation, we can consider two levels of the blocking signals : 10 dBm if there are little distance between the VHF and AIS antennae and 5 dBm if there are a bigger distance.

## For blocking signal 10 dBm and antennae on the same horizontal plan

If we consider a local transmitter with a power of 25 watts (44 dBm) and the blocking signal of 10 dBm for VHF frequencies in simplex band, the isolation between the antennae is : 44 – 10 – 2 - 2 = 30 dB if the antennae are on the same horizontal plan, (44: power  signal, 10 is level of the blocking signal, 2 dB is gain of AIS and VHF transmitter antenna).

The isolation needed correspond to a distance between the antennae of more than 8 meters in horizontal plan (use the formula : isolation = 22 + 20 log(d/λ)-G1-G2)  (λ is the wavelength)

## Antennae in a vertical plan

If the antennae are in a vertical plan, the isolation needed is : 44-10 = 34 dB (44 dBm: power of local VHF transmitter, blocking signal : 10 dBm)

The isolation needed correspond to a distance between the antennae of more 3,3 meters in vertical plan (use the formula : isolation = 25 + 40 log(d/λ)).   (λ is the wavelength)

## Distance of reception

A level of blocking signal of 10 dBm fit with a wanted AIS signal of -60 dBm.  This level is 47 dB higher of the sensitivity (-107 dBm), that fit with a propagation weakening between  two ships and  a theoretical distance (1) between ship reduced from 26 nautical miles to 0,2 nautical mile if the high of the ship antennae are 10 meters.  We find a distance of 1,5 miles with ship Armorique buoy tender and a distance of 0,49 mile with ship SNS in scenario 3.

## Blocking signal of 5 dBm

If we consider a level of the blocking signal of 5 dBm for VHF frequencies in simplex band, the AIS signal level shall be higher than -80 dBm. That suppose an isolation again local transmission in VHF band of :

35 dB if the antennae are on the same horizontal plan

39 dB if the antenna are in a vertical plan,

These isolation values fit with a distance between antennae on the ship of more than 14 m in horizontal plan or 4,4 m in vertical plan.

## Distance of reception

A level of a wanted AIS signal of -80 dBm is 27 dB higher of the sensitivity (-107 dBm), that fit with a propagation weakening between two ships and a theoretical distance (1) between ship reduced from 28 nautical mile to 65 nautical miles if the high of the ship antennae are 20 meters (1).  We find a distance of 14 nautical miles with ship Abeille Bourbon in scenario 3.  We find also in august 2011 a distance of 24 miles but this value is given by the very good propagation condition.  The distance of 41 nautical mile instead of 28 miles for the track without interference confirm these very good conditions.

If these isolations are not possible, especially for small ships, or if the transmission frequencies are in the duplex band or if the AIS receiver is not CEI class A compliant, the AIS signal is blocked, the ship won't be able to receive AIS data.

# Action requested of the Committee

The Committee is requested to adapt the IALA guideline and recommendation for a better explanation of the interferences:

For VTS installation

If the best range for the reception of AIS messages is required, the isolation between AIS reception and other VHF frequencies should be managed for maintain the sensitivity level -107 dBm.

For ship installation

A VHF transmission in simplex band in the same time of the reception of AIS signal has an effect of desensitization or blocking the AIS signal on board.

If some distance can be reserved on board between the AIS and VHF antennae, the AIS signal is not blocking, but the distance of reception of AIS signal is reduce.

For no compliant receiver or for duplex radio VHF frequencies or for ship where the distance between antennae is limited, the reception of the AIS signal in the same time of VHF transmission is not possible.

Therefore an explanation should be provide to the operators to not use on board radio VHF transmission specially the duplex frequencies if they want to receive the AIS signal without trouble, epecially in collision avoidance process.

Change in IALA and IMO documents Res. A917(22) the text “AIS may susceptible to interference from adjacent channels ...”  because that suggest the interference is not sure and only adjacent channel they interfere.  We may indicate : “AIS interfere with all channels of the VHF band, the distance of reception of AIS signal is reduced , depending of the characteristics of the installation,  the frequency used ...”

(1) we use the recommendation ITU-R P:526-7 propagation in diffraction