AISM Association Internationale de Signalisation Maritime  IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

IALA Recommendation V-128

On

Operational and Technical Performance Requirements for VTS Equipment

Edition 4.0 draft

XXX 2010

Edition 1.0 – June 2004

Edition 1.1 – June 2005

Edition 2.0 – December 2005

Edition 3.0 – June 2007

Edition 4.0 – xxxx xxxx

20ter, rue Schnapper, 78100

Saint Germain en Laye, France

Telephone +33 1 34 51 70 01  Fax +33 1 34 51 82 05

e-mail:  iala-aism@wanadoo.fr    Internet:  www.iala-aism.org

Document Revisions

Revisions to the IALA Document are to be noted in the table prior to the issue of a revised document.

| Edition / Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |
| Edition 1.1 June 2005 | Addition of Annex 6 – Hydrological and Meteorological equipment | Annexes added as they are completed to ensure all aspects of VTS equipment are covered. |
| Edition 2.0 December 2005 | Restructured to include operational performance requirements. Annex 2 amended to reflect new annex on operational performance requirements. Annex 6 renamed to Annex 5 Annex 1,3,4,6 added | Annexes added as they are completed to ensure all aspects of VTS operations and equipment are covered. |
| Edition 3.0 June 2007 | Editorial changes to correct errors in paragraph numbering, cross references etc. Structure of annexes harmonised, part of Annex 2 moved to new IALA Guideline (Establishment of Radar Services) Clarification of text, few sentences in annex 1 and 2. | Inconsistence in cross references, table of contents etc. in edition 2.0 Varying structure of individual annexes Users of the document provided ideas to clarification of text on some subjects. |
| Edition 4.0 Xxxx xxxx | Document rewritten and updated to improve user friendliness, to include additional cases (ports, inland waterways and offshore) and to include new technology Annex 7-13 added | New technology emerging Feedback from users indicated need to make the document more user friendly, and to include special considerations for ports, inland waterways and offshore |

Recommendation on Operational and Technical Performance Requirements for VTS Equipment

(Recommendation V-128)

THE COUNCIL:

RECALLING the function of IALA with respect to safety of navigation, the efficiency of maritime transport and the protection of the environment;

NOTING that Chapter V (12) of the International Convention for the Safety of Life at Sea 1974 (SOLAS 74 as amended) requires Contracting Governments planning or implementing VTS wherever possible to follow the guidelines adopted by the Organization by Resolution A. 857(20);

NOTING ALSO that IMO Resolution A.857(20), Annex section 2.2.2 recommends that in planning and establishing a VTS, the Contracting Government or Governments or the competent authority should inter-alia establish appropriate standards for shore and offshore-based equipment;

NOTING FURTHER that National Members provide shore infrastructure to support the aim of IMO to improve the safety of navigation and the protection of the environment;

RECOGNISING that IALA fosters the safe, economic and efficient movement of vessels through improvement and harmonisation of aids to navigation, including vessel traffic services, worldwide;

RECOGNISING ALSO that harmonisation of vessel traffic services would be enhanced by the introduction of international technical performance requirements for VTS;

HAVING CONSIDERED the proposals by the IALA VTS Committee on Operational and Technical Performance Requirements for VTS;

ADOPTS the Operational and Technical Performance Requirements for VTS as set out in this recommendation as follows:

Annex 1  – 	Core Operational requirements

Annex 2  – 	Radar

Annex 3  – 	Automatic Identification System (AIS)

Annex 4  – 	Environmental Monitoring

Annex 5  – 	Electro-Optical equipment

Annex 6  – 	Radio Direction Finders

Annex 7  –  	Long Range sensors

Annex 8  – 	Radio Communications

Annex 9  – 	Data Processing

Annex 10  – 	Human Machine Interface (HMI)

Annex 11  – 	Decision Support

Annex 12 – 	External Information Exchange

Annex 13 – 	Verification and Validation

RECOMMENDS that Competent Authorities providing Vessel Traffic Services take into consideration the appropriate Operational and Technical Performance Requirements contained in the Annexes to this recommendation when establishing appropriate standards for shore and offshore-based VTS.

*    *    *

Table of Contents

Green text has been reviewed by the technical WG

White require review

Yellow require structuring or more work and review

Blue requires substantial work or rewriting

ANNEX 1	Core Operational Requirements	15

1.1	INTRODUCTION	15

1.2	Definitions and clarifications	15

1.2.1	Supporting Documents	15

1.3	Core Operational Requirements for a VTS System	16

1.3.1	Levels of capabilities	16

1.3.2	Allocation of Capabilities to meet Operational Requirements	17

1.4	VTS System Considerations	19

1.4.1	General	19

1.4.2	Availability	19

1.4.3	Back up and fall-back arrangements	21

1.4.4	Precautionary measures to extreme events	21

1.4.5	Recording, Archiving and Replay	21

1.5	Design, installation and maintenance considerations	22

1.5.1	Climatic categories	22

1.5.2	Considerations for determining climatic categories	24

1.5.3	Environmental capabilities and constraints	24

1.5.4	Warning lights	25

1.5.5	Access	25

1.5.6	Power	25

1.5.7	Marking and identification	25

1.6	Documentation	26

1.7	Safety and security precautions	26

ANNEX 2	Radar	27

2.1	Introduction	27

2.2	Definitions and clarifications	27

2.2.1	Definitions	27

2.2.2	IALA target models	29

2.2.3	Supporting Documents	30

2.2.4	Software tools	30

2.3	Radar System solutions	31

2.4	Radar types	31

2.4.1	Pulse Radar	31

2.4.2	Pulse Compression radar	31

2.4.3	Frequency Modulated Continuous Wave	32

2.5	Characteristics of Radar Targets	33

2.5.1	Radar Cross Section	34

2.5.2	Polarization	34

2.6	Operational Requirements	35

2.6.1	The area	35

2.6.2	Targets to be detected	36

2.6.3	Determination of environmental capabilities and constraints	36

2.6.4	Other influencing factors, obstructions, interference etc.	38

2.6.5	Interference	39

2.6.6	Target Detection requirements	40

2.6.7	Calculation of radar detection performance	41

2.6.8	Influence from propagation	43

2.6.9	Target Discrimination	47

2.6.10	Radar dynamic capabilities and constraints	49

2.7	Functional Requirements	51

2.7.1	Operational outputs	51

2.7.2	Operator functions	51

2.7.3	Clutter and noise reduction / Management	51

2.7.4	Elimination of false echoes	52

2.7.5	Built-in test features	52

2.7.6	Service access	52

2.8	Radar design, installation and maintenance considerations	52

2.8.1	Safety precautions	52

2.8.2	Access	52

2.8.3	Design	52

ANNEX 3	Automatic Identification System	54

3.1	Introduction	54

3.1.1	Application	54

3.1.2	Objectives of AIS	54

3.2	Abbreviations and References	55

3.2.1	Abbreviations	55

3.2.2	References	55

3.3	Characteristics	56

3.4	Equipment	56

3.4.1	Class B	56

3.4.2	Limited base stations	56

3.5	Operational requirements	56

3.5.1	Use of virtual MMSI for VTS communication	57

3.5.2	Short safety-related messages	57

3.5.3	Use of binary messages for data exchange	57

3.5.4	Use of the VTS Targets message	58

3.5.5	Assigned Mode	58

3.6	Functional requirements	58

3.7	Graphical presentation	58

3.7.1	Symbol usage	58

3.7.2	Interaction with radar tracks	59

3.8	Data Validity	59

3.9	AIS design, installation and maintenance considerations	59

3.9.1	Transmission Layer	59

3.9.2	Coverage aspects	59

3.10	Interfacing	59

ANNEX 4	Environmental Monitoring	60

4.1	Introduction	60

4.1.1	Scope	60

4.1.2	Objectives	60

4.2	References	61

4.2.1	Definitions	61

4.3	Characteristics of Environmental Sensors in VTS	61

4.4	Functional Requirements	61

4.5	Operational Requirements	62

4.5.1	Information Presentation	62

4.5.2	Malfunctions and Indicators	62

4.5.3	Accuracy	62

4.5.4	Technical requirements	63

4.6	Design, installation and maintenance considerations	64

4.6.1	Durability and Resistance to Environmental conditions	64

4.6.2	Interference	64

4.6.3	Power Supply Requirements / Options	64

4.6.4	Installation	65

4.6.5	Maintenance	65

4.6.6	Technical Challenge	65

4.6.7	Interfacing	65

4.6.8	Backup Arrangements	65

4.6.9	Safety Precautions	66

ANNEX 5	Electrooptical equipment	67

5.1	Introduction	67

5.2	References and abbreviations	67

5.2.1	References	67

5.2.2	Abbreviations	67

5.3	Characteristics	67

5.3.1	Reliability, accuracy, range, and resolution	68

5.3.2	Malfunctions, warnings, alarms and indications	68

5.4	REQUIREMENTS	68

5.4.1	Functional Requirements	68

5.4.2	Operational Requirements	68

5.5	Specific design, installation and maintenance considerations	69

5.5.1	Durability and resistance to environmental conditions	69

5.5.2	Interference	69

5.5.3	Power supply	69

5.5.4	Site selection and Installation	69

5.5.5	Maintenance	70

5.5.6	INTERFACING	70

5.5.7	BACK-UP AND FALL-BACK ARRANGEMENTS	70

5.5.8	SAFETY PRECAUTIONS	70

5.6	CLOSED CIRCUIT TV AS A VTS SENSOR (CCTV)	70

5.6.1	General	70

5.6.2	Detection Performance of CCTV Cameras	70

5.6.3	Characteristics of the CCTV	71

5.6.4	Availability	71

ANNEX 6	Radio Direction Finders	72

6.1	Introduction	72

6.2	Operational Requirements	72

6.2.1	RDF coverage Area	72

6.2.2	Frequency range	72

6.2.3	Number of simultaneously monitored VHF channels	72

6.2.4	Detection performance	73

6.2.5	Bearing accuracy	73

6.3	Functional Requirements	74

6.3.1	VHF channel(s) management	74

6.3.2	SAR functionality	74

6.3.3	Man Over Board EPIRB detection capabilities	75

6.3.4	COSPAS\SARSAT detection and decoding	75

6.3.5	Build In Test and diagnostic	75

6.4	RDF design installation and maintenance considerations	75

6.4.1	RDF antenna installation	75

6.4.2	Lightning protection	75

ANNEX 7	Long Range sensors	77

7.1	Introduction	77

7.2	Long Range Identification & Tracking (LRIT)	77

7.2.1	Specific design, configuration, installation & maintenance considerations	77

7.3	Satellite AIS	78

7.3.1	Specific design, configuration installation and maintenance considerations	78

7.4	HF Radar	78

7.5	Synthetic Aperture Radar (SARSAT)	79

7.5.1	Specific design, configuration, installation & maintenance considerations	79

ANNEX 8	Radio communications in VTS	80

8.1	Introduction	80

8.2	References	80

8.3	Characteristics of Radiocommunication equipment	80

8.4	REQUIREMENTS	81

8.4.1	Radiocommunications Coverage	81

8.4.2	Recording and Playback of Data	81

8.4.3	Malfunctions, warnings, alarms and indications	81

8.4.4	Availability	81

8.5	Specific Design, Installation & Maintenance Considerations	82

8.5.1	Durability and resistance to environmental conditions	82

8.5.2	Interference	82

8.5.3	Power supply	82

8.5.4	Site selection and Installation	82

8.5.5	Maintenance	82

8.6	INTERFACING	82

8.6.1	BACK-UP AND FALL-BACK ARRANGEMENTS	82

ANNEX 9	Data Processing	83

9.1	Introduction	83

9.2	Abbreviations and References	83

9.3	Characteristics	83

9.3.1	Maintaining the Traffic Situation	83

9.3.2	Assessing the environment	89

9.4	Requirements	89

9.4.1	Operational Requirements	89

9.4.2	Functional Requirements	89

9.4.3	Performance Requirements	89

9.4.4	Interface Requirements	89

9.5	Specific Design and Installation Considerations	89

ANNEX 10	HMI	90

10.1	Introduction	90

10.2	Definitions and clarifications	90

10.2.1	Definitions	90

10.2.2	Abbreviations	90

10.2.3	Supporting documents	90

10.3	HCI / MMI / User Interface	90

10.3.1	Principles	90

10.3.2	Chart display	91

10.3.3	Presentation of Vessel Data from the Data Processing Functions (e.g. Track Fusion Process, raw Sensor data, decision support functionality etc.)	92

10.3.4	Status Summary (of VTS sub systems)	92

10.3.5	Audio and Visual Alarms / alerts	93

10.4	Display Hardware and Ergonomics	93

10.4.1	Location (does this belong in this section?)	93

10.4.2	Type of layout	93

10.4.3	Environment	94

10.4.4	Reliability and fall back	94

10.5	Data Interfaces to support the Data Display System	94

10.5.1	Information held in Databases	94

10.5.2	Sensors and vessel Tracking	96

10.5.3	Meteorological and Hydrographic data	97

10.5.4	VTS asset status	97

10.5.5	Decision support tools presentation	97

10.5.6	Hazard Management	98

10.5.7	On line simulation and training facilities	98

10.5.8	Recording system and playback	99

ANNEX 11	Decision Support	100

11.1	Introduction	100

11.2	Definitions	100

11.3	Characteristics	101

11.4	Requirements	101

11.4.1	Operational Requirements	101

11.4.2	Functional Requirements	102

11.5	Specific Design and Installation Considerations	108

11.5.1	Interface Requirements	108

ANNEX 12	External Information Exchange	110

12.1	Introduction	110

12.2	Abbreviations and References	110

12.3	Characteristics of data exchange in VTS	110

12.3.1	Uses	110

12.3.2	Afloat	110

12.3.3	Ashore	111

	User data needs	112

12.3.4	Vessel data	112

12.3.5	Environmental data	112

12.3.6	Regulation and references	112

12.4	Operational, functional and performance requirements	113

12.4.1	Data integrity	113

12.4.2	Architecture of sharing	113

12.4.3	Data models	114

12.4.4	Timeliness	114

12.4.5	Storage	114

12.4.6	Access to data and information	115

12.4.7	Data security and confidentiality	115

12.4.8	Legal limitations	115

12.4.9	Communication links	116

12.5	Specific Design and Installation Considerations	117

12.5.1	Interface Requirements	117

12.5.2	Relevant technical standards	117

ANNEX 13	Verification and Validation	119

13.1	Legal requirements to type approvals	119

13.1.1	Electrical Safety	119

13.1.2	Mechanical Safety	119

13.1.3	Radiation Safety (Radar)	120

13.1.4	ElectroMagnetic Compatibility	120

13.1.5	Radio Spectrum Requirements	120

13.1.6	RoHS	121

13.1.7	Chemical Substances	121

13.2	Additional type tests	121

13.2.1	Equipment in general	121

13.2.2	Hydrographic equipment .....	122

13.3	Software Quality Assurance	122

13.4	Factory Acceptance Test	122

13.4.1	Procedure	122

13.5	Installation and Site Acceptance Test	123

13.5.1	Procedure	124

13.5.2	Scenarios	124

13.6	In Operation Test	125

13.7	In operation monitoring and calibration	125

13.7.1	Continuous monitoring	125

13.7.2	Off line test	125

13.7.3	Calibration of sensors	125

List of Figures

Figure 1-1	Example of assigned capabilities in a Generic VTS area, by the VTS authority	18

Figure 2-1	Target range and visibility	35

Figure 2-2	Coverage diagram, in normal atmosphere (left) and including an evaporation duct (right).	45

Figure 2-3	Example of simulated radar coverage in surface based + evaporation ducting conditions.	45

Figure 2-4	Coverage diagram, elevated duct	46

Figure 2-5	Coverage diagram based on a measured condition at a coastlines adjacent to hot flat deserts. The radar detection using antennas positioned inside the duct (left) and above the duct (right) corresponded to the simulated coverage diagram.	47

Figure 2-6	One hour of recordings with trials (snail tracks) shown in red. The yellow “snake” at sea is an eddy moving forth and back with a speed of app. 4 knots.	47

Figure 2-7	Dynamic characteristics of signal received versus target RCS and target range for point targets, free space	50

List of Tables

Table 1-1  Recommended availability figures	19

Table 1-2  Redundancy	21

Table 2-1 IALA target models	29

Table 2-2  Typical target characteristics.	32

Table 2-3	Targets to be detected	35

Table 2-4 Typical precipitation specification levels for VTS radar	36

Table 2-5 Sea state table	36

Table 2-6 Typical sea state specification levels for VTS radar	37

Table 2-7 Typical range performance for X-band	41

Table 2-8 Typical range performance for S-Band	42

Table 2-9 Target Separation and Accuracy	47

Table 2-10 Maximum side lobe level presented to the operator	50

Table 4-1 Indication of typical minimum accuracy	61

Table 6-1Bearing Accuracy	73

Table 9-1 Radar tracking performance parameters	85

Table 9-2 Track initiation	86

Table 9-3 Radar functions	88

Table 11-1 Decision support functions	101

Table 11-2 CPA/TCPA thresholds	102

Table 11-3 Grounding error thresholds	103

Table 13-1  Recommended equipment standards and specification levels	121

Abbreviations

| º | Degree |
| --- | --- |
|  | Plus or minus |
| > | Greater than |
| ≤ | Less than or equal to |
| ≥ | Greater than or equal to |
| µs | microsecond |
| A R and M | availability, reliability and maintainability |
| AIS | Automatic Identification System |
| AREPS | Advanced Refractive Effects Prediction System |
| ASL | Above Sea Level |
| AtoNs | aids to navigation |
| CARPET | Computer Aided Radar Performance Evaluation Tool |
| CCTV | Closed Circuit Television |
| CPA | Closest Point of Approach |
| CW | Continuous Wave |
| dB | DeciBel |
| dBi | DeciBel isotropic |
| dBm | DeciBel milliWatt |
| DF | Direction Finder |
| EIA | Electronics Industry Association |
| EO | electro optic |
| EPIRB | Emergency Position Indicating Radio Beacons |
| ETA | Estimated Time of Arrival |
| ETA | estimated time of arrival |
| ETD | Estimated Time of Departure |
| ETD | estimated time of departure |
| FMCW | Frequency Modulated Continuous Wave |
| FTC | Fast Time Constant |
| GHz | GigaHertz |
| GIT | Georgia Institute of Technology |
| GLOSS | Global Sea Level Observing System |
| GOOS | Global Ocean Observing System |
| GPS | Global Positioning System |
| HCI | human computer interface |
| IALA | International Association of Marine Aids to Navigation and Lighthouse Authorities |
| ICAO | International Civil Aviation Organization |
| IEC | International Electro-Technical Commission |
| IEEE | The Institute of Electrical and Electronic Engineers |
| IMO | International Maritime Organization |
| IOC | International Oceanographic Commission |
| IP | Ingress Protection |
| ITU | International Telecommunication Union |
| kHz | kiloHertz |
| Ku-band | 12.0 – 18.0 GHz |
| kW | kiloWatt |
| LNFE | Low Noise Front End |
| LRIT | Long Range Identification & Tracking |
| m | metre |
| m/s | metre/second |
| m2 | square metre |
| MDS | Minimum Detectable Signal |
| MHz | MegaHertz |
| mm/h | millimetre per hour |
| MMI | man machine interface |
| MMSI | Maritime Mobile Service Identity |
| MOB | Man over board |
| MRCC | Maritime Rescue Co-ordination Centre |
| MSC | Maritime Safety Committee |
| MTBF | Mean Time Between Failure |
| MTTR | Mean Time to Repair |
| N/A | Not applicable |
| nm | Nautical Mile (also nmi) |
| NMEA | National Marine Electronics Association |
| PD | Probability of Detection |
| PFA | Probability of False Alarm |
| PRF | Pulse Repetition Frequency |
| PW | Pulse Width |
| R | Range |
| RAID | redundant array of independent disks??? |
| RCS | Radar Cross Section |
| RDF | Radio Direction Finder |
| RDF | Radio Direction Finder |
| RF | Radio Frequency |
| RMP | Recognized Maritime Picture |
| SAIS | satellite AIS |
| SAR | Search and Rescue |
| SART | search and rescue transponder |
| S-band | 2.0 – 4.0 GHz (NB military designation is F-band) |
| TBA | to be advised |
| TBC | to be confirmed |
| TCPA | Time to Closest Point of Approach |
| UTC | Universal Time Co-ordinated |
| UTM | Universal Transverse Mercator |
| VHF | Very High Frequency |
| VOIP | voice over internet protocol |
| VTS | Vessel Traffic Services |
| VTSO | Vessel Traffic Services Operator |
| WMO | World Meteorological Organization |
| X-band | 8.0 – 12.0 GHz (NB military designation is I-band) |
| XML | Extensible Mark-up Language |

# Core Operational Requirements

## INTRODUCTION

In 1997 the IMO Maritime Safety Committee adopted Regulations for Vessel Traffic Services (VTS) that have since been included in SOLAS Chapter V (Safety of Navigation) as Regulation 12.This Regulation specifies the responsibilities of contracting governments to arrange for the establishment of VTS in certain vulnerable areas under their control.

The purpose of this Recommendation is to assist the VTS authority in the definition, establishment and upgrades of a VTS system. The document addresses the relationship between the Operational Requirements and VTS system performance requirements. More specifically:

- Recommended capabilities of the System vs. VTS Services provided

- Sensors

  - Radar

  - Automatic Identification System (AIS)

  - Radio Direction Finders (RDF)

  - Electro-Optical Equipment (EO)

  - Hydro/meteo

- Communications between ship and shore

- Data Processing and Display

- External Information Exchange

- Test and acceptance criteria

## Definitions and clarifications

### Supporting Documents

|  | IEEE Std 686-1997 | IEEE Standard Radar Definitions |
| --- | --- | --- |
|  | The International Organisation for Standardisation (ISO) | ISO 8729 Ships and marine technology – Marine radar reflectors |
|  | EIA Standard RS-170 | Electronics Industry Association Recommended Standard RS-170 |
|  | IALA Guideline 1056 | Guideline for VTS radar service |

## Core Operational Requirements for a VTS System

The main functions of a VTS are to mitigate risks associated with shipping and to improve efficiency. The different types of risks and environments have led to various types of VTS including coastal and offshore, port, estuary or inland VTS.

For instance a coastal VTS assist the safe and expeditious passage of shipping through coastal waters, particularly where there is a high density of maritime traffic or an area of environmental sensitivity or through difficult navigation conditions. Similarly, a port, estuarial or inland VTS support shipping when entering or leaving ports and harbours or when sailing along rivers or through restricted waters.

An important task of an offshore VTS is to avoid ships collisions with offshore structures e. g. oil platforms and wind farms.

All VTS types may offer, in principle, all services as defined in the IMO resolution A.857(20). When determining the required performance of a VTS system, the following should be taken into account:

The identified risks

The type of VTS (coastal and offshore, port, estuary or inland VTS)

The VTS services to be provided (INF, TOS, NAS)

Requirements from Allied Services

Types and number of targets

The geographical area

Prevailing meteorological conditions

### Levels of capabilities

All the above factors determine the complexity of the traffic situation. In addition, specific operational requirements such as the need to detect small targets in adverse conditions or ice monitoring, may increase the required performance.

In order to facilitate the definition of required performance, three levels of capabilities for VTS are defined as follows:

Basic – performance for a VTS area with low complexity, where an Information Service and/or a Navigational Assistance Service will be provided.

Standard – performance for a VTS area with low or medium complexity, where an Information Service, Navigational Assistance Service and /or a Traffic Organisation Service will be provided.

Advanced – performance for a VTS area with high complexity and/or specific operational requirements.

Special cases are:

Ports and inland waterways – performance for ports and inland waterways with large nearby structures, but reduced requirements to sea clutter

Offshore – performance for oil platforms and other offshore installations, typically with advanced detection requirements, but reduced  requirements to target separation.

A risk assessment and the determination of the specific operational conditions shall be made by the VTS authority prior to the allocation of capabilities

### Allocation of Capabilities to meet Operational Requirements

Requirements of the VTS equipment may have a high impact on acquisition and life-cycle costs of a VTS system and therefore is paramount to properly allocate capabilities to satisfy Operational Requirements.

A specific capability could be assigned to an entire VTS area or to particular subsections as illustrated in the example given by

Figure 1-1	Example of assigned capabilities in a Generic VTS area, by the VTS authority

Advanced capability was chosen in areas (A, B and C) for the following reasons:

- Dangerous cargo imposes a high risk to environment and populated areas

- Security, including the need for detection of small targets

- Dense traffic in a complex separation scheme including a bridge crossing

Standard capability was chosen in areas (D, E and F) for the following reasons:

- Wind farm close to a traffic lane imposes a navigational hazard

- Traffic in confined areas such as Ports and Inland waterways

Basic capability was chosen for the remainder of the VTS area (G)

## VTS System Considerations

### General

Insert note about systems architecture, stating that functions may be located on various places, e.g. that the full radar processing chain is covered by annex 2, but.....

A VTS System should have the capability to be flexible and easily upgraded and maintained alongside the routine operations of the VTS Centre without the need for interrupting the service.

Any VTS should as a minimum be equipped with automatic identification systems, voice communication and reporting facilities

Depending on the services that a VTS is to carry out, the radar coverage can be:

- nil (automatic identification systems, voice communication and reporting only)

- partly (covered areas chosen intentionally with some blind sectors)

- totally by one radar sensor (without any blind sectors)

Large VTS areas and to cover for shadow effects of other vessels, two or multiple radar sensors may be required. Stereographic processing of images from 2 or more radars may also be utilised for elimination of false (ghost) echoes.

The application of other sensors will also depend on the services that a VTS is to carry out

### Availability

Availability is defined in IMO Resolution A.915 (220 Ref.40) as:

“The percentage of time that an aid, or system of aids, is performing a required function under stated conditions.  The non-availability can be caused by scheduled and/or unscheduled interruptions”.

The recommended availability for VTS services available to the operators are defined by Table 1-1

Table 1-1	Recommended availability figures

...........................

Where the coverage from two or more radar sites provides overlapping coverage, as long as one sensor of the radars is available and provides similar capabilities in the area of waterway normally covered, the requirements for radar service are met.  Thus from a service perspective, unusable time for a waterway can be calculated as the time when any portion of the waterway is without a usable radar.

The radar coverage provided by adjacent countries, may, if agreed, be taken into account when calculating the performance of the VTS radar.  In addition, in areas of high traffic where two waterways meet with converging traffic the location of radar sensor sites should be determined in a manner that minimises shadowing.

Single coverage: Unless dual coverage is provided throughout the whole coverage area, some VTS sites provide a mix of overlapping and single radar coverage. VTS sites that provide single coverage to at least some portion of a waterway are considered to be single coverage sites.

VTS Centers should operate within a dual server environment to minimize disruption to normal operations.

#### Calculation of Availability

Administrations may choose to calculate service availability using one of two methods:

- by waterway model, or

- by  sensor site/  sensor site combination model

Waterway availability model: In this model administrations need to define which waterways are high risk and which waterways are low risks. Separate calculations for high and low risk are required, providing both exist within the coverage area. Individual waterway availability calculations are then averaged to produce one figure for each waterway risk category. If desired, a figure for each waterway may be reported.

Radar Sensor site availability model: In this model, administrations must define which sensors radar sites serve low risk waterways and which serve high risk waterways. The overall availability is calculated by averaging the availability of the associated individual radar sites as illustrated in the examples below.

Example 1: An Advanced site receives complete radar coverage. For example, a VTS area having two radar sites where at Radar site A the availability was 99% and Radar site B’s availability was 99,7%, but both radar sites suffered coincidental outages during 0,1% of the period. Although neither radar site individually met the availability target the combined availability was 99.9% and met the performance goal achieved.

Example 2: A Basic site receives coverage from radar site A (as above) covering a low risk waterway. Radar site C, adjacent to radar site A, provides completely overlapping coverage to the low risk waterway portion served by radar site A. Radar site C’s performance was 99%, but neither unusable period (of radar sites A and C) coincided. The low risk waterway covered by radar site A received usable signals for 100% of the time.

While these two distinctly different situations have different availability requirements based upon the level of risk, the concept of availability remains consistent – the minimum requirement is met for the coverage area.

Note: Higher availability percentage targets may be applicable to more critical parts of the VTS area.  Risk assessment may support lower availability percentage targets in less critical areas.

### Back up and fall-back arrangements

#### Redundancy

Error: Reference source not found specifies the recommended availability for individual radar sites and suggests the redundancy typically employed. Overlap in coverage may reduce the need for equipment redundancies at the individual radar site.

Table 1-2	Redundancy

| Availability and redundancy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Recommendation level |  |  |  |  |  |
|  |  | Basic |  | Standard |  | Advanced |  |
| Recommended availability for the individual radar site |  | 99 % |  | 99.7% |  | 99.9% |  |
| Accessibility to site |  | Easy | Difficult | Easy | Difficult | Easy | Difficult |
| Redundancy typically required | Yes |  |  |  | X | X | X |
|  | No | X | X | X |  |  |  |

### Precautionary measures to extreme events

VTS Authorities responsible for VTS in areas subject to extreme events, such as earthquake and tsunami should specify requirements to construction accordingly.

This will typically include special requirements to equipment shock resistance, alignment capabilities, civil works and power supply.

### Recording, Archiving and Replay

Provision should be made for the storage, security, retrieval and presentation of this information.

The data type, resolution and period of time for which information gathered by a VTS is required to be stored should be identified in internal procedures. This time period should be such that it allows for the full retrieval of data post-incident/accident, in compliance with national requirements and those of the incident/accident investigation procedures of the VTS authority and other authorised parties. This type of information should include:

- Communications, internal and external as defined in IALA Recommendation V-127

- Sensor data, i.e. data used to generate the traffic image such as radar, CCTV, AIS and long-range sensor data.

- Shipping information data, i.e. vessel and cargo data, including vessel movement information.

- Meteorological and hydrological data; and

- Data from other sources if relevant.

- Synchronization of voice / track data

The IMO recommends a minimum of 30 days for the time-period to allow for the full retrieval of data post-incident/accident.  The VTS authority should define the period of time and temporal resolution of sensor data and other tracking performance parameters depending on traffic density and types of tracks.

If required by the VTS Authority, the data should be recorded automatically and capable of being replayed onto a separate replay system.

Recording, Archiving and Replay

The period of time for which information gathered by a VTS is required to be stored should be identified in internal procedures. This period should be such that it allows for the full retrieval of data post-incident/accident, in compliance with national requirements and those of the incident/accident investigation procedures of the VTS authority and other interested parties. This type of information may include:

- Communications (internal and/or external)

- Sensor data (i.e. data used to generate the traffic image such as radar, CCTV, AIS)

- Shipping information data ( e.g. vessel and cargo data, including vessel movement information)

- Meteorological and hydrological data, and

- Data from other sources.

Provision should be made for the storage, security, retrieval and presentation of this information.

IALA Recommendation V-127 On Operational Procedures for Vessel Traffic Services Edition 1 June 2004

## Design, installation and maintenance considerations

### Climatic categories

Based on MIL-STD-810, IALA defined simplified climatic categories suitable for stationary VTS systems, Very Hot, Normal and Severe Cold.

Allowance for daily cycles, primarily based on variations in temperature and relative humidity levels is included.

#### Normal Climatic Category

The Normal Climatic Category covers a broad range of climatic conditions in which equipment materiel should operate and survive storage and transportation. This includes the most densely populated and heavily industrialized parts of the world as well as the humid tropics. The entire range of associated design conditions does not necessarily occur in any one place; however, a single condition (high temperature, low temperature, high humidity) occurs in a wide area. When taken together, the recommendations for the Normal Climatic Category should be valid for equipment used throughout the corresponding area.

The conditions for individual zones of the category are described as follows.

a. Humid tropic zone. Humid tropic areas are included in the Normal Climatic Category rather than being considered an extreme category because humid tropic temperatures are moderate and their humidity levels are equaled at times in some of the other mid-latitude areas. The feature of the humid tropics most important for materiel system design is the persistence of high humidity coupled with moderately high temperatures throughout the year. This combined environmental condition not only promotes corrosion, but also greatly increases insect and microbiological damage.

b. Intermediate zone. These are mid-latitude areas that do not combine higher temperatures with higher humidity throughout the year, and at the same time are not climatically extreme enough to meet the conditions for neither Hot nor Cold Climatic Categories. This zone includes the daily cycles shown in table CI, plus a condition known as "cold-wet" which can occur within the mild cold daily cycle at or near the freezing point (2 to -4°C (35 to 25°F)) with relative humidity tending toward saturation (100 to 95% RH) and negligible solar radiation.

c. Cold zone. In the Cold zone, the temperature during the coldest months, extremes may be very low; however, at open waters cold extremes below -30°C are unlikely.

#### Very Hot Climatic Category

This Climatic Category includes coastlines and inland waterways at the hot-dry low-latitude deserts of the world. During summer in these areas, outdoor ambient air temperatures above 43°C (110°F) occur frequently. However, except for a few specific places, outdoor ambient air temperatures will seldom be above 49°C (120°F). These approximate temperatures of the free air in the shade approximately 1.5 to 2 meters (about 5 or 6 feet) above the ground (in an instrument shelter).

The thermal effects of solar loading can be significant for materiel exposed to direct sunlight, but will vary significantly with the exposure situation. The ground surface can attain temperatures of 17 to 33°C (30 to 60°F) higher than that of the free air, depending on the type/colour of the ground surface, radiation, conduction, wind, and turbulence.

Air layers very close to the surface will be only slightly cooler than the ground, but the decrease in temperature with height above the surface is exponential. Temperatures at approximately 0.5 to 1 meter (about 2 to 3 feet) will be only slightly warmer than that observed in an instrument shelter at about twice that height.1

In winter, temperatures are likely to be in the same range as for the Normal Climatic Category.

Littoral regions are sometimes subject to very high absolute humidity. However, in these hot-wet areas, the highest outdoor ambient air temperatures and highest dew points do not occur at the same time.

#### Severe Cold Climatic Category

In the Severe Cold areas, the temperature during the coldest month in a normal year may be colder than -46°C (-50°F). Temperatures colder than -51°C (-60°F) occur no more than 20 percent of the hours in the coldest month of the coldest part of northern Siberia where temperatures as low as -68°C (-90°F) have been recorded. Because extremely low temperatures are not controlled by a daily solar cycle, they persist for a long enough period of time to cause materiel to reach equilibrium at extremely low temperatures.

It is recommended to seek assistance in military standards for design to such conditions.

### Considerations for determining climatic categories

#### Normal climatic considerations

All outdoor systems should be designed for at least the Normal Climatic Category, meaning that design temperatures will include the outdoor ambient air temperatures range of -30°C through +45°C. Allowance for sun radiation must be made in addition to that.

#### Extreme climatic considerations

Equipment intended to be installed and used in extreme climates (very hot and severe cold), in areas with extreme non-thermal weather conditions such as corrosive agents from oil installations, blowing sand and dust will require additional planning, design, and testing considerations.

#### Special considerations

a. Storage and transit. Environmental conditions for storage and transit modes may be more severe than those of operational modes because of the possibility of induced/combined environments (e.g., heat, humidity, shock, vibration, etc.), higher levels of some factors (e.g., high temperature in temporary open storage or during delays between transit modes), or greater materiel exposure times.

b. Design of sheltered equipment. The shelter becomes the materiel platform, and the environmental characteristics that the sheltered materiel will see depend upon the location and design of the shelter.

### Environmental capabilities and constraints

#### High wind

#### Lightning protection

### Warning lights

It may also include warning lights at the radar towers for air traffic.

### Access

### Power

### Marking and identification

All equipments should be marked with manufacturer name, type and serial number

Local or national legislation may require signposts etc.

National and/or local regulations may require the posting of signs to notify the public that they are under surveillance.

## Documentation

Documentation should be provided in accordance with the IALA VTS Manual

## Safety and security precautions

No specific safety requirements in addition to resolution A. 694(17).

Hazard analysis

--------------------------

Refer to ANNEX 13 for verification

Contract of operational Maintaining

Before the end of the guaranty period, the VTS authority may select a contract for “operational Maintaining”, including preventive and corrective procedures. Such a contract should contained specifications of maximum “none-working” period to be respected. It should also specify procedures of intervention requests and contractual times for acting.

These contracts should also specify protection against corrosion, painting, lubrification,...

Some sensors may request particular maintenance, such as frequent cleaning for the submerged sensors for instance.

# Radar

## Introduction

The purpose of this Annex is to support VTS authorities and integrators in the selection of radar sensors for new and existing VTS systems. The radar solution to a given VTS application may consist of one or more networked radar sensors. Unlike other applications, VTS radars normally need to operate simultaneously on short and long range and this leads to receiver dynamic range requirements that far exceed, for example, those required onboard a ship. However, in some cases marine navigational radar may fulfil requirements in a VTS area with low complexity or as a gap filler within a radar sensor network.

Weather-related phenomena pose significant challenges to the design and specification of radar sensors. Ducting, for example, may influence VTS radars more than ships’ radars. Additionally, more effective clutter suppression for sea, rain and land may be required than that normally associated with a conventional shipboard radar.

Specific security objectives also introduce particular challenges to the radar sensor where there could be a need to detect small targets in heavy clutter conditions or where small versus large target discrimination is essential.

The use of multi-sensor integration, including radar, AIS and other sensors, imposes additional demands on radar sensor performance and accuracy. False and inaccurate information from the, when improperly associated with information from other radars or sensors, may lead to increased risks to safe navigation and security-related decisions.  Such false and inaccurate information can arise from antenna side lobes, time side lobes, Doppler side lobes, ghost targets (multiple reflections), range ambiguous returns and inappropriate data processing.  Appropriate assessment of data integrity and accuracy from the radar sensor(s) is a vital requirement arising from the overall VTS solution.

It is recommenced that the VTS Authority should specify the Operational and associated Validation Requirements rather than Technical Specifications of radar sensor(s).

## Definitions and clarifications

### Definitions

For general terms used throughout this annex refer to:

IEEE Std 686-1997	IEEE Standard Radar Definitions.

Specific terms are defined as follows:

Azimuth (Antenna) Side Lobes - antenna responses (in azimuth) outside the intended radiation beam. Weighting of the illumination function allows a significant reduction of these lobes, but some response outside the intended direction is unavoidable, normally presenting an irregular pattern with "peaks" and "nulls". The side lobes may produce responses from targets in unwanted directions, allowing disturbing signals (intentional or not) to enter the receiver, and to make the radar detectable also by receivers not illuminated by the main beam.

Availability is the probability that a system will perform its specified function when required.

Blind Spots – typically a resulting from either blind range (the Range corresponding to an echo delay of one or more PRIs: the echo then arrives at the receiver while the radar is transmitting a new pulse and the receiver is blanked) or Blind speed (target speeds which produce Doppler shift which are integer multiples of the radar PRF, which are therefore aliased to zero Doppler and cancelled by the MTI filter. Blind spots can also arise behind significant obstructions in the field of view (buildings, land masses, oil tankers)

Chirp -  frequency modulation of the carrier frequency applied within the radar pulse to increase its bandwidth and therefore the range resolution

Coherence - capability of a system to keep a stable phase reference during the target illumination time in order to properly exploit the received phase information for MTI, pulse Doppler processing or other purposes

Doppler - shift in frequency of a wave due to the relative motion between the transmitter and the receiver. Frequency shift is target velocity/lambda. Radar echoes are shifted twice this value because this shift must be accounted for in both the forward and the return path.

Doppler Side Lobes - w hen using Doppler processing (or MTI) the integrated ideal pulse always presents a response outside the integration peak (across all Doppler filters) known as Doppler side lobes. Their main effect is to limit the capability to discriminate weak returns in proximity of strong ones (with side lobes of the same order of magnitude of the weak one).

FMCW - Frequency Modulation - Continuous Wave - A type of radar where a continuous wave instead of pulse is transmitted.  The range information is derived by frequency modulating the carrier with a saw tooth waveform and comparing the echo FM modulation envelope with the reference

Ghost Target

Normal weather and propagation conditions are the conditions persisting 99% to 99.9 % of the time as defined by the individual VTS authority.  The rest of the time is considered having adverse weather and propagation conditions.

Polarisation of a radar signal is determined by the orientation of the electrical field. In the case of circular polarisation the field rotates left or right.

Pulse – typically a pulse (modulated in the case of pulse compression radar) of RF energy transmitted from the radar

Pulse Compression – A technique used to achieve a wide pulse bandwidth (and, therefore, range resolution) using long pulse (for high pulse energy with limited peak power) by introducing an intrapulse modulation (e.g., chirp frequency modulation or Barker discrete phase modulation) and performing a correlation on the received echo.

Radar as referred to in this document relates to all aspects of the radar from sensor through to the presentation of radar to the VTS operator.

Radar Cross Section – an assessment of the cross sectional area presented by a reflector (typically a target or unwanted “clutter”) to the transmitted radar energy. The RCS can vary with frequency and  target attitude,

Radar information ....

Radar PD is the probability of detection at the output of a radar, subsequent to signal processing and plot extraction, but prior to tracking, and presentation.

Radar PFA is the probability of false alarm at the output of a radar

Radar Plots - A likely target report resulting from integration of the received echoes inside the antenna beamwidth. Each target report contains range and bearing information

Radar video – a time varying voltage signal proportional to the sum of the rf signals being received and the rf noise inherent in the receiver itself

Radar tracks - A target report resulting from the correlation, by a special algorithm (tracking) of a succession of radar reported positions (radar plots) for one object. The report normally contains smoothed position and speed vector information.

Range ambiguous returns – measured range of a target typically assumes that the target true range is less than the first range ambiguity (the Range corresponding to an echo delay of one PRI) whereas large targets beyond this range can be detected but typically with (incorrect) ambiguous range measurement

Range Side lobes – see time side lobes (below)

Reliability is the probability that a system, when it is available performs a specified function without failure under given conditions for a given period of time.

Sea characteristics include wave/swell height, direction and speed of waves/swell and distance between waves/swell.

Standard Atmospheric Condition - The International Commission of Air Navigation (ICAN) uses a definition for a standard atmosphere, defining temperature and pressure relative to the height.  In the troposphere (0 meters to 11,000 meters), the temperature is defined to be 15 °C at the surface and changing -6.5 °C/km.

Squint is the angular difference between the axis of antenna rotation and a selected geometrical axis.

Target fluctuations – (also known as Glint) - Fluctuation of a target radar cross section (RCS) (and, therefore, of the received echo amplitude) due to changes in the target attitude (for complex targets, RCS is normally strongly dependent on the angle of observation)

Track swap is the transfer of a track identity (track label) to another track

Time Side Lobes - when using pulse compression the correlated pulse always presents responses outside the correlation peak (before and after it) known as time (or range) side lobes. Their main effect is to limit the capability to discriminate weak returns in proximity of strong ones (with side lobes of the same order of magnitude of the weak one)

### IALA target models

For calculation purposes, IALA defined simplified target models per Table 2-3, modelled as fluctuating point targets.

Table 2-3	IALA target models

### Supporting Documents

| [1] | IEEE Std 686-1997 | IEEE Standard Radar Definitions |
| --- | --- | --- |
| [2] | Merrill I Skolnik | Introduction to Radar Systems, McGraw-Hill Higher Education, ISBN 0-07-290980-3 |
| [3] | P.D.L. Williams, H.D, Cramp and Kay Curtis | Experimental study of the radar cross section of maritime targets, ELECTRONIC CIRCUITS AND SYSTEMS, July 1978. Vol. 2. No 4. |
| [4] | Ingo Harre | RCS in Radar Range Calculations for Maritime Targets. http://www.mar-it.de/Radar/RCS/RCS_18.pdf |
| [5] | IMO | Performance Standards for radar reflectors (latest edition) |
| [6] | International Telecommunications Union (ITU) | ITU-R SM.1541 Unwanted emissions in the out-of-band domain |
| [7] | International Telecommunications Union (ITU) | ITU-R SM.329-9 Spurious emissions |
| [8] | The International Organisation for Standardisation( ISO) | ISO 8729 Ships and marine technology – Marine radar reflectors |
| [9] | IALA | IALA 1056 Guideline for VTS radar service |
|  | ICAN | Std Atmosphere |

### Software tools

| CARPET | Computer Aided Radar Performance Tool TNO (Toegepast Natuurkundig Onderzoek) Physics and Electronics Laboratory, P.O. Box 96864, 2509 JG The Hague, Netherlands, http://www.tno.nl |
| --- | --- |
| AREPS | Advanced Refractive Effects Prediction System Space and Naval Warfare Systems Center, San Diego, http://sunspot.spawar.navy.mil. |

## Radar System solutions

There are several technological solutions available to achieve the performance of the radar system. Most typical solutions include higher average transmission power, larger antennas, reducing the receiver noise, sector blanking and more effective processing and filtering of the transmitted and received signal.

However, the VTS Authority should keep options open and let VTS vendors propose solutions to meet the specified operational and functional requirements based on the recommendations in this document

## Radar types

VTS radars could be of the following types:

- Pulse Radar (usually Magnetron based)

- Pulse Compression radar (usually Solid State)

- Frequency Modulated Continuous Wave, FMCW (usually Solid State)

A general explanation of each radar type will follow.

### Pulse Radar

A pulse radar transmits high peak power RF pulses (10 to 50 kilowatt) for very short duration (50 to 500 nano seconds). The transmission is made with a pulse repetition frequency (PRF) of typically 1000 to 4000 pulses per second. Upon reception, the returned signal is amplified, demodulated and processed.

Main characteristics include:

- It is a well known and proven technology

- Increased pulse duration translates into longer-range detection, but poorer range resolution and less ability to penetrate adverse weather conditions.

- The need for periodic Magnetron replacement

- Fixed transmission frequency(ies)

- Requires for transmission wide frequency band(s)

Additional challenges include:

- The need to reduce out of band transmissions. Note that ITU requirements for shipborne radar are less stringent than for land based radar.

### Pulse Compression radar

A pulse compression radar transmits low peak power modulated chirps (up to 200 watts and eventually higher) with a duration of up to 100 micro seconds. The transmission is made with a chirp repetition frequency of typically 1000 to 20000 chirps per second. Upon reception, the returned signal is amplified, pulse-compressed and processed

The energy in the chirp of a pulse compression radar is comparable to the energy emitted in a pulse from a magnetron radar, The longer chirps are converted into short pulses upon reception by the process of pulse compression, therefore high range resolution can be maintained at all ranges.

Main characteristics include:

- It is based on well known and proven principles, but the high power at high frequency solid state amplifier technology  relies on recent developments

- No need for Magnetron replacement due to solid state power amplifier, reducing the need for periodic maintenance.

- Increase ability to penetrate adverse weather conditions facilitating smaller target detection

- Transmission frequencies can be programmed, which adds flexibility

- Cleaner spectrum, which means that are fewer emissions outside the allocated frequency band(s)

Additional challenges include:

- The need for sophisticated interference rejection due to the longer chirps transmitted

- The need for simultaneous short and long range detection increases complexity

- High power solid state amplifiers operates with large currents therefore requiring careful design to obtain high reliability

- By nature the pulse compression radar creates so-called time side lobes. Avoiding such side lobes, require sophisticated techniques, alternatively side lobes suppression may imply loosing small targets in the vicinity of larger targets

### Frequency Modulated Continuous Wave

Frequency modulated continuous wave radar transmits low peak power continuous wave forms (up to 50 watts). The wave forms are repeated with a typical rate of 500 to 2000 per second. Upon reception, the returned signal is amplified, compressed and processed

The energy in a frequency modulated continuous wave radar is comparable to the energy emitted in a pulse from a magnetron radar, The wave forms are converted into pulses upon reception, therefore high range resolution can be maintained at all ranges.

Main characteristics include:

- It is based on well known and proven principles

- No need for Magnetron replacement due to solid state power amplifier, reducing the need for periodic maintenance.

- The ability to detect from very short range

- Transmission frequencies can be programmed, which adds flexibility

- Cleaner spectrum, which means that are fewer emissions outside the allocated frequency band(s)

Additional challenges include:

- Dynamic limitations restrict the ability to handle small and large target simultaneously. This also affects long range detection

- Target revisit rate is low compared to typical VTS target kinematics

- The need for sophisticated interference rejection, even more than for pulse compression radars

- By nature FMCW creates so-called time side lobes. Suppressing the side lobes may imply loosing small targets in the vicinity of larger targets

## Characteristics of Radar Targets

VTS radar targets are defined by their height above sea level, their radar cross section (RCS) and the fluctuations in RCS.

Table 2-4 provides an overview of characterises for targets, typically of interest to VTS radar,  primarily based on reference [3] and [4], supplemented by experienced data obtained from IALA VTS committee members.

Table 2-4	Typical target characteristics.

| Target |  | Typical characteristics at X-band |  |  |
| --- | --- | --- | --- | --- |
|  |  | RCS | Height | Fluctuations etc. |
| A | Aids to Navigation without radar reflector. | Up to 1 m2 | 1 to 4 m ASL | Rapidly fluctuating, highly dependent on sea characteristics. |
| B | Aids to Navigation with radar reflector. | 10 – 100 m2 |  | Rapidly fluctuating, wind and currents may tilt to blind angles and lobing may cause reflectors to be in blind spots. |
| C | Small open boat, fibreglass, wood or rubber with outboard motor and at least 2 persons onboard, small speedboat, small fishing vessels or small sailing boats. | 0.5 – 5 m2 | 0.5 to 1 m ASL | Rapidly fluctuating may be hidden behind waves up to 50 % of the time. Slow moving targets tend to lie lower in the water than fast moving ones and therefore RCS visible to the radar tends to increase with speed. |
| D | Inshore fishing vessels, sailing boats and speedboats, equipped with radar reflector of good quality. | 3 – 10 m2 | 1 to 2 m ASL | Rapidly fluctuating. |
| E | Small metal ships, fishing vessels, patrol vessels and other similar vessels. | 10 – 100 m2 | 2 to 4 m ASL | Moderately fluctuating. |
| F | Coasters and other similar vessels. | 100 – 1000 m2 | 6 to 10 m ASL | RCS is highly dependent on aspect angle of the individual vessel. Rate of fluctuations is typically moderate. |
| G | Large coasters, Bulk carriers, cargo ships and other similar vessels. | 1000 – 10,000 m2 | 10 to 25 m ASL |  |
| H | Container carriers, tankers and other similar vessels. | 10,000 – 2,000,000 m2 | 15 to 40 m ASL |  |
| I | Buildings, cranes. Stacks of containers and other large structures. | Up to 1,000,000 m2 | Depends on site | Insignificant. |
| J | Floating items, oil drums and other similar items. Birds, floating or flying. | Up to 1 m2 | 0 to 0.5 m ASL | Rapidly fluctuating, highly dependent on sea characteristics. |
| K | Flocks of birds. | Up to 3 m2 | Sea level and up | Rapidly fluctuating, flight paths tend to be characteristic of given species in given areas of interest. |
| L | Jet Skis and other personal water craft | Up to 0.5 m2 | 0 to 1 m ASL | Rapidly fluctuating but virtually independent of aspect angle |
| M | Wind turbines (onshore and offshore) | Up to 1,000,000 m2 |  | Fluctuations for towers are insignificant. Rotating parts give a vide spectrum of Doppler shifts with RCS up to hundreds of m2 |

Note: Modern warship design seeks to minimise RCS and as a result the above figures cannot be related to such vessels.

RCS on targets using S-band is typically 40 % of the RCS in X-band except for small non-metallic targets where difference between the 2 bands can be much higher.

### Radar Cross Section

A target may be observed when transmitted radio energy is reflected back from the target to the receiver. The amount of energy reflected is directly proportional to the radar cross section of the target.

The RCS is defined as the ratio between the power [in W] scattered by the target back towards the radar receiver and the power density [in W/m2] hitting the target. Thus RCS is measured in m2 and has the dimensions of an area.

Note: There is no simple relationship between the physical size of the target and the RCS. The reflected energy depends on several factors, such as the radar operating frequency or frequencies, the angle of incidence of the radio waves, target speed, material and geometry.

The RCS of a target will fluctuate as a result of target movements and environmental effects with consequences on the detection and presentation of the target.

### Polarization

## Operational Requirements

It is recommenced that the VTS Authority should specify the Operational and associated Validation Requirements rather than Technical Specifications of radar sensor(s).

The operational requirements may be determined by:

- Definition of the radar coverage of the VTS area, based on possible radar location(s), the navigation characteristics, and the subdivision into requirements for Basic, Standard and/or Advanced capability

- Definition of targets to be detected

- Determination of environmental capabilities and constraints

- Determination of  other influencing factors, radar location, obstructions

- Definition of targets detection requirements in weather and propagation conditions normal for the VTS area.

- Definition of target discrimination requirements

- Definition of radar dynamic capabilities and constraints

This might be an interactive task involving iterations including evaluation of achievable performance versus overall system cost. It might for example be better to start with simpler solutions, meeting the available budget, than to be left without any radar coverage.

### The area

The VTS area should be subdivided into requirements for Basic, Standard and/or Advanced capability as discussed in ANNEX 1, paragraph 1.3.2 and the need for radar and other sensors coverage shall be defined on the basis of a risk assessment.

The selected location of radar(s) and their height should ensure that the desired coverage requirements are met.

#### Antenna height

The figures below illustrate how the height of the antenna above the water line affects the maximum and minimum detection range performance.

Careful consideration should be given to optimise radar location(s) and antenna height(s) to ensure appropriate coverage of the VTS area.

Figure 2-2	Target range and visibility

### Targets to be detected

The radar in a VTS should be capable of detecting and tracking all types of surface objects defined by the VTS authority in weather conditions normal for the individual site.

Table 2-5 lists the IALA target types to be detected for Basic, Standard and advanced Capability. Refer to Table 2-3 for IALA target definitions.

Table 2-5	Targets to be detected

In addition any special object of interest should be specified separately.

### Determination of environmental capabilities and constraints

The environmental conditions of the VTS area should be described, including the annual cycle. This should include conditions to be expected as normal over a 12-month period as well as extreme events.

Restrictions with respect to operation and access to site(s) due to weather should also be included.

It is highlighted that individual assessments for individual VTS sites must be made. A standard set of specifications for environmental requirements cannot be made to cover all VTS installations, and the tables included in this paragraph are only indicative, giving performance obtainable by traditional magnetron radars.

New technology radars may allow for additional performance within economical reach.

#### Precipitation

Information about precipitation over the VTS areas should be obtained from meteorological services, and the VTS authority should define the required radar performance requirements in clear conditions as well as for normal precipitation conditions.

Note that rising coastline tends to increase precipitation over land at some locations, and normal as well as extreme precipitation over sea may therefore differ substantially from precipitation statistics determined for land-based meteorological stations.

Table 2-6 provides typical precipitation data used in VTS radar specifications. It should be noted that intensities may exceed these values for brief periods of time, but it might not be economically feasible to design for such occasions.

Table 2-6 Typical precipitation specification levels for VTS radar

Also note that designing a system to perform in tropical rain showers will typically call for S-band radars. However, the worst rain conditions may only be present for a few hours per year, and reduction in performance on other parameters, may not justify the additional investment.

#### Sea Clutter

Numerous Sea clutter models exist, however, the CARPET models derive the mean sea clutter reflectivity from the clutter model developed by the Georgia Institute of Technology (GIT). The average wave height is derived from the hydro-graphic sea state so that fully developed sea conditions are assumed. The hydrographical sea state scale with the corresponding wind speeds and average wave heights are shown below.

Table 2-7 Sea state table

This has also been adapted for this recommendation, providing values of the clutter reflectivity under standard propagation conditions for calculation purposes.

Table 2-8 provides typical sea state requirements used in VTS radar specifications, S and X band.

Table 2-8 Typical sea state specification levels for VTS radar

| IALA Target type | Basic |  | Standard |  | Advanced |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | General | Ports and inland waterways | General | Ports and inland waterways | General | Ports and inland waterways |
| 1 | NIL |  | NIL |  | SS 4 | SS 3 |
| 2 |  |  | SS 3 | SS 3 | SS 5 |  |
| 3 | SS 3 | SS 2 | SS 4 |  | SS 6 |  |
| 4 | SS 4 |  | SS 5 |  | SS 7 |  |
| 5 | SS 5 |  | SS 6 |  | SS 8 |  |

### Other influencing factors, obstructions, interference etc.

Obstructions, e.g. topography, buildings and other man made structures may block or reflect radar signals. Other radars and sources of electromagnetic radiation may cause interference.

Inland and harbour VTS will often require special considerations as number of structures, their density and their close ranges can create very complex distortions. Additional care must be taken to assess and mitigate effects caused by natural and man made structures such as bridges, buildings, river banks, sheet metal pilings, and steep bends.

#### Shadowing effects

Radar detection may be blocked by shadowing effects that, to the extent possible, should be avoided. Such effects include:

- Targets being hidden by larger targets or other obstructions

- Masking of small targets by the effects of range and time side lobes

These effects can be minimised by the appropriate siting of radars and selection of equipment with low side lobes.

#### Multi-path effects

Ghost echoes .....

Add illustration

#### Influence from Wind farms

Wind turbines produce large static target-like returns which, from a VTS Operator’s perspective, are normally easy to distinguish from vessel traffic.

The complex return from a wind turbine is made up of two key elements

- The tower and generator housing offering a large static zero-Doppler RCS, in some cases up to 1 million square meters.

- The rotating blades of the turbine offering a complex spread of non-zero-Doppler RCS, typically up to 100 square meters, which will vary with wind direction and speed.

This composite return will be seen as a large static target by a conventional pulse radar,  whereas FMCW and coherent radars using Doppler processing will see a complex target spread across the Doppler domain.

The influence, independent of the applied radar technology, will be reflections causing unwanted ghost echoes and suppression of nearby targets. The large RCS may also result in antenna sidelobe returns, resulting in reduced detectability. The symmetrical layout of wind farms may add further to the disturbances.

Solid state radar technology (with or without Doppler processing) using pulse compression techniques may in addition suffer reduced detectability due to time side lobes and disturbing Doppler shifts.

Add illustrations

### Interference

Interference can be split into susceptibility (received interference) and compatibility (transmitted interference).

To minimise interference, separation between wanted and unwanted transmissions has to be optimised – this can be achieved by a combination of frequency separation, physical separation of transmission sites, antenna directionality, sector blanking, separation by time and also by ensuring that all the systems competing for the same or adjacent spectrum do not transmit excessive and unnecessary transmit power levels or transmit time periods. The radar receiver design will typically be very sensitive (to achieve the required performance), although gain control techniques (swept gain or STC) offer further resistance.

#### Radar susceptibility

In the case of any radar installation, (e.g. a permanent VTS installation of a radar or of each radar in a VTS network), the performance of that radar can be detrimentally affected by received emissions from other radiating sources (physically adjacent VTS radars, maritime shipborne radars, and other users (legitimate or otherwise) of the electromagnetic spectrum. Typically, local legislative regulations and restrictions will control and minimise unwanted received signals but elimination of such signals is likely to be impossible. National spectrum allocation authorities must always be approached by a VTS integrator when considering any changes to a VTS network (radar, microwave link, communications etc.) to enable a holistic view of the changes and how they might affect all users.

The radar design can assist in minimising the susceptibility to unwanted received interference, e.g. by utilising low antenna side lobes, avoiding large reflecting surfaces, minimising receiver front end bandwidth etc.

Note that FMCW and pulse compression radars may typically require larger front end receiver bandwidths than conventional magnetron systems. The multi-pulse waveforms transmitted (and consequently received) by pulse compression radars have to achieve a compromise between pulse chirp bandwidth (related to range cell size and hence range resolution), use of frequency diversity (to optimise performance in clutter), unwanted pulse to pulse interaction etc. vs. spectrum usage and hence unwanted susceptibility with other transmitting spectrum users. FMCW radars transmit and receive (at low levels) for 100% of the time across a swept bandwidth.

#### Radar compatibility with other users

In the case of any radar installation (e.g. a permanent VTS installation of a radar or of each radar in a VTS network), the performance of adjacent systems can be detrimentally affected by transmitted emissions from the radar in question (physically adjacent VTS radars, maritime shipborne radars, and other users (legitimate or otherwise) of the electromagnetic spectrum). Typically local legislative regulations and restrictions will control and minimise unwanted transmitted signals but elimination of the influence of such signals is likely to be impossible. As with susceptibility above, National spectrum allocation authorities must always be approached by a VTS integrator when considering any changes to the RF sub systems within a VTS network.

The radar design can assist in minimising the impact of transmitted signals, for example by utilising low antenna side lobes, avoiding large reflecting surfaces, minimising transmit power, consideration of peak and mean power levels, sector blanking, physical location of the radar  etc.

Note that conventional magnetron radars require larger peak power levels than pulse compression and FMCW radar systems. The magnetron technology can result in unnecessary wideband spectral emissions unless steps are taken to include frequency band pass filtering (which has an inherent loss to the wanted signal). However, pulse compression radars and FMCW radars, although utilising lower peak powers, use techniques which may include frequency modulation, pulse to pulse frequency variation, frequency diversity etc. all of which increase the use of the spectrum and increase the chances of unwanted degradation of adjacent systems.

#### VTS radar network considerations

When considering the design of a network of radars, such as that required by some complex VTS locations, radar-to-radar interoperability cannot be ignored. Although the probability of the antennas of two radars pointing directly at each other is likely to be small, the receivers should each be protected against such an event. The more likely case is when one radar antenna is pointing at the other whilst the other is pointing somewhere else – a calculation of the unwanted received power reduced by the selectivity of the own radar antenna (its peak to sidelobe ratio) is unlikely to be sufficient to avoid unwanted receiver saturation or at least “blindness”. Consequently, the VTS radar network needs to use frequency separation between the unwanted transmission and the wanted receive signals. This is typically known as frequency planning and any competent radar supplier should be able to offer to undertake a study to identify the impacts and propose a suitable transmission plan to ensure that radar interoperability is possible.

Although pulse compression and FMCW radars are typically wider band, the use of digital frequency synthesis enables these radars to offer multiple, selectable transmission centre frequencies, which support the frequency planning and therefore the interoperability of radar systems within a VTS radar network. Magnetron radars are typically tuned in the factory to a single transmission frequency.

### Target Detection requirements

The detection performance of a radar in a VTS should be sufficient to meet the requirement in the individual VTS areas, Basic, Standard and/or Advanced.

The VTS Authority shall ensure that the specified detection performance is achieved. It is strongly advised to consult experts with a sound operational and technical knowledge about the subject.

Factors affecting the detection performance of radar systems including noise, interference, clutter, and propagation shall be taken into account. Special local conditions such as heavy rainfall should be taken into account.

#### Determination of radar coverage

The recommended method for determination of radar coverage and range performance is a combination of site inspections and radar system performance calculations.

The evaluation shall include:

- Calculation of range detection performance, focused on the smallest targets of interest in poor weather conditions. All applicable losses must be included in calculations.

- Evaluation of the effects from propagation conditions and obstructions.

- An evaluation of dynamic requirements.

The calculations may be supplemented by comparison and/or validation test.

It will typically not be possible to encounter for all variables, and calculations are therefore made on the basis of a simplified model of the targets and the environment based on statistical information. It is important to understand the limitations and tolerances this entails.

#### Probability of detection and probability of false alarm

The probability of detection and false alarm rates used for calculations should comply with that required to meet the operational performance required.

Single scan probability of detection values for VTS will typically lie in the range from 0.7 to 0.9.

It is normally desirable not to have noise and clutter spikes presented to the operator in each scan. Therefore, optimal false alarm rates for VTS applications normally lie in the range from 10-4 to 10-5 for the radar video display. The false alarms taken into account in the calculations should include unwanted information from noise and clutter, as presented to the operator or to the tracker (after signal processing), but not signals from other unwanted objects.

Different values may apply for the tracking, on condition that the tracking requirements are meet.

### Calculation of radar detection performance

The achievable target detection range is highly dependant on several factors including antenna height, target characteristics, weather and atmospheric propagation conditions. In the design of radar systems it is furthermore very important not only to focus on maximum detection range but also on radar quality at all ranges, the ability to handle clutter, the ability to suppress interferences and the ability to simultaneous handling of defined (small and large) targets in the VTS area covered by radar.

Calculation for magnetron radars can be performed by the CARPET programme from TNO. CARPET may not be sufficient for determination of performance for pulse compression and magnetron radars, where it may be necessary to rely on vendor furnished information, possibly supported by performance tests.

Warning: The cumulative detection stated by CARPET can be very misleading if used for determination of VTS detection performance.

The below table includes typical examples of detection and tracking ranges for X-band magnetron based radar systems for Basic, Standard and Advanced, in standard atmospheric conditions.

Table 2-9 Typical range performance for X-band

| Antenna elevation | IALA Target type | Basic |  | Standard |  | Advanced |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Clear | 2 mm/h rain | Clear | 4 mm/h rain | Clear | 10 mm/h rain |
| 20 m ASL | 1 | NIL |  | NIL |  | 5 NM | NIL |
|  | 2 |  |  | 7 NM | 4NM | 7 NM | 6 NM |
|  | 3 | 7 NM | 4 NM | 8 NM | 5NM | 9 NM | 7 NM |
|  | 4 | 9 NM | 8 NM | 11 NM | 9NM | 12 NM | 10 NM |
|  | 5 | 12 NM | 10 NM | 13 NM | 11 NM | 14 NM | 13 NM |
| 50 m ASL | 1 | NIL |  | NIL |  | 10 NM | NIL |
|  | 2 |  |  | 10 NM | 7 NM | 12 NM | 9 NM |
|  | 3 | 10 NM | 6 NM | 12 NM | 8 NM | 14 NM | 12 NM |
|  | 4 | 13 NM | 12 NM | 15 NM | 13 NM | 17 NM | 15 NM |
|  | 5 | 16 NM | 15 NM | 18 NM | 17 NM | 20 NM | 18 NM |
| 100 m ASL | 1 | N/A |  | NIL |  | 12 NM | NIL |
|  | 2 |  |  | 13 NM | 5 NM | 16 NM | 10 NM |
|  | 3 |  |  | 17 NM | 10 NM | 18 NM | 16 NM |
|  | 4 |  |  | 20 NM | 19 NM | 22 NM | 20 NM |
|  | 5 |  |  | 23 NM | 22 NM | 25 NM | 23 NM |

It is normally sufficient to calculate detection range for small and medium size targets, therefore IALA targets type 6 and 7 are not included in the table.

The influences from sea clutter increases substantially and high elevated antennas are not applicable for Basic.

The below table includes typical examples of detection and tracking ranges for S-band magnetron based radar systems for Standard and advanced, in standard atmospheric conditions.

Table 2-10 Typical range performance for S-Band

| Antenna elevation | IALA Target type | Standard |  | Advanced |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Clear | Clear | Clear | 16 mm/h rain |
| 20 m ASL | 3 | 4 NM | 4 NM |  |  |
|  | 4 | 7 NM | 7 NM |  |  |
|  | 5 | 10 NM | 10 NM |  |  |
| 50 m ASL | 3 | 7 NM | 7 NM |  |  |
|  | 4 | 11 NM | 11 NM |  |  |
|  | 5 | 14 NM | 14 NM |  |  |
| 100 m ASL | 3 | 10 NM | 10 NM |  |  |
|  | 4 | 14 NM | 14 NM |  |  |
|  | 5 | 18 NM | 18 NM |  |  |

### Influence from propagation

The performance of surface based radar systems is strongly influenced by the electromagnetic properties of the atmosphere and the surface of the Earth. In free space, electromagnetic waves propagate in straight lines from the antenna toward the targets and back. However, radars located near the Earth’s surface must deal with, and adjust to the diffraction and refraction of the propagating wave.

Performance should, in all cases, be evaluated assuming Standard Atmospheric Conditions, combined with precipitation and sea state information for the individual location. Evaluation of the effects from adverse propagation should in addition be included for hot, dry areas of the world, e.g. the Arab Gulf.

#### Propagation in the Standard Atmosphere

An electromagnetic wave observed at a target consists of a summation of an infinite number of contributions from different paths target leading to constructive and destructive contributions at the target. For small low targets this result in shorter detection ranges, than the distance calculated by simple line of sight calculations.

In addition, the barometric pressure and water vapour content of the standard atmosphere decreases rapidly with height, and the temperature decreases slowly with height. This causes the electromagnetic waves to bend a little towards the Earth’s curvature.

Radar parameters, losses in transmission lines (not only waveguide), processing losses, clutter and precipitation add to the complexity and, even for the Standard Atmosphere, it is necessary to combine this with propagation factors by radar calculation tools in order to determine the expected performance for a VTS radar station.

#### Sub-refraction and Super-refraction

Sub-refraction, bending the electromagnetic waves up, and super-refraction, bending the electromagnetic waves down, exists when the atmosphere deviates from the standard.

Sub-refraction can be caused by fast reduction of temperature and slower reduction of water vapour content with height, bending the electromagnetic waves towards space. However, this phenomenon occurs rarely in nature.

Super-refraction (can be caused by temperature increase with height (generally by temperature inversion) and/or rapid decrease of water vapour with height, decreasing N. Decreasing the refractivity gradient will eventually cause it to reach the critical gradient, at which point an electromagnetic wave will travel parallelly to the Earth’s curvature.

#### Ducts and Trapping Layers

Super-refraction will develop into trapping layers, if the refractivity gradient decreases beyond the critical gradient, at which point the electromagnetic wave will be trapped and follow the Earth’s curvature.

Ducts act like waveguides for propagating waves bordered by trapping layers or the Earth’s surface. To couple into and remain in a duct the angle of incidence must be small typically less than 1°.

Ducting can be categorized into three main types:

- Evaporation duct:

  - Weak, caused by evaporation from the sea surface, and only at low levels (maximum of 40 meter ASL).

  - Generally increasing the radar horizon, especially for low mounted antennas.

- Surface-based duct:

  - Surface ducts caused by low level inversion (increase of temperature /decrease of humidity with height), up to 500 meters ASL.

  - Increase of radar range, depending on duct and antenna height.

- Elevated duct:

  - 0.2-2 km above the surface

  - Typical no effect on surface-based antennas.

The effects are typically increased range but also increased amounts of noise and 2nd / multiple time around returns which may appear as falls radar returns..

Notice that the electromagnetic waves are refracted (bent) and not reflected by the trapping layers.

##### Evaporation Ducts

Evaporation ducts exist over the ocean to some degree, almost all the time. A change in the moisture distribution without an accompanying temperature change will lead to a trapping refractivity gradient. The air in contact with the ocean surface is saturated with water vapour, creating a pressure that is decreasing to some value above the surface.

This will cause a steep refractivity gradient (trapping) near the surface, but gradually equalise towards normal refractivity gradient at a certain height, which is defined as the evaporation duct height.

Evaporation ducts are generally increasing the radar detection range and the antenna can be located above the duct and still have extended propagation strength.

For typical coastal radar installations, evaporation duct heights of 6 – 15 meters lead to the longest detection ranges. Evaporation duct heights of more than 10 meters will also introduce an increased amount of clutter, setting additional demands to clutter processing and noise reduction capabilities.

I.e. investigations of weather data from the Arab Gulf area show that evaporation ducts exist all the time with typical duct heights of 5 to 25 meters, resulting in increased radar range in 80% of the time and increased clutter in 50% of the time.

Figure 2-3	Coverage diagram, in normal atmosphere (left) and including an evaporation duct (right).

##### Surface-based Duct

Surface based ducts can be much stronger than evaporation ducts. They occur when the air aloft is hot (and dry) compared to the air at the Earth’s surface. Over the ocean and near land masses, dry continental air may be advected over the cooler water surface; either directly from leeward side of continental land masses or by circulation associated with sea-breeze.

Figure 2-4	Example of simulated radar coverage in surface based + evaporation ducting conditions.

In addition to the temperature inversion, moisture is added to the cool marine air by evaporation, increasing the trapping gradient. In costal areas, this leads to surface trapping ducts. However, as you travel on the open ocean, this trapping layer may well rise from the surface thereby creating an elevated duct.

The electromagnetic wave will be refracted towards the surface of the Earth and be trapped in the duct like in a waveguide. This kind of trapping condition greatly increases the surface detection range - and the amount of noise received. Note that surface detection may occur far beyond the radar horizon with a “dead zone” in between.

Surface based ducts are often combined with evaporation ducts and examples of radar performance in such conditions, as illustrated by Figure 2-4.

##### Elevated Duct

Generally, elevated ducts occur from descending, compressed and thereby heated air, from anticyclones, approaching the marine boundary layer and causing ducts. Elevated ducts may also occur from elevating surface-based ducts.

Surface detection might also occur in this case far beyond the radar horizon with a “dead zone” in between, adding noise to the radar image.

Figure 2-5	Coverage diagram, elevated duct

##### Severe ducting at coastlines adjacent to hot flat deserts

The large temperature variation between night and day in desert areas and the associated pressure differences between land and sea tend to cause very strong temperature inversion during night time and result in strong sea breeze in the afternoons. This can result in very severe ducts. This type of duct can be very low, as little as 15 meters has been experienced

Range performance is very different for an antenna positioned inside or above these ducts, and radar systems with an antenna positioned within such a duct will have substantial increase in the detection range for surface targets. If the antenna is positioned above the trapping layer (outside the duct), the electromagnetic wave will be refracted and the detection range for surface targets will be substantially reduced.

Figure 2-6	Coverage diagram based on a measured condition at a coastlines adjacent to hot flat deserts. The radar detection using antennas positioned inside the duct (left) and above the duct (right) corresponded to the simulated coverage diagram.

The sea breeze also cause eddies over the sea, forming distinctive sea clutter patters. The eddy results in a “snake” like pattern moving forth and back for a few hours in the afternoon on hot days with strong sea breezes (see Figure 2-7). Of course, this will disturb display and tracking, if uncompensated.

Figure 2-7	One hour of recordings with trials (snail tracks) shown in red. The yellow “snake” at sea is an eddy moving forth and back with a speed of app. 4 knots.

### Target Discrimination

In normal weather and propagation conditions, surface objects within the VTS area should be separated in presentation, and individually tracked without track swap, at any applicable target speed when they are positioned apart and with distances as defined by the individual VTS authority.

The system should be designed in such a way that the defined radar accuracy and discrimination can be achieved in the entire area(s) covered by the VTS service. In long measuring distances, the impact of the height and type of antenna on the measuring accuracy and resolution should be taken into account.  The system should also be capable of displaying and tracking all targets of interest simultaneously in normal conditions, preferably without the need for manual adjustments by the operator.

Table 2-11 provides separation and accuracy requirements for point targets, suitable for Basic, Standard and Advanced capability.

For larger (non-point) targets the definition of separation is highly dependant on aspect angles, pulse stretch etc.  Proper transmitter and receiver (dynamic) characteristics, rather than specification of absolute separation, will ensure proper separation of such targets.

The special case of offshore installations will normally require target separation equivalent to Basic capabilities, whereas other operational requirements are Advanced

Table 2-11	Target Separation and Accuracy

| Radar accuracy and physical separation between small point targets for discrimination in display and tracking |  |  | Capability |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Basic |  | Standard |  | Advanced |  |
|  |  |  | Display | Tracking | Display | Tracking | Display | Tracking |
| In range |  | Short range applications (<5 nm coverage – include waterways, harbours etc) | 25 m | 40 m | 20 m | 30 m | 15 m | 25 m |
|  |  | Long range applications (up to 20 nm coverage – littoral waters, offshore etc) | 75 m | 100m | 60 m | 75 m | 50 m | 60 m |
|  |  | Very long range applications (>20 nm coverage) | N/A |  | 100 m | 125 m | 80 m | 100 m |
| In azimuth | X-band | Angle between targets as seen from the radar | 1.2o | 1.3o | 0.7o | 0.8o | 0.55o | 0.6o |
|  |  | Or distance in meters, whichever is the greater | 25 m | 40 m | 20 m | 30 m | 15 m | 25 m |
|  |  | Corresponding –3 dB antenna horizontal beam width | ≤0.7o |  | ≤0.45o |  | ≤0.4o |  |
|  | S-band | Angle between targets as seen from the radar | N/A |  | 3.5o | 4o | 1.8o | 2o |
|  |  | Or distance in meters, whichever is the greater |  |  | 20 m | 30 m | 15 m | 25 m |
|  |  | Corresponding –3 dB antenna horizontal beam width |  |  | ≤2o |  | ≤1.25o |  |

### Radar dynamic capabilities and constraints

#### Dynamic requirements

The dynamic range of the radar should, in normal weather and propagation conditions, detect and process the surface objects specified. This should be done without significant side lobes, degradation of target appearance, degradation of detection and reduction of separation.

The dynamic range is determined by:

- The ratio between the largest nearby objects expected and the smallest distant objects to be detected

- Target fluctuations

Requirements to the radar(s) can be determined from the characteristics of the objects in the coverage area of the individual radar. Table 2-4 summarise the characteristics of objects typically considered and the corresponding dynamic range, as a function of RCS and detection range, can be determined from Figure 2-8

Figure 2-8	Dynamic characteristics of signal received versus target RCS and target range for point targets, free space

The figure represents targets in free space. This is normally sufficient for the determination of VTS radar requirements when combined with 10 dB allowance for target fluctuations and lobing.

Add harbour case and 100.000 m2 to drawing

#### Side lobes

Antenna and range side lobes should be sufficiently low to avoid false targets, especially false targets far from real targets. Table 2-12 recommends the  maximum allowed signal present to the operator, originating from range and/or time side lobes

Table 2-12 Maximum side lobe level presented to the operator

|  | Recommendation level |  |  |
| --- | --- | --- | --- |
|  | Basic | Standard | Advanced |
| Maximum near side lobe level, within +/- 10 in azimuth and +/- 250 meters in range from any target | - 50 dB | - 55 dB | - 58 dB |
| Maximum far side lobe level, outside +/- 10 in azimuth and +/- 250 meters in range from any target | - 65 dB | - 68 dB | - 70 dB |

Azimuth (antenna) side lobes normally are the only contributor in case of traditional  magnetron pulse radar. FMCW and pulse compression radars are subject to time as well as azimuth side lobes.

Note that antenna side lobes are attenuated during transmission as well as reception. The level (in dB) presented to the operator will therefore in general be half of that specified for the antenna, however, other factors may contribute.

Also note that structures near antennas (in some cases up to 100 meters) tend to interfere and increase the azimuth side lobe level in the direction of such structures.

## Functional Requirements

The radar service in a VTS should as a minimum support the operational functions specified by ANNEX 9 as well as the functions listed below.

These functions may be hosted by the radar sensors or other parts of the VTS system

### Operational outputs

The output from a radar service should include radar image and track data.

### Operator functions

Radar functions should be designed and implemented to optimise performance and minimize operator workload to the level practical. Ideally, the operator should only need to control basic functions such as start and stop.

Each radar site should be designed and equipped to reduce the adverse effects of rain and sea clutter and enhance the probability of target detection and it is recommended to make adaptation to changing weather conditions, etc. automatic to reduce operator workload, However, it might be necessary to implement dedicated operational modes e.g. for adaptation to weather.

Manual override of automatic functions should always be possible

### Clutter and noise reduction / Management

Appropriate, clutter reduction or clutter management should be available to meet the performance requirements.

This will typically include:

- White noise suppression

- Interference rejection

- Sea and rain clutter processing

- Adaptation to varying propagation conditions.

The features should preferably be automatic for systems requiring standard or advanced capabilities.

### Elimination of false echoes

The radar should also be designed and installed so as to eliminate, to the maximum extent possible, false echoes caused by side lobes, reflections from nearby structures or second/multiple around echoes.

### Built-in test features

Built-in test features should include monitoring of functions and performance.  It is recommended that results are made accessible for remote monitoring, especially for radars installed in locations that are difficult to access.

### Service access

To the extent practical, service access should be possible at the individual radar station and from remote.

A local service display at each radar sensor, presenting radar information is desirable, the kind of such information is to be decided by the VTS authority.

## Radar design, installation and maintenance considerations

### Safety precautions

Rotating machinery

Radiation hazards

Electrical shock

### Access

### Design

Interface, data communication requirements

WG types, length, losses

WG dehumidifiers

Maintenance

Windload on antennas

4.5	Free sight

The diameter of the platform, the height of the fence, the height of the pedestal and the vertical beam width are all factors affecting the free sight. As a rule of thumb use 1.5 times the lower limit of the vertical beam width (see Fig. 4.9) - in this angle there must be free sight. In blanked sectors or directions of low practical importance this rule may be ignored.

Normally the height of the pedestal is variable to achieve free sight.

Also concerning the lightning conductor TERMA recommends to place this in a blanked sector or in a direction of low practical importance. Due to the tiny size of the rod this will normally not have any practical influence on the radar picture.

When the antenna is mounted, ensure that no equipment or material is blocking the antenna’s free rotation before turning on the antenna motor.

5.1	Mounting the waveguide

It is recommended to use a support for at least every 60 cm and for every 40 cm in critical bends. Do not bend the elliptical waveguide more than allowed by a minimum bending radius of 200 mm (E plane) and 400 mm (H plane) and do not twist it more than 3º per meter.

Be careful when the elliptical waveguide is cut. Humidity and foreign bodies (especially copper dust and copper swarfs) inside the waveguide will cause decreased performance.

#### Extreme events

Authorities responsible for VTS areas subject to adverse weather such as cyclones, typhoon, hurricane, and tornado should consider the potential impact and specify requirements to equipment survival. Radar operation is normally suspended in such conditions.

Refer to Annex 1 section 1.4.4 for general precautions

--------------------------

Refer to ANNEX 13 for verification

# Automatic Identification System

## Introduction

AIS is intended to enhance safety of life at sea, the safety and efficiency of navigation, and the protection of the marine environment.  In addition, AIS may contribute to maritime security.  SOLAS Regulation V/19 requires that AIS exchange data ship-to-ship and with shore-based facilities.  Therefore, the purpose of AIS is to help identify vessels; assist in target tracking; simplify information exchange (i.e. reduce ship reporting using radiotelephony); and provide additional information to assist situational awareness.  In general, data received via AIS will improve the quality of the information available to the VTSO or OOW.  AIS is a useful source of supplementary information to that derived from other navigational systems and sensors, including radar

### Application

These performance requirements are for the use of AIS in a Vessel Traffic Service (VTS).  AIS should improve the safety of navigation by assisting in the efficient navigation of ships, protection of the environment, and operation of VTS, by satisfying the following functional requirements:

- in a ship-to-ship mode for collision avoidance

- as a means for littoral States to obtain information about a ship and its cargo, and

- as a VTS tool, i.e. ship-to-shore (traffic management)

AIS should provide ships and competent authorities, information from the ship, automatically and with the required accuracy and frequency, to facilitate accurate tracking.

Mandating AIS carriage and establishing a service to receive, process and distribute the AIS signals received from vessels enhances safety and security and improves the ability to manage traffic.

Some shore facilities may need to act on the information, others may need to monitor AIS and maintain an information database.  For these reasons, a nationwide or regional network may be set up.

The service should also be capable of information exchange and distribution among several users ashore and afloat.  Government agencies, allied services and commercial maritime interests may have justifiable needs for AIS data.

In order for VTS to take full advantage of AIS, access to the capabilities of an AIS Base Station is required.  This access should preferably be through an AIS service.  With this access, the VTS may change the reporting rate or AIS channel, send short safety-related messages, or perform other functions as necessary

### Objectives of AIS

AIS shall:

- provide information automatically to appropriately equipped shore stations, other ships and aircraft, including the ship’s identity, type, position, course, speed, navigational status and other safety-related information;

- receive automatically such information from similarly fitted ships;

- monitor and track ships;

- exchange data with shore based facilities;

- assist in ensuring the highest possible level of safety and efficiency for vessel traffic in the designated area.

The requirements should not be applied to cases where international agreements, rules or standards provide for the protection of navigational information.  AIS should be operated taking into account the guidelines developed by the International Maritime Organization.

## Abbreviations and References

### Abbreviations

AIS		Automatic Identification System

ASM 		AIS Service Management

BSC		Base Station Controller

FATDMA	Fixed Access Time Division Multiple Access

LSS		Logical Shore Station

OOW		Officer of the Watch

PSS		Physical Shore Station

SOTDMA	Self Organising Time Division Multiple Access

VTSO		Vessel Traffic Services Operator

### References

| [1] | IMO MSC. 74(69), Annex 3 | IMO Recommendation on Performance Standards for a Universal Shipborne Automatic Identification System (AIS) |
| --- | --- | --- |
| [2] | SOLAS Convention | Chapter V Safety of Navigation, Regulation 19 |
| [3] | ITU Radio Regulations | Appendix S18, Table of Transmitting Frequencies in the VHF Maritime Mobile Band |
| [4] | ITU-R M.1371-1 | ITU Recommendation on the Technical Characteristics for a Universal Shipborne Automatic Identification System (AIS) Using Time Division Multiple Access in the Maritime Mobile Band |
| [5] | IEC Standard 61993 Part 2 | Universal Shipborne Automatic Identification System (AIS) Operational and Performance Requirements, Methods of Testing and required Test Results. |
| [6] | IALA Technical Clarification | IALA Technical Clarifications of Recommendation ITU-R M.1371-1 |
| [7] | IALA Recommendation A-124 | IALA Recommendation on AIS Shore Stations and Networking Aspects Related to the AIS Service |
| [8] | Resolution A.694(17) | General Requirements for Shipborne Radio Equipment forming Part of the Global Maritime Distress and Safety System (GMDSS) and for Electronic Navigational Aids |
| [9] | Resolution A.953(23) | World-Wide Radio navigation System |
| [10] | Resolution A.688(17) | Code on Alarms and Indicators |
| [11] | Resolution MSC.39(63) | Adoption of amendments to the Code on Alarms and Indicators |
| [12] | IALA Recommendation V-125 | IALA Recommendation on the Integration and Display of AIS and other information at a VTS Centre. |

## Characteristics

Automatic Identification System (AIS) is a system that makes it possible to monitor and track ships from suitably equipped ships, and shore stations.  AIS transmissions consist of bursts of digital data ‘packets’ from individual stations, according to a pre-determined time sequence.  AIS data consists of shipboard information such as position, time, course over ground (COG), speed over ground (SOG), heading, etc.  AIS uses a broadcast and interrogation technology that operates ship-to-ship and ship-to-shore and includes limited communication capabilities.

Shore stations receive the same information from AIS equipped ships within VHF range. .

The International Maritime Organization (IMO) has established carriage requirements for merchant ships.  The International Telecommunication Union (ITU) has defined the technical characteristics and ratified the global frequencies.  In addition, the International Electrotechnical Commission (IEC) has developed methods for testing AIS for global interoperability.

AIS  makes navigation safer by enhancing situational awareness and increases the possibility of detecting other ships, even if they are behind a bend in a channel or river or behind an island in an archipelago.  AIS also solves the problem inherent with radars, by detecting smaller craft, fitted with AIS, in sea and rain clutter.

## Equipment

AIS equipment is described in detail in IALA Recommendation A-124 on Automatic Identification System (AIS) Shore Station and networking aspects relating to the AIS Service.

### Class B

### Limited base stations

## Operational requirements

The AIS service should provide timely, relevant and accurate information to users to support the decision-making processes of a VTS.  The AIS service may also support port operations by providing information to appropriate shore facilities.  It provides automatic vessel position reports and movement information as it is received at remote sites throughout the service area. In support of incident response, the AIS service, in conjunction with the port authority, can provide information about traffic and the corresponding situational information.  Utilizing the AIS Service, the VTS will monitor safety and security zone boundaries that are established in connection with an incident. The AIS service also provides information to allied services to support their tasks.

a. AIS information needs of the VTS.  Basic information needs for vessel tracking and port security related missions are:

- Up-to-date knowledge regarding the route to be transited.

- Timely, relevant, and accurate information about vessels within the area that might affect safety, security, or the decision making of the VTSO.

- Timely information about emergency and environmental conditions that might affect safety or the decision making of the VTSO.

- Where required, the transmission of relevant information to the mariner in a manner that does not distract from the task at hand, particularly in narrow, confined channels where there is heavy traffic.

AIS, as well as existing aids to navigation and tools, pilotage systems, navigation management systems, and regulations provide information to the mariner but these systems require integrity monitoring to ensure the information they impart is accurate.

b. Integrated Operations.  Where applicable, AIS should support regional Vessel Traffic Services between adjoining VTS Centres.

c. Incident Analysis Support.  The AIS service should support incident response and analysis.

- Immediately after notification of an incident, the AIS service should alert vessels in or planning to enter the area of concern with particulars of the incident.

- The AIS service may provide for review of supporting information and events that occurred before, during, and after an incident.  Otherwise, the VTS should have the ability to support recording and replay of AIS data.

### Use of virtual MMSI for VTS communication

Addressing of AIS units is accomplished via the Maritime Mobile Service Identity (MMSI).  Every base station will have a unique MMSI number. However, when a VTS is operating several AIS Base Stations, it can present a single address, known as a virtual MMSI number.  This will enable the ships to send and receive AIS messages in the VTS area using only one MMSI regardless of the number of base stations in use.

This functionality has to be supported through the AIS Network.  However, shipboard AIS units can not currently respond to messages from virtual MMSIs.

### Short safety-related messages

Short safety-related messages are free format text messages.  They can be addressed either to a specified destination (MMSI) or broadcast to all ships in the area.  Their content should be relevant to the safety of navigation, e.g. an iceberg sighted or a buoy not on station.  Such messages can contain a maximum of 158-162 characters.  Although unregulated, these messages should be kept as short as possible.

Cautionary note: The VTSO should not assume that all short safety-related messages have been read onboard.

### Use of binary messages for data exchange

IMO has selected seven binary messages, as shown below, to be used as a trial set.  The intention is to use this set for a period of 4 years (May 2004-May 2008) with no change.

The messages are:

- meteorological and hydrological data

- dangerous cargo indication

- fairway closed

- tidal window

- extended ship static and voyage related data.

- number of persons on board

- pseudo-AIS targets.

For further details, see IMO SN/Circ.236, dated 28 May 2004, Guidance on the Application of AIS Binary Messages.

### Use of the VTS Targets message

One of the IMO approved binary messages is for broadcast pseudo AIS targets.  This contains information on non-AIS targets derived by means other than AIS. Typically these will be targets that are tracked by a VTS-system based on radars. Sometimes the VTS Targets message is also referred to as VTS Footprint.

The AIS message for VTS Targets can be filled with information for a maximum of 7 targets.  At present, IMO restricts this to a maximum of 4.  Each target comprises:

- Target id (MMSI, IMO number or Callsign)

- Latitude

- Longitude

- Course (COG)

- Speed (SOG)

- UTC time stamp

A VTS Targets message can either be sent as an addressed message to one target or as a broadcast message to all targets that are inside the VHF coverage area of one or more shore stations (i.e., PSS).

For additional information, refer to IMO SN Circ/236 – Guidance on the application of AIS Binary messages.

### Assigned Mode

VTS may use the AIS Service capability to change the reporting mode (from autonomous to assigned mode, for example) of selected shipboard AIS units.  This will enable the ship station to operate according to a specific transmission schedule, as determined by a competent authority.

## Functional requirements

For VTS purposes, an AIS service provides AIS information from one or several base stations to users. In addition to vessel data, an AIS service provides status on AIS equipment and management functions for the control of the AIS network.  The AIS Service may consist of one or more PSS or AIS Networks, or a combination of both.

## Graphical presentation

In the VTS Centre, AIS-data is usually viewed on an electronic chart, either separately or combined with the radar data.

### Symbol usage

The IALA Guidelines on AIS include a description of the recommended AIS target symbols, but these were originally intended for the onboard ECDIS/ECS systems. As the guidelines point out, the given symbols are not necessarily adequate in the VTS context. The main difference is that a VTS-operator may prefer a much wider range of information than is necessary onboard a ship. For example traffic management may necessitate the use of symbols which depict different types and sizes of vessels. Further it may be necessary to show which vessel have pilots embarked, and which do not.

### Interaction with radar tracks

A target that is tracked by radar and also carries an AIS transponder may be displayed with one symbol based on a fusion of the dynamic information received from the two sensor types. The user may have the option to display the input from each sensor with two different symbols. It should also be possible to identify which sensor(s) are used to derive the target position.

## Data Validity

The validity of AIS data received from ships is dependent on the proper installation of AIS, correctly interfaced and functioning ship’s equipment, and correct manual input of static and voyage-related data.

Until further regulations dictate stricter data accuracy requirements in the AIS mobile units, caution has to be taken when using AIS data for processing.

VTS may contribute AIS service data to government agencies, allied services and commercial maritime interests.  This determination will be made by the relevant Competent Authority.

## AIS design, installation and maintenance considerations

### Transmission Layer

AIS is an unencrypted broadcast system and as such, its data is accessible to anyone with a suitable receiver.  National regulations may regulate the use of AIS frequencies. AIS may be susceptible to interference from adjacent channels.  Due consideration should be given to frequency allocations adjacent to AIS channels to avoid possible service disruption.

### Coverage aspects

In general, AIS design coverage ranges should approximate VHF voice communication ranges.  However, actual vessel traffic density or geographic considerations (i.e., mountains or other VHF occlusions) may determine the need for additional base stations

When estimating the size of the operational coverage (operational cell) for shore facilities, an important consideration is the traffic load – number of mobile AIS stations within the area.

For example, calculations in one port have indicated that an AIS Base Station could accommodate less than 300 active AIS units.

For further information, please refer to IALA GUIDELINES ON THE AUTOMATIC IDENTIFICATION SYSTEM (AIS) Volume 1, Part II – Technical Issues, Edition 1.1

## Interfacing

The VTS will select features of the AIS network it requires via the AIS Service.  These features are the Basic AIS Services defined in IALA Recommendation A-124.

# Environmental Monitoring

## Introduction

The protection of the environment is an issue that has grown significantly in importance, both politically and socially, over recent years.  The environmental impact of normal commercial activities is analysed such that risk reduction measures can be determined and implemented.  Traditionally, VTS Authorities have collected environmental data simply to support their VTS activities however, many VTS Authorities have their VTS areas in, or alongside, marine protected areas or Maritime Reserves where any damage to the environment would potentially result in a considerable media interest and political ramifications.

Therefore, the modern VTS should consider two aspects of environmental monitoring:

- Navigation Data Collection

- Environmental Protection

Navigation Data Collection includes the traditional environment monitoring sensors, typically referred to as the hydrological / meteorological (hydro/meteo) systems. Typical meteo variables are those provided by weather stations and include air temperature and humidity, wind velocity and direction, and visibility. In certain locations, hydro variables such as tidal level, and current direction and velocity may also be required.  Hydrological data may be obtained through sensors or available in tables/databases from national authorities.  Sensors providing this data, usually located at remote sites (databuoy) and communicate data to a Vessel Traffic Services Centre (VTS centre) via a telecommunications or radio link.  Alternatively, wave height, direction and current could be derived from the main VTS radar through software processing.

Environmental Protection would include implementing additional capability that would protect the nearby environment from any polluting incidents that may be caused by visiting vessels.  This could be achieved though the software processing of the VTS radar signals or by specialist sensors that are designed to detect oil, or other pollutants, in the water.

At the VTS centre, graphical and/or numeric information is presented for use by the operators.

### Scope

The aim of Annex 5 of this recommendation is to:

- identify functional and operational requirements for environmental data in VTS; and

- provide guidance on design and installation of such equipment

### Objectives

Hydrological, meteorological, Oil Spill and any other environmental information, that the VTS Authority requires, may be integrated into VTS applications to provide the operator a real-time assessment of the environmental situation in the VTS area of responsibility.  Information collected from this equipment can be provided to ships to assist in assessing the conditions.

A number of countries operate tide gauges and current meters to assist the prediction of tidal heights and streams or for the broadcast of real-time information to shipping.  The Intergovernmental Oceanographic Commission (IOC) is responsible for co-ordinating the Global Sea Level Observing System (GLOSS) program to establish global and regional networks of sea level stations for providing essential information for international oceanographic research programmes.  GLOSS operates under the Global Ocean Observing System (GOOS) http://gosic.org/ios/GOOS-Main-Page.htm.  IALA supports and encourages participation in the GLOSS program.

## References

There are many applicable IMO, IEC, WMO and other requirements.  These include, but are not limited to:

| IMO Resolution A.686(17) | Code on Alarms and Indicators (and MSC.39(63) Adoption of amendments to the Code on Alarms and Indicators) |
| --- | --- |
| IMO Resolution A.694(17) | General Requirements for Shipborne Radio Equipment forming Part of the Global Maritime Distress and Safety System (GMDSS) and for Electronic Navigational Aids |
| IMO | SOLAS (i.e. Chapter V, Regulation 12) |
| IEC 529 | Degrees of protection provided by enclosures (IP Code) |
| IEC 721-3-6 | Classification of environmental conditions |
| IEC 60945 | Maritime Navigation and Radio communication Equipment and Systems |
| WMO | International Meteorological Vocabulary Guide to Meteorological Instruments and methods of Observation |
| NMEA 0183 | Standard for Marine Electronic Devices (IEC equivalent) |

### Definitions

For general terms used throughout this document refer to the World Meteorological Organization (WMO).

## Characteristics of Environmental Sensors in VTS

The hydro/meteo sensors used for VTS systems are usually installed at VTS centres or other related sites such as radar sites however they might also be sited on third party sites e.g. a navigation buoy or lighthouse. These sensors measure parameters such as wind speed/direction, barometric pressure, air temperature and current speed/direction  in real time. The hydro/meteo data is then transmitted to the VTS centre and presented to the VTSO in order to support the safety navigation of vessels. In cases of severe weather conditions this information is particularly important.

## Functional Requirements

This section states what the hydro/meteo system should be able to do and which sensors should be integrated into the VTS system.

Measurements are transmitted by the telecommunication system to a VTS centre and displayed in user-selectable format. The measured data is to be presented both numerically and graphically ( in chronological order) and is then stored for at least one year, depending upon the VTS authority. It is essential that a VTS Centre has access to local hydro/meteo information relevant to the VTS area(s) and can, if required by the VTS Authority, disseminate this to their users and allied services.

A VTSO centre can pass the measured hydro/meteo information to vessel(s) in the VTS area(s) by an appropriate radio communication link.

## Operational Requirements

Briefly, operational requirements are qualitative and quantitative parameters that specify the desired capabilities of a system and serve as a basis for determining operational effectiveness.

Operational requirements may include ergonomics, operational controls and information presentation.  Due to the varied nature of hydrological and meteorological equipment in VTS, it is not possible to specify ergonomic or operational control requirements.  – see CI in Appendix 3.

### Information Presentation

The results of the measurements should be transmitted in WMO standard units and displayed in user-selectable format.

The measurements should be available to VTS operators through an integrated display or separate instruments.  Data may be presented numerically and/or graphically.  A log of the latest 24 hour measurements should be available to the VTS operators either numerically or graphically.

### Malfunctions and Indicators

As a minimum requirement, malfunctions, warnings, alarms and indicators should respond to the requirements of IMO Resolution A.686(17) and MSC.Circ.39(63).  Additional requirements may be required, depending on the individual type / purpose of sensor.

### Accuracy

Where a VTS Authority determines a need to establish their own monitoring stations, it should be noted that the individual VTS Authorities should determine the accuracy and availability requirements for each VTS Centre, as these will be based on individual circumstances. Error: Reference source not foundgives an indication of typical measuring range and minimum accuracy requirements.

Note: The target availability should be as prescribed by IMO A.915(22).

Table 4-13 Indication of typical minimum accuracy

| Parameter | Measuring Range | Minimum Accuracy | Remarks |
| --- | --- | --- | --- |
| Height of Tide | 0 to 10 m |   2% |  |
| Rate of Tidal Stream/ Current | 0 to 300 cm/s |   1% |  |
| Direction of Tidal Stream/Current | 0 to 360 degrees |   5 degrees |  |
| Wave height | 0 to 10 m |  0.5m for 5m   10% for >5m |  |
| Wave Direction | 0 to 360 degrees |   20 Degrees |  |
| Wind speed | 0 to 75 m/s |  0.5 m/s for 10 m/s  5% for >10 m/s |  |
| Wind Direction | 0 to 360 degrees |   3 degrees |  |
| Visibility | 10 to 20,000 m |  50 m for ≤ 600 m  10% for > 600 m – ≤ 1 600 m  20% for > 1500 m |  |
| Air Temperature | -10 to +50 degrees |   0.3 degrees | Hot, cold climate category area should be specified the measuring range. |
| Air Humidity | 0 to 100% RH |   2% RH |  |
| Air Pressure | 920 to 1080 hPa |   0.3hPa |  |
| Sea Surface Temperature | -2 to + 40 degrees |   0.1 degrees | Hot, cold climate category area should be specified the measuring range |
| Ice Coverage | - | - | Measured by satellite remote sensing |
| Ice Thickness | - | - | Measured by satellite remote sensing |
| Oil spill | - | - | Measured by satellite or radar remote sensing |
| Salinity | 0 to 70 PSS |   1% |  |

The VTS Authority should specify the time periods over which the various data parameters should be updated and may be averaged, if required, as these factors will depend upon the local circumstances pertaining to the VTS Centre.

#### Reliability, accuracy, range, resolution and units

The reliability, accuracy, range, resolution and units of the measurements should satisfy the minimum requirements as determined by WMO.

### Technical requirements

For hydro/meteo systems within a VTS system measurement sensors should be installed and located according to recommendations from a VTS authority in consultation with hydrologist/meteorologist(s).  The Sensor identification and location should be provided.

The measurements/sensors may include:

- Wind speed / Wind direction / Wind gust

- Air temperature / Relative humidity

- Precipitation

- Barometric Pressure (atmospheric pressure)

- Visibility

- Water temperature / Water level

- Height of tide

- Current speed (may be required at various depths)

- Current direction (may be required at various depths)

- Wave height / direction

- Ice coverage / thickness

- Ice coverage

- Salinity

- Oil spill

## Design, installation and maintenance considerations

Key aspects to design and installation include:

- Durability and resistance to environmental conditions

- Interference

- Power supply requirements /options

- Installation

- Maintenance, and

- Technical challenge

### Durability and Resistance to Environmental conditions

Electronics installed externally should be in an environmental enclosure.  IEC requirements for environmental conditions should be applied as practicable.

### Interference

These sensors comply with applicable international standards and regulations.  IEC requirements (IEC 60945) refer.

### Power Supply Requirements / Options

Relevant IEC requirements should be applied.  In remote locations, due to the low power, consumption of hydro/meteo sensors, authorities should consider use of alternative power (e.g., solar panels, wind vanes, etc.), in lieu of generators, when commercial power is not available.

### Installation

Requirements concerning the installation of sensors, wiring and the arrangement of the equipment providing hydro/meteo information to the VTS centre should be determined.  Operational requirements will determine where sensors are to be located and how many are required.  Sites for sensors should be selected based upon optimizing data relevant to the VTS.  Other considerations include:

- availability of power,

- protection against vandalism,

- housing and collocation with existing VTS, AtoN, or other suitable infrastructure

Relevant IEC requirements should be applied.  For example:

- IEC 529 "Degrees of protection provided by enclosures (IP Code)"

- IEC 721-3-6 "Classification of environmental conditions, Part 3: Classification of groups of environmental parameters and their severities; Ship environment"

- IEC 60945 "Maritime navigation and radio communication equipment and systems - General requirements, methods of testing and required test results"

The requirements of environmental condition should be decided by the VTS authority and referred to Annex 1, 1.5 General design, configuration installation and maintenance consideration.

### Maintenance

Possible requirements, in addition to IMO Assembly Resolution A.694(17) concerning maintenance, should be determined. Citing considerations for sensors should include maintenance, repair, and accessibility requirements.

### Technical Challenge

The new sensing technology has been developed using satellite remote sensing or radar remote sensing technology for detecting ice coverage area, density of surface ice or oil spill area, wave height/direction and so on. The VTS system should be prepared to use emerging technology in order to maintain or improve services.

### Interfacing

The data to be interfaced for the hydro/meteo service are described under ‘Functional Requirements / Sensors’.

For the interfacing of hydro/meteo services to VTS equipment, several different standards are in use.  Among those standards, the Standard for Marine Electronic Devices, NMEA 0183, has been applied for these applications.  In addition, the WMO has developed an interface standard for hydro/meteo applications.

For the interface between a VTS and its users, hydro/meteo data should follow standardized data exchange formats, e.g., XML.  (In addition, a time stamp and source should be provided.)

### Backup Arrangements

Depending on the individual type of the equipment, requirements concerning back-up and fall-back arrangements should be determined.

### Safety Precautions

Depending on the individual type of the equipment, requirements in addition to IMO Resolution A.694(17) should be determined.

# Electrooptical equipment

## Introduction

CCTV information may be integrated into VTS applications to provide the operator a real-time assessment of the situation in the VTS area of responsibility.  Information collected from this equipment can be provided to ships to assist in assessing the waterway conditions.

## References and abbreviations

### References

Electronics Industry Association (EIA) Recommended Standard RS-170

Relevant SOLAS requirements; SOLAS Chapter V (Safety of Navigation) as Regulation 12

General resolutions, such as resolution A.694(17) - General Requirements for Shipborne Radio Equipment forming Part of the Global Maritime Distress and Safety System (GMDSS) and for Electronic Navigational Aids;

Resolutions with more specific requirements, such as:

NMEA 0183 – Standard for Marine Electronic Devices (IEC equivalent 60945)

IEC 529 "Degrees of protection provided by enclosures (IP Code)"

IEC 721-3-6 "Classification of environmental conditions, Part 3: Classification of groups of environmental parameters and their severities; Ship environment"

IEC 60945 "Maritime navigation and radio communication equipment and systems - General requirements, methods of testing and required test results"

To be formatted identical to previous Annexes

### Abbreviations

As defined by Electronics Industry Association (EIA), Motion Picture Experts Group (MPEG) and Joint Photographic Experts Group (JPEG).

To be specific o this Annex - such reference does not help much

## Characteristics

Cameras may be used as a more cost effective solution to radar or in conjunction with radar as an additional sensor, depending on the level of risk.  CCTV may be effective in a limited area not covered by other sensors or provide supplemental information with other sensors (e.g., identification).

The camera may be capable of automatically tracking a vessel selected by the VTS Operator. In addition, there should be the possibility for the VTS Operator to manually de-select any acquired target and manually select another target and/or area, in order for the VTS Operator to perform specified monitoring tasks, such as pilot embarkation and disembarkation.

Where more than one camera is installed to cover a VTS area, it may be desirable for the output from multiple cameras to be provided in one composite picture.

VTS authorities should consider the need for low-light level, infrared, colour, intensified and laser-gated low light level, as well as digital image processing and video compression of CCTV installations.

Characteristics to be considered with regard to CCTV installations include resolution, focus uniformity, shading, lag/image retention, spectral sensitivity, blooming, and light sensitivity.

### Reliability, accuracy, range, and resolution

The reliability, accuracy, range, and resolution should satisfy the minimum requirements of the VTS and as determined by EIA and MPEG.

More specific guidance for VTS ?

### Malfunctions, warnings, alarms and indications

Refer to relevant requirements of Resolution A. 686(17) and Resolution MSC.39(63).

More specific guidance for VTS ?

## REQUIREMENTS

### Functional Requirements

#### Operational controls

The camera may be controllable manually in pan, tilt, and zoom modes.  The extent of this control will be dependent on geography of the waterway, camera installation site, etc.  Camera housings are generally equipped with means for keeping the lens operational (e.g., heaters, lens wipers, protective coating, and washers).

#### Ergonomics

Regarding CCTV controls, one should consider the VTSO position and the use of the CCTV.  It is desirable for the CCTV to be controlled from a single operator position (e.g., touch screen, mouse click, and joystick for CCTV slewing and automatic tracking).

### Operational Requirements

#### Detection Performance

Depending upon the circumstances, cameras should allow the VTSO to identify the type and possibly the name of the vessels concerned.

The type of vessel should be capable of being identified normally at a minimum range of 3 nm from the camera location, but this will depend on the individual circumstances, including local topography. The identity of a vessel by shape, colour and other features should be capable of being determined normally at a minimum range of 1 nm, but again this will depend on the individual circumstances. These ranges are based on a nominal visibility in excess of 10 nm.

#### Recording and Replaying of Data

The data should be recorded automatically and capable of being replayed, if required by the VTS Authority.  Replay of CCTV should not interfere with ongoing operations.  This may require a separate display system for playback.  The VTS Authority must determine the quality of recording and playback (e.g., frames per second, resolution, period).

#### Software requirements

The software requirements for CCTV are a matter for the VTS Authority to determine.

#### Availability

The requirements for the availability of CCTV are a matter for the VTS Authority to determine.

Part of annex 1?

## Specific design, installation and maintenance considerations

### Durability and resistance to environmental conditions

Electronics installed externally should be in an environmental enclosure.  As regards environmental conditions, the IEC requirements should be applied as far as relevant.

### Interference

CCTV equipment is industrial equipment and therefore complies with applicable international standards and regulations.  Refer to the IEC requirements (IEC 60945) as far as relevant.

### Power supply

IEC requirements should be applied as far as relevant.  In remote locations, authorities should consider use of alternative power (e.g., solar panels, wind vanes, etc.) in addition to generators and/or uninterruptible power supplies.

### Site selection and Installation

Requirements concerning the installation of CCTVs, wiring and the arrangement of the equipment in the VTS Centre should be determined.  Operational requirements will determine where CCTVs are to be located and how many are required.  Sites for CCTVs should be selected based upon optimizing the views of areas relevant to the VTS.  Other considerations include:

Note:  Special care must be given to the stability of the camera mounting and preventing glare from direct sunlight.  It may be appropriate to consider the use of software to mitigate unavoidable vibration.

### Maintenance

Requirements in addition to IMO Resolution A.694(17) concerning maintenance should be determined if necessary.

Is an IMO resolution appropriate for terrestrial equipment

Part of Annex 1 ?

### INTERFACING

For the interfacing of CCTV services to VTS equipment, several different standards are in use e.g., EIA, MPEG, JPEG.  A time stamp and source should be provided for recording and playback purposes.

### BACK-UP AND FALL-BACK ARRANGEMENTS

Redundant cameras of the same type are not normally installed in VTS.  However, different camera types may be collocated and can provide backup.

Part of Annex 1 ?

### SAFETY PRECAUTIONS

No specific safety requirements in addition to resolution A.694(17).

Part of Annex 1 ?

## CLOSED CIRCUIT TV AS A VTS SENSOR (CCTV)

### General

The performance requirements of a CCTV service vary depending on traffic density, type of VTS service, special regional features, coverage of the VTS area and the intended use.

VTS Authorities should consider the need for low-light level, colour, intensified and laser-gated low-light level, as well as digital image processing and video compression of CCTV installations.

### Detection Performance of CCTV Cameras

Depending upon the circumstances, cameras should be capable of identifying the type and possibly the name of the vessels concerned.

The CCTV should enable identification of the type of vessel at a minimum range of 3 nm from the camera location, but this will depend on the individual circumstances, including local topography. The identity of a vessel by shape, colour and other features should be capable of being determined normally at a minimum range of 1 nm, but again this will depend on the individual circumstances. These ranges are based on where the nominal visibility is in excess of 10 nm. Image quality and update/refresh rates should meet these requirements.

### Characteristics of the CCTV

Cameras may be used either in low traffic density areas where this is a more cost effective solution to radar or in conjunction with radar as an additional sensor, depending on the level of risk.

The camera should be capable of automatically tracking a vessel manually selected by the VTS Operator via the VTS Screen, if required by the VTS Authority. In addition, there should be the possibility for the VTS Operator to manually de-select the automatically acquired target and manually select another target and/ or area of the acquired target, in order for the VTS Operator to perform specified monitoring tasks, such as pilot embarkation/disembarkation.

Where more than one camera is installed to cover a VTS area it is desirable for the output from both to be fused together to provide one composite picture.

### Availability

The requirements for the availability of CCTV are a matter for the VTS Authority to determine.

Part of Annex 1 ?

# Radio Direction Finders

## Introduction

RDF is a sensor system that supports VTS and SAR operation by indicating the direction/bearing to a VHF transmitting device during time of transmission. Two or more appropriately located RDF are needed to estimate and display the position of a ship, e.g. calling on channel 16 on a VTS console. This may be used to correlate with a radar target to identify a ship not equipped with AIS and therefore assist the evaluation of the traffic image. Because of its co-operative structure RDF is not suitable of being used for continuous tracking.

## Operational Requirements

It is recommended that the VTS Authority should specify the Operational and associated Validation Requirements rather than Technical Specifications of RDF sensor(s).

The operational requirements may be determined by definition of:

- The RDF coverage area, based on possible RDF location(s), the navigation characteristics, and the subdivision into requirements for Basic, Standard and/or Advanced capability

- The frequency range of RDF equipment including SAR frequencies if necessary

- The number of simultaneously monitored VHF channels

- Detection requirements as a function of:

- Types of ships to be detected (with regard to the radio range the antenna height is of interest)

- Meteorological conditions

- Required bearing accuracy parameters

- Other influencing factors, RDF location, obstructions

### RDF coverage Area

The VTS area should be subdivided into requirements for Basic, Standard and/or Advanced capability as discussed in ANNEX 1, paragraph 1.3.1 and the need for RDF and other sensors coverage shall be defined on the basis of a risk assessment.

In order to ensure accurate identification on the VTS display the use of two or more separate RDF bearing stations are required. Bearing angles on the target should be as close to 90º as possible

### Frequency range

Whereas the main purpose of RDF is detection of VHF communication devices, the frequency range of RDF should correspond to the frequencies used for VHF communications. Additionally standard SAR frequencies (121.5 MHz, 243 MHz and 406 MHz) may be required if SAR operations should be supported by VTS.

### Number of simultaneously monitored VHF channels

Depending on its hardware configuration, RDF can simultaneously work on single or multiple frequencies. This achieved by use of one or several VHF receivers (typically 4-8).

The single-receiver RDF can be switched to any VHF channel at any time. It can be done manually or automatically in so-called scanning mode when RDF automatically scan pre-defined list of VHF channels.  Thus, several VHF channels can also be monitored using single-receiver RDF but not simultaneously.

There also may be a need to simultaneous monitor several VHF channels. For example, SAR channels and VHF channel 16 may require to be monitored continuously while all other working VHF channels selectively. In such situation use of multi receivers RDF may be required.

### Detection performance

The detection performance of a RDF in a VTS should be sufficient to meet the requirement in the individual VTS areas, Basic, Standard and/or Advanced.

The VTS Authority shall ensure that the specified detection performance is achieved. It is strongly advised to consult experts with an operational and technical knowledge about the subject.

Factors affecting the detection performance of RDF systems including interference and propagation shall be taking into account. Special local conditions such as heavy rainfall should be taken into account.

The recommended method for determination of RDF coverage and range performance is a combination of site inspections and RDF system performance calculations.

The evaluation shall include:

- Calculation of VHF Radio Range based on RDF antenna height and minimal  VHF antenna height on the target of interest

- Calculation of all applicable losses (target’s VHF transceiver power, required RDF sensitivity, losses in VHF cable etc.)

- Evaluation of the effects from propagation conditions and obstructions

- Influence of meteorological conditions

The calculations may be supplemented by comparison and/or validation test.

### Bearing accuracy

One of the most important parameter of the Radio Direction Finder subsystem is the bearing accuracy. Besides the technical characteristics of the RDF equipment, many other factors may significantly deteriorate the bearing accuracy in real conditions. Therefore, the following aspects should be taken in account when planning bearing accuracy requirements:

- RDF equipment  bearing accuracy  - typically specified by RDF manufacturers for “near to ideal” conditions.

- DF antenna installation environment. Multipath signal propagation  caused by reflection from surrounding objects can significantly deteriorate the bearing accuracy (refer to sub-heading “Specific Design and Installation Considerations)

- Received signal strength. Low level of received signal may significantly deteriorate the bearing accuracy. The major factors affecting received signal strength are:

- Distance to the target

- DF receiver(s) sensitivity and RDF antenna feed losses

- Weather conditions

- Output power and duration of transmitted signal

- Acceptable delay between signal detection and output for presentation. This delay is caused by internal processing of received signal within RDF to achieve declared accuracy and is typically about 1-2 seconds.

Quality of factory and on-site RDF calibration. Proper calibration may help to compensate the factors, which adversely affect the bearing accuracy and achieve better performance.

The recommended bearing accuracy for the types of capability are provided in Error: Reference source not found

Table 6-14Bearing Accuracy

|  | Type of Capability |  |  |
| --- | --- | --- | --- |
|  | Basic | Standard | Advanced |
| Recommended Bearing Accuracy |   5º |   3º |   2º |

6.1.6	Environmental capabilities and constraints

Influence of weather conditions to detection range and accuracy

6.1.7	Other influencing factors, obstructions etc

Influence of surrounding environment and construction to accuracy

6.1.8	Calibration

Requirements for factory and on-site RDF calibration

## Functional Requirements

### VHF channel(s) management

There are two types of RDF systems available on the market: single channel receiver RDF containing one receiver and multi channel receivers RDF containing two or more receivers for simultaneous receive and processing on different frequencies.

Selecting the appropriate RDF type is determined by the operational requirements (see above)

Single channel receiver RDF should as a minimum include:

- Remotely controlled selection of VHF channel

- Automatic channels scan function from pre-defined list of working channels

- Prioritization of SAR channels in  scanning mode

Multi channel receiver RDF should as a minimum include

- Remotely controlled selection of VHF channels for each receiver

- Automatic channels scan function from pre-defined list of working channels for one or arbitrary number of receivers

- Simultaneous output of detected bearings for all receivers

### SAR functionality

In case VTS is purposed to assist in SAR operations, corresponding functionality of RDF equipment may be required. Depending on the tasks, the following functionality may be requested:

- Detection of devices transmitting on SAR frequencies

- Automatic filtering of ELT tones of MOB EPIRB devices

- Receiving and decoding of COSPAS\SARSAT digital data including coordinates and output of decoded data to VTS

### Man Over Board EPIRB detection capabilities

This capability of RDF equipment ensures detection of specific standardized ELT codes transmitted by EPIRB devices. This functionality ensures minimizing the probability of false alarm caused by spurious transmissions on SAR frequencies.

### COSPAS\SARSAT detection and decoding

This capability of RDF equipment ensures receiving and decoding of digital data transmitted by COSPAS\SARSAT radio beacons. Received  data contain identification  number and the measured geographic coordinates of the radio beacon, which can be used for planning of SAR operation.

### Build In Test and diagnostic

Built-in test features should include monitoring of functions and performance.  It is recommended that results are made accessible for remote monitoring, especially for RDFs installed in locations that are difficult to access.

## RDF design installation and maintenance considerations

### RDF antenna installation

The design of a complete RDF system requires careful consideration especially with regard to the DF antenna installation site. The following aspects should be considered when planning RDF antenna installation place:

- RDF antenna should be placed on a very stable support to avoid any rotation or torque as this directly affect RDF performance

- Antenna height should be enough for detection of the VHF transmitting device in interest having minimal VHF antenna height

- Any object causing shadowing in the RDF sector of interest should be avoided

- Any rotating\moving objects (like radar antenna, PTZ CCTV etc.) should be placed on safe distance from the RDF antenna according to RDF manufacturer recommendations (or maybe include some general figures into this doc???)

- Safe distances from RDF antenna to big reflecting objects (like buildings, masts, cranes, forest etc.) should be kept according to RDF manufacturer recommendations (or maybe include some general figures into this doc???) .

- Surrounding landscape (mountains etc.) may significantly decrease the RDF bearing accuracy and distance performance due to multipath signal distribution. So, the landscape around the installation place should be as flat as possible.

### Lightning protection

Typically, the RDF antenna is placed on very top of the mast, so the special attention should be paid to the antenna’s lightning protection. From one side lightning protection should ensure reliable protection of RDF and other installed on the mast equipment, from other side not affect RDF performance causing re-reflection or shadowing of incoming VHF signal.

# Long Range sensors

## Introduction

VTS equipment provides the VTS Authority with real time data from short range line of sight sensors such as radar and AIS. On occasions the use of information derived from long range sensors (typically long range radar, satellite communications systems and satellite AIS) can provide supplementary information for the VTS or Coastal Authority.  It may assist in locating vessels that have not arrived on schedule or detect vessels that arrive unannounced.  This may assist the authorities in assessing any security risks or in other situations may provide input data for search planning should a rescue be required.

Typical Long Range Sensors include:

- LRIT (Long Range Identification & Tracking)

- Satellite AIS

- HF Radar

- Satellite based Synthetic Aperture Radar (SARSAT)

This section will provide an overview of each of the above and identify the benefits, limitations and applicability of these sensors to VTS and Coastal Authorities.

## Long Range Identification & Tracking (LRIT)

LRIT is a mandatory carriage requirement for SOLAS vessels.  It provides a position report at regular intervals based upon the area of operation.  The normal reporting interval is every 6 hours.  LRIT data is received by International Data Centres (IDC) and is available to the flag authority and to the maritime authorities of transit and destination countries.

Based upon the normal reporting cycle, LRIT data may up to 6 hours old when received and therefore not as accurate as the real time data received from the primary sensors such as radar and shore-based AIS.  However, in circumstances where a vessel has arrived unexpectedly or gone missing, the LRIT information could provide additional information to assist in a security assessment or to enable search planning activities to be initiated.

### Specific design, configuration, installation & maintenance considerations

LRIT is an established service that does not require any special design, configuration or installation on the part of the VTS Authority as these are handled by the existing LRIT infrastructure.  The VTS Authority may need to be granted access permission by the national maritime authority and allocated a username by the International Data Centre.  Once access has been granted the VTS Authority will be able to integrate the LRIT data with the VTS Traffic Display as appropriate for his operational requirements.

It should be noted that LRIT data normally carries an airtime cost per position report and that the IDC may charge for the provision of the data.

As the applications for LRIT data continue to evolve, other uses and benefits may be determined and implemented.

## Satellite AIS

An AIS receiver in a satellite will extend the coverage range considerably and make it possible to monitor ship traffic beyond the range of terrestrial AIS receivers and radars.  The Satellite AIS data will be provided through a Service Provider to which the VTS Authority will need to subscribe, possibly for a monthly fee.

The satellite service listens to the AIS environment within its footprint area and stores the data on board the orbiting satellite until it passes over a ground station where it can download the data.  This will mean that the data received by the VTS Authority will not be real time and may be 1 – 2 hours old (possibly more).

In areas with high ship traffic density the sheer number of AIS messages being received at the same time cause data collisions from different self-contained resolution cells occupying the same time-slot. Advanced de-collision algorithms are employed by the different service providers of satellite AIS data but in a high traffic density situation, it may be impossible to reliably extract accurate data.

Satellite based AIS data is now becoming increasingly available via  commercial as well as national government sponsored satellite AIS systems.. The operational systems comprise  several satellites in different constellations, i.e. polar-orbiting constellation or a mix of equatorial and polar orbiting satellites.  The effect of different orbiting constellations will be the availability and location of Ground stations to receive the satellite AIS data.  The more frequently the service can download the data, the less latency between the received data and the real time position of the actual vessels.

### Specific design, configuration installation and maintenance considerations

Satellite AIS is an established service that does not require any special design, configuration or installation on the part of the VTS Authority as these are handled by the satellite AIS Service Providers.  The VTS Authority may need to subscribe to a service on offer from a satellite AIS Service Provider.  Once the service provision has been agreed and implemented, the VTS Authority will be able to integrate the satellite AIS data with the VTS Traffic Display as appropriate for his operational requirements.

As the applications for satellite AIS data continue to evolve, other uses and benefits may be determined and implemented.

The main difference between the terrestrial and satellite AIS data, besides the geographic coverage, is the data latency, i.e. the age of the AIS message when it is actually received by the VTS system.   Aside from the signal path to and from the AIS satellite receiver, a delay is also introduced when the received signal is decoded. Currently two methods of decoding are in use:

## HF Radar

One rarely used technology that can offer long range detection of vessels is HF radar. This has one major advantage over other long range detection technologies in that it does not require cooperation from the vessels to be detected. There are generally two types of HF radars, those that use the low level earth surface “hugging” refraction duct and those that use reflection from the layer to layer boundaries in the ionosphere above the earth (skywave).

Both system types suffer from unpredictable propagation path characteristics which can support medium and large object detection (metal ships) to hundreds of nautical miles in some conditions but often offers very little detection performance. This makes specification of achievable performance and detection “availability” a challenge to both radar customers and radar suppliers. The vagaries of the propagation paths can also introduce unpredictable positional measurement errors affecting both angle and range even when an object is clearly detected.

The HF radar installation requires some careful selection of suitable coastal terrain which may not suit all potential VTS locations. Similarly, suitable sites will rarely support the necessary infrastructure (power, communications, access for installation and maintenance) and these need to be factored into the installation and operational costs.

Optimising the nature of HF radar may impose high workload on specialised, highly trained operators.

If the limitations are acceptable, this technology offers valuable passive detection in open waters, expensive to obtain by other means (airborne sensors and satellite).

## Synthetic Aperture Radar (SARSAT)

Satellite based Synthetic Aperture Radar (SARSAT) can provide vessel target information at ranges beyond that of shore based sensors, including HF Radar. However, such services will probably only provide a single image of a specific area once per day through a few orbiting satellites.  Images are stored on board the satellite until they can be downloaded as the satellite passes over a ground station.  The image is processed, following download from the satellite, to detect ships within the area and radar information (without identity) can be derived that can be used to recognise the type of vessel.  This type of service is for analysis of vessel movement and not for any form of near real time monitoring.  In addition to the latency between the required image capture and the download when passing over a ground station, there is also a further latency related to the processing of the received data.  The application is useful for detecting illegal fishing activity within the deep water areas of a country’s Exclusive Economic Zone and for detecting oil spills and pollution.

### Specific design, configuration, installation & maintenance considerations

SARSAT is available from a variety of established service providers and does not require any special design, configuration or installation on the part of the VTS Authority.  The VTS Authority will need to subscribe to a SARSAT image service and costs are involved on a per image basis.  Once access to such a service has been established, the VTS Authority will be able to integrate the SARSAT target data with the VTS Traffic Display as appropriate for his operational requirements.

# Radio communications in VTS

## Introduction

Radiocommunication equipment is typically integrated into VTS applications to provide the operator with a real-time assessment of the situation in the VTS area of responsibility as well as a means to deliver timely services to VTS participants.  Information collected and disseminated via this equipment can assist in assembling the traffic image and in supporting safe navigation of the VTS area.

## References

Relevant SOLAS requirements;

- SOLAS Chapter IV (Radiocommunications)

- SOLAS Chapter V (Safety of Navigation) – Regulation 12

- SOLAS Chapter V (Safety of Navigation) – Regulation 19

IALA World Maritime Radio Communications Plan Edition 1

Resolution A.694(17) - General Requirements for Shipborne Radio Equipment forming Part of the Global Maritime Distress and Safety System (GMDSS) and for Electronic Navigational Aids;

IEC 529 "Degrees of protection provided by enclosures (IP Code)"

IEC 721-3-6 "Classification of environmental conditions, Part 3: Classification of groups of environmental parameters and their severities; Ship environment"

IEC 60945 "Maritime navigation and radiocommunication equipment and systems - General requirements, methods of testing and required test results"

## Characteristics of Radiocommunication equipment

Radiocommunication is the central ingredient in the operation of VTS.  Radiocommunications links are used to collect position, safety, and general information from shipboard personnel and remote sensing devices. These links are also the primary means through which services are delivered to VTS participants.

Radiocommunication equipments are adapted to guarantee the coverage of the GMDSS ( www.imo.org):

- Area A1 Within range of VHF coast stations with continuous DSC (digital selection calling) alerting available (about 20-30 miles)

- Area A2 Beyond area A1, but within range of MF coastal stations with continuous DSC alerting available (about l00 miles)

- Area A3 Beyond the first two areas, but within coverage of geostationary maritime communication satellites (in practice this means Inmarsat). This covers the area between roughly 70 deg N and 70 deg S.

- Area A4 The remaining sea areas. The most important of these is the sea around the North Pole (the area around the South Pole is mostly land). Geostationary satellites, which are positioned above the equator, cannot reach this far.

Components of the VTS radiocommunication equipment may include:

Very High Frequency (VHF) voice radio - It is usual  practise for the VTS to have its own independent VHF network using specifically designated VHF Channels.  This network may comprise of one or more VHF Channels in different sectors of the VTS Area.  The VTS Authority may require specific VHF Channels to be designated by the National Radio Authority for specific types of operations.  Specific channels are determined to provide safety watch, DSC, and VTS information. The VHF equipment must comply with national and international regulations, particularly with the Master Plan of shore-based facilities for GMDSS.

Medium Frequency (MF) and High Frequency (HF) voice and data – MF and HF may be used on a regional basis where long range communication is required.   The VTS Authority may require specific channels to be designated by the National Radio Authority for specific types of operations.  The equipment must comply with national and international regulations. Exceptionally communication via Sat-com might be required, depending on geographic terrain, shoreline of country and service provided by the VTS.

## REQUIREMENTS

The shipborne equipment must meet the functional requirements of the relevant IMO performance standards and the ITU-R Radio Regulations.

### Radiocommunications Coverage

VTS Radiocommunications shall be in accordance with ITU-R Radio Regulations.  Depending upon the circumstances, radiocommunication equipment should be capable of receiving signals from appropriately equipped ships.  VHF voice radio reception is generally dependent upon the line-of-sight distance between VTS receive site and the ship antenna heights.   This is also true with AIS, although there are performance differences between VHF voice radio and AIS.  As a minimum requirement, the radiocommunications range should facilitate VTS ship communications before the ship enters a VTS area of responsibility.

### Recording and Playback of Data

Radiocommunications data should be recorded automatically and capable of being replayed onto a separate replay system, if required by the VTS Authority. A time stamp and source should be provided for recording and synchronised playback purposes.

### Malfunctions, warnings, alarms and indications

Refer to relevant requirements of Resolution A.686(17) and Resolution MSC.39(63).

### Availability

The requirements for the availability of radiocommunication equipment are dependent on the mandate of the VTS Authority.  GMDSS Master Plan???????a matter for the VTS Authority to determine.

## Specific Design, Installation & Maintenance Considerations

### Durability and resistance to environmental conditions

Electronic equipment  installed externally should be in an environmental enclosure.  As regards environmental conditions, the IEC requirements should be applied as far as relevant.

### Interference

Radiocommunications equipment complies with applicable international standards and regulations - see IEC 60945 which covers the general requirements for navigation and radio equipment and includes interference..  Equipment should be installed in accordance with manufacturer’s instructions and specifications to avoid interference.

### Power supply

IEC requirements should be applied as far as relevant.  In remote locations, authorities should consider use of alternative power (e.g., solar panels, wind vanes, etc.) in addition to generators and/or uninterruptible power supplies.

### Site selection and Installation

Requirements concerning the installation of radiocommunciation equipment, wiring and the arrangement of the equipment in the VTS Centre and in remote sites should be determined.  Operational requirements will determine where radiocommunication transceivers and antennas are to be located and how many are required.  Sites for radiocommunication equipment should be selected based upon optimizing the coverage of the VTS area.    Care must be taken when co-locating antenna sites that proper separation is maintained.  To avoid channel saturation, consideration should be given to subdividing the VTS area into communications sectors based upon channel use with adjacent sectors using separate channels.  Other considerations include availability of power, physical security of the site, , housing, and co-location with existing VTS, AtoN, or other suitable infrastructure.

### Maintenance

In addition to the requirements of IMO Assembly Resolution A.694(17), the siting and installation of radiocommunication equipment should make provision for accessibility, maintenance and repair.

## INTERFACING

Although there is an internationally agreed interface standard for interfacing electronic equipment on board ships, there is however no specific standard for VTS radiocommunication ashore. The interface standard will thus be dependent on the requirements of the VTS authority and the equipment being installed.  .

### BACK-UP AND FALL-BACK ARRANGEMENTS

The provision of redundant radiocommunication equipment is recommended via the use of duplicated equipment in case of equipment failure or UPS/generator in case of electrical power failure.  Additionally, consideration should be given to a business continuity plan such as handing over to another VTS and maintaining a portable radio facility such as satcom or mobile telephone..

# Data Processing

## Introduction

This annex describes the collection and processing of sensor data to establish and maintain the VTS Traffic situation and the related environment. The following data processes  are typical:

- Maintaining the traffic situation

  - Collecting sensor data

  - Tracking and Data fusion

  - Target identification

  - Target data augmentation using data from VTS Databases

- Assessing the environment

  - Weather Conditions

  - Water Conditions

  - Seabed/riverbed profile

## Abbreviations and References

## Characteristics

### Maintaining the Traffic Situation

#### Introduction

A typical VTS will use a variety of sensors to collect information necessary for its operation. Some sensors require cooperation of the target to obtain a measurement. Depending on the systems, available on-board, a vessel may or may not be able to cooperate in the measurement process. A vessel that is able to do so is a cooperative target. However, if it is not able or not willing to do so, it is referred to as non-cooperative target. If a target is cooperative, the measurement data may originate from the target itself. In that case, the sensor is called a dependent sensor, because the target provides the data.

#### Collecting Sensor Data

##### Radar

Plot extraction (see IALA Guideline 1056) should be automatic.  The plot extraction process should be able to handle a minimum of plots per rotation (see Table 55.1).

In most cases, a VTS system will use one or more radar sensors to monitor the real-time traffic situation. Radar is a non-cooperative sensor in the sense that targets are usually detected without any particular action on the side of the target. Note that, especially small targets like sailing boats, may increase their visibility as a radar target by using specially crafted radar reflectors.

The radar image may be presented to the operator without further processing. This allows a trained operator to get a better perception of the target and its environment. If the targets are sufficiently close to the radar, such as during port approach, the resolution of radar allows target features  like an approximate length, width or even orientation can be distinguished. This warrants presentation of the radar image on its own.

On the other hand, radar provides range and bearing measurements, relative to the sensor position. These measurements can be used to automatically determine the position, groundspeed and course over ground of a target. Typical VTS radars use a rotating antenna with a 1s to 10s rotation period. This means that a target is observed once about every rotation. This observation period matches with the expected movements and accelerations of VTS targets.

##### AIS

AIS position information is obtained from the ship navigational system and is, generally, a GNSS-derived position, which may or may not be differentially corrected. This latter fact is indicated in the AIS message.

AIS are basically a digital VHF communication system that provides information from sensors onboard cooperative targets (i.e. targets carrying an AIS transponder). AIS information is sent in packets. This is position information, but also information that identifies the ship (MMSI and ship name) and whether it carries any dangerous cargo or not. Apart from this, AIS can be also used to exchange safety-related information and other information, as detailed in [A.124].

AIS position information is obtained from the ship navigational system and is, generally, a GNSS-derived position, which may or may not be differentially corrected. This latter fact is indicated in the AIS message though.

##### Long-range sensors

This concerns Long-Range Identification and Tracking (LRIT), satellite AIS and information of AIS networks beyond the area of direct interest of the VTS. Long range sensors are not usually necessary for VTS operation (INF, NAS or TOS). They may provide information, though, that assists in long-term planning and may be relevant for allied services as well.

##### Electro-optical

These types of sensors could be divided in two main groups: Short distance gap fillers for a specific area surveillance, and medium range visual identification. Depending on the operational requirements, these sensors also could be used for different purposes e.g. target tracking, target height.

##### Radio Direction Finder

This type of sensor provides the information about the relative bearing to the target transmitting on radio frequency channel. The main purpose of this sensor is assess direction to the speaker you are in relation with.  Depending on the operational requirements, use of multiple Direction Finders may allow to determine absolute position of  transmitting device.

#### Tracking and Data Fusion

##### Introduction

All sensor measurements have limited accuracy and are affected by more or less random errors. In order to obtain a reliable estimate of a target position and speed vector, measurements need to be processed by a tracking filter. A tracking filter uses a model of the sensor and a model of the target movement to provide a best estimate of, at least, the target position, course and speed over ground.

##### Tracking

- Sensor level versus track level fusion

- Tracking filters?

- Reference WGS84, projection

- Performance figures

##### Initiation

Radar

AIS

The accuracy of the AIS position information should be taken into account when initiating an AIS track.

Electro optics

Radio Direction Finder(s)

LRIT

Satellite based

Track initiation should be automatic, semi- automatic or manual depending on the concept of operations.

In automatic track initiation mode, all plots in a scan should be considered potential targets.  Some of the plots will be associated with previously established tracks, while the remaining plots should be considered as candidates for new tracks, i.e. tentative tracks.

Tentative tracks will become confirmed tracks if plots from consecutive scans “fit into the picture” within reasonable physical manoeuvrability limits, otherwise the tentative tracks are discarded.

The tracking system should be able to handle at least a certain number of tentative tracks and to initiate tracks and eventually to confirm tracks under certain conditions of PD and PFA.

It should also be possible to initiate a track manually.  In manual track initiation a plot on the radar display is selected by the operator using a graphical tool.  When selected this plot should form the starting point for a tentative track which eventually should be confirmed or discarded, as in the automatic case described above.

##### Maintaining Track

If automatically or manually, created tentative tracks persist over a certain length of time the tracks should be promoted to confirmed tracks.  Confirmed tracks should be shown on the display.  The tracking system should be able to handle at least a certain number of confirmed tracks (Table 55.1) and to maintain tracks under certain conditions of PD (Table 55.2) and PFA ≤ 0.01.

##### Track Termination

If a confirmed track either moves outside a user defined maximum range, into a user defined non-tracking area, if the quality of the track falls below a predefined minimum, or if the track cannot be updated with new plots over a certain length of time, then the track should be terminated.  In certain cases the operator should receive a warning as defined by the VTS Authority.

Plot extraction and tracking performance

The requirements in respect of plot extraction and tracking should be defined by the individual VTS authority, on the basis of local conditions, number of radar sensors in a system etc.  Table XXX suggests values for each individual radar sensor in a system.

.

Table 9-15 Radar tracking performance parameters

| Plot Extraction and Tracking Performance for each individual radar in a system |  |  |  |  |
| --- | --- | --- | --- | --- |
| Parameter |  | Recommendation level |  |  |
|  |  | Basic | Standard | Advanced |
| Number of plots per antenna rotation |  |  1000 |  2500 |  5000 |
| Number of confirmed tracks |  |  100 |  200 |  300 |
| Time for confirmation of tentative track |  |  1 minutes |  |  |
| Time from track confirmation to achievement of specified accuracy |  |  2 minutes |  |  |
| Time from data loss to automatic track termination |  |  1 minutes |  |  |
| Speed of tracked objects |  |  50 knots |  | 70 knots |
| Turn rate of tracked objects |  |  10°/second |  |  20°/second |
| Accuracy in track position | Range |  0.75 % of range covered by the individual radar or 10m + selected pulse length, whichever is the greater |  |  0.5 % of range covered or 5m + pulse length |
|  | Bearing a |  1, X-band  2, S-band |  |  0.5, X-band  1, S-band |
| Accuracy of track data | Speed a |  2 knots |  1 knot |  1 knot |
|  | Course a |  5° |  2° |  2° |

##### Track initiation and track maintenance

The radar PD should be adapted to the role of the VTS.  The automatic track initiation and track maintenance is optimised accordingly.

Recommendation for the minimum radar PD for track initiation is given in Table 55.2.  For track maintenance a lower minimum radar PD can apply, depending on the tracking principles used by the manufacturers.

Table 9-16 Track initiation

| Minimum radar PD for track initiation |  |  |  |
| --- | --- | --- | --- |
| Priority of the VTS | Recommendation level |  |  |
|  | Basic | Standard | Advanced |
| Surveillance and/or traffic monitoring | 0.9 | 0.8 | 0.7 |
| Safety | 0.9 |  |  |

##### False tracks

False tracks may appear as a result of noise, clutter (including wakes) and ghost echoes.  However, the number should not be significant if the recommended values given in XXX are respected.

The maximum number of false tracks allowed is dependent on role of the VTS.  False tracks should be avoided in safety critical areas and occasionally accepted in areas where surveillance and traffic monitoring is the priority.

There is a trade-off between the time for confirmation of tentative track and the number of false tracks.  A longer confirmation time implies less false tracks and it should be possible to balance this trade-off in the setup of the VTS.

##### Track loss

Track loss may occur as a result of PD < 1 in combination with targets manoeuvring, especially in the vicinity of obstructions such as bridges.

A level generally accepted is that each operator should correct up to one track loss per hour in all areas where the recommended values given in Table 55.1 and Table 55.2 are respected.

The VTS authority should address critical areas, such as the vicinity of bridges, and explain expectations to tracking to allow VTS suppliers to make solutions accordingly.

##### Track swap

Swapping of track identity may occur as a result of targets moving close together or even merging for a period of time, especially if targets are overtaking with small difference in speed and course.

A simple method of manual correction should be employed.

In the case of AIS information being available for the radar track(s) in question, automatic correction should be performed.

The problem may also be addressed by implementing operational procedures to separate targets or to prevent overtaking in critical areas.

#### Target Identification

#### Target data augmentation using data from VTS Databases

#### AIS target maintenance

##### Track Initiation

AIS is an automatic reporting system that doesn’t require any kind of extraction or poling. After receiving of first valid position message AIS track can be initiated. Since dynamic and static information come in different messages, at the time of track initiation the full identification may not be possible and should be confirmed afterward when information is available.

##### Maintaining Track

The AIS track can be maintained as long as the position updates are received with acceptable update rate.

##### Track Termination

##### Sensor Data fusion

Sensor data fusion is a process of collection of data from multiple sources and creation of combined and more robust information. Part of data fusion can be automatic, other part may require operator assistance. The data fusion process should consider the fact that information sources are not time synchronized, and the quality of information coming from different sensors is variable.

##### Automatic data fusion

The automatic data fusion aims at providing the user with combined set of information where data originated from different sensors are linked together for a given target.

Data fusion should not degrade the quality of the information coming from the most reliable source and at certain extend improve it.

By the nature data fusion process provides the redundancy in terms of sensor data use.

##### Manual data fusion

On top of automatic data fusion, the operator may be able to de-associate some of the automatically fused data; or force an association, which could not be done automatically for technical or operational reasons.

##### Centralized fusion

Centralized fusion aims to integrate data from different systems at regional or national level using inter-system data exchange

##### Target identification

Internal and External Databases (Lloyd’s, SafeSeanet, single-hull database, various incident/black lists) – semi-static information

Distribution/Access level/”Need to know”

### Assessing the environment

#### Weather Conditions

#### Water Conditions

#### Seabed/riverbed profile

## Requirements

### Operational Requirements

### Functional Requirements

### Performance Requirements

### Interface Requirements

## Specific Design and Installation Considerations

--------------------------------

Moved from annex 2

Table 9-17 Radar functions

| Parameters / Capability | Basic | Standard | Advanced |
| --- | --- | --- | --- |
| Path, time and track prediction, |  |  | X |
| CPA, | X | X | X |
| TCPA, | X | X | X |
| Anchor watch, |  |  | X |
| Vessels vector, | X | X | X |
| Course, speed and label/identity, | X | X | X |
| Collision alerts. | X | X | X |

# HMI

## Introduction

The purpose of this Annex is to support VTS authorities and integrators in the selection of Data Display sub systems for new and existing VTS systems. The essential features of a Data Display system are discussed (the principal HCI or MMI interface to the VTS Operator) including the need to present the information necessary and not all the data available.

The principles of a good HCI / MMI design and the goals to be achieved are also discussed. Various types of data are categorised (static and dynamic chart data, supporting textual data and separate highlighted presentation of alarms and alerts). Finally the consideration of simulation, recording and playback facilities are discussed.

## Definitions and clarifications

### Definitions

TBA

### Abbreviations

Moved to top of document

### Supporting documents

Documents from eNav portrayal group TBC

(ref S57/S63/S100 data

charting standards (definitions of colours and symbology) TBC)

TBA

## HCI / MMI / User Interface

### Principles

The Data Display system provides the major operational interface between the VTS equipment and the human Vessel Traffic Service providers (the users or VTS Operators). The principal goals of a Data Display system are derived from the fundamental VTS requirements. These derive from the fundamental needs to minimise the probability of hazard occurrence, and to maximise the efficiency of all shipping operations in the controlled area. From these, lower level goals can be derived such as ease of use (to minimise errors and optimise efficiency), high VTS functional availability, the importance of data integrity, and the ability to learn from and improve upon current activities (collection and analysis of performance metrics) and undesirable events.

To achieve these goals an effective Data Display and user interface will have some key features which are discussed further in the sections below. At a summary level these include:

- The Data Display system needs to include the ability to “tune” the user interface to the operators’ roles and particular evolving needs (i.e. The presentation may need to be customisable within defined boundaries, standardised for a particular role or area of responsibility with the ability to adapt on-the-fly to align the information being displayed with the current situation being managed)

- The Data Display system needs to present current and high integrity information. Low integrity data needs to be clearly identified as such

- The Data Display system  should provide appropriate ease of access to valid information which is relevant and used both frequently and infrequently

- The Data Display system design should ensure that key information is highly visible and / or audible, whereas access to lower level information needs  to be “appropriate” (i.e. most used data is most easily accessible, but conversely, access to less frequently used information needs to be intuitive)

- The Data Display system design needs to make use of audio (alerts, alarms etc) where appropriate

- The Data Display system design needs to adhere to “standardised” presentation formats (to minimise training and hazards arising from unfamiliarity)) but this principal should not be so prescriptive as to stifle innovation and adoption of new technology and new commercial standards and products

- The Data Display system needs to include recording of key input data and user interactions and such recordings need to be available for subsequent replay and / or analysis

- NB there is an important difference between data and the appropriate filtering of this data to provide the necessary information required to support current VTS operations (data can be real time, non real time etc)

- This annex needs to recognise other work read across from eNav portrayal group etc. TBC

There are a small number of options available to the Data Display system designer to achieve the necessary transfer of information to the human user. These are discussed in the following sections but fundamentally consist of graphical screen based presentation, textual or tabular screen based presentation, the use of dedicated lights and indicators either within or external to the screens / monitors, the use of audible indicators, the use of audible communications paths. Similarly VTS Operators transmit decisions and information to various “stakeholders” by voice communications, e mail, dedicated messaging and TBA Where in VTS 128 are these output mechanisms discussed?

### Chart display

The principal display format for the VTS Operator is the Electronic chart showing the coastline, shipping lanes, area based hazards etc.. This graphical display combines the static information (current local navigation chart data with port and VTS assets, local hazards etc.) with the dynamic data (vessel traffic locations, asset availability etc.) to enable the VTS Operator to maintain an appropriately detailed (but not over-cluttered) situational picture of the current vessel traffic and potential hazards to the safe and effective operation of the VTS area.

Some important issues to consider include:

- chart data needs to be up-to-date and easily updated at appropriate time intervals

- chart data needs to be augmented with local hazard information (static and dynamic), such as under keel clearance, air draught, high predicted traffic densities, wind, tide and current restrictions on vessel manoeuvrability and required (dynamic?) separations, short term incidents and restrictions to vessel traffic etc.

- Chart data needs to include presentation of AtoNs, SARTS etc.

- Chart presentation needs to utilise standardised symbology (ref TBA)

- Chart presentation needs to utilise standardised colours (ref S57/S63/S100 data and charting standards TBC)

- Chart manipulation features to enable the VTS Operator to control presentation functions such as zoom, chart orientation, scroll, layering / filtering of information presentation

- The monitor / display real estate design needs to consider the appropriate use of multiple windows, pop-up windows, locked and flexible window positioning, overlapping and side by side windows containing chart data, textual information and dedicated status information etc.. the relative importance of each information type needs to be accommodated within the adopted design

- The console design may incorporate multiple displays with dedicated and back-up functionality

### Presentation of Vessel Data from the Data Processing Functions (e.g. Track Fusion Process, raw Sensor data, decision support functionality etc.)

Associated with the relatively static information in the chart based graphical presentation (see above), there are important dynamic (i.e. rapidly changing) pieces of information to be displayed to aid the traffic awareness and decision making processes expected of the VTS Operator. For instance the location of vessels needs to be presented. This should ideally consist of presentation of the fused combination of all available vessel positioning data (radar, AIS, optical sensors, RDF, LRIT etc.). This makes the assumption that the Data Processing function can successfully fuse the data and assess validity and integrity of the fused output. In addition to current location, the VTS Operator may also need to understand the history of vessel tracks, and the tracker predictions for navigation of vessels in the future.

Access to raw sensor data may be seen as a requirement by some VTS system designers but an effective track fusion process should negate this in future VTS systems.  Potential future hazards may need to be highlighted on the display. Standardised symbology and standardised use of colour needs to be established to reduce the chances of errors and to reduce the training and familiarisation time for new and temporary VTS Operators. The Data Display interface should allow the VTS Operator to easily select a vessel and interrogate the vessel database to establish identity, route, pilotage, berthing, communications channels etc. for that object. Similarly the selection of a tracked object should allow the operator to quickly understand the validity, integrity and any sensor issues associated with the fusion process for the track of that object.

The Data Display interface should also allow selection and filtering of the presented information to tailor the display to the task in hand.

There should be a generic search facility that can be accessed to allow displayed object highlighting, hiding or filtering (e.g. highlight a vessel with a certain name, or highlight all vessels requiring access to one particular port asset or group of assets etc..) within a complex traffic situation

The Data Display interface should also allow the introduction and control of a Manual track process by the VTS Operator. (TBC)

The Data Display interface should also allow presentation of advance alert of possible hazards based on the decision support algorithms.

The Data Display interface should also allow Interaction with recording system, although during normal VTS use, this may be automatic and invisible to the operator.

### Status Summary (of VTS sub systems)

The Data Display interface should also allow sub-system status to be summarised (hierarchically) so that the right level of information is presented for each anticipated situation.

The Data Display should ensure that the VTS Operator is immediately aware of any degradation to system status. The use of both audible and visual alerts may be considered appropriate.

On-request, the VTS Operator may need to access detailed VTS sub system status (to include; sensors, tracking (and data processing), decision support, communications, power, satellite based data etc.). Similarly, the VTS Operator may need to access detailed status information relating to port assets (locks, bridges, pilot availability, tug availability, berth availability etc.)

The Data Display system should provide appropriate presentation of the outputs of the decision support processes (traffic capacity alerts, collision alert, weather condition alert etc.)

Interaction with recording system may be a secondary function of the Data Display system.

Interaction with scheduled maintenance and logistics information (and recognition of non critical faults which have reduced available redundancy) may also be required.

### Audio and Visual Alarms / alerts

The Data Display system should provide a hierarchy of alerts with appropriate acknowledgement and cancellation of these alerts as considered necessary to support operational procedures. The types of alerts should be matched to the severity of the information being highlighted. Consideration may need to be given to the use of visual alerts and appropriate audible alerts.

Alert notification, acknowledgement and cancellation activity should be recorded by the recording system.

## Display Hardware and Ergonomics

### Location (does this belong in this section?)

The physical location of a VTS centre needs to consider the objectives of accessibility (wanted and unwanted), security (access control, system integrity, anti terrorism etc.), the need to be central (not necessarily in the physical domain) to hazards and the control of hazard mitigation activities, central to system data paths, and to maximise the access to relevant information (windows to outside world (physical or virtual))

There may be appropriate Security regulations and standards that should be referenced here to minimise the chances of unwanted physical and virtual intrusion into the VTS centre or the systems and sub systems contributing to the VTS functionality.

Consideration should be given to the need for VTS system growth in terms of equipment size, footprint, future redundancy enhancements, separation of simulation, training, and incident playback and analysis in a separate area from “live” VTS operations.

### Type of layout

When specifying the operations room for the VTS centre, consideration should be given to the maximum number of users, the roles to be carried out in parallel in worst case situations, the hierarchy of roles and users, the size of operational consoles, system network design and interfacing to and from the consoles,

The console design should consider the number of screens per user (information to be presented should be appropriate and avoid operator data overload, the size, format (4:3, 16:9) and orientation (landscape, portrait) of the screens.

Where possible open commercial standard formats (architecture and interfaces) should be sought to minimise obsolescence and maximise options for future system growth potential.

The layout should also consider emergency procedures and role of VTS centre in emergencies (maritime, land based, civil unrest, security, military, disaster recovery, environmental etc.) as part of a coherent regional or national infrastructure where required.

### Environment

The VTS centre user environment should be of paramount importance to create a comfortable office type background to facilitate concentration and minimise distractions. Chairs and consoles should offer comfort for long periods of use and a range of settings to optimise the user experience. The environment should consider the advantages of air-conditioning, good and appropriate lighting, minimisation of externally and internally generated noise distractions, nearby rest facilities (to minimise user downtime), and well designed interaction with the available voice communications (e.g.. via voice switching system to combine telephone, hotlines, ship to shore, operator to operator, VTS to VTS, VOIP, etc..). Methods of data entry (keyboard, mouse, etc.) should also be optimised to suit the operator workload during the most stressful of situations.

### Reliability and fall back

The reliability and availability of display system should not be the weakest link in the VTS centre. The A, R and M numerical requirements for the display system should be derived from top-down analysis of critical failures, built-in redundancy and the overall VTS system under consideration.

The display system equipment design should ensure that scheduled maintenance is minimised and non-intrusive. Unplanned maintenance should be considered by suitable design of equipment access and provision of fall-back regimes.

Where necessary to achieve the A, R and M objectives, the possibility of immediate transfer to an adjacent user position (in the event of failure) should be provisioned. The power supply to the VTS centre (and other critical VTS sub systems) should consider emergency power system back up. (NB at system level not just display system). Hardware redundancy within the Display System and as part of the VTS system design concept should be considered to optimise the system level approach to achievement of the A, R and M requirements. This may also need to include the provision of multiple interfaces to the Display System from critical VTS sub systems to minimise the consequence of interface failure.

Based on hazard analysis, a second back-up VTS centre might be appropriate (with all essential facilities of the primary centre location) This should allow easy transfer of current data to another location capable of maintaining the VTS service in the event of an emergency situation resulting in temporary closure of the primary VTS Centre.

Consideration should be given to integration within a National disaster recovery concept (as required) (NB at system level not just display system)

The location of the VTS centre and its backup location(s) should maximise the achieved level of Immunity to natural disasters (flood, earthquake etc.) (NB at system level not just display system). The location of the VTS centre should also involve consideration of anti-terrorism objectives to minimise impact of intrusion and / or destruction (NB at system level not just display system)

## Data Interfaces to support the Data Display System

### Information held in Databases

#### Database principles

Information and data are not the same. The display system needs to present the information required and not the data available. Where possible the entry of data should be automated and read across from originating sources to minimise errors and reduce the possibility of data conflicts. Where possible, data checking (completeness, validity etc.) should be at source (i.e. when first entered into a system which subsequently connects to the VTS system).

Stale data (i.e. that not updated at the expected interval) should be identifiable and flagged to the user as appropriate. In addition, data conflicts (between databases, between sensor updates etc.) should be detected and highlighted to the users (sub systems and Operators) as appropriate.

The database performance, e.g. as measured by interrogation latency, should be appropriate to the data type and the data update rate. Back up regimes should be in place to allow retrieval of data from each critical database affected by an unplanned or catastrophic event.

#### Vessel database (attributes of the record for each vessel.)

See ref TBA for typical data to be held for each vessel (for example Vessel’s Name, Call Sign, IMO Number, MMSI, ETA, ETD, Draft, cargo, originating port, destination). This data is mostly static data relating to each vessel and its current voyage (logistic data) – the database performance requirements are therefore relatively low.

#### Track database

The track database contains sensor plot data, fused track data, validity and integrity measures for the each piece of positional and navigation data. It should also contain predictions about course and time to key manoeuvre and traffic pinch points. Multi hypothesis track predictions may be necessary to allow management of track merging and splitting, false alarm control and sensor error identification. Note that the display system requires unique and unambiguous identification of all the vessels in the track table.

It is likely that time stamping of data entries and data outputs back to a common time reference (local network time server or similar) will be necessary. Coasting and lost tracks should be flagged. Metrics should be collected to allow assessment of the general traffic management “picture” to be monitored and subsequently analysed. The track table size should be sufficient to accommodate n times the heaviest traffic predictions.

The Data Display system design should ensure accurate, frequent and low latency of the information from the track database.

The track database should be able to provide advance warning of track table capacity overload. In addition, the tracking process should assess sensor bias and noise errors and update rates and record issues in the track database. This information should then be presented to the VTS operator and fault managements system as appropriate.

#### Database of Pilots and tugs

The optimisation of VTS operations will probably require data to enable pilot management and tug asset management (availability, capability, current and planned tasks).

It is anticipated that this database will be a relatively slow changing (low performance) database.

#### Database of berths and capabilities plus other port resources

The optimisation of VTS operations will probably require data to enable efficient management of available berth and anchorage locations, plus movement planning restrictions arising from locks and bridges.

It is anticipated that this database will be a relatively slow changing (low performance) database.

#### Charts

The Electronic charts should preferably be compliant with S57/S63/S101.

The Data Display system should facilitate incremental and full set chart updates.

Distribution of published charts to clients should be an easy and, where possible automated, operation.

The Data Display system should include a mechanism to verify published chart status against current to ensure use of latest publications.

#### Local hazards

In support of the presentation of local chart information and to the Decision Support process, a database of the local identified hazards may be required. Typically this might include static and slow changing hazards such as, underwater hazards, bridges, traffic high density points, shore side hazards, and more dynamic hazards such as Oil (and chemical) spills, ship system failure (thrusters, rudder, AIS, radio, etc..), man overboard, coastguard events etc..

It may also be appropriate to capture the generic types of dynamic hazards (even if these are not current)

#### Equipment status, build state, version records

It may be appropriate to integrate logistics management data to allow occasional access to equipment records etc. for various reasons.

#### Spares and consumables stock and storage locations

It may be appropriate to integrate logistics management data to allow occasional access to spares and consumables records etc. for various reasons.

#### Equipment scheduled and unscheduled maintenance

It may be appropriate to integrate maintenance planning and management data to allow occasional access to assess equipment availability records etc. for various reasons.

#### VTS personnel

It may be appropriate to integrate personnel management data to allow occasional access to staff records etc. for various reasons.

#### Fault records

Consideration should be given to holding a database of equipment faults and warning messages to allow failure modes to be analysed and intermittent faults to be highlighted and appropriately managed. Access to such data is unlikely to require the facilities of a full VTS Operator’s console.

#### Accounts with shipping companies and dockside cargo handlers etc.

It may be appropriate to integrate shipping accounts data to automate alignment with cargo movements, shipping movements, handling charges etc. to facilitate account management by systems associated with the port (i.e. Not directly associated with VTS operations)

### Sensors and vessel Tracking

#### Sensor data

It may be necessary, as part of the VTS system concept and operational procedures, to display raw sensor data directly to the VTS Operator. This includes radar video and plots, AIS (and SAIS) plots and associated data, LRIT plots, RDF derived positional data, Camera video etc.

In many cases this sensor data will be normally hidden from the operator, assuming that it is automatically fused with similar data within the Data Processing (track fusion process) yet in other cases (such as optical video from daylight and EO cameras), it may be necessary to display the video on demand or even at all times depending on the operator needs. To minimise information overload, the track fusion process should be designed to reduce the need to display raw sensor data in the majority of situations.

#### Track data and control of tracking and sensor fusion

The Data Display system will overlay the output from the track fusion process (uniquely identified tracks) onto the electronic chart display – as such the Data Display automatically receives the appropriately weighted fused summary from all available sensors

The Data Display should be able to indicate the principal sensors contributing and failing to contribute to the fused output

The Data Display system should be able to display the received assessment of sensor integrity (accuracy (bias and noise), staleness, corruption, faults) and if appropriate an indication of what action to take based on this assessment (repair, warning to ship (potential on board AIS fault etc.)

The Data Display system should also be able to warn the VTS Operator of unexpected and unreasonable Manoeuvres (these may provide an early indication of possible ship system failure, but may also indicate track confusion (merging and splitting) within the Data Processing function.

#### Sensor status information

The Data Display system should also be able to display any detected issues with the normally available sensors (faults, staleness, change of sensor status, etc.)

#### Sensor control capability

The Data Display system should provide the VTS Operator with the ability, where appropriate to enter commands to control the vessel tracking sensors. However, where possible, the sensors should be fully autonomous.

### Meteorological and Hydrographic data

The Data Display system should be able to display the information derived from the available meteorological and hydrographical data (locally gathered and passed from other agencies.

It should also be possible for the VTS Operator to access raw data and recent history and any status or fault information from the local sensors. This may be textual, graphical (e.g. vs. time) or may need to be added to the chart display.

Other information that may need to be accessed include the list of sensors, their current status, fault history, battery life etc.

### VTS asset status

The Data Display system needs to provide the VTS Operator with access to the assets within the VTS area (bridges, locks, tugs, berths etc.) to allow understanding of their current status. In some cases, the Operator may need to directly control some of these assets (bridge raising etc.) and associated monitoring sensors (CCTV, road traffic sensors, water levels, etc)

### Decision support tools presentation

The Data Display system should provide display of Decision Support data (annex 11) to give visibility of (for example):

- Automated alarms and warnings

- Manual input of new situation requiring urgent action

- List of generic situation / hazard and automatic / manual selection of appropriate recommended procedures

- Presentation of recommended procedures and checklists

- Presentation of key contacts

- Display changes to allow focus on management of the situation (automatic / manual)

### Hazard Management

#### Emergency systems interfaces (SAR)

The Data Display system should provide display of information and recognition and reaction to various types of external and internal alerts. Such alerts may consist of:

- External telephone

- Ship communications

- AIS code?

- Unexpected SART detection

- Decision support – automated early warnings

- Liaison with external agencies (monitoring of non VTS events which may become VTS events)

#### External alerts (security etc)

The Data Display system should provide display of information and recognition and reaction to various types of external alerts (not related to SAR) such as:

- VTS asset security breach

- Detection of unknown vessels / water users which may indicate security situation

- Responsibility vs. other agencies for coastal asset security

- Weather warnings

- Use of decision support to standardise response and use of checklists to generic (predictable) situations

#### Disaster management and recovery

The Data Display system should provide display of information and recognition and reaction to various types of potential environmental and wide scale threats to the environmental and to support national or regional solutions to non-maritime events (where required). The display system may need to provide display of:

- Operational procedures etc

- Checklists

- Key contact details for external agencies who might need to be aware / involved

### On line simulation and training facilities

The Data Display system may be required to support various simulation facilities to allow some secondary functions to be carried out. These might include:

- competency assessments of VTS operators

- training of new VTS operators

It is important that such activities are non-intrusive to normal operations. Note that “back room” interaction and control of the stimulators to manage the training, and assessment exercises also need to be specified and appropriately located. A library of scenarios and associated generation files to be used for training and assessment may need to be established, stored and maintained. It may also be useful to be able to load a recorded incident into the simulator for analysis, training illustration and improvement of VTS Operator procedures.

### Recording system and playback

#### Recording and playback principles

The Data and Display system should be able to record sufficient data to allow replay of an event – either as if “live” or just based on what happened. The recording system should be based on hazard assessment.

The Data and Display system should be able to support recreation of an incident in terms of data flow into (and out of) the VTS centre (sensor data, status of various databases, audio and video recording, operator keystrokes, data processing actions during the event, ). Where possible it should also the semi automatic generation of reports for authorities and other stakeholders.

Given the possibly vast amount of data that could be involved, the solution should be cost effective in terms of storage capacity (vs. time  history captured) future expansion, data backup and use of cost effective and reliable storage medium.

#### Recording

The Recording process should work as an autonomous 24/7 service. Preferably the process should be a redundant solution on a separate server.

Recorded data should be compressed/encrypted to ensure un-tampered data sets.

#### Replay

Initiation of replay should be done from the same type of equipment and application as used for live data representation.

It should be possible to search for specific incidents based on geographical area, date/time and vessel particulars.

Sources selected for reply (at least streaming sources) should fully synchronized on replay.

The provision and time and key event based searching through the recordings and a fast forward facility of some critical streams of the recorded data should be considered.

#### Access to recorded data

Data should be recorded to a closed server environment with security policy access to the physical storing media.

Storing medium should facilitate redundancy solutions such as RAID and backup solutions. Procedures for reuse of data storage (after one month or.) should be put in place.

The design should include means to restore historical data for replay.

# Decision Support

## Introduction

This annex describes the uses that can be derived from processed data in order to help the VTSOs to assess the situation or to make decisions. Some useful information for the VTSOs can be directly derived from sensor information display and do not require further refinement whereas the decision support indicators are usually estimated from more complex processes involving multiple sensors information and/or high level processing implying temporal and spatial prediction.

Those decision support tools are designed to assist the VTSOs in their current tasks by synthesizing multiple pieces of information in order to produce an alert when a risk threshold is overcome concerning a vessel situation, an area monitored or a function supervised.

Typical decision support tools include, but are not restricted to, closest point of approach (CPA), time to CPA (TCPA), collision alert, area penetration, route adherence, speeding, anchor watch or grounding alerts.

Though these tools functions current working condition and settings may be preset by the manufacturer at the time of delivery, trainings and documentation should clearly inform the user regarding the basic principles and proper use of these tools, including their capabilities, limitations and possible errors.

## Definitions

Target: any object fixed or moving whose position and motion is determined by measurement of range and bearing.

Tracking: the computer process of observing the sequential changes in the position of a target in order to establish its motion.

CPA: the Closest Point of Approach is the shortest distance separating two ships when observing their course over time. This measure is directly linked to the mutual risks shared by the two ships along their roads. The CPA is permanently computed in real-time on-board ships and in the VTSs in order to anticipate a risky situation.

TCPA: the Time to CPA is the estimated time left before two ships reach the point where their mutual distance is minimal.

Predicted points of collision: A graphical representation of where predicted collision intercept points lie with respect to designated ship and other targets.

Tracking filters: in order to predict the time to reach a position or the position of a mobile in the future, one may use models describing how the observed system evolves in time and space; these models may also be referred to as tracking filters.

- Simple models rely on few data, such as the last speed and heading observed in conjunction with a simple hypothesis such as constant speed and constant direction, in order to predict the spatial and temporal evolution of a mobile at sea.

- More complex filters may use historical data, such as the evolution of speed and the evolution of heading, in conjunction with more complex hypothesis based on human factors, dynamic properties of the mobile, the sea state and sea currents, in order to predict the spatial and temporal evolution of mobiles at sea. Some high level tracking systems may also use multiple model computation simultaneously before choosing the most likely or before computing a weighted mean of the different estimated positions.

History: equally time-spaced past positions of a target which is being tracked.

Map lines:  also called Nav lines, the navigational facility whereby the observer can define lines to indicate channels or Traffic Separation Schemes.

## Characteristics

Support tools are not mandatory so they may differ depending on the needs and functions of the VTS, the system designer, the manufacturers. Some of these services may be provided by sensor manufacturers or integrators whereas some high level functions may me designed at system level, beneficing from the information gathered by multiple sensor networks.

In order to assist VTSOs fulfilling their tasks of surveillance in a specific context, some decision support tools may require user input such as the vessel(s) concerned or the area supervised. On the contrary, some more generic tools or basic warning systems, such as CPA or TCPA, are permanently estimated and should warn the VTSOs if the vessels courses closes within the predefined limits.

Some of these tools may be classified as critical risk assessment tools because they reflect the risk of collision or groundings, those are for instance CPA, TCPA, grounding alert. Other decision support tools are not so critical because they are linked to local regulations or recommendations.

## Requirements

### Operational Requirements

Like other information displayed to the VTSOs, the decision support tools aim at lowering the workload of VTSOs without interfering with the current tasks they are mandated for. The VTSOs should always be able to choose the thresholds of the different alarms displayed, whether they are automated, semi-automated or manual. However, it is recommended that limits to these thresholds cannot be exceeded (for instance, negative values cannot be assigned to thresholds corresponding to distance or time related values).

VTSOs may deactivate such alarms in order to lighten the sum of information displayed. It should be possible to cancel the display of unwanted data displayed within 3s. The cancellation of an alarm raised by any of the decision support tools should be confirmed by the operator.

In specific situations such as high traffic zones, decision support tools such as CPA and TCPA may raise a high number of alarms permanently. It should be possible to suppress the alarms linked to these risk indices for a whole area. In such a case, the area where no alarm can be raised should clearly be indicated on the display according to the display recommendations provided in Annex 10 : Traffic Situation Display.

VTSOs should be aware that permanent cancellation of alarms regarding a specific location or one or more designated vessels, if it is justified at the time of cancellation, may prove itself dangerous moments later by hiding a new hazardous situation, or a recurrent unusual behaviour. It is therefore recommended that if a permanent cancellation of alarms in a given zone is decided, this zone is limited to a well known place on which VTSOs are clearly informed and give a particular attention. In the case where the cancellation concerns a specific vessel, it is recommended that the cancellation is not permanent and does not exceed XX minutes before the tools are able to raise a new alarm related to that vessel.

VTSOs may always be able to switch on/off the different modes of warning, sounds or displays. In the case where all modes of warning are deactivated, a clear indication should state that on the display (see traffic situation display recommendations for more details).

VTSOs may adapt the range of the display in order to have a global situation display or to focus on a specific zone that is to be under close surveillance. Nonetheless, VTSOs should be able to perceive an alarm raised by the system out of the displayed zone. This is possible through different means, the use of multiple screens in the VTS, one dedicated to global surveillance and one allowing for free zooming and displacement of the area under surveillance. If only one screen is present, it can also be obtained through the use of a separate window smaller than the global picture and superimposed on the global picture (see traffic situation display recommendations for more details). In any case, the system should ease the localisation by the VTSO of an alarm in order to warn him or to allow him quick suppression of the alarm.

Alarms such as CPA or TCPA have been validated for on-board as well as for onshore operations and are the only collision alarms shared by both VTSOs and navigators. Though these tools are shared by VTS and vessels, their implementation, the sensors and information they are based on and the thresholds used to raise the alarm may differ.

In the case where the supervising system in charge of the computation of the different decision support tools does not receive any low level information such as sensor information or information produced during the data processing steps (de-noised sensor information, precise and reliable positioning), the system is said to be blind and all positions and alarms are computed on an estimated basis. The system is in dead reckoning mode and should clearly display that information to the VTSOs. Knowing that in dead reckoning mode, the precision of the estimations decreases as time passes, it also should clearly state the confidence level on the alarms displayed, and after a low level threshold of confidence is crossed, the system should not automatically display any alarm which risk of misleading the operator.  The system should focus on the best possible positioning of vessels, and let the VTSOs in charge of requesting estimations of risk indices for the critical decision support tools.

### Functional Requirements

The low level data processing steps may produce a composite indication of position, navigation and timing. High level processing tools such as decision support tools may use this information as an input. The use of such information in high processing steps should not have performance inferior to that which could be obtained using only radar information.

Automatic or manual acquisition should have a performance not inferior to that which could be obtained by the user of the display through the use of a classical geometric tool (use of vectors for CPA/TCPA, use of circle of avoidance for anchor watch or use of drawn limits for area penetration).

The features described in the table below should preferably be automatic for systems requiring standard or advanced capabilities.

Table 11-18 Decision support functions

| Parameters / Capability | Basic | Standard | Advanced |
| --- | --- | --- | --- |
| CPA, | X | X | X |
| TCPA, | X | X | X |
| Collision alerts. | X | X | X |
| Grounding alert | X | X | X |
| Anchor watch, |  |  | X |
| Area penetration | X | X | X |
| Speeding |  |  | X |
| Route adherence |  |  | X |
| Path, time and track prediction |  |  | X |

#### CPA/TCPA

CPA and TCPA are numeric indices characterizing the imminence of a close approach between two vessels. These indices must be interpreted together with a logical AND between the two criterions defined beforehand.

Where the use of VTS is requiring basic capabilities, one threshold may be defined for CPA and one for TCPA. VTS addressing advanced capabilities may also have to monitor zones where basic capabilities are needed. In this case, it may be useful to distinguish areas with different levels of alarm.

If different areas are monitored according to different rules concerning alarms thresholds, it must be possible for the VTSOs to visualize the different zones and the alarm levels associated.

If different alarm levels are accessible, the display of an alarm associated with a given level should allow VTSOs to appreciate immediately the level of criticity urgency?? of the alarm.

Thresholds described in the table below are given as an example and as specified in the operational requirements, these thresholds may be adjusted depending on the needs of the VTS, its capabilities and the traffic density.

Table 11-19 CPA/TCPA thresholds

| Alarms / Capability | Basic | Standard | Advanced |
| --- | --- | --- | --- |
| Level 1 | (CPA < 4 nm) AND (TCPA < 20 min) |  |  |
| Level 2 |  | (CPA < 2 nm) AND (TCPA < 15 min) |  |
| Level 3 |  |  | (CPA < 1 nm) AND (TCPA < 10 min) |
| Collision alert | Whenever two ships are distant of less than 1 nm |  |  |

#### Anchor watch

Anchor watch aims at warning the VTSO that a ship who has been assigned at a given anchorage zone does not drift out of the zone otherwise than in manoeuvre. Usually, only VTS with advanced capabilities have such zones under their surveillance. These areas are defined based on a given position and a distance of avoidance, both of them defining the circle of avoidance inside of which the ship is supposed to stay when not in manoeuvre.

Distances of avoidance may be expressed in standard distance measures such as meters or these distances may also be expressed in number of links or shackles in order for the VTSOs to be able to inform the vessel whatever its mode of count.

In the decision support processing of anchor watch, if the VTS knows of the variation of the circle of avoidance according to tide level, the boundary should be set according to the greatest distance from the anchorage point (low tide limit).

#### grounding alert

Grounding alert is an alarm that depends on the draft of the vessel, the bathymetry and the water height due to the tide. The alarm is raised if the estimated "under keel clearance" crosses a low threshold.

Depending on the capabilities of the VTS, the precision of bathymetric maps, the precision of water height due to the tide and the draft of the vessel, the threshold may be adapted by VTS authorities. The threshold may directly be affected by the precision of bathymetric maps, precision of these maps should be added to the lowest recommended thresholds.

Table 11-20 Grounding error thresholds

| Alarms / Capability | Basic | Standard | Advanced |
| --- | --- | --- | --- |
| Lowest recommended threshold | 2m + precision level of maps and other errors sources | 1m + precision level of maps and other errors sources | 0.5m + precision level of maps and other errors sources |

For instance, given a bathymetric map with an error inferior or equal to 1.5m, the alarm for a standard VTS should at least be set to 2.5m.

#### Area penetration

Area penetration is an alarm that warns the VTSOs whenever a ship is penetrating a predefined area or crosses a navigational line that should not be crossed.

International regulations, national recommendations or VTS authorities may define areas where no shipping is allowed under normal circumstances. These areas may be Traffic Separation Schemes, Special Areas and Particularly Sensitive Sea Areas as defined by IMO or areas defined by national authorities that may be forbidden temporarily to shipping for security or safety measures.

Area penetration support tools are based on the ship's position in regard of the zones under surveillance. If precise positioning of the ship and the limits of its hull are accessible, the alarm may be based on the penetration of any part of the ship in the designated area. If the precision level of positioning is low, the alarm may be based on the crossing of the area by an ellipsoidal shape around the estimated position of the ship, this ellipsoid should be proportioned so as to reflect the limits of the ship with 95% confidence (one may refer to Annex 9 Data Processing for a definition of a precise and reliable positioning).

#### Speeding

Speed alert aims at warning VTSOs whenever a ship's speed is excessive.

National authorities or VTS authorities may define speed limits for navigation in certain areas such as port zones. If precise and reliable speed estimation is accessible the alarm may be based on instantaneous speed observed. If the speed is assessed based on a unique sensor and is subject to noise, special care such as meaning of speed over a short period, outlier filtering through median filtering or such processes should allow for false alarm avoidance.

#### Route adherence

Route adherence is a support tool aiming at warning VTSOs in the monitoring of a ship's routing. This tool is essentially a support tools used in the context of Navigation Assistance Services or Traffic Organisation Services.

Usually, before such a tool can be used, the VTSO must know the routing plan of the ship and according to that plan, he may define a corridor in order to monitor that the ship is following the intended route. The definition of this corridor is bounded by the position of the ship, the destination of the ship or a waypoint, and the width of the corridor that is a distance (expressed in nautical miles or in meters).

In some cases, as in the case of an existing deep water route, this corridor may be predefined. The corridor follows the route composed of one or multiple segments and the width of the corridor depends on the constraints linked to the sea floor. The limits of the corridor are then constituted by polylines.

As in the case of  the area penetration support tool, the distance from the ship to the nominal route may take into account a precise positioning of the ship and its hull, or an ellipsoid linked to the estimated position of the ship (one may refer to Annex 9 Data Processing for a definition of a precise and reliable positioning).

#### Analysis and prediction

##### Traffic monitoring and analysis

Traffic monitoring is a theme which may serve multiple purposes such as information and knowledge of the traffic, determination of input parameters to prediction models, efficiency of traffic management tools or risk assessment tools.

Traffic analysis may be based on ship positioning but it may also use other quantitative data such as speed, bearing, size, sea currents, winds, etc. and qualitative data such as type of ship, destination, origin, flag state or ship owner.

These data may be informed through measurements by one or multiple sensors but they may also come from declarative statements.

Special care should be given on data pre-processing such as filtering of erroneous measurements, sampling rate at constant time or constant distance. The limits of exploitation of the statistics should be known a priori in order to choose the most appropriate approach. The confidence level given to the different data should also be taken into account, be it the noise attached to measurements or the confidence in qualitative data gathered through declarative process (destination or ETA field in AIS data for instance).

The traffic analysis should always state the duration of the analysis, the field of interest and the nature of data analysed (traffic density for all ships or depending on the class of vessel, mean speed, mean speed of the 90th percentile of ships, etc.) and the size of the geographical or time sampling rate such as “per nautical mile square” or “by hour”.

Some processes are static, considering a given set of data of the past as input, whereas some processes will be constantly evolving, considering historical data of fixed depth from the current time.

##### Path time and track prediction

In advanced systems, there may be a complementary constraint on the notion of route adherence which is not only based on distance but is also based on time. In accordance with the routing plan given by the ship, the support tool is able to indicate if the ship accesses certain areas timely as it was planned. These areas are usually located around way points, but the monitoring may also be continuous.

In the case of a continuous estimation of the route adherence, a window of time evolves along the corridor, may it be composed of one or multiple parts, and the support tool controls that the vessel is inside the lateral boundaries of the corridor as well as inside the longitudinal boundaries defined by the space-time window.

The space time window is located around the theoretical position of the vessel considering the mean speed the vessel should have had in order to be timely around the defined ways points. The distance between the limits of the spatial window are such that the distance before or after the theoretical position is equal to the theoretical speed of the ship multiplied by the flexibility of time given to the support tool (10min, 20min, 30min).

The flexibility of time considered may be fixed by the VTSO or automatically defined by traffic statistics considering mean passage time observed, either for all ships or considering the nature and charge of specific ship classes.

In special cases such as when the speeds observed along a route are not usually constant, a speed profile along the route may be estimated through traffic analysis. The speed may not be constant due to different factors such as the tide, the presence of currents or the influence of the wind on the speeds observed. In such cases, models describing the evolution of the speed profiles along this route may be used as an input to determine the centre of the space-time window (theoretical position of the vessel from the initiation of the support tool to the current time).

Special attention should be paid to the way the model is build and used. Data driven approaches (top-down models) and data-informed approaches (bottom-up models) should be distinguished.

In data driven approaches, the model is build in order to stick to the collected data or statistics. The parameters inside the model may not necessarily to observable or physical data. The result is usually closer to the final observed data (as compared with mean behaviour), but as data are often missing or rare for extreme behaviours, those behaviours will likely be more difficult to simulate with good precision.

In data informed approaches, the model is based on hypothesis linking different logical input parameters, the expression of the model is intellectually satisfying because these inputs have meanings. But even though a great amount of tuning is done to clearly establish the links between the input parameters, there may always be missing or fuzzy input parameters, so that in the end, the results may easily diverge from data observed. On the other hand, given the correct input parameters (more easily informed because measurable) they may be able to simulate a wider range of behaviours.

Such models may be used to predict the passage time of ships with greater precision than in the simple route adherence tools based on constant speed hypothesis. They aim at predicting the passage times on a meso-scale basis (from 1 nautical mile to a few tenths of nautical miles) or a macro-scale basis (from tenth of nautical miles to hundreds to nautical miles).

These tools may be used in conjunction with tracking filters which aim at predicting the track of a mobile from micro scale (from a few meters to 1 nautical mile). These tracking filters behave like the dead reckoning tracking in essence but with more complex hypothesis. Dead reckoning is a first order (or linear) simple tracking filter. It does not account for noisy data whereas tracking filters intend to account for noisy measurements and to simulate non linear evolutions.

Usually, dead reckoning is used with a given hypothesis (constant speed and constant bearing) when no further measurements will be available. On the contrary, tracking filter are designed to work in monitored environments in order to improve localisation by constantly updating the model (be it linear or non linear) with a feedback loop and a probabilistic approach accounting for sensor noise (which allows for optimal positioning estimation).

One must remember that though more evolved than dead reckoning approach, these models are based on hypothesis and that they diverge from truth as time passes if they are not updated with measurements. Using them in order to predict the precise track of vessels for the seconds or minutes to come proved to be efficient if updated, but if no update is available, there is no reason to believe that the hypothesis they are based on is more adapted to the situation. Tracking filters may diverge from reality as well as dead reckoning mode if the behaviour of the mobile changes without being observed.

Among those filters we find a whole family based on Kalman approach. The Kalman filter was the first of this family of tracking filters, based on linear prediction but capable of optimal prediction of positioning by accounting for noisy measurements. Extended Kalman filter (EKF) or Unscented Kalman filter (UKF) can account for non linear systems with noisy measurements. The extended Kalman filter (EKF) is probably the most widely used estimation algorithm for non-linear systems. However, it is difficult to implement, difficult to tune, and only reliable for systems that are almost linear on the time scale of the updates. The UKF is more robust to non linear systems.

The more recent approaches based on tracking filter may combine multiple filters of the Kalman family, each of them acting in the framework of a given manoeuvre hypothesis such as constant speed, constant acceleration, constant turning etc. The Interactive Multiple Models (IMM) approach uses two or more filters which run in parallel, each of them using a different model for target motion or errors. The IMM performs an optimal weighted sum of the output of all the filters and is able to rapidly adjust to target manoeuvres.

(One may refer to Annex 9 Data Processing for details on precise and reliable positioning).

#### Incident or Accident management

Incident or accident management is a support tool allowing for monitoring of the resources available and deployed in a crisis context. Depending on the nature of the incident, the operation may focus on SAR, environmental protection, prevention of other incident resulting from the original crisis.

In order to assist the VTS in monitoring the situation and playing a role in crisis management, these tools should help to visualize the allocation of resources depending on the task and on the areas concerned. For instance, these tools should help the VTS to organize different teams composed of vessels and aircraft in order to cover a given areas in case of SAR operation, this can be done with graphical overlay, identification of the resources and historical track display in order to identify the areas already covered during the operation. This can also be achieved by displaying zones unsuitable for navigation, factors influencing the decisional processes such as sea currents and winds, and it may also include an operational assistance for monitoring the operation such as a clear report of the steps, actors and actions organised. This report should be easy to share with other parties in order to manage the interactions with other services, onshore or at sea.

#### Simulation and Training

Simulation and training tools allow VTSOs to simulate operations in a complete environment including all the tools that may be used in real situations. Input data to the system may be based on simulated data or on recorded data.

Usually, simulated data are used in the validation process before systems are implanted in the VTS. They allow for operators to test the ergonomics of the system, test the different tools and express their needs and comments. Once the systems are implanted, connected to the sensors, and if the system is able to perform recording of real data, (as recommended in the operational requirements), the preferred way to exploit the simulation and training tools is based on real data and real situations.

This may help VTSOs to analyse and practice on the most usual configuration as well as on difficult operational procedures that the VTS had to face in the past. The simulation should as much as possible stick to the reality of working of the system in usual conditions, exploit the same sensors in input, exploit the real data recorded even if erroneous or noisy and exploit the same software version of the different tools.

There may be multiple uses for recording and replay, some may be directed towards the VTSOs needs (practice and analysis) whereas some others are directed towards the company who designed the system in order to make its products and tools evolve.

These two approaches should be differentiated. During practice and analysis, if the simulation is based on real data, the system should perform in the same conditions it has worked during the real situation. The system may replay the real situation that has been recorded as an output of the system, in that case, interactions with the VTSOs are limited and the system replay is not interactive. If input data to the system have been previously recorded, the replay should perform the same computation steps than in normal conditions, including calculation steps that made the situation difficult to analyse in real time.

In the feed back and improvement process that implies communications with the industrial, if real data were recorded, the computation steps of the simulation may be changed in different manners in order to better handle difficult situations and to answer more accurately to the VTSOs needs. In that case, the display during the training and simulation process may differ from the display the VTSOs used to visualize the first time during the real situation.

## Specific Design and Installation Considerations

### Interface Requirements

As suggested in VTS manual, the output of decision support tools may be recorded in complement to raw data and the image displayed to VTSOs. This recording is recommended for multiple purposes among which, the review of accidents, incident investigation, technical evaluation, quality monitoring for continuous improvement, statistical analysis, training purposes or for use as evidence following an accident or incident.

VTS authorities should be aware of the usefulness of such practice even when recording raw data or the maritime picture on the other hand. It is of the VTS authority to determine the frequency of sampling but it should be adapted to the goals pursued by the replay options. Low frequency sampling may be sufficient for traffic analysis purposes whereas nominal sampling is preferred if the tool is to be used to simulate the real environment of the VTSOs (recording entirely the raw data or recording entirely the output of the system at its nominal sampling rate).

Depending on the system design, the system may be directly interfaced with the sensors providing data (in the case the company that designed the system has the know how of the low level processing of data) or the system may be of higher level (if the company is an integrator and benefits of pre-processed outputs from the sensors). In the case the system designer masters the processing of raw data information, it is recommended that he uses these raw data as much as possible without chopping too much the data processing flow. Tracking filters for instance work better when processing directly the raw data than after some pre-processing has altered the completeness of the data (including noise).

Decision support tools play an important role in the traffic image displayed to VTSOs, as such, they should comply with data display recommendations listed in Annex 10 - Data portrayal, and the output of the traffic image should also comply with the recommendations listed in Annex 12 - Data information exchange and IVEF format for further readability and exploitation.

# External Information Exchange

## Introduction

VTS centres should be equipped with the ability to communicate with relevant allied services by the use of reliable communication networks.

The scope of this recommendation is to provide members with relevant aspects of exchanging maritime information globally. Details regarding legal issues and processes recommended for sharing maritime data, and more specifically terrestrial and satellite AIS, may be found in IALA guideline N° XX (The global sharing of maritime data and information Ed-0.1-15sept2011).

## Abbreviations and References

IALA dictionary: 5-3-340 Information: The meaning assigned to data by known convention

"Maritime data" is information regarding the maritime domain

IHO

ITU

## Characteristics of data exchange in VTS

### Uses

Comprehensive user needs have been well documented in IMO instruments and other IALA documents. Nevertheless, this technical recommendation offers a brief list of some potential applications or uses for maritime data inftormation. This list is not exhaustive and is offered to provide an indication of the range and diversity of activities that rely on marine data and information for their success, and which will further benefit from mutual exchange of data and information.

VTSs may be specifically concerned either because they are gathering information regarding these subjects and play a key role in providing these to other parts. They may also be concerned because they use, share or need to be aware of certain situations and should have access to this information. Dynamic updating is essential for maintaining the quality of marine data and information. This applies both afloat and ashore.

### Afloat

- Voyage planning and execution

- general VTS interests

  - Risk identification and avoidance (shipboard)

  - Monitoring of cargo, vessel status and resources

  - Track keeping and collision avoidance

  - Planning for sufficient under keel clearance

  - Weather routing

- Mainly port VTS interests

  - Cargo management (planning loading and discharge)

  - Logistics (shipboard)

- Regulatory compliance

  - Reporting

  - Environmental

- Seakeeping (stability and seaworthiness)

- SAR response

### Ashore

- Traffic management

  - VTS operations

  - Anchorage and berth management

- Hazard management

  - Risk analysis

  - Accident investigation

  - Contingency planning

  - Incident reporting

  - Emergency towage and salvage

- SAR

- Pilotage and allied services

- Support to logistic chain

  - Voyage monitoring

  - Port operation

  - Forward planning movements

- Law enforcement

  - Fisheries enforcement

  - Customs

  - Border control/ immigration

- Environmental protection

  - Pollution monitoring and control / response

- Waterways infrastructure management (including inland waterways)

  - AtoN operations and system optimisation

  - Infrastructure

- Maritime safety information (MSI)

## User data needs

An authority, organization or service that intends to undertake any of the above listed activities or operations requires timely, relevant and accurate data and information. Such data and information takes many forms and may be derived from many sources. While primarily focussing on vessels and environmental conditions, it may also address regulatory and technical matters. Furthermore, historical and baseline data and information need to be considered. The following list is, again, not intended to be exhaustive. It is meant to provide an indication of the range of data and information that is gathered, processed and exchanged by shore authorities, organizations and services in the conduct of their business and may be available for sharing.

### Vessel data

- Static

- Dynamic

- Voyage related data (cargo, crew, passengers, route, etc.)

- Defects (including local intelligence on defects)

- Incident reports

- Anomalous activities

### Environmental data

- Hydrographic

- Meteorological

- Physical environment

- Ecological

- Oceanographic (tsunami)

- Special areas of conservation

- Oil spill / pollution detection and reporting

- Signal propagation (atmospheric data)

### Regulation and references

- Port state control

- Technical references

## Operational, functional and performance requirements

### Data integrity

Data integrity is a key concern of both users and providers.

Source data holders are often reluctant to allow access to their data. If the intention is free and open exchange of data there must be a process trusted by all parties (providers and recipients) to enable access.

Users expect that data provided will be accurate and consistent. Furthermore, they expect the data to be authentic in that they are derived from credible sources which can be validated.

It is also of concern that there must be some means of confirming data integrity along the data supply chain because the route from provider to user may be a chain of different links with various opportunities for interference. Loss of integrity may be accidental or occur through deliberate interference.

Data should be transmitted using recognised formats such that the receiver will understand the format used by the sender.

Timeliness can be regarded as a part of data integrity (Sec. 4.4 Timeliness refers)

Quality of data is also very important. Data therefore should include some form of quality marker information (Sec. 4.4 Data quality and integrity refers)

Quality and integrity of data must also be thought into the transmission process between sender and user.

### Architecture of sharing

Suitability for purpose: Users need to be aware of the limitations of the data or information to avoid taking action based on inappropriate, incomplete or inaccurate data.

This suitability for purpose is obtained by transmitting information that has been used on the operational field (used by the transmitting VTS for example).

Sensor level sharing of data is usually less costly, but it provides less integrity and quality to data or the maritime information.

Information may be shared with or without being controlled by the sender. Usually, if the information shared is at sensor level, data may be routed directly from sensor to different receivers or it may be transmitted after transiting through a VTS, but with or without control or processing of the raw data. As such, it is of the responsibility of the receiver to ensure this information meets its own operational requirements before being used in an operational context.

System level sharing of processed data: The information is transmitted after having been controlled after the sensor level according to the specifications given in Annex 10 (Data processing). It usually is information that also has been used by a VTS before being transmitted to other parties. As such, this information fulfils the suitability for purpose mentioned earlier and it is supposed to be exploitable by the end user.

### Data models

Exchange of data requires an understanding of the meaning of each and every data item. This concerns the way in which data values are encoded and the exact meaning of data items. The former is specified by data format, the latter is reflected in the data model.

The data model unambiguously defines the semantics of the data item, the structure of a data item and the permissible values of a data item.

Since the use of data models is fundamental to the exchange of data, IMO, at NAV57, decided to institute the principle of a Common Maritime Data Model and the IHO S-100 has been proposed to be the baseline of this model.

The IHO S-100 standard is a framework standard intended to allow development of data models and associated product specification for a variety of common and maritime specific information. The data models used in the domains of maritime safety, security and more generally that may be used or acquired by VTS are described in the "data registry".

### Timeliness

Information should be received when needed. This may be in advance of an event, real time, near real time or historic as appropriate. Data should be time stamped as appropriate to the nature and use of the information. Time stamp should preferably be at the time of origin, but if not, it should be stamped as soon thereafter as possible. Where time stamp is not the time of origin, it is desirable that the difference involved be flagged.

Quality of service (QoS) covers the prioritisation of certain data above others in order to guarantee a timely delivery of the data. Higher priority data will be delivered faster than a lower priority item.

In real-time or near real-time systems, it may be necessary to ensure that some data types have priority above others. Furthermore, when Data Sharing Networks use infrastructure, which also is used by other data systems (such as internet, email or phone services), it may be necessary to ensure quality of service for the preferred system.

### Storage

The volume of data and information involved in many of the aforementioned uses will be considerable. Given that many of these uses also require access to archive or historic data and information, consideration must be given to providing adequate capacity for retaining and archiving these records.

Some formats are well-suited for transfer and sharing of data and maritime information whereas some formats are more suited for long term archiving of data. (Sec 5.2.2 Common formats gives example of common data formats)

### Access to data and information

The reception and use of broadcast information is subject to ITU-R Radio  Regulations article 17 on Secrecy. Clear and realistic principles and rules regarding access to AIS and other navigation safety data should be defined and adopted by the international community.

National competent authorities could have criteria to ensure that exchanged data is of the highest quality. E.g., the established international system of exchanging Maritime Safety Information. The quality of exchanged information should be made known to the end user.

Exchange of data may be clearly defined for both sender and user through the use of a sharing agreement. This document should state clearly the level of service needed and provided, the security and privacy wanted for the data. Finally, it should clearly state the use of the data, if it is to be used in operational environment, for maritime purposes or for academic or historical studies for instance.

### Data security and confidentiality

Information that requires protection includes localization of sensitive information such as localization of fishing grounds or personal identification information. Personal data includes identity data relating to vessels as well as individuals.

Users are concerned with issues of data security and confidentiality and in particular any commercial sensitivity of data as it relates to release of information that may compromise investors or introduce competitive advantage/disadvantage.

In many cases confidentiality is already protected by legislation but this is not universal throughout the maritime domain. The requirement to protect access to data may go beyond the limits of primary legislation. Confidentiality needs, at least to be protected by appropriate levels of access rights to data exercised through physical security encryption and password protection.

If personal and commercial identification is not needed, for academic studies for instance, all related data may be blanked in order to suppress any doubts on the legal issues linked to the exchange of maritime information.

Data confidentiality should be defined in the sharing agreement. It means that the data is protected against eavesdropping. No other parties other than the sender and receiver are able to read the data. Data confidentiality can be obtained by physical protection, authentication or data encryption. The main objective is to prevent access to the data source or to data link.

Authentication means that the sending and receiving parties are able to unambiguously identify each other. This means that each party knows who he is communicating with.

Encryption is commonly obtained by using a secret key that is only accessible to authorised parties. Depending on the sensitivity of the data, a certain level of data encryption may be required.

### Legal limitations

Many national states, in the lawful exercise of their authority, place legal limits on the exchange and public dissemination of data and information. These include protections on intellectual and commercial property rights, and limitations on third party use of data and information.

In the course of exchanging maritime data and information in the interest of safety, security and efficiency, these limitations must be respected and the authorities involved must be aware of their rights and obligations under law. In particular data received should be consistent with the laws of the national authority receiving the data.

Authorities need to be aware of any exposure to liability that might occur from their actions or inactions with regard to data and information exchange.

### Communication links

The transfer of data from A to B requires connectivity via a data link or more generally a network. A network comprises appropriate hardware and software interconnected by communication channels. In the maritime world, both aboard ship and shore side, data links may be wired or wireless.

Different technical solutions and architectures can be used when establishing a data sharing network. Consideration should be given to:

the physical distance between the sending and receiving parties,

the services provided by the network,

the quality of services requested by the users.

The processes established to guarantee the quality of the services provided by a data sharing network should be carefully defined and monitored and could be part of a Quality Management System.

Global sharing of maritime data and information can take place either through the internet or through dedicated networks. The internet is public, while dedicated networks are generally closed. Consideration should be given to the security related characteristics of theses different network types.

Systems used for global sharing of maritime data and information are in reality a network of networks.

When designing a network for global sharing of maritime data, consideration should be given to transmission protocols, bandwidth limitations, communication / data distribution strategy, security aspects such as authentication and confidentiality as well as data integrity.

A selection between the options available should be based on a number of criteria, including the type of data being transferred, volume of data, types and number of clients connected to the network.

Although bandwidth cost is ever decreasing, there is still a cost associated with the transfer of a certain data item. So the value of the conveyed information has to be balanced against the cost of transmitting it. Another trade-off is the time required to transfer a data item versus a higher required bandwidth (with increased cost). In the future, one can expect to have more flexibility in terms of roaming i.e. dynamic choice of communication links with different bandwidths coverage and costs.

## Specific Design and Installation Considerations

### Interface Requirements

As mentioned earlier, many ways exist in order to ensure one that he is able to exchange and disseminate data and maritime information while meeting the recommended level of security and integrity regarding the data.

If the network used is administrated by the VTS, it is possible to make sure that the data links (sub-network, protocols, ports and sockets) used for data sharing are differentiated from supervising tasks such as sensor monitoring and administration of other parts of the system.

Common authentication and encryption methods and protocols:

Basic rules of authentication and network security

Advanced methods based on specific software, protocols and materials.

### Relevant technical standards

#### Common standards

It is widely acknowledged that there are already a significant number of standards covering description and transfer of data. However, still a few gaps remain that need standardisation, particularly in the field of equipment monitoring and control, interfacing of various sensors etc.

It is necessary to use the most appropriate standard for the current task although there are a number of choices. Some relevant standards and formats are listed below:

ISO 19100 series

19119:2005: identifies and defines the architecture patterns for service interfaces used for geographic information

19115:2003: defines schema (template???) required for describing geographic information and services. It provides information about the identification, the extent, the quality, the spatial and temporal schema, spatial reference, and distribution of digital geographic data.

19139:2007: defines Geographic MetaData XML (gmd) encoding, an XML schema implementation derived form ISO 19115

IHO digital data transfer standards

S-52: Portrayal: provides specifications and guidance regarding the issuing and updating of Electronic Navigational Charts (ENC), and their display in ECDIS. S-52 comprises a number of separate documents.

S-57: The IHO Transfer Standard for Digital Hydrographic Data

S-100: New developed standard for marine data and information data modelling. S-100 will incorporate the requirements of S-57 for ENCs and ECDIS. Significantly it will be aligned with the ISO 19100 series of geographic information standards.

S-63: IHO Data Protection Scheme: An IHO standard used to enable the authentication, integrity and confidentiality of ENC data throughout the data distribution chain from Producer Hydrographic Office to individual seafarer licence holder.

Since maritime information can be spatial information and related to the field of Environment, further guidance can be found in the European INSPIRE directive for establishing an infrastructure for spatial information as well as the CF - Climate and Forecast Metadata convention (WMO).

#### Common formats

Some of these following formats are well suited for real-time data transport, while the others should be considered for storage and retrieval of historical data on a non real-time basis.

| Format name | Responsible organization | description | application |
| --- | --- | --- | --- |
| IEC 61162 | IEC | Maritime navigation and radiocommunication equipment and systems - Specification for communications between marine electronic devices. | Real-time |
| IVEF | IALA | Inter VTS Exchange format (IALA recommendation V-145) | Real-time |
| ITU-R 1371-4 | ITU | Technical characteristics describing the format of NMEA phrases used in the maritime domain. | Real-time |
| NetCDF | UCAR/Unidata | NetCDF is a set of open software libraries of the Open Geospatial Consortium. This self-describing, machine-independent data format, supports the creation, access, and sharing of array-oriented scientific data. | Non Real-time |
| HDF5 | HDF Group | A data model, library, and file format for storing and managing data. | Non Real-time |

# Verification and Validation

VTS equipment shall be verified prior to operation and the most important performance parameters should be monitored continuously or periodically during operation.

This includes:

- Type approval of individual equipments, as required by law in individual countries.

- Other equipment specific verification tests as required by the individual VTS authority

- Verification of equipment prior to delivery in the form of Factory Acceptance Tests

- Verification of individual equipment or systems upon installation and Setting To Work, but prior to  prior to operational used,  in the form of Site  Acceptance Tests

- In operation test, e.g. during an initial operation period of a new or updated VTS

- Continuous or periodic monitoring of selected functions and parameters during operation of the VTS

The overall specifications should be agreed in contractual documents, and it is recommended that FAT, SAT ant other procedures are agreed upon before conducting test.

Procedures may be generic to the individual equipment and/or specific to the individual contract.

Elements below should be adapted to respect legal national texts of the VTS Authority.

## Legal requirements to type approvals

Legal requirements vary from country to country, they are in continuous development and it is the responsibility of the VTS authority to ensure compliance.

However, the tendency is to adapt international and regional standards and the below list reflect the most commonly used legislations and methods to ensure compliance at time of editing this ANNEX 13 (January 2012)

On some aspects, the law requires test by accredited institutions on other aspects the VTS Authority can decide.

### Electrical Safety

Low Voltage Directive (LVD) 73/23/EEC

Compliance is required in order to obtain CE marking (mandatory within EU) and best ensured through fulfilling standard IEC60950-1, 2.ed

### Mechanical Safety

Machinery Directive 2006/42/EC applies to mechanical parts of the radar system, i.e. antennas, gearboxes.

Compliance is required in order to obtain CE marking (mandatory within EU) Compliance can be documented by a mechanical safety analysis.

### Radiation Safety (Radar)

Physical Agents Directive 2004/40/EC

Compliance is required in order to obtain CE marking (mandatory within EU)

Compliance can be documented through measurement of RF power densities from transmitting radio equipment and radar.

However, for pulsed energy, measurement of power densities require sophisticated test equipment and it is normally accepted to verify worst case radiation levels and determine corresponding safety distances through an engineering report on power densities around the antenna utilizing the guidelines laid down by the International Commission on Non-Ionizing Radiation Protection (ICNIRP).

### ElectroMagnetic Compatibility

EMC Directive 2004/108/EC

Generic EMC standards used:

- Immunity as industrial equipment acc. to IEC 61000-6-2

- Emission as residential equipment acc. to IEC 61000-6-3.

Compliance is required in order to obtain CE marking (mandatory within EU)

### Radio Spectrum Requirements

#### EU Requirements:

R&TTE Directive 1999/5/EC.

No harmonized test standards exist, it is recommended to follow the below guidelines:

ECC/Recommendation (02)05 (2004): “Unwanted emissions”

ERC/Recommendation 74-01 (2011): “Unwanted emissions in the spurious domain”

#### ITU Recommendations:

ITU-R Recommendation SM.1541-3 (2011): “Unwanted emissions in the out-of-band domain”

ITU-R Recommendation SM.329-11 (2011): “Unwanted emissions in the spurious domain”

#### US Federal Requirements

NTIA - Manual of Regulations and Procedures for Federal Radio Frequency Management

#### Chinese Radio Transmission Type Approval

Compliance to ITU recommendations.

#### Additional IALA recommendation.

In addition to the legal requirements especially radar performance is highly affected by radiation characteristics, it is recommended that the supplier decides on the most economical solution to meet the individual performance requirement, especially focussed on resolution, coverage and weather penetration

### RoHS

Radar systems shall be designed to comply with the RoHS directive 2011/65/EU (RoHS II) by July 22. 2017.

Other equipment .....

China RoHS is not yet affecting design of radar transceivers, but require additional marking and documentation.

### Chemical Substances

REACH Directive (Regulation (EC) No 1907/2006)

Manufacturer must be able to document that no banned substances in concentrations above the limits are present in the product.

## Additional type tests

### Equipment in general

IALA recommends that all equipment is type tested , preferably by accredited institutes, to the standards and specification levels stated in Table 13-21. This is valid for shore based and off-shore installations above water.

Table 13-21  Recommended equipment standards and specification levels

| Standard | Subject | Condition | Recommended limits versus equipment location |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Conditioned indoor, office or equipment room | Outdoor climatic category |  |  |
|  |  |  |  | Very hot | Normal | Severe Cold |
| IEC 68-2 | Dry Heat | Function | 40C | 55C | 45C | Design to specifics |
|  |  | Storage | 70C |  |  |  |
|  | Cold | Function | 10C | 0C | -30C |  |
|  |  | Storage | - 30C |  |  |  |
| IEC 529 | Protection | Storage and function | IP 20 | IP 54 |  |  |
|  | Sun radiation | In addition to ambient temperature | N/A | 0 - 1120 W/m2 |  | Negligible |

### Hydrographic equipment .....

ESD - They are labeled with yellow/black triangles.

## Software Quality Assurance

Remember network

Check annexes 9, 10 and 12 before writing.

## Factory Acceptance Test

The purpose of the Factory Acceptance Test is to prove that the individual apparatus is conformal to contractual specifications, is well manufactured, and ready for packaging and shipment.

The purpose is also to review the quality control documents and ensure the overall functionality of the complete system.

This will normally include Functional and Performance testing to agreed procedures. Test will normally be performed for individual units and in some cases for pre-assembled systems.

Personnel conducting the test must be familiar with set-up and operation of the equipment in test. Evaluation of e.g. image quality and instrument readouts may demand experienced operators. Be specially aware of Safety Instructions.

### Procedure

Test procedures will normally follow the standards defined by manufacturers. They should include:

- References to project name, customer, software revisions, hardware revisions, part and serial numbers etc.

- List of test instruments and check if “Calibration valid until” date is OK.

- Review of quality control documents including manufactures Declaration of Conformity and Configuration Item Records

- Visual inspection, are the equipment without visible physical defects, properly marked, and connected in accordance with good workman practice (grounding, shielding etc.)

- Functional tests including complete verification of any safety measures, check of power failure and recovery from that, and sample check of functions in general.

- Performance tests at least including measurement of most important parameters. The procedures should also specify measurement tolerances.

- Signatures

The VTS authority may select to witness the FAT. In case of the manufacturer being ISO certified, the VTS Authority might also decide to rely on reporting from the manufactures QA organization.

After FAT, the vendor shall make sure that any finding are corrected, that all settings are in normal operational state and that equipment is ready for shipment, turn power switches off etc.

## Installation and Site Acceptance Test

The VTS Authority should assure:

- Check of equipments after shipment, ensure that all parts and documentation are delivered and in good condition.

- Check of civil works, structures etc. are completed and in good condition prior to installation of any equipment.

- That manufacturer’s instructions are followed when lifting equipment.

- Good workmanship practice is followed when installing equipment, special precautions to safety switches, grounding, shielding and lightning protection may be required.

The VTS Authority should also assure that safety legislations are meet.

CAUTION: 	High voltages may be present at several points in equipments. Such voltages may cause injury and even death.

Equipment may contain parts and assemblies sensitive to damage by electrostatic discharge (ESD). Always use ESD precautionary procedures when touching, removing, or inserting such parts and assemblies.

RADIATION HAZARD may be present. E.g., do not transmit into a stopped radar antenna, and beware of any safety distances specified.

Keep a safe distance from rotating antennas. Always check that the safety switch is turned off before starting work on a rotating antenna unit.

Upon installation and Setting To Work, Site Acceptance test should be performed by the vendor(s) and witnessed by the VTS Authority.

The purpose of the Site Acceptance Test is to prove that the overall system functions and performance is conformal to contractual specifications, is well installed and integrated, and ready for operational use.

This will normally include Functional and Performance testing to agreed procedures. Test will normally be performed for the overall system and possibly also for separate features, services and sections of the system.

Personnel conducting the test must be familiar with set-up and operation of the features and services in test. Evaluation of e.g. image quality may demand experienced operators.

### Procedure

Test procedures will normally follow the standards defined by vendors, but for overall system functionality and performance, tailored to the individual contract. They should include:

- References to project name, customer.

- Verification to check that equipment is as tested during FAT. Verify software revisions, hardware revisions, part and serial numbers etc. If updates, e.g. installation of new software, have been performed after FAT, it may be necessary to repeat elements of testing performed during FAT.

- Inspection of check lists from installation and setting to work.

- Visual inspection of the installed equipment, are the equipment without visible physical defects, properly marked, and connected in accordance with good workman practice (grounding, shielding, lightning protection etc.)

- Verification of safety and security measures including but not necessary limited to physical access barriers, network firewalls, warning signs and protection against radiation hazards.

- Functional tests including complete verification of any safety measures, check of power failure and recovery from that, and test of functions in general.

- Verification of overall most important parameters. It should not repeat those measurements made during FAT, but may e.g. include calculation or measurement  of overall losses in installed systems. The procedures should also specify acceptable tolerances on such calculations and/or measurements.

- Cross check of information from different sensors and check of alignment.

- Verification of performance by scenarios if needed (Not necessary for basics sensors).

- Signatures.

- Deviation and waiver management.

### Scenarios

Site acceptance test should include test scenarios utilizing traffic of opportunity and/or predefined controlled test objects. Such scenarios should address, normal and extreme situations e.g. risks of collision, and handling of rapid manoeuvrings.

#### Continuous coverage

Recording of trajectories over long time, from traffic of opportunity, will test coverage and reveal any unexpected shadows etc.

- insert illustration

This may also include an evaluation of sensor range performance if characteristics of targets are known, i.e. for test of radar target characteristics may be obtained by combination of AIS and database information about individual tests. Likewise, the characteristics and position of AToNs will be known.

#### Test with controlled targets

#### Calibration of reference targets for radar

## In Operation Test

Depending on the complexity of a VTS project, SAT may be followed by In Operation Tests, prior to entering full operation and final reception.

The purpose of the IOT is to ensure operation reliability and continuous performance to contractual specifications.

This will include testing to agreed procedures by operators trained on the actual system, possibly assisted by the system supplier.

## In operation monitoring and calibration

### Continuous monitoring

The majority of equipment used in VTS will include Built-in Test Facilities. This should be monitored, critical failures detected must be reported to operators without significant delay, temporary change to fallback methods (e.g. switch to other sensors) may be necessary and corrective measures shall be performed.

### Off line test

### Calibration of sensors

The VTS authority is recommended to calibrate its sensors at least once per year as required per ISO certification. This calibration, validate/correct the precision, stability and function of sensors.

Results of calibration, metrological corrections etc. may be stored in the sensor itself or it should be applied in data processing.

Within one standard deviation (Gaussian distribution) when sailing on a straight course.  Note that verification may require simulated tracks or other methods due to the fact that it may be impossible to direct a test target to sail with sufficient accuracy.