VTS34           input paper

Agenda item   action 45

Task number

Author              Jean-Charles CORNILLOU

Prevention of collision and action taken following a close quarter situation

by Ushant VTS

SUMMARY

This document presents the process adopted at Ushant Traffic to prevent collision and action taken following a close quarters situation.

1. Purpose of the document: The Committee is invited to note the process adopted by Ushant Traffic and Member States are invited to co-sign and support the guidance presented in appendix 3 as a contribution to sub-committee NAV at IMO.

2. Related documents: IMO Res. A 857(20) guidance for VTS / SOLAS IX – ISM Code / MSC-MEPC.7/Circ.7 guidance on near-miss reporting

BACKGROUND

1.	In compliance to IMO Res. A 857(20), point 2.3.3, Ushant Traffic VTS centre (in French : Ouessant Trafic) is dedicated to monitor the traffic in order to avoid dangerous situation.  The area of Ushant VTS is a circle of 40 NM radius centre on Ushant island, including a TSS and fairways along the coastline and different islands and rocks.

2.	Operators at Ushant Traffic must be aware of the limits of the different sensors they are using. The radar information is refreshed every 12 s and AIS information can be altered because AIS transceivers are not always properly fitted and set on ships. There are still direction finders (D/F) with a 0.5° precision to help operator to locate a ship-borne tranciever on VHF. Hence it is recommended to operators at Ushant Traffic to exercise a critical mind when jauging the information provided by radar, AIS or D/F.

3.	In compliance to COLREG 72, rule 10, attention must be exercised at both ends of the TSS for the density of traffic and cross situations generate a lot of close quarter situations as well as turning point. The weather conditions have to be taken into consideration, in particular the visibility. As a matter of experience in Ushant Traffic, it rises up to a compromise that a call on VHF can be effective to the give-way vessel at last 10 mn before CPA.

4.    When a close quarter situation is detected, the operator tries to contact first the give-way vessel on VHF 13 or 16 or with the help of a DSC at last. In compliance to IMO Res.857 A(20), the VTS centre has no manoeuvring order to summon to the ship. The task of the VTS is but to ensure that the watch-keeper on board ships have the information to decide the proper manoeuvring in order to avoid a close quarter situation. A close quarter decision tree in appendix 1 has been developed as an indication to VTS operator.

DISCUSSION

Action taken in case of infringement to COLREG 72

5. 	In case of infringement to COLREG 72 in the VTS, a report to the Flag State is established with all documentary evidences: radar screen prints and VHF records if necessary. The report is forwarded to the Flag State through central office and foreign affairs ministry. A related message can be allocated to the ship as well as in THETIS the data base of Paris MoU on PSC and requirement to such notification are mandatory under EC directive 2009/016 on PSC.

Action taken in case of a close quarter situation

6.	In case of a close quarter situation detected in the VTS area, in compliance to COLREG 72, it is not obvious to demonstrate an infringement to the rules of the road for the rule 16 relative to the “action by the give-way vessel” should be balance with rule 17 relative to the “action of the stand-on vessel”; hence the following action taken are carried out:

6.1. The master of the ships involved in the close quarter situation is invited to make a sea-report to their Flag State Authority;

6.2. A “dangerous situation notice” (report to appendix 2) is established with an analysis of the situation;

6.3. A mail is sent to the ship company of the give-way vessel with documentary evidences including the dangerous situation notice and radar screen prints if necessary. A copy is sent to the Flag State Authority, the Class Society delivering the ISM certificate and to the company of the stand-on vessel in the case of this latter made any complain to Ushant Traffic following the manoeuvring of the other vessel.

- As MSC-MEPC.7/Circ. 7 guidance on near-miss reporting has been included in the ISM Code since 1st January 2010, the idea of this action taken is :

7.1. To aware watch keepers of merchant ships and fishing vessels to navigate with caution in a VTS;

7.2. To aware companies of merchant ships on the management of the competency of their personnel in charge of watch-keeping;

7.3. To inform Flag State authorities and Class Society acting on behalf of them of the situation in order to exercise attention in a future audit of the company and the ship on the resources and personnel point of the ISM, in particular for personnel in charge of watch-keeping on the bridge.

Feed-back:

8. Ushant Traffic has received positive feed-back from companies, Flag States and Class Societies until now for 4 years this procedure is in force. Internal feed-back is also very positive for operators training and the quality system of the centre.

REFERENCES:

Guidance on close-quarter situation reporting by VTS (appendix 3)

ACTION REQUESTED TO THE COMMITTEE

9. The Committee is invited to note the process adopted by Ushant Traffic and the way close quarter situation are reported. This reporting process should be considered as a declination from MSC-MEPC.7/Circ. 7 guidance on near-miss reporting in order to promote a “just culture” features and atmosphere of responsible behaviour and trust for navigators and shipping companies, as well as VTS personnel. The analysis compilation of close quarter reporting can also be used as training material for VTS.

APPENDIX 1 : close quarter situation Ushant Traffic VTS decision tree

APPENDIX 2 : Dangerous situation notice

APPENDIX 3 : input document to NAV, « Guidance to close-quarter situation reporting by VTS »

## Close quarter situation VTS decision tree

| ## Close quarter situation VTS decision tree |  |
| --- | --- |
| Who | What |
| Speed vectors are converging and indicate that vessels will be in close-quarter situation if no manoeuvring Communicate all info to vessels (relative position of vessels, identification of vessels, course and speed) Draft a contravention report Stop all contact – Follow ships’ manoeuvring Follow carefully ships’ manoeuvring Follow carefully the evolution of the situation till it is over Maintain eventualy a contact with vessels to stress on the emergency if no manoeuvring – Follow the evolution Indicate on monitor CPA and DCPA Is CPA less than 0.5 NM WITHIN NEXT 10 NM ? NO YES Contact vessels / contact first the give-way vessel in compliance to COLREG72. Do the give-way vessel or both vessels answer ? YES NO Try a contact with DSC or Inmarsat if possible. Do the give-way vessel or both vessels answer ? YES NO Ensure that watch-keeper is aware of the situation – Indicate CPA and DCPA Do watch-keepers have all information ? NO YES DCPA is less than 3 minutes (*) YES NO VTS VTS VTS VTS | Draft a close quarter situation notice – Record vessels radar and AIS tracks |

(*) delay to set depending of the VTS navigation conditions and sensors

Dangerous situation notice

1- Nature of the situation						Date / time :

dangerous situation close quarter situation collision

2- Vessels involved

| Ship’s name | IMO n° or Registration n° | FLAG | Type of ship | LoA (m) | Position (longitude & latitude) | Speed (kt) | Course |
| --- | --- | --- | --- | --- | --- | --- | --- |

3- Description of the event

# Most critical situation : CPA : TCPA : Position relative to a mark ashore : Visibility Evolution of vessels

4- Analyse

5-Conclusion

6-Enclosed documents

radar & AIS monitor print               vessels’ data               other :

## Conclusion on the action of the VTS :

Does the vessel(s) change course following VTS information?

Yes   	No  		Vessel does not answer