Suggested Changes to IMO Resolution A.918(22) Standard Marine Communication Phrases):

2.2		Spelling of digits and numbers (P.13)

A few digits and numbers have a modified pronunciation compared to general English:

| Number | Spelling | Pronunciation |
| --- | --- | --- |
| 0 | zero | ZEERO |
| 1 | one | WUN WUN |
| 2 | two | TOO |
| 3 | three | TREE |
| 4 | four | FOWER |
| 5 | five | FIFE |
| 6 | six | SIX |
| 7 | seven | SEVEN |
| 8 | eight | AIT |
| 9 | nine | NINER |
| 1000 | thousand | TOUSAND |

3       Message Markers

(i) 1    Instruction

(ii) 2   Advice

(iii) 3   Warning

(iv) 4   Information

(v) 5   Question

(vi) 6   Answer

(vii) 7  Request

(viii) 8  Intention

4    	Responses (P.14)

4.5  	When an INSTRUCTION (e.g. by a VTS Station, naval vessel or other fully authorized personnel) or an ADVICE is given, respond:

"I will/can ... " - followed by the instruction or advice in full; or,

"I will not/cannot ... " - followed by the instruction or advice in full.

Example: "INSTRUCTION. Slow down your vessel immediately.."

Respond: "I will slow down my vessel immediately”

Example: "ADVICE. Do not overtake the vessel North of you."

Respond: "I will not overtake the vessel North of me."

Example: "WARNING. Obstruction in the fairway."

Response: “Warning received. Obstruction in the fairway.”

Example:  "INFORMATION. MV Noname will overtake to the west of you.”

Response: “Information received. MV Noname will overtake to the west of me.”

Example:   "QUESTION. What is your present maximum draft?"

Response:  “ANSWER. My maximum draft is……meters.”

Example:    “REQUEST. I require two tugs."

Response:  “REQUEST received. You require two tugs”

Example:   “QUESTION. What is your intention?”

Response:   "INTENTION. I will reduce my speed."

6       Standard organizational phrases (P.15)

6.2     When it is advisable to remain on a VHF Channel … / MF/ HF frequency.…, say:

"Stand by on VHF Channel .... / MF/HF frequency .... ".

6.2.1   When it is accepted to remain on the VHF Channel… / frequency indicated, say:

"Standing by on VHF Channel ... / MF/HF frequency ... ".

6.3     When it is advisable to change to another VHF Channel…. / MF/HF frequency…., say:

"Advise (you) change to VHF Channel ... / MF/HF frequency ... ."

"Advise(you) try VHF Channel …. / MF/HF frequency... .".

6.3.1       When the changing of a VHF Channel….. / MF/HF frequency….. is accepted, say:

"Changing to VHF Channel ... / MF/HF frequency ... .".

10       Numbers (P.16)

“Two point five” for 2.5

11       Positions

11.1     When latitude and longitude are used, these shall be expressed in degrees and minutes (and decimals of a   minute / seconds if necessary), North or South of the Equator and East or West of Greenwich.

Example:  "WARNING. Dangerous wreck in position 15 degrees 34 minutes 20 seconds  North

061 degrees 29 minutes 30 seconds  West."

18.2 	The word "can" (P.17)

The word "can" describes either the possibility or the capability of doing something. In the IMO SMCP the situations where phrases using the word "can" appear make it clear whether a possibility is referred to.

Can

Do not say:	"Can I use the shallow draft fairway at this time?"

Say:	" QUESTION. Do I have permission to use the shallow draft fairway at this time?"

GLOSSARY (P.21)

Hoist	 A cable used by helicopters for lifting or lowering persons in a pick-up operation

List	I Inclination of the vessel to port side or starboard side

Recover (to)	 To pick up shipwrecked persons

Resume (to)	 To re-start a voyage, service or search

Elongated spreader	 Step of a pilot ladder which prevents the ladder from twisting (Suggested to move the item up to the letter ‘E’ category)

Stand clear (to)	 To keep a boat away from the vessel

SWL                     	Safe wWorking l Load: maximum working load of lifting equipment that should not be exceeded

VTS	Vessel Traffic Services: services implemented by a Competent Authority, designed to improve the safety and efficiency of vessel traffic and to protect the environment. The service should comprise at least an information service and may also include others, such as a navigational assistance service or a traffic organisation service, or both which should have the capability to interact with the traffic and to respond to traffic situations developing in the VTS area. (from IMO Resolution A.857 (20))

VTS area 	the delineated, formally declared service area of the VTS. A VTS area may be subdivided in sub-areas or sectors (from IMO Resolution A.857 (20))

VTS authority 	the authority with responsibility for the management, operation and coordination of the VTS, interaction with participating vessels and the safe and effective provision of the service. (from IMO Resolution A.857 (20))

VTS centre 	the centre from which the VTS is operated. Each sub-area of the VTS may have its own sub-centre.(from IMO Resolution A.857 (20))

MF	Medium Frequency

HF	High Frequency

A1/1.1		Distress communications (P.30)

Note: A distress traffic always has to commence with stating the position of the vessel in distress as specified in “GENERAL 11 Positions /12 Bearings” if it is not included in the DSC distress alert.

A1/5.2	Ice - breaker operations (P.44)

.2 		Ice - breaker assistance for convoy

A1/6	Vessel Traffic Service (VTS) Standard Phrases (P.45)

Application of Message Markers

In order to especially facilitate shore-to-ship and ship-to-shore communication or when one of the IMO Standard Marine Communication Phrases will not fit the meaning desired, one of the following eight message markers may be used to increase the probability of the message being properly understood.

It is at the discretion of the shore personnel or the ship’s officer whether to use one of the message markers .  The marker to be used would depend on the user’s qualified assessment of the situation. If used, the message marker is to be spoken preceding the message or the corresponding part of the message. The IMO VTS Guidelines recommend that in any message directed to a vessel it should be clear whether the message contains information, advice, warning, or instruction (see below) and IMO Standard Marine Communication Phrases should be used where practicable.

For further standardized VTS communications, also see other sections of Part A1. For VTS Standard Reporting Procedures, see IMO resolution A.851(20) on General Principles for Ship Reporting Systems and Ship Reporting Requirements, including guidelines for reporting incidents involving dangerous goods, harmful substances and / or marine pollutants.

Note: All of the following phrases must come as the culmination (message content) of a radio message exchange between stations covered by the ITU Radio Regulations, and the relevant calling procedures have to be observed.

Message Markers (P.46)

The underlying intent of message markers is to escalate the importance of the message being communicated.

- INFORMATION

This indicates that the following message is restricted to observed facts, situations, etc..

Comment:	This marker is preferably used for navigational and traffic information,etc.

Consequences of INFORMATION will be up to the recipient.

Example:	 "INFORMATON. MV Noname will overtake to the west of you"

- (ii)QUESTION

This indicates that the following message is of an interrogative character.

Comment:	The use of this marker removes any doubt as to whether a question is being asked or a statement is being made, especially when interrogatives such as what, where, why, who, how are additionally used at the beginning of the question. The recipient is expected to return an answer.

Example:	"QUESTION.(What is) your present maximum draft?"

- (iii)ANSWER

This indicates that the following message is the reply to a previous question.

Comment:	Note that an answer should not contain another question.

Example:	"ANSWER. My present maximum draft is seven decimal zero metres."

- (iv)REQUEST

This indicates that the following message is asking for action from others with respect to the vessel.

Comment:	The use of this marker is to signal: I want something to be arranged or provided,

e.g. ship´s stores requirements, tugs, permission, etc..

Note:	REQUEST must not be used involving navigation, or to modify COLREGS.

Example:	"REQUEST. I require two tugs."

- (v)INTENTION

This indicates that the following message informs others about immediate navigational action intended to be taken.

Comment:	The use of this message marker is logically restricted to messages announcing navigational actions by the vessel sending this message.

Example:	"INTENTION. I will reduce my speed."

- (vi)ADVICE

This indicates that the following message implies the intention of the sender to influence others by a Recommendation.  The use of the message Marker ADVICE would normally follow Information and or Warning Messages.

Comment:	The decision whether to follow the ADVICE still stays with the recipient. ADVICE does not necessarily have to be followed but should be considered very carefully.

Example:	WARNING. Vessel departing berth”

"ADVICE. Do not cross the Fairway."

- (vii)WARNING

This indicates that the following message implies the intention of the sender to inform others about danger.

Comment:	This means that any recipient of a WARNING should pay immediate attention to the danger mentioned. Consequences of a WARNING will be up to the recipient.

Example:	"WARNING. Obstruction in the fairway."

- (Viii)	INSTRUCTION

This indicates that the following message implies the intention of the sender to influence others by a Regulation.  The use of the message Marker INSTRUCTION would normally follow the provision of Information, Warning and or ADVICE Messages.

Comment:	This means that the sender, e.g. a VTS Station or government authorized vessel, must have full authority to send such a message. The recipient has to follow this legally binding message unless he/she has contradictory safety reasons which then have to be reported to the sender.

Example:	WARNING. Vessel departing berth”

"INSTRUCTION. Do not cross the fairway.”

A1/6.1	Phrases for acquiring and providing data for a traffic image (P.47)

.2   		What is your flag -?

.2.1 		My flag - is .....

.6   		What is your next port of call -?

.6.1 		My next port of call is ... .

.15  	What is your full sea speed / full manoeuvring speed?

.15.1 	My full sea speed / full manoeuvring speed is ... knots.

.17.1   	Yes, I carry the following dangerous goods:  ... kilograms / tonnes IMO Class/BC Category ... .

.19  	I am constrained by my draft.

A1/6.2   	Phrases for providing VTS services (P.48)

The use of appropriate Message Markers as described above is encouraged in ship-to-shore and shore-to ship communication involving a VTS centre.

.1 Information Service (INS)

These phrases are normally transmitted from the shore.

.1.3	     Traffic information

.3	Salvage operations in position ... from ... (date and time) to ... (date and time).
Wide berth requested. Contact (ship’s name) on VHF Channel ... .

.4	                     Seismic / hydrographic operations by ... (vessel)... from ... (date and time)

to ...(date and time) in position ... Wide berth requested.

Contact (ship’s name) on VHF Channel ..

.1.4	Route information

.1	     Route .../ Traffic Lane ... has been suspended / discontinued / diverted.

1.6	    Electronic navigational aids information (P.50)

.1	  	GPS Satellite ...(number) unusable from ...(date and time) to ... (date and time).

.

.2	LORAN station ...(name number of master / secondary ) unusable from ...(date and time) to ... (date and time).

.2 	Navigational Assistance Service (NAS) (P.51)

Shore based pilotage by navigational assistance service: see also A1/6.4 .3.18 to .3.21 and IALA Guidelines 1068 on NAS.

Navigational Assistance Service provided by VTS: see also A1/6.4 .3.18 to .3.21 and IALA Guidelines 1068 on NAS.

.2.1 	Request and identification

.1       Is shore based radar assistance available?

.1.1         Yes, shore based radar assistance is available.

.1.2         No, shore based radar assistance is not available.

.2       Shore based radar assistance is available from ... to ... UTC.

.3 	Do you require navigational assistance service ... ?

.7      I have located you on my radar screen (radar /AIS)

.8      I cannot locate you on my radar screen (radar / AIS)

.13     You are leaving my radar / AIS screen coverage.

.14 	  Change to VHF Channel ... .

.15	I have lost your AIS signal. Check your AIS equipment.

.2.2     Position (P.52)

.11      MV ... is  metres / cables ... (cardinal points) of you.

~ is ingoing inbound / outgoing oubound.

.2.3 		Course (P.52)

Note: the user of this phrase should be fully aware of the implications of words such

as "track", "heading" and "course made good".

.5.1 		Advise you

~ Keep your present course.

.

~ A new course made good of ..…. degrees.

.3 Traffic Organization Service (TOS)

.3.1     	Clearance, forward planning (P.53)

.10      	The ebb/falling tide is with you / against you.

.3.4 	Enforcement (P.54)

.1 	According to my radar (Radar /AIS) screen, your course does not comply with Traffic Separation Scheme Regulation (Rule 10) of the COLREGs.

.

.4    Have all navigational instruments in operation before entering this area / area ... .

.10	Fairway maximum permitted speed is... knots.

.3.5 	Avoiding dangerous situations, providing safe movements

.12 	You are proceeding at a dangerous speed. (Increase/reduce your speed)

.3.5 	Avoiding dangerous situations, providing safe movements (P.55)

.3 	Pilot request (P.56)

.6 	What is the local time now?

.21 	Do you require shore-based navigational assistance from VTS Centre?

.21.1 Yes, I require shore-based navigational assistance.

.21.2 No, I do not  require shore-based navigational assistance.

Appendix to A1 - External Communication Phrases (P.57)

Standard GMDSS Messages

For further details, see ITU Manual for use by the Maritime Mobile and Maritime Mobile

Satellite Services, Geneva.

1 Standard distress message

.1 Structure

Upon receipt of a DSC Distress Alert acknowledgement the vessel in distress should

commence the distress traffic on one of the international distress traffic frequencies such

as VHF Channel 16 or frequency 2182 kHz (if not automatically controlled) as follows:

MAYDAY (repeated three times)

THIS IS

- the 9-digit Maritime Mobile Service Identity code (MMSI) plus name / call sign

or other identification of the vessel calling

- the position of the vessel

- the nature of distress

- the assistance required

- any other information which might facilitate rescue.

.2 	Example

MAYDAY MAYDAY MAYDAY

- THIS IS TWO-ONE-ONE-TWO-THREE-NINE-SIX-EIGHT-ZERO

MOTOR VESSEL "BIRTE" CALL SIGN DELTA ALPHA MIKE KILO

- POSITION SIX TWO DEGREES ONE ONE DECIMAL EIGHT

MINUTES NORTH

- ZERO ZERO SEVEN DEGREES FOUR FOUR MINUTES EAST

- I AM ON FIRE AFTER EXPLOSION

- I REQUIRE FIRE FIGHTING ASSISTANCE

- SMOKE NOT TOXIC OVER

A2/3.5 	Anchoring

.1 		Going to anchor

.10.1 	The cable is leading

~ ahead / astern.

~ to port / to starboard.

~ round the bow.

~ up and down.

\(suggested to add clock system to indicate the direction of the anchor leading e.g. the anchor is 2 o’clock)

.11 		How is the cable?

.2 Leaving the anchorage (P.62)

.4.1 		The cable is leading

~ ahead / astern.

~ to port / to starboard.

~ round the bow.

~ up and down.

(suggested to add clock system to indicate the direction of the anchor leading e.g. the anchor is 2 o’clock)

IMO STANDARD MARINE COMMUNICATION PHRASES: PART B (P.66)

.2 		Movements

.6 		The course board is written up.(?)

.7 		The next chart is within .... hours.(?)

.3 		Draft

.2 		Present (maximum) draft is ... metres.

B1/1.2 	Briefing on traffic situation in the area (P.67)

.3.2		The vessel will pass ... kilometres / meters/nautical miles/cables ahead / astern.

.7.2 		Attention. There are small crafts in the vicinity not detected on the radar

B1/1.3 	Briefing on navigational aids and equipment status

.3 		GPS / LORAN /AIS is / is not in operation.

B1/1.4 	Briefing on radiocommunications (P.68)

.6 		The Pilot station / VTS station requires

~ flag State.

~ call sign / identification/IMO Number.

B1/1.5 	Briefing on meteorological conditions

.4.1 		The wind is ... (cardinal points) force (Beaufort) ... .

.8 		Visibility is ... metres/kilometres/cables/nautical miles.

.10 		Visibility is expected

~ to decrease / increase to ... metres/kilometres/cables/ nautical miles (within ... hours).

~ variable between ... and ... metres/kilometres/cables/ nautical miles (within .... hours).

B1/1.9 	Briefing on operation of main engine and auxiliary equipment (P.69)

See also B1/1.8.

.4 		There is no problems.

B1/1.11 	Briefing on special machinery events and repairs (P.70)

.5 		Call the Master / Chief Engineer if the revolution of the main engine(s)

is below ... per minute.

B1/1.12 	Briefing on record keeping

.3		The VDR is on/working

B1/2 	Trim, list and stability

1 		The vessel is on even keel (at present).

.1.1 		The vessel is ... metres  trimmedby the head / stern (at present).

B2/1.2 	Briefing crew and passengers (P.71 & 72))

See also B4 “ Passenger care”.

.2.3 		For safety reasons I request all crew members to go to their (assembly) muster stations.

.3.5 		All crew members to (assembly) muster stations.

.3.6 		Follow the escape routes shown.

.3.7 		The route to the (assembly) muster station is ... not clear.

.3.7.1 	The route to the (assembly) muster station will be via ... .

.3.8 		(Assemble) Muster

~ on deck.

~ on the foredeck / afterdeck.

~ on the ... deck on port side / starboard side.

~ on the ... deck forward of ... / aft of ...

.6 		Check the bilge pumps of the lifeboats and report.

.6.1 		All bilge pumps are operational.

.10.4 	Hoist no. ... lifeboat(s).

B2/1.6 	Roll call (P.74)

Suggest replacing ‘Assembly’ with ‘Muster’ in this section

B2/1.8 	In - boat procedures (P.75)

.3.2 		The number of (severe/minor) injured persons is: ..

B2/5.1 	Reporting grounding and ordering actions (P.83)

B2/6.2 	Person-overboard activities (P.86)

19 March 2012 - continue from the WG1 of VTS 33 (P. 86)

.9.2 	Wind force Beaufort......./........m/s  /....... knot

.9.3 	Visibility good / moderate / poor (......miles)

.14 		Use VHF Channel ... MF/ HF frequency….

B3/6.3	Rescue operation - reporting readiness for assistance (P.87)

.1 		Use VHF Channel ... MF/ HF frequency....) for communication

.2 		Observed the following distress signal in....degrees (Bearing)

.4.3		Transmit a MAYDAY - RELAY to ... (radio station shore based station)

.10.2	Activate on SART

B2/6.4  Conducting search (P.88)

.1.2     Inform radio coast station shore based station (s) / MRCC/ vessels in vicinity.

.1.3		Search and rescue operation started at......UTC

.4.1  	Light signals / smoke signals / sound signals / ... signals in ... degrees (Bearing)

.4.2   	Objects / vessel in distress / lifeboat(s) / life raft(s) / person(s) in water

in ... degrees (Bearing)

B2/6.5  Rescue activities

.3      Inform   ... coast radio station shore based station about the name(s) / call sign(s) and destination of the vessel(s) with the survivors.

.4      Inform   ... coast radio station shore based station about the condition of the vessel in distress:

.4.1    The vessel in distress

~ capsized / sunk / adrift (near position ... ) / drifting in ... degrees.

~ grounded (in position: ...).

~ on fire.

~ not under command.

~ flooding

B2/6.6  Finishing with search and rescue operations (P.89)

.3		Inform the coast radio station shore based station / searching vessels

about the cancellation finished of search and rescue.

B3/1.1   Loading and unloading (P.89)

.4      	How many 20 feet/ 40 feet containers will the vessel load?

.4.1     The vessel will load ... 20 feet / 40 feet containers.

.7.1     The vessel can load ... tonnes / cubic metres / ... 20 feet / 40 feet containers on deck.

B3/1.2  Handling dangerous goods (P.92)

.1		Do you carry dangerous goods on board?

.1.1		Yes, My vessel carry dangerous good on board.

.1 2     What is the IMO Class of these goods?

.4       These goods emit produce flammable gases in contact with water.

.3       Reporting incidents (P.93)

.8.4.2    Call the harbour fire brigade Contact fire department / ... .

.4       Action in case of incidents

.9       Only open the container / hold / locker / ... when smoking is has stopped.

.15.3   	Call Contact the ambulance.

.17      Alter course for to the nearest port (inform on radio).

.2       Operating pumping equipment (P.94)

.1      	What is the (maximum) loading rate / discharge discharging rate ?

.2      	Is the COW crude oil washing system / inert gas system operational ?

.2.1   	Yes, COW the crude oil washing system / inert gas system is operational .

.2.2    	No, the COW the crude oil washing system / inert gas system is not operational (yet).

.2.3    	The COW the crude oil washing system / inert gas will be operational in ... minutes.

.8      	Inform ... minutes before loading / discharge discharging will start / finish.

.8.1		Loading / discharge discharging will start / finish in ... minutes.

B3/2.2  Taking measures for cargo care (P.97)

.3.2   	No,  the holds are not clean (dry and free of smell) (yet).

.8     	Enter Record all checks into the log-book.

.2      Describing damage to the cargo (P.98)

.19     ... container(s) were washed overboard (inform on radio to shore based station).

.4      PA Public Address (PA) announcements on emergency (P.100)