AISM Association Internationale de Signalisation Maritime      IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

IALA Guideline No. ####

On

Use of decision support tools for VTS personnel

Edition 1

[Date issued]

[Previous Edition; Date issued]

10, rue des Gaudines

78100 Saint Germain en Laye, France

Telephone: +33 1 34 51 70 01  Fax:  +33 1 34 51 82 05

e-mail:  contact@iala-aism.org       Internet:  www.iala-aism.org

Document Revisions

Revisions to the IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |

Table of Contents

1	Introduction	4

1.1	Objective	4

2	Acronyms and Definitions	4

3	UTILIZATION of decision Support tools	5

3.1	Introduction	5

3.2	Type of decision support tools	5

3.3	Management requirements of decision support tools	6

3.4	Operational requirements of decision support tools	6

3.5	Training	6

4	EXAMPLE decision support tools	6

5	References	7

Use of decision support tools for VTS personnel

# Introduction

According to IMO Resolution A.857(20)，Vessel Traffic Services are implemented to improve the safety and efficiency of vessel traffic and to protect the environment. The service should have the capability to interact with the traffic and to respond to traffic situations developing in the VTS area.

Decision Support is a way to help VTS personnel make decisions in normal and abnormal situations. It is especially useful for VTS personnel facing decisions about evolving situations or emergency situations.

Decision support tools have been used in VTS centres to enhance situation awareness by assisting VTS personnel. These tools can assist VTS personnel decision making activities at an operational, planning and management levels.

## Objective

The aim of this document is to give guidance on the use of decision support tools for VTS personnel.

# Acronyms and Definitions

To assist in the use of this guideline, the following acronyms and definitions, mainly based on IMO resolutions, have been used:

- Acronyms

| IALA | International Association of Marine Aids to Navigation and Lighthouse Authorities |
| --- | --- |
| IMO | International Maritime Organization |
| VTS | Vessel Traffic Services |
| VTSO | VTS Operator is an appropriately qualified person performing one or more tasks contributing to the services of the VTS. |
| INS | An information service is a service to ensure that essential information becomes available in time for on-board navigational decision-making. |
| TOS | A traffic organization service is a service to prevent the development of dangerous maritime traffic situations and to provide for the safe and efficient movement of vessel traffic within the VTS area. |
| NAS | A navigational assistance service is a service to assist on-board navigational decision-making and to monitor its effects. |

- General Definitions

| Alarm | A high priority alert requiring immediate attention and action [MSC.302 (87)] |
| --- | --- |
| Alert | An announcement of abnormal situations and conditions requiring attentions. [MSC.302 (87)] |
| Caution | Lowest priority of an alert. Awareness of a condition which does not warrant an alarm or warning condition, but still requires attention out of the ordinary consideration of the situation or of given information. [MSC.302 (87)] |
| Warning | Condition requiring immediate attention, but not immediate action. [MSC.302 (87)] |
| Emergency alarm | Highest priority of an alert. Alarms which indicate immediate danger to human life or to the ship and its machinery exits and require immediate action. [MSC.302 (87)] |
| Decision-maker | A person or group authorized to make decisions. |
| Decision Support (DS) | A way to help VTS personnel make decisions. |
| Decision support tool (DST) | A VTS decision support tool assists the decision-maker at an operational, planning and management level. This may be in real-time or at a tactical or strategic level. |
| Active decision support tool | A tool that brings out decision suggestions or solutions to support decision making. |
| Passive decision support tool | A tool that aids the process of decision making, but that cannot bring out explicit decision suggestions or solutions. |
| Cooperative decision support tool | A tool that helps the decision-maker to modify, complete, or refine the decision suggestions provided by the tool, and feed back to the tool. The tool again improves, completes, and refines the suggestions of the decision-maker and feedback for validation. The whole process may start again, until a consolidated solution is generated. |

# UTILIZATION of decision Support tools

## General principles

The use of Decision Support Tools may differ depending on the needs and services of the VTS. In order to assist VTS personnel fulfilling their tasks (INS, TOS and NAS) in a specific context, some decision support tools may require user input such as the vessel(s) concerned or the area supervised. In other cases, some tools are working permanently in a self-contained way and should inform the VTS personnel automatically.

The operational procedures of the VTS should clarify the use of decision support tools according to local environment of the VTS area.

The decision support tools can be implemented during or after the establishment of VTS, and if necessary a formal safety assessment can be referred for the application of those tools.

## Situation awareness

The use of different level of alerts can increase the situation awareness with DST. In accordance with V-125, it is recommended that the terminology used for alerts (alarm, warning and caution) reflects the maritime standards contained in MSC.302(87) Adoption of Performance Standards for Bridge Alert Management unless particular local circumstances require otherwise.

The increasing levels of alerts provided by DST starts with caution then warning, alarm and emergency alarm. (figure 1)

Fig 1 – DST increasing alerts

### Caution

In the normal situation the DST provides caution. Caution is condition of awareness which does not warrant an alarm or warning condition, but still requires attention out of the ordinary consideration of the situation or of given information.

Example, of caution

Speed vector gives basic information which can be set by the VTS personnel.

### Warning

In conditions requiring immediate attention, but not immediate action the DST provides warning. Warnings are presented for precautionary reason to make the VTS personnel aware of change conditions which are not immediate hazardous, but may become hazardous if no action is taken.

Example of warning

Vessel approaching shallow water with sufficient manoeuvring space to avoid shallow water.

### Alarm

In conditions requiring immediate attention and action the DST provides alarm.

Example of alarm

Vessel approaching shallow water with minimum manoeuvring space to avoid shallow water.

### Emergency alarm

Alarm which indicate immediate danger to human life or to the ship or endanger to environment exists and requires immediate action.

Example of Emergency alarm

Vessel is not able to avoid shallow water.

## Type of decision support tools

Depending on the relationship between the decision support tool and the decision-maker, a decision support tool can be classified as passive, active, or cooperative.

Whatever the type of decision support tool, the final decision is always at the discretion of the decision-maker according to the VTS standard operational procedures.

### Passive decision support tool

A passive decision support tool is the most common type available in VTS centres. Such a tool is generally used for generating alerts.

Example, shallow water alert

In passive mode, the decision support tool shall raise a visible and/or audible alert whenever a vessel is heading to shallow water and immediate action is required.

### Active decision support tool

In addition to the functions of a passive decision support tool, an active decision support tool is able to provide solutions to the decision-maker. These solutions may be derived from a list of pre-typed solutions that should comply with the standard operational procedures of VTS.

Example, shallow water alert

In active mode, the shallow water decision support tool shall raise a visible and/or audible alert and provide options/suggestions such as course and speed alterations, etc.

### Cooperative decision support tool

In addition to the functions of an active decision support tool, a cooperative decision support tool may induce more exchanges between human and machine in order to find a solution that best suits the needs of the decision-maker. Those tools are used in more complex and/or seldom situations where decision making is difficult.

Example, shallow water alert

In cooperative mode, the shallow water decision support tool provides options/suggestions to the decision-maker who could request new answers to options suggested by the tool.

Reduce speed could be a solution selected and the cooperative decision support tool could calculate the appropriate speed reduction based on the time of available depth of water in order for the vessel to arrive on the position with sufficient depth.

## Management requirements of decision support tools

Management facilities should be provided for the adjustment of alert thresholds and the possibility of de-activation. The appropriate alert thresholds should be part of the agreed operational procedures to ensure that the deployed system is fit for purpose. Alerts should not be excessive.

Management reports may be generated from alert statistics and/or VTSO actions for analysis.

## Operational requirements of decision support tools

Decision support tools may be based upon a real-time assessment of risks associated with the traffic situation. Decision Support tools should:

- reduce the workload;

- be accurate and in real time;

- be configurable with local operational procedures;

- comply with IALA Recommendation V-125 (Use and presentation of symbology at VTS centres);

- have recording capacity.

When the risk level exceeds a pre-defined threshold, an alert shall be raised and the VTSO may be advised of the recommended risk mitigation options.

## Training

VTS Authority should ensure that VTS personnel is adequately trained in the use of decision support tools.

Emphasis should be given on the inputs and limitations of the decision support tools.

# EXAMPLE decision support tools

The following is a short list of common Decision Support Tools that may assist achievement of operational requirements:

- Collision avoidance;

- Anchor watch;

- Grounding preventions;

- Air draught clearances;

- Area related (limited access areas, area to be avoid, etc.);

- Speed limitations;

- Sailing plan compliances;

- Incident or accident management.

# References

The following primary references have been used in the production of this Guideline:

- IMO Resolution MSC.302 (87);

- IALA VTS Manual 2012;

- IALA Dictionary;

- IALA Recommendation V-125;

- IALA Recommendation V-127;

- IALA Recommendation V-128;

- IALA Guideline 1018.

- IALA Guideline 1070;