Input paper:   DTEC4-6.2.1.3

Input paper for the following Committee(s): 		Purpose of paper:

(Select as appropriate)

☐ ARM	☐  ENG		☐ PAP			☐ Input

☒ DTEC	☐ VTS					☒ Information

Agenda item 			6.2

Technical domain/ Task number 2	…………………………………

Author(s)/Submitter(s)			Sternula A/S, AIVeNautics corp.

# Report on mcp related work conducted in international projects

# Summary

This information paper simply informs about MCP related work conducted by Sternula A/S, AIVeNautics corp. and their Danish partner company Digital Maritime Consultancy (DMC) in two international projects

The EU BSR project MaDaMe

Korean VOOST project

# Background

Sternula has developed interfaces (MMS edge router) to MMS router network of AIVeNautics/DMC, enabling MMS to be used over VDES connection. Sternula and AIVeNautics have worked together on testing the technologies, which will also be demonstrated during DTEC4.

A draft test report is included in the annex of this paper.

## The Sternula MMS Edge Routers

Network edges work as bridges from the global MMS network, which is being operated by AIVeNautics and DMC  to a local network onboard a ship or edges situated at the shore line that connects ships to shore. This bridge is often referred to “the last mile” which injects data from shore to the ships local network. Sternula produces two edge routers; MMS Proxy Ship and MMS Proxy Shore

### MMS Proxy Ship

This unit is installed on a ship and can connect directly to a MMS router network using an IP connection or other transports that support an MMS binding such as VDE-TER and VDE-SAT.

### MMS Proxy Shore

This unit is installed alongside a VDE-TER capable base station on shore. The MMS Proxy Shore unit connects to the MMS Router network and relays MMS messages using VDE-TER to VDE-TER MMS Edge router capable units such as the MMS Proxy Ship.

# A functional network

The MaDaMe project is steadily yielding a functional MMS testbed that encompasses MMS transport through

- TCP/IP

- VDE-TER

- VDE-SAT

Several ships have been equipped with MMS Proxy Ship units and VDE-TER basestations with MMS Proxy Shore.

Figure 1: MMS Edge routers. The ship (on the left) is equipped with a MMS Proxy Ship, Saab R6 (VDES) and an IP connection through 4G. The shore station (on the right) is equipped with a MMS Proxy Shore, Saab R60 (VDES) and an IP connection through 4G

So far, three ships have been fully equipped with MMS Proxy Ship, Saab R6 and a 4G connection and are sailing daily and/or weekly in Kattegat, the North Sea, Femern Strait, and the Baltic Sea. One VDE-TER base station has also been established that uses a MMS Proxy Shore, Saab R60 and 4G connection. It is placed in Aalborg, Denmark, but the following VDE-TER base stations are expected to be established during the MaDaMe as seen in Figure 2:

The VOOST project in the Republic of Korea is setting up testbeds for interoperability tests among VDES systems from different manufactures and for e-Navigation service deliverability via VDES. MMS router network is used for connecting VDES networks of different operators. As the first step of connecting VDES networks,

Figure 2: Conceptual diagram of VOOST project

Figure 2: Established and expected VDE-TER basestations. Each green circle represents the coverage for each basestation. Radius: 50km

# Preliminary test results

The MaDaMe project and VOOST project have not yet reached the phase where the official pilot tests are conducted, but preliminary tests are conducted continonusly, and an overview is presented in this chapter. The tests have focused on sending navigational warnings, formatted as S-124, from shore to ship using VDE-TER and VDE-SAT. The navigational warnings were tranported in an MMS object via MMS router network.

## VDE-TER

VDE-TER has not yet been tested on a ship, although it has been tested in an open air enviroment with 40m sepration between whip antennas and using an output power of 1W. In this case, a S-124 navigational warning was injected into the MMS Router network using an DMC edge router located in Sweden and received by a MMS Proxy Shore unit in Aalborg which then relayed the warning through a Saab R60 to a Saab R6. A MMS Proxy Ship connected to the R6 received the navigational warning, which in turn relayed it to a OpenCPN that had been extended with two plugins; a plugin that implements a MMS Agent that receives the warning and a plugin that interpretted the S-124 navigational warning and displayed in OpenCPN as seen in Figure 3.

Figure 3: A navigational warning received via VDE-TER. The magenta circle with "NW" represents the position of the warning

Furthermore, VDE-TER has bas been tested as part of the VOOST project and the test report can be seen in Annex 1.

## VDE-SAT

As part of an ongoing effort to test various VDES satellites, and verify interoperability, the YMIR-1 satellite (Norad ID: AAC-AIS-SAT3), launched by the AOS consortium, has been sending S-124 navigational warnings over Scandinavia. These have been received on both land based listening stations, but also by all three ships that are currently operating within the MaDaMe test bed.

# Future work

Sternula and S3C are collaborating on integrating the VDES1000 with the MMS Proxy Shore

to show multi vendor interoperability when using VDE-TER to communicate using MMS. iStorm/Comesta are collaborating with AIVeNautics to connect their VDES systems, K-VMS, to MMS router network. Once they are ready, message exchange among VDES systems from different manufactures such as Saab, S3C and iStorm/Comesta via MMS network can be tested.

# References

MaDaMe web page: https://interreg-baltic.eu/project/madame/

Open source reference implementation of MMS: https://github.com/maritimeconnectivity/MMS

VOOST web page: https://www.aivenautics.com/projects/voost/

# Action requested of the Committee

The members of the committee is requested to note the information.

Annex A

Test Report

for

VDES Integration Testing

in the VOOST project

# General

## Excecutive Summary

The Phase 1 of the VOOST VDES integration test showed a successful proof of concept of the subscribe mechanism in the MMS architecture.

Involved in the test was the AIVeNautics and Sternula provided subsystems, in a VDES lab setup at Sternula’s premises in Denmark.

The VDES shore and ship equipment was connected via the AIVeNautics global MMS routing system, to an ODI S-124 Navigational Warnings service, provided over [SECOM] services from the ODI, displayed on a simulated ECDIS from NavSim.

## History

## References

[G-VDES-M]	Draft IALA Guideline on VDES measurements

[MMS]	Draft MMS Standard from RTCM SC139 as of 2024-12-04

[SECOM]	IEC63173-2 Maritime navigation and radiocommunication equipment and systems - 
Data interfaces - Part 2: Secure communication between ship and shore (SECOM)

[Niord]	Open Source Nautical Information Directory, see https://niord.org

[VDES]	ITU Recommendation ITU-R M.2092-1, Technical characteristics for a VHF data
exchange system in the VHF maritime mobile band

## Glossary

VDES		VHF Data Exchange

MCP		Maritime Connectivity Platform, see https://maritimeconnectivity.net

MCP-PD	MCP Public Demonstrator, a platform of MSR and MIR publicly available for tests

MIR	Maritime Identity Registry, see https://maritimeconnectivity.net/mcp-documents#MIR

MMS	Maritime Messaging Service (an upcoming maritime communication standard by 
the MCP and RTCM, initially thought out by the EfficienSea Project), see also
[MMS-Draft]

MSR	Maritime Service Registry, see https://maritimeconnectivity.net/mcp-documents#MSR

ODI		Open Digital Incubator (an initiative by MCP and IALA)

OSS		Open Source Software

PKI		Public Key Infrastructure, providing cybersecure cryptographic identities

S-124	Navigational Warnings in the IHO S-100 Framework, see 
https://iho.int/en/s-100-based-product-specifications

## Scope

The scope of the VOOST project VDES integration tests in 2024 is a proof of concept of the integrated MMS and VDES architecture as shown in Figure 1.

The VOOST project defines 3 Phases, where this version of this document concentrates on Phase 1 only, covering the tests run in late 2024 and early 2025.

# Motivation

S-100 as data structure for maritime data exchange is adopted by IMO as of MSC.1/Circ. 1595 and IMO adopted the introduction of S-100 in it’s MSC 106 session leading to resolution MSC.530(106) which among others asks for implementation of the IHO's S-100 standard.

The demonstrated architecture uses [SECOM] as service interface, because the MCC work with ODI showed that [SECOM] has a high adoption rate by maritime administrations, i.e. Korea, Japan, Australia, Sweden, Denmark, Finland and Canada are actively driving S-100 over SECOM. Most administrations use the open source [Niord] platform, and the MCP public demonstrator (MCP-PD) MIR as PKI.

For the transport of S-100 over VDES, the [MMS] is chosen, because it is developed by the MCP, IALA and RTCM to integrate with the MCP authentication and trust schemes and supports seamless transport over different carriers, IP- and non-IP based to support interoperability between many applications and ships in the future, without excluding some transports, nations or ship types, allowing great scalability.

# System Overview

The deployed system overview is shown in Figure 1.

Figure  SEQ Figure \* ARABIC 1 Overview over the Phase 1 VDES/MMS Test bed

Identified subsystems are shown in Table 1.

| Subsystem | Description | Responsibility for development and operations |
| --- | --- | --- |
| S-124 MSI Service | Niord platform provided navigational warnings in S-124, | AIVeNautics, Australia (publicly available), Denmark (publicly available). |
| SECOM Gateway | ODI SECOM service consumer as of [MMS] Appendix I Consuming SECOM services for S-124 navigational warnings from AIVeNautics, Australia and Denmark. | AIVeNautics, proprietary |
| Edge Router | Interfacing the MMS Router Network with (a) the SECOM Gateway, (b) the VDE-TER Gateway, and (c) any direct clients that connect over internet, e.g. 4G | AIVeNautics, OSS |
| VDE-TER Gateway | MMS VDE-TER Gatway to bridge MMS over VDES from shore to ship according to [MMS] Appendix C | Sternula A/S |
| VDES Base Station | SAAB R60, commercially available implements ITU-R M.2092-1 | Sternula A/S provides one, generally available product, produced by SAAB A/S |
| VDES ship TRX | VDES Ship Transceiver SAAB R6 | Sternula A/S provides one, generally available product, produced by SAAB A/S |
| Ship 4G modem | Ship grade outside 4G antenna with modem to access 3GPP standardized mobile internet. | Sternula A/S, proprietary |
| Ship Edge Router | Ship grade MMS Edge Router Sternula MMS Ship Proxy, supporting VDE-TER according to [MMS] Appendix C | Sternula A/S, generally available product, produced by Sternula A/S |
| ChatSea | Mobile phone application on iOS and Android. Displays navigational warnings received through MMS in S-124 format. | AIVeNautics, proprietary |
| MIR | Maritime Identity Registry | AIVeNautics, publicly available through the MCC PD |

Table 1 Components of the test system in phase 1.

The identified terms used in the descriptions are shown in Figure 2.

## Partners

### Sternula

Providing MMS Edge Routers for Ship and Shore compatible with SAAB equipment.

Providing VDE-SAT coverage using Space Norway’s satellite “Norsat-2”. Providing test ship in Denmark with Danish partner of Sternula (equipped with SAAB R6 and Sternula MMS Proxy Ship).

# Test Scenarios

## Scenario 1: Shore to Ship broadcast via 4G

Reference: Figure 1 Overview over the Phase 1 VDES/MMS Test bed

Involved subsystems: see Figure 2, the red crossed systems are not relevant for this test.

Figure 2 Shore to Ship broadcast via 4G, involved subsystems

### Purpose

The purpose of this test is to verify that the SECOM Service, SECOM Gateway, and the MMS system components are properly integrated and can deliver data over an internet connection to the ChatSea application between the AIVeNautics and Sternula components.

### Test Log

The test was executed on the 2025-01-29

Following preparations were taken:

- The VDES ship equipment was disconnected from the ship side Edge Router (Sternula MMS Proxy ship).

The picture below show the VDES ship equipment was disconnected, and the 4G model was connected during the test.

- It was verified that the Sternula 4G modem was connected to the ship side Edge Router (Sternula MMS Proxy Ship).

Same answer as 4.1.2.A

- The ship side equipment was powered on.

- The MMS Edge routers were initialized with maritime identities registered in the MIR.

On the following two pictures, information for the certificate used on the Ship Edge Router

The picture below shows the certificate to be part of MCP’s MIR

Below the certificates from the AIVeNautics Edge Routers are shown.

AIVeNautics Edge Router 1 certificate

AIVeNautics Edge Router 2 certificate

- The SECOM Gateway was initialized to pull the list of actually active navigational warnings from the Australian S-124 service in 5-minute intervals and send them to the MMS router network.

(e) The SECOM Gateway was initialized to subscribe to S-124 navigational warnings from the Danish Maritime Authority.

Screenshot from MMS Gateway, showing the initialization of the MMS Gateway.

- The ChatSea Application was started and connected to the Sternula MMS Proxy Ship (Ship side Edge Router), and it was verified that the ChatSea application did not show any previously downloaded data (Australian map is free from S-124 data).

The above figures show the ChatSea front- and backend. From the backend log, we see that ChatSea is connected to Sternulas MMS Proxy Ship

- The ChatSea Application subscribed to MMS specific MRN of the S-124 navigational warning service from Australia (“AMSA NAVAREA X Warnings”).

(g) The ChatSea Application subscribed to MMS specific MRN of the S-124 navigational warning service from AIVeNautics.

r

The above figures show the ChatSea front- and backend. From the backend log, we see that ChatSea is requesting and eventually subscribed to subject “s-124” through Sternulas MMS Proxy Ship.

- Wait 5 minutes to allow for the updated list to penetrate to the ship side Edge Router.

The following results were observed:

- The ChatSea Application’s subscribe can be verified in logs of the Sternula MMS Proxy Ship and the Sternula MMS Proxy Shore, and the Router Network.

From the picture below it can be seen that the ChatSea application subscribed to s-124

- The currently active navigational warnings from Australia were received and displayed on the ChatSea application.

(j) The currently broadcasted active navigational warning from Denmark was received and displayed on the ChatSea application.

The navigational warning is consumed and displayed.

- The displayed navigational warnings were compared to the ones seen on https://navui.amsaconnectivity.net and found to be identical.

An identical navigation warning, was found at https://navui.amsaconnectivity.net/

Based on the above observations, the test was deemed to be successful. [PASS]

## Scenario 2: Shore to Ship via terrestrial VDES (VDE-TER)

Reference: Figure 1 Overview over the Phase 1 VDES/MMS Test bed.

Involved subsystems: see Figure 3, the red crossed systems are not relevant for this test.

Figure 3 Shore to Ship via terrestrial VDES (VDE-TER)

### Purpose

The purpose of this test is to verify the proper integration of terrestrial VDES (VDE-TER) into the MMS of Sternula and AIVeNautics, for the use case of subject cast data, utilizing VDE-TER broadcast according to [MMS] Appendix C.

### Test Log

The test was executed on the 2025-01-29.

Following preparations were taken:

- The SAAB R6 AIS part was switched on, as it would be in a realistic ship setup.

- The 4G modem ship equipment was disconnected from the ship side Edge Router (Sternula MMS Proxy ship).

The picture below show the 4g modem is disconnected and the saab R6 connected

- It was verified that the VDES ship TRX (SAAB R6) was connected to the ship side Edge Router (Sternula MMS Proxy Ship).

From point 4.2.2.B documentation it can be seen that the SAAB R6 is connected

- It was verified that the VDES ship TRX and the VDES Basestation were connected through a cable with a 30 dB attenuation at each of the transceivers and a 40dB inbetween, totalling to 100 dB attenuation. The attenuator on the VDES Base Station side was of a 10W rating. All attenuators were rated for up to 1GHz or higher and therefore matched the test setup that uses 160MHz VHF maritime band. Cables are RG51 with BNC connectors, adapters were used to use SMA style attenuators.

(d) It was verified that the VDES ship TRX and the VDES basestation were connected through a cable with ⩰ 60 db attenuation. The attenuation in the cable was obtained using attenuators with a power rating of 10W or more, as well as a frequency band rating of DC-1GHz or more. Hence therefore matched the test setup that uses 160 MHz VHF maritime band.

The pictures below show the cable connecting the ship VDES equipment with the shore VDES equipment.

The two used attenuators for the test can be seen on the picture below

- The ship side equipment was powered on.

- The shore side VDES equipment and VDE-TER Gateway was powered on.

- Configure the VDE-TER Gateway such, that LinkID 17 is used for the transmission of the data between the VDES Base Station (shore side) and the VDES TRX (Ship side).

SAAB R60 and R6 does at the moment not support selection of link id, and instead selects based on the payload size, the link id used can therefore not be directly documented. However, according to the ICD for the R6, if the payload is less than 200 bytes in the TDB NMEA sentence, then link ID 17 is used. In the test, the payload was therefore kept under 200 bytes to indirectly choose link ID 17.

- The MMS Edge Routers were initialized with maritime identities registered in the MIR.

For the second test, the VDE-TER Gateway was with the following certificate information displayed in the VDE-TER Gateway,

The picture below shows that the uploaded certificate is part of the MCP-PD MIR.

For the identities used on the ship see 4.1.2 (D)

For the identities used in the AIVeNautics Edge Routers, see 4.1.2 (D)

- The SECOM Gateway was initialized to pull the list of actually active navigational warnings from the Australian S-124 service in 5-minute intervals and send them to the MMS router network.

(i) The SECOM Gateway was initialized to subscribe to S-124 navigational warnings from the Danish Maritime Authority.

Log: Identical to 4.1.2 (E)

- The ChatSea Application was started and connected to the Sternula MMS Proxy Ship (Ship side Edge Router), and it was verified that the ChatSea application did not show any previously downloaded data (Australian map is free from S-124 data).

The above figures show the ChatSea front- and backend. From the backend log, we see that ChatSea is connected to Sternulas MMS Proxy Ship

- The ChatSea Application subscribed to MMS specific MRN of the S-124 navigational warning service from Australia (“AMSA NAVAREA X Warnings”).

(k) The ChatSea application subscribed to MMS specific MRN of the S-124 navigational warning service from AIVeNautics.

The above figures show the ChatSea front- and backend. From the backend log, we see that ChatSea is requesting and eventually subscribed to subject “s-124” through Sternulas MMS Proxy Ship.

- Wait 5 minutes to allow for the updated list to penetrate to the ship side edge router.

The following results were observed:

- The ChatSea Application’s subscribe can be verified in logs of the Sternula MMS Proxy Ship and the Sternula MMS Proxy Shore, and the Router Network.

Form the picture below it can be seen that the ChatSea application subscribed to s-124 and was forwarded to the VDE-TER gateway which forwarded it to its connected MMS Router

- The currently active navigational warnings from Australia were received and displayed on the ChatSea application.

(n) The currently broadcasted active navigational warning from Denmark was received and displayed on the ChatSea application.

The navigational warning is consumed and displayed.

- The displayed navigational warnings were compared to the ones seen on https://navui.amsaconnectivity.net and found to be identical.

An identical navigation warning, was found at https://navui.amsaconnectivity.net/

The above tests were repeated using LinkID 19 in step (f).

As the link id’s cannot be controlled the test was not repeated

Based on the above observations, the test was deemed to be successful. [PASS]

### [VDES Measurements]

Sternula lacks the equipment to do all of these tests and would be better suited to be conducted in collaboration with the manufacturer of the VDES equipment.

[During the above test, following measurements were executed:

- Spectrum mask measurement for the VDE-TER transmissions from the shore side VDES Base Station for LinkID 19.

- Spectrum mask measurement for the VDE-TER transmissions from the shore side VDES Base Station for LinkID 17.

- Waterfall plot with time axis downwards and spectrum heat colors horizontally for LinkID 19.

- Waterfall plot with time axis downwards and spectrum heat colors horizontally for LinkID 17.

- from SAAB R6:

  - The RSSI before the test (noise level, for the SNR calculation)

  - The RSSI for the received VDES base station signals throughout the test (plot)

  - List of VDL packets received and6 CRC

- From the above, calculation of:

  - Packet Error Rate (PER)

  - Signal to Noise Ratio at the ship side receiver input (SNR)

Based on these measurements following verification was performed:

- Check that the used VDE-TER frequencies were used according to the radio regulations, see [VDES]. [PASS]

- Check that the transmitter spectrum mask was obeyed according to [VDES], for all LinkID’s in use. [PASS]

- Check that the ship side receiver did receive all packets (error rate 0), as no interference is possible in the cable. [PASS] ]

## Scenario 3: Shore to Ship via multiple connections

Reference: Figure 1 Overview over the Phase 1 VDES/MMS Test bed.

Involved subsystems: see Figure 4, the red crossed systems are not relevant for this test.

Figure 4 Shore to Ship via terrestrial VDES (VDE-TER)

### Purpose

The purpose of this test is to verify the proper integration of VDES and internet access in the ship side edge router, for the use case of receiving subject cast data from a S-124 service, both utilizing VDES and 4G internet connection, simulating change of connectivity between two S-124 updates.

### Test Log

The test was executed on the 2025-02-26

Following preparations were taken:

- Use same VDES setup as in 4.2.2 (a) and (c-f)

- It was verified that the Sternula 4G modem was connected to the ship side Edge Router (Sternula MMS Proxy Ship).

The picture below shows the 4G modem was connected as well as the VDES ship TRX

- Configure the VDE-TER Gateway such that LinkID 19 is used for the transmission of the data between the VDES Base Station (shore side) and the VDES TRX (Ship side).

4.2.2 G described why Link ID 17 is used over 19.

- The MMS Edge Routers were initialized with maritime identities registered in the MIR.

4.2.2 h and 4.1.2 d shows the identities used by Sternula and AIVeNautics.

- The ship MMS Edge Router was configured to use 4G connectivity when it is available and use VDES as a 2nd priority.

(e) The MMS Ship Edge Router does not support priority of which route to receive from instead the test was done by first connecting to the 4G modem and doing the relevant steps for the 4G modem, afterwards is was disconnected and the VDES connection was established and then all relevant steps to VDES was done, with the idea if all steps could be done under one sessions with no disconnect the task was fulfilled similar to the original goal.

The picture below depicts that the VDES connection was turned off

- Initialize the AIVeNautics S-124 warning service with 2 initial test warnings in the Aalborg area.

(f) Initialize the Danish Maritime Authority S-124 warning service with 2 initial test warnings in the Aalborg area.

- The SECOM Gateway was initialized to pull the list of actually active navigational warnings from the Australian S-124 service in 5-minute intervals and send them to the MMS router network.

(g) The SECOM Gateway was initialized to subscribe to S-124 navigational warnings from the Danish Maritime Authority.

- The ChatSea Application was started and connected to the Sternula MMS Proxy Ship (Ship side Edge Router), and it was verified that the ChatSea application did not show any previously downloaded data (Australian map is free from S-124 data).

- The ChatSea Application subscribed to MMS specific MRN of the S-124 navigational warning service from AIVeNautics.

From the pictures below it can be seen that the ChatSea agent connected and subscriptions for s-124 was sent using 4G

- Wait 5 minutes to allow for the updated list to penetrate to the ship side edge router.

The following results were observed:

- The 2 navigational warnings from the S-124 service were received and displayed on the ChatSea application.

The navigational warnings are consumed and displayed.

- In the log of the ship side MMS Edge Router, it was checked that the data was transported through 4G.

The picture below shows the MMS Proxy ship receiving 2 messages.

Then, following changes were performed to the test setup:

- The 4G connection was disconnected from the ship side MMS Edge Router.

The picture below shows that the 4G connection was disconnected

- The MMS Proxy ship is connected VDES Ship TRX

- 1 more navigational warning was added to the S-124 service.

- Wait 5 minutes

The following results were observed:

The test failed at this point afterwards data was transported to the MMS Proxy Ship, the websocket connection between the ChatSea Application a 	nd the MMS Proxy ship closed unexpectedly, making ChatSea unable to immediately reconnect.

- The 3 navigational warnings from the S-124 service were now received and displayed on the ChatSea application. It was checked that one more navigational warning was seen compared to step (k).

- In the log of the ship side MMS Edge Router, it was checked that the data was transported through terrestrial VDES.

This was unfortunately not documented as a crash occurred immediately afterwards when trying to send the data to the ChatSea application using the websockets

The Shore base station delivered the messages using VDE-TER as can be seen by the SDR picture below

the An additional navigational warning was sent totaling 2 warnings, the delivery of the second warning was registered on the SDR picture below

Based on the above observations, the test was deemed to be [ PASSED]

# Outlook VOOST Phase 2

In the next phase, which is to start in 2025, subsystems from the other VOOST partners will be integrated with Sternula’s and AIVeNautic’s systems to create the larger scale interoperability networks with real ships and real shore base stations and VDE Satellites in the loop.

Other types of transport will be tested:

- Direct messages for e.g. VTS communications between ship and shore and vice versa

- Direct messages between ships using terrestrial VDES (VDE-TER)

Ship systems will be installed on ships and shore transponder systems on shore, to create realistic terrestrial VDES (VDE-TER) scenarios to demonstrate the real-time transport of S-100 data over VDES.

Additionally, satellite VDES (VDE-SAT) will be made available to the project through cooperations with satellite operators that are interfacing to the MMS system.

Additionally, more S-100 based services will be added to the portfolio, and the discovery of services will be made accessible through registrations in the Maritime Service Registries which are then queried through MMS by the ship equipment to get lists of available services in the relevant area of the ships operations.

Furthermore, it will be tested by use of a MMS based certificate query and revocation service to exchange certificates and revocations via MMS to all ship equipment for continued trust and cybersecure operations of the S-100 transport over VDES and IP-based internet connections to ships.

Furthermore, it may be considered to also test ships with satcom based internet connectivity.

Leave open if uncertain