Input paper: 	     ENG4-10.11

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM	  ENG		□  PAP					□  Input

□  ENAV	□  VTS							  Information

Agenda item 			10

Technical Domain / Task Number 2	…………………………………

Author(s) / Submitter(s)			Plymouth University, UK.

# Monitoring and modelling of rock lighthouses

# Summary

The continued survival of historic rock lighthouses cannot be taken for granted in light of their considerable age and the potential increase in wave loading associated with climate change. With their experience in monitoring more conventional coastal structures, Plymouth University has been working with the UK GLAs to develop methods for monitoring these structures. A pilot study on the Eddystone lighthouse has revealed some of the structural behaviour of the tower and has enabled the development of a structural model using Finite Element software. Using the model, the stability of the Eddystone has been assessed and the critical (albeit theoretical) wave that would cause it to fail identified.

This paper describes the monitoring and modelling techniques from the initial project, the tower behaviour revealed, and the planned next steps.

## Purpose of the document

Presentation of the wave loading on rock lighthouses projects: the initial pilot study commissioned by the UK GLAs and the recently announced EPSRC-funded STORMLAMP project, both led by Plymouth University.

# Background

## Introduction

The General Lighthouse Authorities (GLAs) of the United Kingdom and the Republic of Ireland are committed to retaining physical aids to navigation and need to better understand the behavioural response of the iconic rock lighthouses to wave impacts by storm waves. The wave loading will only become more severe with climate change giving rise to higher sea levels and potentially increases in wave height and storm severity [1].

Plymouth University have a strong track record in field measurements on coastal structures and development of suitable instrumentation for determining wave loading. Working with Trinity House and the other GLAs they devised a pilot instrumentation campaign for the Eddystone lighthouse which has line of sight to the university campus. Field monitoring was supplemented by numerical and physical modelling to provide an insight into both the wave loading and dynamic response.

Archive and contemporary observations of wave loading and structural response are available and through the pilot study, measurements have shed light on the behaviour of the iconic Eddystone lighthouse.

## Field campaign

Conventional monitoring of a coastal structure would require wave measurements within a close proximity and comprehensive spatial coverage of the structure by pressure and aeration transducers. These would require sound fixing and considerable power input. Data would be downloadable on a regular basis. These requirements are beyond what is feasible on a rock lighthouse. Instead the incoming waves were tracked with a set of downwards-pointing DC-powered video cameras that were controlled remotely via a wireless bridge to the university campus. Two of the four cameras were aligned to the prevailing southwesterly direction and the other two were equally spaced around the perimeter to track the motion of the wave front, wrapping itself around the tower. In order to minimise power consumption the cameras were only activated during high wave activity as determined by Channel Coastal Observatory wave buoys along the coast. The acquisition rate was generally 1 fps, but if waves ran over the cylindrical upstand the rate increased to 5 fps for at least 30 seconds. Efforts were made to develop a georeferenced grid but these were hampered by the poor resolution of the original cameras (now replaced, but not yet analysed), clouds of spray during the largest wave runup events and darkness when some of the largest impacts took place.

Figure 1	Schematic diagram showing installed equipment and connectivity

Alongside video information on the wave loading, the structural response was determined using battery-powered geophones. These instruments are more commonly used for monitoring seismic motions. Short-duration velocity time histories were transmitted via a GSM mobile phone network once a particular threshold level was exceeded. The threshold was determined according to the background noise level and the size of the events and could be remotely managed as could the acquisition frequency. Given the limited battery-life it was important that large amounts of data were not acquired for transmission.

Figure 2	Video control box (top left); wireless bridge (top right); camera 4 alongside camera 1 (bottom)

Figure 3	Runup estimation: original high resolution photograph showing individual masonry courses (left); colour-distorted image from video camera permitting identification of courses (middle); and resulting grid construction (right)

The equipment was installed by Plymouth University technicians in the summer of 2013 but no data was forthcoming until December 2013 when a catastrophic sequence of 12 individual storms swept across southern UK over a 2 month period. Data from nearly 3,000 individual impact events were recorded and subsequent analysis provided a number of findings including:

- wave runup on the tower completely obscured the cameras (situated just a little lower than the lantern);

- maximum wave runup was not associated with largest motion responses;

- motion time histories and frequency spectra coupled with video footage identified different types of impacts in the same sea-state;

- maximum displacements were of the order of hundredths of a mm;

- accelerations had maximum values of ~0.1g, equivalent to an earthquake;

- modal responses for the tower are ~4 Hz, ~8 Hz, ~15 Hz; and

- response of the tower was related to wave steepness, tide level and wave orientation.

Figure 3	Typical runup event as seen from SW, ESE, NNW (Column 1, Column 2 and Column 3 respectively) with frames taken 1 second apart

## Structural modelling

In order to better understand the structural response, a Finite Element Model of the Eddystone was constructed using LUSAS (v.15). Archive drawings were used as a basis for the geometry, and material properties were acquired by tracing back through the literature on where the granite was quarried. The model was validated using a series of standard test cases and the geophone data. Agreement of the fundamental frequency of the model was to within 0.3 Hz. The numerical model failed to reproduce a response frequency at 8 Hz perhaps due to the fact that the helideck structure was not modelled or because the lighthouse was modelled as a monolithic structure.

A series of failure modes were investigated: material failure, sliding and overturning. The wave loading was based upon expressions for cylinders in ISO/CD 21650 ‘Actions from wave and currents on coastal structures’; whilst not completely appropriate, particularly as the lighthouse is fully-emergent and not surface-piercing, this was the best existing design guidance. For each of the three failure cases and using the most conservative wave loading validated against the largest recorded displacement, the Eddystone was predicted to have large factors of safety. Then considering the failure scenario with lowest factor of safety (overturning) an estimate of wave height to cause failure was determined: 14 m. Since an unbroken wave of this size would not be possible at this location the survival of the Eddystone under wave loading, in its current condition, is assured.

# Discussion

The ambitious pilot project has brought significant insight to the behaviour of the Eddystone. More comprehensive spatial coverage of motion data was sought (to further validate the FEA model) but the second winter season experienced few storms and the geophones, now powered with alkaline batteries due to CAA requirements, produced no further data. Instead, findings from the pilot project [2] are informing the next phase. Funding for a more comprehensive project to consider the most exposed and interesting UK lighthouses, with further academic expertise, has been secured from the UK Engineering and Physical Sciences Research Council. The project is called STORMLAMP (STructural behaviour Of Rock Mounted Lighthouses At the Mercy of imPulsive waves) and will run from Spring 2016 for 3.5 years. Dr Alison Raby at Plymouth will lead the project, undertaking physical modelling in the university’s COAST laboratory and with Prof Deborah Greaves, developing Computational Fluid Dynamics models to properly define the wave loads. Professor James Brownjohn at Exeter University will lead the structural health monitoring and Professor Dina D’Ayala at UCL will conduct discrete and finite element modelling of the structures. Together the team will consider a number of lighthouses which may include: Longships and Wolf Rock which both experience dramatic overtopping; Needles, which has a peculiar form of foundation and is cylindrical in shape; Bishop Rock which previously suffered from damage to the base and was subsequently redesigned; Hanois which was the first lighthouse to be dove-tailed and Skerryvore which has an upper portion with no dove-tailing. After initial investigations one will be selected for further, comprehensive investigations.

The STORMLAMP objectives are as follows:

1	To identify experimentally mode frequencies, shapes etc. of six vulnerable rock lighthouses to find the structure with the most problematic behaviour.

2	To monitor, and process continuously, structural response of one rock lighthouse over an extended period.

3	To develop structural models, based on detailed construction information and calibrated through the dynamic testing campaigns and hydrodynamic modelling, to determine the response of keyed-in block masonry structures to impact loading.

4	To extend physical and CFD modelling of wave interactions with cylinders to include optimised bathymetries, geometries and kinematics in order to investigate worst-case hydrodynamic loading.

5	To formulate guidance on the technologies and procedures for structural condition assessment and management of rock lighthouses.

The programme of dynamic testing with long-term monitoring and real-time interpretation will be the first ever deployed on these structures. The experience of field measurements and modelling will be distilled to the simplest form to guide assessment strategy for other lighthouses, adopted by the GLAs and disseminated to international lighthouse agencies. An ability to identify structural distress and reduce risk of failure through preventative measures has incalculable value for these irreplaceable but vital structures.

# References

Planning Portal (2010) PPS25: Planning Policy Statement 25: Development and Flood Risk. See http://www.planningportal.gov.uk/planning/planningpolicyandlegislation/.

Raby, A., Bullock, G.N., Banfi, D., Rafiq, Y., Cali, F. (2016) Wave loading on rock lighthouses. Proceedings of the Institution of Civil Engineers – Maritime Engineering Vol 169 (1) pp 15-28.

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain