Input paper:   ENG4-10.26

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM	X ENG		□  PAP					□  Input

□  ENAV	□  VTS							X Information

Agenda item 			10

Technical Domain / Task Number 2	TD#2  5.5

Author(s) / Submitter(s)			Dr. Jong-Uk Kim /Korea Association of Aids to Navigation

# Developments in AtoN Simulation Systems

# - Monitoring development of simulation techniques for AtoN simulation in Korea(3) -

The AtoN Simulator provides simulation circumstances, which include the topographical and environmental characteristics of a primary harbour and the characteristics of a navigating ship and the maritime traffic.  The AtoN planning expert can design a safer and more efficient distribution of AtoN using the simulator.

The AtoN Simulator is based on a ship handling simulator and has been developed by KAAN (the Korea Association of Aids to Navigation) and KRISO (the Korea Research Institute of Ships & Ocean Engineering) funded by Ministry of Oceans and Fisheries (MOF) in Korea.  The AtoN Simulator is a management software package (SW) which can place an AtoN and assign it properties such as the shape, colour, light and function, etc.  An AtoN simulator was installed at the Buoy Management Office (Korea Government) in Yeosu.

Full mission bridge of AtoN simulator

Operating and control equipment of AtoN simulator

# Design and Establish of AtoN Database

## Definition of AtoN English abbreviations

In order to use the database, AtoN terms and abbreviations were defined. Table 1 shows some of the abbreviations used in the 88 AtoN properties database.

AtoN Terminology Abbreviated

| Term | Abbreviations |
| --- | --- |
| AtoN ID | AtoNID |
| AtoN Name | AtoNName |
| Class | Class |
| Manned Lighthouse | MannedLH |
| Unmanned Lighthouse | UnmannedLH |
| Beacon | Beacon |
| Leading Lights | LeadingLight |
| Projector Light | ProjectorLight |
| Direction Light | DirectionLight |
| Light Staff | LightStaff |
| Light Buoy | LightBuoy |
| Bridge Light | BridgeLight |
| Traffic Signal Lights | Traffic Signal Lights |
| Unlighted Beacon | UnlightedBeacon |
| Buoy | Buoy |
| Bridge Daymark | Bridge Daymark |

## Definition of Data Type

Prior to the design database, the data types of the AtoN were defined. Definition of the data type is necessary for the effective storage and management of the data. Table 2 shows the items and data type required for the configuration data of the manned lighthouse. A Text type (5) is composed of AtoN ID, AtoN Name, Shape, Comment, and Period. Port ID, Managed office ID, Latitude, Longitude, Direction, Light Rhythm Chart ID, Light height, Nominal Range, Geographical Range, Height, Vertical Angle, Horizontal Angle, VisibilityArc Start, VisibilityArc End, FogSignal ID, Racon ID, Weather Signal ID, Projector Light ID, AtoN AIS ID, 2D ID, and 3D shape model ID are defined as an Int type (20). An Enumeration type (17) includes Class, Type, Status, Office Name, Harbor Name, Light Color, Surface Color, Function, Lantern Type, Flashing Type, Fog Signal, Racon, Weather Signal, AtoN AIS, Projector Light, Monitoring & Control system and the VTS Area.

Definition of Data Type

| DATA Type | Column Name | DATA Type | Column Name |
| --- | --- | --- | --- |
| Text Type | AtoNID | Int Type | Projector Light ID |
|  | AtoNName |  | AtoN AIS ID |
|  | Shape |  | 2D ID |
|  | Comment |  | 3D shape model ID |
|  | Period | Enumeration Type | Class |
| Int Type | Port ID |  | Type |
|  | Managed office ID |  | Status |
|  | Latitude |  | Office Name |
|  | Direction |  | Harbor Name |
|  | LightRhythmChart ID |  | Light Color |
|  | Light Height |  | Surface Color |
|  | Nominal Range |  | Function |
|  | Geographical Range |  | Lantern Type |
|  | Height |  | Flashing Type |
|  | Vertical Angle |  | FogSignal |
|  | Horizontal Angle |  | Racon |
|  | VisibilityArc Start |  | Weather Signal |
|  | VisibilityArc End |  | AtoN AIS |
|  | FogSignal ID |  | Projector Light |
|  | Racon ID |  | Monitoring&Control system |
|  | WeatherSignal ID |  | VTS Area |

## Design of AtoN Properties Database Structure

In order to use 19 kinds of AtoN for the AtoN Simulator, the AtoN properties for the manned lighthouse are described. AtoN ID was classified in accordance with the AtoN status information. The existing facilities are denoted by the EF. New facilities are denoted by the IP. Position fills in degree, minute, and second. In case of Rhythm Characteristics, the light color should be chosen by the user afterwards, Rhythm Characteristics are largely classified for 10 species. 20 kinds of detailed Rhythm Characteristics could be chosen. It applies to the way of entering the flashing cycle of the selected rhythm characteristics. Light and height refer to the average height of the base surface.

Nominal Range refers to the light of the geography of the light house, using ships, function, and light brightness. Surface color and shape fill in the contents of the lighthouse list. A function could record functional contents of the lighthouse list. The name of lantern refers to the currently used name. Vertical and horizontal divergence angles of the light refer to the current value.

Flashing type refers to using flashing type that is divided into rotate, flashing, etc. Visibility fills in the visibility contents of the lighthouse list. Fog signal, Radar beacon, Marine weather station, AtoN AIS, Projector Light, Monitoring and Control system could register the contents of the lighthouse list. VTS management authority refers to the name of the VTS main body. Port and fairway refer to the name of the port and the fairway management authority. 2D and 3D numbers refer to the serial number. The AtoN condition is divided into virtual, expansion, relocation, revocation, etc. Figure 3 represents the attribute table structure of the manned lighthouse. The value of AtoN ID has a primary key and a foreign key. In addition, it refers to the attribute table such as Fog signal, Radar beacon, Marine weather station, AtoN AIS, and Projector light.

Attribute table structure of Manned Lighthouse

## Establishment of AtoN Properties Database

For the inspection and efficient conversion of the collected data, the data were classified with Excel. The collected data were converted to a database record for the AtoN Manager. Figure 4 shows an excerpt of the Excel data of the manned lighthouse.

Excel data sheet of manned lighthouse

The AtoN properties data for each port were converted to the established database. The AtoNInfo table converted to the database is shown in Figure 5.

Established to AtoN properties database

## Application of Database to the AtoN manager

## The database can be created and edited to AtoN through the AtoN properties database management software. The software that manages the AtoN properties database is called the AtoN Manager. AtoN Manager was used for management and deployment capabilities for AtoN and quantifying function of the AtoN layout. The AtoN Simulator was applied to the AtoN Manager for managing AtoN.

Future Tasks

In future tasks, the Ministry of Ocean and Fisheries(MOF) of Korea/Korea Association of Aids to Navigation(Jong-Uk Kim) will provide information about simulation techniques(4) for AtoN simulation. Also we will propose new guideline on the operation techniques for AtoN simulation.

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain