IALA Guideline

1234 [Guideline No.]

working paper on LEADING LINES

Edition x.x

Document date

Revisions to this IALA document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |
|  | all | Collect all input at ENG4 |

1.	ACRONYMS	6

2.	Collection of input papers	7

3.	ENG4-9.12 Revision of IALA documents on leading lines	8

1.1	Summary	8

3.1.	Purpose of the document	8

3.2.	Related documents	8

1.2	Proposals	8

3.3.	References	8

3.4.	Recommendation and/or guideline	9

3.5.	Using SI units	9

3.6.	Spreadsheet program	9

3.7.	Flow chart	9

3.8.	Meteorological Visibility	9

1.3	Formulae	10

1.4	Figures	14

1.5	Action requested of the Committee	14

4.	ENG4-9.14 Aspects of leading lines from Russian Guidelines	15

1.6	Summary	15

4.1.	Purpose of the document	15

4.2.	Related documents	15

1.7	Background	15

1.8	Discussion	15

4.3.	Explanation on the concept of leading line in Russian guidelines	15

4.4.	Two more leading line concepts in Russian guidelines	15

1.9	References	16

1.10	Action requested of the Committee	16

5.	ENG4-9.15 Aspects of lEading lines from Finnish Guidelines	17

5.1.	Summary	17

5.2.	Purpose of the document	17

5.3.	Related documents	17

5.4.	Background	17

5.5.	Discussion	17

5.6.	Concept of channel width vs distance to most the dangerous obstruction from the Finnish guidelines	17

5.7.	Use of retroreflecting film on leading marks in Finnish guidelines	17

5.8.	Using semi-transparent material for enhancing conspicuity/contrast of daymarks observed against the sun	18

5.9.	References	18

5.10.	Action requested of the Committee	18

6.	ENG4-9.16 Comments on the Present IALA Guidelines on Design of Leading Lines	19

6.1.	Summary	19

6.2.	Purpose of the document	19

6.3.	Related documents	19

6.4.	Background	19

6.5.	Discussion	19

6.6.	Comments and additions to consider in the existing IALA Guideline by section numbers	19

6.7.	References	22

6.8.	Action requested of the Committee	22

7.	ENG4-9.17 FTA's leading line calculator software	23

7.1.	Summary	23

7.2.	Purpose of the document	23

7.3.	Related documents	23

7.4.	Discussion	23

7.5.	Action requested of the Committee	24

8.	Need for harmonizing requirements to leading line dayboard dimensions in IALA documentation (Guideline 1023)	25

8.1.	Summary	25

8.2.	Purpose of the document	25

8.3.	Background	25

8.4.	Discussion	25

8.5.	Action requested of the Committee	26

9.	Understanding IALA Leading Line Calculations (unfinished document)	27

List of Tables

Table 1	Example of a table with the significant information in the first column	6

Table 2	Example of a table with the significant information in the first row	6

Table 3	Example of a table with coloured rows	6

Table 4	Example table	9

List of Figures

Figure 1	Example figure	7

Figure 2	Another example figure	7

List of Equations

Equation 1	Geographical range	5

Equation 2	Theory of Special Relativity	5

# ACRONYMS

Body text (To assist in the use of this Guideline, the following acronyms have been used:)

| AAAA | Table text |
| --- | --- |
| BBBB | Table text |
| CCCC | Table text |
| DDDD | Table text |
| EEEE | Table text |
| FFFF | Table text |

# Collection of input papers

At ENG4 several inputs to the guideline and recommendation on leading lines and lights were presented. It was found out that further work should be done intersessional. To keep all the input the working paper contains all papers in a single document.

Papers included

ENG4-9.12	Revision of IALA documents on leading lines (Frank Hermann)

ENG4-9.14	Aspects on leading lines from Russian Guideline (Pärtel Keskküla)

ENG4-9.14	Aspects on leading lines from Finnish Guideline (Pärtel Keskküla)

ENG4-9.16	Comments on present IALA Guidelines on the Design of Leading Lines (Pärtel Keskküla)

ENG4-9.17.1/2	FTA Leading line calculator software

ENG1-9.4.6	Need for harmonizing requirements to leading lines dayboard dimension (Aivar Usk)

Understanding IALA Leading Line Calculations (presented in WG1 at ENG4, Frank Hermann)

# ENG4-9.12 Revision of IALA documents on leading lines

### Summary

IALA Recommendation E-112 on Leading Lights and Guideline 1023 on the Design of Leading Lines are going to be reviewed.

The input paper contains:

- proposals for the review of the document,

- corrected formulae (for copy-paste into new documents)

- new figure for geometric definitions (for copy-paste into new documents)

### Purpose of the document

ENG working group 1 may consider the proposals for the revision of the recommendation and the guideline.

### Related documents

IALA Recommendation E-112 on Leading Lights

IALA Guideline 1023 on the Design of Leading Lines

### Proposals

### References

Both the recommendation and the guideline are based on investigations / papers ('laboratory experiments') which are not available for IALA members. I guess the most important papers are from US Coast Guard. I propose to identify the relevant documents and make them available on IALA Wiki.

### Recommendation and/or guideline

It should be considered whether IALA will keep both a guideline and a recommendation or merge both into a single document. The recommendation currently has no information about the daymark of a leading line.

### Using SI units

The formulae used in the existing documents are not consistent according to current standards. For example some formulae require the input of values for the distance in metres and nautical miles at two different positions in the same formula. Other formulae have hidden units which cause a lot of problems to people trying to read and understand the IALA documents.

Although these abbreviations have a long tradition in IALA, a presentation of each formula consistent with the International System of Units (SI Système international d'unités) should be added. Some proposals are presented in this input paper.

The same had already been done in E-200-2. The definitions made there should be used in the revised leading line documents.

The abbreviation for nautical miles in the list of lights and navigation charts is 'M'.

### Spreadsheet program

Excel is not the only computer spreadsheet program. An alternative is Calc from Libreoffice / Openoffice. The French administration has already used the existing spreadsheet with Libreoffice. Some countries have already decided to use the open source format in their administration.

The guideline can be made independent of the program actually used.

All information which is relevant for the specific computer program should therefore be moved to IALA Wiki.

IALA could provide both a XLSX (Excel) and an ODS (Open Document Spreadsheet) document for download.

The spreadsheet should now use the notation of E-200-2.

### Flow chart

The calculations for the design of leading lines should be presented in a flow chart in the guideline.

### Meteorological Visibility

The design process requires 3 input values for the meteorological Visibility (Minimum / Design / Maximum). The values for maximum (20 M) and design (10 M) may be fixed.

However there should be some proposals for the minimum visibility. E.G. German administration uses values between 2.2 M (Tm = 0.25) to 4.3 M (Tm = 0.5) for minimum visibility.

Unfortunately the calculated daymark size is strictly based on minimum visibility leading to very huge daymark length (some thousands of metres) when putting in existing German leading lines.

I propose to change the calculation process for the daymark size.

### Formulae

Some of the equations in the existing documents seem to be corrupted or have some small typing errors. Therefore the equations have been checked and rewritten with Microsoft Word 2010 formula editor in order to 'copy and paste' them into the revised document.

When an equation is not consistent with SI standard a consistent alternative (alt) is presented.

Chapter 5

γm=2.4+0.2logE+×10-3 	for E1=E2			(1a)

alt: 	γm=2.4+0.2logE+1lx×10-3

γm=2.6+0.14logE+×10-3 	for logE1E2=1		(1b)

alt:	 γm=2.6+0.14logE+1lx×10-3

γm=3.32+0.08logE+×10-3 	for  logE1E2=2		(1c)

alt:	 γm=3.32+0.08logE+1lx×10-3

new:	E+=maxE1,E2

Chapter 6 a)

ΘQ=maxΘ1,Θ2			(2a)

Θ1=0.05×10-3+0.038γΘ1=0.1×10-3+0.028γ for γ≤5×10-3radian    (2b)for 5×10-3≤γ≤20×10-3radian   (2c)

Θ2=0.07γm				(2d)

Chapter 6 b)

ΘD=maxΘ'1,Θ'2			(3a)

Θ'1=0.16×10-3+0.12γΘ'1=0.31×10-3+0.09γ for γ≤5×10-3radian    (3b)for 5×10-3≤γ≤20×10-3radian   (3c)

Θ'2=0.224γm				(3d)

Chapter 6 c)

ΘM=maxΘ"1,Θ"2			(4a)

Θ"1=0.25×10-3+0.19γΘ"1=0.5×10-3+0.14γ for γ≤5×10-3radian    (4b)for 5×10-3≤γ≤20×10-3radian   (4c)

Θ"2=0.35γm				(4d)

Annex 11.1

(5)	E=I*d-2*0.05dV

(6a)	I1*l-2*0.05lV≥1*10-6

alt 1: 	I1*l-2*0.05lV≥1*10-6lx

alt 2: 	I1*l-2*0.05lV≥Emin  with  Emin=1*10-6lx 		(preferred version)

(6b)	I2*l+R-2*0.05l+RV≥1*10-6

alt 1: 	I2*l+R-2*0.05lV≥1*10-6lx

alt 2: 	I2*l+R-2*0.05lV≥Emin  with Emin=1*10-6lx 		(preferred version)

Remarks on 6a and 6b:

According to 'IALA-Recommendation E-200-2' Emin may have the values

Emin=1*10-6lx or

Emin=2*10-6lx or

Emin=2*10-5lx .

(7a)	I1*x-2*0.05xV

(7b)	I2*x+R-2*0.05x+RV

(8)	I2I1=1+Ra*1+Rl*20R2Vu

(9)	I*d-2*0.05dV≥0.2*10-6

alt 1:	I*d-2*0.05dV≥0.2*10-6lx

alt 2:	I*d-2*0.05dV≥Eacq	with Eacq=0.2*10-6lx 		(preferred version)

(10a)	I1*a-2≤0.1

alt 1:	I1*a-2≤0.1lx

alt 2:	I1*a-2≤Emax		with Emax=0.1lx			(preferred version)

(10b)	I2*a+R-2≤0.1

alt 1:	I2*a+R-2≤0.1lx

alt 2:	I2*a+R-2≤Emax		with Emax=0.1lx			(preferred version)

Remarks on 10a and 10b:

- According to recommendation 4 'prevention of glare' Emax may have the value Emax=0.1lx or Emax=0.01lx.

- In the Excel-chart the maximum intensity I1 and I2 is calculated by different equations.

(-10a) I1*a-2*0.05lVmax≤Emax

(-10b) I2*a+R-2*0.05a+RVmax≤Emax

where Vmax is the maximum visibility, typical 20 M (nautical miles)

Annex 11.2

(11)

γm=2.4-0.06*logE2E1+0.26*logE2E12+logE+*0.2-0.02*logE2E1-0.02*logE2E12*10-3

alt:

γm=2.4-0.06*logE2E1+0.26*logE2E12+logE+1lx*0.2-0.02*logE2E1-0.02*logE2E12*10-3rad

Annex 11.3

(12)

γ=H2-b-c-6.75*10-8*x+R2x+R-H1-b-c-6.75*10-8*x2x

alt 1:	
γ=H2-b-c-6.75*10-8m-1*x+R2x+R-H1-b-c-6.75*10-8m-1*x2x

alt 2:	
γ=H2-b-c-ω*x+R2x+R-H1-b-c-ω*x2x

with 	ω=6.75*10-8m

(13)

γ=H2-b-cx+R-H1-b-cx-6.75*10-8*R

alt 1:

γ=H2-b-cx+R-H1-b-cx-6.75*10-8m-1*R

alt 2:

γ=H2-b-cx+R-H1-b-cx-ω*R

with 	ω=6.75*10-8m

(not numbered)

γ≥γm

(14)

H"-b-cu+S-H'-b-cu-6.75*10-8*S≥0

alt 1:

H"-b-cu+S-H'-b-cu-6.75*10-8m-1*S≥0

alt 2:

H"-b-cu+S-H'-b-cu-ω*S≥0

with 	ω=6.75*10-8m

Annex 11.4

(15)

y=θ*x*1+xR

(16)

yD=θD*x*1+xR

(17)

ym=θm*x*1+xR

Annex 11.5

(18)

3849*H-c+b

alt 1:

3849m*H-cm+bm

alt 2:

Rg=3849m*H-c1m+b1m

Remark on 18:

The equation for the geographical range is presented in several IALA documents. However different factors are used, e.g. the guideline on daymarks uses 3760 m instead of 3849 m.

### Figures

The figures are embedded as windows metafile.

figure 1: elevation - vertical section as windows metafile (requires fonts Times New Roman and Greek C)

figure 2: plan - ground plan as windows metafile (requires fonts Times New Roman and Greek C)

### Action requested of the Committee

ENG working group 1 may consider the proposals for the revision of the recommendation and the guideline.

# ENG4-9.14 Aspects of leading lines from Russian Guidelines

## Summary

In ENG3 members were requested to familiarise themselves with the documents relating to Recommendation E-112 for Leading Lights and Guideline 1023 on the Design of Leading Lines and provide input papers to ENG4 to support the update.

### Purpose of the document

The input is presented for information and considering including relevant parts in the IALA Guideline.

### Related documents

Guideline 1023 for the Design of Leading Lines

## Background

The paper includes parts or aspects from Russian guidelines on leading lines that are not covered in the IALA guideline.

## Discussion

### Explanation on the concept of leading line in Russian guidelines

„Sailing along a leading line takes place in the zone where the mariner considers himself being on the centre line of the leading line and only at the edges of that zone perceives deviation of the lights or the centre lines of the marks.

Curves that envelope the zone of leading line are called visor curves. Both curves consist of geometric locations of points where one can perceive the deviation of the marks or lights from one vertical line. Distance from the centre line to the curve is called off-axis distance PK.“

Considering including the information on visor curves to the IALA Guideline to illustrate the aspect of curved shape of the line connecting points at the off-axis distance from a leading line is proposed.

- Elements of a leading line Инструктсия по навигационному оборудованию. Санкт-Петербург. ГУНиО МО РФ. 2001

### Two more leading line concepts in Russian guidelines

„Slot leading line and sight (an arm´s aiming device – translator’s comment) leading line are combination of three fixed navigation marks that are located at the vertices of an equilateral triangle. The axis/centre line of a sight leading line coincides with the extension of the height of the triangle. When using a sight leading line a mariner has to keep the rear mark exactly in the middle of the two front marks. When using a slot leading line a mariner has to keep the rear mark between the two front marks.

Slot and sight leading lines don´t have any substantial advantages compared to linear leading lines. Slot and sight leading lines did not find wide appreciation therefore only their main principles are described. Slot leading lines are more widely used on inland waterways“

With slot leading line the visor curves of “two adjacent leading lines with one common rear mark” bend towards each other and becoming nearly parallel at a distance suit for marking sides of a channel.

- Working principle of a slot leading line. а – the vessel is on the axis of the leading line in the working zone of the leading line, б – the vessel has deviated from the axis of the leading line, в – the vessel is outside the working zone of the leading line. Навигационное оборудование внутренных водных путей. Москва. “Транспорт”. 1984

- Considering relevance of slot leading lines in the context of IALA guidelines on leading lines is proposed.

## References

- Инструксия по навигационному оборудованию (ИНО-2000). Санкт-Петербург: Министерство Обороны Российской Федерации Главное Управление Навигаций и Океанографий, 2001 (There might be a newer one than 2001 but I don´t have one.)

- Навигационное оборудование внутренных водных путей. Москва. “Транспорт”. 1984

## Action requested of the Committee

The Committee is requested to:

- Review the input paper during the discussion of Guideline 1023 for the Design of Leading Lines

# ENG4-9.15 Aspects of lEading lines from Finnish Guidelines

# Summary

In ENG3 members were requested to familiarise themselves with the documents relating to Recommendation E-112 for Leading Lights and Guideline 1023 on the Design of Leading Lines and provide input papers to ENG4 to support the update.

# Purpose of the document

The input is presented for information and considering including relevant parts in the IALA Guideline.

# Related documents

Guideline 1023 for the Design of Leading Lines

# Background

The paper includes parts or aspects from Finnish guidelines on leading lines that are not covered in the IALA guideline.

# Discussion

### Concept of channel width vs distance to most the dangerous obstruction from the Finnish guidelines

Finnish guidelines use the concept of most dangerous obstruction instead of the width of the channel. Most dangerous is considered to be the obstruction at which the angle between axis of a leading line and the line drawn to the obstruction as seen from the front leading mark is smallest.

At first sight it may seem more complicated but in case of no clearly defined channel but isolated shallows the Finnish one would be more accurate approach. Besides it also covers the case of a channel where the most dangerous would be the far end of the channel.

### Use of retroreflecting film on leading marks in Finnish guidelines

“Reflectors are mostly used as alternatives to lights on small craft routes and other less important roures. However, they have importance on lighted fairways as well in case of failure of a light.

At first, the light source is selected for observing the reflectors. On small crafts 10 000 cd and on ships ca 100 000 cd search lights can be expected.

Then the minimum size of the reflector can be calculated by the formula:

A = D4E/ IR0,052D/18520							(2.4.1)

Modified for the treshold of illuminance:

E = AIR0,052D/18520/ D4							(2.4.2)

A – area of the reflector  [m2];

E – treshold of illuminance [lx];

D – observing distance [m];

I – intensity of the light source [cd];

R – reflecting coefficient of the reflector [lx/m2/cd].	(At 3M web page it is cd/lux/m2)

In principle the calculations can be made as for lights but, as the main purpose is to get strong enough reflection, maximum values need not to be calculated. Retroreflecting film is placed on the middle stripe of the leading mark so that it covers ¾ of that stripe, and a quarter of the area of the whole daymark. Film of the front mark begins from the lower edge of the daymark and the film of the rear mark begins from the upper edge of the daymark (so that that the reflectors will not blur making using the leading line difficult). The reflection is decreasing very fast with the distance and the maximum distance does not exceed 4000 m even with powerful searchlights. In practice the distances are 0 ... 2500 m are used. The colour of the reflector is always white.

When the areas to be covered with the reflectors are determined the value of E is determined for both marks with the formula (2.4.2).”

### Using semi-transparent material for enhancing conspicuity/contrast of daymarks observed against the sun

Finnish recommendations have mentioned that using semi-transparent material on daymarks could enhance conspicuity/contrast of daymarks observed against the sun. Attached is a picture from Finnish inland waters where sun from back of the leading mark in foreground makes its dayboard more conspicuous by making it shine.

- A semi-transparent daymark glowing in sunlight on Finnish inland waterways (left)

### References

- Vesiväylien linjalaskennan perusteet. 2-2013. Liikennevitaston oppaita http://www2.liikennevirasto.fi/julkaisut/pdf3/lop_2013-02_vesivaylien_linjalaskennan_web.pdf

### Action requested of the Committee

The Committee is requested to:

- Review the input paper during the discussion of Guideline 1023 for the Design of Leading Line

# ENG4-9.16 Comments on the Present IALA Guidelines on Design of Leading Lines

### Summary

In ENG3 members were requested to familiarise themselves with the documents relating to Recommendation E-112 for Leading Lights and Guideline 1023 on the Design of Leading Lines and provide input papers to ENG4 to support the update.

### Purpose of the document

The input is presented for information with the aim of amending the IALA Guideline 1023.

### Related documents

Guideline 1023 on the Design of Leading Lines

### Background

The paper includes comments and recommendations on the present content of the IALA Guideline 1023.

### Discussion

### Comments and additions to consider in the existing IALA Guideline by section numbers

Section 2.3

„The preliminary decision in designing a leading line is to specify the segment of water to be defined by the leading line.“

I would say „to be marked with the leading line“. Defining segments of a water body are already done by the nature.

Section 2.5.1 Figure 2-5.

Showing the difference in cross-track factors graphically too would be more illustrative.

Section 2.9 Standard Leading Line Characteristics

Actually there is not much written about standard characters there.

For example Russian guideline says about selecting the characters for leading lights:

- When using flashing lights they should be synchronised if possible.

- If leading lights are not synchronised, the duration of the flashes and the periods of the lights on the front marks must be shorter than on the rear marks.

- Duration of flashes must not be shorter than 0,5 s.

- Duration of flashes on rear marks is to be selected so that maximum possible overlap of the flashes of the leading lights is acquired in both frequency and duration.

- As a rule, the lights of front and rear lights have to be of the same colour.

Section 2.8 Considerations Regarding Intensities.

Uniform recommendations about different additional factors for making leading lights “even better” would be good.

The IALA Guideline 1023 recommends to use 90%-of-the-time-visibility, Frank Hermann mentioned using 95%-of-the-time-visibility in Germany, E-200-2 says that „when selecting a light, this [prevailing visibility conditions] should be taken into account“. Could there be one common recommended value of percentage and some standard values in the absence of more detailed data?

In addition to the worst visibility 10 times more intensity is recommended „for better signal“ in the IALA Guideline 1023. It is not recommended anywhere else. Is it leading line specific? Is it still recommended?

Considering background lighting Finnish guideline recommends factors of 2 and 10, IALA recommends 10 and 100 for multiplying the intensity. Could there be danger of blinding after adding up all these coefficients? And there are also factors for service conditions and aging of the light source.

Section 4.3 Problem codes and fixes – editorial

4. – There is one dayboard among all the daymarks.

Section 4.4 Maximum Intensity less than Minimum Intensity

“Another way to reduce glare is to set the focal heights of the lights so as to be significantly different from the primary HOE for vessels using the channel.”

Could tilting/directing the lantern a bit upwards do the trick as well?

Section 4.5 Maximum Intensity less than Recommended Intensity 	editorial

The sixth word in the third sentence should be then instead of than.

Section 5 Leading Line Configurations & Design Constraints

Are all of them (safety, servicing conditions etc) somehow leading marks specific? There is an IALA recommendation for safety aspects.

Section 5.8 Daymarks

“Use of day/night lights is encouraged on ranges requiring marks in excess of 3 m in length, as these marks are the most hazardous to replace.”

Wooden planks are used for building daymarks in Estonia and there has not been any problems with using daymarks higher than 3 m. In Finnish guideline the heights of daymarks start from 3 meters.

Section 5.9 Enclosure 1 : Leading Line Category Selection Aid. Leading Line Category Selection Aid Notes

“...choosing between a single intensity, 24-hour signal or a dual intensity, day/night signal...”

Maybe a bit off-topic, but how is such single intensity 24-hour signal meant to be presented in List of Lights, for example? As it does not have same nominal range at day, it can´t be officially 24 h light. Does it just have information in Remarks that the light is on at daytime as well but with reduced range?

Section 6.5 Separation of Front and Rear Towers

Could conversion from this

R = (M + C) / [W/ (10 (0.34 * 10-3 )( M + C) – 1]

to this

R = (M + C) / [1000W/ (3.4 * 10-3 )( M + C) – 1]

be incorrect?

Section 6.7 Minimum Intensity – editorial

“lights shall be sized such that the illuminance be nearly equal as possible”

Could „as equal as possible“ be better?

In the first formula Distance from the light to the far end of the channel in meters is a lowercase d, in the second one capital D.

Section 6.9 Equality of Illuminances – editorial

In „T = 0.05 1/V,“, the comma has slipped to the exponent.

Section 6.10 Separation of Lights

“Refer to Recommendation Equation 18, (geographical range).

3849 * (SQRT(H – c) + SQRT (b))”

Shouldn´t there be two sides in an equation?

Page 35

At the bottom of the page 35 some addition-clarification seems to me necessary – “The entire rear dayboard (if used) must be visible above the front dayboard at the far end of the useful segment and at least one-half of it visible above the front dayboard at the near end of the segment.”

Same for the next page: “Likewise, the entire rear dayboard (if used) must be visible above the front dayboard when viewed at the far end...”

Finnish guideline has recommendations for using retroreflecting film on leading marks. See my input paper with aspects from Finnish guidelines.

Section 6.11 Sensitivity (Cross Track Factor)

“CD = Position in channel segment being evaluated in meters”

„Distance of the observer from the front light in meters“ would seem more clear to me.

Section 6.12 Dayboards

Finnish guideline has simple formulas for sizes of daymarks:

“Dimensions of dayboards are calculated by the following formula:

k = d × 0,00052 + v1

l = d × 0,0004 + v2

where

k = height of dayboard (m)

l = width of dayboard (m)

d = observing distance (m)

v1, v2 = constants

Observing distances:

Front mark: d = far end of useful segment

Rear mark: d = far end of useful segment + distance between marks

Constants:

Merchant fairways at sea: v1 = 1,9 ja v2 = 1,4

Inland and small vessel routes: v1 = 1,3 ja v2 = 0,9

On merchant fairways at sea minimum dimensions of the dayboards are k×l = 4,5×3,0 m2 and inland and small vessel routes 1,5×1,0 m2. Rear dayboard can, in special cases, be remarkably higher. Dayboard’s maximum size is 100 m2.“

Russian guidelines calculate the width of the daymarks according to the distance to the far end with the formulas 2b1 = 0.58D (front dayboard) and 2b2 = 0.58(D+d) (rear dayboard) and „in most cases“ multipliy it with 3...5 to get the height of the marks.

Finnish guidelines result in biggest daymarks among the guidelines under discussions here.

Would reference to IALA Guideline on Daymarks for AtoN be relevant in the Guildeine on Leading Lines?

On daymarks see also my input paper with aspects from Finnish guidelines.

### References

- Инструксия по навигационному оборудованию (ИНО-2000). Санкт-Петербург: Министерство Обороны Российской Федерации Главное Управление Навигаций и Океанографий, 2001 (There might be a newer one than 2001 but I don´t have one.)

- Vesiväylien linjalaskennan perusteet. 2-2013. Liikennevitaston oppaita http://www2.liikennevirasto.fi/julkaisut/pdf3/lop_2013-02_vesivaylien_linjalaskennan_web.pdf

### Action requested of the Committee

The Committee is requested to:

- Review the input paper during the discussion of Guideline 1023 for the Design of Leading Lines

# ENG4-9.17 FTA's leading line calculator software

### Summary

IALA Recommendation E-112 on Leading Lights and Guideline 1023 on the Design of Leading Lines are going to be reviewed and members were requested to provide input papers to ENG4 to support the update. 
This paper provides information about the leading line calculator software developed by Finnish Transport Agency. The software is available on the file share server.

### Purpose of the document

The input is presented for information. The software with source code is available if IALA members want to use it or develop it further.

### Related documents

IALA Recommendation E-112 on Leading Lights

IALA Guideline 1023 on the Design of Leading Lines

### Discussion

The FTA's leading line calculator software was completed in 2010. It is Java-based and available only in Finnish. The calculation is based on IALA Guideline 1023 with some additional national characteristics.

Some screenshots of the software are presented in figures 1 and 2.

Presentation of leading line profile

Presentation of leading line masts and dayboard dimensions

### Action requested of the Committee

The Committee is requested to:

- Review the input paper during the discussion of Guideline 1023 for the Design of Leading Lines

# Need for harmonizing requirements to leading line dayboard dimensions in IALA documentation (Guideline 1023)

### Summary

A small discrepancy in the current IALA recommendations regarding dayboard size for leading line daymarks between Guidelines 1023 and the newer 1094 has been noted.

### Purpose of the document

This paper sets out the noted discrepancy in the recommendation on daymarks and proposes a discussion.

### Background

The latest source on leading mark dayboard dimensions is the IALA Guideline 1094 On Daymarks for Aids to Navigation (2012) that recommends the Width/Length ratio of 1/2: W=L/2 (pages 34 and 35). The white vertical stripe inbetween red side area elements must be dimensioned as W/4. Paragraph 8.2.4 refers to Guideline 1023 in a way that is not completely unambiguous:

### Discussion

"The daymarks of leading lines are described in IALA Guideline 1023 [5]. As leading lines are used in a small horizontal sector only, the daymark should be flat. The Guideline proposes a dayboard with white and red vertical stripes (W = L/2)."

The older Guideline 1023 on Design of Leading lines (2005) does refer to historic US practices of dayboards with 1/2 ratio (Table E6-1) while noting "...This table expresses dayboard size in terms of height in meters and range in kilometers based on the presumption that the dayboard height will be 1.5 times the width, a common international practice. ... U.S. dayboards are constructed with a 2:1 ratio of height to width (largely due to the North American standard plywood size of 4 feet by 8 feet), ..." (page 38). The calculation spreadsheet that comes with it and is still in daily use calculates the dayboard width as "length / 1.5".

IALA Recommendation E-112 On Leading Lights (2005) says nothing about dayboards.

Considering that "the vertical size of the object is seen as the limiting factor for the range of the daymark" (1094, 5.5), it is obviously reasonable to keep the horizontal dimensions of a dayboard as

small as possible to minimize the wind load, thus 1/2 should be preferred to 2/3?

There are many different daymark solutions used on leading lines that are not covered by IALA documents. These could be collected from IALA members and give guidance for these as well. The following are some pictures of daymarks used for leading lights in Germany and Estonia.

Leading light daymark pictures from Germany kindly provided by Mr Frank Hermann

Leading light daymark pictures from Estonia kindly provided by Mr Aivar Usk

### Action requested of the Committee

The Committee is requested to consider updating either Guideline 1023 and the calculation spreadsheet, or the Guideline 1094 to harmonize the recommendations on leading light daymark dimensions where contradictions currently exist.

# Understanding IALA Leading Line Calculations (unfinished document)

Design Steps

STEP 1	Preliminary Tower Position

Input

- Leading Line Name:	name

- Length of Channel:	C

- Width of Channel:	W

Output

Distance near end to front tower:	a=M=C5=0.2*C			 (equation 1)

Distance between the towers:	R=a+C1000*W3.4*a+C-1=l1000*W3.4*l-1  	(equation 2)

Figure 1: Geometry of channel / segment

Explanation for Step 1:

Equation 1: See 6.4 of the guideline

Equation 2: R for first pass

Figure 2: Angles and Distances

From Figure 2:	sinΘ1=yx	and	sinΘ2=yR+x

In general y≪x<x+R so the angles are very small. Therefore the angles can be calculated in radian by an approximation of the sine-function: sin∝≈α in radian.

Approximation:		Θ1≈yx	and	Θ2≈yR+x

The difference between these angles is the horizontal bearing difference:

Θ=Θ1-Θ2≈yx-yR+x=y*R+x-y*xx*R+x=y*R+y*x-y*xx*R+x=y*Rx*R+x

The relation between the distance from the axis and the horizontal bearing difference is then:

y≈Θ*x*R+xR=Θ*x*1+xR	angle in radian (equation 3)

This equation is used in chapter 11.4 of the recommendation with various distances y.

For the equation for R (first estimation of the distance between the towers) two additional assumptions are made:

- The maximum distance from the axis should be one tenth of the channels width: yD=0.1*W. See 6.5 of the guideline.
As yD2*W is the ‘cross track factor (CTF)’ this means CTF =20%.
The value yD is calculated at the far end of the channel, so: x=l=M+C.

- The (vertical) elevation difference γm is calculated for preliminary calculation with a value of γm=1.5*10-3 radian (see chapter 5 of the recommendation).
This results in a bearing difference of
Θ'1=0.16*10-3radian+0.12*1.5*10-3radian=0.34*10-3radian=0.34*10-3.

When (a) and (b) are inserted into equation 3 the result is:

0.1*W=0.34*10-3*l*1+lR

0.1*W0.34*10-3*l=1+lR

lR=0.1*W0.34*10-3*l-1

R=l0.1*W0.34*10-3*l-1=l1000*W3.4*l-1=M+C1000*W3.4*M+C-1

STEP 2	Intensities (night time)

The intensity is calculated with Allard’s Law and needs the input of a distance, a visibility and a threshold for the illuminance at the eye of the observer.

I=E*d2*0.05-dV	(equation 4)

It is recommended to put in the values with SI units (metres, lux, candela)

Three different calculations for the intensity have to be made:

- Minimum intensity based on a minimum visibility and the far end of the channel

- Maximum intensity based on a maximum visibility and the near end of the channel

- Ratio between the illuminance of front and rear leading lights based on a medium visibility and in the middle of the channel

Input

- Minimum Visibility:	Vmin	(< 10 M)

- Design Visibility:	Vdsg	(= 10 M)

- Maximum Visibility:	Vmax	(= 20 M)

- Required illuminance:	Er	(threshold, 10-6 lx or more for night)

- Maximum illuminance:	Emax	(glare, 0.1 lx or 0.01lx for night)

- Separation between the towers: R

- Distance between the near end of the channel and the front tower: a

- Channel length: C

Output

- Minimum intensity (FL, night):	IF,min=Er*a+C2*0.05-a+CVmin

- Maximum intensity (FL, night):	IF,max=Emax*a2*0.05-aVmax

- Minimum intensity (RL, night):	IR,min=Er*R+a+C2*0.05-R+a+CVmin

- Maximum intensity (RL, night):	IR,max=Emax*R+a2*0.05-R+aVmax

- Intensity ratio (middle of channel):	r1=IRIF=R+a+C22a+C22*Tdsg-RdU

- Intensity ratio (far end of channel):	r2=IRIF=a+C2R+a+C2*Tdsg-RdU

- Recommended ratio:	if r1<2r2  the recommended ratio is r=r1
	If r1≥2r2  the recommended ratio is r=2r2

- Recommended intensity (FL, night):	IF,rec=10*IF,min

- Recommended intensity (RL, night):	IR,rec=r*IF,rec

with dU=1852m

Explanation for Intensity

The ratio between the illuminance of the front and rear light at the eye of the observer depends on the visibility, the distance x and the intensities of the lights.

a) Calculation based on visibilityVdsg

The illuminance values for the design visibility Vdsg at position x are:

- EF=IF*x-2*0.05xVdsg

- ER=IR*R+x-2*0.05R+xVdsg

The ratio is:	EFER=IFIR*R+x2x2*0.05x-R+xVdsg=IFIR*R+x2x2*0.05-RVdsg

The ideal ratio for the illuminance should be equal to one and gives a recommended ratio for the intensities:

- IRIF=R+x2x2*0.05-RVdsg

b) Calculation based on transmissivity Tdsg

- Tdsg=0.05dUVdsg

- dU = 1852 m = 1 M (unit distance 1 nautical mile)

The illuminance values for the design transmissivity Tdsg are:

- EF=IF*x-2*TdsgxdU

- ER=IR*R+x-2*TdsgR+xdU

The ratio is:	EFER=IFIR*R+x2x2*Tdsgx-R+xdU=IFIR*R+x2x2*Tdsg-RdU

The ideal ratio for the illuminance should be equal to one and gives a recommended ratio for the intensities:

- IRIF=R+x2x2*Tdsg-RdU

The ratio IRIF is calculated twice, first for a position in the middle of the channel and secondly for a position at the far end.

Middle of the channel:

The distance from the middle of the channel to

- the front light (FL) is:	x=DF=a+C2

- the rear light (RL) is:	x=DR=R+a+C2

With these the ratio becomes:	r1=IRIF=DR2DF2*TdsgDFdUTdsgDRdU=R+a+C22a+C22*Tdsg-RdU

Far end of the channel:

The distance from the middle of the channel to

- the front light (FL) is:	x=a+C

- the rear light (RL) is: 	x=R+a+C

With these the ratio becomes:	r2=IRIF=R+a+C2a+C2*Tdsg-RdU=R+a+C2a+C2*Tdsga+CdUTdsgR+a+cdU

In the chart the reciprocal value is shown:	r2=IFIR

STEP 3 Required elevation differences (vertical angles to avoid blur)

Input

- Intensity of front light (FL):	IF

- Intensity of rear light (RL):	IR

- Front daymark length:		DLF

Output

- Illuminance from FL:		EF=E1=IF*x-2*TmaxxdU

- Illuminance from RL:		ER=E2=IR*R+x-2*TmaxR+xdU

- Maximum of EF and ER:	E+=maxEF,ER

- Minimum angle:
γm=2.4-0.06*logE2E1+0.26*logE2E12+logE+1lx*0.2-0.02*logE2E1-0.02*logE2E12*10-3rad	(equation 5)

Log is the logarithm for base 10: 	log=log10

Explanation for the vertical angle

The equation for the minimum angle is not explained in the guideline or the recommendation. It is only mentioned that there have been made some laboratory experiments. In addition the recommendation reveals that there were three initial equations which have been put into a single one by interpolation.

However some aspects of equation 5 can be explained. Therefore some features are investigated here.

Example 1

The vertical angle should be conform to the eye resolution which is about 1’ (1 arcminute = 0.000291 rad = 0.291 mrad). For safe navigation the angle should be significantly larger than 1’. Other countries use fixed minimum values for the vertical angle (e.g. Germany: 4’ near end, 5’ far end).

For an equal ratio of EREF=1 (logEREF=0) and at a required illuminance E+=10-6lx the calculated minimum angle is:

γm1=2.4+log10-6lx1lx*0.2*10-3rad=2.4-6*0.2*10-3rad=1.2*10-3rad=4.1'

In this case the minimum angle is very near to other common standards.

Example 2

The typical distance between the observer and the marine light is so large that the geometrical dimensions of the light (viewing angle) is much smaller than the eye resolution.  This means that the light acts as a point source.

However physiological effects produce that a point source may appear in different sizes. The point source appears blurred and the size of the blur depends on the illuminance at the eye of the observer.

Lights with a high illuminance produce a bright and larger blur than those with low illuminance.

The same effect is true for astronomical star observation:  although all stars act as point light sources, they appear with different sizes for the observer (apparent magnitude).

To ensure a sufficient distance between the two lights of a leading line, the vertical angle must be larger, when the absolute illuminance values become larger.

This is done for an equal ratio of EREF=1 (logEREF=0) and at a required illuminance

- E+=2*10-6lx and

- E+=2*10-5lx .

The result is:

- γm2a=2.4+log2*10-6lx1lx*0.2*10-3rad=1.26*10-3rad=4.3'

- γm2b=2.4+log2*10-5lx1lx*0.2*10-3rad=1.46*10-3rad=5.0'

As expected the minimum vertical angle increases as E+increases:

γm1<γm2a<γm2b

Example 3

Normally the lights of a leading light should appear with nearly the same illuminance for front and rear light. If one is free to choose the intensity, it is often easy to ensure that the ratio between the illuminances is inside the interval 0.2≤EREF≤5 for the whole segment and for a minimum visibility of 3 M or more.

The recommendation mentions a ratio EREF=10  (logEREF=1) and even EREF=100 (logEREF=2) and equation 5 supports these ratios.

These ratios may for example occur when a light is used simultaneous as a part of a leading light and an omnidirectional light. Another example is that one part of the leading light has to be driven with solar power (low intensity) and the ratio is not drawn to a value near one, because one light should be strong enough to compete with rival lights.

For large ratios of EREF the brighter light may outshine the darker light. The only way to avoid this, is to increase the angle between these two lights. So equation 5 should give larger values for γm when the ratio EREF becomes large.

This is tested for

- EF=10-5lx and ER=10-6lx		(EFER=10)

- EF=10-4lx and ER=10-6lx		(EFER=100)

Equation 5 gives:

- γm3a=2.4-0.06*1+0.26*12+log10-5lx1lx*0.2-0.02*1-0.02*12*10-3rad=2.6-5*0.16*10-3rad=1,8*10-3rad=6.19'

- γm3b=2.4-0.06*2+0.26*22+log10-4lx1lx*0.2-0.02*2-0.02*22*10-3rad=2.4-0.12+1.04-4*0.2-0.04-0.08*10-3rad=3.32-0.32*10-3rad=3.0*10-3rad=10.31'

As expected the minimum vertical angle increases with the ratio EREF:

γm1<γm3a<γm3b

Remark:

The influence of the ratio EREF is symmetrical for front and rear light. Therefore in equation 5 the ratio is always inserted with the absolute value of the logarithm.

logEREF=-logEREF=logEREF-1=logEFER

STEP 4 Daymark size

The calculation of the daymark size is based on the distance from the tower to the far end of the channel x=a+C (FL) or x=R+a+C (RL) and the minimum visibility Vmin.

The visibility may have an integer value between 1 and 10 nautical miles (M), so there are 10 equations to calculate the size.

Input

- No additional input required

Output

- Daymark length DL (vertical), see table below

- Daymark width is 23 of length (vertical)

| Visibility Vmin [M] | Daymark length (vertical) |
| --- | --- |
| 1 | DL=1 m*exp⁡x-651.8 m806.4 m |
| 2 | DL=1 m*exp⁡x-689.3 m1235.6 m |
| 3 | DL=1 m*exp⁡x-377.8 m2041.5 m |
| 4 | DL=1 m*exp⁡x-242.5 m2689.5 m |
| 5 | DL=1 m*exp⁡x-107.2 m3337.5 m |
| 6 | DL=1 m*exp⁡x+19.9 m3714.7 m |
| 7 | DL=1 m*exp⁡x+159 m4199 m |
| 8 | DL=1 m*exp⁡x+297.5 m4522.8 m |
| 9 | DL=1 m*exp⁡x+326.4 m4896.2 m |
| 10 | DL=1m*exp⁡x+489.2 m5386.1 m |

Explanation for daymark size

The explanation for the equations is referenced to a paper of Blackwell &Duntley and the US Coast Guard Instruction M16500, 4 Range Design (1980).

The daymark length is of an exponential function of the required range. The function itself varies with the visibility (see Figure 3).

Figure 3: Daymark length as a function of range and visibility

The required daymark becomes very large when the visibility is low. As an example the daymark length is calculated for a range of about 10 000 m (= 5.4 M). With a visibility of 10 M the recommended daymark length is 7 m. When the visibility is 4 M the daymark length should be about 37 m and the width about 25 m, which is impractical to install.

On the other hand the luminous intensity of the night light is easy to adjust to a lower visibility. The minimum intensity for a range of 10 000 m and a visibility of 10 M is 504 cd and will be increased to 5700 cd for a visibility of 4 M. Although the increase in intensity is higher than for the size, the resulting intensity is very easy to achieve by common lanterns.

STEP 5 Tower height

The calculation is based on the geographical range, save height above water, the observer height, the distance of the towers to the far end of the channel and the required vertical angles from step 3. If a daymark is used the length of the daymark is added to the height.

Input

- Height of observer b

- Save height above water SAFHW

- Daymark length DL if used

Remark:

The guideline states that the height of the eye is measured from mean low water (MLW). However the ship moves with the tide. At high water the towers are in a lower position to the observer than at low water, whereas the water surface, which blocks the view, is always in the same height to the observer on the ship. So the formulae for tower height are derived at high water (here: mean high water, MHW). The height b of the observer is referenced to the water line of the ship only.

Output

Explanation

Figure 4: Vertical plane

Height of front tower from MLW:	H1=YF+dHF+c

Height of rear tower from MLW:	H2=YR+dHR+c

Angular elevation FL to horizon:		tanγF=YF-bx≈γF 	(γF<0) in Figure 4

Angular elevation RL to horizon:		tanγR=YR-bx+R≈γR

Vertical lowering from earth curvature with α=6.75*10-8m-1:

FL					dHF=α*x2

RL					dHR=α*x+R2

With the equations above the elevation difference between front and rear light becomes

γ=γR-γF≈H2-dHR-c-bx+R-H1-dHF-c-bx

γ=H2-α*x+R2-c-bx+R-H1-α*x2-c-bx

γ=H2-c-bx+R-H1-c-bx-α*x+R+α*x

γ=H2-c-bx+R-H1-c-bx-α*x	with α=6.75*10-8m-1

The height of the front light is estimated first:

There are 3 requirements.

- The light must be inside the geographical range.

- The dayboard should be visible inside the geographical range.

- The lower end of the dayboard should be at a safe height above water.

These three requirements lead to three components for the height of the front light.

The geographical range is:

RG=3849m*dH-c1m+b1m

The height dH-c is the height of a point above mean high water (MHW), see Input, remark. The lowest point of the dayboard should have a height of dH-c above MHW.

Rearranging the

Height of front light (MHW):	HF=a+C3850 m-b1 m2+DLF+SAFHW

- Height of rear light (MHW):	HR=x+R*γ+HF-bx+α*R+b

- Height of front light (MLW):	H1=HF=a+C3850 m-b1 m2+DLF+SAFHW-c

Height of rear light (MLW):	H2=HR=x+R*γ+HF-bx+α*R+b-c