Input paper: 	     ENG4-9.14

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM	X  ENG		□  PAP					X  Input

□  ENAV	□  VTS							□  Information

Agenda item 			9

Technical Domain / Task Number 2	TD#1 - Light and vision physics, Visual Signalling / 34

Author(s) / Submitter(s)			Pärtel Keskküla

# Aspects of leading lines from Russian Guidelines

# Summary

In ENG3 members were requested to familiarise themselves with the documents relating to Recommendation E-112 for Leading Lights and Guideline 1023 on the Design of Leading Lines and provide input papers to ENG4 to support the update.

## Purpose of the document

The input is presented for information and considering including relevant parts in the IALA Guideline.

## Related documents

Guideline 1023 for the Design of Leading Lines

# Background

The paper includes parts or aspects from Russian guidelines on leading lines that are not covered in the IALA guideline.

# Discussion

## Explanation on the concept of leading line in Russian guidelines

„Sailing along a leading line takes place in the zone where the mariner considers himself being on the centre line of the leading line and only at the edges of that zone perceives deviation of the lights or the centre lines of the marks.

Curves that envelope the zone of leading line are called visor curves. Both curves consist of geometric locations of points where one can perceive the deviation of the marks or lights from one vertical line. Distance from the centre line to the curve is called off-axis distance PK.“

Considering including the information on visor curves to the IALA Guideline to illustrate the aspect of curved shape of the line connecting points at the off-axis distance from a leading line is proposed.

Elements of a leading line Инструктсия по навигационному оборудованию. Санкт-Петербург. ГУНиО МО РФ. 2001

## Two more leading line concepts in Russian guidelines

„Slot leading line and sight (an arm´s aiming device – translator’s comment) leading line are combination of three fixed navigation marks that are located at the vertices of an equilateral triangle. The axis/centre line of a sight leading line coincides with the extension of the height of the triangle. When using a sight leading line a mariner has to keep the rear mark exactly in the middle of the two front marks. When using a slot leading line a mariner has to keep the rear mark between the two front marks.

Slot and sight leading lines don´t have any substantial advantages compared to linear leading lines. Slot and sight leading lines did not find wide appreciation therefore only their main principles are described. Slot leading lines are more widely used on inland waterways“

With slot leading line the visor curves of “two adjacent leading lines with one common rear mark” bend towards each other and becoming nearly parallel at a distance suit for marking sides of a channel.

Working principle of a slot leading line. а – the vessel is on the axis of the leading line in the working zone of the leading line, б – the vessel has deviated from the axis of the leading line, в – the vessel is outside the working zone of the leading line. Навигационное оборудование внутренных водных путей. Москва. “Транспорт”. 1984

- Considering relevance of slot leading lines in the context of IALA guidelines on leading lines is proposed.

# References

Инструксия по навигационному оборудованию (ИНО-2000). Санкт-Петербург: Министерство Обороны Российской Федерации Главное Управление Навигаций и Океанографий, 2001 (There might be a newer one than 2001 but I don´t have one.)

Навигационное оборудование внутренных водных путей. Москва. “Транспорт”. 1984

# Action requested of the Committee

The Committee is requested to:

- Review the input paper during the discussion of Guideline 1023 for the Design of Leading Lines

........

Annex Heading 1

Annex heading 2

Annex heading 3

........

- Appendix heading 1

Appendix heading 2

Appendix heading 3

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain