WMO/ETMSS

## Paper for Consideration by WMO/ETMSS

## Weather Overlay Feature Catalogue and Portrayal Development

| Submitted by: | USA (NOAA NWS) |
| --- | --- |
| Executive Summary: | This paper provides a brief status update of the JCOMM/ETMSS Weather Overlay Project. |
| Related Documents: | IHO S-100 |

## Introduction / Background

In May of 2012, the 4th Session of the Joint WMO-IOC Technical Commission for Oceanography and Marine Meteorology (JCOMM) established the need for developing a marine weather overlay for Electronic Chart Display and Information Systems (ECDIS). This ECDIS information overlay, a high priority for JCOMM, is expected to improve marine safety services. It will provide analyses and forecasts for maritime weather and sea state in a new digital medium. The NOAA National Weather Service’s Ocean Prediction Center was designated to manage the Weather Overlay Project on behalf of the JCOMM’s Expert Team on Maritime Safety Services (ETMSS). The Directorate of Hydrography and Navigation (DHN), Brazilian National Hydrographic and Marine Meteorological Service, initiated a dialogue with NOAA to forge a collaborative development effort. Brazil’s “Science without Borders” program awarded a grant to the DHN for a visiting scientist to work on the Weather Overlay portrayal at the Ocean Prediction Center from August to November 2014.

## Analysis/Discussion

NOAA submitted a finalized draft of the Weather Overlay Feature Catalogue for approval to ETMSS in May 2014. This Feature Catalogue had passed three review periods by ETMSS members and included 38 meteorological and oceanographic objects and 84 attributes. These objects and attributes are listed in Appendix A and B, respectively.

ETMSS applied to become an IHO Submitting Organization and for an IHO Registry in September 2014 and was officially granted the S-412 Weather Overlay (WXO) registry. Feature objects and attributes have been loaded into the Feature Catalogue Builder. It is anticipated that numerous changes will be necessary to the Feature Catalogue as the objects and attributes are incorporated into the Weather Overlay Portrayal Catalogue.

DHN’s visiting scientist at the Ocean Prediction Center focused on designing the portrayal and accompanying documents for a set of 13 core maritime weather features most commonly found on international meteorological forecast and analysis charts. The core feature portrayal specifications were developed using Scalar Vector Graphics (SVG) and eXtensible Markup Language (XML). The feature portrayal includes point, symbols, line styles, area coverages, and a specific colour scheme that defines day, dusk and night pallets. These initial features are being reviewed by ETMSS members and other interested international weather agencies in the same manner as the Feature Catalogue: multiple review and comment periods over several months. Remaining features will be developed and reviewed with a phased approach, beginning with the most common elements and ending with the most obscure. Features still under review are displayed in Appendix C.

One of the challenges encountered during the development of the initial feature portrayal was the lack of an available S-100 viewer to test XML line styles and areas against the S-100 schema. Without a display tool, portrayal catalogue development and testing will proceed slowly, with many significant edits to the catalogue likely when test data can be portrayed.

## Conclusions

The S-412 Weather Overlay has seen significant development over the past three years and will continue with forward momentum as the S-100 specification and development tools mature. International collaboration between DHN and NOAA has proven to be mutually beneficial and greatly moved the Weather Overlay project forward. NOAA hopes to continue this relationship with DHN in the future, working toward a product that will be ready for dissemination when S-100-enabled ECDIS are in production and ready for installation on vessels.

## Recommendations

It is requested that IALA:

- Provide advice and support for NOAA/DHN technical issues during S-412 development;

## Justification and Impacts

It is expected that IALA comments will contribute significantly to this project’s development, assuming user requirements in this interdisciplinary project are met. This new product, together with other new products, shall impact industry and users interests transitioning to S-100 standard.

## Action Required of IALA

The IALA is invited to:

a.	Take note of this initiative;

b.	Provide recommendations that may be helpful in developing S-412;

c.	Support JCOMM/ETMSS S-412 activities.

## Appendix A: S-412 Objects under review as of February 2016

| Object Number | Object Name | Acronym | Feature Type |
| --- | --- | --- | --- |
| 1.1 | Air Temperature | AIRTEM | Geo |
| 1.2 | Atmospheric Pressure | AIRPSR | Geo |
| 1.3 | Centre of Anticyclone | CEHIPR | Geo |
| 1.4 | Centre of Depression | CENDEP | Geo |
| 1.5 | Cloud | CLOUDS | Geo |
| 1.6 | Convergent Boundaries | CONVBO | Geo |
| 1.7 | Dew-point Temperature | DPTEMP | Geo |
| 1.8 | Freezing Spray | FZSPRY | Geo |
| 1.9 | Front | FRONTS | Geo |
| 1.10 | Gust | GUSGUS | Geo |
| 1.11 | Ice Edge | ICEDGE | Geo |
| 1.12 | Isoheight | ISOHGT | Geo |
| 1.13 | Limit of Known Icebergs | ICEBRG | Geo |
| 1.14 | Low Water Level | LOWATR | Geo |
| 1.15 | Maximum Air Temperature | MAXTEM | Geo |
| 1.16 | Maximum Dew-point Temperature | MAXDPT | Geo |
| 1.17 | Maximum Pressure Decrease/Minimum Pressure Increase | MAXPDE | Geo |
| 1.18 | Maximum Pressure Increase/Minimum Pressure Decrease | MAXPIN | Geo |
| 1.19 | Maximum Sea Surface Temperature | MAXSST | Geo |
| 1.20 | Metarea | METARE | Meta |
| 1.21 | Minimum Air Temperature | MINTEM | Geo |
| 1.22 | Minimum Dew-point Temperature | MINDPT | Geo |
| 1.23 | Minimum Sea Surface Temperature | MINSST | Geo |
| 1.24 | Observations | OBSERV | Geo |
| 1.25 | Pressure Tendency | PRETEN | Geo |
| 1.26 | Ridge | RIDGES | Geo |
| 1.27 | Sea Surface Temperature | SSTEMP | Geo |
| 1.28 | Significant Wave Height | SIGWAV | Geo |
| 1.29 | Significant Weather | SIGWET | Geo |
| 1.30 | Storm Surge | STOSUR | Geo |
| 1.31 | Surface Visibility | SURVIS | Geo |
| 1.32 | Surface Wind | SUWIND | Geo |
| 1.33 | Swell | SWELLS | Geo |
| 1.34 | Thickness | THKNSS | Geo |
| 1.35 | Tropical Cyclone | TROCYC | Geo |
| 1.36 | Tsunami | TSUNAM | Geo |
| 1.37 | Watch/Warning | WRNING | Geo |
| 1.38 | Wind Wave | WINWAV | Geo |

Appendix B: S-412 Attributes under review as of February 2016

| Attribute Number | Attribute Name | Acronym |
| --- | --- | --- |
| 2.1 | Amount of Pressure Change | AMPRCH |
| 2.2 | Atmospheric Pressure Accuracy | ATPACC |
| 2.3 | Azimuth Degrees of Swell Direction | DEGSWL |
| 2.4 | Azimuth Degrees of Significant Wave Direction | DEGWAV |
| 2.5 | Azimuth Degrees of Surface Wind Direction | DEGWND |
| 2.6 | Azimuth Degrees of Wind Waves Direction | DEGWWA |
| 2.7 | Beaufort Force | BEAUFOR |
| 2.8 | Category of Convergent Boundaries | CATCON |
| 2.9 | Category of Front | CATFRO |
| 2.10 | Category of Swell Wave Height | CATSWH |
| 2.11 | Category of Swell Direction | CATSWD |
| 2.12 | Category of Significant Wave Height | CATSEH |
| 2.13 | Category of Significant Wave Direction | SIWADE |
| 2.14 | Category of Significant Weather | CATSWE |
| 2.15 | Category of Surface Visibility | CATVIS |
| 2.16 | Category of Tropical Cyclone | CATCYC |
| 2.17 | Category of Warning | CATWRN |
| 2.18 | Category of Wind Wave Direction | CATWWD |
| 2.19 | Category of Wind Wave Height | CATWWH |
| 2.20 | Change in Swell Height | CHSWHE |
| 2.21 | Change in Swell Period | CHSWPE |
| 2.22 | Change in Significant Wave Height | CHWAHE |
| 2.23 | Change in Significant Wave Period | CHWAPE |
| 2.24 | Change in Surface Wind Direction | CHAWDI |
| 2.25 | Change in Surface Wind Speed | CHCWDS |
| 2.26 | Change in Wind Wave Height | CHWWHE |
| 2.27 | Characteristic of Pressure Change | CHPRCH |
| 2.28 | Compass Point of Surface Wind Direction | COMDIR |
| 2.29 | Direction of Expected Movement | DREXMO |
| 2.30 | Expected Change in Intensity | EXPINT |
| 2.31 | Front Level | FROLEV |
| 2.32 | Frontal Development | FRODEV |
| 2.33 | Height of Cloud Base | HCLOBA |
| 2.34 | Height of Storm Surge | HEISUR |
| 2.35 | Height Probability | HTPROB |
| 2.36 | Icing Intensity | ICIINT |
| 2.37 | Isallobar Time Interval | ISLOTM |
| 2.38 | Issue Time | ISSTIM |
| 2.39 | Length Units | LUNITS |
| 2.40 | Low Water Level | LOWLVL |
| 2.41 | Lower Isobaric Level | LOWLEV |
| 2.42 | Metarea Number | METNUM |
| 2.43 | Next Update Time | NUPTIM |
| 2.44 | Observation Source | OBSRCE |
| 2.45 | Observation Source Identification | OBSIDS |
| 2.46 | Observation Source Status | OBSTAT |
| 2.47 | Predicted Tsunami Maximum Wave Height | TMWHGT |
| 2.48 | Saffir-Simpson Category | SAFSIM |
| 2.49 | Swell Wave Height | SSWHGT |
| 2.50 | Swell Wave Period | SWLPRD |
| 2.51 | Significant Wave Height | SIWAHE |
| 2.52 | Significant Wave Period | SIWAPE |
| 2.53 | Speed of Expected Movement | SPEXMO |
| 2.54 | Swell Wave Height Change Time Interval | SWHTTI |
| 2.55 | Swell Wave Period Change Time Interval | SWPETI |
| 2.56 | Temperature Accuracy | TMPACC |
| 2.57 | Thickness Height | THKNSS |
| 2.58 | Tidal Datum | LEVREF |
| 2.59 | Total Cloud Cover | TCLOCO |
| 2.60 | Tsunami Wave Arrival Time | ARRTIM |
| 2.61 | Tsunami Wave Period | TSUPER |
| 2.62 | Upper Isobaric Level | UPRLEV |
| 2.63 | Valid Time | VALTIM |
| 2.64 | Value of Atmospheric Pressure | VALPSR |
| 2.65 | Value of Dew-point Temperature | VALTDT |
| 2.66 | Value of Height Contour | VALHGT |
| 2.67 | Value of Sea Surface Temperature | VALSST |
| 2.68 | Value of Surface Wind Gust | VALGST |
| 2.69 | Value of Surface Wind Speed | VAWISP |
| 2.70 | Value of Temperature | VALTMP |
| 2.71 | Velocity Units | VUNITS |
| 2.72 | Visibility Range | VIZRNG |
| 2.73 | Warning End Time | WRNEND |
| 2.74 | Warning Start Time | WSTART |
| 2.75 | Watch/Warning Type | WTCWRN |
| 2.76 | Water Height Units | HUNITS |
| 2.77 | Wave Height Change Time Interval | WAHETI |
| 2.78 | Wave Period Change Time Interval | WASWTI |
| 2.79 | Wind Average Period | WNDAVP |
| 2.80 | Wind Change Time Interval | WNDTIM |
| 2.81 | Wind Wave Height | WIWAHE |
| 2.82 | Wind Wave Height Change Time Interval | WWHETI |
| 2.83 | Wind Wave Period | WIWAPE |
| 2.84 | Wind Wave Period Change Time Interval | WWSWTI |

## Appendix C: S-412 Feature Portrayals under review as of February 2016

| Feature | Acronym | Attribute | Geometry | SVG Symbols Used | Complete Symbol (approximate – no tools available to display actual lines) |
| --- | --- | --- | --- | --- | --- |
| Atmospheric Pressure | AIRPSR | All Attributes | Curve |  |  |
| Atmospheric Pressure | CENHIP | All Attributes | Point |  |  |
| Atmospheric Pressure | CENDEP | All Attributes | Point |  |  |
| Convergent Boundary | CONVBO | Intertropical Convergence Zone | Curve |  |  |
| Convergent Boundary | CONVBO | Squall Line | Curve |  |  |
| Convergent Boundary | CONVBO | Trough Line | Curve |  |  |
| Convergent Boundary | CONVBO | Trough | Curve |  |  |
| Convergent Boundary | CONVBO | Shear Line | Curve |  |  |
| Convergent Boundary | CONVBO | Convergence Line | Curve |  |  |
| Convergent Boundary | CONVBO | Monsoon Trough | Curve |  |  |
| Convergent Boundary | CONVBO | Tropical Wave | Curve |  |  |
| Freezing Spray | FZSPRY | All Attributes | Curve |  |  |
| Freezing Spray | FZSPRY | Light | Point, Curve |  |  |
| Freezing Spray | FZSPRY | Moderate | Point, Curve |  |  |
| Freezing Spray | FZSPRY | Severe | Point, Curve |  |  |
| Freezing Spray | FZSPRY | Very Severe | Point, Curve |  |  |
| Front | FRONTS | Cold Front, Developing | Curve |  |  |
| Front | FRONTS | Cold Front, Dissipating | Curve |  |  |
| Front | FRONTS | Cold Front, Surface | Curve |  |  |
| Front | FRONTS | Cold Front, Above Surface | Curve |  |  |
| Front | FRONTS | Warm Front, Developing | Curve |  |  |
| Front | FRONTS | Warm Front, Dissipating | Curve |  |  |
| Front | FRONTS | Warm Front, Surface | Curve |  |  |
| Front | FRONTS | Warm Front, Above Surface | Curve |  |  |
| Front | FRONTS | Occluded | Curve |  |  |
| Front | FRONTS | Quasi-stationary Front, Surface | Curve |  |  |
| Front | FRONTS | Quasi-stationary Front, Above Surface | Curve |  |  |
| Front | FRONTS | Convergence Line | Curve |  |  |
| Front | FRONTS | Dry Line | Curve |  |  |
| Ice Edge | ICEDGE | All Attributes | Curve |  |  |
| Ridge | RIDGE | All Attributes | Curve |  |  |
| Surface Wind | SUWIND | Value of Surface Wind Speed = 2 | Point |  |  |
| Surface Wind | SUWIND | Value of Surface Wind Speed = 5 | Point |  |  |
| Surface Wind | SUWIND | Value of Surface Wind Speed = 10 | Point |  |  |
| Surface Wind | SUWIND | Value of Surface Wind Speed = 50 | Point |  |  |
| Tropical Cyclone | TROCYC | Tropical Disturbance | Point |  |  |
| Tropical Cyclone | TROCYC | Tropical Depression | Point |  |  |
| Tropical Cyclone | TROCYC | Tropical Storm | Point |  |  |
| Tropical Cyclone | TROCYC | Severe Tropical Storm | Point |  |  |
| Tropical Cyclone | TROCYC | Tropical Cyclone | Point |  |  |
| Tropical Cyclone | TROCYC | Hurricane | Point |  |  |
| Tropical Cyclone | TROCYC | Typhoon | Point |  |  |
| Tropical Cyclone | TROCYC | Super Typhoon | Point |  |  |
| Tropical Cyclone | TROCYC | Post-tropical Cyclone | Point |  |  |
| Tropical Cyclone | TROCYC | Subtropical Cyclone | Point |  |  |
| Tropical Cyclone | TROCYC | Remnant Low | Point |  |  |