Input paper: PAP44-6.1.5.2.1

(ENAV28-5.1.2.2)

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM	□  ENG		□  PAP					R  Input

R  ENAV	□  VTS					     		□  Information

Agenda item 			5.1

Technical Domain / Task Number 2	…………………………………

Author(s) / Submitter(s)   …………………………………

Proposal on amendment of Maritime Services (MS) for MASS

# Summary

## Purpose of the document

As defined by the International Maritime Organization (IMO), an maritime surface autonomous ship (MASS) is:

A ship which can operate independent of human interaction in various degree.

IALA has discussed the rapid development of MASS in the maritime field at its 69th and 70th Council session, and agreed to make full preparations for the introduction of MASS within its scope of work. At present, the ARM, VTS and ENAV Committees have all included MASS in their agenda as follows:

| No. | Committee | Focus | Contents |
| --- | --- | --- | --- |
| 1 | ARM | The impact of MASS on aids to navigation infrastructure | In future, new AtoN services may be needed for MASS (Maritime Autonomous Surface Ships) as well as for traditionally crewed ships. New requirements for availability, redundancy and continuity may be needed. IALA will monitor the development of MASS navigation technology and performance, determine what AtoN services should be provided from shore and develop appropriate guidance. At present it seems likely that an important service that will be required by MASS will be a resilient positioning service. Terrestrial positioning services may be required in some areas to achieve the resilience necessary. So-called "smart buoy" and AIS-specific message (ASM) might also contribute by providing real-time weather and tidal information in digital format direct to MASS. Where VDES or AIS base stations networks are implemented they may provide the necessary secure coastal connectivity for automatic reporting by MASS to shore authorities and VTS. VDES could also provide a satellite tracking capabilities for authorities ashore and VTS. ( IALA, 2019a) |
| 2 | VTS | MASS in VTS area | IALA will prepare for the advent of Maritime Autonomous Surface (MASS) and for their interactions with conventional manned vessels within VTS areas. IALA will cooperate with other international organizations in this preparation work. Initial work in this area will consider the interaction process of autonomous vessels with conventional traffic, the information flow between MASS and shore authorities, and the related information exchange with conventional traffic. IALA envisages that MASS will need services from shore including digital Maritime Safety Information (MSI) perhaps in formats specifically for autonomous vessels. (IALA, 2019a) |
| 3 | ENAV | Digitising service for MASS | Nest generation modular control systems and communications technology will enable wireless monitoring and control function both on and off board. These will include advanced decision support systems to provide a capability to operate ships remotely under semi or fully autonomous control. While the control and navigating of unmanned commercial ships is expected to be by private industry using digital connectivity of its own choice, it could be expected that some digital shore services may be adapted or extended in future to provide Maritime Safety Information (MSI) in an appropriate format for these vessels. At this time, it is not clear what MSI will be needed by autonomous ships, in what sea areas and via what communications. However, IALA will maintain a close monitoring of developments with the intent of providing information and guidance to its members as this field develops. (IALA, 2019a) |

In addition, MASS-related policies were also introduced in some countries and enterprises across the world to support their MASS trials. There are also a number of teams in China engaged in the research and test of MASS in coastal waters and inland waterways respectively. This document, based on the research of our project, puts forward the basic needs of MASS for MS, analyzes the existing Maritime Service Portfolio: Digitising Maritime Services (Edition 1.0), and proposes to amend some of the existing maritime services to accommodate the requirements of autonomous navigation of ships.

## References

[1]MSC.1-Circ.1595 - E-Navigation Strategy Implementation Plan - Update 1

[2]MSC.1-Circ.1604 - Interim Guidelines For Mass Trials

[3]MSC.1-CIRC.1610 - Initial Descriptions Of Maritime ServicesIn The Context Of E-Navigation

[4]IALA Guideline on developments in maritime autonomous surface ships, Edition 1.0

[5]Maritime Service Portfolios: digitising maritime services, Edition 1.0

[6]ENAV26-5.1.1.4 Paper on the Impact of MASS on VTS The Netherlands

# Terms and abbreviations

MASS：Maritime Autonomous Surface Ships

IMO：International Maritime Organization

IALA：International Association of Marine Aids to Navigation and Lighthouse Authorities

MS：Maritime Service

# Background

## Levels of autonomy

To facilitate the progress of the regulatory scoping exercise the IMO identified degrees of autonomy:

Degree one: Ship with automated processes and decision support. Seafarers are on board to operate and control shipboard systems and functions. Some operations may be automated and at times be unsupervised but with seafarers on board ready to take control.

Degree two:Remotely controlled ship with seafarers on board. The ship is controlled and operated from another location. Seafarers are available on board to take control and to operate the shipboard systems and functions.

Degree three: Remotely controlled ship without seafarers on board. The ship is controlled and operated from another location. There are no seafarers on board.

Degree four: Fully autonomous ship. The operating system of the ship is able to make decisions and determine actions by itself.

## IALA Guideline on developments in maritime autonomous surface ships

IALA Guideline on Developments in Maritime Autonomous Surface Ships (Ed. 1.0) puts forward the demands of MASS, which needs navigational services provided by means of physical and virtual aids to navigation. IALA is planning to conduct research on MASS from 15 aspects as follows:

- Risk reduction

- Services in support of safe navigation

- Channels to provide services

- Service requirements of MASS

- Requirements for AtoNs in pilotage waters

- Connection between remote berthing and shore service

- Interaction in VTS areas

- Transfer of routing message

- Awareness of local situation

- Meteorological system and data

- Hydrographic system and data

- Availability of AtoNs

- Vessel traffic flow and density

- Adjustment of traditional AtoN services to support MASS

- Adoption, adjustment and extension of existing technologies

## Research progress of MASS in China

Focusing on the development of MASS technology, China carried out a series of technological researches on perception and awareness of navigation environment, autonomous decision-making, route planning, and remote driving, etc., and has undergone relevant tests on a coastal 300TEU container ship and an inland 64TEU container ship for verification. In order to satisfy the needs of navigation support for MASS, China conducted researches on strategies, methods and key technologies for shore-based navigational support, including communication, cyber security and information services. This work is consistent with IALA draft Guideline on Maritime Autonomous Surface Ships. Moreover, China also launched research on the test method of MASS.

# MASS requirements for shore based information

## Characteristics of MASS requirements for shore based information

(1)Digitization of Interaction: The existing shore based information services are mainly aim at operators on-board, and are displayed by the human interface device, such as text, graphics and so on. With the increasing degree of autonomy, MASS will increasingly rely on the shore based system to provide data and information with machine language, so as to decoding and processing conveniently.

(2) Newly-added remote control center (RCC): MASS will depend on RCC to varying degrees. RCC needs to take over the controls of MASS under specific circumstances and interact with VTS, ship owner and etc.. In the future, a third party may provide standardized remote control and monitor services.

## Applicability of existing MS to MASS

For different degrees of MASS, we analyzed the applicability of the existing 16 types of MS, and the relationship is as follows:

| MS | Degree one | Degree two | Degree three | Degree four |
| --- | --- | --- | --- | --- |
| MS1 VTS Information Service（INS） |  |  |  |  |
| MS2 Navigation Assistance Service (NAS) |  |  |  |  |
| MS3 Traffic Organization Service（TOS） |  |  |  |  |
| MS4 Local Port Services (LPS) |  |  |  |  |
| MS5 Maritime Safety Information Service(MSI) |  |  |  |  |
| MS6 Pilotage Service（IMPA） |  |  |  |  |
| MS7 Tugboat Service |  |  |  |  |
| MS8 Ship Shore Report |  |  |  |  |
| MS9 Telemedical assistance service |  |  |  |  |
| MS10 Maritime Assistance Service(MAS) |  |  |  |  |
| MS11 Chart Service |  |  |  |  |
| MS12 Nautical Publication Service |  |  |  |  |
| MS13 Ice navigation service |  |  |  |  |
| MS14 Weather Information Service |  |  |  |  |
| MS15 Real-time Hydrological and Environmental Information Service |  |  |  |  |
| MS16 Search and rescue service(SAR) |  |  |  |  |

# Proposal on the future work of MS

## Proposal on MS for varying degree of MASS

The existing shore-based services involve a lot of interactions aim at seafarers on board, such as VHF/HF/MF telephone, NAVTEX, which are not applicable to MASS of degree two, three, and four. Therefore, MS for MASS needs digitized and structured information, which can be recognized by machines directly.

In addition, for MASS of Degree two and three, its objects of service include not only the crew members and intelligent systems on board, but also the Remote Control Center (RCC). In this scenario, the public communication network will carry the MS between MS source and RCC instead.

Therefore, the existing implementation methods of MS needs to be supplemented on the basis of fully considering the requirements of each degree of MASS.

## Proposal on adding remote control and monitor service

MASS of Degree one requires the shore to support real-time monitoring and auxiliary decision during normal navigation. MASS of Degree two and three needs continuous remote control service from RCC, and MASS of Degree four requires remote intervention from ashore in case of emergency to ensure navigational safety. Therefore, remote controlling service is as an indispensable shore-based support service for all autonomous degrees of MASS. It is recommended to add remote control and monitor service in MS to meet the requirements of MASS operation.

# Actions requested

ENVA Committee is requested to:

The committee is invited to consider the requirements of MASS for shore-based services.

The committee is invited to note the information provided in Appendix 1 and consider to carry out relevant discussions.

Annex 1: Remote control and monitor service

- Objects of service

Remote control and monitor service are applicable to manned ships and MASS of various degrees.

- Description of the Maritime Service

MASS of Degree one will send back information regarding navigational status, surrounding environment, ship-borne equipment status, and distress alarm, etc. during its voyage process, and RCC will provide real-time monitoring and decision support accordingly.

MASS of Degree two and three will send back ship videos to RCC in real-time, in addition to the information regarding navigational status, surrounding environment, ship-borne equipment status, and distress alarm, etc. under its remote piloting mode. RCC will reconstruct the navigation scene based on the information acquired, and control the ship in real-time by sending remote controlling instructions.

Mass of degree four is in the fully autonomous navigation mode, which requires real-time send back of ship navigational status, surrounding environment, ship-borne equipment status, and distress alarm, etc.. to RCC, and the need for remote taking over control by RCC is determined based on the ship navigation status.

- Purpose

It is intended to provide remote control and monitor service to MASS of different degrees. It can provide monitoring of MASS during its navigation and remote intervention in case of emergency or preparatory conditions.

- Service information

Table 1 lists the information required for remote control and monitor service.

Table 1: Information required for remote control and monitor service

| Related information | Example |
| --- | --- |
| Remote control information | Remote control information includes main thruster instruments, side thruster instruments, steering gear control instruments, navigation lights control instruments, etc. |
| Real-time video | Real-time video stream on board |
| environment information | The surrounding target information and the hydrological and meteorological information detected by the ship |
| navigation status | The course, heading, speed, draft and other information of the ship |
| Equipment status on board | The status information of engine room equipment, communication and navigation equipment, etc. |
| Distress alarm information | The basic information such as the MMSI, ship name, location, time, type of the distress, etc. of the ship in distress |
| High-accuracy positioning information | High-accuracy positioning service provided by shore-side |

- Operational approach

Table 2 lists the operational approach of remote control and monitor service of MASS.

Table 2: The implementation of remote control and monitor service

|  | Degree one | Degree two | Degree three | Degree four |
| --- | --- | --- | --- | --- |
| Remote control information | / | The structured data shall be sent to the shipboard remote response system by RCC. | The structured data shall be sent to the shipboard remote response system by RCC. | The structured data shall be sent to RCC for remote intervention. |
| Real-time video | The streaming media data shall be sent to RCC by shipboard system (optional) | The streaming media data shall be sent to RCC by shipboard system | The streaming media data shall be sent to RCC by shipboard system | The streaming media data shall be sent to RCC by shipboard system |
| environment information | The structured data shall be sent to RCC by shipboard system | The structured data shall be sent to RCC by shipboard system | The structured data shall be sent to RCC by shipboard system | The structured data, shall be sent to RCC by shipboard system |
| navigation status | The simulated or structured data shall be sent to RCC by shipboard system | The structured data shall be sent to RCC by shipboard system | The structured data shall be sent to RCC by shipboard system | The structured data sent to RCC by shipboard system |
| Equipment status on board | The structured data shall be sent to RCC by shipboard system (optional) | The structured data, shall be sent to RCC by shipboard system | The structured data shall be sent to RCC by shipboard system | The structured data, shall be sent to RCC by shipboard system |
| Distress alarm information | The simulated or structured data shall be sent to RCC and search and rescue coordination center (SRCC) by shipboard system | The structured data shall be sent to RCC and SRCC by shipboard system | The structured data shall be sent to RCC and SRCC by shipboard system | The structured data shall be sent to RCC and SRCC by shipboard system |
| High-accuracy positioning information | The high-accuracy positioning information shall be provided for ships through shore-based facilities and satellites (optional) | The RCC obtains high-accuracy positioning information acquired by ships through shore-based facilities and satellites | The RCC obtains high-accuracy positioning information acquired by ships through shore-based facilities and satellites | The shipboard autonomous navigation system obtains high--accuracy positioning information through shore-based facilities and satellites |
| … | ? | ? | ? | ? |

Input document number,to be assigned by the Committee Secretary

Leave open if uncertain