IALA Guideline

VTS46-8.3.3

VTS PHRASELOGY

Edition 1.0

Document date

Revisions to this IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |
| month/year approved by Council |  |  |

1	INTRODUCTION	7

2	DOCUMENT PURPOSE	7

3	DOCUMENT APPLICABILITY	7

4	PRINCIPLES	7

4.1	constant attention	8

4.2	phonetic alphabet	8

4.3	Read-back	8

4.4	Questioning Techniques	9

4.4.1	Closed Questions	9

4.4.2	Open Questions	9

4.4.3	Funnel Questions	9

4.4.4	Leading Questions	10

4.5	Ambiguous Terminology	10

4.6	Responses	11

4.7	Corrections	11

4.8	Repetition	11

4.9	Numbers	11

4.10	Positions	11

4.11	Bearings	12

4.12	Course	12

4.13	Distances	12

4.14	Speed	12

4.15	Time	12

4.16	Geographical names	12

5	MESSAGE MARKERS	12

5.1	Information	13

5.2	Advice	13

5.3	Warning	13

5.4	Instruction	13

5.5	Question	13

5.6	Answer	14

5.7	Request	14

5.8	Intention	14

6	ROUTINE VTS COMMUNICATIONS	14

6.1	General Communications	14

6.1.1	General	14

6.1.2	Request for additional reports	16

6.1.3	Contact / monitoring requests	16

6.1.4	Use of other channels	16

6.2	Standard Reporting Information	17

6.2.1	Vessel identification and ship details	17

6.3	Navigating in VTS area	18

6.3.1	Approach / entry into VTS area	18

6.4	Route movements	18

6.5	Speed changes	19

6.6	Restricted areas	20

6.7	Clearances	20

6.7.1	General / berthing	20

6.8	Pilotage	21

6.8.1	Pilot boarding instructions	21

6.8.2	Specific pilot boat instructions	21

6.8.3	Specific helicopter instructions	21

6.9	Berthing	22

6.9.1	General	22

6.9.2	Departure from berth	22

6.10	Anchor operations	22

6.10.1	Anchoring instructions	22

6.10.2	Weigh anchor	23

6.10.3	Dragging	23

6.11	Vessel operations	23

6.11.1	Engine	23

6.12	Conduct of special activities	23

6.12.1	Specific instructions for conducting special activities	24

6.13	Provision of traffic information	24

6.14	Provision of Weather and hydrographical information	25

6.14.1	Meteorological information	25

6.14.2	Visibility	26

6.14.3	Wind	26

6.14.4	Tide	26

7	NON-ROUTINE VTS COMMUNICATIONS	26

7.1	Warning of navigational dangers and developing situations	26

7.1.1	Warning Notification	26

7.1.2	Requests for action	27

7.2	Management of traffic	28

7.2.1	Confirming intentions	28

7.2.2	Information on the intentions of other vessels	28

7.2.3	Navigational advice	29

7.2.4	Instruction	29

7.3	Response to emergency situations	29

7.3.1	Marine emergency	29

7.3.2	Mayday acknowledgement and relay	30

7.3.3	Pan-pan	30

7.3.4	Upgrading urgency to distress	30

7.3.5	Securite	30

7.3.6	Cancellations	30

7.4	Fishing vessel activity	30

7.5	Impacts on normal vts operations	31

7.5.1	Issues with vts operations	31

7.5.2	Evacuation of the vts centre	31

7.5.3	Radio checks	31

# INTRODUCTION

The use of standardised phraseology is one of the most important factors involved in the process of communication. It provides for quick and effective communication allowing us to overcome differences in language and at the same time reducing the opportunity for misunderstanding. Ambiguous or non-standard phraseology is a frequent causal or contributory factor in accidents and incidents.

# DOCUMENT PURPOSE

The IALA Guideline on VTS Phraseology has been prepared to:

- Assist in the greater safety of navigation in VTS areas.

- Standardise VTS communication with vessels and allied services through the provision of clear, concise, and unambiguous communications that are timely and effective, and reduce the opportunity for misunderstanding.

- Assist VTS Training organisations to incorporate the use of standard VTS phraseology into their course curriculums.

# DOCUMENT APPLICABILITY

VTS Authorities should adopt the phraseology described in this guideline in their day-to-day operations to demonstrate compliance with IALA Standard 1040 – Vessel Traffic Services.

The examples contained in this document are not exhaustive, but merely representative of phraseology in common use.

If it is necessary to use plain language, it should be used according to the same principles that govern the development of phraseologies in that communications should be clear, concise, and unambiguous.

# PRINCIPLES

The key principles to ensuring of clear, concise, and unambiguous communications that are timely and effective include:

- Constant Attention

- Phonetic Alphabet

- Read-Back

- Questioning Techniques

- Terminology

- Message Markers

## constant attention

Constant attention should be given to the correct use of phraseology in all instances in which they are applicable to establish efficient, clear, concise, and unambiguous communications.

VTS Authorities should implement appropriate procedures to ensure the constant and correct use of VTS phraseology in all instances in which they are applicable.

## phonetic alphabet

Phonetic alphabets are used to distinguish between letters, which sound similar when transmitted over the radio.  They are commonly used when transmitting callsigns and in cases where a single letter is used to designate something.

Examples of using the phonetic alphabet are provided in IALA Guideline 1132 – VTS VHF Voice Communications.

Question – Should these examples reside in this document instead of 1132?

## Read-back

Repeating back important information (read-back) is utilised to enhance the effectiveness of communication in many industries.

Read-back is a procedure whereby the vessel repeats a received message, or an appropriate part thereof, back to the VTS to obtain confirmation of correct reception.  Uncorrected erroneous read-back may lead to a deviation from the intention of the message and may remain undetected until the observed through the VTS’s traffic image.

The two key principles for reading back messages is:

- To ensure that the message is received correctly; and

- To benefit other mariners.

Misunderstandings in messages received may include half-heard words or guessed-at numbers. The potential for misunderstanding increases with the complexity of messages and may cause the bridge team to:

- Accept inadequate information received; or

- Determine for themselves the most probable interpretation.

Read back should be used to confirm that messages from VTS operators sent under message markers ‘INSTRUCTION, ADVICE or WARNING’ are correctly received and understood.  This can be achieved through the following steps:

- The VTS operator requesting the recipient to repeat back important information by using the message ‘Read back’.

- The recipient reading back or acknowledging in a manner to clearly indicate they have understood the message and will take appropriate action.

- The VTS operator listening to the read-back to ascertain that the message has been correctly acknowledged and take immediate action to correct any discrepancies revealed by the read-back.

Read-back should not be replaced by the use of terms such as “Roger” or “Copied”.

## Questioning Techniques

Information flow within a VTS is paramount.  VTS often gathers and disseminates information based on real time situations within the VTS area. In the computer world the term ‘garbage in, garbage out’ is often used.  The same applies to VTS communications, if you ask the wrong questions you will probably get the wrong answer.

To ensure effective questioning the following techniques should be used:

- Closed Questions

- Open Questions

- Funnel Questions

- Leading Questions

### Closed Questions

A closed question usually receives a one word answer, or a short factual statement.

| VTS | QUESTION. How many persons onboard? |
| --- | --- |

| VTS | QUESTION. Can you see the Entrance Beacon? |
| --- | --- |

### Open Questions

Generally open questions deliberately seek longer answers and are the opposite of closed questions.  Open questions usually contain terms to elicit further information.

| VTS | QUESTION. What is the nature of your problem? |
| --- | --- |

| VTS | QUESTION. What is your intention? |
| --- | --- |

### Funnel Questions

This questioning technique involves the use of a series of questions. Initially general questions are asked which increase in detail with each subsequent question.  With funnel questioning it may be useful to start with a closed question before following up with more open questions.

| VTS | QUESTION. Can you see the Princes Inner Buoy? |
| --- | --- |
| VTS | QUESTION. Is the buoy lit? |
| VTS | QUESTION. Is the buoy in the correct position? |
| VTS | QUESTION. Describe the damage? |

### Leading Questions

Asking a leading question can influence the recipients answer, closing off undesirable alternatives and guides the recipient in the desired direction.

| VTS | QUESTION. Is it your intention to pass to north of Buoy One? |
| --- | --- |

## Ambiguous Terminology

Some words in English have meanings depending on the context in which they appear.  Misunderstandings frequently occur, especially in VTS communications, and have resulted in accidents.

| VTS | Are you happy with your position? |
| --- | --- |

Other examples:

MAY

Do not say: “You may enter the fairway”

Say: “ANSWER.  You have permission to enter the fairway”

MIGHT

Do not say: “You might have permission to depart if ….”

Say: “ADVICE. Make bridge-to-bridge with (ship name) and report back”

SHOULD

Do not Say: “You should anchor in anchorage Z4”

Say: “ADVICE. Anchor in anchorage Z4”

COULD

Do not say: “You could be running into danger”

Say: “WARNING.  You are running into danger”

CAN

The word “CAN” describes the possibility, or the capability of doing something.

Do not say: “Can you confirm you will pass North of Prince’s Inner buoy?”

Say: “QUESTION.  Is it your intention to pass North of Princes Inner buoy?”

Further, the use of local terminology should be avoided as this can lead to confusion.

| VTS | Vessel swinging and proceeding outward bound |
| --- | --- |

| VTS | SPEED RESTRICTION. Ease down pass Charlton Pier |
| --- | --- |

## Responses

When the answer to a question is in the affirmative, say:

“Yes” followed by the appropriate phrase in full

When the answer to a question is in the negative, say:

“No” followed by the appropriate phrase in full.

When the information requested is not immediately available, say:

“Stand by” followed by the time interval within which the information will be available.

When the information requested cannot be obtained, say:

“No information”

## Corrections

When an error is made in a message, say:

“Correction” plus the corrected part of the message.

| VTS | Your Pilot will board you in 20 CORRECTION - Your Pilot will board you in 30, three zero, minutes. |
| --- | --- |

## Repetition

If any part of the message is considered sufficiently important, say:

“Repeat” followed by the corresponding part of the message.

| VTS | Do not overtake - REPEAT – do not overtake. The tide is 1.2m – REPEAT – one decimal two meters. |
| --- | --- |

When the message is not properly heard, say:

“Say again (please)”

## Numbers

Numbers are to be spoken in separate digits.  For example:

“One-five-zero” for 150

## Positions

When latitude and longitude are used, these shall be expressed in degrees and minutes (and decimals of a minute if necessary), north or south of the Equator, and East or West of Greenwich.

| VTS | WARNING. Dangerous wreck in position 15 degrees 34 minutes North, 61 degrees 29 minutes West. |
| --- | --- |

When the position is related to a mark, the mark should be a well-defined charted object.

## Bearings

The bearing of the mark or vessel concerned is the bearing in the 360 degree notation from North (true north unless otherwise stated), except in the case of relative bearings.  Bearings may be either FROM the mark or FROM the vessel.

| VTS | Pilot boat is bearing 215 degrees from you. |
| --- | --- |

## Course

Always to be expressed in 360 degree notation from north (true north unless otherwise stated). Whether this is TO or FROM a mark can be stated.

Courses should be ‘course made good’ and should include a distance.

Do not use ‘INSTRUCTION’ as a message marker, only ‘ADVICE’.

| VTS | ADVICE. Recommend course to make good 127 degrees for 2 nautical miles. |
| --- | --- |

## Distances

To be expressed in nautical miles or cables (tenths of a mile), the unit always to be stated.

## Speed

To be expressed in knots. Without further notation, speed means through the water, or, “Ground speed” meaning the speed over ground.

## Time

Time should be given in UTC, in a 24 hour format.  If using Local Time, then this should be stated.

## Geographical names

Place names should be those that are on navigational charts and publications.

Where this is not available then latitude and longitude should be used.

# MESSAGE MARKERS

In order to facilitate shore-to-ship and ship-to-shore communications, message markers should be used to increase the probability of the purpose of the message being properly understood.

The following message markers should be used to increase the effectiveness and urgency of VHF communications as required. The VTSO should apply these depending on the assessment of the situation.

The message marker should be spoken preceding the message or at the corresponding part within the message.

Pro-words complement the message marker and prepare the receiver for the nature and content of the message that will follow (eg ‘WEATHER’ before ‘INFORMATION’ and ‘COLLISION’ before ‘WARNING’).

## Information

This indicates that the following message is restricted to observed facts, situations etc.

| VTS | INFORMATION. Dredger “Goomai” is operating in the Williamstown Channel 30 metres West of the Centreline. |
| --- | --- |

| VTS | INFORMATION. ‘XXXX’ Light is not operational. |
| --- | --- |

Note: This marker is preferably used for navigational and traffic information, etc. Information is to assist the on board decision making process.

## Advice

This indicates that the following message implies the intention of the sender to influence others by a recommendation.

| VTS | ADVICE. Container Vessel ‘Maersk Dusseldorf’ is inbound and will take tugs at Swanson Dock swing basin. Slow down and await ‘Maersk Dusseldorf’ to clear the basin”. |
| --- | --- |

Note: The decision whether to follow the ADVICE still stays with the recipient. ADVICE does not necessarily have to be followed but should be considered very carefully.

## Warning

This indicates that the following message implies the intention of the sender to inform others about danger.

| VTS | WARNING. VTS radar indicates you are heading towards shallow water. |
| --- | --- |

Note: This means that any recipient of a WARNING should pay immediate attention to the danger mentioned and confirm the vessel’s position, course and speed in relation to the warning. Consequences are up to the recipient.

## Instruction

This indicates that the following message implies the intention of the sender to influence the intention of others by a regulation.

| VTS | INSTRUCTION. Remain alongside until a further instruction is passed. |
| --- | --- |

| VTS | INSTRUCTION. Do not cross the fairway. |
| --- | --- |

Note: The sender (eg VTS) must have the full authority to send such a message. The recipient has to follow this legally binding message unless they have contradictory safety reasons which then have to be reported to the sender.

## Question

This indicates that the following message is of interrogative character.

| VTS | QUESTION. What is your maximum draft. |
| --- | --- |

## Answer

This indicates that the following message is the reply to a previous question.

| VTS | ANSWER. You have permission to enter fairway. |
| --- | --- |

| SHIP | ANSWER. My present maximum draft is zero seven metres. |
| --- | --- |

Note: An answer should not contain another question.

## Request

This indicates that the following message is asking for action from others with respect to the vessel.

| SHIP | REQUEST. I require two tugs. |
| --- | --- |

Note: The use of this marker is to signal that I want something to be arranged or provided. REQUEST must not be used involving navigation, or to modify COLREGS.

## Intention

This indicates that the following message informs others about immediate navigational action intended to be taken.  Only a ship would normally use this message marker.

| SHIP | INTENTION. I will reduce my speed. |
| --- | --- |

Note: The use of this message marker is logically restricted to messages announcing navigational actions by the vessel sending this message.

# ROUTINE VTS COMMUNICATIONS

## General Communications

### General

| Message Element | Message Intent | Example |
| --- | --- | --- |
| ALL RECEIEVED |  |  |
| APPROVED | Permission for proposed action granted. |  |
| CHECK | Examine (something) in order to determine its accuracy, quality, or condition, or to detect the presence of something. |  |
| CONFIRM | I request verification of: (clearance, instruction, action, information) |  |
| CORRECT | That is correct |  |
| CORRECTION | A change that rectifies an error or inaccuracy |  |
| DISREGARD | Ignore last message |  |
| GO AHEAD |  |  |
| INCORRECT | That is not correct |  |
| I REPEAT | I will state my message again |  |
| I SPELL | Phonetic spelling follows |  |
| I SAY AGAIN | I repeat for clarity or emphasis |  |
| MAINTAIN | Continue in accordance with the condition(s) specified or in its literal sense, eg “Maintain your course” |  |
| NEGATIVE | “No” or “Permission is not granted” or “That is not correct” |  |
| NO |  |  |
| OUT | End of transmission. No answer is required or expected |  |
| OVER | End of transmission. An answer is expected |  |
| READ-BACK | Read back to me the message as you receive it |  |
| REPEAT YOUR LAST |  |  |
| REPORT | Pass requested information |  |
| REQUEST | I should like to know or I wish to obtain |  |
| SAY AGAIN | A request to retransmit all or a portion of a transmission |  |
| STANDBY | Wait and I will call you |  |
| VERIFY | Check and confirm with originator |  |
| WHEN READY | The associated instruction may be completed with at any future time. |  |

### Request for additional reports

| Message Element | Message Intent |
| --- | --- |
| REPORT AT (location or time) |  |
| REPORT AGAIN AT (location) |  |
| REPORT LEAVING (location / VTS area) | Instruction to report when the ship has left the VTS area or a particular location. |
| REPORT PASSING (location) | Instruction to report when the ship has passed the specified location |
| NEXT REPORT AT (location or time) |  |
| REPORT ETA AT (position) | Instruction to report the estimated time of arrival at the specified position |
| REPORT ANY ETA DELAYS OF (time eg 10mins or longer) |  |
| CONTACT (ship) FOR BRIDGE TO BRIDGE |  |

### Contact / monitoring requests

| Message Element | Message Intent |
| --- | --- |
| CONTACT | Establish radio contact with ….(your details have been passed) |
| CONTACT (VTS) ON (frequency) | The ship is required to call the VTS on the specified frequency |
| AT (position) CONTACT (VTS) ON (frequency) | At the specified position the VTS is to be contacted on the specified frequency |
| AT (time) CONTACT (VTS) ON (frequency) | At the specified position the VTS is to be contacted on the specified frequency |
| MONITOR | Listen out on (frequency) |
| MONITOR (VTS) ON (frequency) | The ship is to monitor the VTS on the specified frequency |
| CONTACT (ship) FOR BRIDGE TO BRIDGE |  |
| STANDBY |  |
| MAINTAIN LISTENING WATCH ON (frequency) |  |
| CALL AGAIN IN (x time) |  |

### Use of other channels

| Message Element | Message Intent |
| --- | --- |
| CHANGE CHANNEL TO [XXX] | Request to change channels |
| STANDBY ON CHANNEL [XXX] | Request vessel to standby on a channel |

## Standard Reporting Information

### Vessel identification and ship details

When entering the VTS area the following information about the vessels identity and ship particulars will need to be clarified.   The prefix of “WHAT IS YOUR XXXX” will normally be used.

| Message Element | Message Intent |
| --- | --- |
| NAME |  |
| CALL SIGN |  |
| DESTINATION |  |
| NEXT PORT OF CALL |  |
| LAST PORT OF CALL |  |
| BERTH |  |
| NEXT BERTH |  |
| CARGO |  |
| PRESENT POSITION |  |
| ROUTE |  |
| VOYAGE / PASSAGE PLAN |  |
| COURSE |  |
| HEADING |  |
| SPEED |  |
| ARRIVAL DRAFT |  |
| MAXIMUM DRAFT |  |
| FORWARD DRAFT |  |
| AFT DRAFT |  |
| DRAFT FORWARD |  |
| DRAFT AFT |  |
| GROSS TONNAGE |  |
| LOA |  |
| AIR DRAFT |  |

## Navigating in VTS area

### Approach / entry into VTS area

When a vessel enters the VTS Area there is an exchange of information, such as:

- Verifying the vessels identity

- Confirm reporting requirements

- Provide relevant traffic information

- Provide navigational / fairway information

- Establishing compliance with IMO requirements (charts and publications, passage plan, mechanical defects, personnel shortfalls)

Example of communications with a vessel approaching or entering into the VTS area.

| SHIP | [Ship] APPROACHING / ENTERING [VTS area] |
| --- | --- |
| VTS | [Ship] … For example: - CONFIRM [certain details] - INFORMATION (eg berthing, traffic updates, navigational hazards etc) - REPORT AGAIN AT (location) - PILOT to board at ….. |

The VTS may also need to provide further instructions for the approach/entry to the VTS area such as:

| Message Element | Message Intent |
| --- | --- |
| PERMISSION TO ENTER (details eg VTS area) |  |
| PERMISSION TO CROSS (details eg line / area) |  |
| PERMISISON TO MOVE IN (area eg channel) |  |
| DO NOT PROCEED PAST (location eg pilot boarding ground without pilot) |  |
| DO NOT PASS REPORTING POINT |  |
| DO NOT ENTER |  |
| MAINTAIN ETA TO (location eg pilot boarding ground) |  |
| HOLD POSITION |  |

## Route movements

| Message Element | Message Intent |
| --- | --- |
| PROCEED TO (position / area) | Instruction to proceed directly from the present position to the specified position |
| WHEN ABLE TO PROCEED TO (position) | Instruction to proceed, when able, directly to the specified position |
| AT (time) PROCEED TO (position) | Instruction to proceed at the specified time, directly to the specified position |
| AT (position) PROCEED TO (position) | Instruction to proceed, at the specified position, directly to the next specified position |
| CLEARED TO (position) VIA (route) | Instruction to proceed to the specified position via the specified route |
| CLEARED (route clearance) | Instruction to proceed via the specified route |
| CLEARED (procedure name) | Instruction to proceed in accordance with a specified procedure (eg enter port limits) |
| AT (position) CLEARED (route clearance) | Instruction to proceed from specified position via the specified route |
| AT (position) CLEARED (procedure name) | Instruction to proceed from specified position via the specified procedure |
| EXPECT FURTHER CLEARANCE AT (time / position) | Notification that an onwards clearance may be issued at the specified time or position |
| WAIT for (vessel) TO (details eg clear berth / enter fairway / get underway / leave berth) |  |
| CONFIRM ROUTE AT (position) | Instruction to confirm current route |
| CONFIRM ROUTE |  |
| OVERTAKING PROHIBITED |  |

## Speed changes

| Message Element | Message Intent |
| --- | --- |
| MAINTAIN (speed) | The specified speed is to be maintained |
| INCREASE SPEED TO (speed) | The present speed is to be increased to the specified speed and maintained until further advised |
| REDUCE SPEED TO (speed) | The present speed is to be reduced to the specified speed and maintained until further notice |
| REDUCE SPEED TO SAFE SPEED | <does anyone request a ship to maintain a safe speed> The present speed is to be reduced to the vessels’ safe manoeuvring speed and maintained until further notice |
| DO NOT EXCEED (speed) | The specified speed is not to be exceeded |
| RESUME NORMAL SPEED | Notification that the previously issued speed restriction is no longer required |
| CONFIRM SPEED | Instruction to report present speed |
| SPEED LIMIT OF (x) KNOTS |  |
| SPEED LIMITS IN (area of) |  |
| SPEED LIMITS ENDS AT (location) |  |

## Restricted areas

| Message Element | Message Intent |
| --- | --- |
| RESTRICTED AREA (name) ACTIVE, CLEARANCE REQUIRED | To provide information about a restricted area. |
| RESTRICTED AREA (name) ACTIVE, AVAILBLE FOR TRANSIT | To provide information about a restricted area. |
| RESTRICTED AREA (name) ACTIVE, AVAILBLE UNTIL (time) | To provide information about a restricted area. |
| RESTRICTED AREA (name) ACTIVE, CLEARANCE NOT AVAILBLE | To provide information about a restricted area. |
| APPROVED TO OPERATE IN RESTRICTED AREA (name) | Able to operate in an area |
| HAZARDOUS ACTIVITES IN (area). PROCEED AT YOUR OWN RISK / CLEARANCE NOT AVAILBLE | Unauthorised movements |

| Message Element | Message Intent |
| --- | --- |
| MAINTAIN (speed) | The specified speed is to be maintained |

## Clearances

### General / berthing

| Message Element | Message Intent |
| --- | --- |
| CLEARANCE | Authorised to proceed under the conditions specified |
| CLEARED TO (destination) |  |
| CLEARED TO DEPART (location) |  |
| REMAIN ALONGSIDE UNTIL (details) | When clearance issued will be subject to a delay (may need additional plain text to explain why) |
| REMAIN OUTSIDE (location) AND STANDBY | When clearance will be issued subject to a delay |
| CANCEL CLEARANCE | When clearance has been cancelled |
| PROCEED TO (location eg anchorage area, fairway) |  |
| PERMISSION TO ENTER (details eg port) |  |

## Pilotage

Depending on the VTS there may be local differences in terms used such as pilot station, pilot boarding ground, pilot embarkation point. Similarly when referencing the pilot’s activities such as on bridge, embark or onboard.

### Pilot boarding instructions

| Message Element | Message Intent |
| --- | --- |
| PILOT WILL BOARD /EMBARK AT (time / location) |  |
| PILOT BOARDING TIME IS (time) |  |
| PILOT WILL ARRIVE AT (time) |  |
| WAIT FOR PILOT at (location) |  |
| PILOT CANNOT BOARD / GET OFF |  |
| PILOT DELAYED. ADJUST YOUR ETA. |  |

### Specific pilot boat instructions

There is probably merit in listing some of the more common ones that ports use? Please add to the list.

Examples of instructions that a VTS may provide to vessel:

Message Phrases

Rig ladder on (port/starboard side) (x meters) above water

Maintain boarding speed of (speed)

Make lee on your port/starboard side

### Specific helicopter instructions

There is probably merit in listing some of the more common ones that ports use? Please add to the list.

Examples of instructions that a VTS may provide to vessel:

Message Phrases

## Berthing

### General

| Message Element | Message Intent |
| --- | --- |
| PERMISSION TO PROCEED TO (berth name) |  |
| BERTH AT (time) |  |
| COME ALONGSIDE my (port / starboard side) |  |
| DO NOT COME ALONGSIDE |  |
| BERTH IS NOT CLEAR |  |
| BERTH WILL BE CLEAR AT (time) |  |
| WAIT UNTIL BERTH IS CLEAR |  |
| (Vessel) IS LEAVING HER BERTH |  |
| (Vessel) HAS LEFT BERTH (name) |  |
| (Vessel) WILL LEAVE AT (time) |  |
| BERTHING DELAYED UNTIL (time / by X hrs) |  |
| BERTH CHANGED FROM (A) to (B) |  |

### Departure from berth

| Message Element | Message Intent |
| --- | --- |
| CALL WHEN SINGLED UP |  |
| CALL ON LAST LINE |  |
| CALL (time) BEFORE DEPARTURE |  |

## Anchor operations

### Anchoring instructions

| Message Element | Message Intent |
| --- | --- |
| PERMISSION TO ANCHOR until (Pilot / tugs arrives) |  |
| PROCEED TO ANCHORAGE AREA (location) |  |
| ANCHOR TO AWAIT (details eg tug, berth) |  |
| ANCHOR AT (time) |  |
| ANCHORING IS PROHIBITED |  |
| ANCHOR UNTIL PILOT ARRIVES / ANCHOR BEFORE PILOT ARRIVES |  |
| ANCHORED IN WRONG POSITION |  |
| REPORT WHEN YOU HAVE DROPPED ANCHOR |  |
| ANCHOR CLEAR OF (details eg fairway / outside port limits) |  |
| ANCHOR IN (position) |  |
| REPORT WHEN ANCHOR DROPPED |  |

### Weigh anchor

| Message Element | Message Intent |
| --- | --- |
| WEIGH ANCHOR (immediately / at time) |  |
| CALL BEFORE WEIGH ANCHOR |  |

### Dragging

Where a vessel has been identified to be dragging anchor, it may be appropriate, to issue an all ships broadcast to notify all ships in the area of the developing situation.

| Message Element | Message Intent |
| --- | --- |
| YOU ARE DRAGGING ANCHOR |  |
| ARE YOU DRAGGING ANCHOR |  |
| CHECK YOUR POSITION |  |
| YOU ARE LIKELY TO DRAG |  |

## Vessel operations

### Engine

| Message Element | Message Intent |
| --- | --- |
| REPORT WHEN ENGINES READY |  |
| KEEP YOUR ENGINES READY |  |
| KEEP ENGINES ON STANDBY |  |
| REPORT WHEN UNDERWAY |  |

## Conduct of special activities

| Message Element | Message Intent |
| --- | --- |

### Specific instructions for conducting special activities

There is probably merit in listing some of the more common ones that ports use? Please add to the list (eg approval for hot works, life boat drills etc).

Examples of instructions that a VTS may provide to vessel:

Message Phrases

## Provision of traffic information

Any broadcast of traffic information should be preceded with the message marker of “Traffic Information”.  This leaves the receiving vessel in no doubt as to the message which will follow.

To assist the on-board decision making process, it is necessary to pass relevant information which may be of short to medium term navigational significance.

Consideration should also be given to including tailored information to vessels relevant to their immediate movements, or to the planning of their intended passage.  Generally this will be restricted to one of the following expressions:

| Message Element | Message Intent |
| --- | --- |
| NO REPORTED TRAFFIC |  |
| NO TRAFFIC TO REPORT |  |
| MEET |  |
| PASS |  |
| OVERTAKING |  |
| INWARD |  |
| OUTWARD |  |
| EASTBOUND/ WESTBOUND/ NORTHBOUND/ SOUTHBOUND etc |  |
| TURNING |  |
| ENTERING |  |
| LEAVING |  |
| GETTING UNDERWAY |  |
| DEPARTING |  |
| ANCHORING |  |
| CROSSING |  |
| NOT UNDER COMMAND |  |
| CONSTRAINED BY |  |
| DUE TO TRAFFIC | The associated instruction is issued due to traffic considerations |

It is necessary to identify the vessel by name. Occasionally it may be beneficial to identify the vessel by type, for example ‘container vessel Maersk Rotterdam’.

Traffic information may also include other general maritime safety information to facilitate the safe navigation and awareness of activities in the VTS area. For example, to advise that AtoN equipment not available.

Further, a ship may request traffic information from VTS as required.

An example of traffic information:

| VTS | Traffic Information Meet (Vessel) at (time/location/lat,long) Pass (Vessel) at (time/location/lat,long) Overtake (Vessel) at (time/location/lat,long) (vessel) is ahead of you (Vessel) is leaving (VTS/Area) (Vessel) is entering (VTS/Area) No reported traffic in your area Additional Maritime Safety Information (MSI event eg light, incident, type of operation) in (location/areas/lat,long) Fishing vessels in (Area) Proceed at slow speed/caution when passing (location/details eg diving operations) |
| --- | --- |

## Provision of Weather and hydrographical information

VTS may decide to provide vessels with relevant information from their sensor equipment, or reported information to vessels in the VTS area.

Further detailed weather and hydrographical broadcasts are issued by meteorological departments.  Some, or all of these details may also be deemed useful for vessels in the VTS area.

### Meteorological information

| Message Element | Message Intent |
| --- | --- |
| WEATHER INFORMATION |  |
| PRESENT WEATHER |  |

### Visibility

| Message Element | Message Intent |
| --- | --- |
| RESTRICTED VISIBILITY IN (area) |  |
| VISIBILITY AT (location/area) IS (x meters) |  |
| VISIBILITY IS EXPECTED TO (increase/decrease) |  |
| WAIT (outside/location) UNTIL VISIBILITY (increases/improves/is restored) |  |

Example:

| VTS | Due to restricted visibility, the VTS area is closed to navigation |
| --- | --- |

### Wind

| Message Element | Message Intent |
| --- | --- |
| WIND EXPECTED TO INCREASE TO |  |
| IF WIND EXCEEDS (x knots) THEN (details of action eg vessels will be instructed to leave berths, tug services will be suspended) |  |
| WIND (at) (degrees/cardinal point) at (kts) |  |

### Tide

| Message Element | Message Intent |
| --- | --- |
| TIDE IS (details) |  |
| TIDE at (location) (meters) |  |
| TIDE (meters) above/below prediction. |  |

# NON-ROUTINE VTS COMMUNICATIONS

## Warning of navigational dangers and developing situations

### Warning Notification

| Message Element | Message Intent |
| --- | --- |
| SENSORS INDICATE (eg. Vessel outside of fairway, approaching, left area etc) |  |
| SHALLOW WATER (details eg north of route) |  |
| INCIDENT IN (location/area) |  |
| NAVIGATIONAL HAZARD (details eg derelict vessel, uncharted rock, pipeline leaking gas) |  |
| DREDGING IN (area) |  |
| DIVING OPERATIONS |  |
| SURVEY OPERATIONS |  |

Some examples of immediate risk:

| VTS | WARNING. Shallow water ahead of you. Risk of grounding. |
| --- | --- |
|  | WARNING. You are [getting closer to the shore/approaching the lee shore]. Risk of grounding. |
|  | WARNING. Fishing [net/gear] ahead of you. [Stay/Keep] clear. |
|  | WARNING. Risk of collision with the vessel ahead of you. |

### Requests for action

| Message Element | Message Intent |
| --- | --- |
| STAY CLEAR OF (details eg of fairway) |  |
| KEEP CLEAR OF (details eg of location / thing) |  |
| AVOID (details eg area) |  |
| CHECK YOUR POSITION |  |
| RETURN TO (details eg fairway / route) |  |
| WAIT FOR (details eg vessel leaving berth, ahead of you) |  |
| NAVIGATE WITH CAUTION |  |
| WIDE BERTH REQUESTED |  |
| PASS TO (details eg NW of location) |  |
| DO NOT OVERTAKE |  |
| DO NOT CROSS FAIRWAY |  |
| PROCEED WITH CAUTION |  |
| PROCEED AT SAFE SPEED |  |
| DO NOT PROCEED |  |
| DO NOT ENTER |  |
| KEEP A SAFE DISTANCE |  |

## Management of traffic

To assist in the management of traffic within the VTS area, here are some example of phrases that a VTS may use.   <Note: This is just a list of phrases (please feel free to add / delete as appropriate) at the moment until it is decided whether there is merit to include statements like these>

### Confirming intentions

Message Phrases

A meeting vessel (is) dead ahead of you. What are your intentions?

Will you pass her port to port or starboard to starboard?

A crossing vessel (is) on your starboard bow. What are your intentions with her?

Will you cross ahead of her or (will you) pass astern of her?

(There is a) Crossing vessel on your starboard bow. You must give way.

There are three westbound vessels in the [passage/fairway]. What are your intentions (with them)?

Which vessel will you follow? (When proceeding west)

Which vessel will you pass astern of? (When crossing the westbound lane to join the eastbound lane)

Will you pass astern of the last vessel? (When crossing the westbound lane to join the eastbound lane)

### Information on the intentions of other vessels

Message Phrases

I have contacted Takagi Maru and [checked/confirmed] her intentions. She will pass astern of you.

Takagi Maru has agreed for you to cross ahead of her.

The vessel astern of you will overtake you on your port side.

The vessel on your starboard bow will cross ahead of you.

The vessel on your port bow will pass astern of you.

Takagi Maru will alter course to starboard and pass astern of you.

The vessel (dead) ahead of you will alter course to starboard to pass port to port.

The vessel ahead of you, Takagi Maru, is suggesting port-to-port passing. Do you agree?

The vessel ahead of you, Takagi Maru, will reduce speed [to pick up a pilot/for pilot embarkation]

Takagi Maru will cross ahead of you and proceed west

Takagi Maru will join the eastbound traffic ahead of you

A large bulk carrier will cross the fairway. Please reduce speed and pass astern of her

The vessel on your starboard quarter, Takagi Maru, intends to overtake on your starboard side. Do you agree?

The eastbound vessel on your port bow, Takagi Maru, will enter the westbound lane to avoid fishing nets.

Takagi Maru's intentions are not clear. Approaching vessels, navigate with caution

### Navigational advice

Message Phrases

Crossing the fairway (keeping your present course) is prohibited.

Move as closer to the centre of the fairway as [possible/practicable]

Keep clear of the traffic separation line so far as practicable

Keep the buoy on your [port/starboard] side.

When you cross the fairway, do not impede the safe passage of vessels navigating in the fairway

Entering the passage from [its/either] side is prohibited

The fairway speed in Uraga Traffic Route is one two, twelve knots.

You are in the speed limit section. You must reduce speed.

You are in the [no-overtaking/no-passing] zone. Do not overtake.

Overtaking is prohibited in the West Passage bend.

### Instruction

Message Phrases

Let the vessel on your starboard quarter overtake you

You are too close to the vessel ahead of you

You must give way to the vessel on your starboard bow

## Response to emergency situations

### Marine emergency

Emergency Broadcasts may be made in the event of a major marine, environmental, security incident or on suspension of any VTS services, advising of any special restricted / safety areas and any communication restrictions or changes. In particular:

- All vessels should be requested to maintain current communication watch, minimise all VHF radio traffic and be ready for vessel traffic instructions.

- Any vessel requiring emergency assistance within the VTS area should contact

For example:

| VTS | All Ships, All Ships, All Ships (position and name of vessel) (nature of Incident) (intentions of the vessel / details of assistance required) (any other useful information) |
| --- | --- |

<Examples are probably needed for the following sections>

### Mayday acknowledgement and relay

The call ‘Mayday’ is used to indicate the vessel is in grave and imminent danger to life or the vessel, and require immediate assistance.

Upon receipt of a Mayday, the VTSO should acknowledge the distress message with the vessel concerned.

### Pan-pan

The call ‘Pan Pan’ indicates the vessel is not in grave and imminent danger, but requires urgent assistance.  All urgency messages should be broadcast to all stations or a specific station.

### Upgrading urgency to distress

The need to broadcast an upgrade from Urgency to Distress may arise, when:

- The situation of a vessel deteriorate from a threat to grave and imminent danger requiring immediate assistance.

- The incident is considered to be major in nature

### Securite

The call ‘Securite’ indicates there is an important message regarding navigational warnings or serious weather reports.  All safety messages should be broadcast to all stations or a specific station.

### Cancellations

When the incident is over, an appropriate message should be broadcast to all stations.

## Fishing vessel activity

| Message Element | Message Intent |
| --- | --- |
| FISHING BOATS IN (area/location) |  |
| FISHING BOATS TO THE (area/location) |  |
| PASS FISHING BOATS TO THE (north/south/east/west) |  |
| KEEP FISHING BOATS TO (port / starboard side) |  |
| FISHING GEAR REPORTED AT (location) |  |
| WATCH OUT FOR (details) |  |

## Impacts on normal vts operations

### Issues with vts operations

| Message Element | Message Intent |
| --- | --- |
| (VTS) AT REDUCED SYSTEM CAPACITY | Notification that the VTS is operating at reduced capacity |

### Evacuation of the vts centre

<Question - How do you advise if the VTS centre has been evacuated or operations relocated?  Perhaps “OPERATIONS OFFLINE UNTIL ‘XXXX’>

### Radio checks

<Question – what are the most common phrases used to describe the readability of a transmission?>

When replying to radio check requests from vessels, signal strength may be expressed in the following manner:

| Message Element | Message Intent |
| --- | --- |
| Bad / one / unreadable | One (ie barely perceptible) |
| Poor / two / Readable now and then | Two (ie weak) |
| Fair / three / Readable but with difficulty | Three (ie fairly good) |
| Good / four / Readable | Four (ie good) |
| Excellent / five / Perfectly readable | Five (ie very good) |

This may also provide an opportunity to reciprocate the radio check with the other vessel to ensure VTS transmissions are clear.

| Message Element | Message Intent |
| --- | --- |
| HOW DO YOU READ ME |  |
| READ YOU LOUD AND CLEAR |  |
| READ YOU (signal strength eg Poor) |  |
| I CANNOT READ YOU WELL |  |
| SIGNAL IS (details eg too Weak) |  |
| (there is /I have) (a lot of /too much) NOISE |  |