Input paper:   VTS47-3.1.11

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM		□  ENG		□  PAP						X Input

□  ENAV	X  VTS								□  Information

Agenda item 				3

Technical Domain / Task Number 2	…………………………………

Author(s) / Submitter(s)		The Nautical Institute

Comments from the Nautical Institute on VTS47 10.2.1 - Draft IALA Guideline

# Summary

This paper provides comments from a review of the draft IALA Guideline on VTS Awareness for Navigation Officers (VTS47-10.2.1, ex VTS46-13.2.1)

## Purpose of the document

This paper provides comments for consideration by the IALA VTS Training Working Group (WG3)

## Related documents

Related document includes IALA VTS46-13.2.1.

# Background

The Nautical Institute has reviewed the developing of this document since the initial draft provided in 2018.  A representative of the Nautical Institute will participate in VTS47 and provide further comment as may be required.

# Action requested of the Committee

The Committee is requested to review the comments and proposed changes and address as appropriate.

VTS awareness for Deck officers

Edition 1.0

Document date

Revisions to this IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |
| month/year approved by Council |  |  |

1.	IntRoduction	5

1.1.	BACKGROUND	5

1.2.	APPLICATION	5

2.	AIMS AND OBJECTIVES	5

3.	OVERVIEW OF VTS	5

3.1.	TERMS USED IN VTS	5

3.2.	how to obtain VTS INFORMATION	6

3.3.	Participation in VTS	6

3.4.	AIMs, GOALs, ROLEs AND FUNCTIONs OF VTS	6

3.4.1.	Aim of VTS	6

3.4.2.	Goal of VTS	6

3.4.3.	roles	6

3.4.4.	FUNCTIONS	6

3.5.	Benefits of VTS FOR the bridge team	7

3.6.	VTS and Ship REPORTING SYSTEMS	7

3.7.	VTS PROCEDURES	7

3.7.1.	INFORMATION	7

3.7.2.	Traffic Organisation	8

3.7.3.	Navigational Assistance	8

4.	ALLIED AND OTHER SERVICES	9

5.	COMMUNICATION BETWEEN THE NAVIGATION OFFICER AND THE VTS CENTRE	9

5.1.	TOOLS	9

5.2.	LANGUAGE	9

5.3.	REQUIREMENTS OF COMMUNICATIONS	9

5.4.	SMCP AND MESSAGE MARKERS	9

6.	Training Activities to Increase awareness of VTS	10

6.1.1.	VTS KNOWLEDGE IN THEORY TRAINING	10

6.1.2.	VTS KNOWLEDGE IN PRACTICAL TRAINING	10

6.2.	VHF	10

6.3.	ECDIS	11

6.4.	Considerations for Vessels in the VTS area	11

6.5.	Incidents and Emergencies	11

6.6.	familiarisation activities	12

6.7.	STRENGTHENing PILOTS’ FAMILIARITY WITH VTS	12

7.	ACRONYMS & DEFINITIONS	12

7.1.	ACRONYMS	12

7.2.	DEFINITIONS	13

8.	REFERENCES	13

List of Tables

[IALA Secretariat please update the tables]

# INTRODUCTION

## BACKGROUND

An effective Vessel Traffic Service (VTS) improves the safety and efficiency of navigation, the safety of life at sea and the protection of the marine environment from possible adverse effects of maritime traffic.

SOLAS Chapter 5 Regulation 12 provides the basis for Vessel Traffic Services, and is supported by IMO resolutions and circulars. The importance of VTS is furthermore referenced in STCW 1978.

An efficient VTS relies upon the co-operation between the bridge team and the VTS Operator (VTSO). The effectiveness of information exchange depends upon each party's understanding of their respective functions and responsibilities. Increased knowledge and understanding of a VTS by the bridge team will enhance teamwork between VTS and vessels and improve safe navigation.

It is recognised that not all navigation officers have the same knowledge level of VTS. A higher level of knowledge of VTS will enable better use of their services, resulting in enhanced situational awareness.

## APPLICATION

This Guideline applies primarily to Maritime Education and Training of bridge teams to enhance their understanding of VTS. It could also be used by other relevant stakeholders, such as flag states, harbour and port organisations, pilot organisations and shipping companies.

# AIMS AND OBJECTIVES

This guideline presents guidance and information which should be made available to navigation officers as part of their basic navigational training. The objective is to raise the awareness of the navigation officers of VTS through training, and to facilitate effective communication and exchange of information between the bridge team and VTSO.

# OVERVIEW OF VTS

## TERMS USED IN VTS

- Vessel traffic service (VTS) - a service implemented by a competent authority, designed to improve the safety and efficiency of vessel traffic and to protect the environment. The service should have the capability to interact with the traffic and to respond to traffic situations developing in the VTS area.

- Competent authority - the authority made responsible, in whole or in part, by the government for safety, including environmental safety, and efficiency of vessel traffic and the protection of the environment.

- VTS authority - the authority with responsibility for the management, operation and co-ordination of the VTS, interaction with participating vessels and the safe and effective provision of the service.

- VTS area - the delineated, formally declared service area of the VTS. A VTS area may be subdivided in sub-areas or sectors.

- VTS operator—an appropriately qualified person performing one or more tasks contributing to the services of the VTS.

- VTS reporting line - an electronic virtual line established by the competent authority to report to the VTS centre as required by the specified procedures and requirements when passing through the line.

- VTS centre - the centre from which the VTS is operated. Each sub-area of the VTS may have its own sub-centre.

- VTS voyage plan - a plan which is mutually agreed between a VTS Authority and the master of a vessel concerning the movement of the vessel in a VTS area.

## how to obtain VTS INFORMATION

The IALA VTS Manual provides information on VTS.  For detailed information on VTS consult the latest edition of the VTS Manual.

The services offered by a VTS to the mariner are promulgated to vessels in internationally recognised marine publications. These include details of the VTS, its capabilities, rules, regulations, requirements, radio frequencies and procedures.

## Participation in VTS

As identified in IMO SOLAS, Chapter V, Regulation 12, participation in a VTS may be mandatory within territorial waters. In the mandatory VTS area, VTS may be applicable to all vessels or there may be exemptions to certain types and sizes of vessels. It should be noted that vessels that are not required to participate in a VTS may still choose to do so.

## AIMs, GOALs, ROLEs AND FUNCTIONs OF VTS

VTS is recognised internationally as a navigational safety measure through SOLAS Chapter V (Safety of Navigation) regulation V/12.  VTS is implemented under a national legal framework adhering to international standards and recommendations.

### Aim of VTS

The purpose of a VTS is to improve the maritime safety and efficiency of navigation, safety of life at sea, the protection of the marine environment and/or the adjacent shore area, work sites and offshore installations from possible adverse effects of maritime traffic in a given area.  VTS may also have a role to play in security.

### Goal of VTS

The goals of implementing a VTS are to:

- aid the mariner in the safe and efficient use of navigable waterways;

- afford unhindered access to pursue commercial and leisure activities, subject to any restrictions that may exist;

- contribute to keeping the seas and adjacent environment free from pollution.

### roles

By monitoring the movements of vessels, the VTS Operator is able to interact with participating vessels in accordance with established regulations and procedures within the VTS area.

### FUNCTIONS

VTS functions can be subdivided into internal and external functions. Internal functions are the preparatory activities that have to be performed to enable a VTS to operate. These include data collection, data evaluation and decision-making. External functions are activities executed with the purpose of influencing traffic. They relate to the primary traffic management functions of rule-making, allocation of space, control of vessels and vessel manoeuvres to avoid collisions, as well as to other management functions such as enforcement, remedial and ancillary activities.

Amongst the most important functions that a VTS may carry out are those related to, contributing to and thereby enhancing:

- safety of life at sea;

- safety of navigation;

- search and Rescue;

- efficiency of vessel traffic movement;

- protection of the marine environment;

- supporting maritime security;

- supporting law enforcement;

- protection of adjacent communities and infrastructure.

## Benefits of VTS FOR the bridge team

VTS has real-time knowledge and provides the following benefits for the bridge team:

- information about coastal, port and inland waterways and infrastructures and its key and vulnerable locations, thereby improving maritime traffic efficiency.

- information about vessel activities, including vessel position and intention, etc., thereby improving the situational awareness between vessels.

- navigational assistance in difficult navigation conditions to ensure the safety of vessels.

- knowledge of conventional port operations, including vessel route and port interface activities, thereby reducing the pressure in route planning and help navigation officers fully understand the navigation environment in complex traffic situations.

- information concerning the coordination of medical assistance in emergency situations.

## VTS and Ship REPORTING SYSTEMS

Ship Reporting Systems (SRS) and reporting requirements are used to provide, gather or exchange information through radio reports. The information is used to provide data for many purposes including search and rescue, vessel traffic services, weather forecasting and prevention of marine pollution.

Ship Routeing and SRS are passive in that vessel traffic management is conducted through predetermined reporting requirements from the ship.

VTS is a proactive service, continuously monitoring vessel traffic and providing relevant information to vessels.  The navigating officer should be aware that:

- VTS, Traffic Separation Schemes (TSS) and SRS can be used independently or in combination.

- The SRS often provides data for VTS traffic images.

- VTS may be mandatory or voluntary.

- VTS may be implemented beyond the Territorial Sea.

### VTS PROCEDURES

There are specific procedures within VTS, including the provision of information, traffic organisation and navigation assistance.  More information on VTS Procedures can be found in IALA Guideline 1089.

### INFORMATION

A VTS provides information relevant at appropriate times and on request for the promulgated VTS area.  This information may be provided by digital means or through VHF voice communication.

Examples of when information may be provided by VTS:

- at fixed times (i.e. broadcast) or time -intervals;

- on demand (when requested by vessels);

- when deemed necessary by the VTS.

Examples of what information may be provided by VTS:

- Updated information about vessel traffic and routes;

- Navigational warnings;

- Meteorological and hydrological information;

- update on the status of aids to navigation;

### Traffic Organisation

Traffic organisation is a procedure that helps to prevent the development of dangerous maritime traffic situations and provides for the safe and efficient movement of vessel traffic within the VTS area.

While VHF voice communication may be the primary means of providing traffic organisation, any suitable communication method may also be used.

Traffic organisation should be provided when the VTS is authorised to do so.  Traffic organisation may be provided, for example, when:

- vessel movements need to be planned to prevent congestion or dangerous situations;

- an operating system of traffic clearances or sailing plans, or both, have been established;

- the allocation of space needs to be organised;

- special routes should be followed;

- speed limits should be observed;

- the VTS observes a developing situation and deems it necessary to interact and coordinate vessel traffic;

- nautical activities (e.g. sailing regattas) or marine works in progress (e.g. dredging or submarine cable-laying) may interfere with the flow of vessel movement.

### Navigational Assistance

Navigational assistance is a procedure that provides essential and timely navigational information to assist in the on board navigational decision-making process and to monitor its effects. It may also involve the provision of navigational advice and/or instruction.

Navigational assistance should be provided when the VTS is authorised to do so.   Navigational assistance may be provided when deemed necessary by the VTS, or upon request from the vessel. The procedure for the provision of navigational assistance includes, when time permits, confirmation that the vessel has the capability to respond to the guidance given.

Navigational assistance may be provided, for example, when a vessel:

- is in risk of grounding;

- is deviating from the VTS sailing/passage plan;

- is unsure of its position or unable to determine its position;

- is unsure of the route to its destination;

- needs assistance to find suitable anchoring position;

- experiences defects or deficiencies, such as navigation or manoeuvring equipment failure;

- experiences severe meteorological conditions (e.g. low visibility, strong winds);

- is facing a risk of collision with a fixed or floating object, or with another vessel;

- needs assistance to support the unexpected incapacity of a key member of the bridge team.

# ALLIED AND OTHER SERVICES

Allied services are services that are actively involved in the safe and efficient passage of the vessel through the VTS area, such as port authorities, pilotage organisations, tug companies, salvage, and customs authorities.

Other Services refer to services other than the allied services, which may use VTS data to more effectively undertake their work (e.g. ensuring local security or preventing illegal imports).  More information can be found in IALA Guideline 1102.

# COMMUNICATION BETWEEN THE NAVIGATION OFFICER AND THE VTS CENTRE

## TOOLS

Different communication tools may be used, including VHF radio.   VHF voice communications are generally performed on the channel specified by the VTS centre. Other tools include AIS binary messages, DSC, email and radiotelephony on MF/HF.

## LANGUAGE

Voice communication should be in English, using standard phraseology.  Voice communications may be carried out in the national language of the bridge team and the VTS operator.

## REQUIREMENTS OF COMMUNICATIONS

To ensure consistent and effective voice communications, it is important that:

- VTS VHF working channel is monitored when in the VTS area;

- communication should be concise and unambiguous.

It should be noted that information issued by the VTS centre does not affect the statutory responsibilities and duties of navigation officers.

## Voice Communication In VTS

To ensure the communication efficiency, navigation officers should make use of SMCP, including the use of message markers, and IALA Guideline 1132 on VHF voice communication.  Several tools are provided in the IALA Guideline to improve the effectiveness of communication.

Communication should:

- clearly indicate the identity of the vessel and the VTS centre;

- use the assigned frequencies/channels of the VTS;

- use the correct reporting procedures of the VTS;

- take into account the limitations within radio telecommunication, both when receiving and transmitting communication with a VTS.

# Training Activities to Increase awareness of VTS

VTS should be included in the mandatory training for Navigation Officers, and be reflected in examination and assessment. VTS should also be included in other types of training – such as Bridge Resource Management, voyage planning, and voice communication.

## VTS KNOWLEDGE IN THEORY TRAINING

VTS theory training may include the following:

- introduction of VTS, including functions, responsibilities, and procedures of VTS;

- requirements for transiting through the VTS area;

- benefits of VTS for the Bridge Team;

- communication methods and requirements between the Bridge Team and VTS;

- where to find information on VTS.

## VTS KNOWLEDGE IN PRACTICAL TRAINING

Vessels navigating in an area where VTS is provided should make use of these services. The introduction of VTS in practical training is helpful for Navigation Officers to be familiar with the operation of vessels in the VTS area. Examples of elements for training are provided in Table 1.

Table 1 - Examples of practical training elements

| Training activity | Specific items | Evaluation elements about VTS | Evaluation requirements |
| --- | --- | --- | --- |
| Passage plan | Develop a passage plan | - Obtain the necessary VTS information - Identify the relevant VTS interaction(s) - Include the VTS information in the passage plan | VTS influencing factors should be fully considered and reflected in the formulation of the passage plan. |
|  | Develop a contingency plan | - Consider possible incidents in the VTS area - Identify contingencies that take into account how VTS can assist |  |
| Actual operation through a certain VTS area | Entry to VTS area | - Contact VTS - Include VTS information in your decision making | In the VTS area, vessels should report to VTS as required, receive service, seek help from VTS centre, and cooperate with VTS centre to deal with incidents. |
|  | Passage through VTS area | - Keep smooth communication with VTS Centre - Maintain a high level of VTS awareness. |  |
|  | Incidents in the VTS area | - Contact VTS - Include VTS information in your decision making - Maintain communication with VTS |  |

## Equipment Training

### VHF

Effective VTS VHF communications directly contribute to navigational safety and efficiency. VHF communication training improves communication and coordination between vessels and the VTS centre. This training may also be carried out as simulation.

### ECDIS

The use of ECDIS makes it easy to identify VTS coverage, and location of the VTS reporting line / point.  Training on ECDIS should include reference to the use of ECDIS in a VTS area.  Examples of elements for training are provided in Table 2

Table 2 - Examples of VTS factors that can be considered in ECDIS practical training

| Training activity | Evaluation elements |
| --- | --- |
| Passage plan through the VTS area | - Clarify coverage of VTS and location of VTS reporting points; - Passage planning should include information on the VTS area; |
| Navigational Monitoring in VTS area | - Annotate VTS area and VTS reporting lines/points; |

## Considerations for Vessels in the VTS area

There are a number of factors that should be taken into account by the Bridge Team when a vessel is in a VTS area.  Examples of elements for training are provided in Table 3.

Table 3 - VTS factors that may be considered when a vessel is in a VTS area

| Training activity | Evaluation elements |
| --- | --- |
| Considerations – entering the VTS area | - Providing estimated arrival / departure from VTS area，including format, content and format of the estimated arrival / departure report; - Procedure to follow when entering VTS area，including establishing contact with the VTS Centre, communications and reporting requirements |
| Considerations – within the VTS area | - Passage through VTS area，including the mandatory requirements for ships in the VTS area, communication and reporting requirements; - Anchoring in VTS area, including the designated anchorage, communications and reporting requirements; - Berthing / unberthing in VTS area, including the arrival berth report, speed or other restrictions and reporting requirements. |
| Considerations – departing the VTS area | - Procedures to follow when departing the VTS area, including reporting requirements |

## Incidents and Emergencies

VTS supports safety of navigation and protects the environment through active monitoring and interaction with vessels.  VTS can assist with the coordination of response during incidents and emergencies, while ensuring the safety of the waterway.  Incidents or emergencies may include: search and rescue / man overboard; engine failure; steering gear failure; collision or grounding; encountering adverse weather that may affect safe voyage.

Examples of elements for training are provided in Table 4.

Table 4 -Examples of VTS training in Marine Simulator training

| Training activity | Evaluation elements |
| --- | --- |
| Incident / Emergency | - Report made to VTS in a timely manner. - Relevant information included, depending on the situation. Information may include: time and position of incident, nature of emergency, type of assistance required. - Communication with VTS is clear and concise - Further information requested by the VTS is provided - Request for navigational assistance is made (if required) - VTS is kept advised as the situation develop - VTS is advised when the incident / emergency has been resolved. |

## familiarisation activities

Familiarisation activities may include visiting a VTS Centre or arranging for a presentation from a representative of a VTS.  The objective of the familiarisation is to explain the purpose of VTS to the navigation officer, thereby enhancing VTS awareness.

To achieve the objective, familiarisation activities should be structured, and may use a workbook or other process.  The familiarisation activity may include:

- Equipment used

- Procedures followed

- Legal and regulatory requirements of the VTS

- How a VTS maintains awareness of the traffic in the VTS area

- How the mariner can work most effectively with the VTS

## STRENGTHENing PILOTS’ FAMILIARITY WITH VTS

The participation of the pilot as both a user and provider of information is of benefit to the VTS. Therefore, VTS familiarisation for pilots will strengthen their awareness of VTS, which will enhance the safety of navigation in the VTS area.

# ACRONYMS & DEFINITIONS

## ACRONYMS

AIS	Automatic Identification System

DSC	Digital Selective Calling

ECDIS	Electronic Chart Display And Information System

IALA	International Association of Marine Aids to Navigation and Lighthouse Authorities

IMO	International Maritime Organization

MF/HF	Medium Frequency / High Frequency (radio)

SMCP	Standard Marine Communication Phrases

SRS	Ship Reporting System

STCW	International Convention on Standards of Training, Certification and Watchkeeping for Seafarers

TSS	Traffic Separation Scheme

VHF 	Very High Frequency (radio)

VTS	Vessel Traffic Service

VTSO	Vessel Traffic Service Operator

## DEFINITIONS

The definition of terms used in this Guideline can be found in the International Dictionary of Marine Aids to Navigation (IALA Dictionary) at http://www.iala-aism.org/wiki/dicitonary

# REFERENCES

- IALA VTS MANUAL 2016

- IMO RESOLUTION A.857(20) GUIDELINES FOR VESSEL TRAFFIC SERVICES

- ITU RADIO REGULATIONS, VOLUME IVE, RECOMMENDATION ITU.R M.1171.0 AND SUBSEQUENT CHAPTERS

- IMO RESOLUTION A.918(22) IMO STANDARD MARINE COMMUNICATION PHRASES (SMCP)

- SOLAS 1974- INTERNATIONAL CONVENTION FOR THE SAFETY OF LIFE AT SEA CHAPTER V, REGULATION 12 VESSEL TRAFFIC SERVICES.

- STCW 1978 - International Convention on Standards of Training, Certification and Watchkeeping for Seafarers (STCW), 1978, as amended;

- IALA GUIDELINE 1089 PROVISION OF VTS SERVICES;

- IALA GUIDELINE 1071 ESTABLISHMENT OF A VESSEL TRAFFIC SERVICE BEYOND TERRITORIALSEAS;

- IALA GUIDELINE 1102 VTS INTERACTION WITH ALLIED OR OTHER SERVICES;

- IALA RECOMMENDATION V-120 VESSEL TRAFFIC SERVICES IN INLAND WATERS (EDITION2);

- IALA RECOMMENDATION V-127 Operational Procedures for Vessel Traffic Services (EDITION2);

- ADMIRALTY LISTOF RADIO SIGNALS, ALRS

- INTERNATIONAL CONVENTION ON MARITIME SEARCH AND RESCUE,1979

- IALA GUIDELINE 1132-VTS-VHF-VOICE-COMMUNICATION

- IALA GUIDELINE 1131 SETTING-AND-MEASURING-VTS-OBJECTIVE

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain

IMO Resolution A.851(20)

IALA Guideline 1071 – Establishment of a Vessel Traffic Service beyond Territorial Seas

IALA Guideline 1132 provides detailed information on effective VHF Voice Communication in VTS