NAVGUIDE 2023

Marine Aids to Navigation Manual

9th Edition

International Association of Marine Aids to Navigation and Lighthouse Authorities

Comments by Simon - Generally I am content with the format and running order of the new Navguide - There is no reference to AIS or AIS as an AtoN, This needs adding in chapter 5 Radionavigation - There is an editorial duplication of the Rhythmic character tables around page 49

Table of Contents

Foreword	6

Chapter	1	Introduction to IALA-AISM	7

1.1.	Purpose and Scope	7

1.2.	Background	7

1.3.	Membership	7

1.4.	General Assembly	8

1.5.	The Council	8

1.6.	IALA Secretariat (Minsu to expand on)	9

1.7.	Committees	9

1.8.	Policy Advisory Panel	9

1.9.	Legal Advisory Panel	10

1.10.	IALA World-Wide Academy	10

1.11.	IALA Model Courses and the IALA Accreditation Scheme	10

1.12.	Conferences, Symposia and Exhibitions	11

1.13.	WOrkshops and Seminars	11

1.14.	IALA Publications	11

1.15.	Related Organisations	12

Chapter	2	Concepts and accuracy of navigation	13

2.1.	Navigation Methods	13

2.2.	Accuracy Standards of Navigation	13

2.3.	Phases of Navigation	14

2.3.1.	Ocean Navigation	14

2.3.2.	Coastal Navigation	15

2.3.3.	Harbour Approach	15

2.3.4.	Restricted Waters	16

2.4.	Measurement Errors and Accuracy	16

2.4.1.	Measurement Error	16

2.4.2.	Accuracy	16

2.5.	Hydrographic Considerations	19

2.5.1.	Charts	19

2.5.2.	Datum	19

2.5.3.	Accuracy of Charts	23

2.5.4.	Charted Buoy Positions	24

Chapter	3	Aids to Navigation	26

3.1.	Obligation and regulatory compliance (ARM) (Text Required)	26

3.2.	International Criteria	26

3.3.	AtoN planning (Text Required)	27

3.4.	Operational Requirements	27

3.5.	Visual Aids to Navigation Design Theory	27

3.5.1.	Visual Perception	28

3.5.2.	Daymarks	34

3.5.3.	Lights	40

3.5.4.	Miscellaneous	67

3.5.5.	Light Sources	68

3.5.6.	Integrated Power System Lanterns	70

3.6.	Maritime Buoyage System and other Aids to Navigation	71

3.6.1.	Marking New Dangers	71

3.6.2.	Other Marks	71

3.6.3.	Radar Aids to Navigation	82

3.7.	Level of Service	83

3.7.1.	Benefits	83

3.7.2.	Components	83

3.7.3.	Layers of Service	83

3.8.	Risk Management	84

3.8.1.	IALA Risk Management Tools	85

3.8.2.	Risk Management Decision Process	86

3.8.3.	Risk Level and Acceptability	87

3.9.	Availability Objectives	87

3.9.1.	8.4.1 Calculation of Availability	88

3.9.2.	Definition and Comments on Terms	88

3.9.3.	IALA Categories for Traditional Aids to Navigation	89

3.9.4.	Availability and Continuity of Radionavigation Services	90

3.9.5.	Over and Under Achievement	91

3.10.	Reviews and Planning	91

3.10.1.	Reviews	91

3.10.2.	Strategic Plans	92

3.10.3.	Operational Plans	92

3.10.4.	Use of Geographic Information Systems in AtoN Planning	93

3.11.	Quality Management	93

3.11.1.	Performance Measurement	94

3.11.2.	International Standards	94

3.12.	Maintenance	95

3.12.1.	Guiding Principles for Maintenance	95

3.12.2.	Improving Efficiency	96

3.12.3.	Trends	97

3.12.4.	Maintenance Intervals	97

3.13.	Service Delivery	98

3.13.1.	Service Delivery Requirements	98

3.13.2.	Contracting Out	98

Chapter	4	AtoN design and delivery	99

4.1.	Visual signaling	99

4.2.	Range and performance	99

4.3.	Design, implementation and maintenance	99

4.4.	power supplies	99

4.4.1.	Types	99

4.5.	Electric - Renewable Energy Sources	100

4.5.1.	Solar Power (Photovoltaic cell	100

4.5.2.	Wind Energy	101

4.5.3.	Wave Energy	101

4.6.	Rechargeable Batteries	101

4.6.1.	Principal Types	101

4.6.2.	Primary Battery Cells	102

4.6.3.	Internal Combustion Engine/Generators	103

4.7.	Electrical Loads and Lightning Protection	103

4.7.1.	Electrical Loads	103

4.7.2.	Lightning Protection	104

4.8.	Non-Electric Energy Sources	104

4.9.	Environment	105

4.9.1.	Hazardous Materials	105

4.10.	Preservation of Historic Aids to Navigation	107

4.10.1.	Lens Size and Terminology	108

4.10.2.	Third Party Access to Aids To Navigation Sites	108

Chapter	5	Radionavigation services (PNT)	110

5.1.	Positioning, Navigation and Timing	110

5.2.	Satellite Positioning and timing	110

5.2.1.	Global Positioning System (GPS)	111

5.2.2.	Global Navigation Satellite System (GLONASS)	111

5.2.3.	BeiDou	111

5.2.4.	Galileo	112

5.2.5.	Regional systems	113

5.3.	Terrestrial positioning and timing	113

5.3.1.	Loran-C	113

5.3.2.	eLoran	113

5.3.3.	Ranging mode	114

5.4.	Augmentation Services	114

5.4.1.	Terrestrial augmentation systems	115

5.4.2.	Satellite Based Augmentation Systems (SBAS)	116

5.4.3.	Integrity options	118

5.5.	Radar Aids to Navigation	119

5.5.1.	Radar Reflectors	119

5.5.2.	Radar Target Enhancers	120

5.5.3.	Radar Beacons	120

5.5.4.	Radar Referenced Positioning	122

5.5.5.	Enhance Radar Positioning System	122

5.6.	Non-radio Positioning Systems	122

5.6.1.	Inertial systems	122

5.6.2.	ePelorus	123

Chapter	6	Vessel Traffic Services	124

6.1.	Introduction	124

6.2.	Background	124

6.3.	Definition of VTS	124

6.4.	Purpose of VTS	124

6.5.	Benefits of VTS	124

6.6.	IALA VTS Manual	124

Chapter	7	Training and certification	126

7.1.	Human Resources Challenges	126

7.1.1.	Source of Skills	126

Chapter	8	Digital communication technologies	127

8.1.	Digital communications systems	127

8.2.	IALA‘s Maritime Radiocommunications Plan	127

8.3.	AIS	127

8.4.	VDES	128

8.4.1.	Why VDES includes AIS?	128

8.4.2.	ASM component of VDES	129

8.4.3.	VDE-TER component of VDES	129

8.4.4.	VDE-SAT component of VDES	129

8.5.	R-Mode 	129

8.6.	LTE - M as a communications infrastructure	130

8.6.1.	Merits of LTE-M	130

8.7.	Capture a suite of low-power systems like LORA, etc.	130

8.8.	Autonomous Marine Radio Devices	130

8.8.1.	AMRD Group A	131

8.8.2.	AMRD Group B 	131

8.9.	8.9 Emerging digital technologies 	131

8.9.1.	What is 3GPP?	131

8.9.2.	IALA’s position on 3GPP 	132

8.9.3.	8.9.3 Digitalisation of marine VHF voice channels 	132

8.9.4.	Digital High Frequency radio	132

8.9.5.	How to assess new technologies?	132

8.9.6.	New technologies 	133

8.10.	Digital communications in VTS 	133

8.10.1.	Developments in automation on board ships – implications for AtoN and navigation services 	133

8.10.2.	IALA and GMDSS matters	133

Chapter	9	Information services	134

9.1.	Maritime Services	134

9.1.1.	Background	134

9.1.2.	Terminology and Directives	134

9.1.3.	Definition and Harmonisation of the Format and Structure of Maritime Services	135

9.1.4.	More information on Maritime Services and e-Navigation	135

9.1.5.	Operational Services	135

9.1.6.	Technical Services	136

9.1.7.	Establishing Technical Services	136

9.1.8.	Architectures for e-Navigation	136

9.1.9.	Maritime Resource Names	137

9.2.	IHO’s S-100 Universal Hydrographic Data Model	137

9.2.1.	S-100 Role in e-Navigation	137

9.2.2.	S-100 Dependent Product Specifications	138

9.2.3.	IALA and S-100 development	138

9.3.	Cyber security	139

Chapter	10	Other services	140

10.1.	e-Navigation	140

10.1.1.	Introduction	140

10.2.	IMO’s strategy for the development and implementation of e-Navigation	140

10.2.1.	The case for e-Navigation	140

10.2.2.	Definition	141

10.2.3.	What does the ‘e’ in e-Navigation stand for?	141

10.2.4.	Key elements	141

10.2.5.	e-Navigation solutions	142

10.2.6.	IALA’s Role	142

10.2.7.	IALA’s Strategic Vision 2018-2026	142

10.2.8.	IALA’s work on digital communication technologies, information services and e-navigation matters	143

10.2.9.	A Lay Person’s Description Of E-Navigation	143

10.2.10.	An example of an IALA National Member’s vision	145

10.2.11.	Testbeds	145

abbreviations	147

references	147

List of Tables

Table 1	Fixing Processes and Systems	18

Table 2	Levels Relevant to Marine Aids to Navigation in Coastal and Restricted Waters	22

Table 3	Zones of Confidence (IHO)	24

Table 4	Chart Scales (13??), Applications (14??) and Related Accuracy Considerations	25

Table 5	Contents of SOLAS Convention	27

Table 6	Graphical Range Table in Nautical Miles	35

Table 7	Typical Operational Range of Daymarks	38

Table 8	Guidance on the impact of background lighting and meteorological conditions on light intensity reguise to achieve a particular range	38

Table 9	Night and Day with Background.  This table is intended as guidance only.  It is not to be used for the Nominal Range Publication	40

Table 10	Photometric Units of Measurement	45

Table 11	Table 12 - Maximum Period for Rhythmic Characters of Aids to Navigation Lights	48

Table 12	Rhythmic Characters (4??) of the Lights in the IALA Maritime Buoyage System	57

Table 13	Timing of Astronomical Events	59

Table 14	IALA Conversion Table for Luminous Intensity and Nominal Range for Night Observations	59

Table 15	Night-time nominal range table (rounded off to the nearest nautical mile)	60

Table 16	Table 18 – Night and Day with Background.	61

Table 17	IALA Conversion Table for Luminous Intensity and Nominal Daytime Range	66

Table 18	Required Illuminance in Varying Meteorological Conditions	66

Table 19	Light source performance and other characteristics	69

Table 20	Availability Targets	80

Table 21	Comparison of Different Types of Aids to Navigation	85

Table 22	Availability Objectives by Category	90

Table 23	Power Sources for Operating Lighted Aids to Navigation.	100

Table 24	Silicon Solar Cell Technology	101

Table 25	Preferred terminology for the description of Racon Operating Frequencies	122

Table 26	Applicable cyber security standard to VTS, Ship and Maritime digital devices	140

List of Figures

Figure 1	IALA National Member	10

Figure 2	Nautical Chart (Instituto Hidrográfico)	21

Figure 3	Levelling or Vertical Control Datum (IHO)	23

Figure 4	Examples of GNSS Notes on Charts	24

Figure 5	Illustration of the Colour Zones on the 1931 CiE	32

Figure 6	IALA Chromaticity Areas of Ordinary Surface Colours - As plotted on the 1983 CIE Chromaticity diagram - courtesy of CIE	32

Figure 7	Effect of Exceeding Geographical Range	38

Figure 8	Spectral Sensitivity Distributions or V(λ) and V'(λ) Curves for the Human Observer.	44

Figure 9	Chromaticity Regions of the Recommended IALA Colours for Lights	45

Figure 10	CIE 1931 x,y Chromaticity Chart	46

Figure 11	Illustration of the Inverse Square Law Concept.	47

Figure 12	Daytime Luminous Range Diagram	65

Figure 13	Angle of Uncertainty	76

Figure 14	Sector Light Application	77

Figure 15	Various Applications for Sector Lights	78

Figure 16	Examples of Floating Aids	81

Figure 17	Risk Assessment and Risk Management Process	89

Figure 18	Risk Value Matrix	89

Figure 19	The Cost of Reliability	93

Figure 20	ISO 9001 Diagram on the Emphasis on Satisfying Customer Requirements	97

Figure 21	Existing and under definition SBAS systems (Source: GSA User Technology Report 2018)	119

Figure 22	A Racon (left) and a Radar Display (right) with and without the Racon character	123

Figure 23	VDES frequency usage	130

Figure 24	The Concept of a Client-Server based Maritime Service	145

Figure 25	The relationship between specifications of Maritime Services, Technical Services and data models in e-Navigation	146

Figure 26	The dataflow between Maritime Services, Technical Services and data exchange	146

# Foreword

The NAVGUIDE has been a signature document and information source for IALA members and users for many years. The 2018 edition of the Guide continues this proud tradition and sees it updated with the latest information and developments in the field of Marine Aids to Navigation technology and application.

The Guide is a product of four years collaboration by the world’s leading experts on Marine Aids to Navigation, produced by the four primary Committees, AtoN Requirements and Management (ARM), AtoN Engineering and Sustainability (ENG), e-Navigation (ENAV) and Vessel Traffic Services (VTS). The ARM Committee has the primary oversight for its editing and production.

The Guide plays an important role within the IALA information suite and is regarded as a primary source of information for Marine Aids to Navigation practitioners around the world, along with the IALA Standards, Recommendations, Guidelines, Manuals and other publications.

As one of the essential publications for Marine Aids to Navigation practitioners, the Guide also has recently been included as one of the key guidance documents in the Marine Aids to Navigation Management training syllabus for IALA’s World-Wide Academy.

The Guide has been translated into many languages and IALA encourages this practice and is keen to work with members to assist in this process, and the dissemination of the Guide into all regions of the world where English is not the primary working language.

The 2018 edition of the NAVGUIDE also sees its primary means of distribution being in a digital format which will be available on the website (www.iala-aism.org) along with all the other information sources available to our members and users of Marine Aids to Navigation. I encourage readers of this Guide to also consult the website for other information references that may assist you in your day-to-day work in the field of Marine Aids to Navigation. As always, IALA is receptive to feedback on how the Guide may further be developed for future editions and welcomes suggestions for improvements (contact@iala-aism.org).

I would like to thank the IALA membership for helping to produce this 2018 edition of the NAVGUIDE and reflect on the unique nature of IALA that allows professionals from around the world to contribute their expertise to assist the international maritime community in improving and harmonizing Marine Aids to Navigation.

Francis Zachariae

IALA Secretary-General

June 2023

| # Chapter | # Introduction to IALA-AISM |
| --- | --- |

## Purpose and Scope

The purpose of this manual is to assist Marine Aids to Navigation (AtoN) authorities in the harmonisation of AtoN by offering a first point of reference on all aspects of providing an AtoN service. The manual also references more detailed guidance from other IALA documents as well as related international organisations on specific topics.

## Background

Shipping is a global industry that is regulated through various organisations. Nations have recognized that it is both effective and appropriate to regulate and manage shipping on an international basis.

IALA is a non-profit and non-political international technical association devoted to the harmonisation of Marine Aids to Navigation. IALA was formed in 1957 to provide a framework for Marine Aids to Navigation authorities, manufacturers and consultants from all parts of the world. A process has started to change IALA from that of a non-governmental organisation (NGO) to that of an international intergovernmental organisation (IGO). Elevating IALA’s status to that of an IGO will make it a peer to organisations like the International Maritime Organization (IMO) and the International Hydrographic Organization (IHO), thereby strengthening existing cooperation. As well, it will allow for a broader participation in IALA by States, at government level. This will assist IALA’s aim to promote the greatest possible uniformity in aids to navigation.

The aim of IALA is to foster the safe, economic and efficient movement of vessels through improvement and harmonisation of Marine Aids to Navigation worldwide, and by other appropriate means, for the benefit of the maritime community and the protection of the environment.

The Strategic Vision of IALA for 2018-2026 sets out two main goals:

Goal 1	Marine Aids to Navigation are developed and harmonized through international cooperation and the provision of standards.

Goal 2	All coastal states have contributed to a sustainable and efficient global network of Marine Aids to Navigation through capacity building and the sharing of expertise.

## Membership

IALA has three main types of members:

National membership: Applicable to the national authority of any country that is legally responsible for the provision, management, maintenance or operation of Marine Aids to Navigation.

Associate membership: Applicable to any other service, organisation or scientific agency concerned with Marine Aids to Navigation or related matters.

Industrial membership: Applicable to manufacturers and distributors of Marine Aids to Navigation equipment for sale, or organisations providing aids to marine navigation services or technical advice under contract.

There is also a fourth type of membership: Honorary membership which may be conferred for life by the Council to any individual who is considered to have made an important contribution to the work of IALA.

(the shaded countries are IALA Members)

IALA National Member

## General Assembly

The General Assembly brings together members from all the membership categories every four years and usually convenes in conjunction with the four-yearly Conference.

National members attending the General Assembly are entitled to:

determine the broad policy direction;

elect members to the Council;

decide upon changes to the Constitution; and

approve Standards.

Associate and Industrial members are entitled to attend the General Assembly and take part in the discussions, but they do not have the right to vote.

## The Council

IALA is administered by a Council of up to twenty-one elected and three non-elected Councillors. The elected positions are determined by a ballot of all National members attending a General Assembly. Only one National member from any country may be elected to the Council and there is a general aim to draw Councillors from different parts of the world to achieve a broad representation on the Council.

The Council, among other things:

implements the overall policy of IALA as defined by its aims or by the General Assembly;

decides membership matters;

establishes committees relevant and/or facilitates other such bodies as may be appropriate and their term of reference.

determines rules of procedure for committees and other such bodies as may be appropriate and their terms of reference;

approves recommendations, guidelines, manuals and other appropriate papers;

approves submissions to other organisations;

decides the venue and the year of the next IALA Conferences and symposiums;

establishes rules for participation in IALA Conferences and Symposiums;

convenes General Assemblies;

approves the annual budget and accounts;

determines the rate of contributions;

decides upon the location of the Headquarters and registered office of IALA;

may authorise the purchase, sale, renting or letting of property and the granting and obtaining of loans whether or not secured by mortgage, required for the running of the association;

may grant any power of attorney as required.

## IALA Secretariat (Minsu to expand on)

IALA has a permanent Secretariat based at their headquarters in France, where Technical Operations, Administration and Finance work under the direction of the Secretary-General and Deputy Secretary-General.

## Committees

Committees are established by the Council to study a range of issues, as determined by the General Assembly, including to prepare draft Recommendations and Guidelines, and draft submissions to other international organisations. A Committee may also be asked to provide continuous monitoring of elements of subjects that could influence decisions concerning the provision of Marine Aids to Navigation, including Vessel Traffic Services.

Committees meet regularly, normally twice per year. The programmes for the Committees are for a four-year period, usually from one Conference to the next. Each Committee has a Chair and Vice-Chair, and normally comprises a number of working groups.

Draft Recommendations and Guidelines and other documents created by the Committees may address topics relating to management, operations, engineering, emerging technologies and training. They are forwarded to the Council for approval before being published on the website.

All members are eligible to participate in the Committees, which are presently:

the AtoN Requirements and Management Committee (ARM);

the AtoN Engineering and Sustainability Committee (ENG);

the e-Navigation, Information Services and Communications (ENAV); and

the Vessel Traffic Services Committee (VTS).

## Policy Advisory Panel

The Policy Advisory Panel (PAP) is a group that comprises the Secretary-General, Deputy Secretary-General, members of the Secretariat, the Chairs and Vice-Chairs of each Committee, the Chair of the Legal Advisory Panel, the Dean of the World-Wide Academy, a representative of the Industrial Members Committee and special advisors as necessary. The Panel meets at least once a year to review the work being done by the Committees.

The role of the PAP is to:

consider and advise the Council and the Secretariat on policy and strategy matters concerning the development and harmonisation of Marine Aids to Navigation systems, with specific emphasis on the Strategic Vision;

co-ordinate the work of the Committees and provide a forum for Committee Chairs to share progress, challenges and operations of the Committees to provide a collegiate delivery of the various work plans with the Secretariat; and

carry out such other work as the Council may from time to time require.

## Legal Advisory Panel

The Legal Advisory Panel (LAP) comprises National members with interest in legal affairs, representatives of relevant international organisations, and experts.

Its scope shall include:

providing legal support to the Council as required;

responding to issues and concerns that may be raised through the IALA Secretariat;

responding to requests from IALA Committees and other IALA bodies for legal advice;

providing IALA with information on legal issues that result, or may result from, providing guidance to the membership in the provision of Marine Aids to Navigation services;

preparing draft documentation/guidelines on items of common concern;

identifying where external legal advice may be needed and assisting with the preparation of requests/briefs for such advice, as appropriate; and

reviewing, updating, advising and reporting to the Council on the IALA Risk Registers;

providing a forum to discuss legal matters of common interest.

## IALA World-Wide Academy

The IALA World-Wide Academy is the vehicle by which IALA delivers education, training and capacity building.

The Academy is an integral, but independently funded part of IALA. It focuses on the second goal of the Strategic Vision, namely that “All coastal states have contributed to a sustainable and efficient global network of Marine Aids to Navigation through capacity building and the sharing of expertise”.

The functions of the Academy are:

education and training;

capacity-building; and

research and development.

The Academy is dedicated to assisting coastal states with continuous and sustainable improvement of their AtoN services, including VTS. While its education and training activities are intended for all coastal states, capacity-building activities are aimed at those states which require assistance in fulfilling their international obligations related to Marine Aids to Navigation.

The research and development activities of the Academy are aimed at identifying topical knowledge gaps within the IALA domain, and encouraging research in these areas worldwide. The Academy maintains an overview of its activities in a Master Plan which can be found on the Academy website – www.academy.iala aism.org.

## IALA Model Courses and the IALA Accreditation Scheme

IALA has developed a number of model courses covering the contents of its Standards, Recommendations and Guidelines.

The model courses, which may be found on the Academy website, are primarily intended for adoption by training organisations which have been accredited by a competent authority, normally an IALA national member.

These organisations are called Accredited Training Organisations (ATOs), and the accreditation scheme aims at ensuring the quality of their delivery of the model courses.  There are now several ATOs around the world, delivering both AtoN and VTS related model courses.

The following guidance documents related to training.

Recommendation R0103(V-103) - Standards for Training and Certification of VTS Personnel

Recommendation R0141(E-141) - Training and Certification of Marine Aids to Navigation Personnel

Recommendation O-149 - Accreditation of Training Organisations

Guideline G1014 – The Accreditation and Approval Process of VTS Training Courses

Guideline G1100 – The Accreditation and Approval Process for Aids to Navigation Personnel Training

## Conferences, Symposia and Exhibitions

IALA holds a general Marine Aids to Navigation Conference every four years. Conferences may be attended by members and also by non-member Marine Aids to Navigation authorities.

Papers, presentations and discussions address a wide range of Marine Aids to Navigation issues. The work of IALA over the previous four years is also presented.  All members are invited to submit papers for discussion.

The Industrial Member’s Committee traditionally organizes an Industrial Exhibition in conjunction with the Conference.

In addition, IALA may hold a Symposium on a specific topic of interest to the members every four years, two years separated from the IALA Conference. The Symposium also hosts an industrial exhibition.

## WOrkshops and Seminars

IALA convenes Workshops and Seminars to address topics that may arise during the work term.

Workshops are special meetings convened for the purpose of making maximum use of the technical expertise of participants to further the work of IALA on a specified subject or topic. The workshops also enable skills and understanding of new techniques to be learned by detailed lectures combined with simulation or similar “hands on” methods.

Seminars are meetings of specialists on a specified subject or topic convened for the purpose of consultations by means of the presentation of papers followed by learned discussion.

## IALA Publications

IALA produces a comprehensive set of guidance documents that have the objective of facilitating a uniform approach to Marine Aids to Navigation worldwide.

There are five such documents in a logical hierarchy, as follows.

Standards: IALA Standards form a framework, implementation of which by all coastal states will harmonize Marine Aids to Navigation worldwide. IALA standards cover technology and services and are non‐mandatory.

There are seven standards, covering topics as follows:

Error! Hyperlink reference not valid.

Recommendations: IALA Recommendations specify what practices shall be carried out in order to comply with a Recommendation and may be referenced, in full or in part, in an IALA Standard.

Guidelines: IALA Guidelines describe how to implement practices normally specified in a Recommendation.

Manuals: IALA Manuals provide an overall view of a wide subject area. The IALA Dictionary is considered a Manual.

Model Courses: IALA Model Courses are training documents that define the level of training and knowledge needed to reach levels of competence defined by IALA.

IALA has published a Technical Documents Catalogue which provides further information on the above as well as links to the appropriate documents. This is available on the IALA website – www.iala-aism.org.

IALA publications are governed by a set of principles including:

Usability – The system should be as intuitive as possible.

Visibility - Publications conform to the IALA corporate image, with colour coding.

Validity - The date of issue and date of amendment/edition should be clearly visible to ensure that users have the most up-to-date information.

Availability – Publications are available to download in electronic form from the IALA website at no charge.

- Other documents available from IALA on request include:

Basic Documents, being Constitution, General Regulations, Policy & Procedures – Committees, and Constitution and Bye-Laws of the Industrial Members’ Committee.

Conference proceedings.

Reports of symposia, seminars, committee meetings, workshops and other events.

IALA Bulletin, the house magazine which appears twice per year.

## Related Organisations

IALA works in close cooperation with the International Maritime Organization (IMO) and the International Hydrographic Organization (IHO) and has cooperation agreements with a number of other international maritime organisations.

| # Chapter | # Concepts and accuracy of navigation |
| --- | --- |

## Navigation Methods

IMO Resolution A.915(22) defines navigation as "the process of planning, recording and controlling the movement of a craft from one place to another."

The IALA Dictionary defines Navigation as:

- The art or science of determining the position or course of a ship or aircraft by means of observations on board, whereby difficulties and dangers are avoided and a desired destination is reached as quickly and safely as possible.

- The process of planning, controlling and recording the movement of a craft from one place to another.

The principal methods of marine navigation are briefly described as follows:

Terrestrial Navigation:

Navigation using visual, radar and, (if appropriate) depth sounding observations of identifiable, conspicuous features, objects and marks to determine position.

Celestial or Astronomical Navigation:

Navigation using observation of celestial bodies (i.e. sun, moon, planets and stars) to determine position.

Dead Reckoning:

Navigation based on speed, elapsed time and direction from a known position. The term was originally based on the course steered and the speed through the water, however, the expression may also refer to positions determined by the use of the course and speed expected to be made good over the ground, thus making an estimated allowance for disturbing elements such as current and wind. A position that is determined by this method is generally called an estimated position.

Radionavigation:

Navigation uses radio signals to determine a position or a line of position (e.g. eLORAN, GNSS, DGNSS etc.).

It is recommended that whenever possible, the reliance on a single method of determining position be avoided.

## Accuracy Standards of Navigation

IMO Resolution A.915(22) established accuracy standards for maritime navigation.

Table 1 presents the relevant standards adopted in Appendices 2 and 3 of IMO Resolution A.915(22). Appendix 2 includes the requirement for an accuracy of 10 m on ocean navigation, while IMO Resolution A.1046(27) mentions that “Where a radionavigation system is used to assist in the navigation of ships in ocean waters, the system should provide positional information with an error not greater than 100 m with a probability of 95%."

Minimum Maritime User Requirements

Minimum Maritime User Requirements

## Phases of Navigation

Typically, navigation is divided into three phases: Ocean navigation, coastal navigation and restricted waters navigation. Some documents have introduced other phases, namely harbour approaches, port and inland waterways navigation.

The harbour approach phase is an aspect of the restricted waters phase, but will be treated separately in this manual.

Port and inland waterway navigation are two aspects of restricted waters navigation and will not be dealt with separately in this manual, as the precautions and measures required for restricted waters navigation also apply to these waters.

### Ocean Navigation

In this phase, the vessel is typically:

beyond the continental shelf (200 metres in depth) and more than 50 nm from land;

in waters where position fixing by visual reference to land, charted fixed offshore structures, or to fixed or floating Marine Aids to Navigation, is not practical;

sufficiently far from land masses and traffic areas that the hazards of shallow water and of collision are comparatively small.

Although the IMO has adopted stricter accuracy requirements (see Table 1) the minimum navigational requirements for the Ocean Phase are considered to be a predictable accuracy of 2 to 4 nm, combined with a desired fix interval of 15 minutes or less (maximum 2 hour fix interval). The required accuracy in the Ocean Phase is based on providing the ship with the capability to correctly plan the approach to land or restricted waters.

The economic efficiency aspects of shipping (e.g. transit time and fuel consumption) are enhanced by the availability of a continuous and accurate position fixing system that enables a vessel to follow the shortest safe route with precision.

### Coastal Navigation

In this phase, the vessel is typically:

within 50 nm from shore or the limit of the continental shelf (200 meters in depth);

in waters contiguous to major land masses or island groups, where transoceanic routes tend to converge towards destination areas and where inter-port traffic exists in patterns that are essentially parallel to coastlines.

The vessel may encounter:

ship reporting systems (SRS) and coastal vessel traffic services (VTS);

offshore exploitation and scientific activity on the continental shelf;

fishing and recreational boating activity.

The Coastal Phase is considered to exist when the distance from shore makes it feasible to navigate by means of visual observations, radar and, if appropriate, by depth (echo) sounder. As with the Ocean Phase, the distances from land can be varied to take account of the smaller vessels and local geographical characteristics.

Although the IMO has adopted stricter accuracy requirements (see Table 1), international studies have established that the minimum navigation requirements for commercial trading vessels operating in the Coastal Phase is a navigation system capable of providing fix positioning to an accuracy of 0.25 nm, combined with a desired fix interval of 2 minutes to a maximum of 15 minutes.

More specialised maritime operations within the Coastal Phase may require navigational systems capable of higher repeatable accuracy, either permanently or on an occasional basis. These operations can include marine scientific research, hydrographic surveying, commercial fishing, petroleum or mineral exploration, and Search and Rescue (SAR).

It is not always practical, given the manning of most vessels, to plot fixes at the desired interval of 2 minutes on a chart in the traditional way. GNSS (GPS, DGPS and in the future, in some areas, enhanced Loran (eLORAN)) provide a means of exceeding the IMO Coastal Phase requirements for positional accuracy and fix rates when integrated with Electronic Chart Systems (ECS) or Electronic Chart Display Information System (ECDIS) technology.

### Harbour Approach

This phase represents the transition from coastal to harbour navigation. In this phase the:

vessel moves from the relatively unrestricted waters of the coastal phase into more restricted and more heavily used waters near and/or within the entrance to a bay, river, or harbour;

navigator is confronted with a requirement for more frequent position fixing and manoeuvring the vessel to avoid collision with other traffic and grounding dangers;

- The ship will generally be within:

the coverage areas of Marine Aids to Navigation of varying complexity (including lights, racons, leading lights and sector lights);

pilotage areas;

the boundaries of SRS and VTS.

Safety of navigation issues that arise during the Harbour Approach Phase impose more stringent requirements on positional accuracy, fix rates and other real-time navigational information than those required during the Coastal Phase.

GNSS (GPS, DGPS and in the future, in some areas, enhanced Loran (eLORAN) provide a means of achieving the Harbour Approach requirements for high positional accuracy and fix rates at better than 10-second intervals when integrated with Electronic Chart Systems (ECS) and the Electronic Chart Display Information System (ECDIS) technology.

### Restricted Waters

While similar to the Harbour Approach Phase, in the proximity to dangers and the limitations on freedom of manoeuvre, a Restricted Waters Phase can also develop during a coastal navigation phase, such as in various straits around the world.

The Pilot or Master of a large vessel in restricted waters must direct its movement with great accuracy and precision to avoid grounding in shallow water, striking submerged dangers or colliding with other craft in a congested channel. If a large vessel finds itself in an emerging navigational situation with no options to alter course or stop, it may be forced to navigate to limits measured to within a few metres in order to avoid an accident.

Requirements for the safety of navigation in the Restricted Waters Phase make it desirable for navigation systems to provide:

accurate verification of position almost continuously;

information depicting any tendency for the vessel to deviate from its intended track;

instantaneous indication of the direction in which the ship should be steered to maintain the intended course.

These requirements are not easily achievable through the use of visual aids and ships’ radar alone, but as with Harbour Approach navigation, they can be achieved with a combination of GNSS (and in the future, enhanced Loran (eLORAN)) and Electronic Chart Systems (ECS) or Electronic Chart Display Information System (ECDIS) technology.

## Measurement Errors and Accuracy

Good practice in both navigation and Marine Aids to Navigation design dictates that an indication of the error or uncertainty in measuring a parameter or in obtaining a position fix should be reported along with the derived result.

### Measurement Error

The measurement error is defined as the difference between the true value and the measured value. In general, three types of errors are recognised:

Systematic Errors: Also known as fixed or bias errors. They are errors that persist and are related to the inherent accuracy of the equipment, or result from incorrectly calibrated equipment. This type of error can to some extent be foreseen and compensated for.

Random Errors: Cause readings to take random values either side of some mean value. They may be due to the observer/operator, or the equipment, and are revealed by taking repeated readings. This type of error can neither be foreseen, nor totally compensated for.

Faults and Mistakes: Errors of this type can be reduced by appropriate training and by following defined procedures.

### Accuracy

In a process where a number of measurements are taken, the term accuracy refers to the degree of conformity between the measured parameter at a given time and its true parameter at that time. The term parameter includes: position, coordinates, velocity, time, angle, etc.

For navigational purposes, four types of accuracy can be defined:

Absolute Accuracy (Geodetic or Geographic Accuracy): The accuracy of a position with respect to the geographic or geodetic coordinates of the Earth.

Predictable Accuracy: The accuracy with which a position can be defined when the predicted errors have been taken into account. It therefore depends on the state of knowledge of the error sources.

Relative or Relational Accuracy: The accuracy with which a user can determine position relative to that of another user of the same navigation system at the same time.

Repeatable Accuracy: The accuracy with which a user can return to a position whose coordinates have been measured at a previous time using uncorrelated measurements from the same navigation system.

For general navigation, the Absolute and Predictable Accuracy are the principal concerns.

Repeatable Accuracy: This is of interest to fishermen, the offshore oil and gas industry, ships making regular trips into an area of restricted waters and authorities when positioning floating aids.

Accuracy of a Position Fix: A minimum of two lines of position (LOP) is necessary to determine a position at sea. Since there is an error associated with each LOP, the position fix has a two-dimensional error. There are many ways of analysing the error boundary; however, the radial position error relative to the true position, taken at the 95% probability level, is the preferred method.

Navigational Position Fixing Measurements: Table 2 shows the typical accuracy (95% probability) achieved using common navigational instruments or techniques.

Fixing Processes and Systems

## Hydrographic Considerations

### Charts

The IMO definition[1] of a nautical chart or nautical publication is a special-purpose map or book, or a specially compiled database from which such a map or book is derived, that is issued officially by or on the authority of a Government, authorised Hydrographic Office or other relevant government institution and is designed to meet the requirements of marine navigation.

Nautical Chart (Instituto Hidrográfico)

Nautical charts provide a graphical representation of a plane surface of a section of the Earth’s sea surface constructed to include known dangers and Marine Aids to Navigation.

The principal international organization on charting matters is the International Hydrographic Organization (IHO). A principal aim of the IHO is to ensure that all the world's seas, oceans and navigable waters are surveyed and charted.

The mission of the IHO is to create a global environment in which States provide adequate and timely hydrographic data, products and services and ensure their widest possible use. The vision of the IHO is to be the authoritative worldwide hydrographic body which actively engages all coastal and interested States to advance maritime safety and efficiency and which supports the protection and sustainable use of the marine environment. IMO is the body responsible for determining international standards for the quality of hydrographic surveys and chart production.

### Datum

In its simplest form, a datum is an assumed or defined starting point from which measurements are taken.

A more complex example of a datum is a Geodetic Datum used in the mathematical representation of the Earth’s surface. Many different data (plural of datum) have been devised over time to define the size and shape of the Earth and the origin and orientation of coordinate systems for chart and mapping applications. These have evolved from the consideration of a spherical Earth, through to the geoid and ellipsoidal models, and also the planar projections used for charts and maps.

The geoid model considers the Earth’s surface to be defined as the equipotential surface[2] that would be assumed by the sea level in the absence of tides, currents, water density variations and atmospheric effects.

A further approximation uses an ellipsoid, which is a smooth mathematical surface, to give a best-fit match of the geoid. Early ellipsoid models were developed to suit the mapping and charting of local regions or countries. However, they would not necessarily provide a satisfactory solution in other parts of the world. Some nautical charts still carry a legend referring to a local datum, for instance, Ellipsoid Hayford or International – Datum Potsdam, Paris or Lisbon.

Chart Datum

Chart datum is defined as the datum or plane of reference to which all charted depths and drying heights are related. It is relevant to a localised area and is a level that the tide will not frequently fall below. It is usually defined in terms of Lowest Astronomical Tide (and in some cases by Indian Spring Low Water).

Levelling Datum or Vertical Control Datum

These are generic terms for levelling surfaces that are used to determine levels or elevations. Using nautical charts as an example:

water depths are measured from Chart Datum to the seabed;

elevations of land masses and man-made features are referenced to either Mean High Water Springs (where there are predominantly semi-diurnal tides) or Mean Higher High Water (where there are predominantly diurnal tides)[3];

clearance heights for bridges are generally referenced to Highest Astronomical Tide.

Chart Datum Issues

Until satellite navigation became commonly used, nautical charts were generally produced to the local and national datum.

The now widely used GNSS positioning uses an Earth centred datum referred to as World Geodetic System[4] 1984 (WGS-84) which is considered to be the best compromise for representing the whole of Earth’s surface.

Generally, WGS-84 is the geodetic system associated with the differential correction information broadcast by maritime GNSS stations using the ITU-R M.823/1 signal format.

IHO S-4 Part B Section 200 recommends that all countries that issue national navigational charts should base these on the WGS 84 geodetic system. For many countries, this simple objective represents a formidable workload and will take a number of years to achieve. Consequently, many nautical charts will continue to refer to data other than WGS-84 and discrepancies of several hundred metres can exist between a GNSS derived position and the charted position.

During this transitional period, it is important for navigators and other persons using charts to:

be aware of the datum applicable to the chart in use;

include the applicable reference datum when communicating a measured position;

determine whether or not a satellite derived position can be directly plotted onto a chart. In some cases, a chart will include information for adjusting a satellite derived position to align to the chart datum;

be aware that some GNSS receivers have the facility to automatically convert (and display) WGS- 84 positions into other geodetic coordinate systems. The user should be aware of the settings that have been applied to the receiver.

Examples of the styles of note found on some charts[5] are shown in Figure 5.

Levels Relevant to Marine Aids to Navigation in Coastal and Restricted Waters

Levelling or Vertical Control Datum (IHO)

Examples of GNSS Notes on Charts

Zones of Confidence (IHO)

### Accuracy of Charts

At a national level, it is important that the Authorities responsible for Marine Aids to Navigation and hydrographic services work together to ensure that both the network and the mix of Marine Aids to Navigation provided, and the available charts are appropriate for mariners to navigate safely.

Source quality indication is provided for official ENC charts (ZOC, zones of confidence). Source quality indication might also be indicated on the back of some national paper charts.

Mariners should always consider this information, as official charts (both electronic and paper) might be based on old measurements of poor or unknown quality.

The accuracy requirements for general navigation can be related to the scale of the chart necessary for each part of the passage which in turn will be determined by the local conditions and type of vessel.

Chart scales with the corresponding accuracy requirements recommended by IHO and the equivalent dimension of a 0.5 mm dot on a chart are found in Table 5.

Chart Scales (13??), Applications (14??) and Related Accuracy Considerations

### Charted Buoy Positions

No reliance can be placed on floating aids, always maintaining their exact positions. Buoys should, therefore, be regarded with caution and not as infallible navigating marks, especially when in exposed positions. A ship should always, when possible, navigate by bearings of fixed objects or angles between them and not by buoys.

Notes

- SOLAS Chapter V, Regulation 2.

- These have the same potential gravity at each point.

- It should be noted that elevations of land features on maps are generally referenced to Mean Sea Level.

- The World geodetic system (WGS) is a consistent set of parameters for describing the size and shape of the earth, positions of a network of points with respect to the centre of mass of the Earth, transformations from major geodetic data and the potential of the Earth. (IMO Resolution A.860(20)).

- Examples taken from Australian Charts.

- The allocation of a ZOC indicates that particular data meets minimum criteria for position and depth accuracy and seafloor coverage defined in this Table. ZOC categories reflect a charting standard and not just a hydrographic survey standard. Depth and position accuracies specified for each ZOC category refer to the errors of the final depicted soundings and include not only survey errors but also other errors introduced in the chart production process. [Note: the rest of footnote 1 does not apply to paper charts and is therefore omitted from S-4].

- Position Accuracy of depicted soundings at 95% CI (2.45 sigma) with respect to the given datum. It is the cumulative error and includes survey, transformation and digitizing errors etc. Position accuracy need not be rigorously computed for ZOCs B, C and D but may be estimated based on type of equipment, calibration regime, historical accuracy etc.

- Depth accuracy of depicted soundings = a + (bxd)/100 at 95% CI (2.00 sigma), where d = depth in metres at the critical depth. Depth

- accuracy need not be rigorously computed for ZOCs B, C and D but may be estimated based on type of equipment, calibration regime, historical accuracy etc.

- Typical Survey Characteristics - These descriptions should be seen as indicative examples only.

- Significant seafloor features are defined as those rising above depicted depths by more than:

Depth	Significant Feature

a.	<40 m	2 m

b.	>40 m	10% depth

A full seafloor search indicates that a systematic survey was conducted using detection systems, depth measurement systems, procedures, and trained personnel designed to detect and measure depths on significant seafloor features. Significant features are included on the chart as scale allows. It is impossible to guarantee that no significant feature could remain undetected, and significant features may have become present in the area since the time of the survey.

- Controlled, systematic surveys (ZOC A1, A2 and B) - surveys comprising planned survey lines, on a geodetic datum that can be transformed to WGS 84.

- Modern survey echo-sounder - a high precision single beam depth measuring equipment, generally including all survey echo-sounders designed post 1970.

- The chart scale is generally referenced to a particular latitude e.g. 1:3000,000 at latitude 27° 15'S

- This information may be helpful in assessing the practical accuracy requirements for laying buoy moorings.

| # Chapter | # Aids to Navigation |
| --- | --- |

A Marine Aid to Navigation (AtoN) is a device, system or service, external to vessels, designed and operated to enhance safe and efficient navigation of individual vessels and/or vessel traffic. A marine aid to navigation should not be confused with a navigational aid. A navigational aid is an instrument, device, chart, etc., carried on board a vessel for the purpose of assisting navigation.

This chapter describes the major types of visual and other physical aids to navigation in current use and provides comments on the application and performance of the various technologies.

Vessel Traffic Services (VTS), are also considered by IALA as satisfying the definition of an aid to navigation. However these are covered in separate chapters due to their increasingly significant role in contributing to navigation safety.

The concept of e-Navigation has recently gained significant momentum and a framework is being developed under the auspices of the IMO. IALA has been requested by the IMO to develop the shore-based aspects of the conceptual framework and systems architecture for e-Navigation. Chapter 4 of the NAVGUIDE covers e-Navigation. Radionavigation systems form a key element of the e-Navigation infrastructure and are therefore covered in Chapter 4.

## Obligation and regulatory compliance (ARM) (Text Required)

## International Criteria

The International Convention for the Safety of Life at Sea, 1974 (as amended), or SOLAS is one of the oldest international conventions and originates from a conference held in London in 1914 to address aspects of safety at sea following the sinking of the White Star liner Titanic in 1912. Since then, there have been four other SOLAS Conventions, the latest being the 1974 version that came into force in 1980.

The SOLAS Convention is administered by the United Nations through the International Maritime Organization (IMO). The 1974 Convention (as amended) is divided into twelve chapters and within these are a series of regulations. The contents[1] are outlined in Table 37.

Contents of SOLAS Convention

| Chapter | Contents |
| --- | --- |
| Chapter I | General Provisions |
| Chapter II-1 | Construction - Structure, subdivision and stability, machinery and electrical installations |
| Chapter II-2 | Construction - Fire protection, fire detection and fire extinction |
| Chapter III | Life-saving appliances and arrangements |
| Chapter IV | Radiocommunications |
| Chapter V | Safety of navigation |
| Chapter VI | Carriage of cargoes and oil fuels |
| Chapter VII | Carriage of dangerous goods |
| Chapter VIII | Nuclear ships |
| Chapter IX | Management for the safe operation of ships |
| Chapter X | Safety measures for high-speed craft |

SOLAS Chapter V

SOLAS Chapter V, and Regulations 12[2] and 13 in particular, define the obligations on Contracting Governments to provide vessel traffic services and aids to navigation and related information. These Regulations define the primary roles of IALA National Members.

In December 2000, the 73rd session of the IMO Maritime Safety Committee (MSC) adopted a completely revised SOLAS Chapter V on Safety of Navigation that came into force on 1 July 2002.

In October 2005, IMO adopted IMO Resolution A.973(24) and A.974(24), outlining the IMO Member State Voluntary Audit Scheme which includes all aspects of SOLAS, including Chapter V, Regulations 12 and 13.

SOLAS Chapter V, Regulation 13 - Establishment and operation of aids to navigation states:

Each Contracting Government undertakes to provide, as it deems practical and necessary either individually or in co-operation with other Contracting Governments, such aids to navigation as the volume of traffic justifies and the degree of risk requires.

In order to obtain the greatest possible uniformity in aids to navigation, Contracting Governments undertake to take into account the international recommendations and guidelines (Reference is made to IALA) when establishing such aids.

Contracting Governments undertake to arrange for information relating to aids to navigation to be made available to all concerned. Changes in the transmissions of position-fixing systems which could adversely affect the performance of receivers fitted in ships shall be avoided as far as possible and only be effected after timely and adequate notice has been promulgated.

To satisfy the obligations of Regulation 13, the Contracting Government has to make assessments on:

- whether or not to provide particular types of aids to navigation;

- the type, number and location of aids to navigation;

- what information services are necessary to adequately inform all concerned - principally mariners.

## AtoN planning (Text Required)

## Operational Requirements

The primary objective of marine aids to navigation is to mitigate transit risks and to promote the safe, economic, and efficient movement of vessels by assisting navigators with determining their position, a safe course, and warning them of dangers and obstructions, especially when used in conjunction with other aids within visual, audio, or radar range of the mariner.

## Visual Aids to Navigation Design Theory

Visual marks for navigation can be either natural or man-made objects. They include structures specifically designed as short range aids to navigation, as well as conspicuous features such as headlands, mountain-tops, rocks, trees, church-towers, minarets, monuments, chimneys, etc. Short range aids to navigation can be fitted with a light if navigation at night is required, or left unlit if daytime navigation is sufficient.

Navigation at night is possible, to a limited extent, if the unlit aids are provided with:

a radar reflector, and the navigating vessel has a radar;

retro-reflecting material, and the vessel has a searchlight. This approach would generally only be acceptable for small boats operating in safe waterways and with the advantage of local knowledge.

Visual aids to navigation are purpose-built facilities that communicate information to a trained observer on a vessel for the purpose of assisting the task of navigation.

The communication process is referred to as marine signalling.

Common examples of visual aids to navigation include lighthouses, beacons, leading (range) lines, buoys (lit or unlit), lightvessels, daymarks (dayboards) and traffic signals.

The effectiveness of a visual aid to navigation is determined by factors such as:

type and characteristics of the aid provided;

location of the aid relative to typical routes taken by vessels;

distance (range) of the aid from the observer;

atmospheric conditions;

contrast relative to background conditions (conspicuity);

the reliability and availability of the aid.

Visual aids to navigation can be distinguished by a wide range of characteristics such as:

type; shape; size; colour; names, retro-reflective features; letters and numbers;

lit/unlit; signal character; light intensity; sectors; inclusion of subsidiary aids;

fixed structure; floating platform; construction materials;

location; elevation; relationship to other aids to navigation and observable features.

Refer to IALA publications:

- Recommendation R0130(O-130) on Categorisation and Availability Objectives for Short Range Aids to Navigation;

- Guideline G1035 on Availability and Reliability of Aids to Navigation.

Photo - Courtesy of Danish Maritime Authority

### Visual Perception

When a navigator approaches a visual AtoN, for instance, a buoy, the first thing the navigator will recognise is the shape or colour of the buoy, depending on the viewing conditions.

The navigator will subsequently recognise the topmark and finally its numbers or letters. Thus, the process of identifying a visual AtoN goes through three different stages of perception:

Detection - The observer is aware of an object. The navigator sees an object, but will usually not be able to deduce its shape or colour and will not know that it is an AtoN.

Recognition - The observer is aware that the object is an AtoN.

Identification - The observer is aware which AtoN the object is. At this distance, the navigator can perfectly discern the type of mark it is.

Visual perception requires some understanding of a number of factors that impact the ability of an observer to see an AtoN. These factors are summarised below.

#### Signal Colours

IALA has made recommendations on colours for lighted aids to navigation and for surface colours for visual signals on aids to navigation.

Marine aid to navigation signal lights uses a five-colour system comprising white, red, green, yellow and blue, as defined in IALA Recommendation R0200(E-200) Part 1. The colour regions defined in the IALA recommendation are derived from those given in the International Commission on Illumination (CIE) Standard S 004/E 2001 Colours of Light Signals. [1], with small variances in some of the boundaries.

Recommended surface colours for visual signals on aids to navigation are as follows:

Ordinary colours should be limited to white, black, red, green, yellow or blue[2].

Orange and fluorescent red, yellow, green or orange may be used for special purposes requiring high conspicuity. Refer to IALA publications:

Recommendation R0106(E-106) for the Use of Retroreflecting Material on Aids to Navigation Marks within the IALA Maritime Buoyage System;

Recommendation R0108(E-108) for the Surface Colours Used as Visual Signals on Aids to Navigation;

Guideline G1015 on Painting Aids to Navigation Buoys (including reference to the practical guide on surface colours).

Guideline G1134 Surface Colours used as visual signals on AtoN

Guideline G1145 Application of Retroreflecting material on AtoN

Guideline G1006 On Plastic Buoys

The CIE standard on the measurement of colours (colorimetry) is based on three reference colours (i.e. a tri-stimulus system) that in varying combination can generate the visual spectrum of colours. A particular colour function is described by the symbols; X, Y and Z that represent the proportions of the reference colours.

Using ratios of the tri-stimulus values, such that: X + Y + Z = 1, colours can be defined in terms of chromaticity using just the x = X / (X+Y+Z) and y = Y / (X+Y+Z) values. The advantage of this arrangement is that colours can be mapped on a two-dimensional chromaticity diagram.

CIE colour standards for marine signalling can be depicted as areas on the chromaticity diagram. These areas are defined by boundaries expressed as functions of x and y (equations).

Figure 6 - Illustration of the Colour Zones on the 1931 CIE Chromaticity Diagram - Please note that the colour rendering is only indicative and should not be taken as fully accurate

Illustration of the Colour Zones on the 1931 CiE

Chromaticity diagram - Please note that the colour rendering is only indicative and should not be taken as fully accurate.

IALA Chromaticity Areas of Ordinary Surface Colours - As plotted on the 1983 CIE Chromaticity diagram - courtesy of CIE

If the chromaticity co-ordinates of a coloured light, filter material or a paint product are known, its acceptability for marine signalling applications can easily be determined.

Further information on surface colours can be found in IALA Recommendation R0108(E-108) on the surface colours used as visual signals on aids to navigation. Information for light signal colours is shown in IALA Recommendation R0201(E-200-1) for the colours for light signals on aids to navigation. For further details on this issue, refer to CIE S 004/E-2001 Colours of Light Signals. [3]

#### Meteorological Visibility

Meteorological visibility (V) is defined as the greatest distance at which a black object of suitable dimensions can be seen and recognised by day against the horizon sky, or, in the case of night observations, could be seen if the general illumination were raised to the normal daytime level. It is usually expressed in kilometres or nautical miles.

Photo Courtesy of Australian Maritime Safety Authority

#### Atmospheric Transmissivity

The atmospheric transmissivity (T) is defined as the transmittance, or proportion of light from a source, that remains after passing through a specified distance through the atmosphere at sea level. This is expressed as a ratio. But since the atmosphere is not uniform over the observing distances of most visual aids, a representative value is used:

typically, the atmospheric transmissivity is taken as T = 0.74 over one nautical mile, meaning that 26% of the light is "lost" every one nautical mile due to the atmospheric transmissivity;

a figure of T = 0.86 is occasionally used in regions where the atmosphere is very clear.

A number of countries collect data on atmospheric transmissivity for different parts of their coastline. This enables the luminous range of lights to be:

calculated more precisely;

better matched to local conditions and user requirements.

#### Atmospheric Refraction

This phenomenon results from the normal decrease in atmospheric density from the Earth’s surface to the stratosphere. This causes light rays that are directed obliquely through the atmosphere to be refracted (or bent) towards the Earth in accordance with Snell’s Law.

#### Contrast

The ability to detect differences in luminance between an object and an otherwise uniform background is a basic visual requirement and is used to define the term contrast. It is represented by the equation:

The contrast at which an object can be detected against a given background for 50% of the time, is called the threshold contrast. For meteorological observations, a higher threshold must be used to ensure that the object is recognised.

A contrast value of 0.05 has been adopted as the basis for the measurement of meteorological optical range.

#### Use of Binoculars

While it is generally assumed that observations will be made with the naked eye, mariners will quite often use binoculars. This can allow:

a light being observed, or the characteristics resolved, at a greater luminous range than with the naked eye;

a limited improvement in the sensitivity of leading lights;

about a 30% improvement in the detectable difference from a given bearing;

the identification of a light operating against background lighting conditions.

Generally, the most suitable binoculars for use at sea are considered to be the type with a magnifying power of 7 and an objective lens of 50 mm at night, and 10 x 50 binoculars by day.

#### Geographical Range

This is the greatest distance at which an object or a light source could be seen under conditions of perfect visibility, as limited only by the curvature of the earth, by refraction of the atmosphere, and by the elevation of the observer and the object or light.

As the observer moves further away from the object or light source, there will come a point where the object or light source is obscured by the Earth. This is illustrated in Figure 8.

Geographical Range Table in Nautical Miles

| Geographical Range (NM) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Observer eye height (m) | Elevation of light (m) |  |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 |
| 1 | 2.0 | 4.1 | 4.9 | 5.5 | 6.1 | 6.6 | 7.0 |
| 2 | 2.9 | 4.9 | 5.7 | 6.4 | 6.9 | 7.4 | 7.8 |
| 5 | 4.5 | 6.6 | 7.4 | 8.1 | 8.6 | 9.1 | 9.5 |
| 10 | 6.4 | 8.5 | 9.3 | 9.9 | 10.5 | 11.0 | 11.4 |
| 20 | 9.1 | 11.1 | 12.0 | 12.6 | 13.1 | 13.6 | 14.1 |
| 30 | 11.1 | 13.2 | 14.0 | 14.6 | 15.2 | 15.7 | 16.1 |  |

The values in Table 8 are derived from the formula:

The factor 2.03 accounts for refraction in the atmosphere. Climatic variations around the world may lead to different factors being recommended. The typical range of factors is 2.03 to 2.12.

### Daymarks

A daymark is a structure with defined shape and colour with the purpose of assisting with marine navigation during daylight.

A number of factors impact the suitability and effectiveness of a structure as a daymark, and these are considered below.

#### Visibility of a Mark

The visibility of a mark is affected by one or more of the following factors:

observing distance (range);

curvature of the Earth;

atmospheric refraction;

atmospheric transmissivity (meteorological visibility);

height of the aid above sea level;

observer’s visual perception;

observer’s height of eye;

observing conditions (day or night);

conspicuity of the mark (shape, size, colour, reflectance, and the properties of any retroreflecting material);

contrast (type of background such as lighting, vegetation, snow, etc.);

mark lit or unlit;

intensity and character.

Refer to IALA publications:

Recommendation R0106(E-106) for the Use of Retroreflecting Material on Aids to Navigation Marks within the IALA Maritime Buoyage System.

Recommendation R0108(E-108) for the Surface Colours Used as Visual Signals on Aids to Navigation.

Guideline G1094 Daymarks for AtoN.

#### Range of a Visual Mark

The range of an aid to navigation can broadly be defined as the distance at which the observer’s receiver can detect and resolve the signal. In the case of visual marks the observer’s receivers are his/her eyes. This broad definition of range leads to a number of more specific definitions that are described below.

Effect of Exceeding Geographical Range

Daymark size

The size of a dayboard should be determined for the maximum useful viewing distance and minimum visibility conditions. Daymarks used on leading lines are typically rectangular with the long side vertical. The aspect ratio for the rectangle is commonly 2:1 (height = 2 x width).

The typical operational range of daymarks under different visibility conditions is shown in Table 17.

Typical Operational Range of Daymarks

Guidance on the impact of background lighting and meteorological conditions on light intensity reguise to achieve a particular range

Night and Day with Background.  This table is intended as guidance only.  It is not to be used for the Nominal Range Publication

### Lights

#### Photometry of Marine Aids to Navigation Signal Lights

"The science of observing visible light is called photometry, and provides a basis for creating standards for marine aids to navigation signal lights. Generally, electromagnetic radiation is described by its wavelength in metres and its power in Watts."

However, the study of photometry and the use of lights for signal application has necessitated a parallel set of units to be developed to account for the physiological aspects of how the human eye evaluates a light source, as shown in Table 9.

The spectral sensitivity of the human eye (or the response of the eye to different coloured light) has been evaluated in tests of large numbers of people. The results have been presented as a standard spectral sensitivity distribution or V(λ) curve for photopic (daytime) observers and V’(λ) for scotopic (night-time) observers.

Spectral Sensitivity Distributions or V(λ) and V'(λ) Curves for the Human Observer.

Chromaticity Regions of the Recommended IALA Colours for Lights

Colorimetric Measurement of Lights (Colour Measurement)

The measurement of the colour of lights is described in CIE Publication No 15.2 (1986) Colorimetry. There are two main types of instruments for measuring the colour of a light: one is a colorimeter; the other is a spectroradiometer.

Colorimeters usually comprise three photoreceptors, each with a coloured filter. Each filter is matched to the response of one of the three eye receptors, red green and blue and such devices are called ‘tristimulus’ colorimeters. The colorimeter gives three outputs, one for each filtered receptor, and these correspond to the X, Y and Z functions of the human observer.

Spectroradiometers consist of a monochromator and photoreceptor. The monochromator splits the light into individual wavelengths (much like a prism makes a rainbow) and is usually rotated in steps past an exit slit. The photoreceptor, behind the exit slit, measures different sections of the spectrum as the monochromator is rotated. The output is a series of readings enabling a graph of power against wavelength to be displayed. Results may then be weighted with the X, Y and Z functions of the human observer to produce colour information.

Stepping monochromators of the type described previously are fairly slow in operation and are not suitable for measuring flashing lights. Tristimulus colorimeters, on the other hand, enable much faster measurements of colour. New types of spectradiometer, known as ‘array-based’ spectroradiometers, are now available. Instead of a single photoreceptor and a rotating monochromator, a fixed monochromator has its output directed at an array of charge-coupled devices (CCDs). Such devices are capable of much faster measurement speeds than stepping monochromators.

Recent developments in colour measurement have resulted from the technology of digital cameras. ‘Imaging photometers’, as they are known, are little more than calibrated digital cameras, some with tristimulus filtering. They are capable of fast measurement of a whole scene, making them useful for work outside the laboratory.

In summary:

Tristimulus colorimeters are fast, however cheaper models suffer errors when measuring narrowband light sources such as LEDs;

Stepping monochromators are expensive and slow but very accurate;

Array-based spectroradiometers are fast, relatively inexpensive, but can suffer with stray light errors;

Imaging photometers are expensive and not very accurate, but can record a whole scene and not just one light.

Resultant data from colour measurements are usually displayed on a chromaticity chart, developed by the CIE in 1931. The three X, Y, Z values are reduced to two x, y values as shown in Figure 11.

CIE 1931 x, y Chromaticity Chart

Units of Measurement

Photometric Units of Measurement

| Term | Description | Unit | Abbreviation |
| --- | --- | --- | --- |
| Luminous flux | This is the total light emitted from the source (i.e. lamp) The peak sensitivity of the human eye occurs at about 555 nanometres, a wavelength that corresponds to green. At this wavelength, the photometric equivalent of one watt is defined as 683 lumens. | lumens | lm |
| Luminous intensity | This is the part of the luminous flux in a particular direction. Also expressed as the luminous flux per solid angle (or steradian 15) | candela | cd |
| Luminance | This is the portion of the luminous flux emitted in a specific direction by the | candelas per square meter and | cd/m2 |
| (Brilliance) | surface area of a luminous body. Luminance is an important term for rating the brightness impression of light sources and illuminated objects. | also as candelas per square centimetre | cd/cm2 |
| Illuminance | This is the density of the luminous flux incident on a surface. It is the quotient of the luminous flux by the area of the surface when the surface is uniformly illuminated | lux (lumens/square metre) | lx |
| Luminous efficacy | This is the ratio of luminous output to radiometric output of a light source. It can also be applied to the efficiency with which electrical power is converted to visible radiation. | lumens per watt of electrical power consumed |  |
| Colour | This related to the temperature of a black body. As a body heats up, it goes through | Kelvin | ºK |
| Temperature | A series of different colours from red through yellow and white, to blue white. The colour appearance of a tungsten filament lamp is similar to a black body at the same temperature. | ?? | ?? |
| Colour | Characterises the colour rendering quality of the light from a lamp. It is the same for all incandescent lamps by definition and equal to the maximum value of 100. |  | CRI |
| Rendering index | ?? | ?? | ?? |

Threshold of Illuminance

In physical terms, the threshold of illuminance is the lowest level of illuminance from a point source of light, against a given background level of luminance, that causes a visual response at the eye. For visual signalling applications, the threshold of illuminance (E) is taken to be 0.2 μlux at the eye of the observer. In the case of leading lights of limited range and with a high level of shore illumination, the above figures may be found too low. It is recommended that to observe the relative position of the lights easily and to derive the maximum possible accuracy from leading and sector lights, it is generally necessary to have a minimum illuminance of 1 μlux at the eye of the observer.

This condition is to be met at the outer limits of the useful segment for the minimum meteorological visibility under which the leading lights are to be used. IALA Recommendation R0200 (E-200) series and its associated guidance provides the method of designing AtoN lights for use in daylight. For lights on floating aids, care must be taken to provide adequate vertical divergence so that the minimum illuminance at the observer is maintained as the floating aid rolls and pitches.

Luminous Intensity

The luminous intensity of a navigation light is directly proportional to the luminance of the light source. The luminance of a light source depends on its size and the luminous flux in the direction of observation. The vertical and horizontal divergence is also directly proportional to the size of the light source.

Candela (cd) is the measurement unit used to quantify the luminous intensity of a light.

Inverse Square Law

Light emitted from a source radiates out in all directions. For a point source, the wave fronts of light can be imagined to generate a series of spherical surfaces. As shown in Figure 12, the further the light travels from the source, the greater is the surface area of the sphere and consequently, the lower the illuminance. Since illuminance is measured in lumens per square metre, and the surface area of a sphere increases in proportion to the square of the radius, the illuminance decreases in proportion to the square of the distance from the source. The decline in illuminance with distance is described as an inverse-square law.

Illustration of the Inverse Square Law Concept.

Allard’s Law

The illuminance of a light source reaching an observer’s eye determines whether the light is seen. The relationship between the illuminance produced at the observer’s eye, the luminous intensity of the light source, the distance to the observer and the atmospheric transmissivity is given by the relationships shown in Allard’s Law:

Allard’s law applies only when the luminance of the background is small compared to the average illuminance of the light.

Refer to IALA publication:

- Recommendation R0202(E-200-2) on Marine Signal Lights - Part 2 - Calculation, Definition and Notation of Luminous Range

#### Rhythms and Characters

IALA has produced a recommendation on the characters for light on aids to navigation. The tables of classifications and specifications of aid to navigation characters are provided in Table 10.

Refer to IALA publication:

- Recommendation R0110(E-110) for the rhythmic characters of lights on aids to navigation.

- Guideline G1116 Selection of Rhythmic characters & synchronisation of Lights for AtoN

- The Rhythmic Characters of Lights are provided in Table 11.

- Rhythmic Characters[4] of the Lights in the IALA Maritime Buoyage System[5] Maximum Periods for Light Characters

| Character Class | Maximum period |
| --- | --- |
| Isophase light | 12 s |
| Single-occulting light Single-flashing light Group very quick light | 15 s |
| Group-occulting light of two eclipses Long-flashing light Group-flashing light of two flashes Group quick light | 20 s |
| Group-occulting light of three or more eclipses Group-flashing light of three or more flashes Composite group-flashing light Morse Code light | 30 s |

Refer to IALA publication:

Recommendation R0110(E-110) for the rhythmic characters of lights on aids to navigation.

Maximum Period for Rhythmic Characters of Aids to Navigation Lights

|  | Class | Abbreviation | General description | IALA Specification |  |  | Particular use in the IALA Maritime Buoyage System |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | FIXED LIGHT | F | A light showing continuously and steadily. | A single fixed light should be used with care because it may not be recognized as an aid to navigation light. |  |  | A single fixed light shall not be used. |
| 2 | OCCULTING LIGHT |  | A light in which the total duration of light in a period is longer than the total duration of darkness and the intervals of darkness (eclipses) are usually of equal duration. | A light in which the total duration of light in a period is clearly longer than the total duration of darkness and all the eclipses are of equal duration. |  |  |  |
| 2.1 | Single-occulting light | Oc | An occulting light in which an eclipse is regularly repeated | The duration of an appearance of light should not be less than three times the duration of an eclipse. The period should not be less than 2 s |  |  | A single‑occulting White light indicates a safe‑water mark. |
|  |  |  |  |  | Example: d = 1 s; l = 3 s; p = 4 s | l ≥ 3 d p ≥ 2 s |  |
| 2.2 | Group‑occulting light | Oc(#) e.g. Oc(2) | An occulting light in which a group of eclipses, specified in number, is regularly repeated. | The appearances of light between the eclipses in a group are of equal duration, and this duration is clearly shorter than the duration of the appearance of light between successive groups. The number of eclipses in a group should not be greater than four in general, and should be five only as an exception. The duration of an appearance of light within a group should not be less than the duration of an eclipse. The duration of an appearance of light between groups should not be less than three times the duration of an appearance of light within a group. In a group of two eclipses, the duration of an eclipse together with the duration of the appearance of light within a, group should not be less than 1 s. In a group of three or more eclipses, the duration of an eclipse together with the duration of an appearance of light within the group should not be less than 2 s. |  |  | A group‑occulting Yellow light indicates a special mark. |
|  |  |  |  | Oc(2) | Example: d = 1 s; l = 2 s; c = 3 s; l’ = 6 s; p = 10 s | l ≥ d l' ≥ 3 l c ≥ 1 s |  |
| 2.3 | Composite group‑occulting light | Oc(#+#) e.g. Oc(2 + 1) | A light similar to a group‑occulting light except that successive groups in a period have different numbers of eclipses. | This class of light character is not recommended because it is difficult to recognize. |  |  |  |
|  |  |  |  | Oc(2+1) | Example: d = 1 s; l = 1 s; c = 2 s; l’ = 3 s; l’’ = 9 s; p = 16 s | l ≥ d l’ ≥ 3 l l’’ ≥ l’ c ≥ 1 s |  |

|  | Class | Abbreviation | General description | IALA Specification |  |  |  |  |  | Particular use in the IALA Maritime Buoyage System |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | ISOPHASE LIGHT | Iso | A light in which all the durations of light and darkness are clearly equal. | The period should never be less than 2 s, but preferably it should not be less than 4 s in order to reduce the risk of confusion with occulting or flashing lights of similar periods. |  |  |  |  |  | An isophase White light indicates a safe‑water mark. |
|  |  |  |  |  |  | Example: l = d = 2 s; p = 4 s |  |  | l = d p ≥ 2 s |  |
| 4 | FLASHING LIGHT |  | A light in which the total duration of light in a period is shorter than the total duration of darkness and the appearances of light (flashes) are usually of equal duration. | A light in which the total duration of light in a period is clearly shorter than the total duration of darkness and all the flashes are of equal duration. |  |  |  |  |  |  |
| 4.1 | Single flashing light | Fl | A flashing light in which a flash is regularly repeated (at a rate of less than 50 flashes per minute). | The duration of the interval of darkness (eclipse) between two successive flashes should not be less than three times the duration of a flash. The period should not be less than 2 s (or not less than 2.5 s in those countries where a quick rate of 50 flashes per minute is used). |  |  |  |  |  | A single‑flashing Yellow light indicates a special mark. |
|  |  |  |  |  |  | Example: l = 1 s; d = 3 s; p = 4 s |  |  | d ≥ 3 l p ≥ 2 s |  |
| 4.2 | Long flashing light | LFI | A single‑flashing light in which an appearance of light of not less than 2 s duration (long flash) [1] is regularly repeated. |  |  | Example: l = 2 s; d = 8 s; p = 10 s |  |  | l ≥ 2 s d ≥ 3 l | A long‑flashing White light with a period of 10 s indicates a safe‑water mark. |
| 4.3 | Group flashing light | Fl(#) e.g. Fl(2) | A flashing light in which a group of flashes, specified in number, is regularly repeated. | The eclipses between the flashes in a group are of equal duration, and this duration is clearly shorter than the duration of the eclipse between successive groups. The number of flashes in a group should not be greater than five in general, and should be six only as an exception. The duration of an eclipse within a group should not be less than the duration of a flash. The duration of an eclipse between groups should not be less than three times the duration of an eclipse within a group. In a group of two flashes, the duration of a flash together with the duration of the eclipse within the group should not be less than 1 s. In a group of three or more flashes, the duration of a flash together with the duration of an eclipse within a group should not be less than 2 s (or not less than 2.5 s in those countries where a quick rate of 50 flashes per minute is used). |  |  |  |  |  | A group‑flashing White light with a group of two flashes, in a period of 5 s or 10 s, indicates an isolated‑danger mark. A group‑flashing Yellow light with a group of four, five or (exceptionally) six flashes indicates a special mark |
|  |  |  |  | Fl(2) |  | Example: l = 1 s; d = 2 s; c = 3 s; d’ = 6 s; p = 10 s |  |  | d ≥ l d’ ≥ 3 d c ≥ 1 s |  |
| 4.4 | Composite group‑flashing light | Fl(# + #) e.g. Fl(2 + 1) | A light similar to a group‑flashing light except that successive groups in a period have different numbers of flashes. | Light characters should be restricted to (2 + 1) flashes in general, and should be (3 + 1) flashes only as an exception. |  |  |  |  |  | A composite group‑flashing Red or Green light with a group of (2 + 1) flashes indicates a modified lateral (preferred‑channel) mark. A composite group‑flashing Yellow light indicates a special mark. |
|  |  |  |  | Fl(2+1) |  |  | Example: l = 1 s; d = 1 s; c = 2 s; d’ = 3 s; d’’ = 9 s; p = 16 s |  | d ≥ l d’ ≥ 3 d d’’ ≥ d’ c ≥ 1 s |  |
| 5 | QUICK LIGHT |  | A light in which flashes are repeated at a rate of not less than 50 flashes per minute but less than 80 flashes per minute. | A light in which identical flashes are repeated at the rate of 60 flashes per minute. |  |  |  |  |  |  |
| 5.1 | Continuous quick light | Q | A quick light in which a flash is regularly repeated. |  | Example: l = d = 0.5 s; p = 1 s |  |  | d ≥ l l s ≤ p ≤ 1.2 s |  | A continuous quick White light indicates a north cardinal mark. |

[1]	The term “long flash”, which is used in the descriptions of the long-flashing light and of the light characters reserved for south cardinal marks, means an appearance of light of not less than 2 seconds duration. The term “short flash” is not commonly used and does not appear in the Classification. If an Authority requires discrimination between two flashing lights that only differ in having flashes of different durations, then the longer flash should be described as “long flash” and be of not less than 2 seconds duration, and the shorter flash may be described as “short flash” and should be of not more rhythmic character of such a light is than one third of the duration of the longer flash.

|  | Class | Abbreviation | General description | IALA Specification |  |  |  | Particular use in the IALA Maritime Buoyage System |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5.2 | Group quick light | Q(#) e.g. Q(3) e.g. Q(9) e.g. Q(6) + LFl | A quick light in which a specified group of flashes is regularly repeated. | The number of flashes in a group should be three or nine. An exceptional light character is reserved for use in the IALA Maritime Buoyage System to indicate a south cardinal mark. |  |  |  |  |
|  |  |  |  | Q(3) | Example: l = d = 0.5 s; c = 1 s ; d’ = 7.5 s; p = 10 s |  | d ≥ l d’>d 1 s ≤ c ≤ 1.2 s | A group quick White light with a group of three flashes, in a period of 10 s, indicates an east cardinal mark. |
|  |  |  |  | Q(9) | Example: l = d = 0.5 s; c = 1 s; d’ = 6.5 s; p = 15 s |  | d ≥ l d’>d 1 s ≤ c ≤ 1.2 s | A group quick White light with a group of nine flashes, in a period of 15 s, indicates a west cardinal mark. |
|  |  |  |  | Q(6)+LFl | Example: l = d = 0.5 s; c = 1 s; l’ = 2 s; d’ = 7 s; p = 15 s |  | l’ ≥ 2 s d ≥ l d' ≥ 3 l’ 1 s ≤ c ≤ 1.2 s | A group quick White light with a group of six flashes followed by a long flash of not less than 2 s duration, in a period of 15 s, indicates a south cardinal mark. |
| 6 | VERY QUICK LIGHT |  | A light in which flashes are repeated at a rate of not less than 80 flashes per minute but less than 160 flashes per minute. | A light in which identical flashes are repeated at the rate of 120 flashes per minute. |  |  |  |  |
| 6.1 | Continuous very quick light | VQ | A very quick light in which a flash is regularly repeated. |  | Example: l = d = 0.25 s; p = 0.5 s |  | d ≥ l 0.5 s ≤ p ≤ 1.6 s | A continuous very quick White light indicates a north cardinal mark. |
| 6.2 | Group very quick light | VQ(#) e.g. VQ(3) e.g. VQ(9) e.g. VQ(6)+LFl | A very quick light in which a specified group of flashes is regularly repeated. | The number of flashes in a group should be three or nine. An exceptional light character is reserved for use in the IALA Maritime Buoyage System to indicate a south cardinal mark. |  |  |  |  |
|  |  |  |  | VQ(3) |  | Example: l = d = 0.25 s; c = 0.5 s; d’ = 3.75 s; p = 5 s | d ≥ l d' ≥ 1.5 s 0.5 s ≤ c ≤ 0.6 s | A group very quick White light with a group of three flashes, in a period of 5 s, indicates an east cardinal mark. |
|  |  |  |  | VQ(9) |  | Example: l = d = 0.25 s; c = 0.5 s; d’ = 5.75 s; p = 10 s | d ≥ l d' ≥ 1.5 s 0.5 s ≤ c ≤ 0.6 s | A group very quick White light with a group of nine flashes, in a period of 10 s, indicates a west cardinal mark. |
|  |  |  |  | VQ(6)+LFl |  | Example: l = d = 0.25 s; c = 0.5 s; l’ = 2 s; d’ = 5 s; p = 10 s | l’ ≥ 2 s d ≥ l d' ≥ 1.5 l’ 0.5 s ≤ c ≤ 0.6 s | A group very quick White light with a group of six flashes followed by a long flash of not less than 2 s duration, in a period of 10 s, indicates a south cardinal mark. |
| 7 | ULTRA QUICK LIGHT |  | A light in which flashes are repeated at a rate of not less than 160 flashes per minute and not more than 300 flashes per minute. | A light in which identical flashes are repeated at the rate of 240 flashes per minute. |  |  |  |  |
| 7.1 | Continuous ultra quick light | UQ | An ultra quick light in which a flash is regularly repeated. |  |  |  |  |  |

|  | Class | Abbreviation | General description | IALA Specification |  |  |  | Particular use in the IALA Maritime Buoyage System |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | MORSE CODE LIGHT | Mo(#) e.g. Mo(A) | A light in which appearances of light of two clearly different durations are grouped to represent a character or characters in the Morse Code. | Light characters should be restricted to a single letter in the Morse Code in general, and should be two letters only as an exception. The duration of a "dot" should be about 0.5 s, and the duration of a "dash" should not be less than three times the duration of a "dot". |  |  |  | A Morse Code White light with the single character "A" indicates a safe‑water mark. A Morse Code Yellow light, but not with either of the single characters "A" or "U"*, indicates a special mark. |
|  |  |  |  | Mo(A) |  | Example: l = 0.5 s; d = 0.5 s; l’ = 1.5 s; d’ = 4.5 s; p = 7 s | l = 0.5 s l' ≥ 3 l d ≥ l |  |
| 9 | FIXED AND FLASHING LIGHT | F+ relevant character abbreviation, e.g. FFl, FIso | A light in which a low intensity fixed light phase is combined with a flashing phase of higher luminous intensity compliant with preceding classes of rhythmic characters in this table. | Implementation of an FFl rhythmic character is shown below. Other combinations may be implemented as necessary. |  |  |  |  |
|  |  |  |  |  | Example: l = 1 s; d = 3 s; p = 4 s |  | l ≤ 1 s d ≥ 3 l |  |

|  | Class | Abbreviation | General description | IALA Specification |  |  | Particular use in the IALA Maritime Buoyage System |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | ALTERNATING LIGHT | Al## e.g. AlWR | A light showing different colours alternately. | This class of light character should be used with care, and efforts should be made to ensure that the different colours appear equally visible to an observer. |  |  |  |
|  |  |  |  | AlWR | Example: l = d = 2 s; p = 4 s | l @ d |  |
| 11 | OCCULTING ALTERNATING LIGHT | OcAl | A light showing different colours alternately and a light in which the total duration of light in an period is longer than the total duration of darkness and the intervals of darkness (eclipses) are of equal duration. | OcAlBY | This class of light is particular to the use of Emergency Wreck Marking, and efforts should be made to ensure that the different colours appear equally visible to an observer. |  | An Occulting-Alternating Blue and Yellow light indicates an Emergency Wreck Marking Buoy mark. |
| 12 | FLICKERING LIGHT | Flkr | A character containing flashes with a perceivable flicker. The flickering flash must have a frequency of 5 Hz and a duty cycle of 50 %. | Flkr(3)+Fl(2)Y | This class of light is particular to the use of Mobile AtoN, and is a distinctive character incorporating flicker and non-flicker flashes. The first three flashes comprises of a flicker flash with a flicker frequency of 5 Hz. This is followed by two non-flicker flashes. |  | This character is used solely for the Mobile AtoN application. The light colour is always yellow. |

Headings to match Style

Table captions?

Headings to match Style

Table captions?

Rhythmic Characters (4??) of the Lights in the IALA Maritime Buoyage System

- Rhythmic characters of the lights in the IALA Maritime Buoyage System

| Mark | Rhythmic character of the light | Remarks and further recommendations |
| --- | --- | --- |
| LATERAL | All recommended classes of rhythmic character[1], but a composite group flashing light with a group of (2+1) flashes is solely assigned to modified lateral marks that indicate preferred channels. | Only the colours Red and Green are used. |
| Modified lateral (preferred channel) | Composite group flashing light with a group of (2+1) flashes, in a period of not more than 16 s. |  |
| CARDINAL |  | Only the colour White is used. |
| North cardinal | (a) Continuous very quick light. (b) Continuous quick light. |  |
| East cardinal | (a) Group very quick light with a group of three flashes, in a period of 5 s. (b) Group quick light with a group of three flashes, in a period of 10 s. |  |
| South cardinal | (a) Group very quick light with a group of six flashes followed by a long flash of not less than 2 s duration, in a period of 10 s. (b) Group quick light with a group of six flashes followed by a long flash of not less than 2 s duration, in a period of 15 s. | The duration of the eclipse immediately preceding a long flash should be equal to the duration of the eclipses between the flashes at the very quick rate. The duration of a long flash should not be greater than the duration of the eclipse immediately following the long flash. The duration of the eclipse immediately preceding a long flash should be equal to the duration of the eclipses between the flashes at the quick rate. The duration of a long flash should not be greater than the duration of the eclipse immediately following the long flash. |

| Mark | Rhythmic character of the light | Remarks and further recommendations |
| --- | --- | --- |
| West cardinal | (a) Group very quick light with a group of nine flashes, in a period of 10 s. (b) Group quick light with a group of nine flashes, in a period of 15 s. |  |
| ISOLATED DANGER | (a) Group‑flashing light with a group of two flashes, in a period of 5 s. (b) Group‑flashing light with a group of two flashes, in a period of 10 s. | Only the colour White is used. The duration of a flash together with the duration of the eclipse within the group should be not less than 1 s and not more than 1.5 s. The duration of a flash together with the duration of the eclipse within the group should be not less than 2 s and not more than 3 s. |
| SAFE‑WATER | (a) Long‑flashing light with a period of 10 s. (b) Isophase light. (c) Single‑occulting light. (d) Morse Code light with the single character "A". | Only the colour White is used. |
| SPECIAL | (a) Group‑occulting light. (b) Single‑flashing light, but not a long‑flashing light with a period of 10 s. (c) Group‑flashing light with a group of four, five or (exceptionally) six flashes. (d) Composite group‑flashing light. (e) Morse Code light, but not with either of the single characters "A" or “U”`[2]. | Only the colour Yellow is used. A group‑flashing light with a group of five flashes at a rate of 30 flashes per minute, in a period of 20 s, is assigned to Ocean Data Acquisition Systems (ODAS) buoys. |
| EMERGENCY WRECK MARKING BUOY | Occulting Alternating light with a period of 3s | Only the colours Blue and Yellow are used |
| MOBILE ATON | Three flicker flashes followed by two non-flicker flashes with a period of 10.4 s. | Only the colour Yellow is used. |

[1]	A single fixed light shall not be used on a mark within the scope of the IALA Maritime Buoyage System because it may not be recognized as an aid to navigation light.

[2]	A Morse Code white light with the single character "U" is assigned to offshore structures.

Timing of Astronomical Events

The night-time operation of lighted aids to navigation is emphasised, but daytime role is often as important. The astronomical events that define the transitions from day to night are shown below.

Timing of Astronomical Events

| Event | Condition | Illumination Lux | Comment |
| --- | --- | --- | --- |
| Sunset/Sunrise | Upper edge of the sun’s disc is coincident with the horizon. | 600 |  |
| Civil Twilight (start / end) | Centre of sun is at a depression angle of six (6) degrees below the horizon. | 6 | Large objects are seen, but detail are not discernible. Brightest stars and planets are visible and the sea horizon is clearly defined. |
| Nautical Twilight (start / end) | Centre of sun is at a depression angle of twelve (12) degrees below the horizon. | 0.06 | It is dark for normal practical purposes and the sea horizon is not normally visible. |
| Astronomical Twilight(start / end) | Centre of sun is at a depression angle of eighteen (18) degrees below the horizon. | 0.0006 | Illumination is less than that from starlight and other natural light sources in the sky. |

Switch-on / Switch-off Light Levels

For lighted aids to navigation that only operate at night, the ambient light levels at which an AtoN light switches on should be chosen so that the AtoN light switches on while the ambient light level is sufficiently high to allow safe navigation while not switching on during overcast conditions when the AtoN is not necessary for safe navigation.

Refer to IALA publications:

  - Guideline G1038 on Ambient Light Levels at which Aids to Navigation Should Switch On and Off.

Night Operations

Nominal Range and Luminous Intensity

Table 14 is an extract of the IALA recommendation for the notation of luminous intensity and range of lights and provides a conversion between nominal range and luminous intensity.

IALA Conversion Table for Luminous Intensity and Nominal Range for Night Observations

| Nominal Range (nautical miles) | Luminous Intensity (candela) | Nominal Range (nautical miles) | Luminous Intensity (candela) |
| --- | --- | --- | --- |
| 1 | 1-2 | 14 | 7,140-11,100 |
| 2 | 3-9 | 15 | 11,200-17,100 |
| 3 | 10-23 | 16 | 17,200-26,100 |
| 4 | 24-53 | 17 | 26,200-39,700 |
| 5 | 54-107 | 18 | 39,800-59,900 |
| 6 | 108-203 | 19 | 60,000-89,800 |
| 7 | 204-364 | 20 | 89,900-133,000 |
| 8 | 365-632 | 21 | 134,000-198,000 |
| 9 | 633-1060 | 22 | 199,000-293,000 |
| 10 | 1,070-1,750 | 23 | 294,000-432,000 |
| 11 | 1,760-2,840 | 24 | 433,000-634,000 |
| 12 | 2,850-4,500 | 25 | 635,000-962,000 |
| 13 | 4,540-7,130 | 26 | 927,000-1,135,000 |

This table assumes an atmospheric transmissivity of T=0.74 and a threshold of illumination of 0.2 μlux.

Night-time nominal range table (rounded off to the nearest nautical mile)

- Night time nominal range table (rounded off to the nearest nautical mile)

| Luminous intensity | Nominal range (rounded) | Luminous intensity | Nominal range (rounded) | Luminous intensity | Nominal range (rounded) |
| --- | --- | --- | --- | --- | --- |
| candelas (cd) | nautical miles (NM) | kilocandelas (103 cd) | nautical miles (NM) | Megacandelas (106 cd) | nautical miles (NM) |
| 1 - 2 | 1 | 0.633 – 1.06 | 9 | 0.927 – 1.35 | 26 |
| 3 - 9 | 2 | 1.07 – 1.75 | 10 | 1.36 – 1.96 | 27 |
| 10 - 23 | 3 | 1.76 – 2.84 | 11 | 1.97 – 2.84 | 28 |
| 24 - 53 | 4 | 2.85 – 4.53 | 12 | 2.85 – 4.11 | 29 |
| 54 - 107 | 5 | 4.54 – 7.13 | 13 | 4.12 – 5.93 | 30 |
| 108 - 203 | 6 | 7.14 – 11.1 | 14 | 5.94 – 8.53 | 31 |
| 204 - 364 | 7 | 11.2 – 17.1 | 15 | 8.54 – 12.2 | 32 |
| 365 - 632 | 8 | 17.2 – 26.1 | 16 | 12.3 – 17.5 | 33 |
|  |  | 26.2 - 39.7 | 17 | 17.6 – 25.1 | 34 |
|  |  | 39.8 – 59.9 | 18 | 25.2 – 35.9 | 35 |
|  |  | 60.0 – 89.8 | 19 | 36.0 – 51.2 | 36 |
|  |  | 89.9 - 133 | 20 | 51.3 – 72.9 | 37 |
|  |  | 134 -198 | 21 | 73.0 - 103 | 38 |
|  |  | 199 - 293 | 22 | 104 -147 | 39 |
|  |  | 294 - 432 | 23 | 148 - 209 | 40 |
|  |  | 433 - 634 | 24 |  |  |
|  |  | 635 - 926 | 25 |  |  |

#### Background Lighting

Nominal range at night is calculated with no allowance for glare from background lighting. Excessive background lighting, from street lights, neon signs etc., frequently makes an aid to navigation light less effective and, in some cases, it becomes completely lost in the general background clutter. Such a light can be made more conspicuous by increasing its intensity, changing its colour or by varying its rhythm.

#### Glare

Glare can be caused by bright lights emitted from the shore, such as car headlights, or from another vessel indiscreetly using a search light. An aid to navigation light can also cause glare if it is too bright for the shortest viewing distance, especially when the focal plane of the light and the observer’s eye are at the same height. This situation can arise with two station leading lines. For aids to navigation lights, it is generally accepted that the illuminance at the eye of the navigator from the light:

should not exceed 0.1 lux;

should be reduced to 0.01 lux if the background is very dark.

Refer to IALA publications:

Recommendation R0112(E-112) for Leading Lights (including excel program);

Guideline G1023 for the Design of Leading Lines.

Guideline G1073 Conspicuity of AtoN lights at night

In situations where glare is a problem, one or more of the following alterations may lead to a satisfactory result:

raise the focal plane of the light so that the mariner uses the loom of the light or a less intense part of the vertical distribution of the light;

reducing the intensity of the light source;

reducing the size of the optic;

masking the optic with, for example, perforated metal sheet;

screen unnecessary arcs of the light;

use two or more lower intensity lights instead of one higher intensity light.

Whatever methods are used, it will be necessary to measure or calculate the intensity and distribution of the modified light or lighting system.

Guidance on the impact of background lighting and meteorological conditions on the light intensity required to achieve a particular range is provided in Table 18.

Night and Day with Background.

??

This table is intended as guidance only. It is not to be used for Nominal Range Publication.

#### Intensity Losses

Some lighting equipment has to be installed inside a protective lantern housing. Unless it is practicable to measure the luminous intensity of the complete installation, it is normal practice to apply a de-rating factor to the intensity of the lighting equipment to allow for the reflection and transmission losses at the lantern glazing, generally referred to as the glazing loss factor.

Glazing bars or astragals may reduce the intensity of the light at certain bearings. The installation of non-vertical astragals will overcome this reduction to a certain extent. The focal plane of the light should be positioned away from any horizontal glazing bars or intersections.

IALA recommends that, in the absence of more definitive information, the glazing loss factor be taken as 0.85 for a system in clean condition.

Refer to IALA publication:

Recommendation R0205(E200-5) on Estimation of the Performance of Optical Apparatus.

#### Service Conditions Factor

Under normal operating conditions, the luminous intensity of a light is likely to degrade between service (maintenance) intervals. There are several components to this degradation:

meteorological conditions (which may only be temporary);

dirt and salt deposition (which can be minimised by an efficient regular programme of cleaning of the optical system and housing);

progressive deterioration of the light source over the service interval.

It is clearly impossible to represent such a complex array of factors in any simple way, and a proper assessment of the various effects could only be made by measurements on-site at regular intervals. However, in order to give a more realistic figure for the performance of the light under normal operating conditions than when the luminous intensity is measured in a laboratory or on a photometric range, it may be appropriate to apply a service conditions factor to the measured intensity.

Refer to IALA publication:

Recommendation R0202(E-200-2) Marine Signal Lights – Calculation, definition and notation of luminous range

Guideline G1148 Determination of required Luminous Intensity of Marine Signal Lights.

#### Day Operations

A number of authorities have established daytime lighted leading lines in major ports and waterways to achieve a more consistent performance than is possible with dayboards.

Nominal Daytime Range and Luminous Intensity

Refer to IALA publications:

Recommendation R0202 (E-200-2) On Marine Signal Lights Part 2 – Calculation, Definition and Notation of Luminous Range;

Recommendation R0111 on Port Traffic Signals.

Figure 13 and Table 15 are extracts of Recommendation R0202 On Marine Signal Lights Calculation, Definition and Notation of Luminous Range and provides a conversion between nominal daytime range and luminous intensity.

The Luminous Range Diagram, shown in Figure 13 enables the mariner to determine the approximate range at which a light may be sighted, by day in the meteorological conditions prevailing at the time, and for various levels of sky luminance (refer to Table 16).

Threshold value for illuminance: Et = 1x10-3lx

Daytime Luminous Range Diagram

The graph has been drawn for a sky luminance of 10 000 cd/m². For other values of sky luminance mark off along the scale of abscissae the distance between the luminance of 10 000 cd/m² and that under consideration as it appears on the auxiliary scale.

Example:

Suppose that it is required to calculate the luminous range of a light of 2 000 000 cd for a meteorological visibility of 2 nautical miles under an ordinary overcast sky (luminance 1 000 cd/m²).

Measure the distance A separating graduations 10 000 cd and 1 000 cd on the auxiliary scale. Transfer this distance to the scale of abscissae from the graduation corresponding to 2 000 000 cd (2x106 cd) in the same sense. A point slightly to the right of graduation corresponding to 12 nautical miles is obtained. Erect from this point a parallel to the axis of ordinates to meet the curve for 2 nautical miles visibility. Read off the luminous range on the vertical scale against the point so obtained. It should read approx. 4 nautical miles.

I

ALA Conversion Table for Luminous Intensity and Nominal Daytime Range

- Day time nominal range table (rounded off to the nearest nautical mile)

| Luminous intensity | Nominal range (rounded) | Luminous intensity | Nominal range (rounded) |
| --- | --- | --- | --- |
| kilocandelas (103 cd) | nautical miles (NM) | Megacandelas (106 cd) | nautical miles (NM) |
| 1 – 12.0 | 1 | 1.02 – 1.82 | 7 |
| 12.1 – 45.3 | 2 | 1.83 – 3.16 | 8 |
| 45.4 – 119 | 3 | 3.17 – 5.32 | 9 |
| 120 – 267 | 4 | 5.33 – 8.78 | 10 |
| 268 – 538 | 5 | 8.79 – 14.2 | 11 |
| 539 – 1010 | 6 | 14.3 – 22.6 | 12 |
|  |  | 22.7 – 35.6 | 13 |
|  |  | 35.7 – 55.5 | 14 |
|  |  | 55.6 – 85.6 | 15 |
|  |  | 85.7 – 130 | 16 |
|  |  | 131 – 198 | 17 |

Required Illuminance in Varying Meteorological Conditions

| Meteorological Condition | Luminance, (cd/m2) | Required Illuminance, (μlx) |
| --- | --- | --- |
| Very dark overcast sky | 100 | 13 |
| Dark overcast sky | 200 | 24 |
| Ordinary overcast sky | 1000 | 107 |
| Bright overcast sky or clear sky away from the direction of the sun | 5000 | 506 |
| Bright cloud or clear sky close to the direction of the sun | 10000 | 1000 |
| Very bright cloud | 20000 | 1980 |
| Glaring cloud | 50000 | 4910 |

- Required illuminance under different daytime conditions

#### Meteorological Optical Range

This is the distance through the atmosphere that is required for 95% attenuation in the luminous flux of a collimated beam of light using a source colour temperature of 2700ºK.

The meteorological optical range is related to the atmospheric transmissivity by the formula:

It is often convenient to simplify the above expression by giving the distance term a value of one, such that:

#### Visual Range

This is the maximum distance at which the contrast of the object against its background is reduced by the atmosphere to the contrast threshold of the observer. The visual range can be enhanced if the observer uses binoculars, although the effectiveness depends on the stability of the observer’s platform. Visual Range can be interpreted as the distance that a given light is seen by an observer.

#### Luminous Range

This is the maximum distance at which a given light signal can be seen by the eye of the observer at a given time, as determined by the meteorological visibility prevailing at that time. It does not take into account the: height of the light, observer’s height of eye, or curvature of the Earth.

#### Nominal Range

Nominal range is the luminous range when the meteorological visibility is 10 nautical miles, which is equivalent to a transmission factor of T = 0.74. Nominal Range is generally the figure used in official documentation such as nautical charts, Lists of Lights, etc. Nominal range assumes that the light is observed against a dark background, free of background lighting.

### Miscellaneous

This section includes information on options for increasing the conspicuity of aids to navigation structures.

#### Illumination of Structures

Illumination of structures can provide an important AtoN function. Illumination of fixed structures is frequently called flood‐lighting or facade‐lighting.

The purpose of structure illumination is to assist the mariner to positively identify the object and to allow estimation of distance and relative position to the object. Illumination can be direct or indirect and can be used on structures, signs and daymarks and is considered complimentary to the main aid to navigation light.

Refer to IALA publication:

Guideline 1061 Light Applications - Illumination of Structures.

#### Retroreflective Materials

The use of retroreflective material for aids to navigation has a widespread use in Scandinavian and other high latitude countries. With narrow and complicated waters and fairways, including ice condition and long and dark nights during wintertime, retroreflective material is a cheap and effective way to obtain high level of night-time conspicuity.

The use of retroreflecting material for aids to navigation is particularly in the case of unlighted aids whereby the projection of a light, which may range from a hand-held spotlight to a powerful searchlight, an aid can more easily be located and sometimes identified.

Refer to IALA publication:

Recommendation R0106 - Retroreflecting Material on Aids to Navigation Marks with IALA Maritime Buoyage System.

Guideline 1145 Application of Retroreflecting material on AtoN

### Light Sources

There are a variety of light sources currently used in aid to navigation applications, however due to developments in Light Emitting Diode (LED) technology LEDs are now widely used. Table 19 compares the performance and other characteristics of the various light sources.

Light source performance and other characteristics

| Light source | Max lifetime hours | Max Lumen/Watt | Flashable |
| --- | --- | --- | --- |
| Filament lamp | 2000 | 16 | Yes |
| Tungsten Halogen | 4000 | 25 | Yes |
| Metal Halide | 20,000 | 120 | No |
| LED | 100,000 | 140 | Yes |
| Low Pressure Sodium | 10,000 | 150 | No |
| Xenon | 3,000 | 40 | No |

Light Emitting Diode (LED)

Coloured LED

Electronic semiconductor devices that produce near monochromatic light. The semiconductor junction is encapsulated in a clear plastic housing that usually incorporates a lens. Several LEDs may be grouped together in a cluster, or an array, to provide a light source of the required size and intensity with lamp redundancy. LEDs operate from a low voltage DC supply. Correct operation depends on accurate control of the supply current. LED marine lanterns are sometimes reported as having intense colours and ranges longer than the current IALA calculation method would suggest. IALA is currently investigating this.

White LED

A semiconductor junction emitting blue/violet light is encapsulated with an integral phosphor such that both blue and broad band yellow light are emitted together to form a near white light.

Typical Use:

Lighted beacons on buoys and other short and medium range AtoN, but longer range LED lanterns are increasingly available in the market;

Range lights consisting of flat arrays of LEDs or single high power LEDs;

Signs and signals formed by arrays of LEDs in the shape of letters, numerals, signs etc.

- Technical data:

Power: Single LED: 1 milliwatt to over 32 Watts, Cluster LED: 1 to 60 Watts of higher

Efficiency: Luminous Efficacy of LEDs is improving steadily.

Lifetime: 100,000 hours

“Advantages:”

Very long life (if input power and temperature are carefully controlled) and hence low whole life costs;

Life is so long that lampchangers are not considered necessary;

High luminous efficiency in red and green;

Light produced in saturated signal colours therefore coloured filters not needed;

Mechanically robust when compared with conventional lamps;

Light switching times are very fast;

Relatively cool operation;

Easy to cluster LEDs.

Disadvantages:

Complex electronic control needed to achieve long life and high performance;

Generally difficult to match to existing optics;

Luminous efficiency decreases slowly with life;

White LEDs will be very inefficient with red and green filters;

LED life can be severely reduced if input power and temperature are not carefully controlled.

Safety:

No special hazard.

Disposal:

Consult local and national disposal regulations.

Operating lifetime will depend on the LED junction operating temperature and operating environment.

Detailed information on light sources and their associated operational considerations, lifetime, reliability, operating costs and power consumption is covered in IALA Guideline 1043.

Refer to IALA publication:

Guideline 1043 on Light Sources Used in Visual Aids to Navigation.

### Integrated Power System Lanterns

Integrated Power System Lanterns (IPSL) have application advantages for certain situations. By incorporating modern technologies, they can be small, durable, reliable, cost effective and fully self-contained. Technological advances in light emitting diodes (LEDs), photovoltaics (Solar Panels) and batteries complement each other and facilitate a compact lantern. In order to operate efficiently, these lanterns must be designed for a wide range of solar conditions (i.e. sunlight available to charge the lantern) while maintaining a specified optical output over the expected operating lifetime.

The application criteria for IPSL include nominal light ranges up to 5NM, areas with good solar insolation, areas that suffer from vandalism or theft and small buoys with limited weight carrying ability. They are not suitable where high

duty cycle rhythmic characters are required or in areas suffering from icing. An IPSL device houses power source, power storage, LED light source, rhythmic character coding and switching together in a single unit. IPSL can accept external programming commands and include options for GPS, synchronization, and communication modules.

Refer to IALA publication:

  - Guideline 1064 on Integrated Power Systems Lantern.

Audible Signals

The following provides a brief overview of audible AtoN signals, more detailed information is provided by referring to the following IALA publications.

Refer to IALA publications:

  - Recommendation R0109(E-109) for the Calculation of the Range of a Sound Signal;

  - Guideline G1090 on the Use of Audible Signals.

Nominal Range Audible AtoN signal range is calculated as nominal and is expressed in nautical miles. The nominal range is defined by a probability of 90% of hearing the signal when subjected to a noise as defined in IALA Guideline G1090. Specific ranges cited in the above paragraphs refer to the nominal range calculation.

Hazard Warning It has been IALA policy since 1985 that audible signals, also referred to as sound signals, should only be used as a hazard warning. These hazards refer to certain man-made structures such as offshore structures, renewable energy infrastructure, bridges, breakwaters, and isolated AtoN. The Competent Authority shall determine whether a hazard requires an audible signal and the level of reduced visibility per year that justifies its installation (e.g. 10 days of visibility under 1 nautical mile per year).

Where provided, audible signals for navigational hazards should have a nominal range of at least 1 nautical mile. In addition, Competent Authorities may require a backup audible signal of a reduced range (these do not necessarily need to be separate units); 0.5 nautical mile nominal range is considered adequate for these backup audible signals.

Augmentation of Floating Aids to Navigation Audible signals may also be used to augment buoys, both lighted and unlighted, to enhance their effectiveness to the mariner in reduced visibility. Audible signals on buoys are most often powered by the motion of the sea and include bells, gongs, and whistles. Buoys may also be fitted with electronic horns. Audible signals on buoys should be used to warn mariners of a particular hazard, such as proximity to shoals, rocks or other hazards; or to alert the mariner to a change in navigational requirements, such as the entrance to a restricted channel. Where electronic audible signals are used to augment buoys, they should have a nominal range of 0.25 to 0.5 nautical miles.

## Maritime Buoyage System and other Aids to Navigation

The IALA Maritime Buoyage System (MBS) represents one of IALA’s major contributions to enhancing the safety of navigation. As recently as 1976 there were more than thirty buoyage systems in use worldwide and conflicting sets of rules applied. In 1980 Lighthouse Authorities from fifty countries and representatives from nine international organisations reached agreement on the rules for a single system. In 2010 the MBS was revised. Key changes made included the introduction of an emergency wreck marking buoy and fixed marks. The full name of the revised system is therefore IALA Maritime Buoyage System and other Aids to Navigation, still being referred to as the MBS.

The MBS uses 7 types of Aids to Navigation, which may be used in combination. The mariner can distinguish between these aids by identifiable characteristics. The system includes:

- Lateral Marks[6];

- Cardinal Marks;

- Isolated Danger Marks;

- Safe Water Marks;

- Special Marks;

- Emergency Wreck Marking Buoy;

- Mobile Marine Aids to Navigation

- Other Marks.

The General Principles and Rules of the IALA Maritime Buoyage System can be found in Annex D.

### Marking New Dangers

The Emergency Wreck Marking Buoy (EWMB) is meant for prompt response to mark new dangers such as a wreck. It should therefore only be on station until the Competent Authority is satisfied that information concerning the new danger has been sufficiently promulgated or the danger is otherwise resolved. An appropriate risk assessment should be used to determine how long the EWMB should be deployed. If the new danger is expected to remain, the Competent Authority should mark it with a regular marking scheme.

The EWMB should be equipped and of a size that facilitates its detection under all sea conditions.

Upon a decision to use the EWMB, it should be deployed without unnecessary delay. This can be met by the use of EWMBs that are stored onboard a vessel ready for deployment. It should be taken into consideration that a smaller buoy, in some instances, may be deployed more rapidly. If necessary it could subsequently be replaced with a larger buoy.

Refer to IALA publications:

- Maritime Buoyage System (with supporting guidelines);

- Guideline 1046 on a Response Plan for the Marking of New Wreck.

IALA also has a consolidated recommendation and guidelines for marking areas for specific navigational needs in relation to a variety of man-made structures including aquaculture facilities and offshore resource production and energy generation structures.

### Other Marks

Other Marks are visual marks, intended to aid navigation as information to mariners, not necessarily regarding channel limits or obstructions.

#### Leading Lines/Ranges

Transits / Leading (Range) Lines

A transit is defined as the alignment of two or more marks. A Leading (or Range) light is a specialised application of a transit.

Leading (Rear Range) Light - Photo Courtesy of the Canadian Coast Guard A simple transit can be used to:

Provide a turning reference;

Define a clearing line for the limits of safe navigation;

Provide a distance mark along a waterway.

Leading Lines

A leading line is an aid to navigation system that comprises two separated structures with marks or lights that, when viewed from the centreline or deepest route along a straight section of channel, are aligned.

In a two station leading line, the structures lie along an extension of the centreline of the nominated channel. The rear structure must have a greater elevation than the front structure to enable both marks or lights to be viewed simultaneously.

A leading line provides a vessel with a heading reference and a visual indication of the size and direction of any cross track error.

Purposes of Leading Lines

A leading line may be used to:

- indicate the centreline of a straight section of a navigable channel;

- indicate to deep draught vessels the deepest part of the waterway;

- indicate the navigable channel where fixed and floating aids to navigation are not available[7] or do not satisfy the accuracy requirements for safe navigation;

- define a safe approach bearing to a harbour or river entrance, particularly where there are cross currents;

- separate two-way traffic (i.e. when passing a bridge).

Design Considerations for Leading Lines

A well-designed leading line will enable the type and size of vessels that typically use the channel to:

- Identify the marks or lights when the ship is at the inner and outer sections of the channel and readily detect cross track position errors from the centreline of the channel;

- Detect cross track position errors with sufficient sensitivity that the channel can be utilised without abrupt changes to the vessel’s heading and speed;

- Observe both lights together, by selection of leading light character rhythms that appropriately overlap in their free running condition. In some situations it may be preferable to provide additional equipment to synchronise the light characters;

- Observe the lights in all ambient conditions for which they are designed to be used without glare. If lights are to be used for both day and night operations light intensities will need to be varied.

The characters of rhythmic leading lights should be selected so that the front and rear lights, in their free running states, can generally be observed together.

In some situations it may be preferable to provide additional equipment to synchronise the light characters. If lights are to be used both day and night, the light intensities should be adapted for each situation to avoid glare at night. Radar transponders (RACONs) may be used as leading line markers.

Refer to IALA publications:

- Recommendation R0112(E-112) for Leading Lights (including excel program);

- Guideline G1023 for the Design of Leading Lines;

#### Sector Lights

A sector light is an aid to navigation that displays different colours and/or rhythms over designated arcs. A common means of creating a sector is to fit a coloured filter in front of the main light. However, sector lights with LED light sources are being introduced to the market thereby reducing the need for filters as they produce the coloured light. A sector can also be produced by filtering or by using a secondary light (or several lights) on the same structure. The secondary light can take any of the following forms:

Range (directional) light;

Beacon with a coloured lens, masked to achieve the sector angle;

Beacon fitted with internal or external filter panels;

Beacon or beacons with different coloured light sources, masked to achieve the sector angle;

Precision Direction Light.

The limits or boundaries of a sector are not always precisely cut off due to the characteristics of the light source, fading of colours or changing rhythms between adjacent sectors.

For a beacon fitted with coloured filter panels, the reason for the lack of a precise transition at the sector boundary is readily apparent from Figure 14 which shows the light source, lens and filter geometry. The transition zone is defined by an “angle of uncertainty”. A similar geometry exists with multiple coloured beacons and masking

Bearings, directions of leading (range) lines and limits of sectors should always be stated in terms of the bearings that would be seen by the mariner. Bearings may carry a suffix ‘TBS” or True Bearing from Seaward as confirmation.

Angle of Uncertainty

It can also be noted that:

the observed angle of uncertainty is generally less than the geometric angle due to the relative intensities of sector colours (i.e. colour mixing) as the observer passes through the transition zone;

if space on the aid to navigation structure is not a limiting factor, it is usually possible to achieve an angle of uncertainty of around 0.25° with this type of sector arrangement;

the angle of uncertainty can be reduced by decreasing the physical width of the light source or by increasing the radial distance to the coloured filter;

in situations where the main light has a large projected area, such as a rotating lens or reflector array, it is generally preferable to use a separate sector light rather than installing a coloured filter in front of the main light.

From time to time specialised sector lights have been developed to exhibit different rhythms over different sector bearings. This capability is found in some Precision Direction Lights (PDL)[8].

A PDL is a specialised form of sector light that can generate sharply defined sector boundaries. This feature is particularly useful for applications that require one or several narrow sectors or high precision boundaries. The PDL may use a white light source with coloured filter, but newer designs are utilising LED and possibly laser as a light source.

PDL sector lights are very precise, allowing a complete colour change at a sector boundary to occur over an angle of less than 1 minute of arc in most models.

Applications

The design of sector lights can be a complex task. The process should be carried out with reference to an accurate chart of the area. In some cases good local knowledge is also required.

A sector light may indicate one or more of the following:

- boundaries of a navigable waterway;

change of course position;

shoals, banks, etc.;

an area or position (eg.an anchorage);

the deepest part of a waterway;

position checks for floating aids.

A PDL allows for further applications that include the ability to:

produce narrow sectors with an angle of uncertainty down to approximately one minute of arc;

define the central zone of a channel;

accurately mark one side of a straight channel (a pair of PDLs can cover the permutations of converging, diverging and parallel channels);

define different rhythms over adjacent sectors.

Examples

Some examples of sector lights applications are illustrated in Figure 15 and Figure 16.

Sector Light Application

This illustration follows the IALA Maritime Buoyage System colour convention for Region A (‘red to port when approaching the aid from seaward’). The white sector should, if possible, be wide enough to provide a margin of safety for a vessel that inadvertently leaves the white sector. Curves C and D indicate depth contours or limiting dangers that dictate the boundaries of sectors.

Various Applications for Sector Lights

The function of each light in Figure 16 is described below:

- Light I is a coastal white light with a red sector indicating a danger.

- Light II is a sector light obscured over the shore, with two white sectors indicating a safe channel. When sailing towards the sector light it shows red to port and green to starboard following the IALA Maritime Buoyage System colour convention for Region A and vice versa for Region B. The boundary between the red and the green sector also indicates the position of a buoy.

- Light III is a sector light with a red light and 4 white sectors indicating four anchorage positions. It is obscured over the shore.

- Light IV is a sector light with a white sector indicating a safe channel.

Design Considerations for Sector Lights

Where a single sector light defines a navigable channel the following points should be considered:

Lateral Position: There is no reference of the vessel’s lateral position within the channel until a sector boundary is reached. This may cause a problem in channels subject to a strong cross current. For vessels with local knowledge, the zones defined by the angle of uncertainty can sometimes provide a useful guide to the vessel’s proximity to a sector boundary;

Safety Margin: Where practicable, there should be a margin of safety between the sector boundary and adjacent hazards. If an appropriate safety margin cannot be achieved within the sector boundary, the hazards could be marked separately.

Angle of Uncertainty: Zones defined by the angle of uncertainty should be considered an additional margin of safety over the actual sector boundary;

Vessel Size: The design process for a sector light needs to consider the draught and manoeuvrability of the largest vessels likely to utilise the sector, how quickly they can respond once they cross a sector boundary and the

situations that may develop when other vessels are in the vicinity;

Lights and Filters: When using an incandescent light source the sector design should take account of the spectral distribution of the light source and the proportion of this light transmitted through the filter material as this will affect the resultant colour and intensity of the light exhibited. The process should also check for potential glare problems;

Flash Characteristic: The period of the light flash should be selected to provide ample time for a mariner to recognise the transitional phases that occur at the sector boundary ;

Sector Colours: A white light is normally the first preference for a lighthouse or beacon. If a single coloured sector is added, red is often used. If a white sector light is used to mark a navigation channel, coloured sectors may be used either side of the white to indicate the lateral limits. In such cases it is common practice to use red and green sectors that follow the convention of the IALA Maritime Buoyage System;

Lamp Position and Type: The position of the light source within the optical system is critical for the correct alignment of the sectors. When replacing lamps or using lampchangers, it is important to ensure that the light source (e.g. filament) position is identical. If a lampchanger is incorporated, the sector system should be designed for the widest light source used in the lampchanger.

Refer to IALA publication:

- Guideline 1041 on Sector Lights.

Mobile AtoN

A MAtoN is defined as a non-fixed or un-moored AtoN; it does not include a fixed or moored buoy that is adrift from its station, temporarily or otherwise. A MAtoN can be fitted with an Automatic Identification System (AIS) device transmitting message 21. Note: MAtoN should not be used for marking unmanned vessels; the lights exhibited by these vessels should comply with COLREGS, or other Competent Authority regulations.

TYPICAL USES MAtoN may be used, but not limited to, the following moving/drifting applications: • Ocean Data Acquisition System (ODAS) (e.g. to gather data on currents and weather);

Wreckage (e.g. containers, debris);

Water quality and pollution monitoring equipment;

Dynamic guard zones and convoys;

Underwater operations;

Enhancing navigational safety during military operations (e.g. no sail zones during minesweeping, target exercises areas);

Towed and deployed applications (e.g. cable laying);

Search & Rescue applications.;

Special events (e.g. swimming competitions).

Types of MAtoN

MAtoN can be deployed either physically or virtually. Competent Authorities should determine the most appropriate type of MAtoN for each situation, based on the available equipment and assessment of risk.

Refer to IALA publication:

- Guideline 1154 Use of Mobile AtoN

- Recommendation R1016 Mobile Marine Aid to Navigation

Fixed Aids to Navigation - Lighthouses and Beacons

The IALA International Dictionary of Aids to Marine Navigation defines a beacon as “a fixed artificial navigation mark” that can be recognised by its shape, colour, pattern, topmark or light character, or a combination of these. While this functional definition includes lighthouses and other fixed aids to navigation, the terms lighthouse and beacon are used more specifically to indicate importance and size.

Lighthouse: A lighthouse is generally considered to be a large conspicuous structure (visual mark) on land, close to the shoreline or in the water that:

- acts as a daymark;

- provides a platform generally for higher range marine AtoN signal lights.

Other aids to navigation such as audible signals and radio aids to navigation may be located on or near the lighthouse. A lighthouse may be a staffed or an automated facility, although the staffing of lighthouses is becoming less common. An automated lighthouse may be remotely monitored and in some cases remotely controlled.

Beacon: Visual characteristics of a beacon are often defined by daymarks, topmarks, and by numbers. A marine signalling light, if fitted, would generally be of a lower range than lighthouses. In navigable channels a pile beacon may be used as an alternative to a buoy[9].

Purpose of Lighthouses and Beacons

A lighthouse or beacon may perform one or more of the following navigational functions:

- mark a landfall position;

- mark an obstruction or a danger;

- indicate the lateral limits of a channel or navigable waterway;

- indicate a turning point or a junction in a waterway;

- mark the entrance of a Traffic Separation Scheme (TSS);

- form part of a leading (range) line;

- mark an area;

- provide a reference for mariners to take a bearing or line of position (LOP).

Other purposes for which a lighthouse can be used include:

- base for AIS equipment; racon; radar; radionavigation systems; reference station for GNSS;

- coast-watch or coastguard functions;

- VTS functions;

- base for audible (fog) signals;

- collection of meteorological and oceanographic data;

- radio and telecommunication facilities;

- tourist facilities.

Floating Aids to Navigation - Minor and Major

A floating aid to navigation serves a similar purpose to a beacon or a lighthouse. However the floating aid to navigation is normally associated with locations where:

- it would be impractical due to water depth, seabed conditions or cost to establish a fixed aid;

- the hazard shifts over time (e.g. sand banks, an unstable wreck, etc.);

- the aid is at high risk of damage or loss from ice flows or ship impacts and as a consequence is treated as expendable;

- a temporary mark is required.

- a seasonal mark is required.

Buoys: Buoys are defined as minor floating aids and whilst it is normal that they are lit there are instances where no light is installed. These types of aids to navigation are specifically covered by the IALA Maritime Buoyage System and tend to have circular hull forms up to 3 m diameter. Buoys may be fitted with sound signals. Most buoys are equipped with a radar reflector.

In addition, due to limitations of the structure, the following may apply:

- where lights are exhibited, they are usually solar or primary battery powered. There are gas powered buoys still in operation, although gas powered buoys are not normally used for new installations;

- where lights are exhibited, due to power limitations and/or operational requirements, light ranges are typically restricted to 1 to 5 nautical miles; although longer ranges may be required in some applications;

- additional services are restricted due to limited power on a buoy, but RACONs, AIS AtoN, and remote monitoring units are sometimes installed in addition to a light;

- audible signals are used on buoys in some countries.

Light Vessels, Lightships and Large Navigational Buoys: Light Vessels, Lightships, and Large Navigational Buoys (LNB), sometimes referred to as LANBYs, are defined as major floating aids and may carry one or more of: RACONs, AIS AtoN or audible signals in addition to the aid to navigation light. A light vessel may also display a white riding light to signify a vessel at anchor. All major floating aids should be equipped with a radar reflector and a monitoring unit.

Major aids to navigation:

- generally have high operating costs;

- are only deployed at critical locations;

- are often assigned an aid availability target that is higher than for a buoy;

- are not specifically covered by the IALA Maritime Buoyage System.

Refer to IALA publication:

- Recommendation R0104(O-104) for ‘Off Station’ Signals for Major Floating Aids to Navigation.

- Guideline G1006 on Plastic Buoys

- Guideline G1099 Hydrostatic Design of Buoys

- Guideline G1015 Painting of AtoN Buoys

- Guideline G1098 AIS on Buoys

???

Examples of Floating Aids

Performance Criteria for Floating Aids

Availability is defined as the probability that an aid to navigation or a system of aids to navigation, as defined by the Competent Authority, is performing its specified function at any randomly chosen time. This is expressed as a percentage of total time that an aid to navigation or a system of aids to navigation should be performing their specified function[10].

The availability of a floating aid is the principal measure of performance determined by IALA. The recommended availability targets are indicated in Table 20. The availability objective assigned to floating aids to navigation conforming to the IALA Maritime Buoyage System should also apply to the topmark.

Availability Targets

| Description of Aid | Availability Target |  |
| --- | --- | --- |
| Floating aids to navigation that are considered to be of vital navigational significance | Category 1 | 99.8% |
| Floating aids to navigation that are considered to be of important navigational significance | Category 2 | 99.0% |
| Floating aids to navigation that are considered to be of necessary navigational significance | Category 3 | 97.0% |

Refer to IALA publications:

- Recommendation R0130(O-130) on Categorisation and Availability Objectives for Short Range Aids to Navigation;

- Guideline G1035 on Availability and Reliability of Aids to Navigation.

Technical Considerations for Floating Aids to Navigation

There are various technical considerations that should be taken into account, including: cost, design factors, positioning, water conditions and markings.

Cost

The cost of establishing a floating aid at a given location will generally be less than for a fixed structure. The cost difference increases with increasing water depth and exposure to wind and waves.

In contrast, the maintenance cost of floating aids to navigation tends to be high relative to the capital value. This has caused many authorities to critically examine the potential for savings through design changes, use of alternative materials, alternate service deliveries (contracting out) and amending maintenance practices, generally with the aim of extending maintenance intervals.

Where an competent authority operates a large number of floating aids, it may become practicable to operate a dedicated buoy tender vessel with specialised equipment to minimise buoy change-out times and to improve occupational safety.

Refer to IALA publication:

- Guideline 1047 on Cost Comparison Methodology of Buoy Technologies.

Floating Aid Design

The process of designing a buoy to meet specific requirements is a specialised task. It involves, but is not limited to:

- defining the operational performance characteristics;

- defining the equipment, power requirements and power source(s);

- defining the type and capabilities of the vessels that will be used to service the buoy;

- selecting the initial type proportions and mooring for the buoy;

- integrating of equipment and power supply;

- considering of the maintenance requirements;

- identifying deployment and recovery techniques;

- protecting equipment from damage;

- providing the ability to rectify faults without having to lift the buoy;

- determining the buoy response to the wave, wind and current conditions at the site(s);

- optimising the design.

Refer to IALA publications:

- Maritime Buoyage System (and supporting guidelines);

- Guideline 1006 on Plastic Buoys;

- Guideline 1039-2 on a Standard Method for Defining and Calculating the Load Profile of Aids to Navigation;

- Guideline 1036 on Environmental Considerations in Aids to Navigation Engineering;

- Guideline 1039 Designing solar power systems for AtoN

- Guideline 1043 on Light Sources Used in Visual Aids to Navigation;

- Guideline 1094 on Daymarks for Aids to Navigation;

- Guideline 1099 on the Hydrostatic Design of Buoys;

- Recommendation R0106 on the Use of Retroreflecting Material on Aids to Navigation Marks within the IALA Maritime Buoyage System.

Mooring Design

The mooring system for a floating aid to navigation is the sum of the components that keep the aid within a nominated area. These components have to withstand the forces of wind, wave, current and ice on the floating aid and drag on the mooring line.

The basic assumptions made are that the:

- mooring system tethered to the buoys sinker is usually tangential to the sea bed;

- buoy axis is vertical under the most common conditions of current and wind;

- ratio of the breaking stress of the mooring system to the calculated stress is not less than 5 under the most unfavourable conditions of current and wind;

- reserve buoyancy of the fully equipped floating aid is greater than the combined loads of current and wind under the most unfavourable conditions.

An approximate value for the minimum length of a chain mooring is given by the following formula:

Lmin = 3H for depths less than 50 metres;

Lmin = 2H for depths greater than 50 metres. L = Length of mooring line (m)

H = Depth[11] of water (m)

All moorings should be designed as per Guideline 1066 on Design of Floating Aids to Navigation Moorings.

Refer to IALA publications:

- Guideline 1066 on the Design of Floating Aids to Navigation Moorings.

Positioning of Floating Aids

The charted position of a floating aid defines the nominal (or true) position for the anchor. With most floating aids there is potential for the mooring anchor/sinker to be moved off-station during storms or due to ice flows. Additionally, positional errors can occur while laying the anchors/sinkers.

The positioning process for anchors/sinkers should utilise radionavigation or radio-positioning aids. The use of GNSS position fixing is increasingly seen as the preferred method. The benefits of GNSS position fixing are: convenience, accuracy and repeatability. A buoy tender using GNSS can generally be brought to within 10 metres of the nominal buoy position at the time of releasing the anchor/sinker.

If the anchor/sinker is allowed to free-fall, its final resting position will depend on the prevailing current, water depth, shape of the anchor/sinker and the nature of the seabed. Controlling the decent of the anchor/sinker may serve to improve the positional accuracy of the buoy.

Markings and Topmarks

Markings

Floating aids to navigation are often identified by names, abbreviations of names, letters and/or numbers. Authorities should ensure that the actual marking is identical to the List of Lights reference and the charted marking.

Topmarks

The type, colour and arrangement of topmarks on a buoy are shown in the IALA Maritime Buoyage System, extracts of which are shown in Annex D. Topmarks should conform to Guideline 1094 on Daymarks for Aids to Navigation.

Refer to IALA publications:

- Guideline G1094 on Daymarks for Aids to Navigation.

Notes

CIE website address: www.cie.co.at/cie

Blue surface colours may be used in inland waterways. estuaries and harbours where the colour may be seen at close range. see IALA Recommendation R0108(E-108). in addition, blue lights are being tested for use on emergency wreck marking buoys - IALA Recommendation O-133 refers

CIE website address: www.cie.co.at/cie

A single fixed light shall not be used on a mark within the scope of the IALA Maritime Buoyage System because it may not be recognized as an aid to navigation light.

A Morse Code white light with the single character “U” is assigned to offshore structures.

Lateral marks differ between buoyage regions A and B.

For example, in waterways where the aid may be drifting or destroyed due to ice conditions.

Also known by the trade name of PEL light.

In these situations the beacon will generally show a colour scheme and topmarks in accordance with the IALA Maritime Buoyage System.

As adapted from the IALA Guidelines on Availability and Reliability of Aids to Navigation, Theory and Examples (Edition 2, December 2004).

This is defined as the maximum depth of water and includes the highest tide level and half the maximum wave height at the particular site.

Studies are being conducted on the benefit of expanding the functionality of existing systems by providing a timing signal from which the user may then calculate their position independently from GNSS. This is known as Ranging Mode (R-mode).

At present, the IALA MF beacon system and AIS services are being considered as candidates for modification to add R-mode functionality. By providing timing information over their normal MF or VHF transmissions, a shipboard receiver may then calculate a distance (range) to the transmitter. By calculating the range to several stations, the user is able to calculate the ship’s position. Coverage, geometry and interference issues would need to be investigated.

The provision of R-Mode services would require the availability of an accurate non-GNSS timing source at the transmitter. High stability clocks could be an expensive option and it is more likely that time would be sourced from a low frequency radio time clock or eLoran.

### Radar Aids to Navigation

Radar aids to navigation are devices that provide returns to a ship’s radar that help to locate and/or identify a navigation mark. The IMO carriage requirements contained in Chapter V, Regulation 19 of the SOLAS Convention 1974 (as amended), states all ships of:

300 gross tonnage and upwards are to carry a 9 GHz radar;

3,000 gross tonnage and upwards are to be fitted with a 3 GHz radar or, where considered appropriate by the Administration, a second 9 GHz radar.

Some administrations may impose other carriage requirements.

IMO Resolution MSC.192 (79) Adoption of the Revised Performance Standards for Radar Equipment (December 2004) states that 9 GHz radars should be capable of detecting radar beacons, SARTs and radar target enhancers. By omission, 3-GHz radars are not required to detect radar beacons and SARTS. With the removal of the 3-GHz radar racon detection requirement, ship-owners are free to use higher performing radars, often referred to as New Technology (NT) radars, discussed below.

9 GHz radars are also extensively carried by vessels not covered by SOLAS or local regulation. Due to this high rate of carriage, radar aids to navigation in the 9 GHz band are especially useful.

#### Radar Reflectors

A radar reflector is a passive device designed to return the incident radar pulses of electromagnetic energy back towards the source and thereby enhance the response on the radar display. By design, a radar reflector attempts to minimise absorption and random scattering effects. A radar reflector is generally installed as a supplementary device at sites that would also be marked with a light. The main objectives are to enhance:

- target detection at long ranges (for example, for landfall navigation);

- target detection in areas of sea or rain clutter; and

- radar conspicuity of aids to navigation to reduce the risk of collision damage.

The performance of a radar reflector can be defined in terms of its effective radar cross section (RCS). This is a value determined by comparing the strength of radar signals returned by the radar reflector with the equivalent return from a radar reflective sphere of 1m2 reflecting area.

The range at which a radar reflector target can be detected is dependent on the heights of the radar antenna, the reflector and the output power of the radar. There are analogies to the geographical range of visual marks. The radar (Some text missing here)

## Level of Service

Level of Service (LOS) is the commitment of service by the Competent Authority to mariners who are navigating or operating in an area, as well as clients and/or governments responsible for funding the provision of the relevant service.

Level of service can be articulated through a Level of Service Statement that should be clear, easy to understand and available to all concerned.

### Benefits

An established level of service is integral to efficient planning and delivery and provides users with a clear understanding of the expected services. Moreover, it ensures that services are delivered in a nationally consistent, integrated, predictable, measurable and fair manner.

### Components

A level of service statement should include, at minimum, the following components:

Type

Should describe what the Competent Authority will provide. It is a description of the service provided, such as, visual aids to navigation, radionavigation systems, or Vessel Traffic Services.

Extent

Should describe where and why a service will be provided by the Competent Authority. Most Competent Authorities are bound by the International Convention on the Safety of Life at Sea, 1974 as amended (SOLAS) Chapter 5, Regulation 13, which states that Each Contracting Government undertakes to provide, as it deems practical and necessary either individually or in cooperation with other Contracting Governments, such aids to navigation as the volume of traffic justifies [where] and the degree of risk requires [why]. The extent of service provided may also vary by Competent Authorities for specific areas, category of users, or due to national obligations.

Quality

Should address to what level the Competent Authority will provide a service. It is a minimum standard at which clients can expect a service to be performed, also known as a performance standard. A performance standard is a benchmark against which actual performance of a service can be measured. It may be expressed in the form of a target such as percentage of availability of a service or service response times.

### Layers of Service

A summary of available aids to navigation systems and obtainable accuracies is provided in Table 4.

The various type of AtoN have advantages and disadvantages for the user as well as for the provider as indicated in Table 38.

Comparison of Different Types of Aids to Navigation

| System |  | Users | Providers |  |
| --- | --- | --- | --- | --- |
|  | Advantages | Disadvantages | Advantages | Disadvantages |
| Visual | Can be used to position | Range depends on site, height, colour | For hazard warning, | Maintenance expensive |
|  | Convey immediate | background | traffic regulation, | Planning for maintenance |
|  | information | Limited by visibility | guidance, etc. | depends on weather |
|  | Can be used without a chart if user has a good | Position of floating aids not always accurate | Placement flexible | conditions |
|  |  |  | Maintenance requires | Logistic system required |
|  | local knowledge |  | little training | Training maintenance personnel |
| Radar | Identification with racon | Onboard equipment needed | Can replace visual aids | Radar reflectors needed |
|  | possible in reduced visibility conditions | Racons may interfere if not placed in an | Warnings of dangers | Some vessels do not have |
|  |  | appropriate configuration, aids equipped with | (New dangers) | radar |
|  | With a racon identification | radar reflector are difficult to identify |  | Racon investment |
|  | of low coastline |  |  | expensive |
|  | Only one aid is required |  |  | Training for maintenance of racons |
|  | Rapid deployment |  |  |  |
| Radionavigation | Wide scale coverage | On board equipment needed | Reduced maintenance | May not be under Aids to |
|  | All weather use |  | Automatic monitoring | Navigation Authority's |
|  | Automatic navigation |  | Reduction of visual aids possible | control |
|  | Precision possible |  |  | Monitoring requirement |
|  |  |  |  | Training maintenance personnel |
|  |  |  |  | Large investment |

Refer to IALA publications:

- Guideline 1004 on Levels of Service;

- Guideline 1079 on Establishing and Conducting User Consultancy by Aids to Navigation Authorities.

## Risk Management

Dealing with “risk” is an intrinsic aspect of human existence. The establishment of the early lighthouses represented a tangible way of addressing some of the problems that arose when humans decided to venture out to sea, and then into global trade and the mass transport of people by ships.

The traditional definition of risk is the probability of an unwanted event occurring, multiplied by the impact or

consequence of that event.

R=P*C

Unwanted events include deprival, loss or injury to persons, property or the environment.

Risk management is a term applied to a structured (logical and systematic) process illustrated further below.

The correct, efficient and useful result of hazard identification, assessment of risk and establishment of risk control measures, can be affected by Human Factors. The concept of Human Factors and references to relevant models is included in the IALA Guideline 1018 on Risk Management. It is recommended that organisations and persons involved in a risk assessment process have suitable knowledge in the application of Human Factors disciplines.

With the advances of e-Navigation, the mariner will be provided with additional real time information to assist with navigation. The positive impact on ship control and navigation needs to be incorporated into the formal risk assessment process. For risk control options, the continuous development of e-Navigation and man-machine interfaces may provide new possibilities. However, physical AtoN risk control measures will remain important to address the needs of all user groups.

The risk management approach works equally well for identifying the risks at a detailed or broad level. It can also address the risks from different perspectives.

Refer to IALA publication:

- Recommendation R1002 Risk Management for Marine Aids to Navigation

- Guideline G1018 Risk Management

For example, if the issue is the automation and destaffing of a lighthouse, there are likely to be different sets of risk for:

- service providers (aids to navigation competent authority, lightkeepers, etc.);

- service users (mariners);

- external groups (politicians, local community, conservation groups, etc.).

### IALA Risk Management Tools

IALA continuously develops its Risk Management Toolbox which includes tools that are capable of:

- assessing the risk in ports or waterways, compared with the risk level considered by Authorities and stakeholders to be acceptable. The elements that can be taken into consideration include those relating to vessel conditions, traffic conditions, navigational conditions, waterway conditions, immediate consequences and subsequent consequences;

- identifying appropriate risk control options to decrease the risk to the level considered to be acceptable. The risk control options available include improved co-ordination and planning; training; rules and procedures including enforcement; navigational, meteorological and hydrographical information; radio communications; active traffic management and waterway changes; pilotage; and,

- quantifying the effect on the risk level of an existing port or waterway that may result from a change or reduction of any of the risk control options in use.

Risk management tools can also assist in assessing the risk level of existing ports and waterways as well as determining the probable risk level of proposed new ports and waterways or if substantial changes to existing ports and waterways are being planned. The IALA Risk Management Toolbox includes three different approaches:

- PAWSA Mk II (Port and Waterway Safety Assessment) which is a Qualitative Risk Assessment approach;

- IWRAP Mk II (IALA Waterway Risk Assessment Programme) which is a Quantitative Risk Assessment approach;

- SIRA (Simplified Risk Assessment) which utilizes a basic risk matrix approach.

The three approaches can be used individually, or in combination, sequentially or in parallel. The IALA Risk Management Toolbox is now being used for risk assessments in conjunction with submissions to the IMO. In addition to these three basic IALA tools, various simulation tools are nowadays also widely used for assessing risk in ports and waterways.

The IALA World-Wide Academy provides training on the use of the Risk Management Toolbox through regular training courses. Background information on the elements of the toolbox is provided on a wiki accessible through the IALA website. If further guidance or assistance is required, please contact the IALA World-Wide Academy.

Authorities are encouraged to provide copies of risk assessments made with the IALA Risk Management Toolbox to the IALA Secretariat.

Refer to IALA publication:

- Guideline G1123 The Use of IALA Waterway Risk Assessment Programme (IWRAP Mk II)

- Guideline G1124 The Use of Ports and Waterways Safety Assessment (PAWSA Mk II) Tool

- Guideline G1138 The Use of the Simplified IALA Risk Assessment Method (SIRA)

### Risk Management Decision Process

The Risk Management process described in the IALA Guideline 1018 comprises five steps that follow a standardized management or systems analysis approach:

- Identify hazards;

- Assess risks;

- Specify risk control options;

- Make a decision;

- Take action.

The diagram in Figure 35 provides a guide to the steps involved in the IALA Risk Assessment and Risk Management process.

Hazard – an unwanted event or occurrence, a source of potential harm, or a situation with a potential for causing harm, in terms of human injury; damage to health, property, the environment, and other things of value; or some combination of these.

Risk – the Risk is a measure of the likelihood that an undesirable event will occur together with a measure of the resulting consequence within a specified time i.e. the combination of the frequency and the severity of the consequence. This can be either a quantitative or qualitative measure.

Risk Assessment and Risk Management Process

The central part of the Figure 35 illustrates the five steps in the risk management process. In addition the figure suggests a consultation and reporting element throughout the process.

Stakeholders including practitioners and users shall be consulted and receive feed back continuously to ensure the best possible input to the decision makers, to validate decisions and to ensure ownership of the results and actions taken. The monitoring and review part in the right side of the model is vital to ensure a verification of the decisions, to check if initial conditions have changed and to constantly monitor if control measures are implemented effectively.

### Risk Level and Acceptability

Once the possible unwanted risk scenarios have been identified and described, they must be ranked in terms of their probability and associated consequences. This yields a risk value, enabling prioritization by placing each risk scenario in a matrix similar to the one below. This allows resources to be assigned to mitigate the more serious risks first.

Risk Value Matrix

The SIRA method provides suitable scoring scales for probability (frequency of occurrence) and consequences/impact. It should be kept in mind that the consequences can include both short- and long-term impacts.

Risks with a low risk value (green) may be fully acceptable and require no action, while risks with a high value (red) need urgent attention. The intermediate risk values (yellow and amber) will need to be addressed at some stage, the essence being that all risk values should be As Low As Reasonably Practicable (ALARP) considering the cost and effectiveness of the identified risk control options.

## Availability Objectives

The measurement of ‘Availability’ provides a quantitative measure of performance or service to the mariner.

Availability’ is a useful indicator of the level of service provided by individual or defined groups of aids to navigation because it is representative of all the considerations, within the control of the Competent Authority, that have gone into providing and maintaining the facility.

These include:

quality assurance procedures

design and systems engineering;

procurement;

installation and commissioning;

maintenance procedures;

failure response;

logistics.

To obtain a true representation of Availability, it is necessary to measure the long-term performance of an aid to navigation. To achieve this it is recommended that the calculations should use a time interval greater than 2 years.

### 8.4.1 Calculation of Availability

The availability of an aid to navigation may be calculated using one of the following equations, and is usually expressed as a percentage:

MTBF = Mean Time Between Failures

MTTR = Mean Time To Repair

### Definition and Comments on Terms

Reliability

Ability of a device or system to satisfy the requirements of its intended use within defined limits, and for a stated period of time.

Availability

The percentage of time that an aid, or system of aids, is performing a required function under stated conditions. Non-availability can be caused by scheduled and/or unscheduled interruptions.  It is also defined within IMO resolution A.1046(27) for WWRNS as “The system is considered to be available when it provides the required integrity for the given accuracy level”. IALA generally uses the term as a historical measure of the percentage of time that an aid to navigation was performing its specified function. The non-availability can be caused by scheduled and/or unscheduled interruptions.

Continuity

The probability that, assuming a fault-free receiver, a user will be able to determine position with specified accuracy and is able to monitor the integrity of the determined position over the (short) time interval applicable for a particular operation within a limited part of the coverage area.

For example, if a GNSS station is functioning correctly when a vessel is about to make its approach into a port, the continuity factor is the probability that the GNSS service will not be interrupted in the time it takes the vessel to reach its berth. As for GNSS systems, IALA has proposed that the time interval for continuity calculations be based on a 15-minute time frame in accordance with IMO A.1046(27) for WWRNS.

Redundancy

The existence of more than one means for accomplishing a given function. The various means of accomplishing the function need not necessarily be identical. The concept is such that a complete failure can occur only when all means have failed.

Integrity

This is the ability to provide users with warnings within a specified time when the system should not be used for navigation. IMO Resolution A.1046(27) for WWRNS, stares that this time to alarm should be within 10 seconds.

Failure

This is the (unintentional) termination of the ability of a system or part of a system to perform its required function.

Mean Time Between Failures (MTBF)

This is the average time between successive failures of a system or part of a system. It is a measure of reliability. For components, such as lamps, it is usual to determine the MTBF (or life) statistically by testing a representative sample of components to destruction. As for a system such as a GNSS station, the MTBF is determined from the number of failures that have occurred within a given interval. For example; if four failures occur over a two year interval, the MTBF would be 4380 hours (i.e. =24*365*2/4).

Mean Time to Repair (MTTR)

This is a measure of a Competent Authority’s administrative arrangements, resources and technical capability to rectify a failure. For a small port, the MTTR times might only be several hours. Meanwhile, a Competent Authority with a more distributed network of aids to navigation may have MTTR times equivalent to several days because of the distances and transport mobilisation limitations.

Failure Response Time

This is a sub-set of the MTTR and relates to the time it takes to be notified of a failure, to confirm the details and mobilise personnel to depart for the aid to navigation.

Refer to IALA publications:

Recommendation O-130 on Categorisation and Availability Objectives for Short Range Aids to Navigation;

Guideline 1035 on Availability and Reliability of Aids to Navigation.

### IALA Categories for Traditional Aids to Navigation

IALA provides a method to categorise and calculate aids to navigation availabilities for both individual aids to navigation and systems of aids to navigation as shown in Table 39. IALA Recommendation O-130 does not consider other aids to navigation considered in the mix of aids to navigation such as radionavigation systems or Vessel Traffic Services (VTS). It does provide guidance on suitable and realistic levels of operational performance for competent authorities to adopt.

Availability Objectives by Category

| CATEGORY | AVAILABILITY OBJECTIVE | CALCULATION PERIOD |
| --- | --- | --- |
| 1 | 99.8% | Availability Objectives are calculated over a continuous three-year period, unless otherwise specified |
| 2 | 99.0% |  |
| 3 | 97.0% |  |

Category 1

An Aid to Navigation (AtoN) or system of AtoN that is considered by the Competent Authority to be of vital navigational significance. For example, lighted aids to navigation and RACONs that are considered essential for marking landfalls, primary routes, channels, waterways or new dangers or the protection of the marine environment.

Category 2

An AtoN or system of AtoN that is considered by the Competent Authority to be of important navigational significance. For example, it may include any lighted aids to navigation and RACONs that mark secondary routes and those used to supplement the marking of primary routes.

Category 3

An AtoN or system of AtoN that is considered by the Competent Authority to be of necessary navigational significance.

The Recommendation also states that the absolute minimum level of availability of an individual aid to navigation should be set at 95%.

### Availability and Continuity of Radionavigation Services

The availability objectives for Radionavigation services have been handled somewhat differently from traditional aids to navigation. This reflects the broader policy formulation process that includes IMO Resolution A.1046(27) on a World Wide Radionavigation System and IALA Recommendation R-121.

Refer to IALA publications:

Recommendation R-121 on the Performance and Monitoring of DGNSS Services in the Frequency Band 283.5 – 325 kHz

Recommendation R-121 retains the original definition of availability, but adds a statement about “nonavailability”.

Non-availability is equivalent to “down time” but as proposed includes both scheduled and/or unscheduled interruptions (i.e. preventative and corrective maintenance). The revised equation becomes:

MTBO = Mean time between outages; based on a 2 year averaging period MTSR = Mean time to service restoration; based on a 2 year averaging period

IMO uses a more elaborate definition of Continuity than that given in Section 8.4.2. It states that: Continuity is the probability that, assuming a fault free receiver, a user will be able to determine position with specified accuracy and is able to monitor the integrity of the determined position over the (short) time interval applicable for a particular operation within a limited part of the coverage area. This is the same definition of as “mission reliability”.

If the service is available at the beginning of the operation, then the probability "P" that it is still available at a time "t" later is:

This is the standard expression for reliability and excludes scheduled outages. It uses MTBF and assumes that planned outages will be notified.

The Continuity, or probability that the service will be available after a continuity time interval (CTI), is then:

If MTBF is much greater than CTI, the equation approximates to:

Where:

MTBF = Mean time between failures based on a 2 year averaging period.

CTI = Continuity Time Interval – in the case of Radionavigation calculations, this is equal to 15 minutes (from A1046(27)).

There is no need to include the availability at the beginning of the time period of the operation because if there is no service, then the operation will not commence.

Example 1: Using the figures in the previous example for a system with a 2 year MTBF, the continuity over a 15 minute period is 1-(15/1,051,200), or 99.9986%.

Example 2: Using the figures in the previous example for a system with a 1000 hour MTBF, the continuity over a 15 minute period is 1-(15/60,000), or 99.9750%.

### Over and Under Achievement

The actual availability achieved by an individual aid to navigation is a reflection of the quality of the logistical processes, the maintenance regime and the skill of personnel involved. There is a cost associated with prescribing a higher level of availability for a system such as an aid to navigation. This is irrespective of whether or not the increased availability is required by the mariner. There is also a cost associated with the maintenance of unreliable systems. The interrelationship is complex, but the objective is to find the minimum cost solution as illustrated in Figure 37.

The Cost of Reliability

Over-Engineering vs. Unreliability

For a lighthouse in a remote location, the cost of time and transport to rectify equipment failures can be very high.

From this perspective:

- the one-off cost of over-engineering is generally not as expensive in the long term as the ongoing cost of attending to un-reliable equipment and/or poor system designs;

- a conservative design approach has its merits.

If the aid is not achieving its availability objective, the Authority should ascertain the reasons for this and implement actions that remedy the situation. IALA has recommended that if a facility cannot achieve an availability of 95% (i.e. 50 days out per 1000 days) after reasonable endeavours, consideration should be given to withdrawing the facility (as an aid to navigation).

If a single aid within a group is performing above its availability objective, it could be due to either technical or environmental reasons. If the performance difference occurs between sites using similar equipment, and this trend has been established for some time,  the Authority should investigate the reasons for the difference.

If a group of aids is found to be over performing for a relatively long period of time, there is an opportunity to review the maintenance practices with a view to determining the reasons, and possibly to consider extending the maintenance intervals or reducing the maintenance effort. This could lead to lower operating costs and issues relating to surplus maintenance capacity.

## Reviews and Planning

### Reviews

In many countries, the network of aids to navigation has been built up over a considerable time, in some cases, centuries.

It should be recognised that the nature of shipping is continually changing and this means that the aids to navigation infrastructure should be reviewed periodically.

The rate of change varies from place to place, but it would be reasonable to adopt a review process using one of the change management tools that provides:

a Strategic Plan with a suggested minimum 10 year outlook;

an Operational Plan with a suggested rolling 5 year work program.

The increasing availability of AIS-derived ship data (type, position, speed, cargo etc.) is proving to be a very useful tool in reviewing the relevance of existing aids to navigation and identifying new requirements.

Effective use of AIS data requires a data management strategy and appropriate technology to efficiently store and manipulate very large amounts of data and be able to be integrated with other electronic data, for example electronic nautical charts to display shipping patterns.

### Strategic Plans

A Strategic Plan is the result of an informed and consultative process that sets the long term goals and objectives for an organisation.

For a Competent Authority it would include:

- the role of the authority, for example: -- to promote a high standard of maritime safety; -- to provide infrastructure and information services to support the safety of navigation in a particular area.

- how the authority will go about discharging its responsibilities, for example: -- outline of the corporate values of the authority; -- corporate governance arrangements; -- funding arrangements; -- reviews of industry trends;

- an understanding of the users and navigation requirements.

Because of its importance and its effect on the mariners, any strategic plan should be developed as much as possible in full consultation with the mariners and other stakeholders.

### Operational Plans

The Operational Plan might cover:

- the implementation of the strategic plan, and may include statements on current policy issues such as:

maintenance;

current and new technology;

the design life of new infrastructure -- remote monitoring and control;

historic lighthouses;

environmental culture and safety;

the program for aids to navigation reviews;

contract services (core and non-core);

transport services;

sources of revenue;

external relationships[3];

information, communication and consultation management.

- a list of changes to individual aids to navigation, including any new facilities.

The list would reflect:

decisions resulting from user and stakeholder consultations;

reviews, including those that use:

risk analysis, risk management procedures (see section 8.3), or;

a level of service process, (see section 3.2), or;

the authority’s quality management procedures (see section 8.7) or; the authority’s technical and maintenance policies etc.

- project schedules that reflect known priorities, such as: -- government policies; -- user requirements; -- available resources; -- budget (revenue) forecasts and constraints.

### Use of Geographic Information Systems in AtoN Planning

The use of Geographic Information Systems (GIS) may assist in effective AtoN planning, including evaluation and validation; ensuring that money is invested wisely in new technology.

Coastal waterways are becoming increasingly congested with vessel traffic and developments such as offshore wind farms, tidal turbines and aquaculture sites, which may be required to be marked.

In addition, light pollution through coastal development, the advent of larger and faster ships and the continued growth in small craft usage means that designing suitable AtoN systems becomes ever more complex.

Using GIS, accurate design and provision of AtoN systems as well as suitable simulation can prove very useful and may reduce the chance of costly mistakes being made.

AtoN are distinctly linked to physical locations and their use by mariners invariably involves the use of more than one AtoN at a time, that is, AtoN networks or systems.

These single and interdependent linkages between AtoN and their physical locations mean that GIS technology can provide AtoN authorities with enhancements in many areas of their business, which may ultimately lead to benefits for mariners.

A GIS captures, displays, stores, analyses and manages spatially referenced data. A key feature of GIS is its analytical functionality, which allows a user to interact with spatial data to determine relationships between different types of data and to produce qualitative (diagrammatic/graphical) and quantitative (numeric/tabular) results.

Refer to IALA publications:

- Recommendation O-138 on the use of Geographic Information Systems (GIS) and Simulation by Aids to Navigation Authorities;

- Guideline 1057 on the use of Geographic Information Systems (GIS) by Aids to Navigation Authorities;

- Guideline 1058 on the use of Simulation as a Tool for Waterway Design and AtoN Planning.

## Quality Management

Quality Management Systems have been developed and introduced by numerous businesses, but increasingly are being based on the ISO 9000 series of standards. These standards provide a broadly accepted framework for implementing a quality management system. A generic quality management system is process focused and defines procedures for how things are to be done and what resources are necessary.

It addresses:

- who does what?

- what skills and qualifications are necessary?

- what processes have to be followed to get consistent outcomes?

- what resources are necessary to do the work efficiently? The equipment in aids to navigation systems can be divided into two aspects: the specific AtoN aspect, and the more generic aspect.

Each aspect must comply with applicable standards and regulations. IALA Recommendations and Guidelines provide a basis for the AtoN specific aspect, while international, national or regional regulations apply to the more generic aspects.

Refer to IALA publications:

- Recommendation O-132 on Quality Management for Aids to Navigation Authorities;

- Guideline 1034 on product certification procedures;

- Guideline 1052 on Quality Management in Aids to Navigation Service Delivery,

- Guideline 1054 on Preparing for a Voluntary IMO Audit on Aids to Navigation Service Delivery

### Performance Measurement

Performance measurement is the process of collecting, analysing and/or reporting information regarding the performance of an individual, group, organisation, system or component. It is very important to establish an continuous performance measurement as background for the quality management.

The information obtained can be used to:

- show accountability to government and stake holders;

- demonstrate the efficiency and effectiveness of the service being provided;

- monitor and improve occupational health and safety performance;

- compare the performance of:

similar systems or equipment in different locations;

contract and internally provided services[4];

- amend:

system designs;

procurement decisions;

equipment choices;

maintenance procedures and practices;

increase or reduce maintenance effort;

adjust maintenance intervals.

Refer to IALA publications:

- Guideline 1037 on Data Collection for Aids to Navigation Performance Calculation;

- Guideline 1035 to Availability and Reliability of Aids to Navigation Theories and Examples;

### International Standards

ISO 9000 Series

The 1994 quality standard series of ISO 9001, 9002 and 9003 have been jointly revised and amalgamated into ISO 9001-2000.

The new series of standards designated as ISO 9000 comprises:

ISO 9000 Quality management systems - Fundamentals and vocabulary.

ISO 9001 Quality management systems - Requirements.

ISO 9004 Quality management systems - Guidance for Performance Improvement.

ISO 9001 - 2015

ISO 9001 specifies requirements for a quality management system that can be used for internal application by organisations, or for certification, or for contractual purposes. It focuses on the effectiveness of the quality management system in meeting customer requirements. See Figure 38.

ISO 9004 - 2009

ISO 9004 gives guidance on a wider range of objectives of a quality management system than does ISO 9001, particularly for the continual improvement of an organisation’s overall performance and efficiency, as well as its effectiveness. ISO 9004 is recommended as a guide for organisations whose top management wishes to move beyond the requirements of ISO 9001, in pursuit of continual improvement of performance. However, it is not intended for certification or for contractual purposes.

ISO 14000 Series

This is a collection of voluntary consensus standards that have been developed to assist organisations to achieve environmental and economic gains through the implementation of effective environmental management systems.

There are three standards that deal with Environmental Management Systems (EMS). These are ISO 14001, 14002 and 14004. ISO 14001 is the only standard intended for third party accreditation. The other standards are for guidance.

??

ISO 9001 Diagram on the Emphasis on Satisfying Customer Requirements

ISO 14001

ISO 14001 specifies the requirements for an environmental management system, to enable an organisation to:

formulate a policy and objectives taking into account legislative requirements and information about significant environmental impacts;

apply the requirements to those environmental aspects that the organisation can control and over which it can be expected to have an influence;

demonstrate to itself, and to other interested parties, conformance with its stated environmental policy;

seek certification/registration of its environmental management system by an external organisation;

manage and measure a program of continual improvement.

ISO 14001 does not itself state specific environmental performance criteria.

Refer to IALA publications:

Recommendation R1004 on Sustainability in the provision of Marine Aids to Navigation

Guideline 1036 on Environmental Management in AtoN

Guideline 1137 on AtoN Management in protected areas.

## Maintenance

### Guiding Principles for Maintenance

Maintenance is required to ensure that AtoN equipment and systems continue to perform at the levels required by mariners to safely navigate the World’s waterways.

A maintenance system should be adopted to ensure that AtoN assets deliver the desired performance while minimizing Total Cost of Ownership. This performance is normally defined as the level of availability required. Depending on the criticality or category of the AtoN, the same AtoN type might require different maintenance approaches to deliver the required availability outcome in a given location.

The following guiding principles may assist Authorities in developing their overall AtoN maintenance strategy.

Minimise the Cost of Ownership

AtoN service providers are accountable to their stakeholders for the provision of a reliable AtoN network that meets international standards for a reasonable cost. Maintenance strategies adopted by authorities should seek to reduce the total cost of ownership of their AtoN.

Design for Maintenance

The majority of maintenance costs are determined by the design of the equipment itself. Maintenance costs are usually the most significant component of the total ownership cost of the equipment or system; therefore, it is crucial to account for long-term maintenance and logistics support early on in the design process.

The goal should be to reduce the need for maintenance, extend the time interval between maintenance periods, enable maintenance upon the evidence of need (condition-based maintenance), facilitate the maintenance task by the servicing personnel, and reduce the “logistics footprint” required for maintenance and support.

All of these factors will contribute to reducing the total ownership cost over the entire life cycle of the equipment or system.

### Improving Efficiency

Authorities have been able to achieve significant cost savings by a number of means:

Automation

Automation can reduce the work load of light-keepers or allow for de-staffing altogether. This reduces:

staff costs;

power consumption;

the frequency of stores replenishment;

commitments on infrastructure such as houses or accommodation facilities, water and fuel storage and in some cases jetties and cargo handling equipment;

the requirements for station vehicles and equipment.

Equipment

It may be possible to use more reliable equipment, better system designs, with “fail safe” or “fail by stages” features coupled with:

longer intervals between maintenance visits;

a review of maintenance management procedures.

In addition, it may be possible to use standardised equipment to simplify spare part management.

This could also:

benefit the purchasing power of the organisation;

reduce the range of skills required by maintenance staff (and thus the training burden);

give more flexibility in the choice of basic qualification when recruiting maintenance staff;

provide more opportunity to understand the inherent deficiencies in particular pieces of equipment and for remedial actions to be implemented.

Fixed vs Floating

Depending on location, it may be possible to replace floating aids with fixed structures in waterways of moderate depth; particularly if it also allows a dedicated buoy tender to be replaced by some other means of transport such as smaller vessel or launch.

A whole-of-life cost benefit analysis should be carried out to assist in any such decisions.

Materials

By introducing low maintenance materials such as high density polyethylene, Glass Reinforced Plastic (GRP), stainless steel, etc., it may be possible to reduce maintenance requirements and time on site. This may also decrease the number of ship-day requirements and reduce the need for construction (or structural maintenance) skills.

Remote Control and Monitoring

Remote control and monitoring (RCMS)of distant or isolated aids to navigation can save on the cost of responding to what is later found to be a false outage report. It can also allow for analysis of aids to navigation systems using risk analysis / risk management techniques that may produce cost savings from a rearrangement and or reduction of the aids to navigation within a nominated area. Correctly applied, the monitoring will enable staff to travel to site with the required spares to repair an issue without a return visit. The provision of a control function enables a remote reset which may avoid the need for a site visit.

Refer to IALA publications:

Guideline 1008 on Remote control and monitoring of AtoN

### Trends

IALA Conference papers, IALA Bulletin articles and feedback from IALA Members demonstrate an increasing trend in the extension of maintenance intervals for aids to navigation sites. The ongoing striving for greater efficiency in the delivery of aids to navigation reflected in measures such as the automation of lighthouses has seen alteration of the maintenance intervals from a daily activity to significantly less frequent regimes.

The optimal maintenance interval for aids to navigation is determined from a consideration of national priorities and the Authority’s administrative, technical and environmental constraints.

Where cost efficiency and effectiveness is the driving issue, Authorities are:

using automation, alternative structural materials, more durable coatings and renewable power supplies to contain or reduce costs;

addressing the potential for new technology to reduce acquisition and operating costs

extend maintenance intervals;

remote maintenance by design and remote control to avoid additional visits;

reviewing transport service options.

Extension of maintenance intervals at sites exposed to more extreme weather conditions may result in more extensive maintenance works at each visit which may offset some of the cost savings achieved through extension of service intervals.

### Maintenance Intervals

The maintenance intervals for aids to navigation vary from daily in the case of a manned lighthouse to perhaps five years for a lighted buoy.

It is difficult to establish a clear view of the appropriate maintenance interval as this will vary from authority to authority.  This will vary on the quality of the installation, the prevailing environmental conditions and the performance of the AtoN.

Advances in self-contained beacons, lamps, solar power supplies and remote monitoring make it relatively easy for a well-designed system on a fixed structure to achieve annual or biannual servicing intervals. Systems that can be maintained in multiples of a year can be set up to take advantage of the times of the year that minimise the weather risk on work schedules and disturbance to flora and fauna.

However, a balance has to be found since longer maintenance intervals affect the authority’s knowledge of storm damage, general deterioration to aids to navigation, and control over vegetation growth and dirty glazing that could increase the risk of obscuration and fire damage etc. There may also be a detrimental effect on the detailed level of knowledge held by maintenance personnel.

Refer to IALA publications:

Recommendation R1018 on The responsible design, operation and maintenance in the provision of marine Aids to Navigation

Guideline 1077 on Maintenance of Aids to Navigation;

Guideline 1076 on Building Conditioning of Lighthouses.

## Service Delivery

Authorities responsible for the provision of aids to navigation are generally at a state or national level.

They are usually the sole national regulator of marine aids to navigation infrastructure and services, but are not necessarily the sole provider of these services.

In some countries there is a division of responsibility between the authority representing the national government and other organisations that include;

state and territorial authorities;

local government organisations;

port, harbour or waterway authorities;

local private organisations.

Photo Courtesy of CEREMA

### Service Delivery Requirements

Where more than one local authority provides aids to navigation services, the Contracting Government has the ultimate responsibility to comply with the obligations under the SOLAS Convention as listed in Section 8.1.

### Contracting Out

For a number of decades, national authorities have utilized service providers for the delivery and maintenance of Marine Aids to Navigation (AtoN) services.

Every organisation considering contracting AtoN services should have a clear understanding of what is to be achieved by delivering the service through alternative contracted means as opposed to delivering the service directly by the National Authority. The best practices, advantages, disadvantages, and keys to successfully managing a contracted AtoN servicing program are covered in detail in IALA Guideline 1005, Contracting Out Aids to Navigation Services. It is recommended that National Authorities review this Guideline if they are considering the contracting out their AtoN servicing responsibilities and be mindful of retaining in house expertise to understand the technologies and requirements of AtoN Management and delivery.

Refer to IALA publication:

- Guideline G1005 on Contracting Out Aids to Navigation Services.

- Guideline GNNNN on Quality Control of 3rd Party AtoN service Providers

| # Chapter | # AtoN design and delivery |
| --- | --- |

## Visual SIGNALLING

## Range and performance

## Design, implementation and maintenance

## power supplies

### Types

A wide range of power systems and energy sources have been used or contemplated for operating lighthouses and floating aids. Everything from clockwork to radio-active isotopes have been used. Some of the more common types are listed in Table 35.

Power Sources for Operating Lighted Aids to Navigation.

| Electric Energy Sources & Storage | Non-Electric Energy Sources |
| --- | --- |
| Utility generated electricity | Acetylene |
| Photovoltaic solar modules | Propane |
| Diesel and petrol engine driven generators | Butane |
| Primary & secondary battery cells | Kerosene |
| Wind generators |  |
| Wave activated generators |  |
| Fuel cells using alcohol or hydrogen |  |
| Thermo nuclear devices |  |

The most popular method of powering AtoN is by utility generated electricity if it is available. If it is not available then by solar powered photovoltaic cells charging secondary battery cells.

IALA has created a series of documents to assist in the selection of electrical power systems for aids to navigation.

Refer to IALA publications:

- Guideline 1067-0 on Selection of Power Systems for Aids to Navigation and Associated Equipment

- Guideline 1067-1 on the Total Electrical Loads of Aids to Navigation

- Guideline 1067-2 on Power Source

- Guideline 1067-3 on Electrical Energy Storage for Aids to Navigation.

- Guideline 1039 on Designing solar power systems for AtoN (The Solar Power sizing to ??

Refer to:

- Applicable national standards for the safe handling of gases.

## Electric - Renewable Energy Sources

### Solar Power (Photovoltaic cell

Solar power is an ideal power source for many aids to navigation applications. It offers:

- a power source with no moving parts;

- no maintenance requirements other than being cleaned;

- slight deterioration in power output over its life;

- low life-cycle costs;

- enhanced electrical safety on extra low voltage systems.

When used to power a light, the battery recharging process is separated from the operation of the light source so that the recharge voltage can be optimized without detriment to the light’s operation.

Potential difficulties associated with solar power are:

- AtoN exposed to icing conditions are perhaps the only applications unsuited to the use of solar modules.

Types

The three common technologies employed in the manufacture of silicon based solar modules are listed in Table 36.

Silicon Solar Cell Technology

| Technology | Comments |
| --- | --- |
| Monocrystalline Cells | Made from a thin slice cut from a single large crystal of silicon, usually produced as a circular section rod. Generally have the highest efficiency of the three technologies. If circular wafers of silicon are used, the module fill factor is significantly less than with polycrystalline cells. It is now usual for the cells to be trimmed to approximate a square. |
| Polycrystalline Cells | Made from a thin slice cut from a large cast billet of silicon comprising many crystals. Are slightly less efficient than the mono crystalline cell but they can be shaped to completely fill the module. |
| Thin Film Technology | Made by depositing thin films of silicon directly onto a glass or stainless steel substrate a thin slice cut from a single large crystal of silicon. The cell has a lower efficiency than either of other technologies but can be multi-layered for enhanced performance. Problems have been found with lifetime of these cells. |

In addition to the silicon cell technologies, there are two optional module configurations based on the numbers of series connected cells. The standard module normally has 36 cells in series to give an open circuit voltage of around 20 volts. For all battery charging applications, a voltage (charge) regulator is considered essential.

Modern developments in electronics have allowed voltage (charge) regulators to be developed that use maximum power point tracking (MPPT). This ensures that they operate the solar module at a level to obtain the maximum power, for any given level of irradiance. This operating level is independent to the battery charge voltage level. This technology can lead to up to 30% more output than would be achieved with conventional voltage regulators and can ensure effective solar charging in locations where high ambient temperature exists. It should be noted that better output is achieved when the panel voltage is at least twice that of the nominal battery voltage.

Module or Array Orientation

In the northern hemisphere, solar modules are normally installed facing south and inclined at an angle to the horizontal that is related to the latitude of the site such that they can maximise output during the period of the year when irradiance is least, and vice versa for the southern hemisphere. The inclination angle for solar modules is often optimised for the particular site as part of the sizing calculations.

One of the main problems experienced with solar powered aids to navigation has been bird fouling. Numerous, innovative solutions have been trialled, generally with mixed results. Generally solar modules mounted at an angle or vertically benefit from self washing from rain.

The cost of additional solar modules needed for a vertical installation may be largely off-set by the savings that result from simplifying the mounting arrangements or framework.

Refer to IALA publications:

- Guideline 1012 on Protection of Lighthouses and other AtoN against damage from Lightning

- NEW GUIDELINE due 2021 on What constitutes a good Marine Solar Panel

- Guideline 1091 on Bird deterrents and bird fouling solutions

### Wind Energy

Aids to Navigation Applications

Wind generators (or wind turbines) have been used by a number of IALA Members to power their aids to navigation.

The most popular type are horizontal axis machines with a two or more bladed (propeller type) turbine.

The maintenance requirements arising from the number of moving parts of a wind generator and susceptibility to storm damage, has limited the wide use of wind generators and they are not considered as reliable as photovoltaic cells but can provide a good back up power supply where solar irradiance is poor due to latitude, fog or high power requirements.

Installations

Wind generator installations at aids to navigation sites pose a number of problems:

wind generators tend to require a lot of maintenance if operated in turbulent air flows;

- if the wind generator is installed on a separate mast some distance from the aid to navigation, consideration has to be given to the inherent cable voltage drop;

- operation of wind generators to power aids to navigation needs to take into account the impact it may have on any environmental factors associated with the location, such as flora and fauna.

### Wave Energy

The wave activated generator (WAG) was developed in Japan and has been successfully used to power lighted buoys. The interaction between the buoy and wave motions acts as a simple air pump that is used to drive an air turbine and electricity generator. The WAG is mounted on an extension of a hollow tail tube that passes through the buoy hull. With wave heights of 0.5 metres, the power output can be as much as 100 watts. WAGs have limited life and current systems suffer from excessive wear.

Site conditions will determine the rate at which the tail tube of the buoy accumulates weed and other forms of fouling. These aspects need to be taken into consideration when developing the maintenance regime for the WAG. WAGs can also be very susceptible to storm damage.

## Rechargeable Batteries

### Principal Types

There are various types of storage battery technologies applied to aids to navigation – lead acid Lithium-ion and nickel cadmium. The lead acid type is generally preferred because of its lower cost, good energy exchange efficiency (95% vs. 80%) and general reliability and availability. . Lithium-ion is becoming much more popular with its higher power density though is more expensive to purchase. The nickel cadmium battery can operate in low temperatures and for a greater number of deep discharge cycles.

Recently, new secondary battery technologies have appeared, including nickelmetal-hydride (Ni-MH) batteries, lithium-iron phosphate (LiFePO4) batteries, and lead crystal batteries. These batteries offer good weight and a greater cycle life for a given capacity but come at a premium cost.

Battery technology is a rapidly developing field with power density to size and weight improving all the time, this technology is being driven by the automotive sector and will undoubtedly migrate into AtoN applications.

Lead Acid

The basic form of this battery uses a lead dioxide positive plate and a pure lead negative plate immersed in an electrolyte of dilute sulphuric acid.

Lead acid batteries are available in two main designs, flooded lead acid and valve regulated (VRLA). The VRLA comes in two types, absorbed glass-mat (that use a micro glass separator system to absorb the electrolyte), and gel batteries, that use a jellified electrolyte and polymeric separators to prevent short circuits between the positive and negative plates.

Nickel Alkaline Battery

These batteries use compounds of nickel and, generally, cadmium with a solution of potassium hydroxide as the electrolyte.

Nickel-cadmium cells use perforated steel plates that hold the active material, mainly a nickel hydroxide in the positive plate and a cadmium compound in the negative plate. The construction is generally referred to as a “pocket plate” cell.

A range of valve regulated nickel-cadmium batteries that use a recombination process now complements the traditional flooded cell design. Under normal float charging conditions any gas produced is recombined within the battery and water loss is negligible. However if the battery is overcharged it will vent but water can be added if necessary.

Lithium ion Battery.

The Lithium ion battery is a fast developing technology with many established variants on the market offering proven performance, their general advantages are a high energy density with potential for yet higher capacities, they do not need conditioning when new to acquire maximum capacity, they have a relatively low self-discharge and do not require a periodic discharge to maintain performance. They are also suitable for low and high power applications.

The disadvantages of the Lithium ion battery are that they require a protection circuit to maintain voltage and current within safe limits and are subject to transportation restrictions due to safety concerns. They are expensive to manufacture and there is emerging debate about the social impact of the raw materials required to make them.

Battery Disposal

A number of countries now have standards and regulations relating to the safe and environmentally acceptable methods of disposing or recycling of batteries as well as the environmental and social impacts of acquiring battery raw materials. This should be a key factor when selecting a suitable battery for an AtoN application.

### Primary Battery Cells

Primary battery cells provide electrical energy by a non-reversible chemical process. They were used in large numbers up until the 1980’s to operate buoys and automatic beacon lights. The usage of primary cells has declined sharply since commercial solar power (photovoltaic) modules have become available. A related issue that hastened the decline of primary cells was the tightening environmental standards in a number of countries that required cells to be recovered from site for disposal in an approved manner. Disposal compliance costs, and occupational health and safety aspects of the frequent replacement of primary cells have worked in favour of converting to renewable energy sources. They remain a useful power source where solar irradiance is low or where theft is prevalent as they are less attractive to steal.

Zinc-Air Cell

The zinc-air primary cell was a common energy source for operating buoy and beacon applications. The cell uses a porous carbon block to supply oxygen from the air through an alkaline electrolyte to oxidize a zinc anode. Individual

primary cells have an open circuit voltage of about 1.2 volts and can supply 1000 to 2000 Ah at a maximum rate of about 1 ampere.

Sealed Alkaline Battery

This type is commonly used in some countries, and has the benefits of good low temperature performance.

### Internal Combustion Engine/Generators

Diesel Generators

Diesel engine driven generators are often used as the primary source of electrical power where the location of an aid to navigation is too remote to be supplied from utility generated electricity and the power demand is high. Diesel generators are also used to provide emergency or backup power.

The generator capacity to support the operational and domestic loads of a standard lighthouse is in the region of 10kW. Smaller generators in the range of 2 to 5kW, combined with batteries and inverter-charger systems are now available to meet this variable load. This arrangement can be more suitable and flexible if the load is likely to be light for extended periods, with short periods of heavy loads.

The requirement for diesel generators in lighthouses is decreasing as a result of:

- reduction in electrical load;

- improved efficiency of renewable energy sources.

Petrol Engine Generators

Petrol engine generators are a useful source of power for maintenance work, but are less common in permanent installations due to:

- fuel storage and transportation safety issues;

- maintenance requirements on the spark-ignition system;

- the petrol engine generally being regarded as less durable than a diesel.

Fuel Cell

This is a solid-state device that uses a catalytic process to oxidise fuel to generate an electrical current. A common fuel is hydrogen, or hydrogen rich fuels such as Methanol. It can be thought of as a continuously fed battery, ideally preferring a constant load.

The fuel cell is now commercially available, although the technology is still being further developed. Fuel cells offer a reliable and environmentally friendly energy source for supplementing AtoN power supplies.

Fuel cells do present an environmentally suitable solution, as methanol can be manufactured from sustainable sources and the by-products of the generation of electrical energy is heat and water. There is some interest in the use of fuel cells in hybrid power systems with wind energy or solar energy.

## Electrical Loads and Lightning Protection

### Electrical Loads

IALA has prepared a standard methodology for calculating and defining the load profile of electric aids.

Some of the loads that this methodology covers are:

lights;

RACONs;

AIS AtoN;

audible warning signals;

visibility detectors;

monitoring and telemetry systems;

charge controllers.

Refer to IALA publication:

•	Guideline 1067-1 Total Electrical Loads of Aids to Navigation.

### Lightning Protection

To ensure reliable operation of aids to navigation during electrical storm events, both physical lightning protection and zoned surge protection should be considered. IALA has produced guidelines to describe practical methods for the design, installation, inspection and testing of lightning protection systems. The information covers lightning protection for aids to navigation structures, equipment and systems.

Refer to IALA publication:

•	Guideline 1012 on the Protection of Lighthouses and Aids to Navigation Against Damage from Lightning.

## Non-Electric Energy Sources

Historically, non-electric energy sources were frequently used in aids to navigation, however, the use of electric energy sources is currently the norm and is the recommended practice for new installations. There are various non-electric power supplies, the main types used in aids to navigation are acetylene and propane.

Acetylene

Acetylene (C2H2) has been used to operate lights on buoys and unattended aids to navigation for many years. Acetylene can explode if compressed directly, but can be safely contained under low pressure in special cylinders when dissolved in acetone. The manufacture of acetylene, standards for the cylinders and the filling process are usually controlled by government regulations.

Acetylene has been a convenient and reliable energy source for aids to navigation. However appropriate attention should be given to:

safe handling of cylinders;

the broad range of explosive mixtures with air (between 3 and 82% acetylene);

the purity of the gas;

minimizing leaks in pipe work and fittings.

Propane

Propane gas (C3H8) has been used as an alternative fuel to acetylene, particularly in buoys. Although propane has to be consumed in an incandescent mantle burner to provide a white light, it has several advantages over acetylene:

it is a by-product in oil refining processes;

its abundance and low cost;

propane liquefies at a pressure of 6 atmospheres at 17ºC, and can be transported in low weight and low cost gas bottles;

propane will maintain a positive pressure down to -40ºC and will not freeze in conditions likely to be encountered at sea;

placing the bottles in pockets in the buoy or by filling it directly into the body of an buoy, or pressure vessel;

the comparable containers are the 20 kg propane bottle with gross weight of 48 kg and the 7,000 litre acetylene cylinder, weighing 105 kg;

furthermore the cost of the propane bottle is only about one third of that of a acetylene cylinder;

propane is a particularly safe gas, as only some 6% of all its possible mixtures with air are explosive against a figure of 80% for acetylene;

burns cleanly without the risk of sooting that can occur with a poorly adjusted acetylene burner.

Refer to:

Applicable national standards for the safe handling of gases.

Notes (??)

## Environment

IALA encourages all members to deliver AtoN services in environmentally responsible manner in line with its motto "Successful Voyages, Sustainable Planet"

Aids to navigation play a critical role in protecting the environment by preventing maritime disasters that could have potentially catastrophic ecological consequences at sea and on shore. However, the aids to navigation equipment and activities themselves can create significant environmental damage through pollution, waste generation, and the disruption of ecosystems. It is essential to minimise these negative impacts so that the benefits of aids to navigation are not outweighed by unintended harm to the environment, and to eliminate the potential for pollution and waste of the Earth’s limited resources.

International standard ISO 14001 provides a framework for environmentally responsible service delivery.

IALA is committed to align its activities and that of its members with the 17 United Nations Sustainability Goals

Refer to IALA publications:

Recommendation R1004 on Sustainability in the provision of marine AtoN

Guideline G1036 on Environmental Management.

### Hazardous Materials

Mercury

A number of historic lighthouses still utilise rotating glass lenses and mercury float pedestals. This was a clever method for providing a heavy lens with an almost frictionless bearing so that it could be turned by a clockwork mechanism. However, given the toxic and corrosive properties of mercury, the following information may assist Competent Authorities to implement appropriate safety procedures.

The mercury float pedestal for a first-order rotating lens[5]contains about 13 litres of mercury. Quantities of mercury may also be found in the electrical slip-ring units in rotating lamp array lighting equipment, some tilt-action switches, high current contact breakers, manometers and thermometers. It is being phased out by some Authorities now due to the tighter legislative restrictions about its use and transport.

Physical Properties

Mercury is a heavy metal that has the unusual property of remaining liquid at normal temperatures (above – 38°C).

Spill Risk

The mercury in a lighthouse optic system does not present a significant hazard, unless personnel come into contact with “uncontained” mercury as a result of accidental spills. Such events are usually the result of a mishap during maintenance work, or as a result of a natural disaster such as an earth tremor that displaces mercury from its containment bath.

If spilt, the mercury can enter cracks in floors, and is readily absorbed into porous surfaces such as concrete, masonry and timber. When broken into small globules or droplets, the surface area and vaporisation rate rises rapidly. Minute droplets will adhere readily to dust and can form particles that can be inhaled.

Mercury is a corrosive substance if it comes into contact with metals such as zinc and aluminium.

Occupational Hazard

The occupational hazard associated with mercury relates to:

Vapour Inhalation: Some vaporization from a free mercury surface will occur at normal room temperature and this is the most likely first contact that lighthouse personnel will have with mercury. Unless the mercury vapour levels have been measured, personnel are unlikely to be aware of the hazard. If the work-space around lighthouse equipment containing mercury is not well ventilated, the concentration levels can rise above recommended limits and there is potential for mercury poisoning. Mercury vapour is heavier than air and in still air will tend to concentrate in low parts of the work-space. Well designed ventilation will allow such concentrations to disperse.

Ingestion: This can lead to acute mercurial poisoning.

Skin Absorption: Mercury is not easily absorbed through the skin.

Precautions It is essential for the Authority to have detailed and strictly managed working procedures for all personnel working with, or in close proximity to mercury.

Staff must be trained in these work procedures and regularly medically monitored to ensure that they do not become contaminated with mercury.

The working procedures must follow national health and safety regulations and should be written by an expert in this field.

For work on optics the procedure will cover emptying, cleaning and re-filling the optic bath. Clean-up procedures will detail methods to recover all visible particles of mercury and the use of chemicals to neutralise smaller spills.

Personal protective equipment must be supplied that is specifically designed for use with mercury. This will include overalls, gloves, overshoes, respirator and eye protection. Procedures for the safe storage and disposal of this equipment must be in place.

A mercury vapour meter must be available to monitor the working environment and procedures in place for regular testing and calibration.

Consignment

Mercury is a hazardous substance and the relevant national and international regulations must be followed with regard to the type of container to be used, the packaging of this container for transport and the marking of this packaging.

Both IMO and the International Air Transport Association (IATA) have regulations covering the transportation of mercury.

Paints

Aids to Navigation authorities use a significant quantity and variety of paints and related surfacing materials. There is potential for hazardous situations to arise and for environmental pollution. For example:

storage of inflammable paints and solvents;

during surface preparation and removal of paint prior to repainting;

contact with vapours and solvents during application;

clean-up and waste disposal.

Lead and Isocyanate

Lead and Isocyanate based paints have been widely used in the past, but are now restricted or prohibited in some countries. Authorities maintaining older lighthouses are likely to be faced, at some stage, with having to remove lead based paint and disposing of the waste.

Members are encouraged to assess the risks and to adopt appropriate measures to safeguard maintenance personnel and the environment.

Antifouling Coatings

Antifouling paints contain biocides and are applied to vessels and floating aids to navigation to reduce the accumulation of marine organisms. For service vessels the antifouling paint assists to minimise fuel consumption.

On buoys and lightvessels the build-up of marine growth is not particularly detrimental. In view of the concentration of these types of aids to navigation in port approaches and internal waterways, less toxic paint systems may be preferred to minimise environmental pollution.

A particular group of antifouling paints using Tributyltin (TBT) has been banned from use. For further information, consult the International Convention on the Control of Harmful Antifouling Systems on Ships, published by the International Maritime Organization (IMO).

Asbestos

Asbestos is a carcinogenic substance that was (and in some areas, still is) used in the construction of buildings. Asbestos has excellent properties and with its resistance to fire, mould, rot and electrical insulation made it a material of choice through the early 20th century and many Authorities now have a regulatory duty to manage the safe retention or disposal of all asbestos in their estate. Asbestos is highly dangerous to anyone breathing in the microscopic fibres and it can take many years for the symptoms to become evident to the exposed person in the form of Mesothelioma which is generally fatal. Asbestos must be treated with care and relevant regulatory guidance on handling and disposing of it must be followed.

## Preservation of Historic Aids to Navigation

The IALA Advisory Panel on the Preservation of Lighthouses, Aids to Navigation, and Related Equipment of Historic Interest (PHL) was established by the IALA Council in 1996 in response to membership interest in the heritage value of lighthouses. In 2002, this Panel became part of the IALA Committee on Engineering, Environment, and Preservation (EEP) now AtoN Engineering and Sustainability (ENG) Committee. This work was supported further by the Incheon Declaration signed at the IALA General Assembly in Incheon, RoK in 2018

Objectives of the Incheon Declaration are to:

- Raise awareness of the need for the conservation and sustainable management of historical lighthouses as cultural heritage;

- Recognize that the significance of historical lighthouses extends beyond the navigational and architectural value to include maritime culture and history, social history, environmental aspects and that there is great value in documenting, researching and interpreting these for the benefit and inspiration of future generations;

- Conserve and develop historical lighthouse sites and associated maritime cultural spaces in order that people of all ages and backgrounds can appreciate and enjoy their respective nations’ maritime contribution and culture;

- Support IALA’s efforts to further sustainable lighthouse management through knowledge sharing, training and international cooperation projects; which will strengthen the capacity of coastal nations to manage their lighthouses in a coordinated manner and will encourage good conservation practices and foster interest in lighthouse heritage globally;

- Note the launch of the Incheon Project designed for IALA and its members to provide assistance for developing nations through education, training, technology transfer and capacity building as well as contributing to the advancement of global aids to navigation by means of various programmes and management in efficient and coordinated manner as an important legacy of the 19th IALA Conference in Incheon, Korea in 2018.

Examples of work accomplished to date:

- the creation of the format for an IALA database for recording details of historic lighthouses;

- The development of The Lighthouse of the Year award, based on international nominations and awarded on World AtoN day each year.

- Planning of a Heritage Workshop to share ideas and experiences on Lighthouse heritage.

- The approval of Recommendation R1005 on Conserving the build heritage of Lighthouses and other AtoN

- Publication of the Complimentary Lighthouse use Manual.

| Restoring a Heritage Lighthouse - Photo courtesy of Instituto Hidrografico (Portugal) | Lighthouse Interpretative Display - Photo Courtesy of IALA |
| --- | --- |

Refer to IALA publications:

- Lighthouse Conservation Manual;

- Recommendation R1005 on Conserving the build heritage of Lighthouses and other AtoN

- Guideline G1093 on Management of surplus Lighthouse property

- Guideline G1049 on the Use of Modern Light Sources in Traditional Lighthouse Optics;

- Guideline G1063 on Agreements for Complementary Use of Lighthouse Property;

- Guideline G1074 on Branding and Marketing of Historic Lighthouses;

- Guideline G1075 on a Business Plan for Complementary Use of a Historic Lighthouse;

- Guideline G1076 on Building Conditioning of Lighthouses;

- Guideline G1080 on the Selection and Display of Heritage Artefacts;

### Lens Size and Terminology

Information on terminology for historical glass lens systems and the typical amount of mercury held in mercury float pedestals (for rotating lens systems) is provided in Table 40.

Table 40 - Terminology for Historical Glass Lens Systems and Associated Quantities of Mercury

### Third Party Access to Aids To Navigation Sites

IALA acknowledges that Authorities face an increased demand to share aids to navigation sites with “third parties”. While it is important to ensure that the integrity and security of aids to navigation are maintained, the presence of a third party may be beneficial:

- in reducing the risk of vandalism;

- as a source of revenue or sharing of operational costs (e.g. power, road maintenance, etc);

- as a means of monitoring the operation of the aid.

If an Authority receives a request for a third-party installation, it should first establish whether such involvement is permitted in the Authority’s legislation. If there are no impediments, the Authority may consider negotiating an agreement with the potential third party to clearly establish the responsibilities and liabilities of each party. The agreement may also address:

- conditions to apply to the third party installation and operation to ensure that the equipment does not compromise the integrity and security of the aids to navigation and other property owned by the Authority;

- access to electrical power. At sites with main power, it may be advisable for the Authority to require separate metering of the third party supply so that electricity costs can be recovered;

- if no main power is available, it is reasonable to require that the third party provide its own power supply;

- where practical, the installation of the third party equipment should take into consideration and preserve the heritage value of the aid to navigation.

Authorities should reserve the right to cancel any third party agreement if continued use jeopardizes the performance or functionality of the aid to navigation.

| # Chapter | # Radionavigation services (PNT) |
| --- | --- |

## Positioning, Navigation and Timing

Positioning, Navigation and Timing (PNT) information is widely used across the maritime sector.  It supports vessel navigation, e-Navigation services and provides timing and positioning information for maritime Aids to Navigation.

Over the past decade it’s become clear that reliance on PNT has grown dramatically while at the same time the threat of signal disruption or corruption has grown. There is a clear need for resilience in the provision of PNT information as it underpins so many aspects of maritime navigation, communications and maritime AtoN operations.

Resilience may look like and mean different things depending on the operation at hand, the threat faced and available local options.  It is widely recognised that one solution cannot mitigates all threats in all locations, and what is needed are a scalable, configurable solutions through what is often referred to as a system-of-systems approach.

Global Navigation Satellite Systems (GNSS) are widely used as the primary means of navigation today and that is expected to remain so into the future.  Resilience is achieved when GNSS PNT information is supported by data from other, independent, PNT sources and combined in such a way that the integrity of the data is maintained and the mariner or AtoN service provider can retain their PNT solution, should any of the PNT sources fail.

There are many different systems, which can provide PNT information, and this chapter introduces the most prominent.

Refer to IALA publication:

Recommendation R1017 – Resilient Positioning, Navigation and Timing

## Satellite Positioning and timing

Global Navigation Satellite System (GNSS) is the generic term for satellite navigation systems that provide timing signals that enable the users’ receiver to calculate its position, velocity and time.

GNSS consist of three segments – a satellite constellation, a ground segment and the user’s receiver.  The satellites continuously transmit coded signals in one or more frequency bands. A user’s receiver, positioned anywhere on the earth’s surface, can use these signals to determine their position and velocity in real time, based on ranging measurements.

If a GNSS is recognised by the IMO as a component of its World Wide Radio Navigation System (WWRNS), as set out in IMO Resolution A.1046 (27), the receivers of that GNSS are deemed to satisfy the IMO carriage requirements for position fixing equipment referred to in Chapter V of the SOLAS Convention.

GNSS receivers, in combination with other equipment, can be used to obtain:

absolute positioning;

relative positioning (this can be further processed to derive speed over ground (SOG), course over ground (COG), etc.); and

timing

This information may refer to a stationary observer (static positioning) or to a moving observer (kinematic positioning).

There are several GNSS available today, with GPS, GLONASS, BeiDou and Galileo recognised as components of the IMO’s WWRNS. It is planned that regional GNSS components like QZSS and NAVIC will become operational in the next few years. GPS, GLONASS, Galileo, BeiDou, QZSS and NAVIC operate interoperable services under the framework of the International Telecommunication Union (ITU).

### Global Positioning System (GPS)

The Global Positioning System (GPS) became fully operational in 1995. The system is operated by the United States Air Force on behalf of the United States Government.

GPS provides two levels of service. The Standard Positioning Service (SPS) provides accurate positioning to all users since it is available for peaceful civil, commercial, and scientific use. Otherwise, the Precise Positioning Service (PPS) provides full system accuracy to those designated and authorised users among U.S. and allied national security systems.

The GPS Space Segment consists of a nominal constellation of 24 satellites in six orbital planes. The satellites operate in circular 20,200 km (10,900 nm) high orbits at an inclination angle of 55 degrees and with a 12-hour period.

The GPS SPS is available on a non-discriminatory basis, free of direct user fees, to all users with an appropriate receiver. The service satisfies the requirements for general navigation and harbour approach with a horizontal position accuracy of 9 metres (95% probability) [2]

A modernisation program aims to improve the accuracy and availability for all users and involves new ground stations, new satellites and four additional navigation signals: three new civilian signals known as L2C, L5 and L1C and a new military code termed M-Code.

Further information on GPS can be found at the USCG NAVCEN website (www.navcen.uscg.gov). The website also has a link to the latest United States Federal Radionavigation Plan that provides a comprehensive account of current and future developments for GPS.

### Global Navigation Satellite System (GLONASS)

GLONASS is a three-dimensional positioning, velocity and time system managed by the Russian Space Agency on behalf of the Russian Federation.

It is available on a non-discriminatory basis and free of direct user fees to all users with an appropriate receiver. With a full complement of 24 satellites, the service satisfies the requirements for general navigation and gives an average User Range Error of 1-6m (95%).

Recent satellites have introduced a second civil signal, with future satellites expected to provide a third civilian signal on the L3 frequency.

GLONASS satellites use Frequency Division Multiple Access (FDMA). However new satellites will provide additional signals using code division multiple access (CDMA) to become interoperable with other GNSS.

Equivalent to the Standard Positioning Service (SPS) and the Precise Positioning Service (PPS) of GPS, GLONASS provides a standard precision (SP) navigation signal and a high precision (HP) navigation signal, namely the Channel of Standard Accuracy (CSA) and Channel of High Accuracy (CHA), respectively. The Open Service (OS) provides positioning and timing service with the open access provided by way of the aggregate ranging FDMA signals generated by the GLONASS OC, broadcast at L1 and L2, and whose performance is associated with that of CSA (Chanel of Standard Accuracy) in the GLONASS service volume. The OS with the associated CSA performance is available to any user globally and continuously.

GLONASS High Precision signal is broadcast in phase quadrature with the SP signal providing full system accuracy to those authorised users by the Russian Armed Forces.

Further information on GLONASS and future developments, is available on the Information and Analysis Centre’s website (https://www.glonass-iac.ru/en/).

### BeiDou

BeiDou Navigation Satellite System (BDS) is China’s independently constructed and operated GNSS system. It can be compatible with other GNSS in the world. BDS can provide all-time, all-weather PNT services with high accuracy and high reliability for all kinds of users.

BDS consists of three major components: the space constellation, the ground control segment and the user segment. The global constellation consists of 3 GEO satellites, 3 IGSO satellites and 24 MEO satellites. BDS now can provide the following services:

The Open Service (OS)

With positioning accurate to 10 meters globally and to 5 meters in Asia-Pacific region free of user charge. The OS can be used on one (B1C or B1I) frequency and the better performed B1C is priority recommended. It is also recommended to use two (B1C and B2a ) frequencies.

The Message Service.

As for message services in China and surrounding areas, the service capacity will be increased to 10 million times per hour and the receiver transmission power will be reduced to 1-3W, capable of transmitting 1,000 Chinese characters per message (14,000 bits). As for global message services, the service capability is 40 Chinese characters per message (560 bits).

The Search and Rescue (SAR) service is a contribution of BDS to the Cospas-Sarsat MEOSAR program through the provision of a Forward Link Alert Service (FLS). In addition, Return Link Service (RLS)/BDS is provided through B2b signal.

BDS Precise Point Positioning (PPP) Service Signal PPP-B2b is transmitted by BDS GEO satellites. The PPP-B2b signal serves as the data broadcasting channel for correction parameters, such as satellite precise orbit and clock offset parameters of BDS and other GNSS, and provides PPP services for users in China and surrounding areas, with the dynamic precise position service accuracy at decimetre level and static precise potion service at centimetre level.

Further information on BDS can be found via website (www.beidou.gov.cn).

### Galileo

Galileo is the European GNSS designed to be interoperable with other GNSS, managed and operated under civil control. Galileo uses a constellation of 24 satellites to achieve its positioning performance targets but aims to have a constellation of 30 satellites when fully operational (including in-orbit spares).[5]

Galileo Initial services were declared in 2016 together with the recognition of IMO as part of the World Wide Radionavigation System. Galileo is offering today the following services:

1.	An Open Service (OS): With horizontal position accuracy of 2 metres (95%), free of user charges and providing positioning, time and synchronisation information.

2.	Search and Rescue Services (SAR): EU contribution to the MEOSAR system of COSPAS-SARSAT. Galileo search-and-rescue service forwards since 2017 distress signals to Cospas-Sarsat Mission Control Centres by detecting emergency signals from beacons and relaying messages to them in near real time that are then sent to the Rescue Coordination Centres (Forward Link Service). Galileo also delivers since 2020 an acknowledgment message to beacons with the return link capability to inform the user that the alarm has been received (Return Link Service).

3.	High Accuracy Service (HAS): A free access service complementing the OS by delivering high accuracy data and providing better ranging accuracy, enabling users to achieve sub-meter level positioning accuracy. With HAS, Galileo will be the first constellation able to provide a high-accuracy PPP service globally directly through the Signal in Space. High-accuracy data will be transmitted using an open format in the Galileo E6-B signal and via the internet for Galileo and GPS (single and multi-frequency) to achieve real-time improved user positioning performances (positioning error of less than two decimetres in nominal conditions).

4.	Public Regulated Service (PRS): Encrypted navigation service restricted to government-authorised users, for sensitive applications that require a high level of service continuity. PRS is primarily intended for use by EU Member State government authorised military or civil users, e.g. emergency services and police.

### Regional systems

#### Quasi-Zenith Satellite System

Japan is developing a Quasi-Zenith Satellite System (QZSS). QZSS is based on three satellites in highly elliptical, inclined orbits and one geo-stationary satellite. The final constellation is expected to consist of 7 satellites, with each transmitting 6 signals in the L-band: 3 in L1, 1 in E6, 1 in L2 and 1 in L5. The signal in E6 (L6) aims to support a commercial service with high data rate (2 kbps). Full implementation will also provide augmentation services to GPS and QZSS.

Further information is available at http://QZSS.go.jp/en/.

#### Indian Regional Navigational Satellite System

The Indian Regional Navigational Satellite System (IRNSS) with an operational name of NAVIC (Navigation with Indian Constellation) will be an independent navigation system covering the Indian region through a space segment of 3 GEO satellites and 4 IGSO satellites. The inclination of the orbital plane of the IGSO satellites is low, so that all the satellites can be seen simultaneously over India.

Three NAVIC services are:

Open Service using signals in the L5 and S bands;

Precise Positioning Service using signals in the L5 and S bands; and

Restricted Access Service using signals in the L5 band only.

The Open and Precise services target dual frequency users, but it is also intended to compute and broadcast ionosphere-corrections to support single frequency users. Owing to the limited coverage of the NAVIC network of reference stations, the satellites will, apart from the navigation payload, also include a dedicated C-band uplink/down-link ranging payload to support precise satellite orbit determination.

## Terrestrial positioning and timing

Within this section, different terrestrial navigation and timing systems are considered, noting that the use of Radar for absolute positioning is captured in Section x.

### Loran-C

Loran–C is a hyperbolic radionavigation system that was developed during the 1960’s to meet U.S. Department of Defense requirements. The Russian Federation operates a similar radionavigation system called CHAYKA.

Loran-C chains comprise between three to five stations that have a spacing of 600 to 1000 nautical miles. The signal format is a structured sequence of specially designed radio pulses on a carrier wave frequency centred on 100 kHz. One of the stations is designated as the ‘master’ and transmits groups of 9 pulses. The other stations are called ‘secondaries’ and these transmit groups of 8 pulses.  The spacing between groups of ‘master’ pulses from a single chain is a characteristic unique to that chain and is referred to as the Group Repetition Interval (GRI).

The 100 kHz carrier wave frequency favours the propagation of a stable ground wave over long distances. Careful signal design allows Loran receivers to determine positions using the ground wave and reject the delayed sky wave that would potentially distort the received signal.

The transmissions from each chain are monitored and controlled continuously. System abnormality indicators are built into the signal format and can be identified by the receiver providing inherent integrity warnings.

### eLoran

Enhanced Loran (eLoran) is a terrestrial navigation system developed from Loran-C. It is a Positioning, Velocity, and Timing (PVT) service for use by land, sea and air navigation, as well as other applications reliant on timing data.

eLoran is independent of, and has dissimilar failure modes to GNSS, and therefore complements GNSS in support of resilient PNT. eLoran provides positional accuracy in the region of 8 - 20 metres and time and frequency performance (to stratum-1 level) similar to current GNSS.

eLoran differs from Loran-C as it uses an all-in-view method of operation, calculating the distance to all eLoran stations in view. eLoran stations are synchronised with, but independently of, GNSS time. Synchronising to a common time source allows receivers to employ a mixture of eLoran and other systems, such as GNSS.

eLoran receivers calculate the distance from each station by firstly assuming that the entire earth’s surface is covered in sea-water. By knowing the speed of the signal over sea-water, along with the times of transmission and reception, a pseudorange can be calculated. This pseudorange is then adjusted to take into account the propagation delays due to the signal passing over land. These delays are called Additional Secondary Factor delays (ASFs). ASFs are measured by the service provider and supplied to users as a database built into their receivers. ASFs may change slightly due to weather or seasonal effects, reducing the efficiency of the correction and affecting accuracy. However, this is resolved by installing a differential Loran reference station nearby, which is able to measure the difference and calculate a correction. The correction information is then passed to the eLoran station for dissemination to the user over the eLoran data channel.

The inclusion of a data channel as part of the main transmission is one of the inherent features of eLoran. It can be used to provide other data services in addition to differential corrections.

The eLoran signal is standardised by SAE within its Transmitted Enhanced Loran (eLoran) Signal standard (SAE9990).

Refer to IALA publications:

Recommendation R011- The performance and monitoring of eLoran services in the frequency band 90-110 kHz.

Guideline G1125 – The technical approach to establishing a maritime eLoran Service

### Ranging mode

Ranging mode (R-Mode) is a terrestrial positioning system under development. It uses the frequency bands of existing maritime radio infrastructure for the provision of timing signals that enables GNSS independent position and time estimation. At present, the MF band of the IALA radio beacon system and the VHF bands AIS, ASM and VDE-TER of the VDES are being used in R-Mode testbeds in Europe, Asia and North America.

Three transmitters must be received to perform R-Mode based positioning at sea. Due to the different range of the transmitter of about 250 km for MF and several tens of km for VHF, which strongly depends on the antenna tower height, the coverage of MF and VHF R-Mode service can differ noticeably. Both signals are subject to different effects of signal attenuation, interference and distortion which decreases the performance. The combination of MF and VHF R-Mode signals can noticeably increase the R-Mode positioning performance.

All R-Mode transmitters are synchronized to an R-Mode system time. Depending on the synchronization source and technology to perform synchronization, the system time refers to a time scale which is traceable to UTC. Usually GNSS is used for synchronisation today within the testbeds. To reduce the dependence or to be completely independent from GNSS, R-Mode stations would need to have atomic clocks or another means for synchronization.

Refer to IALA publication:

Guideline G1158 – VDES R-Mode

## Augmentation Services

The aim of GNSS augmentation services such as Differential Global Navigational Satellite Systems (DGNSS) is the improvement of GNSS-based positioning in a given area. In this context, various methods can be applied to increase the accuracy of GNSS-based positioning, and to verify the integrity of applied components (systems, services) and provided data.  An essential basis for the provision of DGNSS service is own GNSS measurements gathered in real time at single reference stations or a network of stations.

There are two main approaches, ground based differential services such as the IALA radio beacon DGNSS system and space-based services, such as Satellite Based Augmentation Systems (SBAS).  The following sections introduce each system in more detail, however they work in similar manners – they both compare the performance of high-grade GNSS receivers to the known location of the antenna in order to observe position errors introduced by the earth’s atmosphere and other such sources.

In safety-critical applications, DGNSS services should also include integrity monitoring functions to ensure signals are valid and fit for use.  Such monitoring can be realised by plausibility and consistency tests, as well as methods estimating error behaviour and budgets, to provide integrity warnings in real time to the end user, informing them is satellite or system should not be used.

The DGNSS service provision is realised by radio signals carrying augmentation, correction and integrity data. Users operating in service areas and equipped with appropriate receivers can use this augmentation data to:

enhance accuracy of GNSS based positioning;

notify of faulty satellites or GNSS failure;

detect satellite signals with increased propagation errors;

exclude disturbed signals from positioning; and

be informed about the usability of services or other information.

Currently, DGNSS services are provided for operational satellite navigation systems such as GPS and GLONASS. In principle, similar DGNSS services can be provided for developing GNSS such as GALILEO, BeiDou and QZSS.

Each DGNSS service can be separated into two parts - generating and distributing the augmentation data. The generation of DGNSS augmentation data requires own GNSS measurements gathered at a single reference station or a network of stations. Different DGNSS messages and services may use different generation methods and means of dissemination. At present, certain communication channels used for the provision of DGNSS augmentation data are assigned to specific DGNSS services. For example, the provision of DGNSS augmentation data is realised by SBAS.

Refer to IALA publication:

Recommendation Rxxx Provision of GNSS augmentation services for maritime navigation

### Terrestrial augmentation systems

#### IALA Beacon DGNSS

IALA beacon DGNSS provide differential corrections, as well as integrity information, to nearby maritime users to improve accuracy and integrity of GNSS based determination of position, velocity and time data (PVT).

The method of differential positioning was developed in the 1990’s, is internationally accepted and supported in most coastal waters, especially in areas of high traffic density. By placing high grade GNSS receivers at know surveyed locations, it’s possible to calculate the difference between the estimated GNSS position and the true surveyed location.  The difference for each satellite’s pseudorange are observed from which corrections are determined and shared with nearby users, enabling their receiver to improve their estimated position.

Additionally, integrity monitoring functionalities assess the usability of GNSS signals, ensuring mariners employing such marine radio beacon DGNSS service use valid and healthy satellite signals.

The radio link used for the provision of DGNSS correction and integrity data defined by ITU (Recommendation ITU-R M.823-3). For such services, the DGNSS signal transmission is realized in the maritime radionavigational band (283.5 to 325 kHz)[6]. At user sites, type-approved DGNSS radio beacon receivers (meeting IEC 61108-4 test and performance standards) are necessary to enable the ship-side use of DGNSS services for an improved PVT data determination.

The full list of radio beacon based DGNSS stations (as notified to IALA by aids to navigation authorities) is at the IALA website (www.iala-aism.org).

Refer to IALA publications:

Recommendation R1115 (Provision of maritime radionavigation services in the frequency band 283.5-315 kHz in Region 1 and 285-325 kHz in Region 2 and 3);

Recommendation R0121 (Performance and Monitoring of DGNSS Services In The Frequency Band 283.5-325khz); and

Guideline G1112: (Performance and Monitoring of DGNSS Services in the Frequency Band 283.5-325kHz).

#### AIS for DGNSS Transmissions

Automatic Identification System (AIS) is a ship to ship and ship to shore data exchange and broadcast system, operating in the VHF maritime band. It is described in more detail in Section 7.

AIS has the capability of providing DGNSS corrections to onboard equipment using standardised transmissions (Message No 17) as described in IALA Recommendation A-124.

#### The BeiDou Ground Augmentation System.

BDS utilizes mobile communication networks or the Internet to provide users within the coverage area of reference station network, with high-precision positioning services at meter, decimetre, centimetre and millimetre levels.

#### Maritime Phase-Based GBAS (MGBAS)

In the past few decades, the development of phase-based measurement techniques was driven by surveying needs, to achieve position accuracies with GNSS in the centimetre level. In IALA Recommendation on the Future of DGNSS (R-135), the RTK technique is mentioned as an approach to meet maritime requirements on high-precision positioning in port areas and for docking. Several manufacturers of maritime GNSS/DGNSS equipment provide solutions supporting RTK-based positioning.

#### RTK over AIS

In survey applications, the RTK correction information is usually distributed to users via VHF/UHF radio modems or via commercial broadband internet. However, when used in hydrographic measurements further away from the shoreline, these communication options might not be always available. The communication options in these areas would then be via satellite or via AIS (the latter is also available only within coastal VHF coverage, usually up to 50 - 70 kms from the coastline).

RTK over AIS is in operational use for selected user groups in some countries and it has been reported to function without major problems and deliver the required positioning accuracy.

When using RTK over AIS, it should be noted that it puts a high demand on the VHF Data Link. Other limitations of this technique are that only one mobile user can be served by one AIS base station at a time, there is reduced understanding of accuracy due to rapid atmospheric fluctuations and that it may not be applicable in areas of high channel loading. The channel loading problem may be addressed in the future by using the additional channels allocated for VDES.

It is noted that RTK is a short-range system, and that there is a need to introduce monitoring and assessment of the integrity of RTK services and RTK based positioning in the context of safety-critical applications.

### Satellite Based Augmentation Systems (SBAS)

SBAS support wide-area or regional augmentation through the collection of data from reference stations located across a region, with differential data provided to the user via satellite-broadcast messages. The basic arrangement is to use a set of monitoring stations (at precisely known positions) to receive GNSS signals. Data from these stations are processed in order to obtain estimations of the errors that are also applicable to the users (i.e. ionospheric errors, satellite position/clock errors, etc.). Once these estimations have been computed, they are transmitted to the users by means of a GEO satellite.

There are many SBAS current in operation or planned, as shown in Figure 1.

Existing and under definition SBAS systems (Source: GSA User Technology Report 2018)

Refer to IALA publication:

Guideline G1129 – The retransmission of SBAS corrections using MF radio beacon and AIS

Guideline G1152 – SBAS maritime service

#### Wide Area Augmentation System (WAAS)

WAAS has been implemented by the US Federal Aviation Authority (FAA) to support the use of GPS for general and commercial aviation over continental United States. It was recently extended to cover parts of Mexico and Canada. At present, the WAAS architecture includes 38 reference stations, 3 master stations, 4 up-link stations, 2 geostationary satellite links and 2 operational control centres. Further information on WAAS can be found on the USCG Navigation Centre website (www.navcen.uscg.gov).

#### European Geo-stationary Navigation Overlay Service (EGNOS)

EGNOS is the European satellite-based augmentation system that provides safety critical navigation services to aviation, maritime and land-based users over most of Europe. EGNOS augments the GPS L1 Coarse/Acquisition (C/A) civilian signal by providing corrections and integrity information.

EGNOS provides three services:

Open Service (OS), freely available to any user. The main objective of the EGNOS OS is achievable positioning accuracy by correcting several error sources affecting GPS signals (EGNOS-SDD)

Safety of Life (SoL) Service, provides the most stringent level of signal-in-space performance developed primarily to support aviation (EGNOS-SDD); A service in the maritime sector is under development with a specific service provision layer including performance monitoring and the promulgation of Maritime Safety Information (MSI). The publication of the IEC Standard for SBAS receiver tests is needed (PNW 80-996 ED1).

EGNOS Data Access Service (EDAS) is the EGNOS terrestrial data service which offers ground-based access to EGNOS data in real time and also in a historical FTP archive to authorised users (e.g. added-value application providers) (EGNOS-SDD). The EGNOS Space Segment comprises 3 geostationary (GEO) satellites. The EGNOS Ground Segment comprises a network of Ranging Integrity Monitoring Stations (RIMS), two Mission Control Centres (MCC), six Navigation Land Earth Stations (NLES), and the EGNOS Wide Area Network (EWAN) which provides the communication network for all the components of the ground segment.

Further information on EGNOS can be found at www.egnos-portal.eu.

#### BeiDou Satellite Based Augmentation System (BDSBAS)

BDSBAS is an important part of BDS. The constellation of BDSBAS includes 3 GEO satellites operating at an altitude of 35,786 km and are located at 80°E, 110.5°E, and 140°E.  These satellites use Pseudo Random Noise (PRN) code 144, 143 and 130, respectively. The coordinate system of BDSBAS is WGS-84.

The deviation of BDSBAS service network time to GPS Time is within 50 nanoseconds. BDSBAS provide the Single Frequency (SF) service through BDSBAS-B1C signal and the Dual-Frequency Multi-Constellation (DFMC) service through BDSBAS-B2a signal for users in China and surrounding areas.

Further information on BDSBAS can be found via website (www.beidou.gov.cn).

#### Multi-Satellite Augmentation System (MSAS)

In Japan, the Multi-Satellite Augmentation System (MSAS) is a SBAS similar to EGNOS and WAAS. MSAS has been commissioned for aviation use, with two GEO-links using the L1 band via dedicated satellites, shared with communications and meteorological missions. The system has been operational since 2007 and there are plans to add additional services in the future.

Further information on MSAS can be found via the website: (https://www.mlit.go.jp/koku/15_hf_000105.html).

#### GPS-Aided Geo Augmented Navigation System (GAGAN)

India has developed its own SBAS called GPS-Aided Geo Augmented Navigation system (GAGAN). The system provides signals primarily for aviation users over the Indian landmass and is the first to cover the equatorial region.  As the system conforms to ICAO standards it is interoperable with other SBAS and provides a link between EGNOS and MSAS.

Further information on GAGAN may be found at www.isro.org.

#### System for Differential Corrections and Monitoring

Russia is developing an augmentation to provide corrections for GLONASS and GPS called the System for Differential Corrections and Monitoring (SDCM). This system will consist of 3 geostationary satellites, assigned PRN codes 125,140 and 141.

#### Korea Augmentation Satellite System (KASS)

Republic of Korea is developing a Korea Augmentation Satellite System (KASS), which is an SBAS similar to WAAS and EGNOS. KASS includes 7 reference stations, 2 master stations, 2 up-link stations, and 2 operational control centres. Upon completion of system development and establishment, scheduled in 2019, KASS will begin to provide its open service in 2020. It will then undergo a series of system certification processes in the next several years before its provision of safety of life (SoL) service in the aviation sector in October 2022. Further information on KASS can be found via website (www.kass-eng.re.kr).

### Integrity options

Augmentation systems check the data provided by each satellite and constellations, to ensure they are valid and operating within normal ranges.  Any satellite or constellation found to be working outside of the norm, is marked as unhealthy and not used in the user’s position estimate.

This test is known as integrity.  There are two different approaches, system level and user level integrity.  System level integrity checks the operation of the system at a set, normally static, location – such as the reference stations used within marine radio beacon DGNSS and SBAS solutions.   Such systems are able to identify system level failures, aspects such as erroneous satellite position data, clock errors or atmospheric effects.  System level tests are unable to determine or capture any integrity errors at the users’ location.

User level integrity is the name given to integrity checks that take place within the user’s receiver as these can include not just system level failure effects, but also local effects such as interference and multipath near to the user receiver.  It is anticipated that user level integrity and system level integrity will be required in the future as the reliance on PNT information grows.

#### Receiver Autonomous Integrity Monitoring

Receiver Autonomous Integrity Monitoring (RAIM) is a technology developed to assess the integrity of GNSS signals, and therefore the user level integrity of GNSS-based positioning. This kind of integrity monitoring is autonomously realized within the user’s receiver with special importance for safety critical applications, such as aviation and maritime.

Range measurements are required from at least four GNSS satellites to enable the determination of position, velocity and time data. However, the application of RAIM in a navigation receiver requires redundancy in the range measurements.

Safety-critical RAIM algorithms might use only “Fault Detection” (FD) or “Fault Detection and Exclusion” (FDE), which enables the continuation of operation in the presence of a single GNSS satellite and signal failures. To detect a faulty satellite, at least five range measurements are required, whereas to isolate and exclude a faulty satellite, at least six range measurements are required. While RAIM can detect many failure modes, it cannot detect some failures affecting multiple satellites.

The upcoming availability of various GNSS will increase the usable number of navigation signals for RAIM-based positioning. New and modernized GNSS supports the provision of GNSS signals in two or more frequency bands and therefore improves the capability of GNSS based ranging.

Future advancement in RAIM algorithms should improve the availability and continuity of RAIM based positioning.

Such enhanced RAIM techniques - called Advanced RAIM (ARAIM) - may become available to maritime users (https://gssc.esa.int/navipedia/index.php/ARAIM).

## Radar Aids to Navigation

Radar aids to navigation are devices that provide returns to a ship’s radar that help to locate and/or identify a navigation mark. The IMO carriage requirements contained in Chapter V, Regulation 19 of the SOLAS Convention 1974 (as amended), states all ships of:

300 gross tonnage and upwards are to carry a 9 GHz radar;

3,000 gross tonnage and upwards are to be fitted with a 3 GHz radar or, where considered appropriate by the Administration, a second 9 GHz radar.

Some administrations may impose other carriage requirements.

IMO Resolution MSC.192 (79) Adoption of the Revised Performance Standards for Radar Equipment (December 2004) states that 9 GHz radars should be capable of detecting radar beacons, SARTs and radar target enhancers. By omission, 3GHz radars are not required to detect radar beacons and SARTS. With the removal of the 3GHz radar racon detection requirement, ship-owners are free to use higher performing solid-state radars (previously known as New Technology (NT) radars), discussed below.

9 GHz radars are also extensively carried by vessels not covered by SOLAS or local regulation. Due to this high rate of carriage, radar aids to navigation in the 9 GHz band are especially useful.

### Radar Reflectors

A radar reflector is a passive device designed to return the incident radar pulses of electromagnetic energy back towards the source and thereby enhance the response on the radar display. By design, a radar reflector attempts to minimise absorption and random scattering effects. A radar reflector is generally installed as a supplementary device at sites that would also be marked with a light. The main objectives are to enhance:

target detection at long ranges (for example, for landfall navigation);

target detection in areas of sea or rain clutter; and

radar conspicuity of aids to navigation to reduce the risk of collision damage.

The performance of a radar reflector can be defined in terms of its effective radar cross section (RCS). This is a value determined by comparing the strength of radar signals returned by the radar reflector with the equivalent return from a radar reflective sphere of 1m2 reflecting area.

The range at which a radar reflector target can be detected is dependent on the heights of the radar antenna, the reflector and the output power of the radar. There are analogies to the geographical range of visual marks. The radar performance of corner cluster reflectors may vary considerably from one make to another.

Use of small radar reflectors can also be subject to multipath fading effects. Please see IALA Guideline No. 1010 on Racon Range Performance for a discussion on multipath fading.

Most radar reflectors are designed for use by 9 GHz radars. The reflectors can also be used with 3 GHz radars; however, the effective radar cross section is about an order of magnitude less.

### Radar Target Enhancers

A Radar Target Enhancer (RTE) is a device that amplifies and returns the pulse from a ship’s radar to give an enhanced image on the radar screen. The returned signal from an RTE is not coded. The RTE was designed primarily for buoys and small vessels that might normally carry a passive radar reflector. RTE testing has shown RTEs to have provided an effective radar cross section (RCS) of about 100m2, compared with an RCS of 20 to 30m2 for passive radar reflectors typically fitted to buoys.

To date, commercially available RTEs only operate in the 9 GHz band. RTE use is subject to multipath fading effects (please see Guideline 1010 on Racon Range Performance).

### Radar Beacons

Radar beacons (racons) are receiver/transmitter devices operating in the maritime radar frequency bands (9 and 3 GHz) that enhance the detection and identification of certain radar targets. Please note that IMO MSC.192 (79) has done away with the requirement for 3 GHz radar to detect racons.

A racon responds to the presence of a ship’s radar by sending a characteristic pulse train. The response appears as a coded mark (or “paint”) on the ship’s radar display (refer Figure 29) that highlights the range and bearing of the racon. The display paint can be fixed to a specified length or can be dependent on the radar range setting. It uses a Morse Code letter character for identification.

A Racon (left) and a Radar Display (right) with and without the Racon character

#### Racon Applications

A racon is generally considered to be a supplementary aid to navigation installed at sites that would also be marked with a light. The number of vessels capable of making use of a racon is effectively unlimited.

A racon can be used for:

ranging and identification of positions in ice conditions or on inconspicuous or featureless coastlines;

identification of aids to navigation, both sea-based and land-based;

landfall identification;

indicating centre and turning points in precautionary areas or Traffic Separation Scheme (TSS);

marking hazards;

indicating navigable spans under bridges; and

identifying leading lines.

#### Frequency-Agile Racon

A frequency-agile racon responds on the frequency on which it is interrogated, and the response can be re-painted on each radar sweep. The purpose of frequency agility is to provide a signal to the radar that is within the receiver bandwidth of the radar.

Racons operate in the 9 GHz band with horizontal polarisation, and/or in the 3 GHz band, with horizontal and optional vertical polarisation.

Preferred terminology for the description of Racon Operating Frequencies

| Preferred Terminology | Alternatives |  |  |
| --- | --- | --- | --- |
| 9 GHZ | 9300 9500 MHZ | X-BAND | 3 CM |
| 3 GHZ | 2900 3100 MHZ | S-BAND | 10 CM |

#### Racon Performance Criteria

The availability of a racon is the principal measure of performance determined by IALA. In the absence of any specific considerations, IALA recommends that the availability of a racon should be at least 99.8%.

#### 5.4.3.4 Racon Technical Considerations

There are a number of technical considerations in the use of racons to assist with the navigation of a ship:

To avoid masking other features on the radar display, the racon response is usually switched on and off on a pre-set cycle;

The angular accuracy of the bearing between the ship and racon depends entirely on the interrogating radar, while the accuracy of the range measurement depends on both the radar and racon;

When racons are used in leading line applications, an alignment accuracy of about 0.3 degrees can be expected; and

When a ship is very close to a racon, side-lobes from the radar antenna can trigger the racon. The resulting multiple responses on the radar display may be a distraction and can mask other targets. Side-lobe suppression techniques are standard features of frequency agile racons.

#### Use with solid-state Radars

All currently available and installed racons are designed for use with high power pulse (magnetron) radars. In comparison, solid-state radars (previously referred to as New Technology radar) use low power transmissions with long pulses. Due to the low received peak signal strength, long pulse at the racon and modulation technique, current racons may not detect solid-state radars and may not transmit a response usable by such radars. Studies have shown that pulsed solid-state radars are able to reliably trigger racons at shorter ranges than would have been achieved with a magnetron pulsed radar. The IMO regulations regarding 9 GHz radars and racons remain unchanged and although detection and triggering range might be reduced, it is the responsibility of manufacturers of 9 GHz solid-state radars meant to satisfy SOLAS carriage requirements to retain racon functionality.

Despite changes to the IMO regulations relating to 3 GHz racons, existing racons with 3 GHz capability will continue to be useful to 3 GHz pulse radars of both magnetron and pulsed solid-state variants. Advanced techniques of solid-state radars do not automatically mean that racons are no longer useable. Some manufacturers are continuing to provide racon compatibility in their solid-state 3 GHz radars.

Please refer to Recommendation eNAV-146 Strategy for Maintaining Racon Service Capability for more information on solid-state radars.

### Radar Referenced Positioning

Algorithms may be developed to allow the radar display to be overlaid upon the electronic chart using detectable recognised navigational features (racons, passive radar beacons or land edge patterns etc.). This technique, although unlikely to challenge the accuracy of a GNSS based position fix, might be adopted as part of a PNT integrity assessment and/or as a back-up in the event of GNSS service or equipment failure or corruption.

### Enhance Radar Positioning System

A new system known as Enhanced Radar Positioning System (ERPS) has been under trial for many years and results have been presented at IALA conferences in 2014 and 2018. ERPS uses enhanced racons (eRacons) that announce their location by encoding their identity and surveyed position into the response signals returned to radars. Enhanced radars (eRadars) then use the encoded data to calculate position solutions. Dynamic position accuracies of better than 28 meters with availability better than 87% have been measured during trials. ERPS is independent of GNSS and can be used as a back-up system in support of resilient PNT.

Refer to IALA publication:

Guideline G1010 on Racon Range Performance

Recommendation R0101 on Maritime Radar Beacons (Racons)

Recommendation R0146 Strategy for Maintaining Racon Service Capability

Recommendation R0113 for the Marking of Fixed Bridges over Navigable Waters

## Non-radio Positioning Systems

### Inertial systems

Many studies have been carried out on the integration of GNSS with Inertial Measurement Units (IMU) for maritime navigation. There exist various grades of IMU, from the very expensive navigation grade, through to tactical grade and low-cost units based on the Micro Electro Mechanics System (MEMS). The IMU grade characterizes the achievable performance of data provision covering velocities and orientations. A small IMU grade is associated with higher drift rates. Depending on the different drift rates, an IMU can provide contingency for various lengths of GNSS outages and support resilient PNT.

In combination with a GNSS compass, an IMU can provide accurate and stable heading data for longer GNSS outages. None of the currently available inertial systems is capable of maintaining all levels of navigation accuracy during a lengthy outage of GNSS. For ocean areas, both navigation and tactical-grade IMUs will give protection for appreciable outages over 15 minutes and navigation grade IMUs approximately 1 hour. For coastal areas, the required accuracy of 10 meter could be obtained for 3.5 minutes with a navigation-grade IMU and 1.5 minutes with a tactical grade IMU.

### ePelorus

An electronic pelorus (ePelorus) is a device for taking bearings of visual marks and converting them to an electronic format for input to an electronic chart system. Such a device would enable the integration of visual AtoN with e-Navigation.

The feasibility of constructing a low-cost ePelorus from commercial off-the-shelf components is being investigated to demonstrate its effectiveness as a backup, and to evaluate the potential for integrating visual AtoN with e-Navigation.

| # Chapter | # Vessel Traffic Services |
| --- | --- |

## Introduction

This chapter provides a first point of reference, basic information and guidance on where more detailed guidance related to Vessel Traffic Services may be obtained.

## Background

SOLAS Chapter V Regulation 12 (Vessel Traffic Services) states, inter alia, that:

Vessel Traffic Services (VTS) contribute to safety of life at sea, safety and efficiency of navigation and protection of the marine environment, adjacent shore areas, work sites and offshore installations from possible adverse effects of maritime traffic.

Contracting Governments undertake to arrange for the establishment of VTS where, in their opinion, the volume of traffic or the degree of risk justifies such services. Contracting Governments planning and implementing VTS shall, wherever possible, follow the guidelines developed by the Organization. The current guideline is IMO Resolution A.1158(32), Guidelines for Vessel Traffic Services (VTS).

## Definition of VTS

A VTS, as defined by IMO Resolution A.1158(32), Vessel traffic services (VTS) means services implemented by a Government with the capability to interact with vessel traffic and respond to developing situations within a VTS area to improve safety and efficiency of navigation, contribute to the safety of life at sea and support the protection of the environment.

## Purpose of VTS

The purpose of VTS is to contribute to safety of life at sea, safety and efficiency of navigation and the protection of the environment within the VTS area by mitigating the development of unsafe situations through:

Provision of timely and relevant information on factors that may influence the ship’s movements and assist onboard decision-making.

Monitoring and management of ship traffic to ensure the safety and efficiency of ship movements.

Responding to developing unsafe situations.

## Benefits of VTS

The benefits of implementing a VTS are that it allows identification and monitoring of vessels, strategic planning of vessel movements and provision of navigational information and assistance. It can also assist in prevention of pollution and coordination of pollution/emergency response.

## IALA VTS Manual

The IALA VTS Manual is acknowledged by the VTS community as being the most comprehensive guide to VTS as well as a point of reference for further detailed study.

The VTS Manual assists Contracting Governments, Competent Authorities and VTS Authorities in the harmonizing the delivery of VTS worldwide by providing a comprehensive overview on all aspects relating to the provision of VTS. In particular, the Manual provides guidance on:

The regulatory and legal framework for implementing and operating VTS.

The obligations of Contracting Governments and Flag States.

IALA Standards relating to the implementation and operation of VTS and their associated Recommendations, Guidelines, and Model Courses.

The Manual is also aimed at a wide readership to encompass all who are in any way involved with the policy for provision, operation and effectiveness of VTS, including those with management responsibility at national level and those who deliver services to the mariner.

The VTS Manual provides further information on, for example:

Regulatory and legal framework of VTS;

VTS Implementation;

VTS Operations;

VTS Communications;

Auditing and Assessing in VTS;

Additional Services;

Data and Information Management;

VTS Technologies;

Training and Assessment for VTS Personnel.

The VTS Manual is intended to complement IALA documentation relating to VTS. It is not intended to replicate the information and guidance in these documents or be prescriptive about the practices described within them. Rather, it provides a roadmap to assist authorities meet their obligations for the establishment and operation of VTS in a consistent manner.

The VTS Manual is available in digital format and will be updated regularly when new or revised documents have been approved. The latest version of the VTS Manual can be downloaded at [provide link]. [IALA Secretariat invited to include information on how to obtain the paper copy of the VTS Manual if appropriate]

| # Chapter | # TRAINING AND CERTIFICATION |
| --- | --- |

## Human Resources Challenges

One of the aims of IALA is to foster the safe and efficient movement of vessels through the harmonisation of aids to navigation services worldwide.

SOLAS (2004 edition) Chapter V, Regulation 13, states that, in order to obtain the greatest possible uniformity in aids to navigation, Contracting Governments undertake to take into account the international recommendations and guidelines when establishing aids to navigation’.

Recommendations and Guidelines produced by IALA clearly identify the role that IALA has to play in ensuring harmonized delivery of AtoN services.

In addition, Resolution 10 of the Standards of Training & Certification for Watchkeepers (STCW) code states that the contribution of vessel traffic service personnel contributes to the safety of life and property at sea and the protection of the marine environment. IALA addresses this aim in several ways, one of which is to recommend that Aids to Navigation and VTS Authorities ensure their staff receive a high standard of training. To assist with this approach, IALA Recommendation V-103 and E-141, together with associated model courses and supporting Guidelines, were developed. This approach provides a means to ensure VTS Personnel are trained to an agreed, minimum, level.

In addition, both the ARM and ENG Committees are currently developing the training requirements for AtoN Management and Engineering Personnel through the World Wide Academy (WWA).

### Source of Skills

Competent Authorities should ensure that all employees have the knowledge, skills and training to perform their duties effectively, and with safety. The term ‘employees’ includes newly hired, part time and temporary employees.

The ISO 9001 Quality Management standard places considerable emphasis on competence, awareness and training.

(See Section 8.7.1).

Refer to IALA publication:

•	Recommendation R0141 - Recommendation on Standards for Training and Certification of AtoN Personnel.

Notes

Reference SOLAS Consolidated edition 2014.

For VTS issues, please refer to Chapter 5 of the NAVGUIDE and the IALA VTS Manual.

For example with national, state, territory, and local governmental bodies and international organisations.

Only where the opportunity arises and where both are engaged in substantially similar work.

The quantity of mercury used in higher order optics is shown in section 8.11.1.

| # Chapter | # Digital communication technologies |
| --- | --- |

## Digital communications systems

This age is one of near-instantaneous communications and digital communications are now being introduced in the maritime environment.  Limitations of communications, such as transmission range, data speed and access cost are slowly and steadily being overcome.

In the coming years, the availability of robust, cost effective, maritime digital communication systems is expected to increase.

IALA has, hitherto, contributed significantly to the development of maritime digital communication system such as AIS and now VDES.

## IALA‘s Maritime Radiocommunications Plan

What is it?  What is its relevance to an AtoN authority?  Status?  Where to find it? Links to ITU work?

## AIS

The Automatic Identification System (AIS) is a VHF maritime mobile system designed as a tool for ship-to-ship collision avoidance and VTSs, and, as a means for littoral States to obtain information about ships and their cargo. AIS also allows for the exchange of safety related data between ships, from ship to shore and from shore to ship. There are numerous types of AIS devices, known as stations, which are identified by a unique Maritime Mobile Service Identity (MMSI) and use the AIS international open standard to communicate. AIS enables the automatic exchange of shipboard data from vessel sensors (dynamic data), as well as manually entered static and voyage related data, between one vessel and another and between a vessel and a shore authority using terrestrial or satellite communications. AIS has been mandated as a shipboard carriage requirement for vessels under the revised Chapter V of the International Convention for the Safety of Life at Sea, 1974 (as amended) (SOLAS 74) section 19.2.4. In addition, some administrations require AIS carriage on non SOLAS vessels. The main benefits of AIS are summarized below:

Data exchange, such as identification and course, between vessels within VHF range of each other, increasing situational awareness;

Data exchange between a vessel and shore authorities, such as a VTS, to improve traffic management in waterways, coastal and remote areas where AIS is sometimes the only mean to exchange data;

Automatic reporting in areas of mandatory and voluntary reporting;

Exchange of safety related information between vessels, and between vessels and shore station(s).

The development of AIS has expanded to include devices such as AIS for Aids to Navigation (AIS AtoN), AIS on search and rescue aircraft and AIS search and rescue transmitters (EPIRB-AIS, AIS-SART and AIS-MOB). The success of AIS has led to augmented use and growth, raising concerns over the reliability of the system as it becomes overloaded. Therefore, the International Maritime Organisation issued Resolution MSC.347(91) Annex 15 for the protection of AIS. This concern was also the driving factor for the development of the VHF Data Exchange System (VDES) discussed in the next section (8.4). Furthermore, the use of Autonomous Maritime Radio Devices (AMRD) on AIS frequencies has been limited by IMO and ITU to only those devices that increase the safety of navigation. These devices are referred to as AMRD group A.. For equipment that does not meet the criterion of improving the safety of navigation, AMRD group B, a new channel, 2006, where AIS technology can be used, was allocated at WRC2019.

IALA Guideline 1082 provides a comprehensive overview of AIS and an extensive listing of all AIS related documentation from various international organisations in its Annex E. The technical characteristics of AIS are available in document ITU-R M. 1371.

## VDES

The VHF Data Exchange System (VDES) is a radio communication system that operates in the VHF Maritime Mobile band. The VDES consists of 4 components:

The AIS uses channels AIS 1, AIS 2, CH75 and CH76. AIS 1 and AIS 2 are for terrestrial communications while satellite uplinks can use all channels, AIS 1, AIS 2, CH75 and CH76.;

The Application Specific Messages (ASM) component of VDES uses channels ASM 1 and ASM 2, for both terrestrial communications and satellite uplinks.

The VHF Data Exchange Terrestrial (VDE-TER) component of VDES uses channels 1024, 1084, 1025, 1085, 2024, 2084, 2025 and 2085 for terrestrial communications.

The VHF Data Exchange Satellite (VDE-SAT) component of VDES uses channels 1024, 1084, 1025, 1085,1026, 1086, 2024, 2084, 2025, 2085, 2026 and 2086, for both satellite uplink and downlink communications.

The sections below provide a short introduction to each VDES component other than AIS which was already discussed above. There are 3 IALA documents that provide additional information on VDES and its sub-systems.

G1117 -  VHF Data Exchange System (VDES) Overview

R1007 – The VHF Data Exchange System (VDES) for Shore Infrastructure

G1139 – The Technical Specification of VDES will be superseded by ITU.R M.2092-1

Additionally,  these ITU documents provide important technical information for VDES:.

ITU-R M.2092, Technical characteristics for a VHF data exchange system in the VHF maritime mobile band.

Report  ITU-R M.2231, Use of Appendix 18 to the Radio Regulations for the maritime mobile service

Report ITU-R M.2435-0 [VDE-SAT]

VDES frequency usage

### Why VDES includes AIS?

VHF Data Exchange System (VDES) comprises a suite of channels in the VHF Maritime mobile band and forms a contiguous block of frequencies that includes both international AIS frequencies as shown in the figure above. For this reason, the VDES was designed in such a way to simplify radio equipment complexity allowing a single “smart” radio box to perform all VDES functionalities, including AIS. Anchored in VDES design is the protection of AIS and the situational awareness it provides to mariners and shore authorities. As such, all AIS functionalities are a part of VDES just as they would be available on an independent AIS device. In principle, a VDES unit could replace the AIS equipment using the same antenna, power and connectivity. For shore authorities to make use of the full capabilities of VDES, improvements to the infrastructure might be necessary.

For more information on AIS, please refer to the section above specific to AIS.

### ASM component of VDES

The Application Specific Message component of VDES uses 2 channels (ASM1 and ASM2). The purpose of ASM is to offload the AIS channels from the growing use of various messages to exchange data that is safety related, but not directly relevant to collision avoidance. The deployment of ASM across the globe should relieve AIS frequencies from such usage and ensure the availability of  AIS for collision avoidance and safety purposes, increasing its effectiveness. Examples of ASM data that are currently carried over AIS frequencies include meteorological / hydrological data, waterways obstructions, recommended routes, etc. The ASM channels use a more efficient signal modulation than the original AIS frequencies which allows for more data, almost twice, to be carried in the same time slot/frame. Existing AIS messages could be simply reused on ASM or VDE channels.

IALA maintains a list of all available Application Specific Messages on all physical links (AIS, VDES-ASM and VDES-VDE)  that have been created and are used across the world for ASM purpose. The list can be found on the IALA website https://www.iala-aism.org/asm/  and where also new proposed ASMs can be registered.

### VDE-TER component of VDES

The purpose of the VDE-TER component is to provide a communication system capable of supporting e-Navigation growing requirements within VHF range (approx. 40 to 60 km). It is expected that ASM will not be able to meet the native transfer requirements of the S-100 suite (S-1xx, S-2xx, S-4xx, etc) of data products. As e-Navigation standardize on S-100 data products, the maritime safety data will be in need of a communication system with expanded capacity to carry this information to and from ships in waterways and coastal areas.  This is the gap that is filled with VDE-TER.

VDE-TER uses 4 duplex channels to achieve 100kHz of bandwidth available to carry data from ship-to-ship, shore-to-ship and ship-to-shore. The maximum total capacity of the channel is approximately 230kbps that is divided amongst the users (ship or shore stations) of the system in a specific area. For example, there could be one shore station and 5 ships using VDE-TER simultaneously to exchange data, each getting about 38kbps of throughput.

VDE-TER is providing much more capacity than AIS and ASM, but it is still an order of magnitude slower than other commercial communication systems such as cellular 3G/4G/5G. This limitation will require VDES to be carefully managed by shore authorities to make sure priority is given to safety related data pertinent to the ship’s context.

### VDE-SAT component of VDES

The purpose of the VDE-SAT is to extend VDE coverage beyond the shore-based coverage of VDE-TER and offer global coverage. As such VDE-SAT is an essential supplement to terrestrial VDE coverage provided by VDE-TER in support of e-Navigation services.

VDE-SAT use 2x150kHz, of which 2x100kHz are shared with VDE-TER and 2x50kHz are identified for VDE-SAT only. The VDE-SAT downlink support data rates ranging from about 2 kbps to 48 kbps, depending on link quality. The VDE-SAT uplink support data rates ranging from about 1 kbps to 95 kbps, depending on link quality and interference environment observed by the satellite. These data rates are total system rates for a 50 kHz channel of which can be allocated to and shared between 1 and 6 users.

VDE-SAT is capable of supporting the same services envisioned for VDE-TER. However, the reduced data rates and inherent store and forward operations mode of VDE-SAT may not be suitable for all services. As for VDE-TER the resources available in VDE-SAT should be carefully managed by the satellite operators to ensure priority is given to the appropriate services.

## R-Mode

As GNSS signals are not always reliable and subject to jamming and spoofing, it is desired to have alternative means of maritime navigation.  One approach for that is to equip communication systems on the shore with the option to transmit ranging signals (R-Mode), which vessels can use to determine their position when within range of these stations. The VHF Data Exchange System (VDES), which is currently in standardisation, can be utilized for this R-Mode application.

The demand for a resilient positioning, navigation and timing (PNT) solution was reflected by the ACCSEAS project and is being further developed by the R-Mode Baltic Project which investigates in the MF and in the VHF band additional ranging sequences.

The proposal for such an alternative system is termed Ranging-Mode (R-Mode).  R-Mode intends to utilize the shore based communication infrastructure, like DGPS (IALA beacons), Automatic Identification System (AIS) or VDE base stations [2] [9] with the existing housing infrastructure.

The accuracy of ranges between the base station on land and the vessel at sea, measured by means on the received radio signal, depends on the utilized bandwidth and of the received signal power versus the power of other noise sources.

## LTE - M as a communications infrastructure

The objective of LTE-Maritime or (LTE-M) is to provide high data rates in the order of megabits per second within the communication coverage of 100km from the shoreline.

In 2017, in order to confirm the feasibility of LTE technology on maritime field, a testbed was developed in the Republic of Korea and several field experiments were conducted. The experiment results show that, although there exist the interference issues with other communication signals, LTE-Maritime can provide the data rates over Mbps and the communication coverage around 100km.

### Merits of LTE-M

LTE-M is based on LTE technology that is a promising solution for wireless maritime network.

To support the requirements of various data services, the maritime communications providing high-speed data rates and extended communication coverage need to be developed. Unfortunately, the conventional communication systems of maritime field such as VHF, MF/HF were operated on the terrestrial radio frequency specified on the maritime missions of GMDSS convention. In case of satellite like Inmarsat, the channel capacity and operative cost are not reasonable for the private user purpose. They could be a good communication system on GMDSS missions globally but not a good solution to be used as a communication system for the various services like e-Navigation, especially in specific local water requiring high data rates for their own services.

LTE is capable of providing increased data rate, capacity, and spectral efficiency even in dynamic propagation environments with the support of advanced techniques such as multiple-input multiple-output (MIMO) and carrier aggregation (CA). Furthermore, it has the potential to provide the communication coverage about 100 km depending on the cell environments, though LTE for commercial mobile communication is designed with a relatively short cell coverage. This superiority of LTE makes us develop a single-hop network enabling ship-to-shore data communication based on LTE technology.

In general, the wireless mesh networks are vulnerable to link failures caused by radio interference and they could not assure reliability. Contrary to existing maritime networks for extending the communication coverage with multi-hop transmission, LTE-Maritime enables ships to directly communicate with onshore BSs and it could improve reliability. Therefore, it is more suitable especially for the safety related maritime services that require high reliability as well as low latency.

## Capture a suite of low-power systems like LORA, etc.

Text – but ensure don’t use propriety names

## Autonomous Marine Radio Devices

There are a number of maritime radio devices which operate autonomously. These include, but are not limited to:

Devices on towed or unpowered ships and barges, “man overboard” devices, diver locating, alerting and radiotelephony devices, fishing net marker buoys, oil spill tracking buoys, oceanographic and other drifting buoys.

There are some types of autonomous maritime radio devices (AMRD) that use AIS or digital selective calling (DSC) technology.  Or they can transmit synthetic voice messages, or with a combination of those technologies.

They have been developed for, and are operating in, the maritime environment.  Their number is expected to increase.

Some of these devices do not enhance the safety of navigation or serve the purpose of communication between coast stations and ship stations, or between ship stations, or between associated on board communication stations, or survival craft stations and emergency position-indicating radio beacon stations, but occupy the spectrum and identities of the maritime mobile service.

There is a need to categorize and regulate the usage of autonomous maritime radio devices.

- ITU, in 2019, adopted the preliminary draft definition of AMRD. The categorization of AMRD and relevant information are contained in the draft new recommendation ITU-R M.2135 and will operate on a new channel (2006). In principle these devices will use AIS technology but is not limited to this. The new recommendation could be published in 2022.

The generally agree IMO position is that:

the integrity of AIS and the Global Maritime Distress and Safety System (GMDSS) should be protected;

autonomous maritime radio devices which enhance the safety (of navigation) should be regulated for the use of frequencies and identities of the maritime mobile service; and

for autonomous maritime radio devices which do not enhance the safety (of navigation), regulation of the use of frequencies, and technical and operational characteristics, should benefit both the user of devices as well as maritime safety.

### AMRD Group A

Will consists of Mobile Aids to Navigation (MAtoN) and Man Over Board Class M (MOB). Their technical and operational characteristics are described in the most recent version of Recommendations ITU-R M.1371 and ITU-R M.493.

Mobile AtoN and MOB should use the numbering scheme defined in ITU-R M.585 “Identities in the maritime mobile service”, i.e. for MAtoN it is 99MIDXXXX.

### AMRD Group B

All other AMRD, that do not enhance the safety of navigation, are categorised as AMRD Group B. The characteristics of AMRD Group B were further developed and contained in two annexes, one for devices using AIS technology and the other for devices using other technologies.

AMRD Group B numbering scheme is still under consideration. The identity 979YYYYYY (not including the manufacturer ID) is proposed for the revised ITU-R M.585-7. However further work is needed and will be carried forward to the ITU WP 5B meetings.

## Emerging digital technologies

IALA has a focus on …………….

Some of the technologies evaluated have been ….3GPP….LTE-M ….others ….

### What is 3GPP?

The use of mobile communications has been being recognized as a very useful means of communications for ships at sea.

3GPP was created in 1998 by the signing of the "The 3rd Generation Partnership Project Agreement". 3GPP unites seven telecommunications standard development organisations (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA and TTC), known as “Organisational Partners”.  It provides their members with a stable environment to produce the reports and specifications that define 3GPP technologies.

The original scope of 3GPP was to produce technical specifications and technical reports for a 3G Mobile System based on evolved GSM core networks and the radio access technologies that they support (i.e., Universal Terrestrial Radio Access (UTRA) both Frequency Division Duplex (FDD) and Time Division Duplex (TDD) modes).  The scope was subsequently amended to include the maintenance and development of the Global System for Mobile communication (GSM) Technical Specifications and Technical Reports including evolved radio access technologies.

The project covers cellular telecommunications network technologies, including radio access, the core transport network, and service capabilities - including work on codecs, security, and quality of service - and thus provides complete system specifications. The specifications also provide hooks for non-radio access to the core network, and for interworking with Wi-Fi networks.

More detailed information about 3GPP can be found in http://www.3gpp.org/about-3gpp

### IALA’s position on 3GPP

IALA’s position is that communication based on 3GPP (currently 4G) technology is as an emerging technology for IALA and the maritime domain.

It may be beneficial to describe IALA members 5G requirements, identify other larger segments with similar requirements, join forces with the custodians of these segments and to work jointly with 3GPP and CIRM, as appropriate to adopt the requirements

In the review of elements related to 3GPP the role of CIRM and the IALA Industrial members was highlighted. In particular, the WG highlighted the opportunities to explore possibilities for closer cooperation between IALA and CIRM on development of technical issues of mutual interest (e.g. 3GPP systems, and digital HF and VHF). This cooperation could include participation in CIRM working groups and technical meetings

### 8.9.3 Digitalisation of marine VHF voice channels

Opening para ….

There are a number of options to implement voice over VHF.  These include:

digital Private Mobile Radio (dPMR) that uses Frequency Division Multiple Access (FDMA) and

Digital Mobile Radio (DMR) that uses Time Division Multiple Access (TDMA) technology.

IALA has assessed that digital voice over VHF, using the example of dPMR, is a suitable candidate for consideration in meeting the needs of IALA members.   However, a suitable vocoder needs to be identified as a standard for maritime use to ensure interoperability.

### Digital High Frequency radio

The data transfer of files and e-mails at a reasonable cost for the mariner is now possible with the digitalisation of the HF frequencies which has been study at ITU-R WP5B. The available HF data transfer protocols currently used in the maritime mobile service (MMS) for the exchange of data and electronic mail on MF/HF frequencies are described in the Recommendation ITU-R M.1798 (Characteristics of HF radio equipment for the exchange of digital data and electronic mail in the maritime mobile service). Three HF electronic mail systems and one wideband HF data exchange system for point-to-point communication are proposed in this Recommendation.

### How to assess new technologies?

IALA has a Guideline on ‘template for review of emerging technologies for possible use by IALA members. The objective of this guideline is to encourage identification of existing or emerging technologies that may be of interest to evaluate and identify their advantages, limitations and applicability of these technologies in consideration of user requirements and needs of IALA membership.  The guideline can be found at Xx …yy..

### New technologies

There are a number that IALA has looked at:

LoRA / LoRaWAN – a low power-wide area network (LP-WAN) system that enables long transmission with limited power consumption.  (https://www.mdpi.com/2079-9292/8/1/15/htm - tracking and monitoring system based on LoRa Technology for lightweight boats; https://www.ncbi.nlm.nih.gov/pmc/articles/PMC6163321/ Experimental study of LoRa transmission over seawater).  This technology will be reviewed at ENAV 25 (ENAV25-2.1.1, action item 26 refers).  This discussion may include a review of Sigfox if expertise is available.

LEO constellation developments, noting the growth in digital communications capabilities from different low earth orbiting satellites.  This technology will be reviewed at ENAV 25 (ENAV25-2.1.1, action item 25 refers)

NAVDAT – noting developments as presented at IMO, including the recent input to NCSR, this technology may be reviewed if expertise and input is received for ENAV25.

LiFi – LiFi, or light fidelity, is a communications capability based on Visual Light Communication (VLC) that uses LEDs to network a wireless system

Make the above generic - remove commercial names - so say for example we looked at for global services, low power WAN etc

It was noted that the listed technologies can be used for proprietary maritime communications but may not address standardisation or security aspects.

## Digital communications in VTS

It is expected that the use of digital communication by VTS will increase significantly in the near future as a means to enhance communications and reduce the opportunity for misunderstanding caused by a miss hearing and/or a language barriers in communication between ship and shore and ship and ship.

In addition to AIS, some VTSs use other digital communication system such as satellites, internet and mobile phone with not only vessels but also other stakeholders and plan to use emerging digital communication technologies such as VDES, NAVDAT and LTE-M for the provision of clear, concise and unambiguous communications in VTS.

### Developments in automation on board ships – implications for AtoN and navigation services

??

### IALA and GMDSS matters

IALA has an interest in maritime mobile services including Global Maritime Distress and Safety System (GMDSS) and radio determination service.

Services include emphasis on the development of VDES, VDE-SAT, AIS and autonomous radio devices (AMRD) operating in the maritime VHF mobile band.

| # Chapter | # Information services |
| --- | --- |

## Maritime Services

### Background

Stated simply, the goal of e-Navigation is to provide harmonised information in electronic formats, in a seamless, customised and efficient manner, to better-designed navigational systems on board.  Ashore, e-Navigation aims to streamline the way maritime authorities, agencies and other stakeholders gather and exchange information.

There are 16 digital maritime services agreed by IMO.  The international community is working to develop and harmonise the supporting digital messages that constitute each service.  The 16 services include three VTS services (information, traffic organisation and navigational assistance), a chart service and Maritime Safety Information (MSI).  These maritime services, provided digitally to ships by shore authorities, will form the core of the navigation services in the coming years.  The timely, bespoke, automated and error-free provision of safety-related information will result in a quantum increase in situational awareness on board ships and a commensurate increase in the safety of navigation.

### Terminology and Directives

Maritime Services in the context of e-Navigation refers to the provision and exchange of maritime-related information and data in a harmonized, unified format.

IMO Resolution MSC.467 (101) (Guidance on the definition and harmonisation of the format and structure of maritime services in the context of e-Navigation) provides more information on maritime services.

Examples of Maritime Services include, but are not limited to, vessel traffic services, maritime safety information, pilotage, tug and nautical chart services.  There may well be other services developed and implemented in the future.

The list of Maritime Services, as described in IMO’s “E-Navigation Strategy Implementation Plan – Update 1 (MSC.1/Circ.1595)”, is presented in the table below.  Each of these defines and describes the set of operational and technical services (and the level of service) provided by a stakeholder in a given sea area, waterway, or port, as appropriate (NCSR 1/ 28 Annex 7).  IALA has submitted initial descriptions for the MSPs 1, 2 and 3 and is recognized as a coordinating body for these, together with IMO (MSC.1/Circ.1610).

### Definition and Harmonisation of the Format and Structure of Maritime Services

At its 101st session, MSC approved MSC.467 (101) (the Definition and Harmonisation of the Format and Structure of Maritime Services in the Context of e-Navigation).  This resolution explains the process and responsibilities for defining a new Maritime Service.  It is important to note that any Maritime Service must be S-100 conformant as a baseline.

### More information on Maritime Services and e-Navigation

As an introduction to e-Navigation, section 0 provides “A lay-person description of e-Navigation”, which includes information on services, data models and operational and technical matters,  explained for a lay-person’s perspective.

Additionally, further IALA Guidance on this topic can be found in the following documents:

IALA Recommendation R1019 - Provision of Maritime Services in the context of e-navigation in the domain of IALA.

- This recommendation recognizes current developments in the domain of Maritime Services, presents general requirements for Maritime Services in the context of e-Navigation and can be seen as a directive for the development of new Guidance documents for this topic.

IALA Guidance on platforms to support the provision of maritime services in the context of e-navigation (G xxxx not assigned yet).

- This guidance document provides information on the establishment of harmonised platforms to support the implementation of VTS and AtoN maritime services in the context of e-Navigation. As new digital platform solutions are approaching in the domain of e-Navigation, a common sense on requirements for these is needed. This document helps IALA members to recognize appropriate platforms and encourages platform providers to adapt their solutions to common requirements. The document is currently developed by ENAV WG 1 as a guideline associated to IALA Recommendation R1019.

### Operational Services

What are they ?  How to establish etc ?  The work that WG 1 and other committees will do on developing guidance

ARM and ENAV Committees need to come to a conclusion on “Operational Services” and the NAVGUIDE should follow.  Based on the reaction to the input paper on Operational Services from ARM, this is not a separate distinction.  Also, would it be worth including the diagrams from the Layperson’s Description of e-Navigation on the relationship between Maritime Services, Technical Services, and Technical Specifications

The ENAV Committee is of the view to develop this via a group consisting of members from PAP, ARM and VTS.  Within this group, inconsistencies in terminology will be resolved and the implementation of maritime services will be described in a uniform way.  We can then use some of that work to populate this section.

### Technical Services

A Technical Service comprises a set of technical solutions, based on an agreed data model and communications means, to provide a Maritime Service as per IMO MSC.467 (101).  Based on the concepts of service-oriented architectures, a Technical Service also refers to a set of related software functionalities that can be reused for different purposes, together with policies that govern and control their use.  Therefore, harmonisation and interoperability of data models and interfaces of Technical services play a key role in the realization of Technical Services.

Technical Services should also be designed modularly.  In this way, services offered by one electronic device can directly be used by another electronic device without any modification.  Often, Operational Services are implemented by electronic devices or software components that rely on one or more Technical Service.

IALA is working on developing the operational and technical aspects of some maritime services.

### Establishing Technical Services

IALA Guideline G1128 on the specification of e-Navigation technical services enables service providers, consumers and regulatory authorities to have a common understanding of a technical service and its implementation.  The Guideline differs between the actual service specification (functionality and interfaces), technical designs (technology specific considerations) and instance descriptions (specific setup of a single service instance).  All sections of the guideline are characterized by a fixed scheme.  This enables the standardized specification of services in a Service Oriented Architecture.

A Technical Service should be formally specified and documented, as described by IALA Guideline G1128.  At the time of writing, this guideline aims at improving the visibility and accessibility of available Technical Services and information provided by them.  Data models of Technical Services should be implemented using the S-100 Universal Hydrographic Data Model (see section 9.2).

### Architectures for e-Navigation

The maritime domain is a complex ecosystem of multiple stakeholders and new and existing digital infrastructures.  Secure, reliable and efficient architectures are required to fulfil the requirements of a harmonized network of technical services.  The Maritime Architecture Framework is a framework for the design of maritime architectures.  It can be used to describe the context of e-Navigation digital architectures and implement them.

In the past few years, platform solutions for the connection of decentralized services and databases have been established in a large part of the transportation and other sectors.  IALA recognizes platforms as an important building block to implement the e-Navigation strategy, as they can facilitate harmonisation, interoperability and collaboration between different services providers and support Service Oriented Architectures (SOA).  SOAs enable services to interact with other services and furthermore, make it possible to build services based on existing services.  Additionally, IP-based communication and web services open the possibilities for an efficient and more fine-grained communication.  However, as this introduces a relatively new way of interacting with Maritime Services in a digital way, there are new requirements and risks that service architects and developers should be aware of.  IALA is currently developing a Guideline to address these issues (G-XXX: Guideline on Platforms to support the provision of Maritime Services in the context of e-Navigation).

Some content of the NAVGUIDE 2018 could also be included here, as it does not seem to be outdated.  Sections 4.6.1.2, 4.6.1.3 and 4.6.2 (without sub-sections) seem to fit well here

### Maritime Resource Names

The notion of identity is vital for all information exchange and communication technologies. Just as human-to-human communication on a global scale would be impossible without unique telephone numbers/email addresses, so too is a globally agreed digital maritime identity required for the various participating ‘actors’. In the maritime sector, there are already many identity systems in place (e.g. IMO number, MMSI). These are managed by different organisations. The challenge in creating a single maritime identity regime is to create a solution that satisfies the most common identification needs of the entire maritime industry, on a global scale, while also being compatible with already established identity systems. A key task is to establish authentication between maritime users, especially in the automated usage with digital maritime services.

Maritime Resource Names (MRN) are a concept for naming and identification of maritime entities, originally developed within IALA Committees.  The system is based on the existing  URN concept from the Internet Engineering Task Force (IETF), and has broad potential for application beyond IALA features.  MRN is included in the S-100 framework documents and applied throughout the S-100 family of products and services. The interconnection between maritime entities (including data) and a MRN is applicable in a wide range of use cases. MRNs could for example be used as a unique Identity representation for SOAs.

On its 65th session, the IALA council adopted the proposal to obtain and manage the ‘MRN’ domain of URN (C65‐11.4.4.1). Furthermore, the IALA ARM committee (in collaboration with the ENAV committee) developed a guideline relating to MRN; G1143, which was adopted by council meeting #69. The ARM committee is currently revising G1143, and edition 2 is currently underway.

For more information, see this footnote.

## IHO’s S-100 Universal Hydrographic Data Model

The S-100 Standard is a framework document that is intended for the development of digital products and services for hydrographic, maritime and GIS communities.  It comprises multiple parts that are based on ISO geospatial standards.

The IHO has developed the S-100 Universal Hydrographic Data Model (UHDM) to cater for future demands for digital products and services.

The S-100 Geospatial Information Registry contains online databases of concepts, features, portrayal information, attributes, metadata and other resources.  These items of information are relevant to those communities developing S-100 based products and services.  A centralised registry also allows for harmonisation between product specifications within the maritime sector.

### S-100 Role in e-Navigation

IMO is responsible for the overarching e-Navigation concept, which seeks to bring about increased safety and security in commercial shipping through better organisation of data on ships and onshore, and better data exchange and communication between the two.

IHO had already established a system of standardised methods of codifying, encapsulating, and subsequently transferring and distributing hydrographic and charting data in its S-57 Electronic Navigational Chart standard.  The concept can be applied into the entire e-Navigation concept by ensuring that the underpinning standards are in place from the beginning, through the S-100 family of product specifications and centralized information registry.  The IHO maintains no controlling role, other than for its own standards. The principles are outlined in IHO publication S-99, Operational Procedures for the Organisation and Management of the S-100 GI Registry, available at: www.iho.int. It is planned to introduce finalized S-100 products in the next years, initially with the simultaneous usage of S-57 products. Later, a complete transition to S-101 and other S-100 products is planned (NCSR7/22/x).

### S-100 Dependent Product Specifications

In order to manage the development of S-100 based products; and with a view to minimizing duplication and encouraging conformity, the IHO Hydrographic Services and Standards Committee (HSSC) allocates S-XXX numbers to be used for the development of S-100 dependent products.  The following Product Specification numbers have been allocated by the HSSC.

International Hydrographic Organization (IHO) (S-101 to S-199)

Some examples are

S-101 Electronic Navigational Chart (ENC)

S-102 Bathymetric Surface

S-103 Sub-surface Navigation

S-104 Water Level Information for Surface Navigation

S-111 Surface Currents

S-112 Open - (See Decision HSSC9/38)

S-121 Maritime Limits and Boundaries

S-124 Navigational Warnings (MSI)

- International Association of Marine Aids to Navigation and Lighthouse Authorities (IALA) (S-201 to S-299)

Some examples are

S-201 Aids to Navigation Information

S-210 Inter-VTS Exchange Format

S-211 Port Call Message Format

S-230 Application Specific Messages

S-240 DGNSS Station Almanac

S-245 eLoran ASF Data

S-246 eLoran Station Almanac

S-247 Differential eLoran Reference Station Almanac

S-2xx Port Collaborative Decision Making (PortCDM)

- Joint Technical Commission for Oceanography and Marine Meteorology (WMO/IOC JCOMM) (S-411 to S-414)

S-411 JCOMM Ice Information

S-412 JCOMM Weather Overlay

S-413 Weather and Wave Conditions

S-414 Weather and Wave Observations

### IALA and S-100 development

The current (as on 1 Jan 2021) edition of S-100 is Edition 4.0.0.  In that there is a section for online data Exchange.  IHO is progressing a refinement of S-100, which aims to include the concept of on-line communications.  Teething issues raised during the testing of a first proposal (from IALA) were dealt with.

As S-100 will be an increasingly used framework for the exchange of maritime information, ways for the secure, reliable, and efficient exchange of S-100 data must be taken into consideration. The standardisation of a common Web Service interface (based on IP-Technology) for exchange of S-100 based products will enable wider technical interoperability where the same service interface can be used for exchanging information regardless of operational use, i.e. common for several Maritime Services. Therefore, the IALA is working on a guideline for S-100 based Web Services (G-XXX: Guideline on Web Service Based S-100 Data Exchange). A general introduction to the topic and an overview of existing standardisation and guidance documents is given.

For the development of the S-2XX Standards, maybe someone from ARM(?) could contribute.

Maybe we can say something about the Product Specifications IALA is currently developing, and the dependencies between IALA and IHO.  If necessary, we could also illustrate the collaboration between IALA and IHO in various committees.  We could also be general, if the specifics aren’t needed (like not using references to the ENAV committee in Chapter 10).

## Cyber security

Classical cyber security solutions have mostly been developed for office IT systems and applications.  Cyber security for the maritime domain requires different priorities, management and operational characteristics and requirements.

According to emerging technology needs in the maritime domain, digital devices and software components will be developed and are expected to provide additional information to mariners. These new maritime digital necessarily require cyber security measures.

Cyber security of VTS, ships and maritime digital devices, etc. is emerging as a critical issue for the maritime environment and safety.  Since each characteristic is different, it is not advisable to apply one cyber security standard to all areas.  Therefore, the following strategic decisions are needed in applying international standards.

Applicable cyber security standard to VTS, Ship and Maritime digital devices

| Category | Standard | Title |
| --- | --- | --- |
| VTS | ISO 27001 | Information security management |
|  | NIST | NIST Cybersecurity framework |
| Ship | IEC 62443 4-2 | Technical security requirements for IACS components |
|  | IEC 62443 3-3 | System security requirements and security levels |
|  | IEC 61162 460 | Maritime navigation and radiocommunication equipment and systems – digital interfaces Part 460: Ethernet interconnection – safety and security |
|  | IEC 63154 | Maritime navigation and radiocommunication equipment and systems – Cybersecurity – General requirements, methods of testing and required test results |
| Maritime digital devices | To be considered above international standards as a reference |  |

JM – Maybe also make a reference to MSC-FAL.1/Circ.3. Not sure about the audience of this section. Maybe there should be a distinction between Users and Producers of digital maritime services / devices?

WG 1 – to develop closing paras

| # Chapter | # Other services |
| --- | --- |

## e-Navigation

### Introduction

The IMO-led initiative termed e-Navigation is a concept which covers many disciplines.  Earlier, section 9.1 has introduced and described the concept.

Stated simply, the goal of e-Navigation is to provide harmonised information in electronic formats, in a seamless, customised and efficient manner, to better-designed navigational systems on board. Ashore, e-Navigation aims to streamline the way maritime authorities, agencies and other stakeholders gather and exchange information.

This chapter provides a brief history of the development of e-Navigation and describes the areas that IALA has been involved in.

#### Background

#### Origins

In 2006, the International Maritime Organization (a proposal from seven of its Member States, which requested IMO develop an e-Navigation strategy.

The aim of the proposal was to “...develop a strategic vision for the utilization of existing and new navigational tools, in particular electronic tools, in a holistic and systematic manner.” (MSC 81/23/10). The sponsors of the submission were concerned that if the introduction of new technology remained uncoordinated, it would result in a lack of standardisation on board and an increased level of complexity. The proposed e-Navigation vision was to create an overarching system that would provide a greater level of safety and incident prevention, resulting in reduced navigation-related accidents.

IMO led other international organisations, notably IALA and the International Hydrographic Organization (IHO) and developed a strategy for the development and implementation of e-Navigation in 2008 (MSC85/26/Add.1, Annex 20).

A plan to implement the strategy, termed the Strategy Implementation Plan (SIP), was completed in 2014 (NCSR1/28 Annex 7).  This was reviewed and updated in 2018 (MSC.1/Circ.1595).

## IMO’s strategy for the development and implementation of e-Navigation

### The case for e-Navigation

The IMO strategy for e-Navigation (MSC 85/26/Add.1, Annex 20) states that about 60% of collisions and groundings are caused by direct human error. Despite advances in bridge resource management training, it seems that the majority of watch-keeping officers make critical decisions for navigation and collision avoidance in isolation. This is partly due to a general reduction in manning, it states.

The IMO strategy also states that in human reliability analysis, the presence of someone checking the decision-making process improves reliability by a factor of 10. If e-Navigation can assist in improving this aspect, through well-designed on-board systems and closer cooperation with vessel traffic management (VTM) systems ashore, the risk of collisions and grounding (and their inherent liabilities and costs to administrations) can be dramatically reduced.

#### Vision

The IMO vision for e-Navigation includes the following general expectations for on-board, ashore and communications elements:

#### On-board

Navigation systems that benefit from the integration of own ship sensors, supporting information, a standard user interface and a comprehensive system for managing guard zones and alerts. Core elements of such a system will include actively engaging the mariner in the process of navigation, to carry out their duties in the most efficient manner, while preventing distraction and overburdening.

#### Ashore

The management of vessel traffic and related services from ashore, enhanced through better provision, coordination and exchange of comprehensive data in formats that will be more easily understood and utilized by shore-based operators in support of vessel safety and efficiency.

#### Communications

An infrastructure providing authorized seamless information transfer on board ship, between ships, between ship and shore and between shore authorities and other parties with many related benefits.

### Definition

The IMO strategy defines e-Navigation as the “harmonised collection, integration, exchange, presentation and analysis of maritime information on-board and ashore by electronic means to enhance berth-to-berth navigation and related services, for safety and security at sea and protection of the marine environment.”

In other words, e-Navigation means:

The harmonised exchange and presentation of navigational information in electronic formats.

Harmonized data exchange and improved communications.

Creation of a "wide area navigation team", which allows the Officer of the Watch (OOW) and the Vessel Traffic Services (VTS) Operator to share tactical and planning information.

Improved design of navigational and communication equipment.

### What does the ‘e’ in e-Navigation stand for?

It is generally accepted that the IMO concept of e-Navigation can be thought of as a brand, without the need for ’e’ to be specifically defined. The concept of e-Navigation was first proposed by seven IMO Member States in 2006 as a process for the harmonisation, collection, integration, exchange and presentation of maritime information. As such, the ’e’ could have stood for ’enhanced’ or ’electronic’ (just like the ’e’ in e-commerce), but this would limit what could be done within e-Navigation. It must be noted that the generic term electronic marine navigation already exists in many forms. It should not be confused with this particular IMO initiative.

### Key elements

The key elements of the IMO strategy for e-Navigation, based on user needs include:

Architecture

Human element

Conventions and standards

Position fixing

Communication technology and information systems

Electronic Navigational Charts (ENC)

Equipment standardisation

Scalability

According to the strategy, the implementation of e-Navigation should be a phased, iterative process of continuous development, taking into account the evolution of user needs and the lessons learned from the previous phase(s).

As part of the basic requirements for the implementation of e-Navigation, it was agreed that e-Navigation should be based on user requirements and needs and not technology-driven.

### e-Navigation solutions

The centrepiece of the current (2018) SIP is the following five e-Navigation solutions:

S1:  improved, harmonized and user-friendly bridge design;

S2:  means for standardized and automated reporting;

S3: improved reliability, resilience and integrity of bridge equipment and navigation information;

S4: integration and presentation of available information in graphical displays received via communication equipment; and

S5: improved Communication of VTS Service Portfolio (not limited to VTS stations).

Solutions S1 and S3 promote the workable and practical use of the information and data on board.  As regards S1, IMO finalised:

Guideline on Software Quality Assurance and Human-Centred Design for e-navigation (MSC.1/Circ.1512)

Guidelines for the standardisation of user interface design for navigation equipment (MSC.1/Circ.1609)

Interim Guidelines for the Harmonized Display Of Navigation Information Received Via Communication Equipment (MSC.1/Circ.1593)

Guidelines for the presentation of navigation-related symbols, terms and abbreviations, (SN.1/Circ.243/Rev.2)

Amendments to the Performance standards for the presentation of navigation-related information on shipborne navigational displays (resolution MSC.191(79))

Guidance on the definition and harmonisation of the format and structure of maritime services in the context of e-navigation (IMO Resolution MSC.467 (101)).

Solutions S2, S4 and S9 focus on efficient transfer of marine information and data between all appropriate users (ship-ship, ship-shore, shore-ship and shore-shore).

## IALA’s Role

### IALA’s Strategic Vision 2018-2026

The aim of IALA is to foster the safe and efficient movement of vessels through the improvement and harmonisation of Marine Aids to Navigation worldwide. This purpose is given effect by two key goals for 2026.

Goal 1 (G1)

Marine Aids to Navigation are developed and harmonised through international cooperation and the provision of standards.

The strategy for e-Navigation is to improve and harmonise VTS, information structures, Maritime Service Portfolios and communications, so as to achieve worldwide interoperability of shore and ship systems.

Goal 2 (G2)

All coastal states have contributed to a sustainable and efficient global network of Marine Aids to Navigation through capacity building and the sharing of expertise.

Here, the strategy is to coordinate the further development of VTS, e-Navigation and short range Marine Aids to Navigation, taking into account new technologies and sustainability. Additionally, to continue to develop capacity building activities to improve the global operations and management of Marine Aids to Navigation systems and related services.

### IALA’s work on digital communication technologies, information services and e-navigation matters

IALA has so far focussed on three broad streams:

Digital Information Systems

Developing guidance on the description and implementation of “Maritime Services in the Context of e-Navigation”.

Facilitating harmonisation in the development of data models, technical services and platforms.

Defining concepts, procedures, services and platforms for new initiatives such as identity management.

Emerging Digital Technologies

Evaluation of new technologies relevant to e-navigation, in particular digital maritime communications

Maritime Autonomous Surface Ships (MASS)

Single Window Data Exchange

Digital Communication Systems

IALA’s Maritime Radio Communication Plan

The technical characteristics and operation of the VHF Data Exchange System (VDES)

Autonomous Maritime Radio Devices (AMRD), Automatic Identification System (AIS) and other digital communication technologies

### A Lay Person’s Description Of E-Navigation

E-Navigation is based on the principles of data exchange as used in the apps that operate on smart phones.

For example, a weather app provides information to a user, this could be called a “Weather Service”.  This weather service satisfies the user’s need for information about the weather. Similarly, a Maritime Service, in the context of e-Navigation, satisfies a user need for information concerning vessel navigation and other maritime considerations including safety, efficiency and the protection of the marine environment.  Our weather service app has to communicate with a server that runs software that can provide weather information.

The Concept of a Client-Server based Maritime Service

The interaction between the app and the server is defined by a technical service specification, which describes the exchange of standardised messages and the language that is used in the message contents. The language is described by a data model.

The software running on the server hardware is described as being an instance of the technical service.

The relationship between specifications of Maritime Services, Technical Services and data models in e-Navigation

If somebody wants to develop an app that is able to communicate with the server or develop a server to be used by the app, then the developer must refer to the technical service specification and the referenced data models.

Similarly, an e-Navigation Maritime Service draws together information through a combination of one or more running instances of a technical services. A technical service facilitates the exchange of data, by receiving messages, processing the data, and sending a result. The data is formatted according to the Common Maritime Data Structure, which is based on the IHO S-100 series of data models, thus ensuring harmonisation and interoperability.

The dataflow between Maritime Services, Technical Services and data exchange

We can further explore the relationship between Maritime Services, Technical Services and Data Models with reference to Figure 4, which further expands on our example of a Weather Service App.

Here we now assume that our weather service app is now part of a set of e-Navigation Maritime Services, and parallels can be made with Figure 3. The Maritime Weather Service App developed by Stormy Corp Ltd. is composed of several Technical Services. Each Technical Service exchanges messages with servers that contain various data about characteristics of the weather. In this example we are using Temperature, Wind and Cloud Cover data. These datasets are obtained from sensors distributed around a geographical region and stored in databases. It is the job of the technical services to extract relevant data depending on the requirements entered into the user interface of the Weather Service app. The Weather Service app connects to an instance of Technical Service 1 for Temperature, Technical Service 2 for Wind, and Technical Service 3 for Cloud Cover (or Sunlight).

Let us examine the characteristics of the architecture outlined in Figure 4:

A single Maritime Service can use several Technical Services. This is an example of a 1:N relationship between Maritime Services and Technical Services.

The instances of these Technical Services can be deployed by a single organisation OR by multiple organisations. In our case for example, BlowFelt Inc. host a Temperature Technical Service on their servers and provide their Temperature Data. BlowFelt Inc. also provide a Wind Technical Service and provide their own Wind database. But the StormyCorp Ltd. Weather Service App has chosen to use Cloud Cover data from GreySkies Inc. BlowFelt and GreySkies servers could be widely separate.

Instances of Technical Services can be re-used by other Maritime Services. In Figure 4 we see a Maritime Service that provides a prediction of the amount of solar energy that can be harvested; the Solar Energy Predictor App developed by a company called SunShine Inc. A user of the app might enter a geographical location into the user interface to gain an understanding of the energy harvesting capability of his remote solar powered autonomous ship at its current location. Notice that SunShine’s App could use the same instance of GreySkies’ Cloud Cover Technical Service as used by StormyCorp’s Weather App, or it COULD have used the one hosted by BlueSkies. Decisions on which service provider’s technical service instance to use could depend on perceived quality of provided data, cost of subscription or numerous other factors.

### An example of an IALA National Member’s vision

In 2019, the Australian Maritime Safety Authority released Navigation Services in Australian Waters—outlook to 2030.  This work provides an insight to the provision of navigation services (not just aids to navigation) in the coming years.  It outlines the emerging trends and drivers in navigation technology and communications. It also describes the anticipated impacts these will have on the maritime industry. Importantly, it lists AMSA’s policy responses to these changes.

It can be found here:

https://www.amsa.gov.au/safety-navigation/navigation-systems/navigation-services-australian-waters-outlook-2030

### Testbeds

Several multi-million dollar projects (completed and underway) have made noteworthy inroads in developing aspects of e-Navigation.

The IALA website (http://www.iala-aism.org/technical/e-nav-testbeds) provides more detail on known testbeds and their results.

IMO and IALA have guidelines to assist with the reporting of testbed results

Guidelines on Harmonisation of Testbed Reporting (MSC.1/Circ.1494)

IALA Guideline 1107 – Planning and reporting of e-navigation Testbeds (June 2016)

### 10.3.6 Answers to Frequently Asked Questions on e-Navigation

IALA has developed answers to some Frequently Asked Questions (FAQs) on certain aspects of e-Navigation. These can be found on the IALA website at .

https://www.iala-aism.org/technical/e-nav-testbeds/enav-faq/ seems to be missing

References for chapter 5

[1]	A registry is simply a bookkeeping device where definitions/specifications are kept in organised locations known as registers. the registry eases the tasks of development of new things, by providing a centralised source for finding definitions/ specifications

[2]	GPS Performance Standards, 2008

[3]	United Nations Office for Outer Space Affairs, “Current and Planned Global and Regional Navigation Satellite Systems and Satellite-based Augmentations Systems”, 2011

[4]	At the time of writing, further information on BeiDou may be found on the internet hhttp://www.en.beidou.gov.cn/csnclist.html

[5]	Further information on Galileo can be found at the following website: http:/ / ec. europa. eu/ growth/ sectors/ space/ galileo/

[6]	A 1kW transmitter will generally allow position fixing to better than 10 metres over a radius of about 200 nautical miles

[7]	United Nation Office of Outer Space Affairs

# abbreviations

# references

10, rue des Gaudines - 78100 Saint Germain en Laye, France Tél. +33 (0)1 34 51 70 01- Fax +33 (0)1 34 51 82 05 - contact@iala-aism.org www.iala-aism.org International Association of Marine Aids to Navigation and Lighthouse Authorities Association Internationale de Signalisation Maritime

The concept of e-Navigation is explained in detail in section 0

As part of the improved provision of services to vessels through e-navigation, maritime services have been identified as the means of providing electronic information in a harmonized way.  It is part of Solution 5 identified in IMO’s SIP.

Weinert, Benjamin & Hahn, Axel & Norkus, Oliver. (2016). A domain-specific architecture framework for the maritime domain.

https://tools.ietf.org/html/rfc8141

Maritime Resource Name (MRN) see: http://mrnregistry.org

IMO is the United Nations specialized agency with responsibility for the safety and security of shipping and the prevention of marine and atmospheric pollution by ships.  Its main role is to create a regulatory framework for the shipping industry that is fair and effective, universally adopted and universally implemented.

Until a few years ago, the term e-navigation was used universally.  The term e-navigation has been left in in this chapter, when outlining some of the history, as words from IMO have been quoted verbatim…but, later on, digital maritime services is used.