Input paper:   DTEC5-x.x.x.x

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM	□  ENG		□  PAP					□  Input

X  DTEC	□  VTS							X  Information

Agenda item 			X.X

Technical Domain / Task Number 2	…………………………………

Author(s) / Submitter(s)			Woo Seong SHIM(KRISO), Bu Young KIM(KRISO)

MS@MS Technology Demonstration and Follow-up Action at DTEC 4

- Summary

The ENAV30 introduced Radio-free wireless communication, the Metal Surface Wave Technology (MS@MS), as a more efficient solution to overcome the limitations of metal surrounding structures in wireless communication, with its primary application in the maritime sector’s wireless IoT network.

The DTEC1 conducted the review of the new technology based on G1153, and the DTEC3 reviewed the results of a trial of MS@MS for intra-ship communications, which was carried out on a coastal passenger vessel in order to validate its wireless communication performance.

The DTEC4 completed the review of the new technology, updating previous reviews with the additional Notes. The issue of intellectual property rights was also discussed. As an outcome of the discussion, the WG noted that KRISO would contact IALA to confirm the status of the technology under the IALA Patent Statement and Licensing Declaration.

This document provides a technology demonstration of MS@MS and presents the IALA Patent Statement and Licensing Declaration.

  - Purpose of the document

The purpose of this document is to demonstrate the MS@MS technology in the field and to report on the actions related to intellectual property rights action following the review conducted at DTEC 4.

  - Related documents

- ENAV30-5.1.2.2 Metal Surface at Magnetic Substance wave

- ENAV30-5.1.2.3 Radio-free wireless communication based on Metal Surface Wave in the maritime sector

- EM1-5.1.2.1 Radio-free wireless communication based on Metal Surface Wave

- DTEC1-5.1.2.1 Review of Radio-free wireless communication based on Metal Surface Wave

- DTEC3-5.2.2.5 Result of trial for the Metal Surface Wave Technology (MS@MS)

- DTEC4-6.2.2.8 Additional Notes of the Metal Surface Wave Technology (MS@MS)

- DTEC4-6.2.2.9 Additional Review of Radio-free wireless communication based on Metal Surface Wave in the maritime sector

- DTEC4-12.1 Report of DTEC4

- Demonstration of the MS@MS technology

  - Demonstration Using Steel Roadside Structures

The first part of the demonstration was conducted using guardrails installed along a road. By employing continuous steel guardrails with only MS@MS devices, measurements were taken to assess both the communication feasibility and the network bandwidth (Mbits/sec) at distances of 100 m, 200 m, and 300 m.

Overview of the First Part of the Demonstration

MS@MS device AP1 was fixed, and AP2 was moved in 100m, 200m and 300m. As shown in Figure 2, during the measurement of network bandwidth using MS@MS at each distance, the presence of Wi-Fi signals was also checked.

Measurement Screens by distance

The communication feasibility at all tested distances was confirmed, and the network bandwidth at each distance was measured as follows. As shown in Table 1 below, the feasibility of communication in all sections was confirmed solely through MS@MS without the use of Wi-Fi. And the network bandwidth of MS@MS, was measured at an average of 140Mbits/sec at 100 m, 65Mbits/sec at 200 m, and 12Mbits/sec at 300 m.

- Table 1	Measurement Results by Distance

| Section |  | communication feasibility |  | Average Network Band Width (Mbits/sec) |
| --- | --- | --- | --- | --- |
|  |  | WiFi | MS@MS |  |
| Distance AP1~AP2 | 100m | X | O | 140 |
|  | 200m | X | O | 65 |
|  | 300m | X | O | 12 |

  - Demonstration on a Coastal Passenger Ship

The second part of the demonstration was conducted on the JAGALCHI CRUISE (iron ship, 397 gross tonnage, length 37.61 meters) which had already been used in the “MS@MS” trial as reported at DTEC3. A total of seven devices were installed on the ship.

Overview of the second Part of the Demonstration

In order to confirm intra-ship communication using MS@MS technology, we used the MS@MS devices in two sections: the first section being between the engine room(AP3) and the B Deck(AP6), and the second section being the bow of Upper Deck(AP5) and the B Deck(AP1), as conducted during the “MS@MS” trial.

MS@MS devices in the engine room(AP3) and the B Deck(AP6)

MS@MS devices in the bow of Upper Deck(AP5) and the B Deck(AP1)

As shown in Figure 6,

Ap detachment

a demonstration was conducted by comparing the cases when AP3 and AP5, installed in the two sections, were attached to and detached from the ship’s steel plates.

Before the APs were detached from the ship’s steel plate, the camera screens connected to the AP were transmitted normally. However, when the APs were detached from the steel plate, the connected camera screen turned blue, indicating that the video was not being transmitted properly.

This demonstrates that magnetic field communication enables data transmission even within a ship composed of multiple bulkheads and decks.

Variation in Communication Status Depending on AP Attachment in the engine room(AP3) and the bow of Upper Deck(AP5)

- IALA Patent Statement and Licensing Declaration

During the review of MS@MS at the DTEC 4 WG2 meeting, the issue of intellectual property rights was discussed. As the outcome of the discussion, the WG noted that KRISO would contact IALA to confirm of the technology under the IALA patent statement and licensing declaration. Accordingly, the attached document is the IALA Patent Statement and Licensing Declaration submitted to the IALA Secretariat.

- future plan

We have conducted research for the introduction of MS@MS technology in the maritime sector.  Furthermore, we will continue research to expand its application in the field of maritime, and we particularly welcome proposals and collaborations in related areas such as MASS and STEEL AtoN.

- Action requested of the Committee

The committee is kindly invited to note the above information, and take actions as appropriate.

- Annex iala patent statement and licensing declaration of ms@ms Technology

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain

This research was supported by Korea Institute of Marine Science & Technology Promotion(KIMST), funded by the Ministry of Oceans and Fisheries(RS-20210644)