Fifth Meeting of the IALA Legal Advisory Panel - LAP 5

| Legal Advisory Panel 5th Session Agenda item 1 | LAP 5 / 4 / 2 February 17-19, 2009 |
| --- | --- |

# Danish position on VTS Communication “Advice on a Course to Steer”

## Introduction

The IALA VTS Committee was directed by the IALA Council to “Review/provide guidance on navigational assistance service” to use as guidelines for national VTS Centres.

However during the work process the drafters experienced difficulties with respect to the distinction between “advice” and “instruction”, especially in situations where a VTS operator performing under a navigational assistance service (NAS) provides a vessel with a course to steer.

The question is whether such could – or should – be given as an advice and if so whether such advice is deemed to be defined and interpreted as a “result-oriented instruction”.

## Danish VTS centres

Denmark has two designated VTS Centres; one operating in the Great Belt, the VTS Great Belt (BELTREP) authorized as a mandatory ship reporting system by IMO providing navigational information and assistance to ships, and the other operating in the Sound between Denmark and Sweden, the Sound VTS (SOUNDREP) as a voluntary ship reporting system providing information to ships about specific and urgent situations, which may cause conflicting traffic movements as well as other information concerning safety of navigation. SOUNDREP is also providing individual information to a ship upon request, particularly in relation to positioning, navigational assistance or local conditions.

As neither of the Danish VTS Centres are providing navigational assistance service to the ships, but only an information service, the IMO SMCP message marker ‘Instruction’ is not adopted for use. However both BELTREP and SOUNDREP use the message marker ‘Advice’, e.g. as in “ADVICE you to guard channel 16 and 71”

## Distinction between advice and instruction

As already stated in the IALA VTS-C on page 10 and 11 with reference to IMO Standard Marine Communication Phrases (SMCP) IMO Resolution A.918(22):

### SMCP on Advice

“Advice” indicates that the following message implies the intention of the sender to influence others by a Recommendation.

SMCP-example: ADVICE. (Advice you) stand by on VHF Channel six nine.

SMCP-comment: Advice does not necessarily have to be followed but should be considered very carefully.

### SMCP on Instruction

“Instruction” indicates that the following message implies the intention of the sender to influence others by a Regulation.

SMCP-example: INSTRUCTION. Do not cross fairway.

SMCP-comment: The recipient [of such instruction] has to follow this legally binding message unless he/she has contradictory safety reasons which then have to be reported to the sender [the VTS].

### IMO SMCP example re advice on a course

The IMO SMCP present in Res. A918(22) section A1/6.2 phrases for providing VTS services. In section A1/6.2.2 regarding navigational assistance service under the heading “course” in section A1/6.2.2.3.5.1 is an example “advice you ~ a new course of … degrees”.

### “Result-oriented instructions”

IMO resolution A.857(20) section 2.3.4 determines that if a “VTS is authorized to issue instructions [deemed as such message markers as defined in IMO Res. A.918(22) section A1/6 (i)] to vessels, [such] should be [given] as result-oriented only, leaving the details of execution, such as course to be steered…, to the master … on board the vessel.”

## Danish position on “Advice or Instruct on a course to steer” and “result-oriented instructions”

It is the opinion of DaMSA that information regarding course to be steered should never be given as an instruction.

It is the opinion that an instruction which is generally understood as a ‘direction or order’ may carry legal implications for the VTS operator especially if the instruction should later prove to be wrong, whereas an advice is generally understood as a ‘guidance or recommendation on future actions’ but leaves it to the recipient to process the advice and act upon it or not.

It is the opinion of DaMSA that IMO Res. A.857(29) section 2.3.4 can be interpreted as even if instructions are given by a VTS operator such should not include a course to steer, but instead leave it up to the master to decide which course to be steered is most suitable.

This should, in the opinion of DaMSA, be taken into account when debating whether an advice by the VTS operator regarding a course to be steered is given to the vessel. Naturally, it will depend on the actual wording whether the VTS operator is deemed to have been given an advice or an instruction – and in the heat of the moment it is not unlikely that what should have been given as an advice will actually be given as an instruction. The latter with potential legal aftermath to be decided should the “advice” or “instruction” prove wrong.

Consequently, it is the opinion of DaMSA – an opinion which is also shared by the parties to the Sound VTS – that a VTS operator should never give a vessel an advice on the course to steer. Not even as a hint or anything which might be interpreted as such. The VTS operator should instead beforehand inform and warn the vessel about potential risks and hazards according to the VTS display as the VTS operator interprets the situation.

As a final remark it might be worth to raise the issue within IMO for a clear guidance on the “acceptability of advising on a course to steer”.

/Sine Rosenborg