IALA Guideline

G1111

Establishing Functional and Performance Requirements for VTS Systems

Edition 2.0

xx 2022

urn:mrn:iala:pub:g1111:ed2.0

Revisions to this document are to be noted in the table prior to the issue of a revised document.

| Date | Details | Approval |
| --- | --- | --- |
| May 2015 | First issue (edition 1.0) of Guideline 1111 on “Preparation of Operational and Technical Performance Requirements for VTS systems”. Originated from IALA Recommendation V-128, edition 3. Annex from former Recommendation changed to Guideline and revised to include additional considerations, new technologies and emerging technologies. Additionally, consistency has been improved and duplications were removed. | Council 60 |
| January 2022 | Edition 1.1 - Editorial corrections published January 2022, in alignment with IMO Resolution A.1158(32) Guidelines for Vessel Traffic Services. | Council 74? |
| June 2022 | Edition 2.0 - Due to the comprehensive content of G1111 (115 pages), the document was split into one guideline and nine sub-guidelines and reviewed to describe the establishment of the functional requirements better. Title changed to “Establishing Functional & Performance Requirements for VTS systems” | Council xx |

1.	INTRODUCTION	4

1.1.	Purpose	4

1.2.	G1111 guideline series	4

2.	Establishing the FUNCTIONAL AND PERFORMANCE Requirements for a VTS System	4

2.1.	Project planning	5

2.1.1.	ISO 21502:2020 Guidance on project management	5

2.1.2.	Project management steps	5

2.2.	Operational requirements	7

2.3.	Security requirements	7

2.4.	Deriving functional and performance requirements	7

3.	Implementation considerations	8

3.1.	Availability and reliability	9

3.2.	Recording, archiving and replay	10

3.3.	Other considerations	11

3.3.1.	Environmental considerations	11

3.3.2.	Equipment shelters	11

3.3.3.	Lightning protection	11

3.3.4.	Warning lights	11

3.3.5.	Site and equipment access	11

3.3.6.	Electrical power	11

3.3.7.	Safety and security precautions	11

3.3.8.	Equipment preservation and monitoring	12

3.3.9.	Marking and identification	12

3.3.10.	Documentation	12

3.3.11.	Equipment standards and approvals	12

4.	DEFINITIONS	13

5.	Abbreviations	13

6.	REFERENCES	17

List of Tables

Table 1	Relationship between downtime and availability	9

List of Figures

Figure 1	Project management steps in establishing VTS systems	6

# INTRODUCTION

This Guideline presents a common source of information to assist competent authorities and VTS providers in the preparation and establishment of functional and performance requirements for VTS systems. Tailoring is required to capture the specific and relevant performance requirements from the generic information included within this document. The Guideline should not be used as a specification without such tailoring.

References and definitions applicable to the separate sections are included in the sections where appropriate.

This Guideline is associated with Recommendation R0128 Operational and Technical Performance of VTS Systems, a normative provision of IALA Standard S1040 Vessel Traffic Services (VTS) [1]. To demonstrate compliance with the Recommendation, the provisions of this Guideline should be taken into account.

## Purpose

This Guideline is one of the G1111 series of guideline documents. The purpose of the G1111 series is to assist the VTS provider in preparing the definition, specification, establishment, technical operation and upgrades of a VTS system. The documents address the relationship between the functional requirements and VTS system performance (technical) requirements and how these reflect into system design and sub system requirements.

## G1111 guideline series

The G1111 series of guideline documents present system design, sensors, communications, processing, and acceptance, without inferring priority. New sub-guidelines can be introduced to the G1111 series in future. The guideline documents are numbered and titled as follows:

G1111		Establishing Functional & Performance Requirements for VTS Systems

G1111-1	Producing Requirements for the Core VTS System

G1111-2	Producing Requirements for Voice Communications

G1111-3	Producing Requirements for RADAR

G1111-4	Producing Requirements for AIS and VDES

G1111-5	Producing Requirements for Environment Monitoring Systems

G1111-6	Producing Requirements for Electro Optical Systems

G1111-7	Producing Requirements for Radio Direction Finders

G1111-8	Producing Requirements for Long Range Sensors

G1111-9	Framework for Acceptance of VTS Systems

# Establishing the FUNCTIONAL AND PERFORMANCE Requirements for a VTS System

IALA Guideline G1150 [2] advises on the planning and implementation of a VTS related to the definition of operational, functional, performance and acceptance requirements of a VTS system. These requirements should be:

derived from a business case and feasibility study in the initiating phase; and

defined in the planning phase of the overall VTS project before the procurement of the VTS system.

The VTS provider should establish operational, functional and performance requirements and acceptance plan for a VTS system at the same time as they establish their procurement plan. These requirements should form the basis for the entire system lifecycle, its definition and its acceptance following implementation.

The G1111 Guideline Series provide generic guidance for all potential equipment and sensors that may be used in designing a VTS system. The guidance is not prescriptive, and the capabilities required may vary between VTS, VTS area and VTS sectors or even specific parts of a VTS area or sector. VTS capability should be linked to risk assessment which will identify the degree of mitigation expected of a VTS and its contribution, together with other mitigation measures, to address a specific hazard or hazards. The extent of risk mitigation provided through the implementation of a offered will be determined by a number of factors that include the equipment fit, equipment capability, local geography and operator authorization/training.

This guideline focuses to provide an overview of the key areas for consideration related to the establishment of a VTS system.

IALA guidelines G1111-1 to G1111-8 provide a specific guidance on initiating and planning functional and performance requirements and implementing VTS systems and VTS equipment, based on operational requirements.

Acceptance steps are typically conducted on the proposed or implemented VTS system to control the compliance to the requirements. These acceptance steps are discussed in G1111-9 Framework for Acceptance of VTS Systems.

## Project planning

IALA Guideline G1150 Establishing, Planning and Implementing VTS recommends a project management approach to ensure that major deliverables, assumptions and constraints are clearly documented when planning and implementing a VTS. This will assist in defining the scope of the VTS, its goals and objectives that need to be met. Project management is considered as a discipline with the purpose to achieve specific goals and objectives by planning, organizing, motivating, and controlling resources.

Relevant international guidance prepared and published by appropriate international organizations regarding project management should be considered, or, where there are national requirements for project management, these should be used.

### ISO 21502:2020 Guidance on project management

This is an international standard issued by the International Organization for Standardization (ISO).

In summary, the standard:

Provides high-level description of concepts and processes that are considered to form good practice in project management.

Can be used by any type of organization, including public, private or community organizations, and for any type of project, irrespective of complexity, size, or duration.

### Project management steps

Project management is undertaken in different steps in order to improve control and quality. Guideline G1150 provides an overview of the five project management phases and the key areas for consideration as they relate to VTS:

Initiating

Planning

Implementing

Controlling

Closing.

It is noted that before completing each stage to define functional and performance requirements, these results could be re-evaluated by risk assessment and cost benefit analysis. The core practice in the planning phase is finalising the requirements. This involves combining the relevant operational requirements with the functional and performance requirements, without unintentionally restricting flexibility in the implementation.

It is important to write well-structured, individual requirement statements within the published requirements documentation.

Note that, for the implementation phase, several possible technical solutions may be identified to achieve the defined requirements and each of these solutions may have different strengths and weaknesses. Scoring systems to address the most critical aspects of the operational requirements may need to be considered when preparing procurement plan.

Project management steps in establishing VTS systems

## Operational requirements

The operational requirements needed to derive the functional and performance requirements should include:

delineating the VTS area and, if appropriate, VTS sub-areas or sectors;

types and sizes of ships required or expected to participate in the VTS;

navigational hazards and traffic patterns;

human/machine interface and human factors including health and safety issues;

tasks to be performed by VTS operators or supervisors;

Operational procedures, staffing level and operating hours of the VTS;

information sharing and co-operation with external stakeholders; and

legal framework.

## Security requirements

The security requirements needed to derive the functional and performance requirements should include:

physical security of the VTS Centre and remote sites;

cyber security; and

business continuity, availability, reliability and disaster recovery.

## Deriving functional and performance requirements

The functional and performance requirements for a VTS system should be derived from the operational requirements. This may be an iterative process, which can be aligned with the phases of Guideline G1150.

In order to define functional and performance requirements, the operational requirements may be grouped into:

communications;

situational awareness;

recording and playback; and

reliability and availability.

The grouping of operational requirements facilitates the creation of functional and performance requirements, for example divided into:

voice and data communication;

the VTS Centre, sites, sensors and processing;

recording and replay including post situational analysis; and

redundancy and resilience.

Deriving system concepts may involve various mathematical, functional and simulation models to visualize different characteristics of the system. Models to consider might include:

radio communications coverage;

sensor coverage;

communications network infrastructure;

data architecture and interfaces;

reliability and availability including any redundancy options; and

lifecycle costs.

The models could assist in establishing the relationship between the system concepts, associated functional and performance requirements and the operational requirements. Feasibility studies (site surveys, equipment trials etc.) may also be appropriate to reduce technical risks which may otherwise not be apparent until implementation.

Additional functional and performance requirements may come from:

environmental considerations;

legal obligations;

ergonomic issues;

safety (other than navigational safety);

security requirements e.g., physical security, personnel security, cyber security incl. software updates during operation; and

operational business rules or operational procedures.

# Implementation considerations

VTS system implementations may require consideration of the following:

VTS System function and performance:

Off the shelf solution or customized solution;

Sensor and radio coverage, including overlapping coverage;

Equipment redundancy;

Communications routes;

External Interfaces;

Electromagnetic issues (EMI/EMC); and

Audit Tracking;

VTS system sustainability:

Lifecycle planning, including environmental impact and sustainability in the choice of materials, power consumptions etc.; and

Maintenance of the VTS system and sensors.

Environmental constraints and impact such as wind, influence from sea, precipitation and possibly ice

Locational and infrastructure design:

VTS Centre location(s);

available land and suitability of sensor sites;

existing infrastructure such as power and data lines;

selection of installation sites with due respect to neighbours; and

security and site access.

Rules and regulations

Business Rules; and

Applicable regulations and required licenses (transmission, building etc.).

Organization and staffing

Administrative functions; and

Authorization levels.

Any VTS system should, as a minimum, be equipped with a means to build a VTS traffic image as well as providing reliable communications.

The system architecture of a VTS system should carefully consider issues such as:

network requirements (bandwidth, latency etc.);

redundant data paths;

data integrity;

security requirements e.g., physical security, cyber security;

data storage; and

reporting and maintenance facilities.

In addition, the system architecture should have built in flexibility for future upgrades and have the capability to be maintained without impacting routine VTS operations.

During the development of the system architecture, comprehensive site surveys could be performed, including but not limited to the above considerations. Involvement of relevant stakeholders in the site survey early in the process adds value and ensures awareness of the potential risks (e.g., design and performance).

## Availability and reliability

The VTS provider should define an overall availability and reliability target for the VTS system including the elements to be considered based on the risk assessment results. The relationship between downtime and availability figures is given by Table 1.

Relationship between downtime and availability

| Availability |  |  |  |  |
| --- | --- | --- | --- | --- |
| Annual downtime | 87,6 hours | 24 hours | 8 hours | 4 hours |
| Corresponding Availability | 99% | 99.7% | 99.9% | 99.95% |

The VTS provider can decide whether individual sub-systems are critical or non-critical. Non-critical sub-systems may be excluded from the overall system availability requirement.

Note that multiple means of communications and overlapping sensor coverage will increase overall availability. Such a solution may result in reduced requirements for the availability of each item of equipment individually.

Scheduled maintenance activities may be included in availability figures.

Also note that required spare parts should be readily available, to ensure the shortest time to repair. Therefore, VTS providers should plan for sufficient spare parts and service arrangements, or business continuation plans in order to meet the availability criteria.

The VTS system availability may be improved by the following redundancy measures:

by duplicating hardware and/or externally hosting VTS services:

In duplicated hardware cases, parameter hand-over from active to stand-by equipment should be considered; and

between sensors and radio base stations, where overlap can provide redundancy, possibly with reduced performance;

between various types of sensors and voice communications, where overlap can provide redundancy, possibly with reduced performance; and

by adding graceful degradation capabilities to individual VTS Equipment.

## Recording, archiving and replay

Within legal limitations, provision should be made for secure storage, retrieval and presentation of VTS data so the relevant data can be consistently recalled.

The data type, resolution and period of time for which data is required to be stored should be derived from operational procedures. A minimum of thirty (30) days' storage capacity is recommended. Consideration including data security, can be given to providing this as online storage. Archiving of older data may be considered for traffic statistics, risks evaluation, strategic planning etc.

The time period should allow for the full retrieval of data post-incident/accident, in compliance with national requirements and those of the incident/accident investigation procedures of the VTS provider and other authorized parties.

Stored and archived data should include:

VTS traffic image;

sensor data;

voice communication; and

other relevant information.

- It may also include (within legal limitations):

internal VTS personnel conversations inside the VTS centre; and

VTS personnel activities.

technical and maintenance purposes e.g. logging, failure tracking, long term failure statistics and analysis, system performance monitoring and improvement;

The data should be recorded automatically and be capable of replay without impact to on-going VTS operations. Synchronization of information is recommended for replay.

## Other considerations

### Environmental considerations

The VTS provider should specify the local environmental conditions for VTS system performance, design and outdoor installations.

### Equipment shelters

A shelter can provide a protective environment with characteristics that depend on the location and design of the shelter. In situations where contained equipment is reliant on the environment created by the shelter, the shelter facilities (e.g., cooling or heating) may become critical to the achieved availability of the equipment.

### Lightning protection

Lightning protection is often subject to national or local legislation taking into account local conditions, severity, earth conductivity, power grid constraints etc. The guidance from country to country differs depending on lightning strike frequency and severity. As a consequence, requirements for the number and type of lightning arrestors, the number of earthing points and the minimum cross section of lightning conductors vary to suit local conditions.

The general principles include:

lightning arresters should be higher than other equipment and be designed to protect the entire installation.

They should have separate down conductor(s) on the exterior of buildings and the down conductors should not be connected to metal parts of buildings such as steel reinforcements, handrails and antenna masts;

safety grounding of equipment should be kept separate from lightning protection; and

potential equalization should be achieved in earth and never at the top of the equipment.

### Warning lights

High structures may require warning lights for air traffic, such as radar towers. Specific light arrangements may also be disturbing to both vessel traffic, the public and to animals. It is therefore recommended to consult local authorities for specific requirements or restrictions in the area.

### Site and equipment access

As part of the design of a VTS system locations, the VTS provider should analyse the need for site access for installation and maintenance. Fencing and other protective means against illegal intrusion will also be needed in many cases.

### Electrical power

The VTS system requires a reliable source of electrical power, which could include a backup power source such as an Uninterruptable Power Supply (UPS).

Where a new or replacement source of electrical power is necessary, renewable sources should be used if possible.

### Safety and security precautions

For each location, the VTS provider should determine safety and security requirements in accordance with local legislation.

Safety requirements should, at least, consider but are not limited to:

safety procedures, such as instructions to personnel performing maintenance;

personnel protection equipment for working at heights;

lone working on remote sites should be avoided;

safety switches to isolate equipment and to stop rotating antennas;

precautions regarding electromagnetic radiation, rotating machinery and electrical shock, railings on masts etc.; and

protection of the general public.

Security requirements should, at least, consider:

access restrictions; and

alarm International Ship and Port Security (ISPS) code requirements.

### Equipment preservation and monitoring

The VTS provider should also consider the following:

fire detection and (remote) alarms;

automated fire extinguishers;

remote monitoring of site status (power, fuel, temperature, site environmental conditions etc.); and

remote monitoring of equipment status.

### Marking and identification

VTS equipment should be marked with manufacturer name, type and serial number.

Legislation may require additional marking or identification, signposts etc.

### Documentation

The VTS provider should specify deliverable documentation to accompany the VTS equipment. As a minimum, documentation should include:

operating instructions;

maintenance instructions (preventive and corrective) inclusive of procedures and spare parts catalogue;

safety information (e.g. regarding radiation, electrical safety and rotating machinery);

certificates and permissions as required by law;

test procedures, test certificates; and

“As built” documentation comprising drawings, equipment configurations, software versions, etc.

### Equipment standards and approvals

Legal requirements for equipment standards and approval (or statements of conformity) vary from country to country. It is the responsibility of the VTS provider to ensure compliance to local, regional and international standards. The VTS Provider should state any applicable standards as part of the acquisition process.

Typical standards and approvals may include the following (Note. This is not a complete list and VTS providers should ensure that all appropriate standards and approvals for their VTS area have been considered):

Electrical Safety

Mechanical Safety

Radiation Safety

Electromagnetic Compatibility

Radio Spectrum licensing

Hazardous / Chemical Substances

# DEFINITIONS

The following definitions apply within this document and in all the G1111 series of guideline documents:

| VTS system | – | within the G1111 guidelines, the VTS system is the VTS software, hardware, communications and sensors. This excludes personnel and procedures. |
| --- | --- | --- |
| VTS equipment | – | within the G1111 guidelines, VTS equipment refers to the individual items of software, hardware, communications and sensors, which make up the VTS system. |
| VTS user | - | within the G1111 guidelines, VTS user is defined as someone with either an operational, technical, or administrative need to use or access the VTS system. |

The other definitions of terms used in this Guideline can be found in the International Dictionary of Marine Aids to Navigation (IALA Dictionary) at http://www.iala-aism.org/wiki/dictionary and were checked as correct at the time of going to print. Where conflict arises, the IALA Dictionary should be considered as the authoritative source of definitions used in IALA documents.

# Abbreviations

º	Degree

	Plus or minus

>	Greater than

≤	Less than or equal to

≥	Greater than or equal to

%	percent

µs	microsecond

AIS	Automatic Identification System

AREPS	Advanced Refractive Effects Prediction System

ASL	Above Sea Level

AtoN	Marine Aid(s) to Navigation

BITE	Built In Test Equipment

BoM	Bureau of Meteorology (Australia)

C	Celsius

CARPET	Computer Aided Radar Performance Evaluation Tool

CCTV	Closed-Circuit Television

CE	Conformité Européenne

CHC	Canadian Hurricane Centre

Circ.	Circular (IMO)

COG	Course over Ground

COSPAS	Cosmicheskaya Sistema Poiska Avariynyh Sudov (Russian; Space System for the Search of Vessels in Distress)

COSPAS/

SARSAT	Search and Rescue Satellite-Aided Tracking

CPA	Closest Point of Approach

CPHC	Central Pacific Hurricane Centre

CS	Coastal Surveillance

CW	Continuous Wave

dB	decibel

dB(A)	A-weighted decibel

dBi	decibel isotropic

DF	Direction Finder

DSC	Digital Selective Calling

DSF	Decision Support Function

DST	Decision Support Tool

D-GNSS	Differential GNSS

EC	European Commission

ECC	Electronic Communications Committee

ECDIS	Electronic Chart Display and Information System

ECS	Electronic Chart System

EIA	Electronics Industry Association

ELT	Emergency Location Transmitter

EMC	Electromagnetic Compatibility

EMF	ElectroMagnetic Force (EU Directive)

EMI	Electromagnetic Interference

ENC	Electronic Navigation Chart

EO	Electro-Optical

EOS	Electro-Optical Sensor

EPIRB	Emergency Position Indicating Radio Beacon

ERC	European Research Council

ETA	Estimated Time of Arrival

ETSI	European Telecommunications Standards Institute

EU	European Union

FAT	Factory Acceptance Test

FATDMA	Fixed-Access Time-Division Multiple Access

FCA	Functional Configuration Audit

FMCW	Frequency Modulated Continuous Wave

FMS	Fiji Meteorological Service

FoV	Field of View

GHz	gigahertz

GIT	Georgia Institute of Technology

GMDSS	Global Maritime Distress and Safety System

GNSS	Global Navigation Satellite System

GPS	Global Positioning System

h/hr	hour

HDF	Hierarchical Data Format

HF	High Frequency (3–30 MHz radio frequency range (band))

HMI	Human / Machine Interface

hPa	hectoPascal

hydro/meteo	hydrological/meteorological

ICNIRP	International Commission on Non-Ionizing Radiation Protection

ID	Identification

IDC	International Data Centre (for LRIT)

IEC	International Electro-Technical Commission

IEEE	The Institute of Electrical and Electronics Engineers

IETF	Internet Engineering Task Force

IHO	International Hydrographic Organization

IMD	Indian Meteorological Department

IMO	International Maritime Organization

IOC	Intergovernmental Oceanographic Commission

IP	Ingress Protection

IP	Internet Protocol

IR	InfraRed

ISO	International Organization for Standardization

IT	Information Technology

ITU	International Telecommunication Union

ITU-R	International Telecommunication Union-Radiocommunication

JMA	Japan Meteorological Agency

JTWC	Joint Typhoon Warning Center

Ka-band	26.4 – 40 GHz (radar band)

kg	kilogram

kHz	kilohertz

km/h	kilometres per hour

KPI	Key Performance Indicator(s)

Ku-band	12.0 – 18.0 GHz (radar band)

kW	kilowatt

LRIT	Long Range Identification and Tracking

LVD	Low Voltage Directive (EU)

m	metre

m/s	metres per second

m2	square metre

m3	cubic metre

MF	Medium Frequency (300 kHz and 3000 kHz radio frequency range (band))

MFR	Météo France

MHz	MegaHertz

MIL-STD	Military Standard (US)

MKD	Minimum Keyboard and Display

mm/hr	millimetre per hour

MMSI	Maritime Mobile Service Identity

MPA	Marine Protected Area(s)

MPEG	Moving Pictures Expert Group

MSC	Maritime Safety Committee (IMO)

MSI	Maritime Safety Information

MTBF	Mean Time Between Failure

MTI	Moving Target Indication

N	The radio refractivity index

N/A	Not applicable

NAVTEX	Navigational Telex

NHC	National Hurricane Centre

NIMA	National Imagery and Mapping Agency

NM	nautical mile

NTIA	National Telecommunications and Information Administration

OFTA	Office of the Telecommunications Authority

OJ	Official Journal of the European Union

PC	Personal Computer

PCA	Physical Configuration Audit

PD	Probability of Detection

PFA	Probability of False Alarm

PRF	Pulse Repetition Frequency

PSLR	Peak Side Lobe Ratio

PSS	Practical Salinity Scale

PSSA	Particularly Sensitive Sea Area(s)

PTZ	Pan, Tilt, Zoom

QoS	Quality of Service

RACON	Radar beacon

RADAR	Radio Detection and Ranging

RCS	Radar Cross Section

REACH	Registration, Evaluation, Authorization and Restriction of Chemical substances

RF	Radio Frequency

RDF	Radio Direction Finder

RH	Relative Humidity

RMS	Root Mean Squared

RoHS	Reduction of Hazardous Substances

R&TTE	Radio and Telecommunications Terminal Equipment

SAIS	Satellite AIS

SAR	Search and Rescue

SARSAT	Satellite-based Synthetic Aperture Radar

SART	Search and Rescue Transponder

SAT	Site Acceptance Test

S-band	2.0 – 4.0 GHz (Note: military designation is F-band)

SLA	Service-Level Agreement

SN	Safety of Navigation (IMO)

SOG	Speed over Ground

SOLAS	Safety of Life at Sea

SPA	Special Protected Area(s)

SS	Sea State

STC	Sensitivity-Time Control

STD./std.	Standard

S-57	Transfer Standard for Digital Hydrographic Data (IHO)

S-100	Geospatial Information Registry (IHO)

S-101	IHO ENC Product Specification (under development in 2015)

TCPA	Time to Closest Point of Approach

TDMA	Time-Division Multiple Access

UPS	Uninterruptable Power Supply

US	United States (of America)

UV	Ultra Violet (light)

VDL	VHF Data Link

VHF	Very High Frequency

VoIP	Voice over Internet Protocol

VTMIS	Vessel Traffic Management and Information System

VTS	Vessel Traffic Service or Vessel Traffic Services (dependent on context)

VTSO	Vessel Traffic Services Operator

W	watt

WGS84	World Geodetic System 1984 (Reference coordinate system used by GPS)

WMO	World Meteorological Organization

X-band	8.0 – 12.0 GHz (Note: military designation is I-band)

XML	Extensible Mark-up Language

# REFERENCES

- IALA. Recommendation R0128 (V-128) Operational and Technical Performance of VTS Systems.

- IALA. Guideline G1150 Establishing, Planning and Implementing a VTS