Input paper:   VTS56-9.3.2

Updated version

Input paper for the following Committee(s): 		Purpose of paper:

(Select as appropriate)

☐ ARM	☐  ENG		☐ PAP			☒ Input

☐ ENAV	☒ VTS					☐ Information

Agenda item 			9.3

Technical domain/ Task number 2	2.8.2

Author(s)/Submitter(s)			Korea Coast Guard

# Proposal for Update on the IVEF Service (S-210) with VTS Data standard at the Sensor Level: Korea Coast Guard Case Study

# Summary

This document serves as an introduction to the operational case of the Cloud VTS development project led by the Korea Coast Guard. Additionally, it proposes updates to the IVEF Service for development of the S-210, incorporating VTS data format at the sensor level. The importance of inter VTS data exchange is highlighted through an analysis of the current state of VTS operations in Korea.

The document outlines the operational status of the Cloud VTS test-bed in Korea elucidating data exchange configurations, and recommends revisions to the IVEF Service in combination with VTS data standard in administrative rules in Korea.

## Purpose of document

The purpose of this document is to introduce the VTS operational environment in Korea and to present the test-bed operation case of the “Cloud VTS development project” conducted by the Korea Coast Guard. The aim is to propose a direction for improving the IVEF service (S-210) revision, with VTS data standard suggested by KCG at the sensor level, as validated by the test-bed operation case.

## Related documents

IALA RECOMMENDATION R0145(V-145) THE INTER-VTS EXCHANGE FORMAT (IVEF) SERVICE

# Background

The IALA Recommendation R0145(V-145) “Inter-VTS Exchange Format (IVEF) Service” issued at first in June 2011.

Discussions related to the IALA Recommendation R0145 which had not been held from 2011, have been resuming application of IVEF to digital VTS Product specification in S-100 world.

VTS51 had resumed revision on IVEF service, so that The KCG introduced the Cloud VTS concept in VTS53 exemplifying the IVEF service with survey of VTS operators regarding the IVEF service and proposed to revise an IVEF to include the need for information exchanging between Radar and VHF.

As an extension of this, this document is aimed at the IVEF protocol with VTS data standard at the sensor level that was invented in Cloud VTS project and is proposed improving IVEF service towards to S-210.

# Discussion

## VTS operation STatus in South korea

South Korea operates 19 VTS centers including 3 Regional VTS. Purpose of operating VTS is for strengthening monitoring and managing ship movements to prevent congestion or dangerous situations and improving overall traffic efficiency. Notably 3 regional VTS centers has been contributing to facilitate continuous maritime traffic management from ship’s departure to ship’s arrival outside the purview of VTS sectors and VTS area in charge.

For continuously monitoring vessel traffic over wide areas, VTS needs to interconnect systems to able to share VTS data. It is challenging to implement on Information exchange with neighboring VTS area due to the use of equipment from different manufacturers. This has highlighted the critical need for inter-VTS information exchange.

Four VTS systems from distinct manufacturers are in operation in South Korea. However, limitations exist in exchanging VTS data beyond Radar Track, Vessel information, Voyage data, and Tagged item data as defined in IVEF. It is caused by differing protocols employed by each manufacturer.

Given the picture below, the following figure depicts vessel tracks passing through VTS areas along the South Korean coast, categorized by manufacturer. All VTS areas in South Korea are interconnected. For instance, a vessel navigating the western direction of the Gyeongin coastal VTS area can proceed to Boryeong Port via the Taean coastal VTS area, or alternatively, enter Mokpo Port. Nevertheless, due to differences in manufacturers between the Gyeongin Port VTS center and the Taean and Boryeong Port VTS centers, seamless VTS information linkage might be hindered, except for data conforming to the IVEF format.

## introduction of cloud vts

To overcome the difficulty of information exchange between different VTS manufacturers, South Korea conducted the Cloud VTS project from 2021 to 2023.

Traditionally VTS has relied on the IVEF communication protocol for limited data exchange with nearby VTS that use different operational systems. However, unlike the operational-level data exchange protocol (IVEF), the standards for data collection within VTS systems vary significantly among manufacturers and are often proprietary. As a result, data acquisition devices and VTS systems are subject to vendor lock-in, which hinders the free flow of information.

As VTS areas expand to ensure the safe and efficient operation of vessels, and various data exchanges are becoming increasingly necessary for VTS operators to make prompt and accurate support of decisions-making, as more data is generated in the maritime industry with the emergence of MASS (Maritime Autonomous Surface Ships). Therefore, it is important to enable the free sharing of data between VTS centers.

So the following figure compares the current VTS system with the Cloud VTS project.

VTS operators can access information from local radar sites or the Cloud platform. In addition to data exchange through IVEF, data from Radar screens, VHF, CCTV, and other sources can also be exchanged, making it possible to utilize them for control support in emergency situations. The Cloud platform enables monitoring of VTS data nationwide in South Korea. And it is planned to expand to all VTS using the Cloud platform without any constraints.

VTS DATA Standard

The purpose of suggesting this data standard by KCG is to specify the requirement of data collection from VTS in various regions in order to collect the data from each VTS operation, integrate the data, and provide customized services required to each circumstances. Each VTS can transmit its surveillance data, and receive other VTS’s data from the VTS operation system on basis of the standardized interface based on IVEF. This standard defines the types of data that can be collected and their respective formats.

KCG is integrating these data format into domestic VTS law and regulations to improve interoperability among diverse equipment and foster data exchange with adjacent VTS. New Equipment to be soon will be required to comply with these data format for communication protocol, enabling a phased transition to IVEF-based communication.

## testbed operation WITH VARIFICATION ON vts DATA FORMAT

The concept of Cloud VTS is to collect VTS information acquired by other centers or external agencies through a central cloud server and provide services by accessing it from VTS centers or other agencies. To achieve this goal, a test-bed has been set up.

The test-bed is composed of the following: Information from Busan Port, Ulsan Port, and Incheon Port is collected on the integrated platform, and the information is displayed on the cloud monitor. The integrated platform can collect and display Radar screen, chart, VHF information, CCTV, and weather information from three VTS centers.

Through the integrated platform, it is possible to track Radar screens, radar, and AIS on charts of Busan Port, Ulsan Port, and Incheon Port that are connected via the platform. In addition, VHF data transmission and reception are possible for control purposes. The integrated platform also enables sharing of information such as Radar screens, CCTV, and VHF in addition to chart information.

A trial had been conducted to facilitate the exchange of information between VTS centers using data Standard from July to December in 2023

The purpose of the trial was to verify the feasibility of collecting standardized data from various sensors like Radar(display and target), AIS, VHF, CCTV, and weather devices installed at the test sites in Busan (Gamcheon and Amnam R/S) and visualizing this data it the Test-bed located Ulsan VTS center in real time whether it could be used for VTS operation.

As a result of operating the test centers, it was confirmed that the operating system developed for the Ulsan test center using the KCG-proposed data format could display traffic control information from the Busan test center and had sufficient functionality to be utilized in actual traffic control operations.

| (a) Radar in data format(Busan VTS) | (b) Radar in data format(displayed in Test-bed) |
| --- | --- |
| (c) VHF status in data format(Busan VTS) | (d) VHF status in data format(displayed in Test-bed) |
| (e) CCTV in data format(Busan VTS) | (f) CCTV in data format(displaed in Test-bed) |
| (g) Weather information in data format(Busan VTS) | (h) Weather information in data format (displayed in Test-bed) |

## updating of ivef service WITH DATA foramt

Test-bed result has validated the effectiveness of VTS data format. By integrating sensor-level data formats  into the IVEF protocol, its efficiency and accuracy across various applications can be significantly enhanced. This integration will enable the exchange of high-resolution data directly from sensors in a standardized format, providing more precise and detailed information. Furthermore, real-time data from diverse sensors, such as radar, AIS, and CCTV can be shared to obtain more comprehensive information and support informed decision-making.

The standardized data format will facilitate seamless data exchange among VTS systems, vessels, and port facilities from different manufacturers. This will enable the development of various applications and the creation of new services based on shared data.

In emergencies, such as the closure of VTS due to global pandemics like COVID-19 or the destruction of VTS systems caused by to conflicts/war, standardized data can contribute to maintaing the stability of VTS operations during recovery.

So It is proposed that an update to the IVEF Service to enable the exchange of Radar video, CCTV and weather information with sensor level. Radar video can increase the accuracy of target tracking, and CCTV data integration enables the consolidation of traffic data from diverse areas into the integrated VTS operation system. Detailed specification on each data format is attached in the Appendix 1. Other major VTS DATA  standards like AIS etc, follow the existing international or organizational data transmission standards where applicable.

# action requested of the committee

VTS committee is invited to note the application of this technology and support to update "IALA Recommendation R0145(V-145) The inter-VTS exchange format (IVEF) service."

details of data format

Radar TARGET data

Data format: !RADAR,TargetID,Status,Lat,N/SIndicator,Lon,E/WIndicator,Course,Speed,Timestamp*H,CR/LF

| No. | Field name | Type | Contents |
| --- | --- | --- | --- |
| 1 | SentenceID | String | Starting letter |
| 2 | Target ID | String | Delimter (0~4,294,000,000) |
| 3 | Status | String | 0=Removed, 1=Lost, 2=Firm, 3=Initial, 4=Coasted, 5=Parked |
| 4 | Latitute | String | Ddd.dddddd |
| 5 | N/S indicator | String | N= North, S= South |
| 6 | Longitude | String | Ddd.dddddd |
| 7 | E/W indicator | String | E= East, W= West |
| 8 | Course | String | Degre (000~359.9) |
| 9 | Speed | String | Knot |
| 10 | Timestamp | String | 0~59s |
| 11 | Checksum | String | H: checksum, Checksum is the XOR value of characters other than "!" and "*" |
| 12 | Terminatior | String | CR/LF: 0x0D/0x0A |

<?xml version="1.0" ?> <RadarTargets> <RadarTarget> <TargetID>44242</TargetID> <Status>2</Status> <Latitude>36.123456</Latitude> <NS_Indicator>N</NS_Indicator> <Longitude>128.123456</Longitude> <EW_Indicator>E</EW_Indicator> <Course>12.3</Course> <Speed>45.6</Speed> </RadarTarget> <RadarTarget> <TargetID>45918</TargetID> <Status>0</Status> <Latitude>34.234567</Latitude> <NS_Indicator>N</NS_Indicator> <Longitude>123.567891</Longitude> <EW_Indicator>E</EW_Indicator> <Course>23.4</Course> <Speed>12.3</Speed> </RadarTarget> <RadarTarget> <TargetID>39481</TargetID> <Status>3</Status> <Latitude>38.193841</Latitude> <NS_Indicator>N</NS_Indicator> <Longitude>129.129431</Longitude> <EW_Indicator>E</EW_Indicator> <Course>129.1</Course> <Speed>8.7</Speed> </RadarTarget> <RadarTarget> <TargetID>37191</TargetID> <Status>1</Status> <Latitude>35.191812</Latitude> <NS_Indicator>N</NS_Indicator> <Longitude>125.284715</Longitude> <EW_Indicator>E</EW_Indicator> <Course>58.3</Course> <Speed>3.3</Speed> </RadarTarget> <RadarTarget> <TargetID>59231</TargetID> <Status>4</Status> <Latitude>36.172481</Latitude> <NS_Indicator>N</NS_Indicator> <Longitude>124.887213</Longitude> <EW_Indicator>E</EW_Indicator> <Course>254.3</Course> <Speed>11.2</Speed> </RadarTarget> <RadarTarget> <TargetID>41918</TargetID> <Status>5</Status> <Latitude>33.982413</Latitude> <NS_Indicator>N</NS_Indicator> <Longitude>126.787534</Longitude> <EW_Indicator>E</EW_Indicator> <Course>301.4</Course> <Speed>16.4</Speed> </RadarTarget> <RadarTarget> <TargetID>58187</TargetID> <Status>3</Status> <Latitude>34.334545</Latitude> <NS_Indicator>N</NS_Indicator> <Longitude>127.898784</Longitude> <EW_Indicator>E</EW_Indicator> <Course>54.2</Course> <Speed>4.5</Speed> </RadarTarget> <RadarTarget> <TargetID>48472</TargetID> <Status>2</Status> <Latitude>35.567852</Latitude> <NS_Indicator>N</NS_Indicator> <Longitude>128.100923</Longitude> <EW_Indicator>E</EW_Indicator> <Course>222.3</Course> <Speed>17.7</Speed> </RadarTarget> <RadarTarget> <TargetID>42485</TargetID> <Status>3</Status> <Latitude>36.667531</Latitude> <NS_Indicator>N</NS_Indicator> <Longitude>129.987654</Longitude> <EW_Indicator>E</EW_Indicator> <Course>36.3</Course> <Speed>12.3</Speed> </RadarTarget> <RadarTarget> <TargetID>38983</TargetID> <Status>4</Status> <Latitude>33.123654</Latitude> <NS_Indicator>N</NS_Indicator> <Longitude>126.654321</Longitude> <EW_Indicator>E</EW_Indicator> <Course>99.1</Course> <Speed>7.8</Speed> </RadarTarget> </RadarTargets>

Radar VIDEO data

Data format: EUROCONTROL Specification for Surveillance Data Exchange ASTERIX Category 240 Radar Video Transmission

<?xml version="1.0" ?> <RadarVideos> <Asterix> <Category>240</Category> <Length>99</Length> <Message> <DataSourceIdentifier> <SAC>51</SAC> <SIC>1</SIC> </DataSourceIdentifier> <MessageType>2</MessageType> <VideoRecordHeader>329082</VideoRecordHeader> <VideoHeaderFemto> <STARTAZ>123.217163085938</STARTAZ> <ENDAZ>123.217163085938</ENDAZ> <STARTRG>0</STARTRG> <CELLDUR>41695516</CELLDUR> </VideoHeaderFemto> <VideoCellsResolution> <Compression>1</Compression> <Resolution>4</Resolution> </VideoCellsResolution> <VideoOctets> <Octets>64</Octets> <Cells>5120</Cells> </VideoOctets> <VideoBlockMediumDataVolume> <Counter>1</Counter> <VideoBlockData>78 9c ed c1 01 0d 00 00 00 c2 a0 f7 4f 6d 0f 07 14 00 00 00 00 f0 6a 14 00 00 01 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00</VideoBlockData> </VideoBlockMediumDataVolume> <TimeOfDay>0</TimeOfDay> </Message> </Asterix> <Asterix> <Category>240</Category> <Length>99</Length> <Message> <DataSourceIdentifier> <SAC>51</SAC> <SIC>1</SIC> </DataSourceIdentifier> <MessageType>2</MessageType> <VideoRecordHeader>329083</VideoRecordHeader> <VideoHeaderFemto> <STARTAZ>123.305053710938</STARTAZ> <ENDAZ>123.305053710938</ENDAZ> <STARTRG>0</STARTRG> <CELLDUR>41695516</CELLDUR> </VideoHeaderFemto> <VideoCellsResolution> <Compression>1</Compression> <Resolution>4</Resolution> </VideoCellsResolution> <VideoOctets> <Octets>64</Octets> <Cells>5120</Cells> </VideoOctets> <VideoBlockMediumDataVolume> <Counter>1</Counter> <VideoBlockData>78 9c ed c1 01 0d 00 00 00 c2 a0 f7 4f 6d 0f 07 14 00 00 00 00 f0 6a 14 00 00 01 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00</VideoBlockData> </VideoBlockMediumDataVolume> <TimeOfDay>0</TimeOfDay> </Message> </Asterix> <Asterix> <Category>240</Category> <Length>99</Length> <Message> <DataSourceIdentifier> <SAC>51</SAC> <SIC>1</SIC> </DataSourceIdentifier> <MessageType>2</MessageType> <VideoRecordHeader>329085</VideoRecordHeader> <VideoHeaderFemto> <STARTAZ>123.480834960938</STARTAZ> <ENDAZ>123.480834960938</ENDAZ> <STARTRG>0</STARTRG> <CELLDUR>41695516</CELLDUR> </VideoHeaderFemto> <VideoCellsResolution> <Compression>1</Compression> <Resolution>4</Resolution> </VideoCellsResolution> <VideoOctets> <Octets>64</Octets> <Cells>5120</Cells> </VideoOctets> <VideoBlockMediumDataVolume> <Counter>1</Counter> <VideoBlockData>78 9c ed c1 01 0d 00 00 00 c2 a0 f7 4f 6d 0f 07 14 00 00 00 00 f0 6a 14 00 00 01 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00</VideoBlockData> </VideoBlockMediumDataVolume> <TimeOfDay>0</TimeOfDay> </Message> </Asterix> <Asterix> <Category>240</Category> <Length>99</Length> <Message> <DataSourceIdentifier> <SAC>51</SAC> <SIC>1</SIC> </DataSourceIdentifier> <MessageType>2</MessageType> <VideoRecordHeader>329088</VideoRecordHeader> <VideoHeaderFemto> <STARTAZ>123.744506835938</STARTAZ> <ENDAZ>123.744506835938</ENDAZ> <STARTRG>0</STARTRG> <CELLDUR>41695516</CELLDUR> </VideoHeaderFemto> <VideoCellsResolution> <Compression>1</Compression> <Resolution>4</Resolution> </VideoCellsResolution> <VideoOctets> <Octets>64</Octets> <Cells>5120</Cells> </VideoOctets> <VideoBlockMediumDataVolume> <Counter>1</Counter> <VideoBlockData>78 9c ed c1 01 0d 00 00 00 c2 a0 f7 4f 6d 0f 07 14 00 00 00 00 f0 6a 14 00 00 01 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00</VideoBlockData> </VideoBlockMediumDataVolume> <TimeOfDay>0</TimeOfDay> </Message> </Asterix> <Asterix> <Category>240</Category> <Length>99</Length> <Message> <DataSourceIdentifier> <SAC>51</SAC> <SIC>1</SIC> </DataSourceIdentifier> <MessageType>2</MessageType> <VideoRecordHeader>329089</VideoRecordHeader> <VideoHeaderFemto> <STARTAZ>123.832397460938</STARTAZ> <ENDAZ>123.832397460938</ENDAZ> <STARTRG>0</STARTRG> <CELLDUR>41695516</CELLDUR> </VideoHeaderFemto> <VideoCellsResolution> <Compression>1</Compression> <Resolution>4</Resolution> </VideoCellsResolution> <VideoOctets> <Octets>64</Octets> <Cells>5120</Cells> </VideoOctets> <VideoBlockMediumDataVolume> <Counter>1</Counter> <VideoBlockData>78 9c ed c1 01 0d 00 00 00 c2 a0 f7 4f 6d 0f 07 14 00 00 00 00 f0 6a 14 00 00 01 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00</VideoBlockData> </VideoBlockMediumDataVolume> <TimeOfDay>0</TimeOfDay> </Message> </Asterix> </RadarVideos>

cctv vessel target data

Data format: !CCEXT,CameraID,Count,X1,Y1,X2,Y2,SHIPTYPE*H,CR/LF

| No. | Field name | Type | Contents |
| --- | --- | --- | --- |
| 1 | SentenceID | String | Starting letter |
| 2 | CameraID | String | CCTV camera Delimter (1~65,000) |
| 3 | Count | String | Number of vessel (maximum n) |
|  | Start interval (n times) |  |  |
| 4 | X1 | String | Pixel point (Left, Up) |
| 5 | Y1 | String |  |
| 6 | X2 | String | Pixel point (Right, Down) |
| 7 | Y2 | String |  |
| 8 | ShipType | String | AIS standard (ITU-R M.1371) |
|  | End interval (n times) |  |  |
| 4+(5*n) | Timestamp | String | 0~59s |
| 5+(5*n) | Checksum | String | H: checksum, Checksum is the XOR value of characters other than "!" and "*" |
| 6+(5*n) | Terminator | String | CR/LF: 0x0D/0x0A |

<shipDataExtractedFromCCTV> <CCTVs> <cameraID>1</cameraID> <shipCount>1</shipCount> <shipData> <shipInfo> <leftPoint>9</leftPoint> <topPoint>173</topPoint> <rightPoint>112</rightPoint> <bottomPoint>30</bottomPoint> <shipType>70</shipType> </shipInfo> </shipData> <timeStamp>35</timeStamp> </CCTVs> <CCTVs> <cameraID>1</cameraID> <shipCount>2</shipCount> <shipData> <shipInfo> <leftPoint>20</leftPoint> <topPoint>172</topPoint> <rightPoint>121</rightPoint> <bottomPoint>31</bottomPoint> <shipType>70</shipType> </shipInfo> <shipInfo> <leftPoint>36</leftPoint> <topPoint>58</topPoint> <rightPoint>127</rightPoint> <bottomPoint>177</bottomPoint> <shipType>70</shipType> </shipInfo> </shipData> <timeStamp>36</timeStamp> </CCTVs> <CCTVs> <cameraID>1</cameraID> <shipCount>3</shipCount> <shipData> <shipInfo> <leftPoint>26</leftPoint> <topPoint>173</topPoint> <rightPoint>127</rightPoint> <bottomPoint>30</bottomPoint> <shipType>70</shipType> </shipInfo> <shipInfo> <leftPoint>35</leftPoint> <topPoint>60</topPoint> <rightPoint>127</rightPoint> <bottomPoint>180</bottomPoint> <shipType>70</shipType> </shipInfo> <shipInfo> <leftPoint>110</leftPoint> <topPoint>100</topPoint> <rightPoint>213</rightPoint> <bottomPoint>130</bottomPoint> <shipType>0</shipType> </shipInfo> </shipData> <timeStamp>36</timeStamp> </CCTVs> <CCTVs> <cameraID>1</cameraID> <shipCount>4</shipCount> <shipData> <shipInfo> <leftPoint>30</leftPoint> <topPoint>171</topPoint> <rightPoint>127</rightPoint> <bottomPoint>30</bottomPoint> <shipType>70</shipType> </shipInfo> <shipInfo> <leftPoint>35</leftPoint> <topPoint>60</topPoint> <rightPoint>127</rightPoint> <bottomPoint>180</bottomPoint> <shipType>70</shipType> </shipInfo> <shipInfo> <leftPoint>110</leftPoint> <topPoint>100</topPoint> <rightPoint>213</rightPoint> <bottomPoint>130</bottomPoint> <shipType>0</shipType> </shipInfo> <shipInfo> <leftPoint>155</leftPoint> <topPoint>106</topPoint> <rightPoint>232</rightPoint> <bottomPoint>169</bottomPoint> <shipType>0</shipType> </shipInfo> </shipData> <timeStamp>36</timeStamp> </CCTVs> <CCTVs> <cameraID>1</cameraID> <shipCount>5</shipCount> <shipData> <shipInfo> <leftPoint>8</leftPoint> <topPoint>173</topPoint> <rightPoint>111</rightPoint> <bottomPoint>32</bottomPoint> <shipType>70</shipType> </shipInfo> <shipInfo> <leftPoint>33</leftPoint> <topPoint>61</topPoint> <rightPoint>126</rightPoint> <bottomPoint>178</bottomPoint> <shipType>70</shipType> </shipInfo> <shipInfo> <leftPoint>111</leftPoint> <topPoint>99</topPoint> <rightPoint>213</rightPoint> <bottomPoint>129</bottomPoint> <shipType>0</shipType> </shipInfo> <shipInfo> <leftPoint>155</leftPoint> <topPoint>106</topPoint> <rightPoint>232</rightPoint> <bottomPoint>167</bottomPoint> <shipType>0</shipType> </shipInfo> <shipInfo> <leftPoint>235</leftPoint> <topPoint>137</topPoint> <rightPoint>145</rightPoint> <bottomPoint>215</bottomPoint> <shipType>70</shipType> </shipInfo> </shipData> <timeStamp>37</timeStamp> </CCTVs> </shipDataExtractedFromCCTV>

VHF status data

Data format: !VHFST,VHFID,Channel,VHFStatus,Timestamp*H,CR/LF

| No. | Field name | Type | Contents |
| --- | --- | --- | --- |
| 1 | SentenceID | String | Starting letter |
| 2 | VHFID | String | VHF Delimter (1~65,000) |
| 3 | Channel | String | Channel (1~88) |
| 4 | VHF Status | String | 0= Off, 1= PTT, 2= Busy |
| 5 | Timestamp | String | 0~59s |
| 6 | Checksum | String | H: checksum, Checksum is the XOR value of characters other than "!" and "*" |
| 7 | Terminator | String | CR/LF: 0x0D/0x0A |

* vhf data excludes xml conversion

weather status data

Data format: !WEATH,DeviceID,Temp,Humidity,WindDirection,WindSpeed,Visibility,Timestamp*H,CR/LF

| No. | Field name | Type | Contents |
| --- | --- | --- | --- |
| 1 | SentenceID | String | Starting letter |
| 2 | Device ID | String | Device Delimter (1~65,000) |
| 3 | Temp | String | Temperature (℃) |
| 4 | Humidity | String | Humidity (%) |
| 5 | WindDirection | String | Wind direction (0~359.9°) |
| 6 | WindSpeed | String | Wind speed (m/s) |
| 7 | Visibility | String | Visibility (km) |
| 8 | Timestamp | String | 0~59s |
| 9 | Checksum | String | H: checksum, Checksum is the XOR value of characters other than "!" and "*" |
| 10 | Terminatior | String | CR/LF: 0x0D/0x0A |

<?xml version="1.0" ?> <Weathers> <Weather> <DeviceId>1</DeviceId> <Temperature>23</Temperature> <Humidity>2</Humidity> <WindDirection>62.3</WindDirection> <WindSpeed>54</WindSpeed> <Visibility>0.7</Visibility> </Weather> <Weather> <DeviceId>2</DeviceId> <Temperature>20.3</Temperature> <Humidity>4</Humidity> <WindDirection>290</WindDirection> <WindSpeed>4.1</WindSpeed> <Visibility>0.8</Visibility> </Weather> <Weather> <DeviceId>3</DeviceId> <Temperature>22.5</Temperature> <Humidity>6</Humidity> <WindDirection>32</WindDirection> <WindSpeed>10.3</WindSpeed> <Visibility>1.1</Visibility> </Weather> <Weather> <DeviceId>4</DeviceId> <Temperature>19.8</Temperature> <Humidity>2</Humidity> <WindDirection>282</WindDirection> <WindSpeed>4.0</WindSpeed> <Visibility>0.9</Visibility> </Weather> <Weather> <DeviceId>5</DeviceId> <Temperature>21.5</Temperature> <Humidity>5</Humidity> <WindDirection>10</WindDirection> <WindSpeed>7.5</WindSpeed> <Visibility>0.4</Visibility> </Weather> <Weather> <DeviceId>6</DeviceId> <Temperature>18.1</Temperature> <Humidity>1</Humidity> <WindDirection>279</WindDirection> <WindSpeed>1.5</WindSpeed> <Visibility>0.9</Visibility> </Weather> <Weather> <DeviceId>7</DeviceId> <Temperature>15.2</Temperature> <Humidity>8</Humidity> <WindDirection>52</WindDirection> <WindSpeed>9.4</WindSpeed> <Visibility>0.9</Visibility> </Weather> <Weather> <DeviceId>8</DeviceId> <Temperature>17.6</Temperature> <Humidity>3</Humidity> <WindDirection>243</WindDirection> <WindSpeed>4.4</WindSpeed> <Visibility>0.2</Visibility> </Weather> <Weather> <DeviceId>9</DeviceId> <Temperature>25.4</Temperature> <Humidity>4</Humidity> <WindDirection>306</WindDirection> <WindSpeed>4.8</WindSpeed> <Visibility>0.3</Visibility> </Weather> <Weather> <DeviceId>10</DeviceId> <Temperature>21.1</Temperature> <Humidity>2</Humidity> <WindDirection>15</WindDirection> <WindSpeed>2.7</WindSpeed> <Visibility>0.4</Visibility> </Weather> </Weathers>

rEVISION OF IVEF WITH VTS DATA FORMAT

- <MSG_IVEF> - <Header> <!-- HEADER CONTENT --></Header> - <Body> - <CustomData> - <!—NEW INSERTION OF RADAR VIDEOS DATA--> - <RadarVideos> - <Asterix> - <Category>240</Category> - <Length>99</Length> - <Message> - <DataSourceIdentifier> - <SAC>51</SAC> - <SIC>1</SIC> - </DataSourceIdentifier> - <MessageType>2</MessageType> - <VideoRecordHeader>329082</VideoRecordHeader> - <VideoHeaderFemto> - <STARTAZ>123.217163085938</STARTAZ> - <ENDAZ>123.217163085938</ENDAZ> - <STARTRG>0</STARTRG> - <CELLDUR>41695516</CELLDUR> - </VideoHeaderFemto> - <VideoCellsResolution> - <Compression>1</Compression> - <Resolution>4</Resolution> - </VideoCellsResolution> - <VideoOctets> - <Octets>64</Octets> - <Cells>5120</Cells> - </VideoOctets> - <VideoBlockMediumDataVolume> - <Counter>1</Counter> - <VideoBlockData>78 9c ed c1 01 0d 00 00 00 c2 a0 f7 4f 6d 0f 07 14 00 00 00 00 f0 6a 14 00 00 01 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00</VideoBlockData> - </VideoBlockMediumDataVolume> - <TimeOfDay>0</TimeOfDay> - </Message> - </Asterix> - </RadarVideos> - </CustomData> - <ObjectDatas> - <!—ORIGINE OTHER IVEF MASSAGE DATA --> - </ObjectDatas> - </Body> - </MSG_IVEF>

- It is recommended to adding the customdata tag to the end of the body tag in XML tree to avoid interfering with other existing data elements.

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain