Input paper: PAP56-2.21.1

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM	□  ENG		X  PAP					X  Input

□  ENAV	□  VTS							□ Information

Agenda item			2.21.1

Author(s) / Submitter(s)			Secretariat

# Review of concepts, features and attributes of S-201 AtoN information and development of an AtoN concept registry

# Summary

This document reviews the concepts, features, and attributes of S-201, a standardized Product Specification for Marine Aids to Navigation (AtoN), under the IALA S-200 framework. It highlights the challenges of evolving terminologies, identifies discrepancies between IALA and IHO definitions, and proposes harmonization strategies. The paper aims to create a shared understanding of terms and definitions to ensure consistent and reliable AtoN data for IALA Member States and global maritime stakeholders.

# Background

AtoN are essential infra used to ensure the safety and efficiency of maritime traffic. These aids include buoys, beacons, lights, sound signals, and other systems that guide mariners and enhance navigational accuracy. Traditionally, the management and understanding of these AtoN have relied on established terms, symbols, and definitions, many of which have remained unchanged for decades.

The introduction of S-201, a Product Specification under IALA S-200 framework, marks a significant step in modernizing the digital representation and exchange of AtoN information. S-201 ensures that data on marine AtoN is structured, interoperable, and easily integrated with other systems of stakeholders, including nautical charts and other tools.

# purpose of the paper

This paper addresses a key challenge posed by S-201: the evolution of terms and definitions. Many of the traditional terms used in marine AtoN have been updated, creating potential discrepancies between IALA's definitions, the IHO's terms, and broader maritime usage. This document compares these definitions, identifies inconsistencies, and proposes a path toward harmonization.

The ultimate goal is to create a shared understanding of terms across Member States and stakeholders. Once internal agreement is reached within IALA, the changes will be communicated to the IHO for integration into their Geospatial Information (GI) Registry (registry.iho.int), which serves as a global standard for S-100 data. The agreed definitions could also form the basis of an IALA user manual or publication to support mariners and other users of S-201.

# key findings from the review

During the comparative analysis, several observations emerged:

- Discrepancies between IALA and IHO terminologies:

  - IHO terms and definitions are designed primarily for use in nautical charts and focus on the portrayal of AtoN on paper and ECDIS.

  - IALA’s terms emphasize practical application and operational use by mariners, such as the management and maintenance of AtoN.

- Adaptation and harmonization:

  - Some concepts and terms from the IHO are suitable for adoption by IALA.

  - In cases where agreement cannot be reached, it is recommended that IALA informs the IHO and works to address these discrepancies through updates to the IHO GI Registry.

- Terminology framework:

  - All terms must be registered as concepts, and depending on their use in Product Specifications, they can be registered as features or attributes.

    - Concepts: Broad ideas that encompass various aspects of marine navigation, such as safety, efficiency, and operational management.

    - Features: Physical entities represented within the S-201 framework, such as buoys, beacons, and lighthouses.

    - Attributes: Specific characteristics of features, like colour, shape, light patterns, or operational status.

These terminologies align with the principles of the IHO’s S-100 framework, which defines the general structure for geospatial data in maritime contexts.

Figure 1 Example of Comparative table of IALA and IHO term and definitions

Figure 2 Example of Reference and legend of IALA AtoN related concepts and symbols

# Timeline and Next Steps

To ensure comprehensive review and input from stakeholders, the following timeline is proposed:

- First Half of 2025: Committees review the findings and provide feedback.

- Second Half of 2025: The Secretariat incorporate feedback, refine the document, and recirculate it for final approval and publication.

Recognizing the dynamic nature of maritime technology and data standards, the publication will be treated as a living document, updated periodically to reflect ongoing developments and consensus.

# Action Requested of the PAP

is invited to:

- Review the annexed list of terms, definitions, and discrepancies between IALA and IHO terminologies.

- Provide feedback and recommendations to the Secretariat.

- Support the harmonization efforts by identifying additional areas requiring attention or improvement.

Annex A Comparative table of IALA and IHO term and definitions

See attached document

Annex B Reference and legend of IALA AtoN related concepts and symbols

See attached document