Input paper:   VTS57-9.2.2

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM	□  ENG		□  PAP					√  Input

□  ENAV	X  VTS							□  Information

Agenda item 			9.2

Technical Domain / Task Number 2	Task 2.2.2b

Author(s) / Submitter(s)			Korean Coast Guard, Republic of Korea

# PROPOSAED REVISION TO G1111-1 PRODUCING REQUIREMENTS FOR THE CORE VTS SYSTEM

## Summary

This paper introduces operational examples of cloud VTS development projects led by the Coast Guard and proposes amendments to G1111-1 5.4 CHARACTERISTICS OF EXTERNAL INFORMATION EXCHANGE IN VTS for data exchange between VTS systems and external systems.

Cloud VTS is a centralized method of data and can be viewed as the concept of big data, which is a large amount of data. With the introduction of digital VTS services in the future, not only existing VTS data but also more data will be digitized and integrated, so big data is essential for the development of artificial intelligence technology for VTS advancement.

Revision to G1111-1 5.4 CHARACTERISTICS OF EXTERNAL INFORMATION EXCHANGE IN VTS is recommended because radar video and voice communication, which are the most important data in the VTS system, need to be included within the data exchange.

## Purpose of the document

The purpose of this paper is to introduce our country's VTS operating environment and cloud VTS, and to propose the revision of G1111-1 5.4 CHARACTERISTICS OF EXTERNAL INFORMATION EXCHANGE IN VTS to include radar video and voice communication, which are the most important data in the VTS system, in the data exchange.

## Related documents

G1111-1 Ed1.0 Producing Requirements for Core VTS Systems and Equipment.

# Background

The Korea Coast Guard continued to propose the need to revise the IVEF service recommendations by introducing the cloud VTS' data exchange system from the 51st to 56th committees of the VTS.
Radar and voice among cloud VTS data exchange systems have synchronization problems when applied to IVEF in the form of real-time streaming, so the VTS 57th Committee proposed an amendment to include radar video and voice communication in traffic management in G1111-15.4 Table 4.

# Discussion

## VTS operation Status in South Korea

South Korea operates 19 VTS centers including 3 Regional VTS. The purpose of operating VTS is to strengthen monitoring and managing ship movements to prevent congestion or dangerous situations and improving overall traffic efficiency. Notably 3 regional VTS centers have been contributing to facilitate continuous maritime traffic management from ship’s departure to ship’s arrival outside the purview of VTS sectors and VTS area in charge.

## Introduction of Cloud VTS

To consolidate data from multiple VTS centers into a single location South Korea conducted the Cloud VTS project from 2021 to 2023.

Traditionally VTS has relied on the IVEF communication protocol for limited data exchange with nearby VTS that use different operational systems. However, unlike the operational-level data exchange protocol (IVEF), the standards for data collection within VTS systems vary significantly among manufacturers and are often proprietary.

As VTS areas expand to ensure the safe and efficient operation of vessels, and various data exchanges are becoming increasingly necessary for VTS operators to make prompt and accurate support of decisions-making, as more data is generated in the maritime industry with the emergence of MASS (Maritime Autonomous Surface Ships). Therefore, it is important to share data to support more efficient vessel traffic service based on the results of integrating and processing data from several VTS centers.

So the following figure compares the current VTS system with the Cloud VTS project.

VTS operators can access information from local radar sites or the Cloud platform. In addition to data exchange through IVEF, data from Radar screens, VHF, CCTV, and other sources can also be exchanged, making it possible to utilize them for control support in emergency situations. The Cloud platform enables monitoring of VTS data nationwide in South Korea. And it is planned to expand to all VTS using the Cloud platform without any constraints.

## Testbed operation with verification on VTS data format

The concept of Cloud VTS is to collect VTS information acquired by other centers or external agencies through a central cloud server and provide services by accessing it from VTS centers or other agencies. To achieve this goal, a test-bed has been set up.

The test-bed is composed of the following: Information from Busan Port, Ulsan Port, and Incheon Port is collected on the integrated platform, and the information is displayed on the cloud monitor. The integrated platform can collect and display Radar screen, chart, VHF information, CCTV, and weather information from three VTS centers.

## Introduction to the use of voice communication data through the establishment of a VTS big data system

The Korea Coast Guard established a big data system at Daesan Port VTS to implement vessel traffic service (VTS) support functions and installed pilot operated software that digitizes the VHF Logbook, among other functions.  In this process, the STT (Speech to Text) artificial intelligence model, which was able to convert well in general phone calls, was unable to properly convert the special radio wave environment called VHF and the English pronunciation of ship operators from various countries. To solve this problem, we were able to improve STT performance by acquiring a large amount of VHF voice data over a long period of time and training an AI model.

The Korea Coast Guard converts voice communication into text to automatically extract key content related to vessel traffic service (VTS) and converts this into a database so that it can be combined with static information on existing ships. If necessary, we have developed technology that allows controllers to view past voice communication history in text and voice.

- Example of automatic extraction of         (b) Check voice communication content by time

voice communication content

# Proposals for amendments to G1111-1

## Revising 5. EXTERNAL DATA EXCHANGE AND INFORMATION SHARING

For data exchange between the VTS system and the external system, all data of the VTS must be gathered in a specific place, and functions such as a backup VTS control system and an integrated VTS decision-making system can be performed in this process. This is the concept of cloud VTS in Korea, and for this purpose, radar and voice communication, which are the most important data in the VTS system, need to be included in the data exchange content.

## Revising 5. 4 CHARACTERISTICS OF EXTERNAL INFORMATION EXCHANGE IN VTS project development process

Big data necessary for learning is essential for the development of artificial intelligence technology to advance VTS, and for this purpose, radar and voice communication, the most important data in the VTS system, should be included in the data exchange content.

The table 4. Typical information exchange between VTS and shore-based entities in G1111-1 5.4 CHARACTERISTICS OF EXTERNAL INFORMATION EXCHANGE IN VTS suggests adding 'Radar video' and 'Voice Communication' to the existing content of Traffic Management.

| Purpose | Type of information exchange |
| --- | --- |
| Traffic management | VTS support including real-time Traffic Image Anchorage and berth management Bridge and lock management Radar Video Voice communication |
| Hazard management | Risk analysis Incident reporting and investigation Contingency planning Emergency towage and salvage |
| SAR | Medical and aeronautical support Incident assistance |
| Logistic chain support | ETA at Berth Cargo handling Voyage monitoring Port operation Forward planning movements Pilotage and allied services |
| Law enforcement | Maritime contraventions Fisheries enforcement Customs Port state control Border control / immigration Port health inspections Security |
| Environmental protection | Pollution monitoring Incident response Waste management |
| Waterways infrastructure management (including inland waterways) | AtoN operations and system optimization Infrastructure maintenance and update |
| Maritime safety information (MSI) | Navigation warnings (S-124), etc |

The expected effect of this proposal is to integrate and analyze big data such as traffic images and radar videos to implement AI-based ship anomaly detection or collision avoidance function that predicts the opponent's ship's trajectory.

In addition, through natural language processing of voice data, voice can be converted into text, analyzed, and key information such as emergency messages other than the channel being listened to can be summarized and delivered to VTSO.

# Action requested of the Committee

- The Committee is requested to:

- To consider the proposal, and continue to revising the draft Guideline.

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain