IALA Guideline

G1141

Operational Procedures for Delivering VTS

Edition 3.0

June 2024

urn:mrn:iala:pub:g1141:ed3.0

Revisions to this document are to be noted in the table prior to the issue of a revised document.

| Date | Details | Approval |
| --- | --- | --- |
| 14 December 2018 | First issue Guideline prepared to align Recommendation R0127 (V-127) – VTS Operations, following adoption of IALA Standards | Council 68 |
| June 2021 | Edition 2.0 Revised to reflect new and amended IALA documentation and ensure the guideline provides an affective document to assist authorities to implement and operate a VTS in a global and harmonized manner. | Council 73 |
| January 2022 | Edition 2.1 Approved by Council December 2021 and published January 2022, in alignment with IMO Resolution A.1158(32) Guidelines for Vessel Traffic Services. | Council 74 |
| July 2022 | Edition 2.2 Editorial corrections. |  |
| June 2024 | Edition 3.0 Revised to align with other IALA documentation. | Council 80 |

1.	INTRODUCTION	5

2.	Purpose of the document	5

2.1.	ReLAtionship to other documents	5

3.	Content of the document	6

4.	Internal VTS Procedures	6

4.1.	Routine Procedures	6

4.1.1.	Document Control	6

4.1.2.	Gathering and Recording of Information	6

4.1.3.	Equipment Operation, Maintenance, Calibration and Updating	7

4.1.4.	Public Relations	7

4.1.5.	Security	7

4.1.6.	Training	7

4.1.7.	Watch Handover	8

4.1.8.	Vessel Handover	8

4.1.9.	Maintenance of Nautical Publications and charts	8

4.1.10.	Casualty, Incidents and near-miss Recording	8

4.1.11.	Administrative/ Management	9

4.2.	Emergency Procedures	9

5.	External VTS Procedures	10

5.1.	Routine Procedures	10

5.1.1.	VTS Voice Communication	10

5.1.2.	Pre-Arrival Information	11

5.1.3.	Vessels Entering VTS Area	11

5.1.4.	Vessels movements Within VTS Area	11

5.1.5.	Monitoring and management of vessel traffic	12

5.1.6.	Permission to Proceed From or To an Alongside Berth or Anchorage	12

5.1.7.	Responding to developing unsafe situations	13

5.1.8.	Vessels at Anchor	13

5.1.9.	Vessels at Berth	13

5.1.10.	Vessels Departing the VTS Area	14

5.1.11.	Transition between Adjacent VTS Areas	14

5.1.12.	Adverse Environmental Conditions	14

5.1.13.	Environmental Protection	14

5.1.14.	Digital Maritime Services	15

5.1.15.	Interaction with Allied Services	15

5.2.	Emergency Procedures	15

5.2.1.	Collision, Capsizing, Sinking, Grounding, Fire Onboard, Man Overboard	15

5.2.2.	Pollution	16

5.2.3.	Places of Refuge	16

5.2.4.	Medical Emergency	16

5.2.5.	Vessel Not Under Command (NUC)	16

5.2.6.	Security Incident	16

5.2.7.	Protest Action	17

5.2.8.	Natural Disaster	17

6.	Evaluation of Procedures	17

7.	Defintions	17

8.	ABBREVIATIONS	17

9.	References	17

ANNEX A	NAVIGATIONAL Support – Pre-Assessment considerations	19

ANNEX B	CONSIDERATIONS FOR INFORMATION THAT A VTS May request FROM a VESSEL IN EMERGENCY SITUATIONS	20

INTRODUCTION

To ensure the harmonized delivery of VTS worldwide processes and procedures should be implemented in a manner that are consistent with Recommendation R0127 (V-127) VTS Operations [1] relating to implementing processes and procedures to ensure standards are consistently maintained and the service is delivered accurately.

For the purposes of this Guideline:

Process: “is defined as a series of actions or steps taken in order to achieve a particular end.”

Procedure: “is defined as an established or official way of doing something.”

Processes and procedures provide the mechanism to:

Ensure tasks are performed correctly and that the provision of VTS is consistent globally;

Ensure conformance with the entity’s policy and regulatory framework;

Provide guidance for decision-making; and

Streamline internal and external processes.

IMO Resolution A.1158(32) Guidelines for Vessel Traffic Services [2] states that:

  - Vessel traffic services are provided worldwide and make a valuable contribution to the safety of navigation, improved efficiency of traffic flow and the protection of the marine environment.

  - The level of safety and efficiency in the movement of maritime traffic within an area covered by vessel traffic services is dependent upon close cooperation between those operating the vessel traffic services and participating ships.

  - The use of differing procedures may cause confusion to ship masters, and that Vessel Traffic Services should be established and operated in a harmonized manner and in accordance with internationally approved guidelines.

Purpose of the document

The purpose of this Guideline is to provide a framework for authorities to implement processes and procedures associated with the provision of VTS.

ReLAtionship to other documents

IALA Guideline G1141 Operational procedures for Vessel Traffic Services [3] is associated with Recommendation R0127 (V-127) VTS Operations, a normative provision of IALA Standard S1040 Vessel Traffic Services (VTS)[4]. To demonstrate compliance with the Recommendation, the provisions of this Guideline need to be implemented.

Note: This Guidance should be read in conjunction with Recommendation R0132 Quality Management for Aids to Navigation Authorities (O-132) [5] and Guideline G1052 Quality Management Systems for AtoN Service Delivery [6], noting that G1052 states that documentation for the Quality Management System (QMS) should include, inter alia:

Procedure documents, e.g., Operating guidelines, operating procedures, work instructions

Record keeping

Content of the document

The Guideline describes processes and procedures associated with:

Day-to-day operations to;

provide timely and relevant information on factors that may influence the vessel’s movements and assist onboard decision-making;

monitor and manage vessel traffic to ensure the safety and efficiency of vessel movements; and

respond to developing unsafe situations.

Management and administration to;

ensure the VTS operates in accordance with relevant international conventions and IMO instruments, IALA standards and national law; and

set operational objectives for the VTS that are consistent with improving safety and efficiency of vessel traffic and protection of the environment and routinely evaluating that they are being achieved.

These processes and procedures are separated into activities that are either internal or external to the VTS, and subsequently split into routine or emergency:

Internal Procedures – procedures that cover the day-to-day running of a VTS, including but not limited to the operation of systems and sensors, interactions among the staff and the internal management, etc.

External Procedures – procedures that cover the interaction with participating ships and allied services.

Internal VTS Procedures

VTS authorities should implement processes and procedures for all internal activities related to the VTS. A clear distinction should be made between routine and emergency activities as described below.

Recognizing that activities may vary between VTS, these may be further adapted to suit local needs.

Routine Procedures

Document Control

Processes and procedures should be documented, regularly reviewed, and version control maintained. These documents should be available to VTS personnel and form an integral part of training where adherence is routinely monitored.

Further, to achieve collaboration, it is recommended that these procedures (or part of them) may be shared with allied services.

Gathering and Recording of Information

Provision should be made for the storage, security, retrieval and presentation of this information. This type of information may include:

Communications (internal and/or external)

Sensor data (i.e., data used to generate the traffic image such as radar, CCTV, AIS)

Vessel data and voyage information (e.g., vessel particulars, cargo data, including vessel movement information)

Meteorological and hydrographic data

Data from other sources

The internal procedures should specify the time period for which VTS information is stored. This time period should be such that it allows for the full retrieval of data post-incident/accident, in compliance with national requirements and those of the incident/accident investigation procedures of the VTS provider and other interested parties.

Equipment Operation, Maintenance, Calibration and Updating

All manuals and handbooks for equipment operation, maintenance (preventative and remedial), calibration and updating should be properly maintained and readily available to the appropriate personnel.

Key considerations include:

Descriptions of equipment and systems used in the VTS

Operation of equipment to cover all normal and emergency procedures

Determining performance availability criteria for equipment, including the availability of VTS during maintenance or other outages

Categorization and prioritization of maintenance and defects

Calibration of sensors within a specified tolerance level

Updates to equipment (hardware and software) and their associated manuals/handbooks

VTS availability due to maintenance or other outages.

Public Relations

There should be documented processes and procedures to deal with media and public relations issues. Issues for consideration may include:

Nominated lead department for public relations

Publication of ongoing activities within the VTS area

Protection of sensitive information

Security

Processes and procedures should be clearly documented and consistent with local and national requirements. As a minimum, they should ensure the security for:

Data transmission and storage (e.g., Cyber Security);

VTS personnel; and

VTS buildings and structures.

Procedures should reflect any involvement of the VTS with the Port Facility Security Plan (PFSP) as per the International Ship and Port facility Security Code (ISPS) [7].

Training

Processes and procedures should be available to ensure that:

VTS personnel are appropriately trained and qualified for their VTS duties;

Qualifications and operational performance are maintained and confirmed through performance assessments undertaken at regular intervals; and

Training records of VTS personnel are maintained.

Please note, that Guideline G1156 Recruitment, training and assessment of VTS personnel [8] provides further guidance on the training and assessment of VTS personnel.

Watch Handover

Watch handover arrangements should be formalized and include, as an example, information on:

Present traffic situation;

Expected/developing traffic situations;

Incidents and special operations (e.g., SAR or military operations);

Waterway conditions;

Environmental conditions (e.g., meteorological conditions and tidal conditions);

Status of AtoN;

Equipment performance/availability;

Status of allied services (e.g., pilotage, port and tug services);

Personnel availability;

Appropriate times for watch handover; and

Method for documenting the handover.

Vessel Handover

The arrangements for vessel handover between adjacent sectors or areas should be clearly laid down. Components may include:

Mutual understanding of the handover procedures.

Identification of information to be communicated between operators (e.g., communication channel, vessel identification, etc.).

Method for documenting the vessel handover.

Maintenance of Nautical Publications and charts

A VTS provider should ensure that arrangements are in place for maintaining, updating and disseminating nautical publications and charts (paper and/or electronic).

Charts should be the latest editions as confirmed by the issuing authority.

Casualty, Incidents and near-miss Recording

The arrangements for the gathering and exchange of information on incidents, accidents and/or near-misses in the VTS area should be described. This may include processes and procedures to ensure that all recording equipment is operating correctly. This may also include the procedures for incident reporting and dissemination of information to relevant parties.

Please note, IALA Guideline G1118 Marine casualty / incident reporting and recording, including near‐miss situations as it relates to VTS [9] provides further information on developing and establishing processes for the reporting, recording and analysis of marine casualties, incidents and near-miss situations.

Administrative/ Management

Processes and procedures should be implemented to demonstrate that the responsibilities, practices, policies and procedures exercised by an authority to provide strategic direction, ensure objectives, manage risk and use resources responsibly and with accountability are monitored and evaluated on a routine basis. This should include documentation relating to:

VTS Objectives – Procedures to monitor and assess that the objectives set for the VTS are met and regularly reported to management. Aspects and considerations include:

Setting performance measures, including both positive (leading) and negative (lag) performance measures;

Establishing a framework to regularly assess the performance measures; and

Reporting to management.

Evaluation – Procedures to regularly carry out an evaluation to ensure that the VTS operational objectives have been met, and the problems identified and defined for implementing the VTS have been either alleviated or at least reduced to an acceptable level. Aspects and considerations in undertaking the evaluation include:

The need for the VTS, which may include monitoring and identifying changes that may have occurred since the VTS was implemented or when previous evaluation was made regarding the volume of traffic and degree of risk;

An operational evaluation of the VTS; and

An evaluation of the operational objectives and the list of problems requiring attention.

Policies – Procedures for maintaining policies associated with VTS, such as qualifications and training, compliance and enforcement.

Please note Guideline G1131 Setting and Measuring VTS Objectives [10] provides further guidance for competent authorities and VTS Authorities for setting objectives for a VTS and achieving the obligations associated with SOLAS regulation V/12 (Vessel Traffic Services) [11] and IMO Resolution A.1158(32)[2].

Emergency Procedures

A VTS provider should have documented contingency plans to ensure the safety of VTS personnel and for the continuity of operations in the event of an emergency. The VTS provider should have plans to address events such as:

System and equipment failure:

Loss of external communications;

Loss of internal communications;

Loss of functionality of sensor equipment; and

Loss of information management systems.

Internal emergencies, for example, fire and flood

Forced evacuation of VTS centre

Personnel medical emergencies

Security incidents

The following issues may be included in these plans:

Remedial action

Callout procedures

Fall-back options

Media or allied services communications

Recording of incident

Data safeguarding

Post-emergency debriefing

External VTS Procedures

A VTS provider should implement processes and procedures for all external activities related to the VTS, such as the interaction between VTS and:

- Vessel traffic, including:

the provision of timely and relevant information on factors that may influence the vessel's movements and assist onboard decision making;

the monitoring and management of vessel traffic to ensure the safety and efficiency of vessel movements;

responding to developing unsafe situations; and

communication and processing of mandatory reporting requirements from vessels and other information as deemed appropriate.

- Allied services.

A clear distinction should be made between routine and emergency activities as described below.

Recognizing that activities may vary between VTS these may be further adapted to suit local needs. Similarly, the exchange of information should be standardized where possible.

Routine Procedures

VTS Voice Communication

Processes and procedures should be established to ensure that:

VHF Communication is timely, clear, concise and unambiguous, taking into consideration:

IMO Resolution A.918(22) Standard Marine Communication Phrases [12]);

IMO Resolution A.954(23) Proper use of VHF Channels at Sea [13]);

IALA Guideline G1132 VTS Voice Communications and Phraseology [14]; and

IALA Guideline G1111-2 Producing Requirements for Voice Communications

The VHF channels used and monitored are clearly communicated.

Voice communications in VTS areas are generally conducted using VHF channels allocated in ITU Radio Regulations Appendix 18. The use of simplex channels will ensure that participating ships are able to monitor all VHF communication in the VTS area. When simplex channels are unavailable due to insufficient channels or interference, duplex channels may be used.

Pre-Arrival Information

Prior to arriving at the VTS area, a vessel may be required to provide information and notify of its intent to enter the VTS area. The VTS provider should specify the format and timing of pre-arrival information, if possible, by non-verbal means, see section 5.1.14. This may, for example, include:

Route information

ETA

Vessel Identity (Name, IMO number, Call Sign, MMSI)

Vessel dimensions as relevant

Vessel draft

Air draft

Hazardous, dangerous or polluting goods details

ISPS security level

Information about any vessel defects or deficiencies

Other specified details

Further, processes and procedures to support the arrival of vessels to the VTS area should be established. These should include:

communication requirements for participating ships;

reporting requirements for pre-arrival information;

non-compliance with reporting requirements; and

information exchange with allied services

Vessels Entering VTS Area

Processes and procedures should be established for when vessels enter the VTS Area. These may include:

Establishing communications and verifying vessel identity, position and intention.

Information exchange, such as:

Reporting requirements

Provide relevant traffic information

Provide navigational / fairway information

Vessel defects or deficiencies, such as navigation or manoeuvring equipment failure

Updating information with allied services

Vessel’s movements Within VTS Area

Procedures should be established for vessel movements within a VTS area. These may include:

Reporting formalities

Provision or exchange (AIS, VDES or other means) of relevant information to participating ships at regular intervals, on request or as deemed necessary by the VTS, including:

environmental conditions;

traffic situation;

navigational conditions;

traffic separations;

overtaking restrictions;

warnings and restrictions concerning the movement of traffic in the area; and

Maritime Safety Information.

Special provisions for vessels carrying hazardous, dangerous or polluting cargo

Compliance with pilotage requirements

Non-compliance with the requirements and procedures in the VTS area

Monitor and, if possible, communicate with vessels not required to participate in the VTS

Information exchange/update on allied services

Monitoring and management of vessel traffic

Procedures for the monitoring and management of vessel traffic should be established. These may include:

- Forward planning of vessel movements

- Organizing vessels underway

- Organizing space allocation

- Establishing a system of voyage or passage plans

- Ensuring compliance with the regulatory provisions

Permission to Proceed From or To an Alongside Berth or Anchorage

A vessel’s departure from or arrival to a berth or anchorage or when entering a lock or confined waterway is a critical moment when a vessel’s movements may have a direct influence on other vessels nearby.

In particular, the departure of a vessel from a berth is a critical moment when situational awareness of bridge staff on the departing vessel may be diverted and impaired by internal or jetty-side issues and when other vessels in the immediate vicinity might be surprised by the unexpected movement of another vessel into the fairway.

Processes and procedures for a VTS responding to a request from a vessel for permission to proceed from or to a berth or anchorage may include:

- Requirements for a vessel to request permission to proceed when it is ready to depart

- Provision of relevant traffic information to the departing vessel prior to departure

- Standard and formal message for approval from the VTS for a vessel to proceed

- Standard and formal message for refusal from the VTS for a vessel to proceed

Permission for a vessel to proceed means that, based on the information available, the VTS assesses that it is safe and gives approval for the vessel to proceed on its intended course of action, subject to the discretion of the master.

Permission for a vessel to proceed may be subject to conditions (e.g., details received from the vessel, known fairway and traffic) which may be contained in the message.

Refusal for a vessel to proceed should be unambiguous, clear and may be issued as an instruction.

Responding to developing unsafe situations

Procedures for responding to developing unsafe situations should be established, and may include:

A vessel unsure of its route or position

A vessel deviating from the route

A vessel requiring guidance to an anchoring position

A vessel that has defects or deficiencies, such as navigation or manoeuvring equipment failure

Meteorological conditions (e.g., low visibility, strong winds)

A vessel at risk of grounding or collision

Emergency response or support to emergency services

A vessel deviating from passage plan

Assistance to a vessel to support the unexpected incapacity of a key member of the bridge team

IALA Guideline G1089 Provision of VTS Services [15]states that “before navigational support is provided and if time permits, a VTS should make an assessment of capabilities and conduct other relevant checks” whilst recognizing that, when the need is observed to be necessary by the VTS, early intervention is likely to be necessary, which may preclude pre-assessment checks being carried out.

Some considerations for making a pre-assessment prior to the provision of navigational support are provided in annex A.

Please note, Guideline G1089 Provision of VTS Services provides further guidance on the provision of a VTS to participating ships in a harmonized manner in accordance with IMO Resolution A.1158(32) and IALA Standards.

Vessels at Anchor

Procedures should be established for vessels at anchor in a VTS area. Depending on the capability of the VTS to monitor the vessel position under prevailing conditions, these may include:

Anchorage assignment

Communication requirements

Reporting requirements for vessels prior to leaving the anchorage

Non-compliance with the requirements and procedures for the VTS area

Information exchange/update on allied services

Vessels at Berth

Processes and procedures should be established for vessels at berth in a VTS area. Depending on the capability of the VTS to monitor the vessel position under prevailing conditions, these may include:

Reporting requirements for vessels on arrival at berth

Non-compliance with reporting requirements

Security requirements, including security level

Special requirements to maintain a communication watch

Need for restrictions for other vessels passing the berth (e.g., during bunkering or crane operations)

Reporting requirements for vessels prior to leaving the berth

Information exchange/update on allied services

Vessels Departing the VTS Area

Processes and procedures should be established for vessels departing the VTS area. These may include:

Reporting requirements for vessels prior to departing the area

Non-compliance with reporting requirements

Handover requirements with adjacent or next VTS

Transition between Adjacent VTS Areas

Processes and procedures should be established for vessels transiting between adjacent VTS areas. The handover arrangements may include:

Transfer of vessel information such as identification, cargo, destination and ETA

Process for communication procedures

Process to ensure vessel monitoring

Adverse Environmental Conditions

In situations of adverse environmental conditions within the VTS area, such as poor visibility, strong currents or tidal streams, high winds, ice, etc. special processes and procedures may be required. These may include:

Restriction or prohibition on vessel movements

Additional reporting requirements

Additional separation between vessels

Additional requirements (e.g., mandatory tug service, pilot, etc.)

Special consideration may need to be given depending on vessel characteristics and local geography and conditions.

Environmental Protection

The role of VTS to assist with environmental protection may include marine conservation measures is increasingly being recognized internationally as a means to:

mitigate risks such as ship collisions with cetaceans and disturbance of marine mammals in nursery areas;

mitigate the effect of ship wash on the shores in the vicinity of low-lying communities; and

support the protection of Particularly Sensitive Sea Areas (PSSAs) or locally declared environmentally sensitive areas.

It may be appropriate to develop processes and procedures to:

broadcast relevant information on times and locations;

interact with individual vessels in the vicinity of marine mammals;

keep protected areas clear of traffic;

advise speed restrictions where marine mammals have been sighted, or to reduce ship wash on the shores;

re-route traffic away from sightings; and

collect information to identify potential interaction hotspots to assist in planning future mitigation measures.

Digital Maritime Services

Where the VTS provides information in digital format, processes and procedures on digital information exchange should be established. This information may include, for example, the use of AIS messages to provide information on weather, virtual AtoN or the exchange of route information between vessels and VTS.

The procedures should include a description of the digital information delivered and the communication means used for the information exchange.

Interaction with Allied Services

Processes and procedures should be established to cover the interaction between VTS and allied services, for example:

Pilots

Tugs and tug operators

Icebreakers and icebreaker operators

The organizers of marine events

Shipping agents

Government agencies, including law enforcement agencies

Please note, IALA Guideline G1102 VTS Interaction with Allied or Other Services [16] identifies issues to be considered and the principles to successfully interact between VTS and allied or other services.

Emergency Procedures

The activities of the VTS centre should be maintained during any emergency response.

Other authorities (e.g., Maritime Rescue Co-ordination Centre (MRCC)) might be responsible for handling external emergencies within the VTS area. In that case, procedures for the interaction between the VTS and these authorities need to be established.

Incidental co‐operation with emergency services, such as Search and Rescue and Pollution Control may be conducted in accordance with pre‐established contingency plans in which the procedures for interaction with allied or other services are laid down and responsibilities established.

Despite this, the VTS may be one of the first to respond to a call from a vessel that has an emergency situation. It is important that VTS requests and collects any further information to help in response activities and to pass on to other authorities. Some considerations for information that a VTS may try to establish from the vessel under these situations can be found in annex B.

Collision, Capsizing, Sinking, Grounding, Fire Onboard, Man Overboard

Processes and procedures should be established to deal with situations such as collision, capsizing, sinking, grounding, fire onboard and man overboard. Actions may include the following actions:

Alert MRCC;

Inform and co-operate with relevant emergency services;

Inform relevant regulatory authority/ies;

Act on local call-out procedures;

Support on-scene coordination;

Consider back-up VTS personnel;

Promulgate or relay information concerning situations with vessels in VTS area;

Restrict traffic in the area;

Alert allied services and other support units; and

Ensure that a proper log is maintained.

Pollution

Pollution incident processes and procedures should be established. The following actions may be included:

Alert relevant environmental authority and/or service(s);

Alert relevant response authority and/or service(s);

Inform and co-operate with relevant regulatory authority/ies;

Assess scale of incident and call in specialist support, as appropriate;

Promulgate information concerning incident to vessels in VTS area; and

Restrict traffic movements in the area.

Places of Refuge

Places of Refuge processes and procedures should be developed, depending on national requirements and the particular arrangements arising out of the implementation of IMO Resolution A.949(23) Guidelines on Places of Refuge for Ships in Need of Assistance [17].

Medical Emergency

Processes and procedures for medical emergencies should be established. Actions may include:

Inform MRCC ;

Inform coast radio station;

Consider special manoeuvring requirements; and

Relay information from responsible health authority to arriving vessels suspected of being infected with a contagious disease that requires special care e.g., quarantine.

Vessel Not Under Command (NUC)

Processes and procedures in the event of a “vessel not under command” should be established. Actions may include:

Promulgate information concerning incident to vessels in the VTS area;

Obtain detailed information about onboard situation;

Maintain communication with vessel;

Assess vessel’s proximity to danger (danger to vessel itself and other traffic); and

Alert allied services and other support units, if appropriate.

Security Incident

In the event of a security incident, processes and procedures should be established. Procedures should reflect any involvement of the VTS with the Port Facility Security Plan (PFSP) as per the International Ship and Port facility Security Code (ISPS)[7].

Protest Action

Processes and procedures should be established to respond to protest action in the VTS area. Throughout any protest action, the safety of vessels and protestors is paramount.

Actions may include:

Alert responsible authority;

Act on local call-out procedures, including but not limited to VTS manager; and

Promulgate information concerning incidents to vessels in the VTS area.

Natural Disaster

Natural disaster procedures should be established to deal with situations such as earthquakes, tsunamis, tidal waves, fire, and exceptional weather conditions such as hurricane/typhoon. Actions may include:

Alert vessels in the VTS area, including navigational warnings containing urgent information relevant to safe navigation;

Act on local call-out procedures; and

Alert MRCC of external emergencies, as set out  in 5.2.1 and 5.2.4.

Evaluation of Procedures

All processes and procedures should be regularly reviewed and evaluated for their adequacy and to support the objectives of the VTS.

- Such relevant times may include:

- Changes to regulatory requirements

- Changes to VTS infrastructure, including systems and equipment changes and upgrades

- Changes of VTS areas and sectors

- Changes of port or fairway infrastructure

- After a significant near-miss or incident

- As part of the ongoing evaluation of the VTS

The VTS provider should also ensure that VTS personnel are updated with changes to procedures and competence verified through revalidation training.

DefinItions

The definitions of terms used in this Guideline can be found in the International Dictionary of Marine Aids to Navigation (IALA Dictionary). Where conflict arises, the IALA Dictionary should be considered as the authoritative source of definitions used in IALA documents.

ABBREVIATIONS

VTS	Vessel Traffic Service or Vessel Traffic Services (dependent on context)

References

IALA. Recommendation R0127 (V-127) VTS Operations

IMO. Resolution A.1158(32) Guidelines for Vessel Traffic Services

IALA. Guideline G1141 Operational Procedures for Vessel Traffic Services

IALA. Standard S1040 Vessel Traffic Services

IALA. Recommendation R0132 Quality Management for Aids to Navigation Authorities

IALA. Guideline G1052 Quality Management in AtoN Service Delivery

The International Ship and Port Facility (ISPS) Code

IALA. Guideline G1156 Recruitment, Training and Assessment of VTS Personnel

IALA. Guideline G1118 Marine Casualty/Incident Reporting and Recording, Including Near-Miss Situations as it Relates to VTS

IALA. Guideline G1131 Setting and Measuring VTS Objectives

IMO. International Convention for the Safety of Life At Sea

IMO. Resolution A.918(22) IMO Standard Marine Communication Phrases

IMO. Resolution A.954(23) Proper use of VHF channels at sea

IALA. Guideline G1132 VTS Voice Communications and Phraseology

IALA. Guideline G1089 Provision of a VTS

IALA. Guideline G1102 VTS Interaction with Allied or Other Services

IMO. Resolution A.949(23) Guideline on Places of Refuge for Ships in Need of Assistance

NAVIGATIONAL Support – Pre-Assessment considerations

Examples of considerations for making a pre-assessment prior to the provision of navigational support may include:

Capabilities:

- VTS operator workload.

VTS equipment capabilities and limitations, performance, serviceability and back-up (particularly key elements of communications, radar and AIS).

Capability of the vessel and bridge staff to continue passage under navigational support:

Status of the ship’s navigational/communications equipment and machinery;

Knowledge and understanding of VTS and knowledge of the local area;

Language ability; and

Carriage of up-to-date charts.

- Operational Considerations:

- Positive identification of ship to be supported.

- Alternative options (e.g., anchor, provision of pilot, amendment to voyage or passage plan).

Cargo carried.

Risk if navigational support is not provided.

Environmental conditions (wind, day/night, visibility, tidal height, tidal stream).

Other ship traffic.

Communications channel for navigational support.

- Checks:

- Master’s understanding that VTS navigational support does not absolve the master from responsibility for the safety of their vessel or for collision avoidance.

Master’s acceptance of navigational support.

Commencement of navigational support.

Completion of navigational support.

CONSIDERATIONS FOR INFORMATION THAT A VTS May request FROM a VESSEL IN EMERGENCY SITUATIONS

When a VTS is the first to become aware of an actual or potential emergency, the International Aeronautical and Maritime Search and Rescue Manual (IMSAR) Volume II notes that "the information collected and the initial action taken are often critical to successful operations” It also notes that:

“The success of a SAR operation depends on the speed with which the operation is planned and carried out. Information should be gathered to help the competent response authority to determine the nature of distress, the appropriate emergency phase and what action should be taken”.

In gathering information, the VTS should ensure safety of other traffic in the VTS area and provide details to the relevant authority such as a rescue coordination centre. Some considerations for information that a VTS may request from a vessel under emergency situations are listed below. These considerations are not intended to be exhaustive or mandatory:

| Situation | Considerations |
| --- | --- |
| COLLISION | When and where did the accident happen Any injuries to persons? Any dangerous or polluting cargo? Is there any pollution? Are you taking water? Are you flooding? Are you sounding your tanks? Can you proceed by yourself? What is the damage/casualty situation? What assistance is required? Are you venting hazardous / polluting gases? |
| GROUNDING | When and where did the accident happen? Any dangerous or polluting cargo? Is there any pollution? Are you aground? Are you attempting to re-float? Are you taking water? Are you flooding? Are you sounding your tanks? Do you have any list? What is the damage/casualty situation? What assistance is required? |
| FLOODING / SINKING | Is the flooding under control? Can you proceed by yourself? What assistance is required? |
| FIRE / EXPLOSION | When and where did the accident happen? What part of the ship is on fire/has exploded? Is the fire under control? What is the damage/casualty situation? Is there any pollution? What assistance is required? |
| MARINE POLLUTION | When and where did the pollution/discharge happen? What type of oil (diesel, oil, HFO, bilge) / dangerous goods or hazardous substances have been discharged? What is the approximate size of the spill (length and breadth)? What direction is it heading? If the source of pollution is from the reporting ship: - What time did the discharge occur? - How did the discharge occur? - How much oil / dangerous goods or hazardous substances have been discharged? - Has the discharge stopped? - Can you stop the discharge? - What assistance is required? |
| MAN OVERBOARD | When and where the accident happen? How many persons overboard? Can you still see them? Did anyone see the person go overboard? When was the person last seen? Was the person overboard wearing a life jacket? What assistance is required? |
| NOT UNDER COMMAND (NUC) | What problems do you have? (e.g., , nature of the mechanical failure) Can you repair by yourself? How long [will it take] to repair? What kind of assistance is required? |
| CONTAINERS / CARGO OVERBOARD | When and where the accident happen? What kind of cargo overboard? Any dangerous or polluting cargo? How much cargo/ how many containers overboard? Are the containers visible? Floating or sinking? |
| MEDICAL ASSISTANCE | What assistance is required? Is the casualty conscious and breathing? Is the casualty male /female? What is their age? Does the casualty speak English? Does the casualty have any known health problems? What treatment has been provided to the casualty on board? Has the casualty been taking any medication? Is your vessel able to accept a helicopter (i.e., to land or use of winch only)? |
| LOSS OF PROPULSION | Are you in danger of grounding? Do you need towage assistance? What is the cause for loss of propulsion? When will propulsion be restored? |

Duplex channels should instantaneously be retransmitted on the outgoing frequency to ensure that both sides of any exchange can be heard by other ships, thus replicating a simplex channel (refer to Guideline G1111-2 Producing Requirements for Voice Communications).