Input paper: VTS59-9.1.2

Input paper for the following Committee(s): 	                              		Purpose of paper:

□  ARM	□  ENG		□  PAP					☑  Input

□  ENAV	X  VTS							□  Information

Agenda item			9.1

Technical domain/ Task number	2.5.2a

Author(s)/Submitter(s)	               China Maritime Safety Administration

# Proposals on "Service Specification for VTS Traffic Clearance Edition 1.5"

# Summary

This proposal proposes revisions to Service Specification for VTS Traffic Clearance Edition 1.5 from the aspects of framework, operational context, use cases, functional and non-functional requirements, operational nodes, service overview, service data model and service dynamic behaviour.

## Purpose of the document

The purpose of this document is to provide input document for the VTS Committee to update the Service Specification for VTS Traffic Clearance Edition 1.5.

## Related documents

- IALA VTS Committees Task Plan 2025-2027

https://iala-task-register.com/committee-homepage?id=1

[2] 	IALA VTS53-6.3.2 Development of technical service specifications for digital data exchange between VTS and other entities - primarily ships

[3] 	IALA VTS54-9.1.3 Proposals on the New Task of "Developing Technical Service Specifications for   Digital Data Exchange between VTS and Other Entities - Primarily Ships"

[4] 	IALA VTS55-9.1.1 Proposals on “Service Specification for VTS Traffic Clearance Service_V1.0"

[5] 	IALA VTS56-9.1.1 Proposal on "Service Specification for VTS Traffic Clearance Service Edition 1.3" and "Service design"

[6] 	IALA VTS57-9.1.2 Proposals on "Service Specification for VTS Traffic Clearance Edition 1.4"

[7] 	IALA VTS57-12.3.7 Service Specification for VTS Traffic Clearance Edition 1.5

[8] 	IALA VTS58-12.2.3 Service Specification for VTS - Vessel Route Exchange Edition 1.1

[9] 	IALA Revised G1128 Specification of e-Navigation Technical Services Edition 1.7

[10] 	IALA  C03-10.4.3 Revised G1191 Maritime Service Registry (MSR) Technical Specification Edition 1.1

[11]       IALA VTS58-12.1.1 New Guideline on Digital Communications

# Background

The IALA VTS Committee Task Plan 2023-2027 raises the task "Development of technical service specifications for digital data exchange between VTS and other entities - primarily ships" (task 2.5.2), and aims to finalize the pioneer technical service specifications before VTS57. Service Specification for VTS Traffic Clearance Service Edition 1.5 had been finalized at VTS57. The working document was not updated during the intersessional meeting on November 26, 2025.

As the co-sponsor of  the new work task proposal (VTS 53-6.3.2) at  VTS53, China MSA continued to submit "Proposals on the New Work Task of Developing Technical Service Specifications for Digital Data Exchange between VTS and Other Entities (VTS54 9.1.3) " at VTS54, "Proposals on Service Specification for VTS Traffic Clearance Service Edition 1.0" (VTS55-9.1.1) at VTS55, "Proposals on Service Specification for VTS Traffic Clearance Service Edition 1.3 and Service design" (VTS56-9.1.1) at VTS56, "Proposals on Service Specification for VTS Traffic Clearance Edition 1.4"(VTS57-9.1.2) .

# Discussion

Service Specification for VTS Traffic Clearance Edition 1.5 is based on G1128 Specification of e-Navigation Technical Services Edition 1.5. In June 2025, Council 02 approved G1128 Edition 1.7. In December 2025, Council 03 approved related documents including Service Specification for VTS - Vessel Route Exchange Edition 1.1, Service Design for VTS - Vessel Route Exchange Service using SECOM Edition 1.1, and G1191 Maritime Service Registry (MSR) Technical Specification Edition 1.1. After comprehensive analysis, the following revision suggestions are proposed:

## Proposals on Improving the Framework

Compared to Service Specification for VTS - Vessel Route Exchange Edition 1.1 approved by Council 03 in December 2025, the cover, table of contents, and names of certain sections of Service Specification for VTS Traffic Clearance Edition 1.5 do not effectively correspond with G1128 Specification of e-Navigation Technical Services Edition 1.7 and its Annex A Service Specification Template. Key elements such as service unique identifiers and document revision are missing. It is suggested to update the framework.

## Proposals on Improving Operating Context

Section 3. Operating Context of Service Specification for VTS Traffic Clearance Edition 1.5 contains redundant and potentially conflicting content. It is suggested to remove the following redundant material fully quoted from the New Guideline on Digital Communications: Paragraph 1 ( (A.1158(32) ), Paragraph 2 (G1089), Paragraph 4 (MS1: VTS), and Paragraph 6 (VTS Digital Communications). Retain only the essential descriptions required for Traffic Clearance Services. Furthermore, as the term "prior" does not appear in other sections of this specification, and considering the context of the entire document, it is recommended to remove the two instances describing the intent of "prior to entering a VTS area".

## Proposals on Improving Use Cases

Section 3.1 Use Cases of Service Specification for VTS Traffic Clearance Edition 1.5 contains incomplete content and non-standardized expressions. Specific suggestions are as follows:

### Enriching Sufficient Technical Detail in use cases

G1128 emphasizes that "The Service Specification must provide sufficient detail in the use cases and logical data model ", as well as "the operational context shall describe any requirement the service shall fulfil or adhere to. This refers to functional as well as non-functional requirements at a high level". The Use Cases section of "Service Specification for VTS - Vessel Route Exchange Edition 1.1" provides important parameters such as route status, route version, route information description, and confirmation, as well as their value changes, on multiple occasions. In terms of the detailed correspondence between user cases and subsequent functional and non-functional requirements, Use Case 1 of "Service Specification for VTS - Vessel Route Exchange Edition 1.1" states, "For the complete list of requirements, please refer to RESF007."Use Cases 3 points out that "for more details, see RESF012" and others are all related to the logical data model of functional requirements and other subsequent sections.

By comparing the new edition of G1128 with "Service Specification for VTS - Vessel Route Exchange Edition 1.1", it is suggested that the Use Cases details be further enriched in the context of the specification. For example, timestamps parameters such as ETA/ETD/PTR/PTD/RTA/RTD be added according to different application scenarios and business processes. And the differentiated values of the message parameters of ClearanceMessage and MessageStatus should provide sufficient details for the subsequent sections related to functional requirements and non-functional requirements, data models, etc.

### adding "Vessel can discover Traffic Clearance Service" use case

"Vessel can discover compatible route services" is provided in Use Cases 14 of Service Specification for VTS - Vessel Route Exchange Edition 1.1. Combined with the relevant requirements of the newly released G1191 Maritime Service Registry (MSR) Technical Specification Edition 1.1 in December 2025, and considering that the service specification and service design of VTS Traffic Clearance Service already have relevant contents, it is suggested to add "Vessel can discover Traffic Clearance Service" Use Case in this specification.

### UPDATING Figure 2

In combination with the relevant requirements of G1128 and multiple newly released service specifications such as route exchange, it is suggested that this figure 2 be renamed "High-level view of Traffic clearance", and supplement the relevant processes of MSR registration discovery.

Figure 1 High-level view of Traffic clearance

## Proposals on Improving Functional and Non-functional Requirements

### ADJUSTING Functional and Non-functional Requirements

In combination with the Service Specification for VTS - Vessel Route Exchange Edition 1.1 and G1191 Maritime Service Registry (MSR) Technical Specification Edition 1.1, it is suggested to add "Requirements for identities", "Requirements for authenticity", "Requirements for retrying operations" and "Requirements for encryption", and the non-functional requirements TCSNF001 Authenticity and TCSNF005 Reliability be simultaneously removed to better reflect MSR and SECOM.

Figure 2 Adjusting Functional and Non-Functional Requirements

### CLEARANCE MESSAGE RELATED PROPOSALs

"ClearanceMessage" includes attributes related to time windows. As far as TCSF005 "Approve ETA/ETD from vessel" and TCSF006 "Send RTA/RTD from VTS to vessel" are concerned, for specific guidance on providing different time windows and the status of "ClearanceMessage", it is suggested to provide the Clearance time range for permission when necessary, and to simultaneously supplement and improve the relevant details in the user cases and related figures as well.

"ClearanceMessage" includes attributes related to "VTSLocation affectedArea". Considering that "The service must provide its area of interest in the geometry used when registering to a service registry", it is suggested to refer to Service Specification for VTS - Vessel Route Exchange Edition 1.1, add the relevant functional requirements of "Provide area of interest of VTS to vessel", and simultaneously supplement and improve the relevant details in the use cases and related figures.

## Proposals on Improving Operational Nodes

There are only two operational nodes, namely Vessel and VTS centre. There is a lack of nodes such as Service and Consumer that are mandatory by G1128, as well as common business nodes for VTS technical services like VTS centre, VTS operator, VTS system, and Marine. It is also recommended to add the following operational nodes in combination with the full content of the specification and apply them in relevant contexts such as Use Cases.

- Figure 3 Suggested Additional Operational Nodes

## Proposals on Improving Service Overview

As the Edition 1.5 that has been released for testing, compared with Service Specification for VTS - Vessel Route Exchange Edition 1.1 and the relevant requirements of G1128, the content of this section is relatively insufficient. Specific suggestions are as follows:

### Logical operations RELATED PROPOSALs

There are only three logical operations in section 4.1. It is suggested to delete the content of "As an example..." and add the following four logical operations:

- Discover service capabilities: The service must have a way to discover if the consumer supports payload encryption or requesting the Traffic Clearance

- System status: For service registries and service consumers to be able to check if a service is up, an interface must be defined that allows the checking of service without sending a valid message.

- Service registry updates: The service must be able to update its own metadata in the service registry according to the service registry definitions. This is required to maintain updated certificates in the service registry as well as to allow other automatic updates to service information.

- Search service registry: The service must have a way to search a maritime service registry for the consumer endpoint if it is registered.

### Logical Parameters RELATED PROPOSALs

The full text of section 4.2 Logical Parameters references to the relevant principles of G1128 and lacks substantive content. It is suggested to add the following three logical parameters:

1	Callback: For most operations initiated by the consumer a callback parameter is expected that defines how or where the service may respond to the consumer. This parameter is not required, but if it is not present, the consumer must be available in a service registry for discovery. The actual method of the callback depends on the service design.

2	Message identifier: In all operations where an acknowledgment is requested, a unique identifier of the message must be present that the acknowledgment can refer to. This is typically referred to as a transaction identifier, correlation id and so on.

3	Service discovery: The typical parameters used in service discovery are defined. It is important to note that searching for multiple versions of a service design that can be used is possible to ensure that the search returns all compatible versions and not just a single version. It is also important to note that from a service discovery point of view the version of the actual implementation should not have any significance.

## Proposals on Improving Service Data Model

- The enumeration values of "subjectOfCommunication" in "VTSDigitalInformationMessage", namely "9 Pilotage" "10 Port Visit" "11 Ready to Sail Operation", and the enumeration values of "messageStatus", namely "3 Confirmed" and "5 Cancelled", are not referenced in other relevant sections of this specification such as user instances and functional requirements. They also do not comply with the requirements of G1128 Version 1.7 in terms of how to use these enumeration values when implementing this service. Additionally, considering user cases and the context, it is suggested to add "3 passby" in "location Type".

## ProposalS on Improving Service Dynamic Behaviour

- In accordance with the relevant requirements of G1191 Maritime Service Registry (MSR) Technical Specification Edition 1.1, it is suggested to add a new dynamic behaviour of "service discovery"  and the following description:

- The method for service discovery using maritime service registries (MSRs) is described in more detail in G1128. To allow service discovery in the maritime domain, the service must be registered in a maritime service registry that participates in the global search. If the consumer allows automated requesting of route, it must be discoverable in a MSR that participates in global search. The consumer must support searching for compatible instances of the service in MSRs by using the geometry of the route and an applicable version of a service design based on this specification. The geometry of the route may be simplified if required. The service must support searching for consumers based on the vessel's MRN; or IMO or MMSI number.

## ProposalS on Improving References

- G1128 clearly presents relevant industrial standards and references in the form of a list rather than a table. In combination with the newly approved Service Specification for VTS - Vessel Route Exchange Edition 1.1 and G1191 Maritime Service Registry (MSR) Technical Specification Edition 1.1 by Council03 in December, 2025, it is suggested that the tables of Relevant Industrial Standards and References in 3.4.1 be changed to list, and G1128 be updated to version 1.7. New normative references to standards such as MSC.1.Circ.1610, Rev.1, IALA G1183, G1191, G1111, IEC 6373-2, and SECOM should be added.

# Action requested of the Committee

The Committee is requested to consider the proposals in this document and take actions as appropriate.