ialA Technical service

SSXXXX

Service Specification for VTS Under Keel Clearance Service

Edition 0.7

January 2026

urn:mrn:iala:techsvc:ssXXXX:0.1

| Date | Details | Approval |
| --- | --- | --- |
| September 2025 | Initial draft for input to VTS58 |  |
| October 2025 | Extending use cases |  |
| January 2026 | Extending use cases, BPMN |  |

1	introduction	5

1.1	Purpose	5

1.2	Intended readership	5

1.3	Input from other sources	5

2	Service Identification	6

3	Operational context	7

3.1	Use cases for VTS UKC Service	7

3.1.1	Use case 1 – Vessel requests UKC maximum allowed draught information from a VTS area (pre-plan)	9

3.1.2	Use case 2 – Vessel requests UKC tidal window information from a VTS area (pre-plan)	10

3.1.3	Use case 3 – Vessel subscribes to UKC information from a VTS (actual plan)	12

3.1.4	Use case 4 – Vessel requests UKC information from a VTS (actual plan)	13

3.1.5	Use case 5 – Vessel requests UKC information from a VTS (actual plan) after a route change	15

3.1.6	Use case 6 – Vessel provides detailed stability parameters to get a detailed UKC information from a VTS (actual plan)	17

3.1.7	Use case 7 – Vessel cancels subscription	19

3.1.8	Use case 8 – Sharing critical UKC planning information to VTS	20

3.1.9	Use case 9 – UKC cancels subscription automatically	21

3.2	Data flows	22

3.3	Functional and non-functional requirements	24

3.3.1	Functional requirements	24

3.3.2	Non-functional requirements	26

3.4	Other constraints	27

3.4.1	Relevant Industrial Standards	28

3.4.2	Operational nodes	29

4	Service Overview	30

4.1	Logical operations	30

4.1.1	Search Service Registry	31

4.1.2	Service registry updates	32

4.1.3	System status	32

4.1.4	Send and receive acknowledgment	32

4.1.5	Send route	32

4.1.6	Receive message	32

4.1.7	Subscribe to information	32

4.1.8	Discovering service functionality	32

4.2	Logical parameters	32

4.2.1	Callback	32

4.2.2	Geometry	33

4.2.3	Message identifier	33

4.2.4	Service discovery	33

5	Service Data Model	34

5.1	Container type	34

5.2	End-to-end verification of data	34

5.3	Message markers	34

5.4	Coordinate systems	35

6	Service Dynamic Behaviour	36

6.1	Acknowledgement messages	36

6.2	Service discovery	36

7	Definitions	38

8	Abbreviations	39

9	References	40

List of Tables

Table 1 Operational Nodes	29

Table 2 Logical interfaces	30

List of Figures

Figure 1 High-level view of VTS UKC information use cases	8

Figure 3 Pre-plan (UC1,2) VTS Information Business Process	22

Figure 2 Subscription (UC3) VTS Information Business Process	23

Figure 4 Actual Plan (UC 4 – 8) VTS Information Business Process	24

Figure 5 Abstract object model of the VTS information data	34

Figure 6 Service discovery flow	37

# introduction

This document was produced as part of the work of IALA VTS task group on development of technical service specifications for VTS. The document is structured according to the IALA Guideline G1128 The Specification of e-Navigation Technical Services [1].

## Purpose

The purpose of this Service Specification is to provide an overview of the information that VTS may provide or request from vessels via VTS Information in a technology-independent way, according to the guidelines given in G1128. It describes a well-defined baseline of the service by clearly identifying the service version.

The aim is to document the key aspects of the VTS UKC Service at the logical level, including:

the operational and business context of the service:

requirements for the service (e.g., information exchange requirements);

involved nodes: which operational components provide/consume the service;

operational activities supported by the service; and

relation of the service to other services.

the service description:

service use cases;

service operational sequence;

logical operations;

logical data model; and

dynamic behaviour.

## Intended readership

This Service Specification is intended to be read by service architects, system engineers and developers in charge of designing and developing an instance of the VTS Information Service.

Furthermore, this Service Specification is intended to be read by enterprise architects, service architects, information architects, system engineers and developers in pursuing architecting, designing and development activities of other related services.

## Input from other sources

N/A

# Service Identification

This section provides a unique identification of the Service and describes where the Service is in the engineering lifecycle.

| - Name | - VTS UKC Service |
| --- | --- |
| - ID | - urn:mrn:iala:techsvc:ss:vts:ukcs:0.7 |
| - Version | - 0.7 |
| - Description | - The VTS UKC Information Service specification describes a standardized service implementing the communication between ship and VTS used to request and deliver UKC information from vessel to VTS (and vice versa) in a standardized format. |
| - Keywords | - VTS, UKC Service, Ship Traffic Management, S-129, S-421 |
| - Architect(s) |  |
| - Status | - Draft |

# Operational context

WG1

The aim of a VTS UKC Service is to contribute to the safe and efficient navigation of vessels by delivering relevant UKC information from VTS perspective. The service can be implemented to deliver all the UKC information, or it can fulfil and/or enrich the data from other S-100 services. VTS UKC Service contains structured messages. The content of the message field is structured or unstructured. All messages must start with message markers information, request, question or answer. The message can be sent to one or multiple vessels.

VTS providers should define which UKC information will be provided via VTS UKC Service based on for example risk assessment.

The VTS UKC Service is intended to contribute to the safe and efficient navigation of vessels by delivering UKC information that VTS determines is relevant. The aim is to reduce the amount of broadcasted information via VHF to all vessels in the area and utilize digital tools to deliver timely and relevant UKC information to relevant parties.

In addition to delivering information, VTS can use the service to request information from vessels that is not covered in other reporting requirements.

There are no requirements that a VTS must use the UKC information service for any given use. All of the above are examples and a VTS may choose to not use VTS UKC Information Service in day-to-day operations.

WG1 to contribute more on operational context, could include text on e.g.

Information delivered,

What is a request

What is a question

What is an answer

## Use cases for VTS UKC Service

The use cases below describe an attempt of describing how and what types of uses there may be for the service. Not all the use cases are expected to be relevant to all VTSs and service implementations may ignore any of the use cases deemed unnecessary.

Consumer implementations must implement support for all of the use cases.

In all use cases the VTS system must be capable of receiving information from the service on the delivery status of the message to the vessel.

Figure 1 High-level view of VTS UKC information use cases

S-129 datasets are typically intended to be overlays to S-101 ENC and always displayed with S-101 ENC data in the background. Systems that support the display of S-129 datasets should provide the user with simple functions to turn the display of S-129 datasets on and off.

Optionally, S-129 datasets can be viewed as overlays to a combination of S-101 ENC and S-102 (Bathymetric Surface) datasets, with S-104 (Water Level Information for Surface Navigation) datasets incorporated where applicable. The same requirements to allow the user to easily toggle the S-129 dataset on/off persist.

### Use case 1 – Vessel requests UKC maximum allowed draught information from a VTS area (pre-plan)

Description – Vessel fetches pre-planning UKC information from VTS. In the pre-planning use case, a ship requests a maximum allowed draught for an arrival/departure port or waterway, typically days or weeks in advance. In this case, the VTS will compute a maximum allowed draught based on predicted tides, to forecast navigable depths, including safety/manoeuvrability margins, speed and squat predictions (default value), other forecast environmental conditions and the route provided by the vessel. In this scenario, the VTS UKC Service could return a single Pre-plan dataset, and generally no updates are required.

Typical sequence:

The vessel sends the route to the UKC service, with at least waypoints and a schedule. The vessel indicates the desired maximum allowed draught calculation result.

The UKC service calculates the pre-plan by using route, waterlevel (astronomical) and UKC waterway limits

- UKC service sends UKC pre-plan including:

  - UKC purpose: pre-plan

  - Under Keel Clearance Calculation Requested: Max allowed draught

  - Geometry for non-navigable areas [optional]

  - Geometry for almost non navigable areas, including distance above UKC limit [optional]

### Use case 2 – Vessel requests UKC tidal window information from a VTS area (pre-plan)

Description – Vessel fetches pre-planning UKC information from VTS. In the pre-planning use case, a ship requests a tidal window for an arrival/departure port or waterway, typically days or weeks in advance. In this case, the VTS will compute the tidal window based on predicted tides, expected vessel maximum draught, to forecast navigable depths, including safety/manoeuvrability margins, speed and squat predictions (default value), other forecast environmental conditions and the route provided by the vessel. In this scenario, the VTS UKC Service could return a single Pre-plan dataset, and generally no updates are required.

Typical sequence:

- The vessel sends the route to the UKC service, with at least the maximum draught, waypoints and a schedule. The vessel indicates the desired global time window calculation result.

- Vessel requests tidal window for the acknowledged route

- VTS sends UKC pre-plan including:

  - UKC purpose: pre-plan

  - UKC time window with fixed time ranges start and end global time windows

  - Geometry for non-navigable areas [optional]

  - Geometry for almost non navigable areas, including distance above UKC limit [optional]

### Use case 3 – Vessel subscribes to UKC information from a VTS (actual plan)

Description – Actual Plan datasets are produced closer to the arrival/departure (for example, 48-24 hours prior) and provide the mariner (crew and/or pilot) with a more detailed passage plan. This plan is generated from more frequent and/or precise weather forecasts/observations. Closer to the time of a ship entering the VTC Operational Area, the ship will need a more detailed UKC plan. This Actual Plan usually considers more up to date information and will typically need to be updated more frequently. In this case, the non-navigable and almost non-navigable areas, any tidal windows (via Control Points), and some metadata will have changed. Depending on the variability of the met-ocean conditions, the update frequency could vary, for example between 10 and 60 minutes.

Typical sequence:

- Vessel queries the VTS Service registry

- Vessel subscribes to the VTS UKC Service

- Vessel provides an update link to exchange UKC Service details

### Use case 4 – Vessel requests UKC information from a VTS (actual plan)

Description – Actual Plan datasets are produced closer to the arrival/departure (for example, 48-24 hours prior) and provide the mariner (crew and/or pilot) with a more detailed passage plan. This plan is generated from more frequent and/or precise weather forecasts/observations. Closer to the time of a ship entering the VTC Operational Area, the ship will need a more detailed UKC plan. This Actual Plan usually considers more up to date information and will typically need to be updated more frequently. In this case, the non-navigable and almost non-navigable areas, any tidal windows (via Control Points), and some metadata will have changed. Depending on the variability of the met-ocean conditions, the update frequency could vary, for example between 10 and 60 minutes.

Pre-conditions:

- Vessel has subscribed to the VTS UKC Information Service and the vessel is able to display geometries from received messages as a layer on an ENC.

- The route is shared with the route exchange service (including schedule and draught) is known upfront by the VTS.

Typical sequence:

- VTS uses route, draught information, waterlevel and current forecast, UKC waterway limits to calculate an actual UKC plan

- UKC service sends actual UKC Plan to the vessel including:

  - UKC purpose: actual plan

  - Under Keel Clearance Calculation Requested: Time Window

  - vessel max draught used in calculation

  - UKC time window with fixed time ranges start and end time tidal windows

  - Geometry for non-navigable areas [optional]

  - Geometry for almost non navigable areas, including distance above UKC limit [optional]

  - Control points details with

    - Distance above UKC limit

    - Expected passing speed

    - Expected passing time

    - Fixed time ranges start and end time local tidal window

### Use case 5 – Vessel requests UKC information from a VTS (actual plan) after a route change

Description – a vessel has subscribed to the UKC service and changes its route (geometry and/or timing) and requests an update on the UKC plan.

Pre-conditions:

- Vessel has subscribed to the VTS UKC Information Service

- The UKC plan has been shared to the vessel for a previous route

- The changed route has been shared between ship and VTS by using e.g. the Route Exchange Service

Typical sequence:

- VTS uses route, draught information, waterlevel and current forecast, UKC waterway limits to calculate an update of the actual UKC plan

- UKC service updates the UKC plan to the vessel(s)

  - Each update must refer to the original message that is being updated

UKC purpose: actual plan

  - Under Keel Clearance Calculation Requested: Time Window

  - vessel max draught used in calculation

  - UKC time window with fixed time ranges start and end time tidal windows

  - Geometry for non-navigable areas [optional]

  - Geometry for almost non navigable areas, including distance above UKC limit [optional]

  - Control points details with

    - Distance above UKC limit

    - Expected passing speed

    - Expected passing time

    - Fixed time ranges start and end time local tidal window

### Use case 6 – Vessel provides detailed stability parameters to get a detailed UKC information from a VTS (actual plan)

Description – Actual Plan datasets are produced closer to the arrival/departure (for example, 48-24 hours prior) and provide the mariner (crew and/or pilot) with a more detailed passage plan. This plan is generated from more frequent and/or precise weather forecasts/observations. Closer to the time of a ship entering the VTC Operational Area, the ship will need a more detailed UKC plan. This Actual Plan usually considers more up to date information and will typically need to be updated more frequently. In this case, the non-navigable and almost non-navigable areas, any tidal windows (via Control Points), and some metadata will have changed. Depending on the variability of the met-ocean conditions, the update frequency could vary, for example between 10 and 60 minutes.

Pre-conditions:

- Vessel has subscribed to the VTS UKC Information Service and the vessel is able to display geometries from received messages as a layer on an ENC.

- The route is shared with the route exchange service (including schedule and max draught) is known upfront by the VTS.

Typical sequence:

- Vessel shares or updates ship details and stability parameters:

  - Length L.O.A. (lengthOverall)

  - Breadth width (breadth)

  - Front, mid, end vessel draught [optional] (draftOrDraught)

  - Deadweight [optional] (deadweightTonnage)

  - Metacentric Height (GM) [optional] (metacentreOrMetacenter)

  - Waterdisplacement [optional] (displacementTonnageLoaded)

  - Free fluid correction (GG’) [optional]

  - Ship roll period [optional]

  - Centre of Buoyancy, KB [optional] (centreOfBuoyancy)

  - Centre of Gravity, KG [optional] (centreOfGravity)

  - Block Coefficent, Cb [optional]

- VTS uses route, draught information, waterlevel and current forecast, UKC waterway limits and available stability parameters to calculate an actual UKC plan

- UKC service sends actual UKC Plan to the vessel including:

  - UKC purpose: actual plan

  - Under Keel Clearance Calculation Requested: Time Window

  - vessel max draught used in calculation

  - UKC time window with fixed time ranges start and end time tidal windows

  - Geometry for non-navigable areas [optional]

  - Geometry for almost non navigable areas, including distance above UKC limit [optional]

  - Control points details with

    - Distance above UKC limit

    - Expected passing speed

    - Expected passing time

    - Fixed time ranges start and end time local tidal window

### Use case 7 – Vessel cancels subscription

Description – a Vessel has subscribed to the UKC service and wants to end this subscription. This is not implying that the route to the VTS area is cancelled.

Pre-conditions:

- Vessel has subscribed to the VTS UKC Information Service

Typical sequence:

- Vessel sends subscription cancellation to the service

- UKC service is no longer required to publish the UKC planning

### Use case 8 – Sharing critical UKC planning information to VTS

Description – The UKC service has made an actual UKC plan for which the UKC clearance is not meeting the fairway design draughts.

Pre-conditions:

- The route is shared with the route exchange service (including schedule and draught) is known upfront by the VTS.

- An actual UKC plan has been calculated

Typical sequence:

- UKC service detects the UKC plan is not meeting the fairway design draughts

- An alert is sent to the VTS operator to notify UKC plan risk

### Use case 9 – UKC cancels subscription automatically

Description – The UKC service cancels subscription automatically after certain time to avoid sending outdated information to inactive vessel.

Pre-conditions:

- Vessel subscribes to UKC service.

Typical sequence:

- UKC service cancels subscription after certain time

- UKC service is no longer required to publish the UKC planning

## Data flows

Figure 2 gives an overview of the dataflows for the VTS Information Service as described in the use cases.

Figure 2 Pre-plan (UC1-2) VTS Information Business Process

Figure 3 Subscription (UC3,7,9) VTS Information Business Process

Figure 4 Actual Plan (UC 4 – 8) VTS Information Business Process

## Functional and non-functional requirements

### Functional requirements

| - Requirement ID | - RESF001 |
| --- | --- |
| - Requirement Name | - Receive basic route from vessel |
| - Requirement Text | - A vessel must be able to send basic route information to the service. This consists of at least one waypoint and an estimated time of arrival at that waypoint. |
| - Rationale | - The basic route information is required to be able to calculated any UKC information. |
| - Author |  |

| - Requirement ID | - RESF002 |
| --- | --- |
| - Requirement Name | - Calculate UKC pre-planning (max draught) |
| - Requirement Text | - The UKC Service of the VTS must facilitate the calculation of an UKC pre-planning with max allowed draught. |
| - Rationale | - The UKC pre-planning information with max allowed draught information must be shared to the vessel to allow the vessel to know the limitations applicable within the VTS area at the expected time of arrival. |
| - Author |  |

| - Requirement ID | - RESF003 |
| --- | --- |
| - Requirement Name | - Calculate UKC pre-planning (global tidal window) |
| - Requirement Text | - The UKC Service of the VTS must facilitate the calculation of an UKC pre-planning with UKC global time tidal window to cross the VTS area and reach the destination safely. |
| - Rationale | - The UKC pre-planning information with global time tidal windows information must be shared to the vessel to allow the vessel to know the planning limitations applicable within the VTS area at the expected time of arrival. |
| - Author |  |

| - Requirement ID | - RESF004 |
| --- | --- |
| - Requirement Name | - Calculate UKC actual planning |
| - Requirement Text | - The UKC Service of the VTS System must facilitate the calculation of an UKC actual planning with UKC global time tidal window to cross the VTS area and reach the destination safely. |
| - Rationale | - The UKC actual planning information with global time tidal windows, safety areas and information must be calculated and shared to the vessel to allow the vessel to know the planning limitations applicable within the VTS area at the expected time of arrival. |
| - Author |  |

| - Requirement ID | - RESF005 |
| --- | --- |
| - Requirement Name | - Support route exchange service |
| - Requirement Text | - Valid detailed route exchange support is required for actual plan UKC Service exchanges. |
| - Rationale | - For actual plan UKC Service calculations, a valid route must be exchanged. This is covered in the route exchange service. |
| - Author |  |

| - Requirement ID | - RESF006 |
| --- | --- |
| - Requirement Name | - Service must be able to send updated actual plan to the vessel automatically. |
| - Requirement Text | - The service must be able to send updated actual plan to the vessel when it changes. - This means that the ship system can get the latest actual plan when the route or environment condition have changed. |
| - Rationale | - Due to complexity, this is optional to support on the vessel side, but support in the service is required so that vessels that do support this functionality and can receive better service. |
| - Author |  |

| - Requirement ID | - RESF007 |
| --- | --- |
| - Requirement Name | - UKC Service details must be published in service registry for service discovery. |
| - Requirement Text | - The availability of the UKC Service must be published in a service registry for service discovery. Information of the service details availability (pre-plan / actual plan) must be described in the service registry. |
| - Rationale | - The vessel should be able to discover in a service registry the level of UKC Service provided. A limited UKC Service (e.g. only pre-plan) or a more extended UKC Service with an actual plan or detailed actual plan could be provided. |
| - Author |  |

| - Requirement ID | - RESF008 |
| --- | --- |
| - Requirement Name | - Requirements for a valid route received from vessel |
| - Requirement Text | - The following information must be present in the route shared by the vessel: All elements required in the schema must be present Vessel must be identifiable, see RESF009 Vessel voyage must be identified All waypoints must have at least one associated schedule element Waypoint schedule elements must not be of type “recommended” All waypoints except the first and last waypoint must have a non-zero turn radius or have both ETA and ETD defined and ETD must be later than ETA. All action points must have either a time to act or distance to act All legs must be defined. First waypoint must not have a leg. All legs must have both port and starboard CL defined All legs must have routeWaypointLegDraft defined All waypointIDs must be unique in the route. A route should not have more than 1000 waypoints or action points All dates and times must have an accompanying time zone. Time zone should be UTC. Character encoding of shared route must be UTF-8. routeInfoVesselHeight and RouteInfoVesselAirDraft must be set |
| - Rationale | - Additional business rules are required in addition to schema validation to validate a route shared by the vessel for checking purposes. These requirements fulfil the requirements from the VTS System side on what is required to be able to check and verify the route shared from the vessel. - This requirement also takes into account the business rules defined in S-421 [5] and the minimum requirements placed on ECDIS devices |
| - Author |  |

| - Requirement ID | - RESF009 |
| --- | --- |
| - Requirement Name | - Requirements for identities |
| - Requirement Text | - The service must have a valid maritime identity. See Guideline G1183 [3] - A consumer must have a valid maritime identity. - The maritime identity certificate must include the vessel’s MMSI and IMO number and they must match with the MMSI and IMO number in the route. If vessel does not have an IMO-number then only MMSI is required. If the certificate information and the identifying information in the route do not match an error must be returned and no further processing of the route is allowed. |
| - Rationale | - The consumer must be able to verify the identity of the service and its provider. The consumer must reject any connections from a service that does not have a valid maritime identity. - The service must require verification of the identity of the consumer and must decline service if a valid identity is not provided or if the identity provided does not match the metadata of the route. |
| - Author |  |

| - Requirement ID | - RESF010 |
| --- | --- |
| - Requirement Name | - Requirements for authenticity |
| - Requirement Text | - The consumer must be able to verify the authenticity of the route from the originator of the route (VTS system or this service) to the actual consumer application (e.g. ECDIS). This requires that if the communication between the service and the end-user application is not direct, the signature of the route is also delivered to the consumer application hand in hand with the route. - The service must be able to verify the authenticity of the route from the actual vessel it identifies. Thus, the route must be signed with the signature of the same vessel it is from and the signature must be delivered to the service. |
| - Rationale | - ECDIS is mandated to verify the authenticity of all incoming external data. Thus the signature of a received route must be delivered to the ECDIS hand in hand with the route. See IACS UR E27 section 4.2.6 [7] - VTS systems and other end users of shared routes must also be able to verify that the route they receive actually originates from the ship identified so that spoofing or any other kinds of bad actions from third parties can be prevented. |
| - Author |  |

| - Requirement ID | - RESF010 |
| --- | --- |
| - Requirement Name | - Requirements for sharing a pre-plan to the vessel |
| - Requirement Text | - The pre-plan information shared by the UKC Service to the vessel should be a valid to S-129 message specifications, with attribute underKealClearancePurpose=1 (prePlan). The attribute underKeelClearanceCalculationRequested should be 1 (timeWindow) or 2 (maxDraught) |
| - Rationale |  |
| - Author |  |

| - Requirement ID | - RESF010 |
| --- | --- |
| - Requirement Name | - Requirements for sharing an actual plan to the vessel |
| - Requirement Text | - The actual plan information shared by the UKC Service to the vessel should be a valid to S-129 message specifications, with attribute underKealClearancePurpose=2 (actualPlan) or 2 (actualUpdate). |
| - Rationale |  |
| - Author |  |

### Non-functional requirements

| - Requirement ID | - TCSNF001 |
| --- | --- |
| - Requirement Name | - Integrity |
| - Requirement Text | - It must be clear to both service provider and consumer whether changes have been made to the information after the dataset was created. All messages must be signed with the correct certificates so that the contents of a message can be validated. The technical designs must describe how this is managed. |
| - Rationale |  |
| - Author |  |

| - Requirement ID | - TCSNF002 |
| --- | --- |
| - Requirement Name | - Availability |
| - Requirement Text | - The actual SLA of the service must be defined by instance owner. The service should be highly available and be considered a critical component of the VTS system. |
| - Rationale | - In IALA G1111 Section 3.1 [8] the availability requirements for VTS systems are defined. As the service is required for successful digital exchange of VTS information and such exchange may occur at any time of the day the service should be available whenever VTS system is available. |
| - Author |  |

| - Requirement ID | - TCSNF003 |
| --- | --- |
| - Requirement Name | - Performance – timeliness |
| - Requirement Text | - The service must provide a technical response to an incoming request instantly. This response is by necessity a technical delivery acknowledgement and not a business process response. This applies both to requests coming from vessels and VTS System. The technical designs must describe how this is managed. |
| - Rationale | - Especially from a vessel’s point of view it is important to get an acknowledgement that the service has received a request so that the vessel’s system does not need to try resending the request. |
| - Author |  |

| - Requirement ID | - TCSNF004 |
| --- | --- |
| - Requirement Name | - Uniqueness |
| - Requirement Text | - All requirements for uniqueness during route exchange are guaranteed to exist only during the transmit of a route. |
| - Rationale | - On either side of the exchange changes to non-technical identifiers or names in the route are possible and may conflict with each other. Relying on user editable names or identifiers in communication must be done carefully by ensuring that the same element is being discussed. |
| - Author |  |

| - Requirement ID | - TCSNF004 |
| --- | --- |
| - Requirement Name | - Throttling – rate limitations |
| - Requirement Text | - The UKC Service should be able to limit the number requests |
| - Rationale | - Depending on the capacity of the UKC Service and VTS System, the UKC information requests should be throttled. Many actual plan requests can trigger a high load on the UKC Service. The maximum parallel capacity should be defined and used to limit the load. |
| - Author |  |

## Other constraints

### Relevant Industrial Standards

- IALA Guideline G1128 1.6 The Specification of E-navigation Technical Services

IALA Guideline G1143 3.1 Unique identifiers for maritime resources (MRN)

IHO Standard S-100 5.2.0 IHO Universal Hydrographic Data Model https://iho.int/uploads/user/pubs/standards/s-100/S-100_5.2.0_Final_Clean.pdf

IALA Guideline G1183 1.1 Provision of MCP Identities

IEC 63173-2 Committee Draft 80/1149/CD Secure communication between ship and shore (SECOM)

IEC 63173-1 Committee Draft 80/1148/CD S-421 Route Plan based on S-100

IACS UR E26/27 Rev1 International Association of Classification Societies https://iacs.org.uk/resolutions/unified-requirements/ur-e

IALA Guideline G1111 2.0 Establishing Functional Performance Requirements

### Operational nodes

Table 1 Operational Nodes

| - Operational node | - Remarks |
| --- | --- |
| - Route plan or route | - Route plan is the common definition of what is being shared. For brevity, route is used as a synonym for route plan throughout this specification. |
| - Vessel | Participating ship that is required to participate with vessel traffic services and is sailing or expected to sail in a VTS area where there is coverage of technical service. |
| - Mariner | - In this document mariner means a person who is part of the bridge team that is responsible for the navigation of the vessel. |
| - VTS | - Vessel traffic services (VTS) means services implemented by a Government with the capability to interact with vessel traffic and respond to developing situations within a VTS area to improve safety and efficiency of navigation, contribute to the safety of life at sea and support the protection of the environment. |
| - VTS centre | - VTS centre responsible for a one or several VTS Areas for which the VTS provider is authorized to deliver vessel traffic services. A VTS centre is responsible for VTS information service within its coverage area. |
| - VTS operator | - The personnel of the VTS centre, means persons performing tasks associated with vessel traffic services, trained in vessel traffic services operations and appropriately qualified. |
| - VTS system | - The VTS system is the VTS software, hardware, communications and sensors. This excludes personnel and procedures. |
| - Service | - Implementation of the VTS UKC Service. This is used to differentiate the service from a server as there may be multiple servers capable of accepting incoming requests / data. |
| - Registry | - The service registry of the VTS system to allow service discovery on the availability and level of services provided by the VTS system. |
| - Consumer | - The vessel system interacting with the service. There may be a proxy server between the consumer and the service to facilitate data exchange but the consumer can always be understood as the on-board system(s) used to interact with the service. |

# Service Overview

## Logical operations

The following logical operations must be provided in the designs that follow this specification ((x = required, o = optional):

Table 2 Logical interfaces

| Operation | Description | Required |  |
| --- | --- | --- | --- |
|  |  | [Node X] | [Node Y] |
| - Send information | - An operation to send a message. Message may be an pre-plan with max draught information, pre-plan with global tidal windows or actual plan. | x | x |
| - Receive information | - An operation that allows the receiving of messages. When message is received and delivered to the end system (on-board or VTS) an acknowledgement must be sent. | x | x |
| - Send acknowledgement | - An operation that allows sending of an acknowledgement that VTS information has been delivered to end system or opened by end user. | x | x |
| - Receive acknowledgement | - An operation that allows the reception of an acknowledgement. | x | x |
| - Subscribe to UKC actual plan | - VTS System must have a way to inform the vessel of changes in the actual UKC plan. - The consumer must have a way to subscribe to the results of long running processes or processes that require human interaction and thus an asynchronous response. | x | x |
| - Push UKC information | - The consumer must support the service pushing UKC information when the consumer subscribed to the UKC Service. | x | o |
| - Discover service capabilities | - The consumer must have way to discover if the UKC service supports sharing of actual plan information. | x |  |
| - System status | - For service registries and service consumers to be able to check if service is up an interface must be defined that allows the checking of service without sending a valid message. | x | x |
| - Service registry updates | - The service must be able to update its own metadata in the service registry according to the service registry definitions. This is required to maintain updated certificates in the service registry as well as to allow other automatic updates to service information. | x | x |
| - Search service registry | - The consumer must have a way to search a maritime service registry for VTS information services that it is compatible with its area of interest. - The service must have a way to search a maritime service registry for the consumer endpoint if it is registered. | x | X |
| - All operations defined in the route exchange specifications | - A route exchange service must be supported for the UKC service to calculate actual plan UKC. | x | x |

In this table the requirement that a vessel has an interface means that the vessel must have access to an interface that ensures that the incoming data is transported to the vessel’s on-board systems. It does not mean that the vessel must have connectivity all the time or that the vessel must accept random incoming connections. However, the vessel systems must have a way to send data to the service and get the technical sign-off of a successful transmit in a synchronous call.

What this means is that the end-to-end acknowledgment that is described in use cases and sequence diagrams differs from the technical acknowledgment that signals a successful end of transmission. This service and others that work between shore and vessel are often components of a more complex system. As such, the technical ok of a single transmission being complete does not indicate that a message has been delivered end to end. Thus, a separate mechanism for acknowledging delivery of end-to-end transmission is required. This is done by adding a method for acknowledging when the VTS information has been delivered to the end system and opened by a recipient and visualizing this acknowledgement in the sending systems to facilitate e.g. operational acknowledgment.

### Search Service Registry

Service discovery is a required functionality of both the service and its consumers.

The consumers must be able to search for valid route exchange services that the consumer supports and cover its area of interest. For this functionality to work, the following parameters are expected:

The technical service design MRN and version number that the service implements that the consumer supports.

Geometry of the area of interest. See 4.2.1

If the service has not received a callback parameter, the service must be able to search the service registry for the consumer to discover valid endpoints for the consumer. The following parameters are expected in this case:

The technical service design MRN and version number that the service implements.

The MRN of the consumer.

The actual method and logic of service discovery is outlined in IEC 63173-2.

### Service registry updates

The service should be able to update the supported versions of the technical service design, its certificates and endpoint automatically to the service registry. The service must also define the geometry of the area that the service covers to allow for service discovery.

If registered in a service registry, the consumer should also be able to update the supported versions of the technical design, its certificates and endpoint automatically to the service registry.

The actual method and logic of service discovery is outlined in IEC 63173-2.

### System status

Both the service and consumer endpoints must support a way of checking that the endpoint is available via e.g. a ping interface without sending a valid message.

### Send and receive acknowledgment

Both the service and consumer must have a method of sending and receiving acknowledgments of delivery to end systems. As many of the operations are asynchronous and may depend on multiple intermediate systems there is a need to be able to show to the end users of the systems the status of message delivery that exceeds the technical status of the initial message transfer. This interface must require the presence of the message identifier (see 4.2.3).

### Send basic route

… using S-212

### Send route

The consumer must have a method of sending the route. When sending the route from the vessel it must conform to the requirement RESF007.

### Receive pre-plan request message

### Receive actual-plan request message

The service must have a method of receiving the route.

The vessel may require that any routes it receives are related to an earlier shared route unless the route is returned as a result of a search to a VTS route library. In this case the message identifier of the message in which the route was initially shared by the vessel is passed as a parameter. The vessel must reject all incoming routes that do not come from a service with a valid maritime identity.

The service must be able to accept any route sent by a vessel with a valid maritime identity.

When receiving a route all parties must validate the route according to the S-421 XML Schema and according to the requirements RESF007 and RESF008 as applicable.

A longer route may be larger than what the transfer mechanism allows depending on the service design. If so, the service design must define interfaces that allow the sharing of arbitrarily large messages to ensure that sharing of routes is not limited by transfer protocol limitations and routes of at least 1000 waypoints, 999 legs, and 1000 action points with all associated additional attributes can be shared.

### Subscribe to information

When the UKC Service generates regular updates on the actual plan (eg when route changed, environmental changes detecte), the new actual plan needs to be shared with the vessel. Thus, this asynchronous interaction requires and interface to allow subscriptions to share updated planning details.

### Discovering service functionality

The consumer must have the ability to discover all of the interfaces provided by the service. The service must provide an interface that supports the discovery of available functionality.

## Logical parameters

Here we will cover the abstract logical parameters that are common for multiple interfaces. Actual parameter structures, response structures or error handling is not specified. These will be defined in more detail in the technical design documents.

### Callback

For most operations initiated by the consumer a callback parameter is expected that defines how or where the service may respond to the consumer. This parameter is not required, but if it is not present, the consumer must be available in a service registry for discovery (see 4.1.1). The actual method of the callback depends on the service design.

### Geometry

When sharing geometries as a search parameter the geometry may either be an actual representation of the route with all waypoints and legs represented or a simplified representation of the geometry that can be any of the following (non-inclusive)

List of waypoint coordinates

A simplified polygon of the route

A simplified version of the route

Any padding that is needed for the geometry when creating the geometry (e.g. for searches of services along a route) must be added by the creator of the query. Recipients can use the incoming geometry as is.

### Message identifier

In all operations where an acknowledgment is requested, a unique identifier of the message must be present that the acknowledgment can refer to. This is typically referred to as a transaction identifier, correlation id and so on.

### Service discovery

The typical parameters used in service discovery are defined in 4.1.1. It is important to note that searching for multiple versions of a service design that can be used is possible to ensure that the search returns all compatible versions and not just a single version.

It is also important to note that from a service discovery point of view the version of the actual implementation should not have any significance.

# Service Data Model

The basis of the data model used in this service is S-212 and S-129. In addition to the validations in the schema and the business rules defined in S-212 and S-129 the following requirements are placed on conforming services.

S-212, including route details via S-421

S-129

Figure 5 S-129 Abstract object model of the UKC Service planning

## Container type

The service is expected to create the S-129 message. The consumer will require capabilities to exchange S-212 messages and the route exchanges (S-421).

While S-100, S-129 and S-212 support transmitting a route in an S-100 exchange set or dataset; or a plain S-129 or S-212 message without any additional metadata, for the use of this service both approaches will be supported.

The rationale for this is that there might be a need to create a globally unique filename and/or path combination for the message that is required in both the S-100 exchange set and dataset metadata. The UKC information being shared in this service cannot always be considered ephemeral and the S-129 and S-212 formats of the UKC information may need to be kept in storage for either the consumer or service.

## End-to-end verification of data

Because the VTS information shared in conforming applications may have the full set of metadata and external structures of an S-100 exchange set, the signature of the message required in RESF010 must accompany the message in other means. In normal direct communication between consumer and service this is normally handled by communication headers and may be provided by the protocol. The service design must specify how the signature is transmitted.

## Coordinate systems

The coordinate system used must be defined. It should be EPSG:4326 according to S-100 and the coordinate order when using EPSG:4326 must always be latitude longitude. Service and consumer are not required to ensure correct handling of other coordinate systems or coordinate orders.

It is adequate to define the used coordinate system for the bounding box of the message. All subsequent geometries must default to the coordinate system defined for the root bounding box unless another coordinate system is explicitly defined for that geometry.

# Service Dynamic Behaviour

In all of the following diagrams ship systems include the ECDIS and any other systems on-board that mariners can use to plan and change routes. It is a requirement that at least the ECDIS [11] supports all of the functionality described in this specification, other on-board systems may communicate with route exchange service directly or via ECDIS.

Every time the service receives a route it must check that the route is valid according to schema. If route does not pass schema validation, then the appropriate error message must be returned depending on the service design.

The use cases described earlier in this document are looked at from a more technical perspective and the actual data and flow of information from one component to another are described here.

While service discovery may be needed, it is left last in the description of dynamic behaviour as it is already described in more detail in G1128 and may not be a required part of each use case.

In each case we will assume that a route is a valid route according to the XML schema. The handling of routes that are valid according to schema but do not fulfil the business rules of the use case will be described here.

While the service may often be integrated into a VTS system, from the perspective of this specification there is a boundary between the VTS system and the service as the service may also deliver and interact with other backend systems.

Due to the nature of vessel connectivity, we do not assume that all connections between the service and on-board consumers are direct. There may be shore based intermediate components involved as well depending on the actual service design. These components are not of interest from the perspective of this specification but the addition of multiple components in a single message transmission (multi-hop) is taken into account and adds complexity to the specification and implementations.

## Acknowledgement messages

The sequence diagrams shown have multiple references to acknowledgments being sent of delivery to end systems (ship or VTS system) or opening of the message. This is due to the asynchronous and multi-hop nature of the communication where the technical transmission of a message from one component to another does not always signify the successful transmission of the message from one end system to another.

Thus, the sequence diagrams show and prescribe when acknowledgments must be sent either on the successful delivery of the information to the end system or of the opening of the information by a user. If the received information is automatically displayed on VTS or ship systems, the opened acknowledgment can only be sent after the user has in some way interacted with the information.

## PrePlanning

The basic use of the UKC Service is initiated by the vessel sharing basic route details.

This consists of at least a destination waypoint and an estimated time of arrival.

With this (basic) route details, the UKC Service can provide a high level UKC pre-plan to provide the vessel information of the maximum allowed draught indication (use case 1) or the estimated tidal window (use case 2). This information will be provided using the S-129 format.

Figure 6 Use case 1 sequence diagram (pre-plan max draught)

Figure 7 Use case 2 sequence diagram (pre-plan tidal window)

## Service discovery - Subscription

The method for service discovery using maritime service registries (MSRs) is described in more detail in G1128.

To allow service discovery in the maritime domain, the service must be registered in a maritime service registry that participates in global search.

Figure 8 Service discovery flow

## Actual Plan

When the vessel has provided the full route to the VTS System (using Route Exchange Service), a more detailed actual plan can be calculated by the UKC Service. This actual plan will provide a more accurate tidal window and can indicate area’s which are safe to navigate and area’s which are not safe to navigate. This information will be provided using the S-129 format.

Figure 9 Use case 4 Sequence diagram (actual plan)

Whenever the route information changes, or the VTS area environmental conditions (e.g. weather information, hydrometric information, bathymetric information) change, the UKC actual plan may need to be updated. On regular basis, the UKC Service will need to validate the conditions and criteria of the voyage and when needed, the actual plan will need to be actualized. The vessel subscription will allow the vessel to retrieve any update of the actual plan to make sure the latest planning information is available to the vessel.

Figure 10 Use case 5 Sequence diagram (actual plan)

## Detailed actual plan

For some VTS areas, it might be useful to generate more detailed UKC planning. To model such detailed UKC planning, extra vessel stability information might be required. Using the UnderKeelClearanceMessage of S-212, such extra vessel stability information can be provided to the UKC Service. Based on this information, the same actual plan information in S-129 will be provided to the vessel, yet the planning provided will be more accurate.

Figure 11 Use case 6 Sequence diagram (detailed actual plan)

## Ending UKC Planning

When the vessel no longer needs the UKC planning, or when the voyage has ended, the UKC planning for the vessel will no longer be needed. The vessel can cancel the subscription with the UKC Service or the UKC Service can cancel the subscription based on the voyage status. This will allow the UKC Service to limit the number of UKC plannings to calculate, since, depending on the level of details and modelling, this might be a high load calculation for the UKC Service.

Figure 12 Use case 7 sequence diagram cancellation by vessel

## Critical UKC planning for VTS

Based on the UKC Planning generated by the UKC Service, notifications within the VTS System may be desired. If an issue with an UKC Planning is detected (e.g. high bottom touch probability), an alert / notification of this risk may be notified to the VTS System and VTS Operator.

Figure 13 Use case 8 sequence diagram VTS alert

Ending UKC Planning by VTS

Use case 9

# Definitions

| Operational Node | A logical entity that performs activities. Note: nodes are specified independently of any physical realisation. Examples of operational nodes in the maritime context are: Maritime Control Centre, Maritime Authority, Ship, Port, Weather Information Provider, etc. |
| --- | --- |
| Service | The provision of something (a non-physical object), by one, for the use of one or more others, regulated by formal definitions and mutual agreements. Services involve interactions between providers and consumers, which may be performed in a digital form (data exchanges) or through voice communication or written processes and procedures. |
| Service Consumer | A service consumer uses service instances provided by service providers. All users within the maritime domain can be service customers, e.g., ships and their crew, authorities, VTS centres, organizations (e.g., meteorological), commercial service providers, etc. |
| Service Data Model | Formal description of one dedicated service at logical level. The service data model is part of the Service Specification. Is typically defined in UML and/or XSD. If an external data model exists (e.g., a standard data model), then the service data model shall refer to it: each data item of the service data model shall be mapped to a data item defined in the external data model. |
| Service Interface | The communication mechanism of the service, i.e., interaction mechanism between service provider and service consumer. A service interface is characterised by a message exchange pattern and consists of service operations that are either allocated to the provider or the consumer of the service. |
| Service Operation | Functions or procedure which enables programmatic communication with a service via a service interface. |
| Service Provider | A service provider provides instances of services according to a Service Specification and service instance description. All users within the maritime domain can be service providers, e.g., authorities, VTS centres, organizations (e.g., meteorological), commercial service providers, etc. |

# Abbreviations

API	Application Programming Interface

MRN	Maritime Resource Name

MSR	Maritime Service Registry

VTSO	VTS Operator

XML	Extendible Mark-up Language

XSD	XML Schema Definition

# References

IALA. 2024. Guideline G1128 The Specification of e-Navigation Technical Services

IALA. 2022. Guideline G1143 Unique Identifiers for Maritime Resources (MRN)

IALA. 2022. Guideline G1183 Provision of MCP Identities

IHO. 2024. Universal Hydrographic Data Model Standard S-100 5.2.0

IEC. 2021. 63173-1 S-421 Route Plan based on S-100 Committee Draft 80/1148/CD

IEC. 2021. 63173-2 Secure communication between ship and shore (SECOM) Committee Draft 80/1149/CD

IACS. 2024. UR E26 & E27 Rev1 International Association of Classification Societies https://iacs.org.uk/resolutions/unified-requirements/ur-e

IALA. 2022. Guideline G1111 Establishing Functional Performance Requirements

IEC. 2024. 61174-1 Electronic chart display and information system (ECDIS) – Operational and performance requirements, methods of testing and required test results IEC TC80 MT7 December 2024 meeting outcome

IMO. 2024. MSC.530(106)REV.1 Performance Standards For Electronic Chart Display And Information Systems (ECDIS) https://wwwcdn.imo.org/localresources/en/KnowledgeCentre/IndexofIMOResolutions/MSCResolutions/MSC.530(106).pdf

IMO 2024. MSC.1CIRC.1610REV.1 INITIAL DESCRIPTIONS OF MARITIME SERVICES IN THE CONTEXT OF E-NAVIGATION https://wwwcdn.imo.org/localresources/en/OurWork/Safety/Documents/enavigation/MSC.1-CIRC.1610%20- %20Initial%20Descriptions%20Of%20Maritime%20ServicesIn%20The%20Context%20Of%20E-Navigation%20(Secretariat)%20(1).pdf

Author to insert actual service name and version number