Input paper:   VTS59-9.1.5

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM	□  ENG		□  PAP					X  Input

□  ENAV	X  VTS							□  Information

Agenda item 			9.1

Technical Domain / Task Number 2	2.5.2.a

Author(s) / Submitter(s)			Fintraffic VTS

# Key Findings and Feedback from the Digital VTS Traffic Clearance Service Piloting

# Summary

Fintraffic VTS has been conducting simulator and onboard piloting of the digital Traffic Clearance Service, developed based on the IALA Service Specification for digital VTS Traffic Clearance. The piloting included both technical validation and the collection of end-user experiences and was carried out within the Interreg Baltic Sea Region co-funded MaDaMe project.

During the 58th session of the VTS Committee, the initial findings from the pilots, focusing on end-user feedback, were presented to WG2 – Technology, and an expert review was conducted. Based on the pilot findings and the expert review discussions, Fintraffic VTS has summarised key findings related to the digital TCS service specification, as well as feedback on the implementation of future digital VTS services, which may be relevant to other IALA working groups and external stakeholders.

## Purpose of the document

This document is intended to support the committee’s evaluation and decision on whether the findings from Fintraffic’s digital TCS service pilots indicate a need to amend the Service Specification for digital VTS Traffic Clearance.

Furthermore, the document aims to bring to the committee’s attention several topics arising from end-user feedback related to the implementation of digital VTS services. This information may be of value not only to the Technology Working Group, but also to other working groups within the IALA VTS Committee.

Some of the topics presented in this document may also be of interest to equipment manufacturers and other stakeholders outside IALA.

## Related documents

IALA SS0001 Service Specification for VTS Traffic Clearance, version 1.5.1.

IALA SD0001 Service Design for VTS Traffic Clearance using SECOM, version 1.2.1

IALA S-212 Product Specification (draft) for VTS Digital Information, version 0.7.4

Report of the 58th session of the IALA Vessel Traffic Services (VTS) Committee

# Discussion

## Findings related to the Service Specification for digital VTS Traffic Clearance

### Message acknowledgement

Certainty that a message has been received and understood is a cornerstone of maritime communication. The automatic system acknowledgements (message delivery and message opening) used in the pilots did not provide the test participants sufficient assurance to feel confident that the recipient had read and understood their message.

Due to the nature of bridge operations, it is typical that reading a message may be interrupted by a more urgent task. This means that message opening does not necessarily guarantee that the message has been read. Especially in the case of safety-critical messages, any break or uncertainty in communication between VTS and a vessel can pose a significant safety hazard.

Based on the pilot results and the IALA VTS58 committee expert review, improvement of the TCS message acknowledgement practices towards closed-loop communication by including manual acknowledgements in the specification should be considered.

The current TCS specification mentions “manual interactions”, but they are not defined in more detail. Fintraffic proposes that the manual interactions be clarified in the specification and that the manual acknowledgement be added to service functionality and the S-212 data model.

### Traffic Clearance Service use cases

The straightforward use cases, in which a permission is requested and confirmed, were successfully completed during the pilots. However, in the more complex use cases, the service logic confused some of the test navigators. These included, for example, a two-stage anchoring permission request process, the denial of an already confirmed permission due to an unexpected occurrence, and situations in which the VTS operator required additional information before making the decision.

Any confusion between the VTS centre and a vessel may create a safety hazard. The topic raised discussion in the expert review, and using the digital VTS information service or VHF communication was identified as a potential risk mitigation measure for safety critical use cases, such as when a VTS centre denies a requested permission.

Fintraffic notes that reviewing the TCS specification and existing use cases from a safety perspective, and adding further example use cases, could support clearer understanding.

## Feedback related to implementation of future digital VTS services

### Common situational picture

Many test participants expressed concern about a common situational picture if departure permissions are handled digitally between a single vessel and VTS, since other vessels in the area cannot “hear” the exchange as they currently do when permissions are requested by voice over VHF.

The expert review pointed out that the possibility of sharing traffic information digitally with all vessels in the area is included in the plan for the digital VTS Traffic Functions Service. However, the challenge with smaller commercial vessels, fishing boats, and pleasure craft not equipped with S-100 compatible ECDIS remains to be solved.

The IALA VTS58 committee expert review recognized some possible options for sharing digital TCS information with vessels not carrying S-100 compatible equipment e.g., converting the digital permission information into speech and broadcasting it by VHF or using AIS ASM messages.

Digital VTS services are likely to be among the first two-way S-100 services to be implemented. The information exchanged through these services must be shared with other vessels in the area to ensure a common situational picture, creating both a need and an opportunity for IALA to drive the discussion and shape future solutions. Fintraffic suggests that IALA could consider initiating work to explore and define solutions for sharing information generated by digital VTS services with vessels not equipped with S-100 compatible equipment.

### Parallel use of digital and voice communication

Concerns about how digital and voice communication can be safely used in parallel in VTS services during the transition period were raised by the test participants. Although there was a wish that digital services would only complement existing voice-based communication instead of replacing it entirely, it was a common concern that switching back and forth between the two communication methods could cause confusion and potentially create a safety hazard.

The way in which digital VTS services are introduced has a significant impact on how much parallel communication is required and how it can be performed safely. So far, IALA has not defined whether digital VTS services will be introduced in phases or all at once. Early development of an IALA implementation plan for digital VTS services would support the timely planning of future VTS operational procedures for parallel communication.

The expert review recognized that the transition should be addressed in the IMO e-Navigation Implementation Plan. However, it also noted that IALA can play an active role in contributing to the IMO work. Fintraffic suggests that IALA consider initiating work on an implementation plan for digital VTS services.

### Effect of digital services on bridge operations

Test participants provided feedback on the potential impact of the transition to digital services on bridge workflow. Voice communication over VHF enables navigators to monitor VTS channels passively while navigating or performing other operational tasks. In contrast, digital systems may require active interaction, which can divert attention from the primary task and potentially reduce situational awareness.

This concern was also reflected in observations from the simulator pilot: in high workload situations, the test navigators prioritized operational tasks, and incoming digital messages—including potentially safety-critical ones—went unnoticed. This indicates a risk that important information may not be received or acted upon in a timely manner.

During the expert review discussions, the importance of the issue was recognized, and it was noted that human–machine interaction should be studied in more detail. The use of text-to-speech tools on the bridge during the transition phase was also suggested as a possible solution that could enhance safety.

As the matter falls within the responsibility of equipment manufacturers, IALA has limited possibilities to influence it directly. However, given the safety-critical nature of the issue, Fintraffic suggests IALA consider facilitating a dialogue with equipment manufacturers on this topic.

# Action requested of the Committee

The committee is invited to:

- Consider the findings from Fintraffic’s digital TCS service pilots and amend the Service Specification for digital VTS Traffic Clearance and the S-212 data model accordingly.

- Identify relevant IALA working groups for whom the feedback related to the implementation of future digital VTS services may be of value, and take appropriate steps to disseminate the information.

- Consider whether the topics presented in this document may also be of interest to equipment manufacturers or other external stakeholders, and how a dialogue on these matters could be initiated.

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain