Input paper:   VTS59-9.1.6

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM	□  ENG		□  PAP					X  Input

□  ENAV	X  VTS							□  Information

Agenda item 			9.1

Technical Domain / Task Number 2	2.5.2, 2.8.1

Author(s) / Submitter(s)			Sewoong OH(KRISO), Yunjee KIM(KRISO), Deukjae Cho(KRISO)

# Proposed approach for the Test Version of the S-212 Product Specification

# Summary

S-212 is being developed with the objective of establishing a product specification that includes the data models required to support the four functions and eleven services defined under VTS Digital Communications by the VTS Committee. At present, the S-212 data model supports the TCS service and the UKC service. Therefore, from a data model perspective, it is necessary to identify areas requiring extension to enable the development of digital VTS services. In accordance with the IHO S-97 Product Specification Development Guidelines and IALA Guideline G1106, Producing an IALA S-200 Series Product Specification, preparatory actions for the development of a test version (Edition 1.0) are proposed.

# Background

The VTS Committee is currently undertaking Task 2.5.2, Develop Technical Service Specifications for VTS, and Task 2.8.1, Develop a Product Specification S-212 for VTS on Digital Information, in support of the development of digital VTS services.

# Discussion

## Guideline on VTS Digital Communications

The VTS Digital Communications Guideline, currently under development by VTS Committee WG1, defines the Operational Functions of MS1 Vessel Traffic Service under the IMO Maritime Services in the context of e-Navigation. These Operational Functions include Route Functions, Planning Functions, VTS Information Functions, and Traffic Functions. For each function, detailed services and corresponding use cases have been defined.

For the development of digital VTS services, it is necessary to establish service definitions, service standards, and data models (product specifications). Accordingly, the corresponding service standards and product specifications have been mapped to each service included within the Operational Functions of digital VTS services.

Under Task 2.5.2 and Task 2.8.1, the VTS Committee is developing the service standards and data models. The results of mapping the completed and ongoing developments are presented in Table 1.

Table 1. Service Specifications and Related Data Models for Digital VTS Services

| Num | Operational Functions | VTS Digital Services | Technical Service Specification | Product Specification (Data Model) |
| --- | --- | --- | --- | --- |
| 1 | Route Functions | Route Exchange Service | Service specification for VTS – Vessel Route Exchange | S-421 |
| 2 | Route Functions | Route Reference Service |  |  |
| 3 | Route Functions | Route Crosscheck Service |  |  |
| 4 | Route Functions | Route Monitoring Service |  |  |
| 5 | Planning Functions | Traffic Clearance Service | Service Specification for VTS Traffic Clearance | S-212 |
| 6 | Planning Functions | Slot Management Service |  |  |
| 7 | VTS Information Functions | VTS Information Service | Service Specification for VTS Information Service | S-212 |
| 8 | VTS Information Functions | VTS Reporting Service | Service Specification for VTS Reporting Service | S-212, S-127 |
| 9 | Traffic Functions | Traffic Image Service |  |  |
| 10 | Traffic Functions | Navigation Assistance |  |  |
| 11 | Traffic Functions | Intended Track Exchange Service |  |  |

## Data Model Extension Requirements for Digital VTS Service Development

The VTS Committee is currently developing the S-212 data model (Product Specification) under Task 2.8.1. The existing data model has been established by adding feature types for TCS messages and UKC messages based on the common information structure of the VTS Digital Information Message. Further discussions are planned to extend the VTS Information Message, and the addition of a VTS Reporting Message may also be considered. Since the development approach of the S-212 data model involves adding detailed feature types for each specific service under the VTS Digital Information Message, it is anticipated that the S-212 data model can be extended, as illustrated in Figure 1, to support the information exchanges required for the detailed service operations defined in the VTS Digital Communications Guideline currently under development by VTS Committee WG1.

The contents of each feature type are defined as follows:

- Route Exchange Message: A message for exchanging route-related information, including route exchange, route reference, route validation, and route monitoring.

- Slot Management Message: A message exchanged for slot management and priority management.

- Traffic Image Message: A message used to share traffic situation information.

- Navigation Assistance Message: A message exchanged to provide navigation assistance services.

- Intended Track Exchange Message: A message for exchanging intended track and traffic information in accordance with the voyage plan.

Fig. 1 Proposed Extension Approach for the S-212 Data Model

## Requirements for the Preparation of the S-212 Ed.1.0 Test Version

The VTS Committee is developing the S-212 Product Specification in accordance with the IHO S-97 Product Specification Development Guidelines and IALA Guideline G1106, Producing an IALA S-200 Series Product Specification. S-212 Edition 1.0 is defined as a test version. From Edition 1.x up to, but not including, Edition 2.0, revisions will be made through testing and validation processes. Edition 2.0 and subsequent editions are defined as the operational versions intended to support the implementation of digital VTS services.

According to the IHO S-97 standard, the S-100 Readiness Levels are defined as illustrated in Figure 2. The components included in the Product Specification package are as follows:

- Main Document: The primary document of the Product Specification.

- Default Encoding: The recommended default data encoding format.

- S-100 Compliant Feature Catalogue: A catalogue expressing the contents of the data model in XML format.

- Data Classification and Encoding Guide: Guidance on data classification and encoding.

- S-100 Compliant Portrayal Catalogue: A catalogue describing symbolization and portrayal methods based on the data model.

- Data Quality Checks: Validation items related to data quality.

- Test Data Sets: Test datasets for verification purposes.

- Data Validation: Validation items to ensure data integrity and compliance.

- Encryption/Digital Signatures: Data encryption and digital signature mechanisms.

- Interoperability: Interoperability catalogue.

- Alerts and Indications: Provisions related to alerts and indications in ECDIS.

- Operational Data: Operational datasets produced in accordance with the operational version of the Product Specification.

Fig. 2 S-100 Readiness Levels

For the development of the S-212 Product Specification test version, the Main Document, Default Encoding, S-100 Compliant Feature Catalogue (FC), and Data Classification and Encoding Guide (DCEG) are mandatory components. Therefore, in addition to the main document currently under development, it is considered necessary to prepare the GML Schema, Feature Catalogue, and DCEG.

Since the S-212 data model is a message exchange standard for digital VTS services, there are no specific portrayal requirements. However, in order to support system implementation of certain included feature types, the application of the IHO-provided BPC (Basic Portrayal Catalogue) may be considered.

Figure 3 illustrates the structure of the BPC provided by IHO. It is considered that applying the BPC would be beneficial when testing S-212 test datasets within the S-100 Testbed or ECDIS environment.

Fig. 3 Basic Portrayal Catalogue의 구조

# Action requested of the Committee

The Committee is requested to:

- review the content contained in this input paper

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain