Input paper:   VTS59-9.1.7

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM	□  ENG		□  PAP					X  Input

□  ENAV	X  VTS							□  Information

Agenda item 			9.1

Technical Domain / Task Number 2	2.5.2, 2.8.1

Author(s) / Submitter(s)			Sewoong OH(KRISO), Yunjee KIM(KRISO), Deukjae Cho(KRISO)

# Data Model for the VTS Reporting Service

# Summary

The VTS Committee is currently developing guidelines and a use cases document for VTS Digital Communications, and has defined procedures for the VTS Reporting Service. In relation to the discussion made at VTS58 to use the IHO S-127 Marine Traffic Management Product Specification for VTS reporting requirements, sample S-127 data were produced, and the addition of a VTS Reporting Message to the S-212 data model was proposed for the VTS Reporting Service.

# Background

The VTS Committee is currently carrying out Task 2.5.2, Develop Technical Service Specifications for VTS, and Task 2.8.1, Develop a Product Specification S-212 for VTS on Digital Information, in order to support the development of digital VTS services.

# Discussion

## Use Cases for the VTS Reporting Service

The VTS Reporting Service is designed to enable ships to report information to VTS in accordance with requirements defined by VTS. The information reported by ships includes departure and arrival reports, position reports, and specific information that may affect vessel traffic. Ship reporting requirements may be based on FAL47/22.Add.1, and may also include reporting requirements specific to VTS.

The use cases for the VTS Reporting Service are defined as follows:

- Use Case 1 – Vessel makes initial contact and receives reporting requirements (Vessel subscribes VTS Reporting Service and receives all reporting requirements)

- Use Case 2 – Vessel sends complete report (Vessel sends report)

- Use Case 3 – Vessel sends invalid report or no report at all (Vessel sends incomplete report, no report or report with incorrect information)

- Use Case 4 – Vessel sends updated report (Vessel updates requested report)

In accordance with the above use cases, VTS shall provide reporting requirements to the vessel, and the vessel shall submit a complete report in compliance with those reporting requirements. Within the VTS Reporting Service procedure, the reporting requirements and the complete report exchanged between VTS and the vessel shall be transmitted as standardized digital data.

## Review of S-127 MTM and Development of Sample Data for VTS Reporting Requirements

At VTS58, discussions were held on reviewing a data model for the development of VTS Reporting Requirements data and on applying the IHO S-127 Marine Traffic Management (MTM) data model, which is being developed under Phase 2 of the S-100 Implementation Roadmap, to the production of VTS Reporting Requirements data. The S-127 MTM Product Specification, currently under development by the IHO NIPWG, addresses marine traffic management and includes a variety of feature types and information types.

In relation to VTS services, the Vessel Traffic Service Area feature is defined. This feature is derived from the Reportable Service Area, Supervised Area, and Organization Contact Area feature types. With respect to reporting requirements, an additional information type, Ship Report, may be incorporated.

Fig. 1 VTS Service Area Data Model in S-127 MTM

Fig. 2 Ship Report Information Type in the S-127 MTM Data Model

The Ship Report information type defines the type of ship report and the IMO SRS Format Code, and additionally includes notice time and text content. In order for VTS to provide VTS Reporting Requirements in response to a vessel’s request, S-127 MTM data are required. Using the IALA S-200 Test & Validation Tool, Korean S-127 sample data (VTS component) were developed. The latest data model and Feature Catalogue were verified through the IHO S-127 standard resources available on GitHub. The corresponding Feature Catalogue was then applied to the S-200 Test & Validation Tool, and S-127 MTM (VTS Service Area + Ship Report) sample data were produced as shown in Figure 3.

Fig. 3 Sample data of S-127 MTM (VTS Service Area & Ship Report)

In the case of the Republic of Korea, Navigation Reports and Anchoring/Berthing Reports were defined as follows. The SRS Format Code was applied in accordance with the Ship Report information type defined in the S-127 MTM data model, using the Ship Reporting Format set out in IMO Resolution A.851(20). For each detailed report, the conditions were specified as follows:

(1) upon arrival in the VTS area;

(2) 10 minutes prior to departure or movement;

(3) at the time of departure or movement; and

(4) when anchoring or berthing.

To indicate these conditions, the composite attribute Notice Time of the Ship Report information type was used, including Notice Time Hours, Notice Time Text, and Operation. However, since reporting conditions may differ among IALA Member States, the method of entering Ship Report data may vary accordingly. Therefore, it is considered necessary to share case examples of S-127 MTM data production among IALA Member States.

Fig. 4 Korean VTS Reporting Requirement Case and the IMO Ship Reporting Format Code under IMO Resolution A.851(20)

## Proposed Addition of a VTS Reporting Message to S-212 for the VTS Reporting Service

In accordance with the procedures defined in the VTS Digital Communication Use Case document, VTS provides reporting requirements to the vessel, and the vessel subsequently submits a VTS Report to VTS in compliance with those reporting requirements.

For the reporting requirements provided by VTS to the vessel, the S-127 MTM data model may be applied. The VTS Report submitted by the vessel to VTS may be based on the S-212 data model. In this regard, the addition of a VTS Reporting Message to the S-212 data model is required.

Fig. 5 Data Model for the VTS Reporting Service (S-212 / S-127)

In relation to the agreement reached at the intersessional meeting held in Ostend, Belgium, in January 2026, to proceed with the development direction of the S-212 data model by extending detailed feature types under the VTS Digital Information Message, the figure below illustrates the addition of a VTS Reporting Message feature type to the existing S-212 data model. he detailed attributes included are as follows:

- Category of Ship Report: Type of ship report

- SRS Format Code: Format type in accordance with the IMO Ship Reporting Format Code under IMO Resolution A.851(20)

- SRS Format Content: Report content in accordance with the IMO Ship Reporting Format Code

Since reporting requirements vary by VTS area, multiple reporting formats and corresponding contents may be required depending on the specific reporting requirements. Accordingly, the SRS Format Code and SRS Format Content attributes were designed to allow multiple entries. These entries are to be provided in sequence to ensure that each SRS Format Code correctly corresponds to its matching SRS Format Content.

Fig. 6 Approach to Add a VTS Reporting Message to the S-212 Data Model

# Action requested of the Committee

The Committee is requested to:

- develop VTS reporting requirements of IALA Member States in S-127 format, share relevant case examples, and encourage IALA Member States to produce S-127 data related to VTS reporting requirements.

- request a review of the proposed revised S-212 data model to enable the exchange of VTS reporting information using the S-212 data model.

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain