A number of queries and suggestions arose when this Guideline was being reformatted to the new IALA image, as shown in the comments throughout the document. The ENG Committee is requested to respond to these queries and suggestions and submit to the Secretariat to enable completion of the reformatting. IALA Guideline

1043

Light Sources used in Visual Aids to Navigation

Edition 1.2

December 2011

Revisions to this IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |
| May 2008 | Entire document | Reviewed and updated at IALA Floating Aids 2008 workshop (April 08) and EEP11 |
| December 2011 | Sections 2, 3.1, 3.2, 3.4, 4, 6, 7.3 Pages 9, 10, 17 to 20, 31 to 3 Removal of LED lifetime estimation example | EEP Work Programme Task |

1	INTRODUCTION	5

2	SCOPE	5

3	LIGHT SOURCES	5

3.1	Brief History	5

3.2	Categories of Light Sources	5

3.3	Incandescent Lamps	6

3.3.1	Tungsten Filament Lamps	8

3.3.2	Tungsten Halogen Lamps	10

3.4	Lamps Using Electric Discharge	14

3.4.1	Fluorescent Tube	15

3.4.2	Mercury Vapour, High-pressure Sodium	15

3.4.3	Low-pressure Sodium	15

3.4.4	Metal Haide	15

3.4.5	Xenon Lamp	17

3.5	Light Emitting Diodes	19

3.5.1	Advantages	20

3.5.2	Disadvantages	20

3.5.3	Operational, Environmental and Financial Issues	21

3.6	Laser	22

3.6.1	Guiding Approach	23

3.6.2	Types	23

3.6.3	Advantages	23

3.6.4	Disadvantages	23

3.6.5	Operational, Environmental and Financial Issues	24

3.7	Gas lamps	25

3.7.1	Acetylene	25

3.7.2	Propane and Butane	25

3.7.3	Advantages	26

3.7.4	Disadvantages	26

3.7.5	Filter Information	26

3.7.6	Typical Application	26

4	SUMMARY AND COMPARISON OF LIGHT SOURCES	26

5	Spectral Distribution of Light Sources	29

6	LIGHT SOURCE LIFETIME CONSIDERATIONS	29

6.1	General Considerations	29

6.1.1	Example	30

6.2	LED Light Source Degradation	30

6.3	LED Product Lifetime Estimation	31

6.4	Performance Measurement of LED Light Sources in Field Conditions	32

7	SPECIAL APPLICATIONS OF LIGHT SOURCES	32

7.1	Light Poles and Light Pipes	32

7.2	Signage	34

8	DEFINITIONS	36

9	ACRONYMS	36

10	REFERENCES	36

ANNEX A	COMPARISON OF THE ANGULAR INTENSITY PROFILE OF A 12V 100W CAPSULE HALOGEN LAMP AND A 10W LED BARE CHIP	38

List of Tables

Table 1	Example of a table with the significant information in the first column	4

Table 2	Example of a table with the significant information in the first row	4

Table 3	Example of a table with coloured rows	4

Table 4	Example table	4

List of Figures

Figure 1	Example figure	4

Figure 2	Another example figure	4

List of Equations

Equation 1	Geographical range	4

Equation 2	Theory of Special Relativity	4

# INTRODUCTION

Through the work of the IALA Committees and various workshops, IALA has been providing timely and accurate information to members on developing light sources.  These documents have been integrated into a single Guideline on existing and developing light sources that should be more beneficial to members.

# SCOPE

This Guideline is intended to provide information to members on existing and developing light sources used in maritime aids to navigation (AtoN) systems.  It provides information on associated operational considerations, such as AtoN light source lifetime, reliability, operating costs and power consumption.

It also gives examples of typical applications.  It covers the light sources themselves and not the associated optical parts of an AtoN system, unless those optical parts are integral to the light source.

# LIGHT SOURCES

## Brief History

Until the first application of electricity to lighting in the early twentieth century, all artificial light was produced by fire.  Illuminants progressed from pyres of wood (used up until the 1700’s), to oil wick lamps, vaporised oil and gas burners, electric arc and tungsten filament lamps.  Optical devices matched these developments, first with reflector systems and later with lenses.

It is interesting to note that the efforts to understand human perception of light and to improve the efficiency and effectiveness of AtoN illuminants and optical apparatus were for many years at the forefront of scientific endeavours.

The lens design pioneered by Fresnel, around 1820, remains a principal element of the modern AtoN light.  However, modern lenses are often made of plastic rather than glass.

The majority of AtoN lights use electric light sources of various types.  Increasingly, these light sources are powered from renewable energy sources such as solar, wind and wave (see IALA Guideline 1042 on Power Sources for Aids to Navigation). Very few countries may still have gas lights, that burn acetylene or propane. Consequently, this guideline does not cover this light source technology.

Optical equipment for lighthouses and beacons are generally proprietary products although, from time to time, lighthouse authorities have developed their own equipment.

Some electric lamps have been specifically designed for AtoN applications..  However, lamps selected from the enormous range of commercial products have also been used or adapted for AtoN equipment.

Light emitting diode (LED) technology hass developed as the preferred light source, used either singly or in arrays.  Development of powerful and efficient white LED components is ahead of the development of colour LED components, driven by mass market lighting application needs. More detailed information on LEDs is given in ANNEX B.

Some common types of incandescent light sources are shown in Figure 2.

## Introduction to the Categories of Light Sources

Figure 1 shows the categories of the various light sources that are commonly used in AtoN.  The light sources that are described in this Guideline are identified by a reference to the sub-section in which the information is contained.

3.3 Lamps using
Electric Discharge

3.2 Lamps using filament
Incandescence

3.4 Light Emitting Diode

Low Luminance

Tungsten Filament (§ 3.2.2)

High Luminance

Low pressure Sodium (§3.3.3.)

Fluorescent (§ 3.3.1)

Compact Fluorescent

Tungsten Halogen (§ 3.2.3)

High Pressure Sodium

Mercury Vapour (§3.3.2)

Metal Halide (§ 3.3.4)

Coated Envelope Mercury Vapour

Xenon (§ 3.3.5)

3.5 Laser

Categories of light sources

### Incandescent Lamps

Incandescent lamps are thermal radiators and generate light by heating a solid body to a high temperature - the higher the temperature, the ‘brighter’ the light.  In electric incandescent lamps, this solid body (usually a filament) must also be an electrical conductor.

The incandescent material must fulfil two requirements in order to be useful as a light source:

high melting point;

low rate of vaporization.

Early electric lamps used carbon for the incandescent filament; modern incandescent lamps use tungsten.

A selection of lamps manufactured for AtoN applications

(Courtesy of Tideland Signal Corporation, USA)

### Tungsten Filament Lamps

Although tungsten is not quite as good a thermal radiator as carbon, it is a more suitable incandescent filament material due to its low rate of vaporization at elevated temperatures approaching its melting point.

Historically, the manufacture of tungsten filaments presented a number of problems due to the brittleness of pure tungsten and the difficulty of forming fine wires.  In modern lamps, tungsten alloys are used that enable the properties to be controlled within wide limits.

The emissivity or radiation from a hot tungsten source has a spectral distribution over the ultraviolet, visible and infrared (heat).  At the highest practicable temperatures, the radiation distribution peaks at about 850 nanometres.  In this case the energy balance is typically:

visible light 	5%;

heat losses 	12%;

radiated infrared 	83%.

Over the operating life of the lamp, vaporized filament material is deposited on the inner wall of the glass bulb (envelope) and blackens it in the process.  The blackening reduces the amount of light emitted from the lamp.  Increasing the envelope size is a means of distributing the tungsten deposits more widely.  An example of this can be seen with the 3.0 amp, CC8 filament, P30s lamp that is available in an S8 or S11 envelope.  The S8 has a higher initial lumens output than the S11, but degrades more quickly through blackening.

Lamps that have been specifically developed for AtoN beacons generally consist of:

a coiled or coiled-coiled tungsten filament;

a precision glass envelope (made of soda glass) that is either:

filled with an inert gas such as nitrogen or argon;

evacuated (less common).

With a prefocus cap base, such as the:

P30s (as used in four or six position lamp changers);

BA22d-3 (twin filament lamps);

3 pin Bayonet (twin filament lamps).

#### Advantages

low investment cost;

reliable;

availability on small size lamps;

proven technology;

small size dual filament available (giving redundancy due to spare filament);

can be switched on and off to provide character with appropriate power supply design and character ratio;

can be used with coloured filters;

universal lamp operating position;

choice of filament geometry to suit optic;

relatively simple to operate;.

Flexibility to run at lower than nominal voltage to significantly prolong life to reduce maintenance costs

#### Disadvantages

high operational costs;

relatively short lifetime;

relatively low colour temperature (with some possibly not in the IALA white preferred region);

aging during storage;

sensitive to shocks and vibration;

intensity drops due to blackening during the lifetime;

availability of lamps is becoming more limited;

care must be taken when selecting commercial lamps to ensure filament geometry, lamp life, etc., are appropriate for the required application (e.g. inconsistencies such as filament size/position that are inconsequential to commercial applications may cause issues in AtoN installations) .

Takes time to reach full brightness

High inrush current

When running at lower than nominal voltage, the spectrum may change and luminous output will reduce

#### Operational, Environmental and Financial Considerations

voltage: From 6 to 300 volts;

current: From 0.125 to 15 amps;

colour temperature range from 2,200 to 3,000°K;

efficacy up to 16 Lumens/Watt;

nominal lamp life ranges from 50 to 1,500 hours;

envelope material is fragile;

envelope temperature - risk of burning;

problems with flashing for larger lamps above 100W (longer time to full incandescence);

for larger lamps, maintain a ‘simmering’ current of about 10-20% of the rated current on the lamp during the eclipse times;

filament position tolerances should be considered when re-lamping;

supply voltage control, must be sufficiently precise to ensure long life;

filament geometry may affect the beam profile;

protection required against direct contact with water;

envelope and filament design and tolerance may affect distribution of light output;

general precautions in handling glass apply;

requires mechanical lamp changer for redundancy and extended service interval;

approximate transmittance when used with acrylic coloured filters (% of remaining light intensity):

red Plexiglas 501	24%;

green Plexiglas 1677	34%;

green Plexiglas 701	20%;

yellow Oroglas 2246	69%;

yellow Plexiglas 1989	65%.

#### Application Area

Applicable to all AtoN (from buoys to large optics) but use is diminishing due to high operation cost.

### Tungsten Halogen Lamps

Tungsten halogen lamps feature a tungsten coil filament mounted in a quartz glass envelope that has been filled with an inert gas (usually krypton or xenon) mixed with traces of a halogen element (usually bromine or iodine).

A selection of tungsten halogen lamps

When the lamp is operating under normal conditions, convection currents are set up between the hot filament and the cooler walls of the lamp.  The circulating gas mixture drives the ‘halogen cycle’ in which tungsten molecules, vaporized from the incandescent filament, combine with oxygen and halogen atoms in the cooler region near the envelope wall (around 700°C) to form tungsten oxyhalide molecules.  These molecules remain in a vapour phase at the lower temperatures around the envelope wall, which prevents the tungsten condensing onto the envelope.  The tungsten oxyhalide vapour is then carried on convection currents back towards the filament.  In the hotter region around the filament (3000°C), the tungsten oxyhalide separates, by dissociation, allowing the tungsten to be deposited back onto the filament or to continue as tungsten vapour on the convection currents towards the envelope.  The result of this cycle is that the glass envelope stays relatively clean.

In order for the halogen cycle to function correctly, the temperatures at the envelope wall need to be controlled within certain limits.  This is done by making the envelope the correct size and shape relative to the filament and the resultant envelope is usually much smaller than those of non-halogen tungsten filament lamps.  With a small envelope, a more expensive glass can be used such as quartz glass, and the combination of quartz glass and a smaller envelope allows higher gas pressures and the efficient use of more expensive gas mixtures (such as krypton or xenon).  The higher gas pressure (up to 20 bar) helps suppress the rate of vaporization of the incandescent filament.  The reduced rate of vaporization of the incandescent filament can be used to either increase the lamp life, or increase the filament temperature.

The visible spectrum lies between 380 and 780 nanometres.  The luminous efficacy decreases as the lamp voltage / colour temperature is lowered.

During operation, the surface temperature of a tungsten halogen lamp envelope can be 600°C or more, however, lampholder temperatures are generally limited to around 250°C to prevent oxidation of the conductors and premature lamp failure.

Operating tungsten halogen lamps significantly below their rated voltage can lower temperatures to an extent that may inhibit the halogen cycle and lead to blackening of the envelope and a shorter life.  Interruption of the halogen cycle can also occur when the lamp is flashed for short durations at low duty cycles.

Users should be informed on occupational safety issues relating to tungsten halogen lamps.

The issues include:

The high operating temperatures and the need to wait a sufficient time for lamps to cool down once extinguished.

The risk of eye damage due to glare and UV emissions:

The high average luminance values of (up to 3,000 cd/cm2 at the filament) can cause glare problems and the lamp housing should not be viewed with the naked eye.

Depending on the applied voltage and colour temperature, tungsten halogen lamps with quartz glass envelopes will emit about 0.2 % to 0.3 % of the electrical power in the form of UV radiation (i.e. below 380 nanometres).  The UV radiation is higher than for standard filament lamps, but still very low compared to other lamps.  UV-free lamps are now widely available, whereby special absorbing elements are introduced into the glass envelope; these are recommended for use where possible.

A tungsten halogen lamp should only be held by the base.  Any fingerprint residues left on the quartz glass will burn when the lamp is operated and cause the glass to devitrify.  This can make it opaque, reduce the strength of the glass and increase the risk of the envelope rupturing.

#### Advantages

high luminance allows the design of lights with a very narrow angular distribution, as often required for direction lights;

relatively low investment cost;

reliable;

availability on small size lamps;

the lamp envelope can be made smaller;

there is less degradation of the light output over the life of the lamp compared to tungsten filament lamps;

higher colour temperature than tungsten filament lamp (‘whiter’ light ~3,000°K);

universal lamp operating position for most low-power lamps (some lamps with high power consumption have limited operating positions);

longer operational life than a tungsten filament lamp;

longer shelf life than a tungsten filament lamp;

higher efficacy than a tungsten filament lamp;

robust compared with a tungsten filament lamp;

can be switched on and off to provide character with appropriate power supply design and character ratio;

physically and electrically compatible with existing tungsten filament lamp equipment;

relatively simple to operate;.

Higher transmittance with red filters compared with LEDs

#### Disadvantages

lamp must operate above the temperature necessary to sustain the halogen cycle;

high in-rush current;

higher wattage lamps are sensitive to cold starts;

limited choice of filament geometry;

filaments are usually compact which may be too small for some applications, especially with respect to wide vertical divergence;

more sensitive to power supply variations than tungsten filament lamp (filament burnout);

flash length may be restricted due to halogen cycle;

high temperatures may cause problems with lampholder contacts;

envelope and filament design and tolerance may affect distribution of light output;

poor manufacturing tolerances may cause selection or adaptation to be required for marine AtoN applications otherwise consistency in performance will be poor;

special lamps, e.g. for marine applications, are less easily manufactured;

care must be taken when selecting commercial lamps to ensure filament geometry, lamp life, etc., are appropriate for the required application;

more handling precautions are required than with tungsten filament lamps;

training in handling may be required.

#### Operational, Environmental and Financial Considerations

voltage:  From 6 to 300 volts;

current:  From 0.5 to 15 amps;

colour temperature range, from 2,900 to 3,400°K (IALA preferred region);

luminous efficacy of up to 25 lumens per Watt;

nominal lamp life of up to 4,000 hours;

batteries usually have very low internal resistance and a cold lamp experiences a high in-rush current that places a very heavy load on filament and lead-in wires, welds and connecting foils.

when a battery is fully charged or is being recharged, the terminal voltage may exceed the rated lamp voltage to an extent that brings the filament of high output lamps close to its melting point and causes premature lamp failures.

To ensure the correct lamp operating conditions, the use of voltage regulators is highly recommended.

avoid operating the lamp below the temperature necessary to sustain the halogen cycle;

avoid having high wattage lamp experience continuous cold starts;

This situation can be overcome by using a current limiting ‘soft start’ or by maintaining a ‘simmer’ current of about 10-20% of the rated current on the lamp during the eclipse times. However, a simmering current may result in periods of operation without the halogen cycle. Tests should therefore be conducted to verify satisfactory lifetime.

halogen cycle operation failure results in rapid lamp blackening;

- The lamp must by operated at sufficient voltage and flash length to allow the lamp to reach the correct temperature and therefore enable the halogen cycle operation

envelope material;

envelope temperature;

small, compact filament;

problems with flashing for larger lamps above 100W (halogen cycle);

when used with coloured filters, red transmittance is slightly lower than with a tungsten filament lamp;

supply voltage control, must be sufficiently precise to ensure long life;

safety in handling, as contamination from fingerprints on the envelope can reduce life;

compatibility with existing equipment may be a problem because the envelope shape or size and/or the filament shape and size may not match optic (especially in larger optics where the optic was designed for a large light source and the modern light source is much smaller causing incorrect focusing);

bi-pin capsule lamps with high current may suffer pin/base contact problems;

a tungsten halogen lamp is usually more durable than the equivalent tungsten filament lamp due to its rugged filament and compact envelope;

safety procedures should be defined to include gloves, goggles and advice on glare;

The gloves can also protect the envelope by minimising finger print contamination.

some protection around the lamp may be advisable;

training in handling is required but this applies to all high performance lamps;

care should be taken in disposal due to the high pressure in the envelope;

general precautions in handling glass apply;

when UV-free lamps are not used, UV radiation should be considered, especially for high power lamps;

The use of UV safety goggles is recommended in this case.

usually requires less energy than conventional filament lamps, which is a positive effect on the environment;

longer life produces less waste;

smaller lamps produce less waste;

longer life than conventional filament lamp reduces maintenance visits;

higher efficiency reduces power (design) requirements.;

lamps are usually more expensive than tungsten filament equivalent but this is offset by increased life;

cost of training may be slightly higher;

requires mechanical lampchanger for redundancy and extended service interval;

reduced stock holdings due to longer life;

transmittance when used with acrylic coloured filters (% of remaining light intensity):

red Plexiglas 501	19%;

green Plexiglas 1677	36%;

green Plexiglas 701	18%;

yellow Oroglas 2246	65%;

yellow Plexiglas 1989	65%.

#### Typical Application

It is applicable in all AtoN (buoy to large optics).  It is still a common solution in long range flashing applications.

## Lamps Using Electric Discharge

Discharge lamps differ from filament lamps in that their operation depends not on the incandescence of a hot piece of wire but on electrical discharge through an ionised gas or vapour causing an arc of hot gaseous particles.  All discharge lamps require some form of igniter to start the ionisation.  Once started, electrical current can be enormous and cause damage to the lamp.  To prevent this a ‘ballast’ is placed in series with the arc tube in order to limit or regulate current.

Low-pressure discharge lamps typically have long arc tubes and low igniter voltages.  At low pressures, the spectral content of ionised gas or vapour tends to consist of a few narrow spectral lines (for instance, a mercury vapour lamp has four spectral lines in the visible spectrum).  These lamps do not provide good colour rendering, in other words details and colours of surfaces viewed under such light are poorly defined.  The spectral output of low-pressure lamps can be used to excite a phosphor coating, deposited on the inside of the lamp envelope, so that an improved spectral distribution of light is emitted.  This is the case with fluorescent tubes that can give coloured or white light depending on the type of phosphor coating.

When the pressure inside the arc tube is increased, the spectral content of the ionised gas or vapour becomes more spread.  Some high-pressure discharge lamps, with arc tube pressures of up to 50 bar, give a very broadband, white light.  Such lamps have very short arcs and very high strike voltages.  The handling and operation of such lamps can be risky, with the potential for arc tube explosion.  Because of the very high luminance of such lamps, they are often called ‘high intensity discharge’ (HID) lamps.

When metal halides were introduced into the arc tubes of mercury and sodium vapour lamps, it was found that only moderate pressure was needed in the arc tube to yield a white light.  By-products of lower arc tube pressure were improved safety and lower ignition voltage.  Metal halide and high-pressure sodium lamps are commonplace in street and floodlighting applications.

With the exception of coated envelope multi-vapour and fluorescent lamps, low-pressure discharge lamps are generally unsuitable for AtoN.  One exception is low-pressure sodium lamps that are sometimes used as fixed yellow AtoN lights.

### Fluorescent Tube

Fluorescent tube lights are sometimes used to mark breakwaters’ jetty heads and to provide leading lines.  This is a low cost approach that may be suitable for meeting the needs of recreational and fishing vessels.  Typical colours used are red, green, white and blue, although they may not comply with IALA Recommendations for the colours of light signals on AtoN.

Other application areas are sign illumination and direction arrows.

### Mercury Vapour, High-pressure Sodium

The spectral distribution from uncoated mercury and high-pressure sodium lamps is not well suited to white AtoN signal light applications.  These lamps are not normally used as AtoN light sources.

### Low-pressure Sodium

Examples of low-pressure sodium lamps

A low-pressure sodium lamp has two lines very close together (so that they may be considered as one) in the yellow preferred region.  This makes them useful for applications such as markings for inland waterway channels or small waterways (e.g. Kiel Canal).  Since service life is about 5 years in practice and their efficacy is about 5-times higher than a yellow LED, they remain a popular choice of light source.

Low-pressure sodium lamps are sometimes used for marking structures (e.g. bridge supports and pier heads) where they are particularly useful during foggy conditions.

### Metal Haide

Examples of Metal Halide Lamps

Metal halide lamps consist of an arc tube that contains mercury vapour, various metal halides, and argon.  Some have an outer envelope containing the arc tube and other controlling devices.  These lamps generally require a ballast circuit to regulate the lamp current, an igniter or ‘striker’ to initiate ionisation during start up and possibly some regulator or autotransformer to accommodate supply voltage fluctuations.

When the lamp is operating, the metal halides are vaporised and disassociated in the inner core of the arc into the halide and the metal, with the latter radiating their appropriate spectrum, thereby adding to the spectral content.

When a lamp is turned on, it takes several minutes to reach normal operating conditions (including vapour pressures).  If the supply voltage is interrupted sufficiently for the arc to be extinguished, the lamp will not relight until it cools and arc-tube vapour pressure decreases to a level that allows the arc to re-strike.  This may take as long as fifteen minutes.  The metal halide lamp cannot be flashed for AtoN applications and is only used in rotating lens and rotating shutter optics.  Hot re-strike circuits, that apply a very high voltage to the hot arc tube electrodes, are available but these can cause arcing in single ended lamps and can drastically shorten lamp life.

#### Advantages

high efficiency (up to 120 Lm/Watt);

long operational life (6,000 to 20,000 hours);

rugged construction;

lower relative operating temperature;

choice of white available (3,000 - 6,000°K);

choice of colours available (floodlighting effects).

#### Disadvantages

cannot be switched on and off to provide a rhythmic character;

some lamps have restricted burning positions;

lamp warm up period required;

restarting time in the event of power interruption;

light output reduces significantly over operating life;

colour changes significantly over operating life;

complex power supply required;

inherent Radio Frequency Interference (RFI) is a potential problem;

significant UV output;

lamp bases are often not precision/pre-focus (re-focus on replacement);

limited choice of arc tube geometry (usually tall and thin);

poor transmittance and colour variance with red filters.

#### Operational, Environmental and Financial Issues

wide range of wattages available;

attention must be given to the proximity between the ballast, the igniter and the lamp due to the capacitance of the cable;

low ambient temperatures may affect ballast operation;

monitoring systems are more complex;

select a lamp colour temperature to match AtoN requirements, typically white;

select a lamp with low UV output due to environmental concerns;

due to colour variation and light output reduction over age, scheduled lamp replacement is important;

shape of the arc may resemble a teardrop and care must be given to focus to ensure light centre placement gives correct beam direction;

wider variety of bases exists;

high strike voltage hazard;

mercury content may create disposal problems;

protective clothing such as goggles, gloves, etc., required due to potential arc tube explosion and high operating temperature;

high efficiency saves energy;

lower disposal rate due to longer life;

low maintenance cost due to longer life;

lamp cost is higher than equivalent tungsten halogen or tungsten filament lamp;

initial installation costs higher;

training of personnel required;

when using with coloured filters, care should be taken to ensure spectral content of lamp yields correct colour;

transmittance when used with acrylic coloured filters (1kW MBI Lamp; % of remaining light intensity):

red Perspex 4401	6%;

green Plexiglas 1677	36%;

green Plexiglas 701	18%.

#### Typical Application

Long range beacons, lighthouses and upgrading of the light source using existing rotating optics (modernization and/or longer range).  Also sometimes used for floodlighting.

### Xenon Lamp

The xenon arc lamp has a short-arc length, high arc tube pressure and provides a compact light source.  They typically reach 80% of their final output immediately after start.

A Xenon lamp

The xenon lamp has the highest luminance of all artificial light sources.  Therefore, lights with very narrow angular distributions can be realized with them.  A typical application is the use of a 2000W xenon lamp in a rotating optic or a precision sector light.

Application example of Xenon lamps in a drum lens

A 150W xenon lamp developed for automotive headlights has been considered for the light source for a rotating reflector array for lighthouse applications.  The lamp operates on a voltage between 16 and 20 volts dc, supplied from a 12 volt dc input.  It is claimed to have a relatively flat spectral output over the visible wavelength band allowing for excellent colour projection in both red and green as well as providing a 6000°K ‘daylight’ coloured white light.

A combination of xenon and metal halide technologies has resulted in compact low-wattage lamps (typically 10 - 60W) for use in bicycle and automotive headlights.

#### Traditional short-arc xenon lamps

##### Advantages

highest luminance available;

high energy efficiency;

approximately 3000 hours’ life;

##### Disadvantages

the lamp is hazardous: extremely high UV output, it may explode, it is very hot and ignition needs 40 kV;

an inability to flash the lamp at a rate suitable for AtoN applications;

a complex power supply monitor/control circuit;

a service life that is heavily dependent on the number of ignitions experienced;

limited availability;

expensive.

#### New developments in automotive headlamps

##### Advantages

high energy efficiency;

compact light source;

low wattages available (10-60W);

low voltage DC control available;

relatively long life;

automotive specification (rugged).

##### Disadvantages

high initial cost;

not suited to flashed rhythmic characters;

not efficient when used with red filters;

a complex power supply monitor/control circuit.

## Light Emitting Diodes

A Light Emitting Diode (LED) is a solid-state light source (photo-luminescent semiconductor junction) that is fundamentally different from an incandescent lamp.  It emits radiation in a narrow spectral band in the infrared, ultra-violet or visible spectrum when a current is passed through the junction.  Spectral distribution is narrow, in the order of 50 nm.  To obtain white light, rather than a single colour, two techniques are employed: phosphor- conversion (pcLEDs) that use a blue or UV LED surrounded by a yellow phosphor; multi-chip, typically three LEDs of red, green and blue (RGB).  The mixture of blue and yellow or RGB produces a near-white light.  The RGB mixture can be varied to produce different colours.  This is useful in colour displays but can present problems with colour consistency for a signal light.

There are currently two main types of LED junction, InGaN and AlInGaP.  The InGaN or nitride types operate in the shorter wavelengths from green to ultraviolet, reaching down to yellow light with phosphor conversion technology.  The AlInGaP or phosphide types are confined to the longer wavelengths from yellow to infrared. New organic types of LEDs (OLED’s) are in development but are not yet suitable for light signalling application.

When compared with incandescent technology, coloured LEDs are much more efficient than incandescent lights with filters, and white is presently at least twice as efficient as incandescent.  However, white LEDs are fast being developed for use in general illumination and recent developments have delivered efficacies of over 200 lumens per Watt.

Power ratings for LEDs vary from a few milliwatts to over 40W per junction/component. LEDs can be arranged into groups or arrays to improve intensity and light source geometry.  When used in groups or arrays wiring arrangements can vary considerably, but are typically series/parallel, with consequences on the number of LEDs that may be lost if a single LED fails.

LEDs require careful thermal management and complex drive circuitry that can vary in efficiency.  Within an LED, up to 15% of the energy is emitted as light and the remainder as heat.  Unlike conventional light sources, which dissipate heat by radiation, convection and conduction, all heat from the LEDs must be conducted away by the mounting or luminaire.

High power LED

LEDs with a luminance (in candela/sq. m) comparable to the luminance of tungsten halogen lamps are available.  However, the light output of a typical LED component has a Lambertian distribution, it is not omnidirectional like an incandescent bulb (see the comparison in Appendix 1).  Since the performance of a particular manufacturing batch cannot be precisely predicted, manufacturers use a system of binning by luminous intensity, forward voltage, spectral properties etc. to map the batch-to-batch differences.

LEDs can be made to work effectively in large optics.  Multiple LEDs or One-Chip-LED-arrays can be used to provide the same intensity and intensity distribution as a tungsten halogen lamp in large optics.  Many individual LEDs have an integrated lens that produces a beam.  Secondary lenses may be used to modify the beam to the required shape.  Bare LED chips usually emit light through a limited angle, typically Lambertian, i.e. in one direction from a flat surface so that the radiated intensity, by angle, obeys the cosine law.  The optical components used with an LED should be adapted to the particular angular distribution of the LED in order to produce the required beam pattern efficiently.  For example, replacing a halogen lamp with an LED in a light with a narrow beam and a large mirror would not be an efficient way of producing a similar beam.  For intended use in traditional optics, it is important to position the LED light source towards the lens and/or reflector to make best use of the collection angle.  In many cases it may not be possible to position a large LED array, or even a single LED component, to achieve the intended beam parameters and provide adequate cooling.

Earlier LEDs encapsulated in clear epoxy had both lower reliability and shorter life than newer LEDs encapsulated in optical silicone.  Epoxy LEDs can lose as much as 50% of their original brightness in as little as 3000 hours when driven using maximum allowed current, whereas those encapsulated in silicone can maintain 70% of their original brightness at over 50000 hours.

### Advantages

High reliability;

energy efficient white and coloured light generation;

light produced in saturated signal colours. LEDs generally maintain the colour through whole lifetime;

physically rugged and robust;

Slow continuous lumen depreciation and rarely catastrophic failure;

long life depending on driving techniques, thermal management, environmental conditions, duty cycles and type of LED;

instantaneous on-off switching of light;

Wide range of flash shapes can be generated

possible advantage of improved conspicuity due to the colour (narrow spectral distribution) and square-wave flash profile (e.g. the use of flickering light);

does not have a high inrush current;

no shadows created by filament supports

no complex maintenance requirements;

LED groups or arrays can substantially reduce the probability of total lamp failure;

no mechanical moving parts such as lampchangers.LED technology continues to develop.

Cost per lumen is reducing.

### Disadvantages

light output reduces with increasing LED operating temperature;

colours also shift with LED operation temperature (about 2nm per 1°C for red and yellow) and in some cases age;

LED efficiency decreases at increased current levels (so called ‘droop’) and so does the operational lifetime.

heat management of LED devices is essential to limit degradation of performance and maintain life;

multi-chip (RGB) LEDs are not recommended for white light sources because their colour consistency varies with operating conditions and age;

individual LEDs within an array can vary in their light output and beam distribution, affecting overall beam pattern;

LED technology is changing and developing rapidly;

Long term experience with these devices is therefore limited while LED component types tend to change in few years from introduction to obsolete phase, presenting challenges for product manufacturing life cycle management.

relatively complex electronic control needed to achieve long life and high performance;

some additional considerations when using LEDs as a replacement for lamps in traditional lenses, especially with coloured sectors due to the lower transmittance when used with red and green filters.

monitoring systems are more complex;

LED efficiency decreases at increased current levels (so called ‘droop’) and so does the operational lifetime.LED colours vary and may be outside the IALA preferred colour regions and therefore may have to be sorted;

Susceptible to electrical overload (e.g. nearby lightning strikes)

### Operational, Environmental and Financial CONSIDERATIONS

#### Operational

the very long operating life and low power requirement may reduce requirements for maintenance resources and infrastructure;

LEDs allow the production of small, self-contained lanterns;

reduces complexity of maintenance and, hence, the technical competence and instruction required.

test results obtained using standard photometric equipment may contain large errors;

luminous efficacy of LEDs is improving steadily, so LED AtoN lights will continue to improve in efficiency for the foreseeable future;

lLED lights are now widely used;

for short range applications LED are used almost exclusively;

region-specific safety labelling requirements.

purchase cost depends on range and features. Low cost lights available for low intensity, up to high intensity where the initial purchase cost can be higher than incandescent lamp lanterns.

Since coloured LEDs are available, coloured filters may be replaced by the appropriate coloured LED. White LEDs are still used with coloured filters where appropriate but the transmissivity is lower compared to filament lamps.

Due to the fast switching capability of LEDs, pulse width modulation (PWM) may be used to control intensity, generate various flash shapes and generate a “fixed and flashing” character.

While the LED component does not exhibit a high inrush current, high power LED lanterns may exhibit this characteristic.

When using an LED lantern, any spare often consists of an entire spare lantern thus increasing the initial capital cost.

Due to the high reliability and long life of LEDs many authorities no longer use lampchangers to provide additional redundancy. However many choose to use a main and standby lantern, where the standby lantern will be activated in the event of the main failing. Another approach utilises two lanterns running simultaneously where each lantern alone meets the required nominal range. In the event of one lantern failing the nominal range is still met. Yet another approach utilises two lanterns simultaneously run at 50 % intensity each, in the event of one failing the remaining lantern is increased to 100 % intensity to compensate.  .

depending on drive circuitry, the light output can vary with ambient temperature but this variation may be compensated by controlling the current;

#### Environmental

LED AtoN lights present no more environmental issues than other AtoN lights;

self-contained LED AtoN lights that contain a battery can present disposal problems.  It is recommended that they be returned to the manufacturer for recycling;

less power consumption leads to less batteries, solar panels, fuel requirements, etc., possibly resulting in smaller buoys and smaller moorings;

These factors reduce the environmental impact of the AtoN to which LEDs are fitted.

less frequent servicing reduces impact on environment from ships, aircraft, etc.;

LED itself is better than a lamp in terms of toxic materials;

The very tiny amount of solid-state electronics involved is encased in epoxy or silicon, and there are no discarded lamps during life of the AtoN light.

LEDs allow the production of small, robust, self-contained AtoN lights;

purchase cost depends on range and features;

Low cost lights are available for low intensity applications; in case of high intensity applications, the initial purchase cost can be higher than incandescent lamp AtoN lights.

powerful cool white LED light sources emit significant blue spectral component presenting elevated level of eye hazard at short distances and causing ‘blue light pollution’ in the atmosphere.

#### Issues that should be considered when purchasing an LED light

it may be complex to select optimum LED AtoN lights for applications as there are fairly large differences in drive circuitry, LED array arrangements, etc.;

intensity and colour should be specified over operational temperature range;

horizontal consistency of intensity should be specified;

angle of vertical divergence FWHM and FWTM should be specified (refer to IALA Guideline 1065);

lifetime expectation depends on LED current and junction temperature being in compliance with LED component manufacturer’s specifications;

to optimise the lifetime of an LED AtoN, the electronic equipment and enclosure must be as robust as possible;

required minimum operational lifetime (working hours) of the light;

power source requirements, including power consumption when light is on and off;

the operational effect of the reduction in intensity with time;

temperature effect on intensity and colour;

the effect of variation in voltage on LED intensity;

effect of loss of a single LED or group of LEDs;

LEDs must be current controlled (incandescent lamps are voltage controlled), the intensity can be controlled via pulse width modulation (PWM) if required;

detecting LED failure by automated monitoring equipment can be difficult;

LED AtoN products may draw electric current without any LED producing light.  Therefore, the use of an integrated photometric sensor to sense the light (optical feedback) is recommended since LED may fail to open circuit, short circuit or partial short, failures are difficult to sense.

compliance of LED with the IALA Colours for Signal Lights (IALA Recommendation E-200-1);

ISO/IEC standards for Electromagnetic Interference/Immunity and Electromagnetic Compatibility, or other national standards;

additional lightning protection may be required if not included;

requirements for mechanical vibration/shock.

#### Typical Application

LEDs are being increasingly used in AtoN equipment.  These include LED AtoN lights for buoys, beacons, range lights, sector lights and illuminated ‘dayboards’, which may be encapsulated, sealed units.  The night time nominal range for LED omnidirectional beacons can exceed 18 nautical miles, with greater ranges beginning to appear for directional lights.  Application of LEDs in AtoN lights is described in the IALA Guideline 1048 on LED Technologies and their use in Signal Lights.

## Laser

Laser technology has been trialled for AtoN applications but has not been adopted for operational systems.  A laser is a device that produces a coherent highly collimated beam of monochromatic light.  Several types of laser are available on the market but for marine applications green solid-state (e.g. Nd:YAG) laser and red semiconductor (GaAs) lasers must be considered first because of their high power efficiency and their robust nature.  A laser should only be considered for applications that require high intensity narrow beams.  Electric discharge lasers are also available but tend to be used for high power applications and are therefore not recommended for AtoN because of eye hazard.

### Guiding Approach

a laser aimed directly at mariners requires low power and works in mostly all weather conditions;

a laser using scattering in the atmosphere, requires high power and works only during night and when there are enough scattering particles in the atmosphere;

a concentrated, high power narrow light beam with a highly directional light source is possible;

an approximately 0.1 degree beam angle with a 25mm diameter optic;

behaves as a point source (source of the order of micrometers);

monochromatic (very narrow wavelength bandwidth);

laser power 20-300 mW (Black Board pointer laser uses 1-5 mW).

### Types

lasers are available in different colours;

red semi-conductor devices are recommended and have a life of approximately 10,000 hours;

green solid state lasers are recommended and have a life of approximately 30,000 hours;

gas lasers offer a large variety of colours but are not as efficient or robust.

They have a life of at least 2,000 hours.

### Advantages

good source for high intensity, narrow and accurate beams;

long day time ranges are easily achievable.;

short range systems are easy to install;

high power efficiency, even long day time ranges only require low power, possibly solar;

does not require large optics, 25mm is enough for an angle larger than 0.1 degree;

low maintenance system as laser need only be changed approximately every 3 years;

no colours change with atmospheric propagation;

can be pulsed to further increase power efficiency;

highest electrical efficiency for high intensity narrow beams;

good source for compact system;

very accurate sector cut off allows exact marking of hazards.

### Disadvantages

not fully developed yet for all applications;

systems having more than 3 coded sectors are difficult to construct;

due to the large coherence length and the small diameter of the beam the light tends to show flicker or speckle that may conflict with flash characters;

replacement of laser more expensive than replacement of standard light source;

not cost effective for low range wide-angle applications;

maximum width of narrow beam restricts the use of laser lights to narrow channels;

difficult to have other colours than green and red in circumstances where efficiency and robustness have to be taken into account;

All colours are available where these circumstances do not apply.

at high ambient temperature, of 40o C and above, the system efficiency decreases due to cooling device consumption;

tower needs to be very stable;

training is required for safety reasons;

special provision must be made for eye safety at close range.

### Operational, Environmental and Financial Issues

#### Operational

require one tower only for laser range light projector;

easy to install;

a gun scope is necessary for alignment;

a short-range system can be installed by one person;

low power consumption;

a system designed for a range in the order of 3 km and a divergence of 4.3 degrees when operated around 15ºC has a power consumption of 5W;

At low temperature (approximately - 20ºC) the power consumption is 18W.

a system designed for longer ranges (30 km) would have power consumption in the order of 100W.

#### Environmental

no need for special disposal, arrangements for disposing of electronic components are suitable;

components can be replaced on site;

the system can easily be designed to be safe for observation with the naked eye and with binoculars at the operational range;

the system is environmentally friendly due to very low power requirements.

#### Financial

quantity production and anticipated new, cheaper lasers should decrease the price.

installation costs are very low.

the system can use small, cost efficient sources of power supply, solar in many cases.

example of LASER Light Application: Hay River Artic two colour laser.

A two-colour laser range light (20mW Red and Green lasers) with a 3 km range was designed and installed for the Hay River entrance channel.  The laser lights are powered by solar equipment and the output power varies during night and day, with reduced power at night.

The equipment is easy to install and maintain, was aligned during installation with a gun scope and the lasers have a planned service life of 10,000 hours.

The laser code used in the Hay River project

The front tower of the existing leading lights has been used to install the laser range and, for safety purposes, the laser system is used in parallel with the existing system until its reliability is proven.  The laser system is eye-safe for mariners and the objective of the trial is to evaluate the possibility of replacing the existing system of ranges.

The laser range was installed in June 2000 and has performed up to expectations. Development of the system cost 60,000 US$ and the cost of replicating the system is estimated as 30,000 US$.

## Gas lamps

### Acetylene

The acetylene light has a special place in the history of AtoN, primarily for being the first reliable means of automating lighthouses, buoys and beacons during the earlier part of the 20th century.  The predominant acetylene lighting systems carry the AGA  brand and these originate from the inventions of Gustaf Dalen.  The key inventions included:

•	Production methods for generating, purifying and drying large quantities of acetylene;

•	The design of a transportable cylinder for storing acetylene gas under pressure;

•	The development of a reliable open flame burner system (and low gas consumption pilot burner) that could generate a regular flash rate;

•	The development of a sun valve  to economise on gas consumption  by limiting the operating light to night time conditions.

Acetylene lighting technology was further enhanced by the development of the Dalen ‘mixer’ that allowed gas and air to be drawn into a chamber and then consumed in an incandescent mantle to produce a brighter light source than the open flame type.  The incandescent mantle could be operated as a flashing source inside a fixed lens or as a continuous source inside a rotating lens.  Related developments included a gas-operated mechanism for rotating a lens and a clockwork powered automatic mantle changing device.

### Propane and Butane

Propane and butane gas have been used as an alternative fuel to acetylene.  The lighting equipment typically uses a mantle burner similar to the Dalen design.

Propane is recommended for regions where temperatures lower than 0°C occur.  Butane liquefies at temperatures lower than -0.5°C in the normal atmosphere and the flame is extinguished.

### Advantages

proven technology;

gas equipment is less likely to be vandalized or stolen than other types of light sources;

mechanical components have a long life.

### Disadvantages

high service and maintenance costs;

limited suppliers / expensive;

low light intensity;

safety (explosion hazard).

### Filter Information

yellow Plexiglas 1989	70% (for gas).

### Typical Application

buoy light.

# SUMMARY AND COMPARISON OF LIGHT SOURCES

A summary of the light sources described in the section of the Guidelines and a comparison of their attributes is given in Table 1.

Measurement results of a comparison of the angular intensity profile of a 12V 100W capsule halogen lamp and a 10W LED bare chip are provided in Appendix 1.

Light Source Summary and Comparison

|  | Usual AtoN use | Cost of light Source (USD) | Lifetime (hours) | Robust-ness | Maximum Input Power (Watt) | Lumen pr. Watt | Emission Geometry | Colour Spectrum | Flashable | Safety and Other issues |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Filament Lamp | All Round | 5-20 | 300-2,000 | Low | 3,000 | up to 16 | Spherical | Broadband | YES | Hot envelope Glass may break. |
| Tungsten Halogen Lamp | All Round | 10-20 | Up to 4,000 | Medium | 3,000 | Up to 25 | Spherical | Broadband | YES | Very hot envelope Glass may break Fingerprints on envelope may cause failure. |
| Metal Halide Lamp | Rotating Optics and medium range Range Lights | 30-100 | 6000-20,000 | Low (vibration) | 10,000 | Up to 120 | Spherical | Strong spectral lines and UV | NO | Hot envelope. Glass may break Arc tube can explode. UV Hazard |
| Lasers | Range Lights | 1,000 | 10,000 | Medium | 20 | ?? | Narrow Beam | Monochromatic | YES | May cause damage to the eyes. |
| Low Power LED | All Round except long range | 0,25-1 (Need many 4-600) | >100,000 White >50,000 | High | 250mW | 25 - 50 | Medium to wide Beam | Narrow Bandwidth (50nm) but pcLED broadband | YES | May cause damage to the eyes. |
| High Power LED | All Round | 3 – 100 | May exceed 100,000 | High | > 32 | > 100 | Medium to wide Beam | Narrow Bandwidth (50nm) but pcLED broadband | YES | May cause damage to the eyes. Require proper current conditioning and thermal management. |
| Low Pressure Sodium | Inland waterways & marking structures | 10-20 | >10,000 | Medium | 135 | 150 | Spherical | Monochromatic (yellow) | No | Glass may break |
| Xenon | Sector lights & high range lights | 300 | Up to 3,000 | Medium | 35 – 15K | 40 | Spherical | Broadband | No | Hazardous: UV, explosion risk, very hot envelope, glass hazard. |

# Spectral Distribution of Light Sources

Spectra of Various Light Sources

# LIGHT SOURCE LIFETIME CONSIDERATIONS

## General Considerations

The lifetime of an AtoN light source is considered to have reached the end-of-life stage when one of the following conditions is evident:

light output falls below the luminous intensity level necessary to provide the light signal with its required nominal range (either a critical failure, or smooth degradation due to aging);

changes in spectral parameters rendering the colour of the light incompatible with the requirements of IALA Recommendation E200-1;

changes in timing performance distorting the rhythmic character beyond acceptable limits.

For the classical light sources (incandescent and discharge lamps), the end-of-life event is easily predictable and arrives in the form of a critical failure.  The manufacturers provide various statistical numbers for the lifetime of lamps (e.g. average lifetime, 2% failure lifetime, recommended replacement interval).  The situation is different with LED light sources: in most cases, LEDs are not used in AtoN lights as individual components but supplied enclosed in a complete product ('maintenance free over lifetime').  Very often such LED AtoN lights are fully sealed and there is no way to replace a failed LED component.

When discussing LED based AtoN applications, the lifetime of an LED AtoN light (the enclosure of the light source with electronics and optics) and the lifetime of an individual LED component should be clearly distinguished:

lifetime of an LED component: manufacturers provide estimates for the lifetime of a single LED component under precisely defined conditions in a laboratory, combining the failure statistics and extrapolations based on on-going experiments;

lifetime of an LED based AtoN light: the lifetime of a complete product depends on:

the mechanical design (e.g. heat transfer, ingress protection);

electronic design (e.g. LED arrangement, power supply quality, protection from electrical overstress);

the actual conditions of use (e.g. ambient temperatures, flashing character, duty cycle, daytime use, vibration).

In general, the estimated lifetime of an individual LED component and the actual lifetime of an LED AtoN light can vary significantly (upwards or downwards) depending on particular design and applications.

### Example

When an LED AtoN light is designed with LEDs driven using a current exceeding the limits specified by LED component manufacturer, the AtoN light will undoubtedly encounter premature failure.  When an LED AtoN light is driven with a very low current and is well designed, it may provide failure free operation within specifications for longer than the quoted LED component lifetime.

A slow degradation of luminous intensity due to LED component aging is a dominating failure mode only if the AtoN light is well designed.  Lifetimes of LED components from different manufacturers may differ significantly due to differing technologies used and variations of the quality of the manufacturing process.

An example for the failure of an LED AtoN light due to poor electronics is the use of low quality electrolytic capacitors in the power supply or control system.  The average lifetime of an electrolytic capacitor with non-solid electrolyte is shorter than typical lifetime of properly driven LED components. Besides, that average lifetime varies considerably according to the operating temperature.  Therefore, although the lifetime of other components (including LEDs) is potentially much longer, the average lifetime of an electronic device with such features can be limited considerably when the circuit is operated at high temperatures induced by the operational environment or self-heating.  It is important to select the components of the electronic system properly, in accordance with the expected operating temperatures, so that they will not become a limiting factor of the lifetime of whole equipment.

## LED Light Source Degradation

Modern LED based light sources are expected to demonstrate high reliability and long lifetime, with the dominating failure mode in the form of light output degradation causing luminous intensity to fall below acceptable levels.  While the lifetime of LED components themselves may exceed 100000 hours when used in strict compliance to component manufacturers specifications, actual lifetimes of LED based light signalling products may vary significantly depending on design quality and operational conditions.

LED component manufacturers currently provide information on components with the end-of-life condition when the luminous intensity falls to 70% of its initial level.  In order to account for foreseeable degradation of an LED based AtoN light, it is recommended to procure a product with sufficient spare capacity of luminous intensity to maintain the desired nominal range within the proposed lifetime of the AtoN.

In certain applications, the lifetime of a properly designed LED AtoN light may be limited by factors other than LED component lifetime, which may reach tens of years in a carefully chosen setting.  Aging of plastic lenses due to solar UV radiation, temperatures generated by the AtoN light itself, humidity breaching the enclosure, and mechanical wear are the external factors to be considered.

The lens aging rate depends on the amount of UV radiation and the lens material. The amount of UV radiation incident on a lens is affected by the geographical position and any protective structures (e.g. lantern house).  In the case of range and projector sector lights also the direction (azimuth) of the AtoN light affects the level of exposure to UV radiation.

Mechanical wear of the lenses takes place mainly in the floating AtoN used in ice conditions. Ice rubbing may scratch the lens and contribute significant changes in light intensity and distribution.  Abrasion from wave action or wind-blown sand may be relevant in certain AtoN light applications.

The correcting factors that are considered as necessary must be added up, in order to guarantee that the needed intensity is maintained during the service (useful) life of the AtoN light.

In operation, cooler temperatures will reduce the LED aging process, therefore proper ventilation of lantern rooms is beneficial in minimizing the inner atmosphere heating up due to sunlight and heat radiated from the product itself.

High humidity can lead to a shortening of LED lifetime, but this a significant factor mainly for LED light sources that are not protected from environmental effects.  Cold and hot temperature cycling may adversely affect LED light source lifetime, creating mechanical problems leading to failure of enclosures.  LED AtoN lights need a very robust enclosure to ensure that humidity does not reach the LED chip within the designed lifetime of the AtoN light in field conditions.

## LED Product Lifetime Estimation

Unless optimized for a specific use, actual lifetime of an LED AtoN light will be dependent upon the operational mode of the product as well as upon actual environmental conditions at the site of installation.  The main factor adversely affecting the lifetime of even properly designed LED products is a high temperature environment.  For most LED products, the lifetime is shortest in the case of steady burning mode, and longest in the case of flashing mode for rhythmic characters with long eclipses and low duty cycle pulse width modulation (PWM).  Latest research suggests that LED aging may increase significantly in case of driving the LED chip at higher current densities even in the conditions where the temperature of the LED chip is maintained within manufacturer recommended range.

In the course of the AtoN light’s life cycle, irreversible decreasing of the luminous intensity of the LEDs occurs depending on operational time and conditions, while only the time of the powered state of LEDs causes the internal degradation (except for UV radiation induced changes in lenses).  As a result, the efficiency of the light may decrease below an acceptable level long before a critical failure occurs.  Such a condition is defined by LED component manufacturers as a lumen maintenance failure.  This is expressed with reference to a threshold that describes the LED’s light output compared to its initial value, below which the LED is considered faulty.  In this case, any unit with more than 30% light output degradation (i.e. less than 70% of initial output) is classified as a lumen maintenance failure.  Failure rates of the components are expressed in terms of the time period by which 50% of LEDs are expected to degrade.

It is extremely complicated to estimate realistic LED product lifetimes without knowing the exact temperature profile of the actual site of application, therefore many manufacturers are unwilling to provide LED product lifetimes.  When such lifetimes are provided, they are typically calculated for a particular mode of operation focusing on the information released by LED component manufacturers.  Such lifetimes usually do not refer to a time to a critical failure but to a lumen maintenance failure.  To obtain a realistic estimate for any AtoN light lifetime (product lifetime), if provided by the manufacturer, needs to be combined with a factor derived from the ambient temperature profile of a particular site of application (lantern room, space under a buoy cupola, etc.).

## Performance Measurement of LED Light Sources in Field Conditions

There are several ways of monitoring the performance of LED light sources in the field or in situ.  The intensity of the AtoN light can be measured using the field light measurement techniques described in IALA Recommendation E-200-3 ‘On Marine Signal Lights – Measurement’.  Such measurements can be carried out at regular intervals to assess the degradation of intensity over time.  During the measurement process, the rhythmic character can also be checked.  A spectroradiometer could be used in situ to measure the colour of the AtoN light.  However, there are two problems with this approach: a field light measurement can be expensive to carry out especially if the AtoN light is offshore; and the intensity is usually only measured in one direction.

The relative luminous intensity of a light can be monitored in situ by measuring a sample of the light output with a photodiode or photo-dependent resistor coupled to a photoamplifier; the output of the photoamplifier being directly proportional to the light intensity.  The monitor output can be set to raise an alarm if it drops below a predetermined level.  However, once again, it is difficult to measure the AtoN light intensity in all directions and the output of the LED(s) monitored may not be representative of the total light output.  Monitoring the rhythmic character is more complex; although it is a relatively simple matter to monitor whether the light is flashing and this can guard against a ‘fixed light’ failure mode.

Some LED AtoN lights have integral intensity monitoring and the monitor output can be used either to plot the intensity degradation over time or to raise an alarm if it drops below a predetermined level.  Intelligent systems can adjust the intensity level as it degrades to maintain a steady level.

# SPECIAL APPLICATIONS OF LIGHT SOURCES

## Light Poles and Light Pipes

Light poles and pipes may be used to give a conspicuous shape or to enhance the outline of a building or structure.  Different shapes and colours can improve the conspicuity of an AtoN in areas where background lighting is a problem.  Such devices can be realised with LED technology, with fluorescent lamps or with prismatic diffusion.

Conspicuity is increased with the size of the light signal viewed and decreased with density of background lighting.  High-density backgrounds cause less reduction in conspicuity of ‘extended’ signals than ‘point source’ signals.  Whereas, flashing enhances conspicuity of ‘point source’ signals more than extended ones.

Light Pipes for Swedish Maritime Administration (0.4W per lamp)

Example of Application of Light Pipes

Light Pole Detail

Light Pole in Operation

Prismatic Diffusion Technique

## Signage

Light sources may be used to illuminate signs that give information to the mariner or navigator.  The requirement is usually that the light source illuminates a recognisable area rather than a point source.  Several point sources could be used together to form shapes or extended light sources could be used to provide illuminated areas.  A typical use of such signage is shown below, where the sides of the triangles illuminated indicate the movement of vessels further along an inland waterway.

Mandatory warning sign showing vessel traffic

  - Indirect Illumination

Light sources may be used to illuminate special structures erected on the shore side or deployed in the sea bed with the purpose of visual light signalling using large illuminated surfaces.  In such cases of indirect illumination, the structure itself becomes a source of the reflected light for the mariner, while the properties and condition of the reflecting surface become important factors influencing the properties of the resultant navigational signal.  Considerations for illumination of objects are provided in the IALA Guideline No. 1061 on Light Applications - Illumination of Structures.

‘HIB’ navigation marks

# DEFINITIONS

The definition of terms used in this Guideline can be found in the International Dictionary of Marine Aids to Navigation (IALA Dictionary) at http://www.iala-aism.org/wiki/dictionary.

It is suggested that the definitions of correlated colour temperature, organo-metallic vapour phase epitaxy, colour temperature, colour purity, colour rendition, die chip are relevant to this guideline.

# ACRONYMS

AlInGaP	Aluminium Indium Gallium Phosphide

AtoN	Aid(s) to Navigation

CCT	Correlated Colour Temperature

cd	candela

CIE	Commission Internationale de l'Eclairage (International Commission on Illumination)

DC	Direct Current

ESD	Electrostatic discharge

FWHM	Full width at half maximum

FWTM	Full width at tenth maximum

GaAs	Gallium Arsenide

GaP	Gallium Phosphide

h	hour(s)

HIB	High speed craft indirect lighting (Translated)

HID	High Intensity Discharge

IALA	International Association of Marine Aids to Navigation and Lighthouse Authorities – AISM

IAVG	Average current

IEC	International Electrotechnical Commission

IESNA	Illuminating Engineering Society of North America

InGaN	Indium Gallium Nitride

ISO	International Standardization Organisation

km	kilometre

LED	Light-Emitting Diode

Lm	lumen

Nd:YAG	Neodymium-doped Yttrium Aluminium Garnet

nm	nanometre(s) (wavelength

NY	New York

OLED	Organic LED

OMVPE	Organometallic vapor phase epitaxy

PC	Personal Computer

pcLED	Phosphor Converted LED

PWM	Pulse Width Modulation

RFI	Radio Frequency Interference

RGB	Red Green Blue

TIP	Truncated-Inverted-Pyramid

USA	United States of America

UV	Ultra Violet (light) (10 – 380 nm)

vs	versus

W	watt

C	Celsius

K	Kelvin

# REFERENCES

IALA Recommendation E-200-1 On Marine Signal Lights – Colours.  12/2008.

IALA Recommendation E-200-2 On Marine Signal Lights - Calculation, Definition and Notation of Luminous Range. 12/2008.

IALA Recommendation E-200-3 On Marine Signal Lights – Measurement.  12/2008.

IALA Recommendation E-200-4 On Marine Signal Lights - Determination and Calculation of Effective Intensity.  12/2008.

IALA Guideline No. 1042 on Power Sources for Aids to Navigation.

IALA Guideline No. 1061 on Light Applications - Illumination of Structures.

IALA Guideline No. 1048 on LED Technologies and their use in Signal Lights.

Recommendation E-112, for leading lights.  May 1998.

IALA Guideline 1012 for the protection of lighthouses and Aids to Navigation against damage from lightning.  December 2000.

Survey of Laser Range Light Developments, INO 02-5455 LR RFI N/A February 2003.

Conspicuity of Aids-to-Navigation: Extended Light Sources, B. Mandler USCG, 12th IALA Conference, 1990.

Report on the Use of Extended Light Source – Light Pole, S. Kurin, Swedish Maritime Administration, April 2007.

Test Report – Photovoltaic Floodlighting, S. Kurin, Swedish Maritime Administration, November 2001.

IESNA Lighting Handbook Reference, Volume 1981.

IALA Guideline 1065 on Aids to Navigation Signal Light Beam Vertical Divergence

COMPARISON OF THE ANGULAR INTENSITY PROFILE OF A 12V 100W CAPSULE HALOGEN LAMP AND A 10W LED BARE CHIP

Polar Plot

Cartesian Plot

Measurements were conducted at the Trinity House light range.

FURTHER INFORMATION ON LED LIGHT SOURCES

# LED TYPES

There are many types of LEDs, differing in packaging methods, but sharing the same basic technology.  LEDs can be generally divided into two groups by power: low power and high power.

## Low power LEDs

Low power LEDs (sometimes called indicator LEDs) often are used for direct viewing in arrays in signs and signal lights.  The most popular are the 5 mm precision optical performance LED and the Super Flux LED.

### T-1 3/4 or 5 mm LED

The 5 mm package is one of two original packaging configurations adopted from the incandescent lamp industry designated, T-1 and T-1 ¾. (the number indicates lamp diameter in 1/8ths of an inch).  Thus, the T-1 ¾ has a diameter of approximately 5mm.

Details of the 5mm lamp are shown in Figure 1.  The cathode lead, which mounts a small reflecting cup holding the LED chip, is used both, for electrical path and heat extraction.  The upper surface serves as a lens, projecting light into the forward direction.  This package is popular due to low cost and ease of incorporation into an array for signage or other high flux applications.

T-1 3/4 or 5 mm LED

(Source: LumiLeds)

### Superflux LEDs

The Superflux LED package is constructed using double lead frames for reduced thermal resistance and improved ruggedness.  Improved heat removal, along with a larger die area, permits a power rating of twice that of the 5mm device.  Light is projected through a 3mm diameter convex lens on the top surface of the package.

Superflux LED

## High power LEDs

High power or illumination LEDs are used to illuminate other objects or as high intensity, concentrated light sources.

The high power LED, Figure 3, employs a large metal slug for heat conduction with electrical leads isolated from the thermal path.  This structure dramatically improves heat transfer, supporting greater die size and increased operating current.

High Power LED

(Source: LumiLeds)

Other improvements include the use of an ultraviolet resistant silicone encapsulant as an optical coupling medium between the die and transparent epoxy outer package, greatly attenuating yellowing of the epoxy.  In addition, the silicone material, being pliable, results in reduced mechanical and thermal stresses acting upon the die structure and electrical leads.  Improved phosphor conformance coating provides greater uniformity in correlated colour temperature (CCT) for white LEDs.  These advances have virtually eliminated severe lumen depreciation and colour variation, which are notorious in 5 mm LEDs.

## Colour

The material composition determines emitted light colour.  Variations in alloy composition produce differences in band gap energy and, in turn, different characteristic light colour.  Currently, two classes of materials are used in most high brightness LEDs.

There are currently two main types of LED junction, InGaN and AlInGaP.    New organic types of LEDs (OLED’s) are in development but are not yet suitable for light signalling application.

### AlInGaP material

Longer wavelengths from yellow to infraredare produced by a four alloy structure of aluminium, indium, gallium and phosphorous (AlInGaP).

Hewlett-Packard, after a decade of research, mastered the technique of the complex OMVPE growth process (see definitions) allowing growth of the four-alloy structure on GaAs substrates, leading to commercialization of high brightness red and yellow LEDs in 1990.

### InGaN material

The shorter wavelengths from green to ultraviolet, reaching down to yellow light with phosphor conversion technology are produced by an alloy of indium, gallium nitride (InGaN).  Researchers at Nichia Chemical and Nagoya University perfected the growth process for this alloy structure on a sapphire substrate, completing the full colour range of LEDs.

### White LED source

Currently two approaches are used to create a white LED source.  The first uses colour mixing from an array of red, green and blue dies.  This complex technique requires precise control of the LED driver including a feedback system ensuring colour integrity.  This source type is used mostly in television back lighting.  A second simpler approach uses phosphors deposited onto a blue LED die; achieving acceptable colour rendition.

In early 5mm LEDs, during the encapsulation process, a measured quantity of phosphor and epoxy in slurry form was deposited into the containment cup surrounding a blue die.  This process resulted in variations as large as 700°K in CCT with viewing angle.  (The human eye can detect a change of the order of 50-100°K.)

Colour variation with viewing angle along with colour shift with die temperature and time result in small variation in chromaticity.

## Luminous EFFICACY of LEDs

With a goal of producing a viable high intensity light source for signal lights, LED manufacturers have focused on developments in:

heat extraction;

luminous efficacy;

overall electro-optic efficiency;

cost.

### Heat Extraction

The balance between heat generation and dissipation is a key limitation on power handling capacity.  Manufacturers specify a maximum die temperature consistent with acceptable service lifetime.  A thermal resistance stated in °C/Watt specifies the rise of die temperature above that of the LED mounting contact point or surface.  Maintaining acceptable temperature is process of balancing:

environmental conditions, i.e. ambient temperature from direct sunlight, lantern size vs. power, etc.;

driving power;

heat sink design.

Recent designs have emphasized intimate thermal contact between LED die and external mount.  Thermal resistance has been reduced to between 10-15°C/Watt in high power devices in comparison with 100-150°C/Watt for low power LEDs.

### Luminous efficacy

For all practical purposes, luminous efficacy is determined by internal quantum efficiency and extraction efficiency.

#### Internal quantum efficiency

Internal quantum efficiency depends on several factors such as material impurity, optimisation of crystal layers, crystal growing technique, etc.

AlInGaP (red, orange and yellow) is the most widely understood material;

Consequently, its internal quantum efficiency approaches nearly 100% which means that every electron-hole pair creates a photon.

InGaN (green, blue and white) is less well understood at the present time.

Typical internal quantum efficiency of the green device is 20 to 40% with blue 40 – 60%.

Typical Structure of AlInGaP Die

(Source: Compoundsemiconductor Magazine –.’AlGaInP LEDs break performance barriers’ by Mari  Holcomb, Pat Grillot, Gloria Hfler, M. Krames, Steve Stockman - April 2001.)

#### Extraction efficiency

Once photons are created, it is necessary to transport them out of the die, a process called light extraction.  Extraction efficiency depends on absorption within the substrate material, chip geometry, and relative refractive indices of the die, optical coupling material and transparent package.

Boundary reflections.

The refractive index of the die material is extremely large in comparison with that outside of the die.  This large discontinuity produces strong reflections, trapping the light within the die.  Extraction efficiency is improved by an encapsulant of intermediate refractive index, which acts as a bridge between the die and external medium.

The extraction process for a rectangular solid die with epoxy encapsulant is shown in Figure 5.  Photons within the die are generated and propagate in all directions.  However only a small fraction is able to pass through the boundary within the small cone, with the remainder reflected from the surface.  Outside of the critical cone angle, photons experience total internal reflection and do not contribute to useful light.

Absorption by substrate material (internal absorption).

Photons generated within the die structure must propagate within the die to reach boundary walls.  Substrate materials are chosen with due consideration for minimizing absorption along this path.

Showing the Extraction Process

(Source: Compound Semiconductor:  Article ‘Buried Micro-reflectors Boost Performance of AlGaInP LEDs’ by Stefan Illek.)

Die geometry has a large impact upon extraction efficiency.

To date, the most efficient geometry is the truncated inverted pyramid (Figure 6), which minimizes internal reflections.  To date, the highest extraction efficiency is 55% at 650nm.

AlInGaP/GaP Truncated-Inverted-Pyramid (TIP) Die

(Source : LumiLeds)

## Life and Lumen Depreciation

Generally, a light level reduction over a short time is not noticeable until the level reaches 80% of its initial value (Kryszczuk and Boyce.) Figure 7 shows range reduction in nautical miles versus initial visual range with percent intensity degradation of the light source as a parameter.

IALA, to date, has not addressed this issue or recommended a source replacement scheme for AtoN signal lights based on degradation levels.  Some of the administrations accept a 20% degradation before replacing lantern while IALA Recommendation on the determination of the luminous intensity of a marine aid-to-navigation light quotes 75% as ‘service conditions allowance’ for reduction in intensity (this loss must be budgeted for with spare capacity).

Range Reduction vs. Initial Visual Range in Nautical Miles

Lumen degradation depends upon the following factors:

packaging (discussed above);

Die operating temperature;

drive current;

colour.

### Lumen depreciation of low power LEDs (or 5mm package)

Due to packaging methods discussed in section 3.1.1, the lumen degradation of 5mm LEDs is greater than that of high power LEDs mainly due to poor heat transfer and epoxy yellowing of the package.

Different colour LEDs have different rates of degradation since they are manufactured with different semiconductor materials with different degradation properties.  Figure 8 shows typical lumen degradation of red, green, blue and white indicator type LEDs operating at rated current of 20mA.

Lumen Degradation of Red, Green, Blue and White Indicator LEDs (5mm)

(Data based on literature of Lighting Research Center)

#### White Indicator type (5mm) LED degradation

The primary cause of degradation of white indicator type LEDs is yellowing of the epoxy encapsulant, caused by several factors:

excessive junction temperature;

photodegradation of the epoxy material subjected to short wavelength radiation.

Figure 9 shows typical lumen degradation of white 5mm LEDs for several drive currents.

Degradation of the 5mm LED with different drive currents

(Data based on Literature of Lighting Research Center)

As seen in the figure above, indicator type white LEDs reach 75% lumen maintenance around 2000 hours when operated at rated current (20mA).  Degradation is detailed in the test report entitled ‘Solid-State Lighting: Failure Analysis of White LEDs’ by N. Narendran at Lighting Research Center, Troy, NY.

#### Lumen degradation due to Pulsed Drive

It is common practice for many AtoN light manufacturers to employ a pulsed drive technique in an effort to obtain increased light output.  However, due to declining electro-optic efficiency with increasing drive current, the average light output is less than that produced by a constant current of comparable power.  In addition, increased peak current comes at a price of light output degradation, as shown in Figure 10.

Projected Long Term Light Output Degradation for Pulse Drive Condition with The LED On Time Duty Cycle Adjusted to Provide an Average Current IAvg = 30mA

(Source: Agilent Application Brief I-024)

### Lumen depreciation of high power LEDs

Due to improvements in packaging as described above, manufacturers of high power LEDs claim to retain over 70% average lumen output over 50,000hours.

On Going Test Data From the RPI Lighting Research Center – Troy, NY

#### Lumen Degradation of High Power White LEDs

The use of silicone as the coupling material, along with improved techniques in phosphor coating, enable high power white LEDs to retain excellent light output degradation characteristics as shown in Figure 12.

Ongoing test data from the RPI Lighting Research Center –Troy, NY

## Temperature compensation

The electro-optic efficiency of LEDs is temperature dependent.  In general, output flux decreases with increasing temperature under constant drive current conditions.  This effect is most pronounced for the AlInGaP series, although all colours experience the effect to some degree.

Thus, in order to maintain a constant luminous output over the nominal range of operating temperatures, it is necessary to adjust the drive current, due care being taken to avoid thermal run-away.  In addition, the drive current vs. temperature algorithm must be designed in such a way that maximum junction temperature and maximum drive current limits are not exceeded at high ambient temperature and with large duty factors.

## Important factors to be considered when evaluating a LED light

Based upon information from the above overview, LED light specifiers and end-users should consider the following issues:

luminous intensity should be specified over operational temperature range;

horizontal consistency of intensity should be specified, noting that IALA E-122 recommends the 10th percentile figure of horizontal intensity be quoted;

required angle of vertical divergence and beam location, noting that IALA Recommendation E-122 recommends that vertical divergence be quoted at 50% of maximum intensity;

lifetime expectation depends on correct LED current and junction temperature;

efficiency of beacon is proportional to total luminous flux versus power used;

required working hours of the light;

power source requirements, including power consumption when light is on and off;

the operational effect of the reduction in intensity with time;

temperature effect on peak intensity and temperature compensation to maintain constant luminous output;

voltage effect on intensity, pulse or DC-driven;

effect of loss of a single LED or group of LEDs;

method of control i.e. current controlled, voltage controlled or power controlled, series or parallel;

compliance of LED with the IALA Colours for Signal Lights, noting that colour may change with character exhibited;

CE standard for Electromagnetic Interference/Immunity and Electromagnetic Compatibility, or other national standards;

additional lightning protection may be required if not included;

requirements for mechanical vibration/ shock;

process at the end of useful life:

LED module or array replacement?

discard and replace with new light?

are life and colour performance specifications being met?

## LED Eye Safety

LED eye safety issue has been discussed in many LED manufacturers’ application notes and publications.  Guidelines can be found in IEC document (IEC-825-1).  Due to the intense, concentrated source, it is recommended to avoid direct viewing of LED sources at short distance where possible.

# CURRENT AND EMERGING APPLICATIONS OF LEDS IN AIDS TO NAVIGATION

LEDs are being used in almost all AtoN equipment.  These include LED lanterns for buoys, small beacons, range lights and illuminated ‘day boards’ which may be encased sealed units.  The typical nominal range for LED omni-directional beacons is 1 to 15 nautical miles, with greater ranges beginning to appear for directional lanterns.

## Sector light

Light-emitting diodes revolutionize the light signalling of all areas of traffic at the moment.  For many applications (e.g. traffic lights, braking lights, sea lanterns) solutions have already been worked out in LED technology and used in practice.

The sector lights using the projection principle represent a special signal type of lights that is exclusively used in AtoN.

The development of high power LEDs (typically 1 watt, standard LEDs 0.1 watt) could be useful in the development of LED projectors.

In principle the lamp of a projector could be replaced by LEDs (Figure 13).  However, the use of red and green filters in conjunction with an LED light source would severely reduce the intensity of the red and green sectors.

General principal of a projector

The light gain (emitted luminous flux per electrical power) of white diodes is only comparable with that of incandescent lamps.  The projector in Figure 13 then would correspond to a sector light with a 1 watt incandescent lamp.  It would be a very weak light and unsuitable for most applications.

An improvement is possible by using an own projector for every sector (Figure 14).  The luminous intensity of the coloured sectors could clearly be raised since the colour doesn't arise from filtration any more.

Beam arrangement for 3 projectors

However, this construction has two disadvantages:

Adjustment of the sector limits is complex and difficult.  It is almost impossible to adjust the sectors on-site.  Since the white LEDs are of lower intensity (at same electrical power) in comparison with the coloured, either the power of the coloured diodes must be reduced or a lower intensity white sector must be accepted.

To avoid these disadvantages, the white sector can be made by colour mixture from green and red (Figure 15).  To produce the colour white by so called additive colour mixture, it is necessary to mix the three colours red, green and blue.

Colour addition

The green colour used in the traffic signals is always one bluish green and contains therefore also blue colour shares.  A skilful choice of the luminous intensity of red and green a warm white arises, which fulfils the requirements for white signal lights (Figure 16).

Colour diagram

To create the white sector, let overlap the two coloured sectors.  This can be made by the use of two projectors (Figure 17).

Beam arrangement for 2 projector

In some cases (e.g. at small distances or if the projection lens gets very great) it is advisable to mix the light already in the projector.  A colour factor can be used for this.

A colour factor is a semi permeable mirror which reflects the light or lets through depending on colour.  The colour factor used here is designed to an angle of 45° to reflect red light and to pass blue/green through.  It is put between the frame for the sector limits and the projection optics.  The two light sources are ordered at the colour factor that both rays are brought together again in the projection objective (Figure 18).

Beam arrangement for 1 projector

The following comparison is from the measures between a commercial sector light in incandescent lamp technology and the described experiment construction.

Incandescent lamp technology compared with LED technology

| Parameter | Incandescent lamp technology | LED-technology |
| --- | --- | --- |
| Electrical Power | 5 Watts | 2 Watts |
| Luminous intensity (white) | 1500 cd | 1300 cd |
| Luminous intensity (red) | 300 cd | 650 cd |
| Luminous intensity (green) | 300 cd | 650 cd |

By further improvements at the visual system as well as at the LEDs it is foreseeable that the luminous intensity is gradable (with an increased power consumption) around the factor 10. It then would be possible to replace sector lights with up to 50watts by LEDs with half of the power consumption.

## Traditional Optics

IALA Guideline 1049 on the Use of Modern Light Sources in Tradition Lighthouse Optics provides further information on  applications of LEDs in AtoN lights.

# INTENSITY

LED marine lanterns are sometimes reported as having intense colours and ranges longer than the current IALA calculation method would suggest. Current work by IALA is investigating this.

# CONCLUSION

Recently, several independent institutions have developed programs dedicated to research, testing, and evaluation of LEDs.  Information from LED manufacturers and independent sources enables engineers and end users to gain a better understanding of design and operational requirements.

LED manufacturers have made great strides toward improving efficiencies and power capacities of new generations of high power LEDs.  These improvements render great design opportunities for new AtoN signal lights.  Many high power LEDs fitted with special lenses, offer flexibility in designing light sources for retrofitting in existing marine optics, realizing considerable savings for AtoN signal light end-users.

More research and development has to be done in other issues as vertical divergence, horizontal uniformity, spectral characteristics, measurement of intensity.

# ACRONYMS

AlInGaP	Aluminium, Indium, Gallium and Phosphorous

AtoN	Aid(s) to Navigation

CCT	Correlated Colour Temperature

cd	candela

CIE	Commission Internationale de l'Eclairage (International Commission on Illumination)

DC	Direct Current

ESD	Electrostatic discharge

GaP	Gallium and Phosphorous

IALA	International Association of Marine Aids to Navigation and Lighthouse Authorities - AISM

IAVG	Average current

IEC	International Electrotechnical Commission

IES	Illuminating Engineering Society

InGaN	Indium Gallium Nitride

LED	Light-Emitting Diode

mA	milliamp(s)

mm	millimetre(s)

nm	nanometre(s)

NY	New York

OMVPE	Organometallic vapor phase epitaxy

PC	Personal Computer

pcLED	LED strips

TIP	Truncated-Inverted-Pyramid

UV	Ultra Violet (light) (10 – 380 nm)

vs	versus

°C	degrees Centigrade

°K	degrees Kelvin

# REFERENCES

Optoelectronics/Fiber-Optics Applications Manual, Second Edition – Hewlett-Packard Optoelectronics Division

Bierman, A.  1998. LEDs: From indicators to illumination?

Kryszczuk, K. M. and P. R. Boyce. 2002.  Detection of low light level reduction.  Journal of the Illuminating Engineering Society

Lighting Research Center.  Solid-State Lighting.  Accessed on 28 May 2003 at http:/www.Irc.rpi.edu/programs/solidstate/

N. Narendan, Y. Gu, J.p. Freyssinier, H. Yu, L. Deng – Solid-state lighting : Failure Analysis of white LEDs.  2004

G. Craftford, P. Martin (LUMILEDS) – various LEDs technical presentations (2003 – 2004)

D. Steigerwald, et al . Illumination With Solid State Lighting Technology. IEEE Journal on Selected Topics in Quantum Electronics, Vol. 8, No. 2, March/April. 2002

Lumileds : Application Brief AB-12.  Custom Luxeon Design Guide

Agilent :  Application Brief I-024.  Pulse Operating Ranges for AlInGap LEDs vs. Projected Long Term Light Output Performance

Agilent : Application Note 1109.  LED Eye Safety

IES Lighting Handbook – Reference Volume

Mari Holcomb, Pat Grillot, Gloria Hfler, M. Krames, Steve Stockman – AlGaInP LEDs break performance barriers (Cover Story) – Compoundsemiconductor Magazine, April 2001.

Traffic Technologies Centre in Koblenz/Germany (http://www.wsv.de/fvt/lichtte1/led_sektorenleitfeuer/led_sektorenleitfeuer_02.html)

IALA Recommendation E-122 on photometry

IALA Guideline 1043on Light Sources

IALA Guideline 1041 on Sector Lights

IALA Conference 2002: ‘Well balanced ratio between horizontal luminous intensity and vertical divergence of marine lanterns for light buoys / criteria for the assessment of luminous intensity’, German Federal Waterways Administration, Traffic Technologies Centre Koblenz, Otto Schellhase/ Frank Hermann.

IALA Conference in Hawaii: Paper on buoy light divergence

Research on white LED aging, Lighting Research, New York

Application notes from LumiLED, Agilent

IALA Input paper [EEP6/06i/4] Overview of Current of LED Technologies, P.C. To

IALA NAVGUIDE Ed.5

Tungsten has a melting point of 3656Kelvin (≡ 3383C)

The location of the filament high up in the envelope compounds the rate of deterioration in light output because it is closer to the area that blackens first.

An Osram publication ‘Tungsten Halogen Low Voltage Lamps – Photo Optics’ indicates that generally there are no problems with 5 to 10% reductions in rated voltage and that some modern tungsten halogen lamps can be dimmed without detriment.

IALA Conference 1994:   “New Visual Signals” USCG – reference to Vega Industries XAB 17.

The Swedish AB Gas accumulator company

Gustaf Dalen was awarded the Nobel Prize for Physics in 1912 in recognition of these inventions.

Typically, a steel cylinder filled with a porous mass containing a quantity of acetone that absorbs many times its own volume of acetylene in suspension under a modest pressure of around 20 Bar

The principle of the sunvalve uses the differential expansion between two metal bodies, one polished and the other blackened, to close a gas valve when exposed to daylight

The combination of replacing a continuous flame with a flashing character and the sunvalve achieved a gas savings typical around 80%.