IALA Guideline

Gnnnn

demonstrating the contribution of VTS to the safety of navigation and protection of the environment

Edition 1.0

Date (of approval by Council)

urn:mrn:iala:pub:gnnnn

Revisions to this document are to be noted in the table prior to the issue of a revised document.

| Date | Details | Approval |
| --- | --- | --- |
| <Month Year> | Edition 1 | Council XX |

1.	introduction	4

2.	objective	4

3.	Framework for demonstrating VTS contribution	5

3.1.	Advantages in adopting the framework	6

4.	Methodology	6

4.1.	Hypothetical examples	7

4.2.	Contribution – clarifying terminology	8

4.3.	Developing unsafe situations - Definition	9

4.4.	Challenges	9

4.5.	Key principles	10

4.6.	Data requirements	11

4.7.	Time series data structure	12

4.7.1.	Purpose of the Time-Series Dataset	13

4.7.2.	Categorisation of interactions	14

4.8.	Statistical Techniques	15

4.8.1.	Normalisation	15

4.8.2.	Percentage Change	16

4.8.3.	Selecting the Appropriate Technique’s	17

4.8.4.	VTS addressable situations	18

4.8.5.	Evaluation – VTS Attributable	18

4.8.6.	Assessment and Review	19

4.9.	Examples	19

5.	Application of the methodology	20

5.1.	Preparing the dataset	20

5.2.	Generating Graphical Outputs	21

5.3.	Assessment and review	21

5.4.	Interpreting the outputs	21

5.5.	Reporting VTS Contribution	21

6.	Integration, assurance and continuous improvement	22

6.1.	Integration with the Wider Safety System	22

6.2.	Assurance and Regulatory Reporting	22

6.3.	Organisational Learning and Continuous Improvement	22

6.4.	Communication with Stakeholders	23

6.5.	Embedding the Methodology into Routine Practice	23

7.	Definitions	23

7.1.	References	24

ANNEX A	25

EXAMPLE DATASET STRUCTURE AND CODING SCHEMES	25

ANNEX B — WORKED EXAMPLE	27

ANNEX C — DEFINITIONS AND TERMINOLOGY	28

ANNEX D — TEMPLATE FOR REPORTING VTS CONTRIBUTION	29

ANNEX E — TEMPLATE FOR INTERNAL VTS OPERATIONAL REVIEW	31

ANNEX F — EXAMPLE GRAPHS	32

# introduction

Vessel Traffic Services (VTS) are a significant investment to enhance navigational safety and protection of the environment.

Demonstrating their contribution has historically been challenging because:

- Most VTS interactions prevent developing unsafe situations before they occur.  As a result, the impact of VTS is often indirect and difficult to quantify.

- VTS operates within a wider socio-technical system including pilotage, routeing measures, aids to navigation, decision support tools and regulatory frameworks which makes it difficult to isolate the direct influence of VTS from other complementary safety measures.

Quantifying the role of a VTS in mitigating unsafe situations involves measuring incident and casualty reduction factors, analyzing causal interaction, and tracking near-miss escalations.

This Guideline introduces a practical qualitative methodology to demonstrate how VTS interactions in responding to developing unsafe situations, through the issue of advice, warnings and instructions improves the safety of navigation and protection of the environment.

The methodology uses a structured time-series dataset to illustrate VTS contribution over time and to contextualise VTS actions alongside the introduction or evolution of other safety and environmental measures. It is simple, scalable and applicable across all VTS environments, relying largely on information already recorded as part of routine VTS operations.

For a VTS conforming with IALA Guideline G1118 - Marine Casualty / Incident Reporting and Recording, including Near-miss Situations, only minor changes should be required to their processes and procedures for recording situations to embrace the methodology.

# objective

This Guideline provides a qualitative framework for VTS providers to demonstrate how VTS interactions in responding to developing unsafe situations through the issue of advice, warnings and instructions, improves the safety of navigation and protection of the environment.  Specifically, it focuses on:

- How VTS interaction with participating ships contributes to:

- Safety of navigation – for example, mitigating developing unsafe situations and reducing the likelihood of marine casualties, incidents and near‑misses.

- Protection of the environment – for example, reducing the risk of groundings, collisions and pollution events.

- The interrelationship between VTS and associated navigational safety and environmental measures, such as pilotage, decision support tools, routeing measures, aids to navigation and regulatory frameworks.

Note – It is important to recognise that no matter how effective VTS contribution is, or has been, in mitigating developing unsafe situations, incidents and casualties remain possible due to the inherent risks of human error, environmental conditions and mechanical failure.

However, through ongoing assessment of mitigation measures and a commitment to continuous improvement, these risks can continue to be effectively managed.

# Framework for demonstrating VTS contribution

VTS plays a central role in supporting safe and efficient vessel movements, but its contribution is best understood within the wider context of associated safety and environmental measures, such as pilotage, routeing measures, aids to navigation, decision-support tools and regulatory frameworks. Within this integrated system, VTS provides an operational safety layer by monitoring vessel traffic, identifying developing unsafe situations and interacting with vessels through advice, warnings and instructions when required.

Demonstrating VTS contribution is challenging because maritime safety outcomes are influenced by multiple interacting measures and operational conditions. Published studies provide useful insight into the potential safety benefits of VTS, but differences in methodology, assumptions, data availability, waterway characteristics and historical context mean that results are not always directly comparable.

Existing studies and publications often rely on high-level statistical methodologies, which can lead to debate over assumptions, coefficients and interpretation. Therefore, this Guideline focuses on a practical qualitative methodology based on incident reports, VTS interactions and structured time-series data to demonstrate VTS contribution over time.

In this Guideline, the term contribution refers to the role and support that VTS provides within this integrated system. It describes how VTS assists safe and efficient navigation through its interactions with vessels, provision of information and management of traffic. For example, VTS may support ships that deviate from their planned route by providing guidance informed by the VTS traffic image, situational awareness and decision-support tools such as route monitoring, electronic corridors, and defined risk areas and waypoints.

Published information indicates that VTS interaction may contribute between 15% and 100% to the reduction of collisions and groundings. Examples include more recent studies such as the Great Belt VTS and Reef VTS, as well as earlier work undertaken between the 1970s and early 1990s. Although methodologies vary, the published results consistently demonstrate that VTS can make a significant contribution to navigational safety across a wide range of waterway types and operational contexts. Table 1 lists some examples of these studies as a source of reference.

Table1 — Examples of Published Studies on VTS Contribution

| Study | Region / Waterway | Year | Estimated Reduction in Collisions / Groundings | Ref. |
| --- | --- | --- | --- | --- |
| Reef VTS | Great Barrier Reef & Torres Strait | 2025 | Powered groundings reduced to zero | [1] |
| Great Belt VTS | Denmark | 2013 | 60–90% | [2] |
| Asyall (IAMU) | Various | 2002 | 50–60% | [3] |
| Park & Redfern | Various | 1995 | 50–60% | [4] |
| USCG Port Needs Study | USA | 1991 | 0.52–0.68 (collisions), 0.25–0.46 (groundings) | [5] |
| Tokyo Bay Study | Japan | 1990 | 52% | [6] |
| COST 301 | European waters | 1980s | 60% (collisions), 55% (groundings) | [7] |
| Canadian Coast Guard Study | Canada | 1984 | 15–70% (avg. 43.3%) | [8] |
| Dover Strait Research | UK | 1978 | 54.7% | [9] |
| USCG Port Needs Study | USA | 1973 | 30–32% (mixed accidents), 60–65% (collisions) | [10] |

While these studies provide valuable insight into the potential safety benefits of VTS, their findings should be interpreted with care.  Differences in methodology, data availability, waterway characteristics and historical context mean that results are not always directly comparable.

These considerations highlight the importance of complementing quantitative estimates with qualitative evidence drawn from incident reports and operational experience, as outlined in the methodology presented in this Guideline.

## Advantages in adopting the framework

Adopting a structured framework enables a VTS to demonstrate its contribution to navigational safety and environmental protection in a consistent, transparent, and evidence-based manner. It supports long term trend analysis, facilitates organisational and regulatory reporting, and provides a foundation for continuous improvement across different VTS contexts.

There are several advantages for a VTS, including:

- VTS objectives

Demonstrating that operational objectives set for the VTS are being achieved. Guideline G1131 – Setting and Measuring VTS Objectives provides an example:

  - Objective: Capability to interact with a vessel to influence the decision‑making process on board.

  - Measurement: Percentage of interventions that successfully influenced the decision‑making process on board (i.e., interventions that mitigated a developing unsafe situation).

- Audits

Demonstrating that the risks identified when determining the need for the VTS have been alleviated or reduced to an acceptable level (refer to Guideline G1101 – Auditing and Assessing a VTS).

- Evaluation

Demonstrating that regular evaluations are undertaken to ensure the problems identified and defined when implementing the VTS have been alleviated or reduced to an acceptable level (refer to Guideline G1141 – Operational Procedures for Delivering VTS).

- Funding

Supporting requests for ongoing or additional funding from the responsible authority (e.g., government, port authority or board) by clearly demonstrating the contribution of VTS to navigational safety and environmental protection.

The framework can also assist in securing adequate funding when the cost and value of the VTS are challenged, particularly in the context of changing operational requirements, technologies and financial circumstances.

- Benchmarking and communication

Providing a structured way to benchmark and communicate the contribution of VTS and its interrelationship with associated safety and environmental measures such as pilotage, decision-support tools, routeing measures and environmental legislation.

# Methodology

This section describes a methodology used to analyse and demonstrate the contribution of VTS. It outlines the principles, data requirements, and analytical steps needed to produce consistent, meaningful outputs. The methodology is designed to be flexible and adaptable, allowing VTS providers to tailor it to their operational context, available datasets, and specific safety objectives.

To support understanding of how the methodology can be applied, Section 3.3.1 provides simple hypothetical examples using a 30‑year time‑series dataset. These examples illustrate how graphical displays can be generated to show the effect of safety measures and VTS interactions on navigational safety outcomes.

The following subsections outline the components of the methodology, describing the terminology, data requirements, analytical structure, and categorisation principles needed to apply it consistently across different VTS contexts.

## Hypothetical examples

This section provides illustrative examples demonstrating how the methodology can generate graphical displays that show the contribution of VTS together with other safety and environmental measures to mitigate navigational risk and protecting the environment. The examples draw on a hypothetical 30‑year dataset reflecting the staged introduction of associated safety measures (e.g. pilotage, routeing, environmental regulations) and VTS.

These examples demonstrate how VTS providers can apply the methodology and tailor it to their operational context. For instance, demonstrating interactions may be further broken-down by permutations highlighting types of developing unsafe situations, types of interaction and type of situation as described below:

Types of developing unsafe situations, such as:

- ship unsure of its route or position

- ship deviating from the planned route

- ship missing a critical waypoint

- ship requiring guidance to an anchoring position

- ship experiencing defects or deficiencies

- ship at risk of grounding or collision

- emergency response or support to emergency services

Types of interaction:

- warning

- advice

- instruction

- escalation of the above

Type of Situation:

- marine casualty

- incident

- near-miss situations

This level of categorisation enables VTS providers to demonstrate, with clarity and consistency, how VTS contributes to mitigating unsafe situations and improving navigational safety.

It is also important to recognise that VTS interactions frequently prevent developing unsafe situations from escalating into accidents or casualties. These near‑miss events form a critical part of the dataset, as they demonstrate the preventative effect of VTS intervention even when no incident occurs.

Further examples of how the types of developing situations, types of interaction and type of situation can be presented is provided in Annex F.

Example 1 - VTS and Pilotage introduced to replace LPS due to increasing traffic

This example provides a simple example of a growing port and the transition from operating a Local Port Serice to introducing pilotage and VTS as a means to reduce the number of collisions.

The hypothetical dataset demonstrates a clear downward trajectory in collision incidents as measures are adopted and refined.

Example 2 - Safety Measures and VTS Introduction

This example provides a high‑level overview of how the introduction and improvement of safety measures can be represented graphically to show their effect on reducing groundings or collisions.

The hypothetical dataset demonstrates a clear downward trajectory in incidents as measures are adopted and refined:

  - Early years (Y1–Y7) exhibit elevated grounding rates.

  - Mid‑period values decline steadily.

  - By Y19–Y20, groundings fall to zero.

This simple representation illustrates how cumulative safety interventions contribute to long‑term reductions in navigational incidents.

Example 3 – VTS Interaction

This example provides a high‑level overview of how VTS interactions—warnings, advice, and instructions—can be represented to show their role in responding to developing unsafe situations.

In summary, the example highlights:

Groundings and collisions reduce significantly over the observed period, with multiple years achieving zero incidents.

The introduction of VTS corresponds with a marked and sustained reduction in operational risk.

## Contribution – clarifying terminology

This section clarifies how the term ‘contribution’ is used within the methodology, ensuring consistent interpretation of the role and support that VTS provides within an integrated system of safety and environmental measures.

In this Guideline, the term contribution is used to describe the role, input and support that VTS provides within an integrated system of safety and environmental measures. It does not imply comparison with other mechanisms such as pilotage, routeing measures, aids to navigation, policies, procedures or environmental regulations. Each of these mechanisms plays a complementary role, and overall outcomes arise from their combined influence.

In this Guideline, contribution refers specifically to the role and support that VTS provides through its interactions with vessels in response to developing unsafe situations and the management of vessel traffic.  Examples include:

- interacting with vessels to identify and mitigate developing unsafe situations through warnings, advice and instructions;

- providing timely and relevant information on hazards, traffic, weather and port conditions;

- supporting the management of vessel traffic within the VTS area.

Contribution therefore reflects the functions and interventions delivered by VTS and how these support the broader safety and environmental framework.

## Developing unsafe situations - Definition

A developing unsafe situation is a circumstance in which a vessel’s actions, position, condition or intentions indicate an increasing risk to navigational safety or protection of the environment. These situations may arise from vessel behaviour, equipment deficiencies, environmental conditions or operational factors, and may be observed by the VTS or reported by the vessel. A developing unsafe situation does not necessarily involve immediate danger, but represents a trend or condition that, if not mitigated, could lead to a casualty, incident or near miss.

Developing unsafe situations are identified so that VTS can take timely action through the issue of advice, warnings or instructions, consistent with its purpose. The purpose of VTS interaction is to mitigate the situation and contribute to the protection of navigation and the marine environment.

Examples of developing unsafe situations include, but are not limited to:

- ship unsure of its route or position

- ship deviating from the planned route

- ship missing a critical waypoint

- ship requiring guidance to an anchoring position

- ship experiencing defects or deficiencies (e.g., navigation or manoeuvring equipment failure)

- ship at risk of grounding or collision

- emergency response or support to emergency services

A single event may involve multiple related developing unsafe situations. For example, a vessel deviating from its planned route may subsequently be identified as missing a waypoint and later require guidance to avoid a potential grounding. Each situation should be recorded to reflect the full sequence of events and the associated VTS interactions

## Challenges

There are several practical and analytical challenges that may arise when applying the methodology, including data availability, categorisation consistency, and interpretation of near miss events.

Quantifying the contribution of VTS is challenging because maritime safety outcomes are influenced by multiple interacting factors, including traffic patterns, vessel behaviour, environmental conditions, pilotage, routeing measures, aids to navigation and local procedures.

For this reason, there is no single agreed quantitative methodology for isolating and measuring VTS contribution across all operational contexts. The methodology therefore uses structured data, qualitative evidence, incident reports and selected performance indicators to support consistent interpretation. Processes and procedures should take account of the inherent challenges described below.

1. Complexity of Maritime Operations

- Multiple variables: VTS supports vessel safety and environmental protection, each influenced by external factors such as weather, crew competency and local hazards.

- Dynamic conditions: Maritime traffic and operating environments change continuously, making it difficult to isolate the specific contribution of VTS.

2. Difficulties in Defining Success

- Diverse metrics: Contribution may be reflected through reduced incidents, effective interaction or improved situational awareness.

- Indirect outcomes: Many benefits of VTS are preventive and may not be directly observable, limiting measurable data.

3. Data Availability and Quality

- Limited access: High‑quality operational data may not be consistently available, particularly where VTS capability is developing.

- Context gaps: Movement data alone may not capture causal factors, making long‑term correlations difficult.

4. Causality and External Factors

- Attribution challenges: Improvements may result from multiple influences, making it difficult to attribute outcomes solely to VTS.

- Non‑VTS influences: Broader industry changes may overshadow VTS contribution.

5. Lack of Standardised Metrics

- No universal benchmark: There is no globally accepted set of metrics for assessing VTS contribution.

- Regional variation: Differences in local conditions and objectives limit comparability.

6. Human Factors

- Operational variability: VTS contribution often depends on human decision‑making, which varies with training and experience.

- Subjective assessments: Evaluations may rely on subjective interpretation, introducing potential bias.

7. Emerging Technologies and Change

- New capabilities: Technologies such as autonomous vessels may alter VTS roles.

- Uncertain impacts: Effects may not be immediately measurable.

These challenges underline the importance of adopting a practical and consistent methodology that enables VTS centres to demonstrate their contribution despite the inherent complexities of maritime operations.

## Key principles

The methodology is based on several key principles that support consistent recording, interpretation and communication of VTS contribution within an integrated system of safety and environmental measures.

1. System‑based perspective

VTS contribution is interpreted within the broader context of pilotage, routing measures, aids to navigation, port procedures and regulatory frameworks. The methodology does not isolate VTS from these mechanisms or compare their relative influence. Instead, it recognises that overall outcomes arise from the combined effect of multiple complementary measures.

2. Focus on interactions and developing unsafe situations

The methodology centres on VTS interactions that address developing unsafe situations, including warnings, advice and instructions. These interactions provide the most direct insight into how VTS contributes to mitigating casualties, incidents and near misses.

3. Use of qualitative evidence supported by structured data

Incident reports, operator logs and operational experience form the primary evidence base. Structured time‑series data enhances this qualitative evidence by illustrating trends, changes and the introduction or evolution of other safety and environmental measures over time.

4. Recognition of shared outcomes

Reductions in collisions, groundings and near misses cannot be attributed to VTS alone. The methodology therefore focuses on demonstrating VTS contribution rather than assigning sole responsibility for outcomes. This principle avoids misinterpretation and supports accurate communication with stakeholders.

5. Practicality and applicability across all VTS environments

The methodology relies on information already recorded as part of routine VTS operations. It does not require complex statistical analysis or specialised tools, making it suitable for all VTS centres regardless of size, capability or technological maturity.

6. Transparency and consistency in interpretation

Clear definitions, consistent categorisation of interactions and transparent documentation support reliable interpretation of VTS contribution over time. This consistency is essential when communicating findings to authorities, auditors and stakeholders.

7. Adaptability to change

The methodology accommodates changes in VTS capability, technology, procedures and associated safety measures. As new systems or practices are introduced, they can be incorporated into the time‑series dataset to maintain continuity and relevance.

## Data requirements

To support consistent application of the methodology, VTS centres should record core data elements for each situation involving VTS interaction, including incident records, VTS interaction logs and contextual information relating to associated safety measures. These elements provide the minimum information required to describe the circumstances of the event, the vessel involved, the developing unsafe situation and the nature of the VTS contribution.

Key elements to be considered for monitoring, documenting and recording include:

- Date and time – The date and local time at which the situation occurred.

- Source of information – Origin of the report or data, such as VTS operational records, incident/accident/near‑miss reports (refer to Guideline XXX), maritime safety investigation reports or other relevant documentation.

- Vessel identification – Name, call sign and IMO number.

- Vessel type – General vessel classification (e.g., container ship, tanker, bulk carrier, passenger vessel).

- Vessel length – Length overall (LOA).

- Flag State – Flag State of the vessel.

- Area – Where applicable, the VTS sector or locally defined operational area in which the situation occurred.

- Location – Latitude and longitude of the vessel at the time of the situation.

- Pilotage status – Whether the vessel was under pilotage (Yes/No).

- Type of situation – Depending on the role of the VTS, this may include one or more developing unsafe situations such as:

  - ship unsure of its route or position

  - ship deviating from the planned route

  - ship missing a critical waypoint

  - ship requiring guidance to an anchoring position

  - ship experiencing defects or deficiencies (e.g., navigation or manoeuvring equipment failure)

  - ship at risk of grounding or collision

  - emergency response or support to emergency services

A single event may involve multiple related situations. For example, a vessel observed deviating from its route may subsequently be identified as missing a waypoint and later require guidance to avoid a potential grounding.

- Type of interaction – The nature of the VTS intervention (i.e., warning, advice or instruction).

- VTS attributable (Y/N) – Whether the VTS interaction contributed to mitigating the developing unsafe situation.

- Volume of traffic by year – For example, the number of transits.

## Time series data structure

The time series dataset provides a structured means of recording VTS interactions and associated safety and environmental measures over time. Its purpose is to illustrate how VTS contributes to mitigating developing unsafe situations and how this contribution relates to the introduction, evolution or enhancement of other measures within the wider maritime safety system. The dataset should be maintained in chronological order to support clear interpretation of trends and changes.

To ensure consistency, the time‑series dataset should include the following categories of information:

1. VTS Interaction Records

These entries form the core of the dataset and should include the data elements described in Section 3.3.5. Each record should capture:

- the developing unsafe situation(s) observed

- the VTS interaction(s) undertaken (warning, advice or instruction)

- whether the interaction contributed to mitigating the situation (VTS attributable Y/N)

- any escalation or sequence of interactions associated with the event

Recording these elements over time enables VTS centres to identify patterns in vessel behaviour, recurring types of developing unsafe situations and the nature of VTS interventions.

2. Introduction or Evolution of Other Safety and Environmental Measures

To reflect the integrated nature of maritime safety, the dataset should also record the introduction, revision or withdrawal of other measures that may influence navigational safety. Examples include:

- Pilotage arrangements – introduction of compulsory pilotage, expansion of pilotage zones or changes to pilot boarding locations

- Routing measures – implementation or modification of traffic separation schemes, recommended tracks, precautionary areas or designated shipping routes

- Decision Support Tools (DSTs) – introduction of new or enhanced technologies such as traffic prediction tools, risk assessment modules, integrated decision‑support platforms or route‑optimisation systems

- Environmental regulations – establishment or revision of designated shipping areas, environmentally sensitive areas, speed restrictions, emissions control zones or other regulatory measures

- Aids to navigation and hydrographic updates – installation, removal or modification of aids to navigation, changes to buoyage, updates to charts or hydrographic information

These entries should be recorded with their effective date and a brief description of the measure. This allows the dataset to show how the broader safety system evolves and how VTS contribution interacts with these changes.

3. Operational or Procedural Changes within the VTS

Changes within the VTS itself should also be recorded, such as:

- updates to VTS operating procedures

- changes in staffing levels, qualifications or training

- introduction of new communication protocols

- upgrades to surveillance systems (e.g., radar, AIS, CCTV)

- changes in sectorisation or VTS area boundaries

These entries help contextualise changes in VTS contribution over time and support interpretation of trends.

4. External Influences

Where relevant, the dataset may include external factors that could influence vessel behaviour or VTS operations, such as:

- significant weather events

- major port developments

- changes in traffic patterns

- introduction of new shipping services or vessel types

These factors are not part of VTS contribution but may help explain observed trends.

5. Volume of Traffic

The time‑series dataset should include the volume of traffic (e.g., number of transits) to enable the data to be normalised where appropriate (refer Section 3.3.10).

### Purpose of the Time-Series Dataset

This subsection outlines the e role of the time series dataset in demonstrating trends, identifying changes in operational risk, and supporting evidence-based assessments of VTS contribution. The combined dataset enables a VTS to:

- demonstrate VTS contribution over time;

- contextualise VTS interactions within the broader safety system;

- identify trends in developing unsafe situations;

- understand how changes in other measures influence outcomes;

- support audits, evaluations and stakeholder communication;

- provide a structured evidence base for assurance activities.

This structure ensures that VTS contribution is interpreted accurately and consistently, without isolating VTS from the wider system of complementary safety and environmental measures

### Categorisation of interactions

VTS interactions should be categorised to ensure clarity and consistency when analysing their impact on developing unsafe situations.

VTS achieves its purpose through the issue of advice, warnings and instructions, as defined in Standard Marine Communication Phrases and used as message markers in accordance with IMO guidance. These interactions are undertaken in response to developing unsafe situations observed by the VTS or reported by vessels. Categorising interactions consistently supports clear interpretation of VTS contribution within the time‑series dataset.

1. Message Markers

The following message markers should be used when recording VTS interactions:

- Advice – information provided to assist the vessel in making safe and efficient navigational decisions.

- Warning – information provided to alert the vessel to a hazard or developing unsafe situation.

- Instruction – a directive issued by the VTS when necessary to prevent or mitigate a developing unsafe situation, consistent with VTS authority and the IMO VTS Guidelines.

These message markers should be applied consistently to support reliable interpretation of VTS contribution.

2. Types of Developing Unsafe Situations

The IMO VTS Guidelines identify a range of situations in which VTS may interact with vessels to support safe navigation. These situations should be recorded in the dataset and may include one or more of the following:

- Ship unsure of its route or position – vessel reports uncertainty or is observed to be off track without clear situational awareness.

- Ship deviating from the planned route – vessel observed departing from the expected track or routing measure.

- Ship missing a critical waypoint – vessel fails to alter course at a designated waypoint or navigational turning point.

- Ship requiring guidance to an anchoring position – vessel requests or requires assistance in locating or approaching an anchorage.

- Ship experiencing defects or deficiencies – examples include failure of navigation equipment, propulsion or steering systems, or other defects affecting safe navigation.

- Ship at risk of grounding or collision – vessel observed in close proximity to hazards, other vessels or shallow water, or operating in a manner that increases collision or grounding risk.

- Emergency response or support to emergency services – situations involving fire, flooding, medical emergencies, search and rescue or other urgent circumstances requiring coordination.

A single event may involve multiple related situations. For example, a vessel deviating from its planned route may subsequently be identified as missing a waypoint and later require guidance to avoid a potential grounding. Each situation should be recorded to reflect the full sequence of events.

3. Interaction Record

For each developing unsafe situation, the dataset should record:

- the message marker used (advice, warning, instruction)

- the reason for the interaction (type of situation)

- any escalation or sequence of interactions

- whether the interaction contributed to mitigating the situation (VTS attributable Y/N)

This categorisation supports consistent interpretation of VTS contribution and provides a clear link between observed vessel behaviour, VTS actions and the wider safety system.

## Statistical Techniques

Many high-powered statistical methods can be used to analyse maritime safety data and quantify the contribution of VTS. Approaches such as regression modelling, multivariate analysis, and probabilistic risk assessment can provide deep insight but often require specialist expertise, complex tools, and extensive data preparation.

This guideline instead focuses on two simple, practical techniques—normalisation and percentage change - that are easy to implement, transparent to interpret, and suitable for consistent use across all VTS centres. These methods provide clear, repeatable results without the need for advanced analytical capability.

The two techniques used are:

- Normalisation

- Percentage Change

Together, they provide a balanced basis for comparing performance across different periods, traffic volumes, and operational contexts.

### Normalisation

Normalisation adjusts data to a common reference point so that trends can be interpreted independently of changes in vessel traffic volume. It is essential when analysing time-series data where movements vary significantly over time. Without normalisation, increases or decreases in developing unsafe situations may simply reflect changes in traffic rather than changes in vessel behaviour or VTS contribution.

Normalisation is particularly useful when:

- transit volumes vary over

- new services or vessel types are introduced

- port expansions alter traffic density

- comparisons are required across periods or VTS centres

When applying normalisation:

- Select a consistent reference value (e.g., 10,000 transits per year).

- Apply the same method across all years.

- Retain both raw and normalised values for transparency.

- Use normalisation only where traffic volume is relevant to interpretation.

Normalisation does not replace qualitative assessment. It ensures that observed trends—such as reductions in near misses following increased VTS interaction—are interpreted in the correct operational context

Normalising Incident Rates

Normalisation removes the effect of changing traffic volume:

RtGtTt×10000

Where:

- G(t) = [incident type] in year t

- T(t) = transits in year t

- R(t) = [Incidents] per 10,000 transits

This produces a fair, comparable rate over time.

Pros

- Enables long-term trend comparison across decades

- Removes distortion caused by fluctuating movement volumes

- Supports fair comparison across ports and periods

- Reduces debate over raw numbers

- Clarifies the operational impact of interventions

Cons

- Does not show absolute incident counts

- Sensitive to input accuracy

- Can produce misleading values if formulas are misaligned

- Requires explanation for non-technical audiences

- Not suitable when absolute numbers are required

### Percentage Change

Percentage change is a simple but powerful technique for interpreting how a measure evolves over time. While more advanced statistical approaches—such as time-series modelling or regression analysis - can quantify trends with greater complexity, percentage change provides an accessible, transparent way to show direction and magnitude without requiring specialist tools.

It can be applied directly to raw counts or, more effectively, to normalised rates. When used in combination with normalisation, percentage change highlights whether safety performance is improving or deteriorating independently of traffic volume or operational scale.

Percentage change can then be applied to the normalised rate:

Δt=Rt-Rt-1Rt-1×100

Where:

- R(t) = the normalised rate for year t (e.g., incidents per 10,000 transits)

- R(t-1) = the normalised rate for the previous year

- Δ(t) = the percentage improvement or deterioration between the two years (or periods).

This shows the magnitude and direction of improvement or deterioration, independent of traffic volume, raw counts, or service changes.

Pros

- Makes increases and decreases immediately visible

- Effective for “before vs after” comparisons

- Encourages focus on trends rather than absolute values

- Easy to communicate

Cons

- Can exaggerate changes when numbers are small

- May hide operational significance

- Sensitive to unusual baseline years

- Cannot be applied when the baseline value is zero

Summary — Combined Use

Normalisation provides a fair rate. Percentage change shows how that rate changes.

Used together, they provide a clear, defensible analytical picture:

- Groundings decreased significantly after VTS introduction.

- Interactions increased as reporting matured.

- The rate of groundings per movement fell sharply and remained low.

This combined approach supports consistent, transparent interpretation without reliance on raw counts or scale-driven distortions.

### Selecting the Appropriate Technique’s

The following guidance has been prepared to assist determining which technique/s to use in demonstrating the contribution of VTS.

Step 1 — Is traffic volume changing significantly across years?

- Yes → Use Normalisation Adjust incident counts to a common reference volume (e.g., per 10,000 transits).

- No → Raw counts acceptable Traffic volume is stable; normalisation is not required.

Step 2 — Do you need to show improvement or deterioration?

This determines whether the analysis must show whether performance has improved or worsened between two periods, such as before and after a VTS intervention, procedural change, or operational shift.

- Yes → Use Percentage Change Shows direction and magnitude of change over time.

- No → Use Normalisation or raw counts only Trend analysis is not required.

Step 3 — Are both conditions true?

(Traffic volume changes and you need to show trend)

- Yes → Use BOTH Normalisation + Percentage Change

  - Normalisation = fair rate

  - Percentage change = direction + magnitude Together they provide a defensible, scale-independent trend.

- No → Use the single appropriate method Apply only Normalisation or Percentage Change as needed.

Step 4 — Is the baseline value zero?

- Yes → Percentage Change cannot be used Use normalised rate or raw counts instead.

- No → Percentage Change applicable Trend comparison is valid.

### VTS addressable situations

Developing unsafe situations may occur anywhere within the VTS area; however, not all of these situations are VTS addressable. VTS contributes to navigational safety, but it is not a panacea, nor is it responsible for mitigating every unsafe circumstance that arises within its area of responsibility.

A VTS‑addressable situation is one in which:

- the developing unsafe situation is observable by the VTS (through surveillance systems, reports or other means);

- the situation is within the scope of VTS authority, as defined by the IMO VTS Guidelines and local regulations;

- the vessel is able to receive and act upon VTS advice, warnings or instructions; and

- VTS interaction can reasonably influence the vessel’s behaviour or the evolution of the situation.

Situations that fall outside these criteria may still be recorded for contextual awareness, but they are not considered VTS‑addressable for the purpose of demonstrating VTS contribution.

Examples of non‑addressable situations may include:

- incidents occurring outside the VTS area or beyond surveillance coverage;

- situations where the vessel is unable to receive or respond to VTS communications;

- circumstances governed entirely by other authorities (e.g., pilotage decisions, tug operations, terminal instructions);

- events where the unsafe situation develops too rapidly for VTS to intervene;

- situations arising from factors that VTS cannot influence (e.g., sudden mechanical failure with no prior indicators).

Distinguishing between addressable and non‑addressable situations ensures that VTS contribution is interpreted accurately and fairly. It prevents over‑attribution of responsibility to VTS and supports a balanced understanding of how VTS interacts with the wider system of complementary safety and environmental measures.

This principle also reinforces that VTS contribution is demonstrated through interactions, not through the mere presence of developing unsafe situations within the VTS area.

### Evaluation – VTS Attributable

Not all VTS interactions result in a measurable change in vessel behaviour, and many responses are routine. For example, a vessel may temporarily deviate from its track to avoid small craft and then return to the planned route without requiring further intervention. These interactions should still be recorded, but they are not necessarily VTS attributable.

A VTS‑attributable interaction is one in which the VTS action (advice, warning or instruction) contributed to mitigating a developing unsafe situation. The purpose of identifying VTS‑attributable interactions is to demonstrate VTS contribution accurately and consistently within the time‑series dataset.

A VTS interaction is considered attributable when:

- the developing unsafe situation was addressable by VTS (see Section 3.4.2);

- the VTS interaction was timely, relevant and within VTS authority;

- the vessel’s subsequent behaviour indicates that the interaction influenced the outcome;

- the situation was mitigated, stabilised or prevented from escalating into a casualty, incident or near miss; and

- the mitigation cannot reasonably be attributed solely to other measures (e.g., pilotage, tug operations, terminal instructions).

Routine interactions that do not influence the evolution of the situation should be recorded but marked as not attributable. This distinction ensures that the dataset reflects genuine VTS contribution rather than normal operational communications.

Examples of VTS‑attributable interactions include:

- a warning that prompts a vessel to alter course to avoid shallow water;

- advice that assists a vessel experiencing equipment failure to maintain safe navigation;

- an instruction that prevents a close‑quarters situation from escalating into a collision;

- coordinated communication that supports emergency response and reduces risk to the vessel or environment.

Examples of non‑attributable interactions include:

- routine traffic information that does not influence vessel behaviour;

- interactions where the vessel was already correcting its course or situation independently;

- situations resolved entirely by other parties (e.g., pilot, tug master, terminal operator);

- interactions where the unsafe situation developed too rapidly for VTS to influence the outcome.

To achieve maximum benefit, all interactions—whether generated by Decision Support Tools (DSTs), initiated by operators, or requested by vessels—should be recorded and flagged in the dataset as VTS attributable (Y/N). This supports consistent evaluation, transparent reporting and accurate communication with stakeholders.

### Assessment and Review

Consideration should be given to regularly reviewing recorded situations and the assessment made.  For example, it is a standing item at quarterly management meetings to ensure the assessments made in a consistent manner.

## Examples

Example worksheets for establishing and maintaining the time‑series dataset, and for illustrating how information can be presented and communicated, are provided in the following Annexes:

- Annex A – Example Dataset Structure and Coding Schemes

Provides a sample layout for recording VTS interactions, developing unsafe situations, traffic volumes and associated safety measures. Includes coding conventions to support consistency across VTS centres.

- Annex B – Worked Example (One Month of VTS Data)

Demonstrates how the methodology can be applied in practice using a short, representative dataset. Shows how interactions are categorised, how developing unsafe situations are recorded and how VTS‑attributable outcomes are identified.

- Annex C – Definitions and Terminology

Contains standardised definitions used throughout the methodology, supporting consistent interpretation and communication across VTS centres, authorities and stakeholders.

- Annex D – Template for Reporting VTS Contribution

Provides a structured reporting format for summarising VTS contribution over a defined period, including narrative explanation, key metrics, normalised data and contextual information.

- Annex F – Example Graphs

Illustrates how time‑series data may be visualised, including normalised trends, developing unsafe situations, VTS‑attributable interactions and the introduction or evolution of other safety and environmental measures.

These annexes support practical implementation of the methodology and provide examples that can be adapted to local requirements, operational contexts and regulatory frameworks

Example worksheets for establishing / maintain the time series dataset and how information can be presented / communicated is provided in the following Annexes:

- ANNEX A - Example Dataset Structure and Coding Schemes

- ANNEX B - Worked Example (One Month of VTS Data)

- ANNEX C - Definitions and Terminology

- ANNEX D - Template for Reporting VTS Contribution

- ANNEX F - Example Graphs

# Application of the methodology

This section describes how the methodology set out in Section 4 can be applied in practice to demonstrate VTS contribution using graphical outputs, structured reporting formats, and long‑term trend analysis. It outlines the steps required to convert time‑series data into clear visual representations, interpret developing unsafe situations, and communicate findings.

The examples provided illustrate how VTS centres can adapt the methodology using existing operational records and interaction logs. The aim is not to produce complex statistical models, but to present clear, structured evidence showing how VTS contributes to mitigating developing unsafe situations and supporting navigational safety.

## Preparing the dataset

Before graphical outputs can be generated, the time‑series dataset must be assembled using the data elements described in Section 3.3.7. This includes:

- VTS interaction records

- developing unsafe situations

- VTS‑attributable outcomes

- introduction or evolution of associated safety measures

- operational changes within the VTS

- external influences

- traffic volumes

The dataset should be maintained in chronological order to support interpretation of trends and changes over time.

## Generating Graphical Outputs

Graphical outputs provide a clear and accessible means of demonstrating VTS contribution. These may include:

- incident trends (e.g., collisions, groundings, near misses)

- developing unsafe situations over time

- VTS interactions by type (advice, warning, instruction)

- VTS‑attributable interactions

- normalised trends based on traffic volume

- introduction or evolution of associated safety measures

Graphs should be simple, clear and focused on illustrating relationships between VTS interactions, developing unsafe situations and broader safety measures.

## Assessment and review

Consideration should be given

## Interpreting the outputs

Interpretation should focus on:

- identifying long‑term trends

- understanding how VTS interactions influence developing unsafe situations

- recognising the preventative effect of VTS interventions

- contextualising changes in risk with associated safety measures

- distinguishing between VTS‑addressable and non‑addressable situations

- highlighting near‑miss mitigation

Interpretation should avoid attributing outcomes solely to VTS. Instead, it should demonstrate how VTS contributes within an integrated system of complementary measures.

## Reporting VTS Contribution

Reporting should be structured, transparent and consistent. Reports may include:

- narrative explanation of trends

- graphical outputs

- normalised data

- contextual information

- examples of VTS attributable interactions

- description of associated safety measures

- explanation of external influences

The reporting template in Annex D provides a standardised format that can be adapted to local requirements.

# Integration, assurance and continuous improvement

This section explains how VTS centres can use the methodology outputs, including structured datasets, graphical displays and reporting, to support assurance, regulatory reporting, performance review and continuous improvement. By maintaining a structured dataset and applying the methodology consistently, VTS centres can demonstrate their contribution clearly and reliably over time, while strengthening operational performance, stakeholder confidence and compliance with existing organisational processes.

The methodology is not intended to replace existing reporting or investigation processes. Instead, it provides a structured, repeatable means of demonstrating VTS contribution within an integrated system of safety and environmental measures, including the identification of training needs and evaluation of decision-support tools.

## Integration with the Wider Safety System

VTS contribution must be interpreted within the broader context of pilotage, routeing measures, aids to navigation, environmental regulations and port procedures. The methodology supports this integrated perspective by:

- contextualising VTS interactions alongside other safety measures

- recognising shared outcomes and complementary roles

- avoiding over‑attribution or misinterpretation

- supporting balanced communication with authorities and stakeholders

This integrated approach ensures that VTS contribution is understood as part of a system of systems, rather than as an isolated function.

## Assurance and Regulatory Reporting

The methodology supports assurance activities by providing:

- structured datasets

- clear categorisation of interactions

- normalised trends

- graphical outputs

- narrative interpretation

These elements strengthen internal assurance processes and support external audits, regulatory reporting and compliance with IMO VTS Guidelines. The methodology also provides a consistent basis for demonstrating how VTS contributes to mitigating developing unsafe situations and supporting navigational safety.

## Organisational Learning and Continuous Improvement

The methodology supports continuous improvement by enabling VTS centres to:

- identify recurring developing unsafe situations

- evaluate the effectiveness of interactions

- assess operator performance

- identify training needs

- evaluate decision‑support tools

- refine procedures and communication protocols

- understand how changes in associated safety measures influence outcomes

Long‑term trend analysis supports evidence‑based decision‑making and helps VTS centres adapt to evolving operational environments.

## Communication with Stakeholders

Clear reporting formats and graphical outputs improve communication with:

- port authorities

- maritime administrations

- pilots

- terminal operators

- emergency services

- environmental agencies

Structured reporting supports transparent communication and strengthens stakeholder confidence in VTS operations.

## Embedding the Methodology into Routine Practice

To achieve maximum benefit, VTS centres should:

- maintain the dataset continuously

- apply categorisation consistently

- update records when safety measures evolve

- use graphical outputs in internal reviews

- incorporate findings into training and procedures

- use reporting templates for periodic communication

Embedding the methodology ensures that VTS contribution is demonstrated consistently and supports long‑term organisational improvement.

# Definitions

(Section to be included, if required)

## References

(Section needs reordering)

[1] LehnSchiøler, T., Hansen, M.G., Melchild, K., Jensen, T.K., Thomsen, S.R., Glibbery, K.A.K., & Rasmussen, F.M. Collision and Grounding of Ships and Offshore Structures. Taylor & Francis Group, 2013.

[2] IALA e-Bulletin. Reef VTS – 21 Years of Safer Navigation through Interaction with Vessel Traffic. 18 December 2025.

[3] Park, J-S., & Redfern, A. Quantification of VTS Effectiveness. Transactions on the Built Environment, Vol. 11, 1995.

[4] Asyall, E. A Ship-Based Approach to Determine the Effectiveness of VTS Systems in Reducing Vessel Accidents in Congested Waters. 3rd Annual General Assembly, International Association of Maritime Universities (IAMU), 2002.

[5] COST 301 Project. European Community (EU), early 1980s.

[6] Canadian Coast Guard. National Vessel Traffic Service Study. 1984.

[7] USCG. Vessel Traffic Systems: Analysis of Port Needs. 1973.

[8] Dover Strait Research. Channel Navigation Information Service (CNIS) Safety Assessment. 1978.

[9] Safety Assessment of Waterway Network in Tokyo Bay Area. 1990.

[10] USCG. Port Needs Study. 1991.

# ANNEX A

# EXAMPLE DATASET STRUCTURE AND CODING SCHEMES

This Annex provides supporting material for applying the methodology described in Section 3. It includes:

- a recommended dataset format for VTS interactions,

- a separate dataset format for other safety and environmental measures,

- interaction coding scheme,

- developing unsafe situation coding scheme.

These elements support consistent recording and interpretation of VTS contribution across all VTS centres.

Note – these examples could be adopted as standalone simply foy recording VTS interaction or incorporated into processes and procedures associated with G near miss etc and Procedures

A.1 VTS INTERACTION DATASET — EXAMPLE FORMAT

This dataset records all developing unsafe situations and VTS interactions. It forms the core of the methodology.

| Date/Time | Vessel | Type | LOA | Flag | Location | Area | Pilotage | DUS Code | INT Code | VTS Attributable | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2026 03 14 09:42 | MV Example | Container | 280 m | AUS | 27°28.1’S 153°09.4’E | Sector 2 | Yes | DUS 02 | INT 01 | Y | Vessel deviated from planned route; corrected following VTS advice |
| 2026 03 14 11:10 | MT Sample | Tanker | 210 m | SGP | 27°27.5’S 153°10.2’E | Sector 1 | No | DUS 05 | INT 03 | Y | Steering gear alarm reported; VTS issued instruction to reduce speed |
| 2026 03 15 02:20 | FV Coral | Fishing | 35 m | NZL | Sector 3 | No | DUS 07 | INT 02 | N | Emergency response coordinated by SAR; VTS provided warning only |  |

A.2 OTHER MEASURES DATASET — EXAMPLE FORMAT

This dataset records changes to other safety and environmental measures, such as pilotage arrangements, routing measures, DSTs, environmental regulations, and aids to navigation.

It is separate because:

- entries are infrequent,

- changes often have long-term influence,

- the dataset is used primarily for contextual interpretation.

Example Other Measures Dataset

| Effective Date | Measure Type | Description | Area/Sector | Notes |
| --- | --- | --- | --- | --- |
| 2026 03 01 | Routing Measure | New Traffic Separation Scheme implemented | Sector 2 | Introduced following risk assessment |
| 2026 02 20 | Anchorage Procedure | Revised anchorage allocation and approach guidance | Port Limits | Communicated to all users via Notice to Mariners |
| 2026 04 20 | DST Upgrade | Enhanced traffic prediction module deployed | All sectors | Operator training completed |

This dataset is referenced when interpreting trends in the interaction dataset.

A.3 INTERACTION CODING SCHEME

VTS interactions should be coded according to the message markers defined in IMO standard marine communication phrases. These codes reflect the nature of the VTS intervention.

| Code | Interaction Type | Description |
| --- | --- | --- |
| INT 01 | Advice | Information provided to assist safe and efficient navigation. |
| INT 02 | Warning | Information alerting the vessel to a hazard or developing unsafe situation. |
| INT 03 | Instruction | Directive issued by VTS to prevent or mitigate a developing unsafe situation. |

Coding Principles

- Codes must be applied consistently across all events.

- Multiple interactions may occur within a single event; each should be coded.

- Escalation (e.g., advice → warning → instruction) should be recorded sequentially.

- Codes should be used in the time-series dataset and in internal reporting.

A.4 DEVELOPING UNSAFE SITUATION CODING SCHEME

Developing unsafe situations should be coded to support consistent categorisation and analysis. These codes align with the IMO VTS Guidelines and the examples provided in Section 3.3.7.

| Code | Situation Type | Description |
| --- | --- | --- |
| DUS 01 | Route/position uncertainty | Vessel unsure of its route or position. |
| DUS 02 | Route deviation | Vessel deviating from the planned or expected route. |
| DUS 03 | Missed waypoint | Vessel failing to alter course at a critical waypoint. |
| DUS 04 | Anchoring guidance required | Vessel requiring guidance to an anchoring position. |
| DUS 05 | Defects/deficiencies | Navigation, propulsion, steering or manoeuvring equipment failure. |
| DUS 06 | Risk of grounding | Vessel operating in proximity to shallow water or hazards. |
| DUS 07 | Risk of collision | Vessel operating in proximity to other vessels or structures. |
| DUS 08 | Emergency response | Fire, flooding, medical emergency, SAR or other urgent situations requiring coordination. |

Coding Principles

- Codes should reflect the primary developing unsafe situation.

- Secondary or subsequent situations should also be recorded if relevant.

- Codes support trend analysis and identification of recurring patterns.

- Codes should be used consistently across all VTS sectors and operators.

# ANNEX B — WORKED EXAMPLE

This Annex provides a worked example illustrating how a VTS centre may apply the methodology over a one‑month period.

B.1 Example: Summary of VTS Interactions — April 2026

| Effective Date | Measure Type | Description | Area/Sector | Notes |
| --- | --- | --- | --- | --- |
| 2026 04 01 | Pilotage Procedure | Revised pilot boarding arrangements | Sector 2 | Implemented following consultation with pilots |
| 2026 04 10 | Chart Correction | Updated hydrographic information published | Sector 1 | Included new depth contours |
| 2026 04 20 | DST Upgrade | Enhanced traffic prediction module deployed | All sectors | Operator training completed |

Interpretation

Across the month:

- Six developing unsafe situations were recorded.

- Five were mitigated through VTS interaction.

- Three coincided with the introduction or revision of other safety measures.

- Two involved escalations from advice to warning or instruction.

The separate datasets make it easier to:

- analyse VTS interactions independently,

- interpret trends in context,

- identify correlations between system changes and vessel behaviour.

B.2 Other Measures Dataset — April 2026

| Effective Date | Measure Type | Description | Area/Sector | Notes |
| --- | --- | --- | --- | --- |
| 2026 04 01 | Pilotage Procedure | Revised pilot boarding arrangements | Sector 2 | Implemented following consultation with pilots |
| 2026 04 10 | Chart Correction | Updated hydrographic information published | Sector 1 | Included new depth contours |
| 2026 04 20 | DST Upgrade | Enhanced traffic prediction module deployed | All sectors | Operator training completed |

Interpretation

Across the month:

- Six developing unsafe situations were recorded.

- Five were mitigated through VTS interaction.

- Three coincided with changes in other safety measures.

- Two involved escalations from advice to warning or instruction.

The separate datasets make it easier to:

- analyse VTS interactions independently,

- interpret trends in context,

- identify correlations between system changes and vessel behaviour.

# ANNEX C — DEFINITIONS AND TERMINOLOGY

This Annex provides definitions of key terms used throughout the Guideline. These definitions support consistent interpretation and application of the methodology.

Vessel Traffic Service (VTS)

A service implemented by a Competent Authority to improve the safety and efficiency of vessel traffic and to protect the environment.

Contribution

The role, input and support that VTS provides within an integrated system of safety and environmental measures.

Developing Unsafe Situation

A circumstance indicating increasing risk to navigational safety or protection of the environment, requiring mitigation through VTS interaction.

VTS Interaction

An action taken by VTS through the issue of advice, warnings or instructions.

Advice / Warning / Instruction

Standard marine communication message markers used by VTS.

Time-Series Dataset

A chronological record of VTS interactions, developing unsafe situations and associated safety and environmental measures.

VTS Attributable

Indicator identifying whether a VTS interaction contributed to mitigating a developing unsafe situation.

Decision Support Tools (DSTs)

Technologies supporting VTS decision-making.

# ANNEX D — TEMPLATE FOR REPORTING VTS CONTRIBUTION

This Annex provides a template that VTS centres may use when reporting VTS contribution to Competent Authorities, port authorities or other stakeholders.

1. Overview

Reporting period: VTS Centre: Competent Authority: Prepared by: Date:

2. Summary of VTS Interactions

| Metric | Value |
| --- | --- |
| Total developing unsafe situations recorded |  |
| Total VTS interactions |  |
| Situations mitigated through VTS interaction |  |
| Escalations (advice → warning → instruction) |  |
| Emergency responses supported |  |

3. Types of Developing Unsafe Situations

| DUS Code | Description | Count |
| --- | --- | --- |
| DUS 01 | Route/position uncertainty |  |
| DUS 02 | Route deviation |  |
| DUS 03 | Missed waypoint |  |
| DUS 04 | Anchoring guidance required |  |
| DUS 05 | Defects/deficiencies |  |
| DUS 06 | Risk of grounding |  |
| DUS 07 | Risk of collision |  |
| DUS 08 | Emergency response |  |

4. Interaction Summary

| INT Code | Interaction Type | Count |
| --- | --- | --- |
| INT 01 | Advice |  |
| INT 02 | Warning |  |
| INT 03 | Instruction |  |

5. Summary of Other Measures Introduced or Changed

| Effective Date | Measure Type | Description | Area/Sector |
| --- | --- | --- | --- |

6. Interpretation and Commentary

Provide a narrative summary describing:

- key trends observed,

- notable events,

- changes in vessel behaviour,

- influence of other safety measures,

- operational insights,

- any recommendations for improvement.

7. Supporting Data

Attach or reference:

- the VTS Interaction Dataset, and

- the Other Measures Dataset.

## ANNEX E — TEMPLATE FOR INTERNAL VTS OPERATIONAL REVIEW

This Annex provides a template for internal VTS operational review meetings, supporting continuous improvement.

1. Review Details

- Date:

- VTS Centre:

- Participants:

- Reporting Period:

2. Summary of Developing Unsafe Situations

- Total recorded

- Most frequent DUS codes

- Recurring patterns

- Sector‑specific observations

3. Summary of VTS Interactions

- Total interactions

- Interaction types (A/W/I)

- Escalations

- VTS‑attributable outcomes

4. Operational Insights

- Situations requiring escalation

- Situations resolved without VTS intervention

- Communication challenges

- DST performance

5. Training and Competency

- Identified training needs

- Operator performance observations

- Recommendations

- Procedural Review

- Procedures followed

- Deviations

- Recommendations for updates

7. Safety System Context

- Changes in associated safety measures

- External influences

- Port developments

8. Actions and Recommendations

- Immediate actions

- Medium‑term improvements

- Long‑term considerations

## ANNEX F — EXAMPLE GRAPHS

This Annex illustrates how time‑series data may be visualised.

Graph Types <to be developed>

- Developing Unsafe Situations (Annual Trend)

- Shows total DUS events per year

- Highlights long‑term trends

- Interaction Types (Advice/Warning/Instruction)

- Shows distribution of interaction types

- Identifies escalation patterns

- VTS‑Attributable Interactions

- Shows proportion of interactions that mitigated unsafe situations

- Normalised Trends

- Adjusts DUS and interactions for traffic volume

- Supports fair comparison across years

- Introduction of Safety Measures

- Timeline showing pilotage changes, routing measures, DST upgrades, environmental regulations

- Combined Safety System Timeline

- Integrates VTS interactions with other safety measures

- Near‑Miss Mitigation Trends

- Shows reduction in near misses over time

- Demonstrates preventative effect of VTS

Note – this guideline does not address the contribution of VTS to navigational efficiency.  This will be considered in a future guideline.

Refer to IMO Resolution A.1158(32) Guidelines for Vessel Traffic Services (Section 3.1 and 3.2)