Input paper:   VTS60-6.2.7

Input paper for the following Committee(s): 		Purpose of paper:

(Select as appropriate)

☐ ARM	☐  ENG		☐ PAP			☒ Input

☐ ENAV	☒ VTS					☐ Information

Agenda item 			6.2

Author(s)/Submitter(s)			CHINA MSA

# Proposal for a guideline outline to support ESTABLISHMENT OF A NEW TASK FOR REVISING G1110

# Summary

The collaborative development of Artificial Intelligence (AI) and Maritime Autonomous Surface Ships (MASS) is fundamentally transforming the operational environment and capability boundaries of VTS decision support tools. Decision Support Tools (DSTs) will evolve from a VTS duty assistance tool to an intelligent decision-making hub that connects humans, AI systems, and MASS in mixed-traffic environments. With the advancement of the IMO MASS rule framework, G1110 urgently requires systematic revision to adapt to the changes in the new shipping industry.

## Purpose of the document

To propose that the IALA VTS Committee establish a new task to the 2027-2030 work plan to revise Guideline G1110, thereby facilitating the synchronized evolution of the standard framework for decision support tools with technological developments and operational needs.

To provide proposals for revising the outline of Guideline G1110.

## Related documents

Resolution A.1158(32) GUIDELINES FOR VESSEL TRAFFIC SERVICES.

G1110 Use of Decision Support Tools for VTS Personnel.

TC03-10.5.3 New draft Guideline on VTS interaction with MASS.

MSC 111/5 International Code of Safety for Maritime Autonomous Surface Ships.

# Background

The collaborative development of AI and MASS is profoundly reshaping the operational logic and decision support requirements of VTS. Under the combined influence of AI and MASS, decision support tools should not remain confined to their traditional role of providing information integration and alerts for VTS personnel; they must evolve towards "cognitive assistance and collaborative decision-making".

At the 59th session of the VTS Committee, the China MSA, based on the application of the Yangshan VTS Advanced Decision Support System, submitted to the Committee the proposal "proposals for the Work Plan of the VTS Committee Based on the Advanced Decision Support System of Yangshan VTS (VTS59-8.7.5)” and the task registration (VTS59-8.7.5.1). These submissions received significant attention from the delegates and the DETC Committee, and the China MSA was invited to give a dedicated presentation at the 6th meeting of the DETC Committee. The DETC Committee is also actively promoting the collection and dissemination of decision support tools that meet MASS requirements, and has called for a global solicitation of excellent application cases of advanced decision support tools.

Currently, IMO and IALA are at a critical stage of actively advancing MASS rules and guidance documents. IMO implemented the non-mandatory MASS Code in July 2026 and plans to formally implement the mandatory MASS Code in 2032. IALA has restarted the workplan on developing MASS-related guidelines. To ensure that the standard framework for decision support tools evolves in step with technological developments and operational needs, it is necessary to revise G1110 in light of the new developments in the maritime transport scenarios.

# Discussion

The combination of AI and MASS has resulted in a significant synergy effect. The complexity of interaction, the massive amount of data, and the real-time decision-making requirements brought by MASS can only be effectively addressed through AI-driven DST. In the future, DST will not only be a passive centre for collecting and distributing information, but also an active intelligent decision-making hub that integrates AI capabilities and is tailored for MASS.

## The Impact of AI Development on DST

### Transformation of the Operating Logic

The traditional DST essentially responds to past or ongoing situations. However, the AI-driven decision support system can conduct predictive analysis of ship trajectories, traffic flows, and environmental data through deep learning, achieving a transformation from "passive response" to "active pre-control".

### Enhancement of Data Fusion Capability

Traditional DSTs focus on data aggregation, whereas AI has obvious advantages in multi-source heterogeneous data fusion, intelligent perception, and multi-model fusion applications. It can conduct in-depth intelligent analysis based on data collection, thereby enhancing the effectiveness of situational awareness, precise computation, and intelligent management.

### Adjustment of the Human-Machine Collaboration Mode

The introduction of AI reshapes the relationship between VTS personnel and DSTs. AI can undertake tedious monitoring and primary assessment tasks, allowing VTS personnel to focus on more important decision-making tasks and more targeted traffic management, enabling human value to be leveraged at a higher level.

### New Governance Challenges Introduced by AI

The introduction of AI brings new risks such as algorithmic bias, data poisoning, model drift, and adversarial attacks, and also raises deep governance requirements in areas such as data governance, algorithmic accountability, and ethical compliance.

## The Impact of MASS on DST

### Transformation of the Functionality of DST

The emergence and development of MASS have placed new demands on the automation and autonomy of DSTs. DSTs are no longer merely an information integration and early warning platform; instead, they have evolved into functional system that integrates active pre-control, intelligent decision-making, system interaction, and distributed collaboration.

### Adjustment of Interaction Mode

The current G1110 guideline implicitly assumes the basic premises of "ships are manned with at least a minimum crew " and "VTS communicates with the ships via VHF voice". The introduction of MASS has broken this premise. DST must support digital two-way communication with MASS, capable of sending and receiving machine-readable navigation instructions, situational information and intention expressions. The interaction mode between VTS and the ships will also change from "VHF voice communication" to "a combination of VHF digital data exchange".

### Changes in the Subject of Situation Awareness

In a mixed traffic environment, situational awareness is no longer concentrated on the VTS operators, but is distributed among the VTS centre, the MASS remote control centre (ROC), and the MASS onboard autonomous system. The decision support tools need to support the fusion and sharing of situational awareness across systems.

### Reconstruction of Decision-making Authority and Responsibility Boundaries

The introduction of AI and MASS has blurred the issue of "who makes the decision". DSTs and the autonomous navigation system of ships may make decisions such as adjusting the route and avoiding collisions on their own. How to effectively convey the instructions and suggestions from VTS to the decision-making system of the ships and how to ensure the execution of the instructions have become major challenges.

## Proposals for Revising the Outline of G1110

### Update the definition and classification framework of DST

Based on the level of intelligence and the interaction objects, DSTs can be classified into two categories: traditional DSTs and Advanced Decision Support Tools (A-DSTs). Traditional DSTs are based on deterministic rules and thresholds and is suitable for convention ship interactions. A-DSTs can be divided into three types according to their level of intelligence and autonomy: AI-enhanced DSTs, semi-autonomous DSTs, and fully autonomous DSTs. AI-enhanced DSTs integrates technologies such as deep learning, large models and possesses capabilities such as predictive analysis, anomaly detection, intelligent recommendation, digital communication, data exchange, and autonomous ship behaviour understanding. Semi-autonomous DST adds some autonomous interaction and handling capabilities on the basis of AI-enhanced DST. Fully autonomous DST adds full autonomous interaction and handling capabilities on the basis of AI-enhanced DST. Advanced decision support tools can be adapted to the management requirements of mixed traffic.

### Reconstructing the collaborative framework

Under the dual influence of AI and MASS, the DST interaction framework needs to consider human-AI collaboration, human-MASS interaction, AI-MASS interaction, and trust calibration.

Human-AI collaboration should clearly define that the role of the AI system is "assisting decision-making" rather than "replacing decision-making", with VTS personnel retaining the final decision-making authority. For human‑MASS interaction, the DST should act as an intermediary, converting human instructions into digital information that MASS can understand, and converting MASS's "intentions" into situational information that personnel can understand. For AI‑MASS direct interaction, the boundary conditions and human supervision mechanisms for such scenarios should be clearly defined. When AI is involved, a trust calibration mechanism should be established to guide VTS personnel to correctly understand the credibility of AI suggestions, avoiding excessive reliance or complete disregard of AI output.

### Increase the AI-based DST capability requirements for MASS

AI-based DST capabilities include digital interaction, MASS behaviour understanding and prediction, mixed-traffic flow management, remote control support, and MASS type identification and adaptation.

Digital interaction requires that DSTs support digital two-way communication with MASS, capable of sending and receiving machine-readable navigation instructions, situational information, and intent expressions. MASS behaviour understanding and prediction require that AI-driven DSTs can understand and predict the decision-making logic and navigation intent of MASS at different autonomy levels. Mixed-traffic flow management means that, in scenarios where conventional ships, automated ships, and MASS coexist, DSTs can provide VTS personnel with differentiated situational awareness and decision recommendations. Remote control support requires that, in situations where MASS requires remote intervention, DSTs can provide necessary situational information and operational recommendations to remote control centres. MASS type identification and adaptation require that DSTs can automatically identify the type of MASS and accordingly adjust information presentation, interaction protocols, and decision support strategies.

### Add Multi-Source Data Fusion and Data Quality Requirements

DSTs should have the capability to fuse radar, AIS, VHF direction finders, meteorological sensors, and unique data sources (e.g., shipborne sensor data, remote control status information), and clearly define data quality assessment standards. It should ensure that in cases where data is incomplete or inconsistent, DST can accurately assess the data's credibility and provide corresponding prompts.

### Add Performance Evaluation and Testing Requirements for Decision Support Tools

The introduction of AI and MASS has imposed new requirements on the real-time performance, reliability, interoperability, system redundancy and fault tolerance of DST. Before DST is put into use, it should undergo interoperability testing to assess whether the performance requirements and redundancy design requirements in data processing, risk assessment and alarm response are met, ensuring the correctness and consistency of digital communication and data exchange, as well as the availability and effectiveness of the backup system.

### Expansion of the Risk Management Framework

The introduction of AI and MASS has significantly expanded the risk scope of DSTs. The unique risks of AI include algorithm bias, data poisoning, model drift, and adversarial attacks. The interaction risks of MASS include incompatible communication protocols, misinterpretation of intentions, conflicts between autonomous decisions and VTS instructions. The mixed traffic risks mainly manifest in the unpredictability caused by ships of different autonomous levels navigating in the same water area. The system risk management framework should clearly define the backup procedures and manual takeover mechanisms in case of failure of AI systems or MASS communication.

### Revise VTS operator Training Requirements

For the training content for VTS personnel, it is suggested to add new items such as AI literacy, MASS interaction skills, and human-machine collaborative decision-making capabilities. Specifically, AI literacy requires VTS personnel to understand the basic principles, capabilities, and limitations of AI-driven DST. MASS interaction skills require VTS personnel to master the skills for digital interaction with MASS and understand the behavioural characteristics of ships with different autonomous levels. The human-machine collaborative decision-making ability focuses on training VTS personnel on how to effectively evaluate and make decisions when AI suggestions are inconsistent with their own judgments.

# References

Resolution A.1158(32) GUIDELINES FOR VESSEL TRAFFIC SERVICES.

DISCUSSION PAPER FUTURE VTS A LIVING DOCUMENT (Edition 2.2).

G1110 Use of Decision Support Tools for VTS Personnel.

VTS59-12.5.1.5 WP G1110 Use of Decision Support Tools for VTS Personnel.

VTS49-8.2.8 The Application of a VTS Decision Support Tool based on Artificial Intelligence.

MSC 107-5-1-Report of the MSC-LEG-FAL Joint Working Group.

# Action requested of the Committee

The Committee is requested to:

Note the content of this proposal;

Establish a new task to the 2027-2030 work plan to revise Guideline G1110, ensuring that the standard framework for decision support tools evolves in tandem with technological developments and operational needs, and promoting the coordinated development of advanced decision support tools globally.

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain

Footer will automatically populate