Input paper:   VTS60-6.2.9

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM	□  ENG		□  PAP					X   Input

□  ENAV	X  VTS							□  Information

Agenda item 			6.2

Author(s) / Submitter(s)			Shri Dharamveer Shrivastava, Director, DGLL, India

# Recommendation by DGLL for IALA (2023-2027) quadrennial 
work programme for Vessel Traffic Services

# Summary

I, Shri Dharamveer Shrivastava, Director, DGLL posted at Port Blair under the guidance of Shri AK Debata, Dy Director General, VTS, Gandhidham, Gujarat, India and with assistance of Shri Digvijay Singh, Deputy Director and Shri Jayanta Chakraborty, Deputy Director, all having experience in working in VTS intended to prepare a paper related to VTS. Accordingly, discussion was made in detail with all officers on various standards, recommendation, guidelines, model courses, manuals of Vessel Traffic Services (VTS) Committee quadrennial work programme of IALA. The observations and recommendations made after through study of different IALA Recommendations and Guidelines are incorporated in the subsequent paragraphs.

Purpose of the document

After detail study of Publications, Manuals, and based on discussions with other officers and under the guidance, it is decided to share some inputs for more effective ways to enhance the efficiency and operational Capabilities of VTS system. The points may be considered in the meeting and thereby may be implemented in later stage after the due approval.

## Related documents

- IALA Guideline G1118 (marine casualty / incident reporting and recording, including near-miss situations as it relates to a VTS) and G1111 (Preparation of Operational and Technical Performance Requirements for VTS Systems)

- G1111-2 Producing requirements for voice communications

- IALA VTS Manual

# Background

# The scheme of establishment of VTS-GoK (Gulf of Kachchh) under Directorate General of Lighthouses and Lightships, INDIA has been planned to facilitate safe movement of vessels in the area, the safety and protection of environment and the security of the area.

# GoK has a few of the most important ports in Gujarat including three largest ports of the country in terms of volumes.

# Over 70% of India’s crude oil imports are handled here through 11 SBMs

# With the area close to the western border it is of considerable strategi importance

# To enhance the operational capabilities and efficiency, some suggestion has been made for incorporation and amendments to the referred documents

Vessel Traffic Services (VTS), established under SOLAS Chapter V, Regulation 12 and delivered in accordance with IMO Resolution A.1158(32), depend today on software: the platform that fuses radar, AIS and other sensor data into a traffic image, raises alarms, records evidence and supports operator decisions. While IALA guidance — Recommendation R0128 and the G1111 series — assists competent authorities in preparing their own specifications, no IALA instrument defines a normative minimum set of software features. Consequently, "IALA-compliant VTS software" remains an unverifiable, self-declared claim, and the burden of specifying hundreds of safety-critical functions falls unevenly on coastal States, hardest on those establishing their first VTS. This paper reviews the existing framework, identifies the gap, and proposes a minimum feature set — covering the traffic image, sensor fusion, data integrity, alerting and decision support, route monitoring, search and rescue support, recording and replay, vessel information, system availability, and security — each with operational justification, giving particular weight to surface water surveillance and search and rescue. It recommends that IALA should publish this minimum as a guideline, with a model declaration of conformity aligned to G1111-9 acceptance testing, so that compliance claims become verifiable and procurement rests on a common floor of safety functionality.

# Discussion

It is observed that some of the guidelines and recommendations are generalised in nature and thus may be standardised and homogeneous for all the VTS operations. To make the suggestions more fruitful and prudent, a table is prepared with relevant references for better understanding

| Sr No | Reference Document of IALA | Existing Provision | Proposed Revision | Justification for the proposed revision |
| --- | --- | --- | --- | --- |
| 01 | IALA Guideline G1118 (marine casualty / incident reporting and recording, including near-miss situations as it relates to a VTS) and G1111 (Preparation of Operational and Technical Performance Requirements for VTS Systems) | IALA recommends a minimum of 30-day storage period of VTS data as the time-period to allow for the full retrieval of data post-incident. It can be assumed that this requirement is appropriate for all data sets that may be used for incident replay. | A minimum of 180 days storage period of VTS data may be ensured. | (i) As the VTS may also have a role to play in Maritime security (IALA VTS Manual),for future reference and investigation, the recording period needs to be increased. (ii) The recorded data/incidents of past performances may be utilized for training to the newly recruited VTS Operators/Supervisors in order to better understanding/familiarization of the VTS Operational areas which will, in turn enhance the efficiency of the overall performance of the VTS. The team recommends for 180 days minimum in stead of 30 days. |
| 02 | IALA. Guideline G 1111 (Preparation of Operational and Technical Performance Requirements for VTS Systems) | VTS equipment refers to the individual items of software, hardware, communications and sensors, which make up the VTS system. | No standard has been devised by IALA on VTS Software. As the international sea trading is growing significantly, the minimum mandatory facilities/features may be incorporated in the VTS software features thereby standardization of VTS Software is felt necessary. Hence a separate guideline should be prepared. The Software should offer a complete digital platform protected by the highest level of cybersecurity and a strict application certification process for all third-party services and applications. The benefits of the Standardization of Software are: - The Operational Staffs such as VTS Operators, Supervisors may work in a standardized platform which will help for faster adaptability of the VTS system having working experience from various VTS scenario. - The Software developers to have clear and specific guidance to develop the same standardized Software. - Updating of software to become easy and guided. In view of the above, The following suggestions may be incorporated while implementation of new VTS or modification of existing VTS systems :- - Chart View - 3D Perspective View - Auto follow - OpenGL graphics - Generic search feature - Chart Feature View: - Radar Video - Metrological and hydrological readings - Responsibility Areas - Setting up Standard Operating Procedure - Track symbol - Track hover - Track select - Customized dynamic track lists - Route Mesh - Calculation of ETA - Reports and Statistics - Simulator module - Collision warnings - Statistical Data, Reports and MIS software features and specifications | - Chart View: Any number of Chart Views should be able to display at once, at any range, and at any chart position. Chart views should be highly configurable and should be able to save to show or hide any of the features such as the View Control Area, Notifications Area, Ribbon Buttons, Status bar etc. - 3D Perspective View: Real-time VTS Traffic Image. 3D Perspective View must be displayable at any range and at any chart position. The Operators must be able to control the 3D Perspective View’s pitch, latitude, longitude, yaw, and field of view with keyboard and/or mouse functions. - Auto follow: Any Chart View or 3D View should be able to be commanded to auto follow a track. When auto follow is engaged for a specific track, that track should remain in the centre of the view until auto follow is disengaged or the track is lost. - OpenGL graphics: Chart Views and 3D Views should be visualised in operating console using OpenGL, the industrial standard and most widely used and supported 2D and 3D graphics application programming interface (API). As the OpenGL benefits operating console with stable, high quality, and high performance graphics for rendering and visualising charts, video, tracks, and 3D models. - Generic search feature: Operator Client shall have a generic search feature that will search for any vessel name, call sign, MMSI, geo locations (place names, chart objects etc). - Chart Feature View: Electronic navigation charts should contain a large amount of data. In order to limit the amount of information the operator must process, and in order to present an uncluttered traffic situation, the system administrator should able to select chart features and information and organize these into View Classes. Any number of View Classes should be able to created, showing varying combinations of chart data. Operators should be able to easily apply another View Class to a Chart View. - Radar Video: The radar video presented in the Operator Client should be digitised radar information that has been further processed to the Web Map Service (WMS) format by the Radar Video Server. Radar returns should be visualised as clusters of filled, coloured polygons or fragments. The fill colour depends on both the intensity and the age of the digital video fragment. The radar image is redrawn once every scan. The video intensity should be indicated using a set of colours combined with decreasing opacity to visualise intensity levels. The afterglow should be realised using another set of colours and opacity levels, thus providing the possibility for a very smooth and at the same time dynamic presentation. When an echo has more than one radar video source, Operating Console should always display the radar with the highest quality signal at a higher opacity level than the weaker echo, which should results in a cleaner, more easily understood radar video presentation. - Metrological and hydrological readings: These shall be superimposed on top of chart in the Operator Client with the option to open a permanent view that displays sensor readings in gauges or text. - Responsibility Areas: Each Operating Console should be able to configure to cover a specific responsibility area – a unique geographical area in which a single operator is responsible for total traffic surveillance. - Setting up Standard Operating Procedure :It should be possible for an operator to link any drawn objects or vessels with a Standard Operating Procedure (SOP). Either as a path to a local document or an URL to a remote location. This feature gives automatic information to operator whether the vessel is following SOP or not. - Track symbol: The track symbol should be able to differentiate depending on the track data source. AIS and/or info-linked tracks should be able to shown with a unique symbol, while tracks from other sources should be able to visualised with another. - Track hover: Track presentation should be able to change when the mouse hovers over the track. Colour changes and additional information such as history, heading vector, rate of turn vector displayed are possible configurations. - Track select: The presentation of a track that is selected should also be configurable. Additional label data should be able to displayed, a box, circle, or triangle should be able to appear around the symbol, and colours should be able to change as well. - Customised Dynamic track lists : It shall be possible for an operator to create and save customised dynamic track lists. Track list shall be based on any track properties (static and dynamic. E.g: Name, speed, dimensions). - Route Mesh : The system shall be able to support creation of route mesh. The route mesh will indicate possible routes to destinations and their waypoints. - Calculation of ETA : It shall be possible to assign vessels to destination and the system shall calculate ETA’s for the route. - Reports and Statistics : Operating console should be able to set up with links to reports and statistics generated by vessel movement and information received. - Simulator module : System shall contain a simulator module. The simulator module shall be fully integrated and utilise “live” operator clients. This feature is very essential for (OJT) on job training. The simulator instructor shall create scenarios by defining vessels and their routes, as well as other assets, such as buoys, SAR assets, etc. - Collision warnings : shall be dynamically escalating and shall graphically display vessels involved in the close quarter situation and be colour coded to represent the TCPA escalation. - Statistical Data, Reports and MIS software features and specifications: Real-time track data from the VTMS shall be utilised to generate inside/outside area events and to log vessel position, speed and course as part of events, e.g. vessel incidents and contraventions, in the MIS. |
| 3 | IALA. Guideline G 1132 (VTS voice communications and phraseology) Clause 4.2 | Language: English language should be used for all VTS communications with ships and allied services unless use of an alternative language has been agreed. If an alternative language be used, VTS personnel should be mindful that not all participants may understand what is being communicated. | The standardization of the communication language protocol and the minimum educational qualifications for VTS Operators should be established to ensure uniformity, operational efficiency, and the required level of communication proficiency. | The educational system and standards for teaching and learning English vary from country to country. Consequently, the educational qualifications or degrees awarded in one country may not accurately reflect an individual's proficiency in the English language. In view of this, it is proposed that each country be allowed to recruit VTS Operators in accordance with its own educational qualification standards and criteria, ensuring that candidates possess the level of English proficiency required for the role. |

# References

- IALA Guideline G1118 (marine casualty / incident reporting and recording, including near-miss situations as it relates to a VTS) and G1111 (Preparation of Operational and Technical Performance Requirements for VTS Systems)

- IALA VTS Manual

# Action requested of the Committee

The IALA VTS Committee is requested to: Incorporate the recommendations after considering the suggestions made by DGLL team as described in Para 3.

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain