IALA Guideline

Gnnnn

Guidance on the marking of different restrictions areas

Edition x.x

Date (of approval by Council)

urn:mrn:iala:pub:gnnnn

Revisions to this document are to be noted in the table prior to the issue of a revised document.

| Date | Details | Approval |
| --- | --- | --- |

1.	Introduction	5

2.	SCOPE	5

2.1.	OBJECTIVES	5

3.	FIELD OF APPLICATION	5

4.	MARKING OF TRIAL AREAs FOR AUTONOMOUS SHIPS AND VEHICLES	5

4.1.	In general	5

4.1.1.	Characteristics of vehicles and operations	6

4.2.	Types of Trial area	6

4.2.1.	Environmental description	7

4.2.2.	Navigational Considerations	7

4.2.3.	Type of area	7

4.3.	Marking	8

4.3.1.	Marks of trial area	8

4.3.2.	Design and setting for marks of trial area	9

4.3.3.	Information release	9

4.4.	safety mangement	9

4.5.	Risk assessment	10

4.6.	Coordination	10

5.	Marking of ice roads	10

5.1.	In general	10

5.2.	Type of area	11

5.3.	Marking	11

5.4.	Risk assessment	12

5.5.	Coordination	12

6.	Marking of the Recreation zone	13

6.1.	In general	13

6.2.	Type of zones	13

6.3.	Type of events	13

6.4.	Marking	14

6.5.	Risk assessment	14

6.6.	Coordination	14

7.	Marking of PROHIBITED AREAS	15

7.1.	In general	15

7.2.	Type of area	15

7.3.	Marking	15

7.4.	Risk assessment	16

7.5.	Coordination	16

8.	DEFINITIONS	17

9.	abbreviations	17

10.	references	17

- Introduction

IALA recognises the necessity to mark restrictions areas to navigation and has developed this guideline to assist its

members and other Competent Authorities, when they consider the use of Marine Aids to Navigation (AtoN) to

mark and avoid this kind of hazard for mariners.

SCOPE

This guidance is for stakeholders such as competent authorities, lighthouse, port, and other maritime authorities, aids to navigation providers, and the contractors, developers and operators involved about maritime restrictions areas mentioned in the following sections.

  - OBJECTIVES

This Guideline has the following objectives in the context of the requirements for stakeholders:

1 To provide and increase a broad understanding of different kinds of maritime restrictions areas.

2 To strengthen the practice of maritime restrictions areas by providing standardization based on of know-how and best practices.

3 To offer general guidance about the choice of different kinds of maritime restrictions areas.

FIELD OF APPLICATION

The guidance contained in this document applies to different kinds of marine restrictions areas fixed in position temporarily or permanently, which can be extend above or below the surface of the sea and which are obstructions to navigation.

Restricted areas present different characteristics, which have been grouped as follows:

- Section 4: trial areas for autonomous and unmanned/remote controlled ships and vehicles

- Section 5: ice roads

- Section 6: recreation zones, and

- Section 7: prohibited areas

- MARKING OF TRIAL AREAs FOR AUTONOMOUS SHIPS AND VEHICLES

  - In general

This section provides guidance on the marking and navigational safety considerations for areas designated for the testing and experimentation of autonomous and unmanned/remote-controlled ships and vehicles, or other technologies in subsurface, surface (land and water), and aerial environments (MASS UxV).

Such areas are increasingly being established by competent authorities to support innovation and testing activities. Depending on the nature of the operations, restrictions to navigation may be applied within these areas to ensure safety, security, and environmental protection.

Test areas should be established and operated in accordance with applicable national legislation and regulatory frameworks. From a navigational perspective, it is essential that these areas are clearly defined, appropriately marked, and effectively communicated to all maritime users.

    - Characteristics of vehicles and operations

When determining marking and safety measures, competent authorities should consider the general characteristics of the vessels and systems operating within the area, as these may influence navigational risk.

The type of systems or vehicles to be operated within the area must be clearly identified. This includes surface autonomous vehicles (ASVs), unmanned surface vehicles (USVs), autonomous underwater vehicles (AUVs), remotely operated vehicles (ROVs), aerial drones (UAVs), and hybrid or multimodal systems capable of transitioning between domains (designated at MASS UxV in this guideline).

Relevant factors may include: size and manoeuvrability of the vessels; operating speed and patterns of movement; degree of autonomy and level of human oversight; number of vehicles operating simultaneously and presence of support or escort vessels.

These characteristics should be taken into account when assessing the extent of the area, marking requirements, and the need for additional safety measures.

  - Types of Trial area

Trial areas are dedicated to experiments and operational testing of autonomous ships and vehicles, as well as associated sensors and technologies operating on the surface, subsurface, land and aerial environments (MASS UxV).

For the purpose of navigational awareness and marking, these areas may be distinguished according to various criteria, such as:

- Location (offshore, coastal, onshore where applicable);

- Operational domain (surface, subsurface, aerial or combined);

- Operational status [or type] (permanent, temporary or dormant).

Competent authorities should ensure that the location and extent of test areas are determined with due consideration to navigational safety, including proximity to shipping routes, traffic density, and other maritime activities.

Where necessary, specific restrictions or recommendations may be applied within the area based on a risk assessment.

Restrictions and recommendations may relate to factors such as:

- Characteristics of the ships and vehicles involved, including size, performance, and operational capabilities;

- The presence of a support operational vessel;

- Degree of autonomy and level of human oversight;

- The number of simultaneous trials or operations conducted within the area;

- Seasonal and prevailing meteorological and oceanographic conditions;

- Limitations due to restricted visibility or adverse environmental factors];

- Proximity to high-density traffic areas, other restricted zones, recommended routes, or sensitive areas;

- Environmental characteristics and constrains of the trial areas;

- Any other operationally relevant, or safety-related factors identified through risk assessment.

In any case, such restrictions should be clearly stated in the authorizations [or permits] issued by competent Government authorities.

    - Environmental description

Environmental and physical characteristics of the area may influence both the safe conduct of testing activities and the design and effectiveness of AtoN.

Factors to be considered may include:

- Bathymetry and seabed conditions (relevant to AtoN placement);

- Tidal range, currents, and wave conditions;

- Meteorological conditions and visibility;

- Presence of natural or artificial hazards;

- Seasonal or environmental constraints (e.g. ice conditions).

These factors should be taken into account to ensure that marking remains effective and reliable under prevailing conditions, and that risks to navigation are minimized.

Where test areas are located in or near active navigation zones, appropriate mitigation measures should be implemented to ensure the safety of navigation both within and in the vicinity of the area.

    - Navigational Considerations

The establishment and use of test areas should take into account their interaction with existing maritime traffic and navigational arrangements.

Considerations may include:

- Proximity to shipping lanes, recommended routes, and port approaches;

- Traffic density and patterns of vessel movement;

- Presence of fishing activities or other marine uses;

- Potential overlap with other restricted or regulated areas.

To support safe navigation, measures may include:

- Promulgation of Maritime Safety Information, such as Navigational Warnings (NAVWARNs) and Notices to Mariners (NtM);

- Depiction of the area on nautical charts and Electronic Navigational Charts (ENCs);

- Coordination with Vessel Traffic Services (VTS), where available;

- Establishment of recommended routes or precautionary measures, where appropriate.

In all cases, coordination with relevant authorities, including hydrographic offices and VTS centres, should be undertaken to ensure consistency with existing navigational systems and practices.

    - Type of area

Trial areas may be classified according to their operational status, frequency of use and AtoN requirements. Three main categories are defined in this guideline, as follows:

Permanent Areas – These areas are continuously available for testing and demonstration activities. They are typically equipped with established infrastructure such as marine aids to navigation (AtoN), communication systems, and safety monitoring equipment. Permanent areas are continuously available and may be equipped with fixed or long-term AtoN.

Temporary Areas – These areas are designated for testing during specific timeframes or for particular projects. Activation is limited in duration and may require advance notification to relevant maritime or aviation authorities. After testing concludes, the area reverts to normal use. It is recommended that temporary trial areas observe a rest period following each operation to allow recovery of normal navigational and environmental conditions before reactivation. Temporary areas are activated for defined periods and require timely promulgation and, where appropriate, temporary marking.

Dormant Areas – Dormant test zones are pre-approved and inactive but can be rapidly reactivated when required. They provide operational flexibility for future missions or contingency scenarios without the need for full re-authorization procedures. Dormant areas are pre-defined but inactive, and may be reactivated when required, with appropriate notification and marking.

Competent authorities should ensure that the location and extent of test areas are determined with due consideration to navigational safety, including proximity to shipping routes, traffic density, and other maritime activities.

Where necessary, specific restrictions or recommendations may be applied within the area based on a risk assessment.

When defining a test area, the following parameters should be specified:

- Size and Coordinates – Clearly define geographic boundaries (latitude and longitude).

- Type – Indicate whether the area is permanent, temporary, or dormant.

- Date and Time of Activation – For temporary areas, specify the operational window.

- Status – Define the operational status (Open, Restricted, Confidential).

- Navigational Routes and Fairways – Identify existing shipping lanes, fishing zones, or aerial corridors intersecting or adjacent to the test area.

- Traffic Intensity – Assess vessel and aircraft density to evaluate potential conflicts and determine necessary safety measures.

Based on the intensity of navigation and operational risk assessment, it may be necessary to install or activate AtoN, in accordance with IALA (International Organization for Marine Aids to Navigation) standards and be integrated into the local maritime traffic management system.

  - Marking

    - Marks of trial area

The trial area should be clearly defined through an effective marking system to ensure that mariners are aware of their presence and any associated restrictions.

In accordance with R1001 The IALA Maritime Buoyage System, marking may include:

- Physical AtoN such as special marks;

- Lateral mark or cardinal mark, where necessary to protect adjacent navigation routes or when crossing the fairways;

- Light characteristics to enhance visibility;

- Virtual or digital AtoN, such as AIS AtoN or VDES based systems.

Additional visual aids may be used where appropriate to enhance situational awareness.

The choice of marking should take into account:

- The size and location of the area;

- The duration of its activation (permanent or temporary);

- Environmental conditions affecting visibility;

- The level of risk to navigation.

    - Design and setting for marks of trial area

The marking scheme should be designed and setting should comply with relevant IALA standards and regulations and be adapted to the specific conditions of the trial area. The consideration should include:

- Ensuring marks are clearly visible and recognisable to all users. It should be designed and setting to address the needs of various users, including large commercial vessels (e.g. tankers, cargo ships, passenger ferries), small crafts (motorboats, yachts), and the autonomous vehicles.

- Selecting appropriate types, colours, and characteristics in accordance with standards. The color, shape and size of the mark should comply with relevant standards, which to ensure that meaning of the marks can be identify and recognized clearly and accurately.

- Accounting for environmental factors that may affect performance. For different water area scenes, corresponding mark should be set. Considerations include, but are not limited to water resources, waterway location, environmental factors, navigation rules, etc.

- Marks should be kept clear, obvious and easily identifiable to ships. Digital (physically and virtual) marks can be used as substitutes or to enhance the display if the area affected by factors such as environment, weather or light.

- Digital marks must ensure that the displayed information is consistent with the physical entity, and the essence of the information must not be altered due to digitalization. It is also necessary to ensure that digital devices are functioning properly.

- The marks must be categorized according with R0130, so they can be inspected and maintained according to the requirements of different levels. If damaged, they need to be repaired or replaced in time to ensure the clarity and effectiveness of signs.

    - Information release

The Maritime authority shall determine the scope of the trial area based on factors such as the scope of the operation or activity water area, natural environment and traffic conditions, and release it to the public according to the needs (except in cases involving national security or military confidential information). The information announced includes: the size of the area, the date range, the waterway/route, and the traffic volume. If any changes are needed, they shall be re-verified and announced with navigation warning by the maritime administrative authority.

The limits of the test area shall be clearly depicted on nautical charts, paper and Electronic Navigational Charts (ENCs). Additionally, it is possible to mark the corners or boundaries of the test area with virtual Automatic Identification Systems (AISs). Virtual AISs should be published also in ENC. All markings and relevant information in the chart products will be removed when the area is no longer reserved as a test area.

Maritime authority can list out the circumstances under which sea trials may affect the safety of vessel traffic and announce them to the public based on the actual situation of their jurisdiction. The system should also provide contingency in the event of communication loss or loss of control, NAVWARNs must be issued.

  - safety mangement

To support navigational safety, competent authorities should ensure appropriate monitoring and situational awareness within and around the test area.

This may include:

- Monitoring of vessel traffic and potential conflicts;

- Monitoring the status and performance of AtoN;

- Ensuring reliable communication capabilities;

- Providing timely warnings or guidance to vessels when necessary.

Where applicable, coordination with VTS services can enhance safety and situational awareness.

  - Risk assessment

Risk assessments should be conducted to evaluate the impact of test areas on navigation and to determine appropriate mitigation measures. The IALA Risk Management Toolbox is described in IALA Guideline on Risk Management. Assessing maritime risks pertaining to a spatial plan can be done according to the IMO Formal Safety Assessment (FSA) procedure. The IALA risk assessment tools, and simulation can be used in the process

  - Coordination

Effective coordination among relevant stakeholders is essential to ensure the safe establishment and operation of test areas.

The Competent authority will inform the applicant about the area designated for trial, about the progress of the application review and provide all interested parties with information related to ensuring the safety, reliability and environmental friendliness of the trial.

In case of emergency situations, if necessary, the Competent authority, should make changes into the trial program or stops the trial. This is especially important when the tests are conducted in the VTS area or the Mandatory Ship Reporting Systems (MRS) area. The competent organization monitors the readiness for testing in accordance with internal requirements.

While the applicant has the uninterrupted responsibility for the trial, the VTS should be actively involved in all stages of the trial process, depending on the case from application stage to the debriefing meeting. VTS should be provided with relevant data relating to MASS UxV and the typology of trials that are planned to be carried out. VTS, depending on the risk assessment and type of trials, should have the ability to:

- Communicate directly with other «conventional» traffic that may be affected by the trials;

- Supervise the trials;

- Promulgate any information related to the trial/s within the VTS area to «conventional» vessels that may be affected by the trial (information, warnings, instructions, etc.).

MRCC/SAR activities are carried out in accordance with the requirements International Convention for the Safety of Life at Sea (SOLAS), Chapter V, Regulation 7 and International Convention on maritime search and rescue (SAR).

Information of hazards received from any reliable source should be promptly reported by the Responsible organisation to all concerned participants including notification to other interested Responsible organization.

Marking of ice roads

In general

In areas where ice roads are constructed in winter, for example on lakes, rivers and in archipelago areas in the coastal zone for car and truck traffic, it is very important to prevent ice channels for maritime traffic from being established in the same area.

Figure 1. Ice roads in coastal zone. Source: Estonia.

Type of area

Ice roads can be established temporarily on frozen lakes, rivers, and within archipelagos in the coastal zone, in order to allow passenger and goods transportation during winter.

Figure 2. Roadmap for ice roads in archipelagos. Source: Luleå Municipality.

Marking

In accordance with the IALA MBS, special mark AtoN are suitable for the purpose of marking ice roads using physical and/or virtual marking. If needed, physical AtoN can be enhanced with pictograms. The restricted areas for the ice roads will be marked by AtoN at suitable intervals along the borders.

Figure 3. Ice roads in archipelagos (physical AtoN and Pictogram). Source: Swedish Maritime Administration.

Figure 4. Ice roads in archipelagos (Virtual AtoN). Source: Swedish Maritime Administration.

Risk assessment

To avoid hazards and accidents in areas where ice roads are planned to be established, a risk assessment must first be carried out by the competent authority, in order to determine whether any safety-enhancing measures need to be taken relating to sea traffic before opening of ice roads.

Coordination

- Information

The competent maritime authority are responsible for informing mariners of the establishment of ice roads using Notice to Mariners (NtM) and Maritime Safety Information (e.g. navigational warning).

The competent authority should also make regular public announcements on the current status of ice roads through public service announcements (e.g. local radio stations, websites). Any changes must be promptly communicated by the competent authority, to ensure that information provided to the public is up to date and accurate.

- Readiness

The operator of the ice road is responsible for preparing emergency procedures for rescue operations and coordination with the relevant maritime competent authority.

- Monitoring

If VTS monitoring is available within the geographical area, the risk of maritime traffic damaging the ice road when entering the restricted area could be reduced.

- Maintenance

The operator of the ice road is responsible for the inspection and maintenance of AtoN to ensure compliance with any competent authority licencing conditions.

Any failure must be corrected within the specified time for level of service as directed by the competent authority within the ice road operator’s licence.

Marking of the Recreation zone

In general

Recreation zone which to reserve for organizing various types of competitions and events may be established temporary or permanent. Examples include sailing or powerboat races, public demonstrations, or other similar activities that may temporarily influence vessel navigation safety in this area.

To ensure safety of navigation and protection of participants, these zones should be clearly marked and coordinated with the competent authorities.

Figure 5. Swimrun. Source: ÖTILLÖ.

Type of zones

Recreation may be temporary and short-term in nature or organized on a recurring basis. For recurring use, a permanent recreation zone may be designated for continuous or seasonal use. For temporary and short-term use, temporary zone can be designated for this, and it will be restored after the recreation is finished.

Type of events

Recreations may be temporary and short in duration or organized on a regular basis. And also, may involve motorized or non-motorized activities, examples include powerboat or personal watercraft races, sailing regattas, and rowing or canoeing events.

Marking

In accordance with the IALA MBS, special marks are suitable for marking competition and event areas using physical and/or virtual aids which called recreation zone marks.

Figure 6. Physical AtoN sailing Race Course. Source: Swedish Maritime Administration.

Risk assessment

To avoid hazards and accidents, a risk assessment shall be provided by the event organizer or the proponent of a permanent event area. The assessment shall identify potential risks and define appropriate measures to ensure safe navigation and the protection of participants and other waterway users.

Assessing maritime risks pertaining to a spatial plan can be done according to the IMO Formal Safety Assessment (FSA) procedure. The IALA risk assessment tools, and simulation can be used in the process the IALA Risk Management Toolbox is described in IALA Guideline on Risk Management.

Coordination

- Information

Where applicable, the maritime competent authority should issue information on recreation through a NAVWARN or via the Vessel Traffic Service (VTS) to inform mariners. Permanent event areas shall, where appropriate, be marked on nautical charts.

- Readiness

The recreation organizer is responsible for preparing emergency procedures for rescue operations and coordination with the relevant maritime competent authority.

- Monitoring

- VTS information. If VTS monitoring is available within the geographical area, the risk of accidentally entering the recreation zone could be reduced.

- safety boats arranged by the recreation organizer can be equipped at the boundary of the zone to monitor and deal with emergencies.

- Maintenance

The event organizer or permanent area operator is responsible for the inspection and maintenance of recreation zone marks ensuring compliance with the requirements set by the competent authority.

- Marking of PROHIBITED AREAS

  - In general

Prohibited areas at sea can vary depending on purpose of the restriction and location. This may have an impact on sea traffic's room for manoeuvre, and freedom of navigation, in for example a fairway or route.

Restrictions can generally be temporary, or permanent, depending on the need of action.

  - Type of area

Prohibited areas in this regard may be nature reserves, bird protection, freshwater intakes, work, dredging, harbour, training, live fire exercise, and environmentally sensitive sea areas (ESSA).

Figure 7. Physical AtoN bird sanctuary. Source: Swedish Maritime Administration.

Figure 8. Physical AtoN freshwater intakes. Source: Swedish Maritime Administration.

  - Marking

In accordance with the IALA MBS, special mark AtoN are suitable for the purpose of marking prohibited areas using physical and/or virtual marking. If needed, physical AtoN can be enhanced with pictograms. The prohibited area should be marked by AtoN at suitable intervals along the borders.

Figure 10. Physical AtoN and Pictogram. Source: Swedish Transport Agency.

Figure 11. MAtoN by physical and virtual AtoN. Source: Swedish Transport Agency.

Figure 12. Virtual AtoN military exercise area. Source: Swedish Maritime Administration.

  - Risk assessment

To avoid hazards and accidents in areas where prohibited areas are planned to be established, a risk assessment must first be carried out by the competent authority, in order to determine whether any risk mitigation measures need to be taken, relating to marine traffic, before implementing the prohibited area.

  - Coordination

- Information

The competent maritime authority are responsible for informing mariners of the establishment of prohibited areas using Notice to Mariner (NtM) and Maritime Safety Information (e.g. navigational warning).

- Readiness

The organisation operating the prohibited area is responsible for preparing necessary procedures for search and rescue operations in the area and coordinating these with the relevant maritime competent authority.

- Monitoring

If VTS monitoring is available within the prohibited area, the risks to marine traffic in the restricted area could be reduced.

- Maintenance

The organisation operating the prohibited area is responsible for the inspection and maintenance of any associated AtoN to ensure compliance with any competent authority licencing conditions.

Any failure to the AtoN must be corrected within the specified time for level of service as directed by the competent authority within the operating authorities licences/permits.

DEFINITIONS

The definitions of terms used in this Guideline can be found in the International Dictionary of Marine Aids to Navigation (IALA Dictionary) at https://www.iala.int/wiki/dictionary/index.php/Main_Page and were checked as correct at the time of going to print. Where conflict arises, the IALA Dictionary should be considered as the authoritative source of definitions used in IALA documents.

abbreviations

AIS	Automatic Identification System

ASV	Surface Autonomous Vehicles

AtoN	Marine Aid(s) to Navigation

AUV	Autonomous Underwater Vehicles

EEZ	Exclusive Economic Zone

ENC	Electronic Navigational Chart

ESSA	Environmentally Sensitive Sea Areas

FSA	Formal Safety Assessment

IMO	International Maritime Organization

MASS	Maritime Autonomous Surface Ships

MAtoN	Mobile Marine Aids to Navigation

MPA	Marine Protected Area

MRCC	Maritime Rescue Coordination Centre

MRS 	Mandatory Ship Reporting Systems

NAVWARN	Navigational Warnings

NtM	Notice to Mariner

PIANC	World Association for Waterborne Transport Infrastructure

PSSA	Particularly Sensitive Sea Areas

ROV	Remotely Operated Vehicles

SAR	Search and Rescue

SOLAS	International Convention for the Safety of Life at Sea

UAV	Unmanned Aerial Drones

UxV	Unmanned Vehicles

VDES	VHF Data Exchange System

VTS	Vessel Traffic Services

references

Recommendation R1001 IALA MBS

Recommendation R1002 Risk Management for Marine Aids to Navigation

Recommendation R1010 Marine Spatial Planning

Recommendation R1016 Mobile Marine Aids to Navigation

Recommendation R0130 Categorisation and Availability Objectives for Short Range Aids to Navigation

Recommendation R0143 Provision of Virtual Aids to Navigation

Guideline G1018 Risk Management

Guideline G1036 Environmental Management in Aids to Navigation

Guideline G1081 Provision of Virtual Marine Aids to Navigation

Guideline G1122 The Use of Pictograms on Aids to Navigation

Guideline G1137 AtoN management in protected areas

Guideline G1154 Use of Mobile Marine Aids to Navigation