IALA

working paper on USE CASES for VTS Digital Communications

TO DO FOR IALA SECRETARY: KINDLY ADD TABLE OF CONTENTS ALSO FOR FIGURES

THIS IS A WORKING DOCUMENT

SEC: THE DOCUMENT WILL BE FULLY REVIEWED ONCE IT IS SUBMITTED BY THE WG TO THE COMMITTEE

Edition x.x

Date (of approval by Council)

urn:mrn:iala:pub:gnnnn

Revisions to this document are to be noted in the table prior to the issue of a revised document.

| Date | Details | Approval |
| --- | --- | --- |
|  | Updated Traffic Clearance Service use cases Added use cases for VTS Information Service Added use cases for VTS Reporting Service Updated service names according to GXXXX- Digital Communications |  |

Document PURPOSE

The Guideline on VTS Digital Communications [ G….] outlines digital communication functions and their associated services. It has been released in accordance with the official IALA procedure.

To provide practical insight, use cases are developed that illustrate these digital services in real-world scenarios. A single digital service as described in the guideline may have one or more use cases.

It is recognised that use cases can evolve over time. Therefore, they are not included in the guideline itself. Instead, the use cases are reviewed and approved by the IALA VTS community and published separately on the IALA website under Topical Matters.

The description of the functions and the services can be found in the Guideline on VTS Digital Communications [ G….].

DOCUMENT STRUCTURE

The VTS Digital Communications guideline describes operational functions. Each operational function has its own chapter in this document. The associated services are paragraphs within the chapters.

Figure 1 Document relationship

Document workflow

If a use case requires revision due to new insights, an input paper shall be prepared. This paper will be reviewed and discussed during a VTS Committee meeting in working group 1 (operations) and working group 2 (technology). Based on these discussions, the use cases may be updated and approved by the VTS Committee.

If an updated or new use case affects the technical implementation, an IALA action item shall be created. This action item will be included on the agenda of the next committee meeting.

Figure 2 Document workflow

ROUTE functions

DIGITAL Route exchange service use cases

Description is missing

- Figure 3 Example of arriving vessel to berth

Initial Sharing of the route from Vessel to VTS

Description: 	Vessel shares route with VTS before entering VTS area, leaving from berth/anchorage, departing from port/anchorage.

- Typical sequence:

- The route is planned in the planning station by the mariner

- Planning station crosschecks the route and upload route to the ECDIS/ECS

- The ECDIS/ECS should send the route to the “Route Exchange Function” before departure, but the route must be shared at latest according to local rules

- The Route Exchange Function checks that at least following information is included in the route

- Vessel Identification Information

- Waypoints (WP)

- Schedule / times of WPs

- Legs (including cross track distance limit (XTDL)

- Wheel over point (WOP)

- The Route Exchange Function sends this planned route to the VTS System

- The Route Exchange Function sends “received” acknowledgement automatically

- VTS System can display the route as needed to the VTS personnel

VTS gives route recommendation to vessel within a geographically defined area

Description: 	VTS gives route recommendation to vessel for example due to:

    - A certain part of the route is inaccessible, for example due to navigational danger, environmental conditions, or for monitoring and managing vessel traffic

      - Changing the geography of the route

      - Changing the ETA to a specific waypoint

    - Enhanced navigational assistance

      - Changing the radius of the turn

- Typical sequence:

- Vessel has already sent route to VTS

- VTS personnel creates the recommendation for vessel

  - VTS system can assist VTS personnel to create the route recommendation

- VTS system sends back the recommended route to ECDIS (planning station)

  - Route can contain changes to waypoints and/or schedule

- Vessel sends “route received” acknowledgement automatically

- One of the following

  - Vessel does not agree with changes

  - Vessel process the route and sends it to VTS

- The Route Exchange Service sends “received” acknowledgement automatically

- VTS System can display the route as needed to the VTS personnel

VTS requests route from vessel

Description: 	VTS has not received route from vessel and requests route from vessel for situational awareness and/or traffic management.

- Typical sequence:

- VTS request route from vessel

- Vessel sends route as requested

- The Route Exchange Service sends “received” acknowledgement automatically

- VTS System can display the route as needed to the VTS personnel with ability to highlight any changes

Vessel´s route changes

Description: 	Vessel wants to change its route

- Typical sequence:

- Vessel has already sent route to VTS

- Mariner makes changes to its route

- ECDIS sends updated information to VTS as monitored route

  - If VTS has requested updates according to use case 3.1.3 that request must be honored

- VTS system sends “received” acknowledgement automatically

- VTS System can display the route as needed to the VTS personnel with ability to highlight the changes

Vessel does not arrive to VTS area as planned

Description: 	Vessel changes route and does not arrive to VTS area as part of its voyage (use case 3.1.1)

- Typical sequence:

- Vessel has already sent route to VTS

- Mariner makes changes the route where no waypoints are located inside geometry area

- Vessel sends cancellation to VTS system by means as route cancelled message

- VTS system sends “received” acknowledgement automatically

- VTS System can display the route as needed to the VTS personnel with ability to highlight the changes/cancellation

VTS acknowledges the route

Description: 	VTS acknowledges vessel’s route without changes

- Typical sequence:

- Vessel has already sent route to VTS

- VTS system sends the route to Route Crosscheck Function if available to checks the route

- Route Crosscheck Service checks the route and marks it as compliant

  - If the route is not compliant go to use case 3.1.2 or 3.1.8

- VTS personnel takes in consideration the monitoring and managing vessel traffic and marks the route “ok” on the VTS system

  - If the route is not suitable go to use case 3.1.2

- VTS system sends VTS acknowledge message to ECDIS

- ECDIS can display the VTS acknowledged status of the monitored route to mariner

VTS personnel acknowledges the route

Description: 	VTS personnel acknowledges vessel’s route without changes

- Typical sequence:

- Vessel has already sent route to VTS

- VTS personnel checks the route and marks the route “ok” on the VTS system

  - VTS personnel also takes in consideration the monitoring and managing vessel traffic

  - If the route is not suitable go to use case 3.1.2 or 3.1.8

- VTS system sends VTS acknowledge message to ECDIS

- ECDIS can display the VTS acknowledged status of the route to mariner

VTS sends route back with comments

Description: 		VTS does not acknowledge vessel´s route and sends it back with comments

- Typical sequence:

- Vessel has already sent route to VTS

- Route Crosscheck Function or/and VTS personnel checks the route and finds issues with the route

- VTS personnel decides not to make a recommendation

- VTS sends part of the route back to the vessel with comments, which can be:

  - Recommendations (route modification readily to be used by the mariner, including human readable explanation why this recommendation is given)

  - Issues (including human readable text to explain the issue)

  - Errors (technical issues of the route, not necessarily human action)

  - Route incomplete (VTS decides there are too many issues to be explained to the mariner, so this route incomplete message is a request to the mariner to re-plan the route)

- Vessel changes its route [go to use case 4]

Vessel gets multiple changes to their route

Description: 	Vessel sends its route to many VTS areas and gets multiple change recommendations to their route.

- Typical sequence:

- Vessel has already sent route to multiple VTS areas

- VTS areas can give multiple recommendations to vessel´s route

  - VTS can send recommended route or/and comments (issues, errors)

- Vessel receives multiple change recommendations to their route and adjust it

DIGITAL Route Reference service

- With the Route Reference Service VTS and other service providers offer routes and waypoints, in electronic format. Route Reference Service is designed to assist mariners in their voyage planning to define the suitable route on commonly used passages, such as shipping lanes, approaches to ports, and coastal routes. Reference routes can be fully predefined and/or can be calculated based on predefined waypoints.

DIGITAL Route Crosscheck service

- The purpose of Route Crosscheck Service is to validate a planned or monitored route from the vessel and compare the information with expertise of the VTS and its information regarding the specific VTS area (traffic separation, depth, speed restriction, etc). When the VTS receives a route from a vessel the VTS should be able to execute a Route cross check. The cross-checking may be done before the vessel’s departure or before arrival at a certain geographical area (for example a VTS area). The cross-check may include Under Keel Clearance, air draft, no violation of no-go areas, Maritime Safety Information and compliance with mandatory routing.

DIGITAL Route Monitoring Service

The Route Monitoring Service is used to monitor vessels´ movement in vicinity and in the VTS area. Within this service the VTS may detect vessel´s deviating from their routes or schedules, allowing the VTS to intervene promptly in case of potential safety hazards and navigational issues.

TRAFFIC functions

DIGITAL Traffic Image Service

The Traffic Image Service enables the exchange of real-time traffic information between VTS and vessels. For example using sources like AIS, radar, or CCTV, it provides vessels and VTS with an up-to-date traffic picture and supports the creation of a common traffic image within the VTS area.

DIGITAL Intended Track Exchange Service

The intended track exchange service is designed to (automatically) broadcast the intended trajectory of the vessel covering a route segment of a certain amount of time which is significantly shorter than the complete planned route (minutes rather than hours). This intended trajectory may consist of the current track and speed or a limited set of waypoints of the route including time at each waypoint. Depending on the applied control of the vessel, for example: manual control, autopilot or track pilot it broadcasts the track and speed or a set of waypoints. Vessels share their intentions from vessel to vessel as well as from vessel to VTS. This allows vessels to avoid potential conflicts because this service provides the vessels insight in the developing traffic situation. With this service, the VTS receives all vessels intentions within the VTS area, allowing the VTS to intervene promptly in case of potential safety hazards and navigational issues.

DIGITAL Navigation SUPPORT Service

Digital Navigation Support Service supports VTS in responding to developing unsafe situations. These situations may include a vessel unsure of its position, deviating from its route, needing guidance to anchorage, experiencing equipment failure, facing severe weather, being at risk of grounding or collision, or requiring emergency support. The service provides navigational information, advice, warnings or instructions to individual vessels or to groups within the VTS area. All messages are transmitted reliably to ensure that vessels receive them and can maintain safe navigation. Add the severity eg. low, medium, high – every VTS is responsible to define the severity level on their area????? Need to discuss in Paris

VTS INFORMATION functions

DIGITAL VTS Reporting Service

- The VTS Reporting service is designed for vessels to report information to the VTS as specified by the VTS. This includes an arrival/departure report, position report and specific information about the vessel which affects ships traffic.

- Reporting requirements can be based on the IMO RESOLUTION A.851(20) but they can include VTS specific reporting requirements. Reporting requirements include reporting position and/or time.

VESSEL makes initial contact and receives reporting requirements

Description: 		Vessel subscribes VTS Reporting Service and receives all reporting requirements

- Typical sequence:

- Vessel subscribes VTS Reporting Service

- VTS system sends all reporting requirements automatically to vessel

- Vessel receives all reporting requirements

- Vessel system informs automatically VTS Reporting Service that reporting requirements are received and opened

VESSEL sends complete report

Description: 	Vessel sends report

Pre-conditions:	Vessel has all reporting requirements

- Typical sequence:

- Vessel reports requested information to VTS

- VTS receives the report

- VTS systems sends delivery acknowledgement automatically

- VTS validates the report

- VTS sends to vessel that report is according to requirements

VESSEL sends invalid report

Description: 	Vessel sends report that is not according to requirements

Pre-conditions:	Vessel has all reporting requirements

Typical sequence:

- Vessel sends the requested report to VTS

- VTS receives the report

- VTS systems sends delivery acknowledgement automatically

- VTS validates the report and finds that report is not according to requirements

- VTS system sends to the vessel that the report is not according to requirements and details which information need to be corrected

Post-conditions:	If vessel does not correct the report as requested other VTS procedures take place.

vts does not receive report at all

Description: 	VTS does not receive report

Pre-conditions:	Vessel has all reporting requirements

Typical sequence:

- VTS does not receive report at correct position or time

- After a preset time limit, other VTS procedures take place.

VESSEL sends updated report

- Description: 		Vessel updates requested report

- Pre-conditions:	Vessel has already sent the report according to requirements and wants to update information in the report.

Typical sequence:

- Vessel resends a required report with updated information

- VTS receives report

- VTS systems sends delivery acknowledgement automatically

- VTS system validates the information

  - Information is valid [go to use case 5.1.2]

  - Information is invalid [go to use case 5.1.3]

DIGITAL VTS Information Service

The aim of a VTS Information Service is to contribute to the safe and efficient navigation of vessels by delivering relevant information from VTS perspective. The service can be implemented to deliver all the information, or it can fulfil and/or enrich the data from other S-100 services. VTS Information Service contains structured messages. The content of the message field is structured or unstructured. All messages must start with message markers information, request, question or answer. The message can be sent to one or multiple vessels.

VTS providers should define which information will be provided via VTS Information Service based on for example risk assessment.

Message field includes one or more of the following elements:

Geometry + defined set of features

Time

Pictures + pdf

Form (eg questions that require structured answer)

Free text (including links)

Time frame

VTS informs vessel(s) without time frame

Description: 	VTS sends message to a vessel(s)

Example:	INFORMATION: Marine mammal(s) observed in a certain point and heading to certain direction.

- Typical sequence:

- VTS sends message to vessel(s) that includes

  - Message marker: information

  - Time: Without time frame

  - Message can be in a format that VTS provider chooses (see 5.2)

- Vessel system informs automatically VTS that information is received and opened

VTS informs vessel(s) with time frame

Description: 	VTS sends message to a vessel(s)

Example:	INFORMATION: Sailing competition in the VTS area from 1200 UTC to 1500UTC

- Typical sequence:

- VTS sends message to vessel(s) that includes

  - Message marker: information

  - Time: With time frame

  - Message can be in a format that VTS provider chooses (see 5.2)

- Vessel system informs automatically VTS that information is received and opened

- Circumstances changes and VTS updates the provided information

  - There can be multiple updates

- Vessel system informs automatically VTS that update is received and opened

- Vessel´s system cancels information automatically after the time frame has ended

VTS informs vessel(s) with unknown time frame

Description: 	VTS sends message to a vessel(s)

Example:		INFORMATION: Diving operation starting 1200UTC on the fairway. Ending time unknown.

- Typical sequence:

- VTS sends message to vessel(s) that includes

  - Message marker: information

  - Time: Unknown time frame

  - Message can be in a format that VTS provider chooses (see 5.2)

- Vessel system informs automatically VTS that information is received and opened

- Circumstances changes and VTS updates the provided information

  - There can be multiple updates

- Vessel system informs automatically VTS that update is received and opened

- Situations ends and VTS sends cancels the provided information

- Vessel system informs automatically VTS that cancellation is received and message has been deleted from the map

VTS asks question from vessel(s)

Description: 	VTS asks question from vessel(s).

Example:		QUESTION: What is your ETA to pilot boarding position

- Typical sequence:

- VTS asks question from vessel(s) that includes

  - Message marker: Question

  - Message can be in format that VTS provider chooses (see 5.2)

- Vessel system informs automatically VTS that question is received and opened

- Vessel answers to VTS that includes

  - Message marker: Answer

  - Message can be in format that vessel chooses (see 5.2)

- VTS system informs automatically vessel´s system that answer is received and opened

Vessel asks question from VTS

Description: 	Vessel asks question from VTS

Example:		QUESTION: What is my anchorage area

- Typical sequence:

- Vessel asks question from VTS that includes

  - Message marker: Question

  - Message can be in format that vessel provider chooses (see 5.2)

- VTS system informs automatically vessel´s system that question is received and opened

- VTS answers to vessel that includes

  - Message marker: Answer

  - Message can be in format that VTS chooses (see 5.2)

- Vessel system informs automatically VTS that answer is received and opened

VTS asks question(s) by using form

Description: 	VTS asks questions from vessel(s) by using form

Pre-conditions:	VTS has established validation rules

Example:		VTS wants to confirm pre-arrival information from vessel

- Typical sequence:

- VTS asks question from vessel(s) that includes

  - Message marker: Question(s)

  - Message is in form format

- Vessel system informs automatically VTS that question(s) is received and opened

- Vessel answers to VTS that includes

  - Message marker: Answer(s)

  - Each answer is related to question form format

- VTS system informs automatically vessel´s system that answer is received and opened

  - If the information received is not valid go to step 1 with explanation

  - Or go to use case 5.2.1

- Circumstances changes and vessel updates the provided information

  - There can be multiple updates

  - Go to step 4

Vessel informs VTS without time frame

Description: 	Vessel sends message to VTS

Example:	INFORMATION: Marine mammal(s) observed in a certain point and heading to certain direction.

- Typical sequence:

- Vessel sends message to VTS that includes

  - Message marker: information

  - Time: Without time frame

  - Message can be in a format that vessel chooses (see 5.2)

- VTS informs automatically vessel´s system that information is received and opened

Vessel informs VTS with time frame

Description: 	Vessel sends message to VTS

Example:	INFORMATION: Onboard training from 1600UTC to 1700UTC

- Typical sequence:

- Vessel sends message to VTS that includes

  - Message marker: information

  - Time: With time frame

  - Message can be in a format that vessel chooses (see 5.2)

- VTS informs automatically vessel´s system that information is received and opened

- Circumstances changes and vessel updates the provided information

  - There can be multiple updates

- VTS informs automatically vessel´s system that update is received and opened

- VTS cancels information automatically after the time frame has ended

Vessel informs vessel(s) with unknown time frame

Description: 	Vessel sends message to VTS

Example:		INFORMATION: Testing main engine onboard starting from 1400UTC. Ending time unknown.

- Typical sequence:

- Vessel sends message to VTS that includes

  - Message marker: information

  - Time: Unknown time frame

  - Message can be in a format that vessel chooses (see 5.2)

- VTS informs automatically vessel´s system that information is received and opened

- Circumstances changes and vessel updates the provided information

  - There can be multiple updates

- VTS system informs automatically vessel´s system that update is received and opened

- Situations ends and vessel sends cancels the provided information

- VTS informs automatically vessel´s system that cancellation is received and message has been deleted from the map

Vessel request information from VTS

Description: 	Vessel requires information to VTS

- Typical sequence:

- Vessel requires information from VTS for example “Request. Send pilot boarding place.”

- VTS system informs automatically vessel that question has been received and opened

- VTS applies to request

USE CASE: CANCEL

USE CASE: GEOFENCING

planning functions

digITAL Traffic Clearance Service

Traffic Clearance is asked from Traffic Clearance position e.g berth, VTS-area line, anchorage, buoy.  ETA/ETD from traffic clearance position.

- Digital Traffic Clearance refers to the process of ensuring that there is sufficient space and time for vessels to navigate safely through an area, taking into account other vessels, obstructions, regulatory and environmental factors. The Digital Traffic Clearance Service provides vessels with permission to proceed, set conditions and/or denies clearance. Within this digital service the VTS coordinates, authorises, and monitors the approach and passage of vessels through the area.

VTS gives traffic clearance to vessel

Description:	Vessel sends intended ETA or ETD to the VTS as a part of requesting a traffic clearance. VTS gives traffic clearance to vessel.

Typical sequence:

- The vessel requests traffic clearance with intended ETA or ETD

- VTS system has received information and sends received acknowledgement automatically to vessel

- VTS has opened request and sends read acknowledgement to vessel

- VTS gives a traffic clearance to vessel including the ETA/ETD time

- Vessel system has received information and sends received acknowledgement automatically to VTS

- Mariner has opened information and sends read acknowledgement to VTS

VTS denies traffic clearance

Description:	Vessel sends intended ETA or ETD to the VTS as a part of requesting a traffic clearance. VTS denies traffic clearance from vessel.

Typical sequence:

- The vessel requests traffic clearance with intended ETA or ETD

- VTS system has received information and sends received acknowledgement automatically to vessel

- VTS has opened request and sends read acknowledgement to vessel

- VTS denies traffic clearance from vessel with further instructions for actions (e.g contact VTS in 2 hours) and possibly explanation for denial

- Vessel system has received information and sends received acknowledgement automatically to VTS

- Mariner has opened information and sends read acknowledgement to VTS

- Go to use case (6.1.1)

VTS gives traffic clearance with a specific time

Description:	Vessel sends intended ETA or ETD to the VTS as a part of requesting a traffic clearance. VTS denies the request and gives a new time for vessel.

Typical sequence:

- The vessel requests traffic clearance with intended ETA or ETD

- VTS system has received information and sends received acknowledgement automatically to vessel

- VTS has opened request and sends read acknowledgement to vessel

- VTS gives a new traffic clearance time with possible explanation to vessel

- Vessel system has received information and sends received acknowledgement automatically to VTS

- Mariner has opened information and sends read acknowledgement to VTS

- Vessel sends acceptance or cancellation (use case 6.1.4 and use case 6.1.5) of the proposed new time

Cancellation of granted traffic clearance initiated by the vessel

Description:	Vessel has a granted traffic clearance from VTS as described above (use cases 6.1.1 and 6.1.3). Vessel cancels granted traffic clearance.

- Typical sequence:

- Vessel cancels granted traffic clearance with possible explanation

- VTS system has received information and sends received to vessel automatically

- VTS has opened cancellation and sends read acknowledgement to vessel

- Go to use case (6.1.1) if required

Cancellation of pending traffic clearance initiated by the vessel

Description:	Vessel has requested traffic clearance from VTS as described above and VTS has not yet granted the vessel´s traffic clearance request (use cases 6.1.1 step 2 and 6.1.3 step 2). Vessel cancels pending traffic clearance.

- Typical sequence:

- Vessel cancels pending traffic clearance with possible explanation

- VTS system has received information and sends received to vessel automatically

- VTS has opened cancellation and sends read acknowledgement to vessel

- Go to use case (6.1.1) if required

VTS updates traffic clearance

Description:	Vessel has a granted traffic clearance as described above (use cases 6.1.1 and 6.1.3). VTS updates granted traffic clearance.

Typical sequence:

- VTS updates granted traffic clearance with possible explanation

- Vessel system has received information and sends received acknowledgement automatically to VTS

- Mariner has opened information and sends read acknowledgement to VTS

VTS Cancels granted traffic clearance

Description:	Vessel has a granted traffic clearance from VTS as described above (use cases 6.1.1 and 6.1.3). VTS cancels granted traffic clearance.

- Typical sequence:

- VTS cancels granted traffic clearance with possible explanation

- Vessel system has received information and sends received to VTS automatically

- Mariner has opened information and sends read acknowledgement to VTS

- Go to use case (6.1.1) if required

DIGITAL Slot management service

- The Slot Management Services is designed to provide a time slots for vessels in advance to ensure safe, efficient, and organised movement within the VTS area. A time slot includes scheduling and allocation for vessel(s) within a VTS-area. The service includes assigning specific time slots for vessels' arrival, departure, or transit. The time slot may be based on for example weather and/or hydrologic conditions (for example tide, fog), port/area resources (for example berth, anchorage), traffic density, infrastructure (for example bridge, lock), type/size of vessel. This integrated approach enables the VTS to manage traffic flow while maintaining safety and operational efficiency.

VESSEL makes initial contact and receives time slot requirements

Description: 		Vessel subscribes Slot Management Service and receives requirements

- Typical sequence:

- Vessel subscribes Slot Management Service

- VTS system sends all planning requirements automatically to vessel

- Vessel receives requirements

- Vessel sends “received” acknowledgement automatically to VTS

VTS sends time slot for vessel

Description:	Prior to the arrival the VTS sends time slot for vessel

Pre-conditions:	Vessels have subscribed the VTS Information Service and VTS knows the ETA to certain point/s

Typical sequence:

- VTS sends time slot for vessel

- Vessel receives time slot and sends “received” acknowledgement automatically to VTS

Publishing the allocation schedules by VTS (CHINA MSA)

Description:	When the allocation schedules change, VTS synchronously publishes to the service.

Note: The unallocated items in the allocation schedules refer to the available resource plans for vessels to request. (The same below)

Typical sequence:

- VTS system publishes the latest allocation schedules (full or incremental) to the “Scheduling and Allocation Service”

- “Scheduling and Allocation Service” acknowledges automatically upon receipt

Querying the allocation schedules (CHINA MSA)

Description:	The service receives a query message and feedback on the query result.

Typical sequence:

- “Scheduling and Allocation Service” acknowledges automatically upon receiving the query message

- “Scheduling and Allocation Service” generates the query result message based on the allocation schedules

- “Scheduling and Allocation Service” sends the generated message to the initial receiving path

Allocating an anchorage (CHINA MSA)

Description:	Prior to the arrival of the vessel, the service receives the vessel’s request message of an anchorage, and VTS allocates an anchorage space and a time period for the vessel to use.

Typical sequence:

- “Scheduling and Allocation Service” acknowledges automatically upon receiving the vessel’s request message of an anchorage (ETA and ETD at anchorage)

- “Scheduling and Allocation Service” compares:

- If the request is suitable for the published available resource plans [go to step 3]

- If the request is not suitable, the service automatically generates a prompt message [go to step 8]

- “Scheduling and Allocation Service” sends the request message to VTS system

- VTS system acknowledges automatically upon receipt

- VTS system automatically or VTS personnel manually allocates a vacancy with a time period for the vessel to anchor, and updates the allocation schedules of anchorage

- VTS system publishes the latest allocation schedules of anchorage via use case 1

- “Scheduling and Allocation Service” generates the allocation message (start and end times of anchorage reservation) based on the latest allocation schedules

- “Scheduling and Allocation Service” sends the generated message to the initial receiving path

Cancelling the allocated schedule from vessel (CHINA MSA)

Description:	Due to changes in vessel’s voyage plan or route plan, the service receives the vessel’s cancel message, and VTS cancels the allocated schedule for the vessel.

Typical sequence:

- “Scheduling and Allocation Service” acknowledges automatically upon receiving the vessel’s cancel message

- “Scheduling and Allocation Service” sends the request message to VTS system

- VTS system acknowledges automatically upon receipt

- VTS system automatically or VTS personnel manually cancels the allocated slot or anchorage for the vessel, and updates the corresponding allocation schedules

- VTS system publishes the latest allocation schedules via use case 1

- “Scheduling and Allocation Service” generates the feedback message (cancellation completed) based on the latest allocation schedules

- “Scheduling and Allocation Service” sends the generated message to the initial receiving path

Cancelling the allocated schedule by VTS (CHINA MSA)

Description:	Due to special cases, such as receiving a cancellation notice from the relevant department, VTS cancels the allocated schedule for the vessel.

Typical sequence:

- VTS system automatically or VTS personnel manually cancels the allocated slot or anchorage for the vessel, and updates the corresponding allocation schedules

- VTS system publishes the latest allocation schedules via use case 1

- “Scheduling and Allocation Service” generates the notification message (cancelled schedule) based on the latest allocation schedules

- “Scheduling and Allocation Service” sends the generated message to the vessel

Reallocating slots of traffic flow (CHINA MSA)

Description:	When external influences are eliminated, VTS unfreezes the allocation schedules and reallocates traffic slots for those waiting vessels.

Typical sequence:

- VTS system automatically or VTS personnel manually unfreezes the allocation schedules of traffic slots

- VTS system automatically or VTS personnel manually reallocates passage slots for those waiting vessels, and updates the allocation schedules of traffic slots

- VTS system publishes the latest allocation schedules of traffic slots via use case 1

- “Scheduling and Allocation Service” generates the re-allocation messages corresponding to vessels based on the latest allocation schedules

- “Scheduling and Allocation Service” sends the generated messages to the relevant vessels

Modifying the allocated schedule from vessel (CHINA MSA)

Description:	Due to changes in vessel’s route schedule, the service receives the vessel’s modification message, and VTS cancels the allocated schedule for the vessel, and then allocates a new traffic slot or an anchorage resource.

Typical sequence:

- “Scheduling and Allocation Service” acknowledges automatically upon receiving the vessel’s modification message (new ETA/ETA and ETD)

- “Scheduling and Allocation Service” distributes the modification message, allocates new schedule via use case 3 or use case 4, and cancels the allocated schedule via use case 5.

DEFINITIONS (do we make them bold)

Traffic Clearance position

ETA

ETD

VTS=  can be system or human

VTS personnel = human

VTS system = machine

Vessel = can be system or human

Mariner = human

Vessel system = machine