Input paper: VTS60-8.3.2

Input paper for the following Committee(s): 	 				Purpose of paper:

□ARM	□ENG		□PAP					X  Input

□ENAV	XVTS							□  Information

Agenda item			8.3

Technical Domain / Task Number2	Task 1.3.1

Author(s) / Submitter(s)			China Maritime Safety Administration

# Proposal on Adding a VTS Digital Navigation Clearance Height Service

# Summary

This document, based on the practical operational requirements of VTS, identifies the need for exchanging navigation clearance height information between VTS and vessels in a standardized, structured digital format. It proposes the addition of a new VTS digital “Navigation Clearance Height Service”, and discusses associated use cases.

## Purpose of the document

The document aims at providing an input paper for task group to promote Task 1.3.1 on “Develop guidance on VTS digital communications (operational aspects)”.

## Related documents

IMO A.1158 (32) Guidelines for Vessel Traffic Services

IALA G1089 Provision of a VTS

IALA VTS59-12.1.5 draft Guideline on VTS Digital Communications_FINAL

IALA VTS59-12.2.1 Service Specification for UKC Digital Service 1.0

IALA VTS58-12.1.6 Use Cases on VTS Digital communications

IALA VTS58-12.2.3 Service Specification for route exchange ed1.1 rev2

IALA VTS57-12.3.6 Service specification for VTS traffic clearance 1.5_final

IALA VTS57-8.3.1 Proposal on the Description and Use Cases of VTS Scheduling and Allocation Service

# Background

The VTS Committee has been continuously advancing the research and development of VTS-specific digital services, and has successively conducted discussions on use cases and technical specifications for Route Exchange Service, Traffic Clearance Service, VTS Information Service, Under Keel Clearance Service, and Slot Management Service.

China MSA has been closely tracking the progress of this work. We have found that in the vertical dimension, in addition to the vessel's draught, its air draught (height above water) is also a critical factor for navigation safety. Vessel-bridge collisions pose one of the major risks to both maritime traffic safety and public safety. Accidents caused by excessive air draught may lead to severe consequences and losses. In recent years, typical global cases of this type include:

- In July 2020, a container vessel collided with the Willemsbrug bridge in Rotterdam, the Netherlands;

- In April 2022, a bulk carrier collided with the Gaoxinsha Bridge in Guangdong, China;

- In May 2025, a Mexican Navy training ship struck the Brooklyn Bridge in New York, USA;

- In March 2026, a container vessel collided with a railway bridge over the Rhine River in Neuss, Germany.

With the growth in vessel size and the increasing frequency of oversized cargo transport operations on water, facilities such as bridges and overhead cables spanning navigable waters have become critical factors that cannot be ignored for navigation safety. The effective use of the navigation clearance height beneath these facilities directly affects the safety and efficiency of vessel traffic within a VTS area. To address this need, current research on VTS-specific services lacks corresponding operational functions/services. It is therefore necessary to introduce a new VTS digital service—the Navigation Clearance Height Service—and to develop its use cases.

Meanwhile, the industry has also provided a data foundation for the implementation of the VTS digital “Navigation Clearance Height Service”. Some S-100 product specifications and data models, such as S-101, S-212 and S-421, have already taken into account relevant parameters in the vertical dimension (e.g., vertical clearance under bridge, air draught of vessel) during their development. It is thus feasible to add this new VTS digital service.

# Discussion

## Operational Basis

According to IMO resolution A.1158 (32) Guidelines for Vessel Traffic Services, the purpose of VTS can be achieved through “monitoring and managing ship traffic to ensure the safety and efficiency of ship movements”, which may include: “

- Planning ship movements in advance;

- Establishing a system of traffic clearances. ”

In Section 4.2 of IALA G1089 Provision of a VTS, it states that

“The monitoring and management of ship traffic to ensure the safety and efficiency of ship movements may include:

- Forward planning and prioritization of ship movements to prevent congestion or dangerous situations and improve overall efficiency.

- Establishing a system of traffic clearances.”

In waters where navigation clearance height is restricted, the vertical dimension above the water surface is a critical component of navigation safety. Providing navigation clearance height information not only assists VTS in achieving its purpose by monitoring and managing vessel traffic to ensure safety and efficiency, but also enables vessels to formulate rational route plans beforehand. Furthermore, it helps protect bridges, overhead cables, and other facilities, reducing the risk of collision.

## Operational Requirements

Navigation clearance height refers to the vertical distance between the underside of bridges, overhead cables, or other facilities and the water surface, which allows for the safe passage of vessels. Vessels must maintain at least the required safety margins when passing such areas.

In practice, considering the combined effect of water levels and tides, a vessel's air draught when passing under bridges or overhead cables must be lower than the navigation clearance height, with a sufficient safety margin maintained. The safety margin is set by the relevant authority and may not be a constant value.

The new VTS digital “Navigation Clearance Height Service” can transmit navigation clearance height information between vessels and VTS in a standardized, structured digital format.

For vessels, the Navigation Clearance Height Service may be used to:

- calculate the maximum allowable navigation clearance height for the intended route,  i.e., the maximum air draught of the vessel;

- calculate the date/time windows available for the vessel’s safe passage under bridges, overhead cables, and other designated facilities, after its air draught is confirmed.

For VTS, the Navigation Clearance Height Service may be used to:

- validate vessels’ route plans;

- compare vessel dimensions against the navigation clearance heights at corresponding times, in order to monitor and manage vessel traffic;

- obtain the real-time and predicted navigation clearance height information for each bridge and overhead cable within a VTS area, in order to provide navigation support service when needed.

This proposal provides some use cases in the annex for reference.

## Relationship with other VTS-specific Services

### Relationship with Under Keel Clearance Service

"Navigation Clearance Height Service" and "Under Keel Clearance Service" share similarities. Both services provide information related to the vertical dimension of navigation safety in restricted waters – the former focuses on the height above the water surface, while the latter concerns the depth below the water surface. They can be used independently, and sometimes in combination.

For example, when preparing route plan, a vessel needs to obtain relevant information in advance based on the restrictions of the VTS area: if the planned route is to pass through an area with height restrictions, "Navigation Clearance Height Service" is invoked; if it is to pass through an area with depth restrictions, "Under Keel Clearance Service" is invoked. When both height and depth restrictions exist in the same area, the two services shall be used combined for calculation and analysis.

### Relationship with Slot Management Service

In practice, there may be a priority hierarchy between "Navigation Clearance Height Service" and "Slot Management Service".

For example, regarding the time window for a vessel to pass through a height-restricted area (e.g., a bridge), the information provided by "Navigation Clearance Height Service" is related to natural environmental conditions (such as water levels) and is derived from relevant data (e.g., S-104). In contrast, the information provided by "Slot Management Service" comes from the VTS's division and assignment of navigable time slots. The vessel should pass through that area following the time window provided by "Slot Management Service", unless it affects the safety of vessel navigation.

In general, digital services involving active VTS management have a higher priority.

### Relationship with Traffic Clearance Service

"Navigation Clearance Height Service" can be used in conjunction with "Traffic Clearance Service".

For example, when a vessel applies to VTS via "Traffic Clearance Service" for passage through a height-restricted area, VTS may use the information provided by "Navigation Clearance Height Service", together with vessel dimensions and other parameters, to comprehensively assess the vessel's capability for safe passage within a specific time frame, and then inform the vessel of the result through "Traffic Clearance Service".

# Action requested of the Committee

The Committee is requested to:

1.	consider the contents of this document;

2.	take actions as appropriate.

# ANNEX

use cases of navigation clearance height service

In this document, the "maximum allowable navigation clearance height" refers to the navigation clearance height corresponding to a water level set by VTS above the lowest navigable water level for safety considerations.

Use Case 1 – Vessel requests information of predicted maximum allowable navigation clearance height from a VTS area (pre-plan)

Description – Vessel fetches pre-planning information from VTS. In the pre-planning phase, a vessel requests a predicted maximum allowable navigation clearance height of bridges or overhead cables, typically days or weeks in advance. The predicted data is calculated based on forecast information and may not be very accurate. In this use case, the service could return a single pre-plan dataset, and generally no updates are required.

Pre-conditions – VTS has obtained the vessel’s route plan, with at least waypoints and schedules.

Typical sequence:

Vessel sends the request for a predicted maximum allowable navigation clearance height for its route plan.

“Navigation Clearance Height Service” automatically informs vessel that the data is received.

“Navigation Clearance Height Service” calculates the pre-plan data by using route plan, water level (astronomical and/or upstream inflow), limits of bridges or overhead cables, and safety margins.

“Navigation Clearance Height Service” sends the calculation result to vessel, including:

- Purpose: pre-plan

- Calculation Requested: dataset of “preDate, premaxallowableClearanceHeight, bridgeMRN/aloftcableMRN, bridgeOpeningMRN[optional] “

- If the vessel’s route involves multiple height-restricted areas, the value of the predicted allowable maximum navigation clearance height shall be the minimum among them.

Vessel system automatically informs “Navigation Clearance Height Service” that the data is received.

Use Case 2 – Vessel requests information of date window from a VTS area (pre-plan)

Description – Vessel fetches pre-planning information from VTS. In the pre-planning phase, a vessel requests a date window via bridges or overhead cables, typically days or weeks in advance. In this use case, the service could return the dataset containing a date window, which is closest to the schedule.

Pre-conditions – VTS has obtained the vessel’s route plan, with at least the freshwater air draught, waypoints and schedules.

Typical sequence:

- Vessel sends the request for a date window for its route plan.

“Navigation Clearance Height Service” automatically informs vessel that the data is received.

“Navigation Clearance Height Service” calculates the pre-plan data by using the vessel’s freshwater air draught and route plan, water levels (astronomical and/or upstream inflow), limits of bridges or overhead cables, and safety margins.

If the air draught exceeds the predicted maximum allowable navigation clearance height on every date within a date range, then “Navigation Clearance Height Service” sends the “None” result to vessel.

Else, “Navigation Clearance Height Service” sends the calculation result to vessel, including:

- Purpose: pre-plan

- Calculation Requested: dataset of “startDate, endDate, bridgeMRN/aloftcableMRN, bridgeOpeningMRN[optional]“

Vessel system automatically informs “Navigation Clearance Height Service” that the data is received.

Use Case 3 – Vessel requests information of time window(s) from a VTS area (execution plan)

Description – Vessel fetches executable information from VTS. In the execution plan phase, a vessel requests time window(s) via bridges or overhead cables, typically 0~24 hours before the vessel entering into the VTS area. Hydro-meteorological information will be more accurate and reliable. The service could return the dataset containing time window(s) for the scheduled date, typically specified in hours.

Pre-conditions – VTS has obtained the vessel’s route plan, with at least the freshwater air draught, waypoints and schedules.

Typical sequence:

- Vessel sends the request for time window(s) for its route plan.

“Navigation Clearance Height Service” automatically informs vessel that the data is received.

“Navigation Clearance Height Service” calculates the execution plan data for each height-restricted area, by using the vessel’s freshwater air draught and route plan, water level (astronomical and/or upstream inflow), limits of bridges or overhead cables, and safety margins.

If the air draught﹥the maximum allowable navigation clearance height, then “Navigation Clearance Height Service” sends the “None” result to vessel.

If the air draught﹤the navigation clearance height without time frame, then “Navigation Clearance Height Service” sends the calculation result to vessel, including:

- Purpose: execution-plan

- Calculation Requested: dataset(s) of “startTime[=all], endTime[=all], bridgeMRN/aloftcableMRN, bridgeOpeningMRN[optional]“

If the navigation clearance height without time frame﹤the air draught﹤the maximum allowable navigation clearance height, then “Navigation Clearance Height Service” sends the calculation result to vessel, including:

- Purpose: execution-plan

- Calculation Requested: dataset(s) of “startTime, endTime, bridgeMRN/aloftcableMRN, bridgeOpeningMRN[optional] “

Vessel system automatically informs “Navigation Clearance Height Service” that the data is received.

Use Case 4 – VTS gets information of navigation clearance height within the VTS area

Description – VTS obtains the time-series navigation clearance height information for each bridge or overhead cable within the VTS area.

Typical sequence:

- “Navigation Clearance Height Service” calculates the data for each height-restricted area of navigation clearance height at fixed time intervals, by using water levels, limits of bridges or overhead cables, and safety margins. The default is to start from the current moment.

“Navigation Clearance Height Service” sends the calculation result to VTS system, including:

- Purpose: overall

- Calculation Requested: datasets of “timeSequence, timeseriesClearanceHeight,  bridgeMRN/aloftcableMRN, bridgeOpeningMRN[optional] “

VTS system automatically informs “Navigation Clearance Height Service” that the data is received.

Use Case 5 – VTS publishes daily information of navigation clearance height within the VTS area

Description – VTS publishes the navigation clearance height without time frame and the maximum allowable navigation clearance height for each bridge or overhead cable within the VTS area for the day, where “the day” means local time 00:00:00 to 23:59:59.

Typical sequence:

- “Navigation Clearance Height Service” calculates the data for each height-restricted area of navigation clearance height for the day, by using water levels on the day, limits of bridges or overhead cables, and safety margins. The day refers to local time from 00:00:00 to 23:59:59.

“Navigation Clearance Height Service” sends the calculation result to VTS system, including:

- Purpose: the-day

- Calculation Requested: datasets of “localDate, notimeframeClearanceHeight, maxallowableClearanceHeight, bridgeMRN/aloftcableMRN, bridgeOpeningMRN[optional] “

VTS system automatically informs “Navigation Clearance Height Service” that the data is received.

VTS publishes the information of navigation clearance height on a regular basis.

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain