Input paper:   VTS60-8.3.3

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM	□  ENG		□  PAP				X  Input

□  ENAV	X  VTS							□  Information

Agenda item 			8.3

Technical Domain / Task Number 2	Task 1.3.1

Author(s) / Submitter(s)			China Maritime Safety Administration

# Proposal to develop Pilotage-related Operational Information Exchange Use Cases for Digital Slot Management Service

# Summary

This input paper proposes that the VTS Committee consider including pilotage information exchange as a new use case area in the Use Cases for VTS Digital Communications.

From the VTS operational perspective, pilotage-related information, such as whether pilotage has been requested, whether the pilotage request has been confirmed, whether pilotage has been arranged, delayed, cancelled or completed, may have a direct impact on traffic planning, vessel sequencing, traffic clearance, route monitoring, berth coordination, towage coordination, UKC planning and interaction with allied services.

This proposal is limited to use cases for the exchange of pilotage-related information relevant to VTS operations. The preliminary use cases requiring consideration are set out in the Annex for reference.

## Purpose of the document

The purpose of this document is to:

- identify the relevance of pilotage-related information exchange to VTS operations;

2. propose that pilotage-related information exchange be considered for inclusion in the Use Cases for VTS Digital Communications;

3. provide preliminary operational use cases for consideration by WG1;

4. clarify the scope and responsibility boundary of the proposed work; and

5. invite WG2 to provide input on potential technical implications, as appropriate.

## Related documents

IMO Resolution A.1158(32), Guidelines for Vessel Traffic Services

IMO MSC.1/Circ.1610/Rev.1, Initial descriptions of Maritime Services in the context of e-navigation

IMO MSC.1/Circ.1595, E-navigation Strategy Implementation Plan

IMO FAL.5/Circ.42/Rev.4, Guidelines for setting up a maritime single window

IMO FAL.5/Circ.52, Data for Port Calls

IMO Just In Time Arrival Guide, 2020

IALA Guideline G1128, The Specification of e-Navigation Technical Services, version 1.7, 2025

IALA Guideline G1089, Provision of a VTS, Edition 2.0, 2022

IALA Guideline G1143, Unique Identifiers for Maritime Resources (MRN), 2022

IALA Guideline G1183, Provision of MCP Identities, 2022

IALA Service Specification for UKC Digital Service, version 1.0

IALA Service Specification for VTS - Vessel Route Exchange, version 1.0

IALA Service Specification for VTS Traffic Clearance, version 1.5

IHO S-127, Marine Traffic Management

# Background

In recent years, IALA digitalisation work has made significant progress in areas such as Route Exchange Service, VTS Information Service, Traffic Clearance Service and UKC Digital Service. These developments provide an important basis for structured digital information exchange between ship and shore. However, there is currently no dedicated technical service specification addressing pilotage-related information exchange in the VTS context.

According to IMO Resolution A.1158(32), one of the important objectives of VTS is to improve the safety and efficiency of vessel traffic. In many ports and restricted waters, the availability and timeliness of pilotage services are important factors affecting traffic organisation and navigational safety.

IMO MSC.1/Circ.1610/Rev.1 identifies Pilotage Service as an important Maritime Service within the e-navigation framework. It indicates that pilotage service should support information exchange between ships, pilotage organisations and shore-based organisations in order to ensure that pilotage activities are conducted safely and efficiently. Meanwhile, IMO MSC.1/Circ.1595 proposes to enhance the safety and efficiency of berth-to-berth navigation and related services through the harmonised collection, integration, exchange, presentation and analysis of maritime information. Pilotage service is one of the key shore-based services in the berth-to-berth navigation process.

In addition, the IMO Just In Time Arrival Guide identifies the Pilot Boarding Place as the first key point where a ship enters the port service chain, and as an important basis for port coordination and Just-In-Time Arrival. The readiness of the pilot, the accuracy of the pilot boarding time and the timely updating of the pilotage plan may directly affect ship speed optimisation, waiting time at anchorage, tug scheduling, berth resource utilisation and the overall efficiency of port operations.

IMO FAL.5/Circ.52 further includes data such as estimated time of arrival at the Pilot Boarding Place, planned time of arrival at the Pilot Boarding Place, actual time of arrival at the Pilot Boarding Place and requested time of arrival at the Pilot Boarding Place. This indicates that pilotage-related data has become an important component of international maritime digitalisation.

At present, however, many ports and pilotage organisations still rely on telephone, VHF voice communication, e-mail, local dispatch systems and other arrangements for the exchange of pilotage-related information. A lack of common standards between these systems makes it difficult to achieve interoperability with the VTS digital service framework.

The absence of a common framework for pilotage-related information exchange may lead to inconsistent information exchange patterns between ports; limited interoperability between ships, pilotage organisations and VTS centres; difficulty in integrating with Route Exchange, Traffic Clearance and UKC Service; inability to automatically synchronise pilotage plans following ETA changes; excessive reliance on voice communication and manual coordination; limited support for JIT Arrival and port emission reduction; and difficulties in supporting future machine-to-machine communication and MASS development.

Within the S-100 framework, S-127 Marine Traffic Management may provide static or semi-static information related to pilotage, such as pilotage areas, pilot boarding places, contact information and instructions for using pilotage services. However, S-127 does not fully cover dynamic pilotage-related operational information, such as pilotage requests, acknowledgments of pilotage requests, pilotage arrangements, pilot boarding time, pilotage delays, pilotage cancellations and pilotage completion.

This indicates that a gap remains between existing S-100 product specifications and the operational needs of VTS. S-127 may tell ships where pilotage services are available and how to use them, but it does not fully address the dynamic operational question of what the current pilotage status of a particular ship is and whether that status affects VTS traffic organisation.

It is therefore necessary to further consider operational use cases for pilotage-related information exchange within the framework of VTS Digital Communications. This work may complement the static and semi-static pilotage information provided by S-127, and focus on dynamic pilotage-related information exchange associated with VTS traffic planning, traffic clearance, route exchange, UKC windows, berth coordination and coordinated operations.

# Discussion

Digital Slot Management Service is intended to allocate time slots for vessel arrivals, departures and transits in order to ensure safe, efficient and organised vessel movements within a VTS area. According to the draft IALA Guideline on VTS Digital Communications, slot allocation may depend on a range of operational constraints, including environmental conditions, UKC, traffic density, port resources and infrastructure.

Pilotage is one of the key operational resources affecting the availability and execution of a vessel’s allocated time slot. Changes in pilot assignment, pilot boarding time, pilot availability or pilotage status frequently require adjustment of vessel arrival schedules, traffic sequencing and traffic clearance decisions. Consequently, pilotage-related operational information has a direct influence on the planning and coordination processes performed under the Digital Slot Management Service.

Introducing standardised use cases for pilotage-related information exchange under the Digital Slot Management Service would enable VTS, pilot organisations and participating vessels to maintain a common operational picture of slot availability and execution. This would support dynamic slot allocation, reduce delays caused by manual coordination, improve traffic planning efficiency and strengthen the digital coordination of vessel movements within the VTS area.

Also, as outlined in the background above, pilotage-related information may directly affect the planning, monitoring and management of vessel traffic from a VTS perspective. It may intervene at several stages of the vessel arrival and departure process, including pre-arrival planning, confirmation of pilotage information, changes or cancellation of the pilotage plan, abnormal situations before the commencement of pilotage, reports during pilotage, pilot handover and completion of pilotage activity.

The time-based use case nodes are illustrated below.

Figure 1 - Time-based use case nodes for pilotage-related information exchange

The figure illustrates how pilotage-related information exchange may support VTS throughout the vessel arrival, pilotage and completion process. The proposed use cases are intended to identify where information exchange may affect VTS planning, monitoring and management, without defining pilotage procedures or changing the responsibilities of any party.

# Action requested of the Committee

The Committee is requested to:

- invite WG1 to consider and further develop the preliminary operational use cases set out in Annex A;

2.invite WG2 to provide input on potential technical implications, as appropriate; and take further action as appropriate.

# ANNEX A

# Preliminary use cases for pilotage information exchange

General

This annex describes operational use cases for pilotage-related information exchange within the framework of VTS Digital Communications. It is intended to complement, rather than duplicate, the information already covered by existing S-100 product specifications.

Within the S-100 framework, S-127 Marine Traffic Management provides static and semi-static information related to pilotage, including pilotage areas, pilot boarding places, contact information and instructions for using pilotage services. However, S-127 does not define the dynamic operational information required to support real-time coordination between VTS, pilot organisations and vessels during pilotage operations.

The use cases described in this annex focus on the exchange of dynamic pilotage-related operational information, such as VTS receive pilotage dispatch information, pilotage acknowledgements, pilotage plan change or cancellation, abnormal situation before or during pilotage, completion of pilotage. These operational exchanges support VTS planning and coordination activities, particularly within the Digital Slot Management Service, and contribute to traffic organisation, traffic clearance, route execution, UKC management, berth coordination and other collaborative operational processes.

The use cases defined in this annex address the exchange of pilotage-related operational information rather than pilotage procedures themselves, and do not alter the respective responsibilities of VTS or pilot organisations. Instead, they provide a common digital communication framework for standardised information exchange that supports VTS traffic planning, coordination and the digital management of vessel movements.

Actors

Depending on national or local arrangements, the following actors or functional roles may be involved:

•	vessel;

•	master / bridge team;

•	VTS centre;

•	VTS operator;

•	pilotage organisation or pilot;

•	pilot transfer assets, such as pilot boat or helicopter, where applicable.

•	Pilotage Dispatch System

UC-1 VTS receives pilotage dispatch information

Description

The pilotage organisation or other relevant party has received a request for pilotage service from a vessel or its agent and has preliminarily arranged the vessel pilotage plan. VTS receives pilotage planning or dispatch information from a pilotage organisation, pilot dispatch system or other authorised source. Information flow: pilotage organisation to VTS.

Typical sequence

VTS receives pilotage dispatch information. Information that may be exchanged:

•	pilot boarding place;

•	pilot boarding time;

•	pilotage not yet arranged;

•	pilot assigned;

•	pilot changed;

•	no pilot currently available;

•	pilotage pending;

•	pilotage delayed;

•	pilotage not possible;

•	pilotage direction, such as inbound, outbound or shifting;

•	applicable operational remarks, such as the side on which the pilot ladder should be rigged and the height above the water.

UC-2 Vessel confirms pilotage information with VTS before port entry or arrival

Description

A vessel confirms pilotage-related information with VTS before arriving in the VTS area, port approach waters or at the pilot boarding place. Information flow: VTS to/from vessel.

Typical sequence

Vessel confirm pilotage plan. Information that may be exchanged:

•	vessel identification;

•	ETA at pilot boarding place;

•	requested pilot boarding place;

•	vessel draught;

•	destination or berth;

•	route or schedule reference;

•	whether pilotage has been requested;

•	whether pilotage has been confirmed;

•	other information relevant to pilotage arrangements.

UC-3 Pilotage plan change or cancellation

Description

The pilot boarding plan is changed or cancelled due to changes in the vessel movement plan. Such as ETA changes, weather, traffic situation, UKC window, berth readiness, pilot availability or other operational reasons. Information flow: vessel to/from VTS, VTS to/from pilotage organisation, and pilotage organisation to/from vessel.

Typical sequence

Pilotage plan change or cancellation. Information that may be exchanged:

•	original planned pilot boarding time;

•	revised pilot boarding time;

•	reason for revision;

•	expected delay;

•	operational impact;

•	pilotage cancellation.

UC-4 Abnormal situation report before commencement of pilotage

Description

An abnormal or safety-related pilotage situation occurs before commencement of pilotage causes the pilotage plan to be suspended or terminated, and the relevant information is reported to the VTS centre or pilotage station in a timely manner.

Typical sequence

Abnormal situation report before commencement of pilotage. Information that may be exchanged:

•	pilot unable to board;

•	pilot boat failure;

•	unsafe pilot boarding conditions;

•	adverse weather at the pilot boarding place;

•	vessel defect affecting pilotage;

•	vessel not ready to receive the pilot;

•	pilotage cancelled after vessel movement has commenced;

•	pilotage status affecting other vessels in the traffic sequence;

•	pilot illness or the master relieving the pilot of duty.

UC-5 Situation reports during pilotage

Description

During pilotage, abnormal operational or environmental situations are reported to the VTS centre or pilotage station in a timely manner. VTS assesses the impact on traffic safety and efficiency according to the type of incident and takes action in accordance with local safety management requirements.

Typical sequence

An abnormal navigational or environmental situation occurs during pilotage. Information that may be exchanged:

•	oil pollution position;

•	vessel not under command;

•	man overboard position;

UC-6 Pilot handover

Description

According to local regulations or operational requirements, a single pilot cannot complete the whole pilotage service from the first pilot boarding place to the berth, and pilot handover is required during the passage.

Typical sequence

Pilot handover. Information that may be exchanged:

•	pilot handover position;

•	next pilot boarding place;

•	next pilot boarding time;

•	method of boarding for the next pilot, such as pilot boat or helicopter;

•	disembarkation position of the pilot on board;

•	disembarkation time of the pilot on board;

•	method of disembarkation for the pilot on board, such as pilot boat or helicopter.

UC-7 End of pilotage

Description

After pilotage is completed, cancelled or otherwise ended, the relevant pilotage record is archived.

Typical sequence

End of pilotage. Information that may be exchanged:

•	pilotage completed;

•	pilotage cancelled;

•	archive instruction, where applicable.

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain